| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0521 |
| --- | --- |

| Title: | BoG report on Adaptive Loop Filter (ALF) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document of JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | T. Yamakage |  | tomoo.yamakage@toshiba.co.jp |
| Source: | BoG on in-loop filtering (ALF) |  |  |

_____________________________

# Abstract

BoG on in-loop filtering (ALF) has studied input contributions related to ALF.

Date: 11 July 2012

Time: 15:15-20:40

Number of attendees: approximately 25 people

Room: 357

This is a BoG report on ALF.  Several normative changes are recommended to be adopted in WD8/HM8.  Several non-normative encoder enhancements are recommended to be adopted in HM8.

# List of studied input documents

(Normative changes)

JCTVC-J0036 ALF coefficient coding with a single k-table [K. Ugur (Nokia), M. Budagavi (TI)]

Abstract:

This contribution proposes to remove position dependent coding of coefficients and instead use EG(0) for all the coefficients. Two versions of signed EG(0) VLC were tested – (1) Unsigned EG(0)+separate sign and (2) se(v). The results show that there is 0.0%-0.1% change in coding efficiency.

Benefit:

Luma & chroma coefficient coding consistent

Removes kes(v) parsing process entirely from the specification

Removes position dependency k-tables

Coding efficiency:

(1) 0.06% loss proposal 1 (Unsigned EG(0) + separate sign)

(2) 0.07% loss proposal 2 (se(v) – Signed Exp-Golobm coding)

Cross-check:

Originally, this was proposed by Nokia.  Later, TI jointly proposed.  TI and Nokia cross-checked the results individually.

Availability of text:

Available in the contribution.

Discussion:

kes(v) is used in other place, so no hardware reduction.

Current text has differences leading zero or leading one, but can be unified.

Same coding engine is used as coefficient level coding (but ALF coef. is signalled in APS).

There is no prediction in coding, therefore, EG(k) is preferable to compensate this.

Since JCTVC-J0346 is relevant, revisit after reviewing JCTVC-J0346.

JCTVC-J0346 Unifying ALF coefficient coding with coeff_abs_level_remaining coding [J. Lou, Y. Yu, L. Wang (Motorola Mobility)]

JCTVC-J0434 Cross-check for J0346 on Unifying ALF coefficient coding with coeff_abs_level_remaining coding [Zhan Ma, Felix C. A. Fernandes (Samsung)] [late] [miss]

JCTVC-J0493 Cross-check of ALF Coefficient Coding in JCTVC-J0346 [W.-S. Kim (TI)] [late]

Abstract:

In the current HEVC, fixed k-parameter Exp-Golomb code is used for ALF coefficient binarization. However, k-th Exp-Golomb code is only used for ALF coefficient coding which introduces extra complexity. It is proposed to unify the ALF coefficient coding with coeff_abs_level_remaining coding.

There are three options in this contribution, but discussion was focused on scheme 3; Both the Luma and Chroma ALF coefficients are binarized (CABAC binarization process) with a unary code and a variable length code with parameter 3.

Benefit:

Simplification and negligible loss (on average) of coding efficiency.

Coding efficiency:

QP=22-37, negligible loss in luma, gain in chroma (0.2%)

QP=32-47, less than 0.1% loss in luma, more gain in chroma (0.4%)

Cross-check:

TI cross-checked, but no opinion.

Availability of text:

Available in the contribution.

Discussion:

Better than current one, but new coding is not preferable (TU+fixed length).

Sharing CABAC engine for coefficients and header is not preferable.

Do we have any information about ALF header information size? No.  homework

An expert checked the size of ALF header using Kimono 1080p LB, QP22, 100 frames.

The ratio of those two approaches was around 15% increase in bits.

And usual range of bits used in coeff coding was around 1000-2000 bits.

Position dependency is not necessary.

Unify luma/chroma syntax process is preferable.

Visit one of the text editors about kes() (Kemal and In Suk).  Among those two proposals, one expert expressed that the cleaner text (J0036) is preferable if we will modify the current text.  Only concern is coding loss (less than 0.1% on average).  The editor suggested J0036.

Recommendation: Revisit the adoption of JCTVC-J0036 (se(v) syntax).

To confirm whether these much increase in APS signaling is ok before adopting se(v) instead of EG(k) is requested during drafting the BoG report. (If it is not an issue, the BoG recommends to adopt JCTVC-J0036.)

JCTVC-J0337 Fix for ALF Padding Process [P. Chen, W. Wan (Broadcom)]

JCTVC-J0427 Crosscheck of JCTVC-J0337: Fix for ALF Padding Process [M. Budagavi (TI)] [late]

Abstract:

Virtual boundary processing has been adopted into the HEVC draft text to remove the line buffer requirement for ALF processing in LCU-based decoder implementations. The idea behind virtual boundary processing is to adjust the filter support depending on the filter location such that the last several LCU rows do not need to be stored in line buffers and wait to process these rows until the bottom neighboring LCUs become available. The current padding defined for chroma processing contradicts this purpose.

A modification of the padding process for chroma is proposed to remove the current dependency and resulting line buffer requirement. When chroma top edge pixels are extrapolated upwards, it is proposed to extrapolate by two rows, except when the top edge is also the picture boundary, where it is proposed to extrapolate by three rows.

WD7						Proposal

Benefit:

Removal of the current dependency in chroma ALF padding process.

Coding efficiency:

No loss.

Cross-check:

Source code and proposed text changes were checked and found to match. BD-Rate results matched too.  Positive to support.

Availability of text:

Available in the contribution.  Only two characters need to be modified.

Discussion:

Fine slice granularity is not supported.

Since JCTVC-J0050 is relevant, revisit after reviewing JCTVC-J0050.

JCTVC-J0050 AHG6: ALF with modified padding process [C.-Y. Tsai, C.-Y. Chen, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-J0325 Crosscheck of J0050: AHG6: ALF with modified padding process [M. Budagavi (TI)] [late]

Abstract:

Another solution for chroma padding.  (1) A later block in processing order has a lower priority than a prior block to extend boundary samples when the two blocks have an overlapping to-be-padded area (i.e., only use LCU0).

Additionally, (2) the horizontal size and the vertical size of the ALF filter shape are nine samples and seven samples, respectively. Therefore, it is proposed to change the number of padded samples in the vertical direction from four to three, while keeping the number of padded samples in the horizontal unchanged as four. Only seven lines of modifications are required in the HM software.

WD7						Proposal

Benefit:

Can work with FGS. (But we do not support FGS in Main profile at this moment.)

Coding efficiency:

No loss.

Cross-check:

BD-Rate results were observed to match.

Availability of text:

Available in the contribution.

Discussion:

If filter shape is 7x5, there is no problem.

The purpose to fix chroma padding process is the same as JCTVC-J0337.

Recommendation: If FGS is not supported in the specification text, J0337 is recommended for vertical padding since it is very simple modification.  If FGS is supported in the specification text, J0050 is recommended for vertical padding.  For horizontal padding, J0050 (2) is recommended for both luma and chroma.

JCTVC-J0266 AHG6: Modification to loop filtering across slice boundaries [S. Esenlik, M. Narroschke, T. Wedi (Panasonic)]

JCTVC-J0448 AHG6: Cross check of Modification to loop filtering across slice boundaries (JCTVC-J0266) [T. Ikai (Sharp)] [late] [miss]

Abstract:

The proposal advocates aligning all three loop filters by modifying the SAO and ALF slice boundary control operation. The problem is emphasized especially in the case of top-to-down gradual decoder refresh operation (the most common GDR implementation) where slices in a frame are refreshed starting from the top-left corner of a frame. It is proposed that all three loop filters are controlled jointly at the top/left slice boundaries by the slice_loop_filter_across_slices_enabled_flag in the slice header and not at the bottom/right slice boundaries. In other words If slice_loop_filter_across_slices_enabled_flag is equal to 0 in a slice, then the following are applied according to the proposal:

- Deblocking is disabled at both sides of the top/left slice boundary. (No change with respect to current HEVC [2])

- SAO is controlled at both sides of the top/left slice boundary.

- Padding is used for ALF at both sides of the top/left slice boundary.

Benefit:

Typical GDR implementation support with reasonable cross_slice processing control.

Do not need to store control information of cross boundary.

Same rule (as AVC) for disabling cross boundary processing control for DF, SAO and ALF.

Coding efficiency:

No loss.

Cross-check:

Partial results perfectly matched (simulation on-going).  Support this proposal.

Availability of text:

Available in the contribution.

Discussion:

An editor suggested to cleanup text. Proponent will work for text revision with the editor.

The revised text will be circulated to the interested parties.

Recommendation: Adopt this proposal.

JCTVC-J0320 Multicore-friendly ALF luma region coding [M. Tikekar, M. Budagavi, V. Sze (TI)]

JCTVC-J0399 AHG6: Crosscheck of multicore-friendly ALF luma region coding in JCTVC-J0320 [C.-Y. Tsai, Y.-W. Huang (MediaTek)] [late]

Abstract:

The region coding scheme for luma ALF in HM-7.0 does not allow regions 0 and 15 to share filters although they are positioned adjacent to each other. This contribution proposes the addition of an extra flag to allow that share for a more coherent design.  The proposed solution introduces an extra flag alf_filter_pattern_flag[0] which signifies regions 15 and 0 are merged if the flag is 0 (i.e., circular merging between filter#0 and filter#15).

Benefit:

Multicore friendly for encoding, since 0 and 15 can be merged.

It is beneficial for tile.

When coding with multi-processor, filter can be independently derived and merged.

Coding efficiency:

No loss, no gain for single-core simulation (i.e., common test conditions).

Cross-check:

The verification task was done successfully, and the results match exactly.

Availability of text:

Available in the contribution.

Discussion:

There still be a problem, e.g. filter#2 and filter#11can not be merged in subpic0 (2 division).

There are many ways to realize filter merge.  This is sub-optimal solution.

No results for multi-core-based encoding scheme, which means the gain is unknown.

Two supports, two oppositions.  No consensus.

JCTVC-J0048 AHG6: ALF with non-normative encoder-only improvements [C.-Y. Chen, C.-Y. Tsai, C.-M. Fu, Y.-W. Huang, S. Lei (MediaTek), T. Yamakage, T. Itoh, T. Chujoh (Toshiba), I. S. Chong, M. Karczewicz (Qualcomm)] --- Non-normative

JCTVC-J0253 AHG6: Cross-check for non-normative ALF improvements (JCTVC-J0048) [S. Esenlik, M. Narroschke (Panasonic)]

Abstract:

Non-normative encoder-only ALF improvements are proposed. The major changes are reusing up to eight previous adaptation parameter sets and estimating rates more accurately in the rate-distortion optimization (RDO) process. The bug-fix of ticket #574 (i.e., swapping coef[2] and coef[4] in the bitstream) is also included. Compared with the ALF in HM-7.0, the proposed ALF can increase luma coding gains by 0.2-1.1% in terms of BD-rate.

- Consider the rate of LCU on/off control flags during picture-level on/off decision  Bug fix

- Use CABAC for rate estimation of LCU on/off control flags during LCU on/off decision

- Reuse up to eight previous adaptation parameter sets during picture-level filter selection

- Recheck picture-level ALF-off after LCU on/off decisions (only for high efficiency mode)

- Increase from one to three redesigns of filter coefficients (only for high efficiency mode)

Benefit:

Coding gain with non-normative (encoder) improvement.

Coding efficiency:

Compared with ALF-off using common test condition sequences, the coding gains of ALF-on are 2.9%, 4.0%, and 4.0% for Y, Cb, and Cr, respectively, when Main-AI/RA/LB/LP and HE10-AI/RA/LB/LP are averaged.  (In comparison with the ALF in HM-6.0, the luma coding gain and the chroma coding gain have been increased by 0.2-0.6% and 1.4-3.3%, respectively.)

Compared with ALF-off using 15 720p and 10 1080p sequences that were commonly seen during the study period of KTA software, the corresponding coding gains of ALF-on become 5.2%, 5.4%, and 5.3% for Y, Cb, and Cr, respectively, where the ALF luma coding gains in Main-LP and HE10-LP are even higher than 9% and 10%, respectively.

Cross-check:

The bitrate and PSNR measurements that are provided by the proponents have been verified. Additional simulation results for KTA sequences are going to be provided in a later revision.  The source code was found to follow the description.

Availability of text:

Available in the contribution.

Discussion:

Why KTA sequences gain is higher?    Only high resolution sequences are used.

One expert expressed that KTA sequences are noisy and ALF works better for objective gain, but not for subjective picture quality.  The reason why ALF shows better gain would be noisy sequences.   Not noisy sequences can be also improved.

Recommendation: Adopt this non-normative improvement for HM8.  Adopt Bug-fix ticket #574.

JCTVC-J0390 AHG6: Further cleanups and simplifications for the ALF in JCTVC-J0048 [C.-Y. Chen, C.-Y. Tsai, C.-M. Fu, Y.-W. Huang, S. Lei (MediaTek), T. Yamakage, T. Itoh, T. Chujoh (Toshiba), I. S. Chong, M. Karczewicz (Qualcomm)] [late]

JCTVC-J0440 Crosscheck of JCTVC-J0390: AHG6: Further cleanups and simplifications of the ALF in JCTVC-J0048 [D.-K. Kwon, M. Budagavi (TI)] [late] [miss]

Abstract:

This proposal presents further cleanups and simplifications for ALF, which is mainly in response to some expert’s request.

- Software package 1:

On top of the JCTVC-J0048 software, four modifications are added as follows.

- Reduction of the filter coefficient precision from 9-bit to 7-bit

- Reduction of filter shape from cross9x7+square3x3 to cross7x7+square3x3

- Fix of RDO for considering a previous APS

- Code cleanups

- Software package 2

On top of the software package 1, when samples are equal to 8-bit (i.e., Main conditions), filter coefficients are normatively constrained on the encoder side as follows.

- Sum of positive non-center filter coefficients times 510 plus center filter coefficient times 255 shall be in the range of [0, 215-1-32).

- Sum of negative filter coefficients times 510 shall be in the range of [-215, 0).

In this way, 16-bit accumulation can be achieved for filtering 8-bit samples in Main conditions.

- Software package 3

All the cleanups, fixes, non-normative improvements, and simplifications in JCTVC-J0048 and the previous two software packages are integrated in the software package 3. In addition, the followings are also included.

- Cleanups and fixes for APS in JCTVC-J0047

- Unifying exponential golomb coding of ALF with other parts by using leading zeros

- Applying virtual boundary processing for the last luma LCU row and the first chroma LCU row (a missing adoption in software and text)

- What is more, the software package 3 is based on HM-7.1, where the ALF part can be easily reused for developing HM-8.0.

Benefit:

Simplification to enable 16-bits accumulation operation for highly parallelized processing.

Code cleanups.

Coding efficiency:

0.1% loss in luma compared with JCTVC-J0048 (Software 2), which mainly comes from reduction of one coefficient.

Cross-check:

It is confirmed that the cross-check results of two softwares match to ones by the proponents.

Availability of text:

Available in the contribution.

Discussion:

Whether 7x7+3x3 is desirable (cf. current: 9x7+3x3)?  Show the results to the person who requested.

If picture size is large, larger filter gives better coding gain.

Several experts expressed their opinion that filter shape should be unchanged at this stage.

About 4x speedup by SIMD.  It is similar to transform.

Encoder complexity is concerned.  Current coefficients modification scheme is similar to RDOQ.  One expert expressed his opinion that other than encoder, this is nice to have, and simulation to turn off the violated filter is desirable.

Constraint to 16-bits accumulation is necessary?  Yes. Recommend to adopt this at this meeting, and provide the better/simpler encoder at the next meeting.

Recommendation:

(from software1) Reduction of the filter coefficient precision from 9-bit to 7-bit

(from software1) Fix of RDO for considering a previous APS

(from software1) Code cleanups

(from software2) Sum of positive non-center filter coefficients times 510 plus center filter coefficient times 255 shall be in the range of [0, 215-1-32)

(from software2) Sum of negative filter coefficients times 510 shall be in the range of [-215, 0).

(from software3) Cleanups and fixes for APS in JCTVC-J0047 except the part3 of JCTVC-J0047 has to be confirmed by HLS experts

(from software3) Applying virtual boundary processing for the last luma LCU row and the first chroma LCU row (a missing adoption)

Only filter shape needs to be further discussed in Track B.

Conduct subjective viewing by using the simplest one (i.e., Software package 2)

JCTVC-J0288 On Loop Filter Disabling [G. Van der Auwera, R. Joshi, Y.-K. Wang, M. Karczewicz (Qualcomm)]

- Should be discussed in session B.

(Non-normative changes)

JCTVC-J0144 AHG6: Hue/saturation-based chroma ALF design and LCU-based on/off control by encoder [T. Yamakage, T. Itoh, Takeshi Chujoh (Toshiba), C.-Y. Chen, C.-Y. Tsai, C.-M. Fu, Y.-W. Huang, S. Lei (MediaTek), I. S. Chong, M. Karczewicz (Qualcomm)]

Abstract:

This contribution provides hue/saturation-based chroma ALF design and on/off control scheme at encoder side. Coding error in Cb/Cr domain is converted to hue/saturation domain. Based on the coding error statistics in hue/saturation domain, Cb/Cr filter coefficients are designed by Wiener filter design method. In addition, LCU-based ALF on/off control for Cb/Cr is decided based on hue/saturation domain.

Benefit:

Reduce hue change that is easily recognized with a small loss of coding efficiency in chroma.

Coding efficiency:

0.2% loss in chroma.

Cross-check:

None.

Availability of text:

N/A

Discussion:

It is interesting since the same idea can be applied to other RDO part.

Informational contribution   no action.

(Bug fixes) Originally categorized to syntax cleanup, but the BoG discussed this.

JCTVC-J0047 AHG6/AHG9: Syntax for APS ID [C.-Y. Tsai, C.-Y. Chen, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-J0332 AHG6/AHG9: Cross check of Cleanups and fixes for APS (JCTVC-J0047) [T. Ikai (Sharp)] [late]

Abstract:

1.	Cleanups of APS (non-normative change)

2.	Fix of APS – Send an APS only when ALF is enabled for the picture (non-normative change)

3.	Fix of APS ID – Send an APS ID only when ALF is enabled for the slice (normative change)

Benefit:

Cleanups and fix of APS.

Coding efficiency improvement.

Coding efficiency:

No gain since aps_id is always 0 in HM7.  In practical application, since aps_id can be 0 to 63, some gain is expected.

Cross-check:

The software is reportedly carefully checked and verified. The test results exactly match with those provided by the proponent.

Availability of text:

Available in the contribution.

Discussion:

Recommendation: Adopt 1 and 2 to HM8.

Recommendation: Adopt 3 to WD8/HM8, however, if other APS element(s) is adopted, this proposal should be modified accordingly.

(Evaluation and profile)

JCTVC-J0049 AHG6: Comparison between ALF and bi-prediction MC [C.-Y. Chen, C.-Y. Tsai, Y.-C. Chang, C.-Y. Cheng, Y.-W. Huang, S. Lei (MediaTek)]

Abstract:

ALF and bi-prediction MC are compared using 15 1080p sequences, where five of the sequences are common test condition class B sequences and the rest were commonly used during KTA software study period.

Coding efficiency:

Main-LP without ALF is used as the anchor. Main-LP with ALF and Main-LB without ALF are tested against the anchor. The software is in the uploaded package of JCTVC-J0048, which contains non-normative and encoder-only improvements for ALF.

| Anchor: Main-LP, ALF-off | ALF (Main-LP, ALF-on) | Bi-Prediction MC (Main-LB, ALF-off) |
| --- | --- | --- |
| BD-rate | -8.3% | -10.5% |
| HM Encoding Time | 100% | 140% |
| HM Decoding Time | 116% | 109% |
| MC Bandwidth | 99% | 161% |

Cross-check:

None.

Availability of text:

N/A

Discussion:

Power consumption is more serious issue (see the table below).

If integer only interpolation is used for bi-prediction, the gain by bi-prediction becomes lower.

Based on the above reasons (trade-off between coding efficiency and power consumption), for real-time low-delay encoding-decoding applications (e.g. video phones and video conferencing) with full HD resolution, ALF could be a better trade-off than bi-prediction MC.

Visual quality is different between bi-prediction and uni-prediction (with ALF).

Informational contribution   no action.