| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0522 |
| --- | --- |

| Title: | Mental crosscheck of JCTVC-J0063: Syntax for NAL Packet Priority |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Rickard Sjöberg Ericsson Research Färögatan 6 16480 Stockholm | Tel: Email: | +46 761153563 rickard.sjoberg@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This document provides a "mental" cross-check for JCTVC-J0063: "Syntax for NAL Packet Priority".  The contribution has been reviewed and understood by the author of this document.

JCTVC-J0063 proposes to include signalling of relative NAL priorities, either by re-defining nal_ref_flag or by including a 4-bit priority_id into the access unit delimiter. The motivation is that the priority signalling can help a decoder to conceal lost slices.

# Comments

The current HEVC specification contains priority marking by temporal layers. It can be expected that a network node would choose to prioritize NAL units of lower temporal layers in case of congestion. This proposal suggests adding priority signalling within the same temporal layer to the specification to enable a finer granularity of priortization.

It is true that nal_ref_flag is not needed for the decoding process and this flag could be redefined.