| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0523 |
| --- | --- |

| Title: | Mental cross-check of JCTVC-J0122 solution5 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yoshitomo Takahashi Ohji Nakagami Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2746 yoshitomo.takahashi@jp.sony.com ohji.nakagami@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This document provides a mental cross-check for JCTVC-J0122 solution5.

# The problem statement

The cross-checker basically understands that the proposal JCTVC-J0122 is describing motion vector prediction (MVP) issues of the seven scenarios in the 3DV extension contexts.

The cross-checker comments that Scenario6 and Scenario7 are also related to Ver1 spec.

# The proposal

The cross-checker confirmed that what described in Section6 of JCTVC-J0122 matches with the proposed semantics in Section7.

The cross-checker understands that JCTVC-J0122 solution5 proposes to introduce additional condition checks on spatial and temporal MVP derivation process to solve the issues in the seven scenarios.

On the MVP process, the following condition checks are added to the condition checks in the current spec. If at least one of the added conditions is NOT true, the predicted vector is marked as unavailable.

- The reference pictures of current vector and predicted vector are same view component or same layer. For this condition check, a function AddPicId( pic ), which returns always 0 in Ver1 spec, is introduced.

- Both the reference pictures of current vector and predicted vector are short-term reference pictures.

- Both reference pictures of current vector and predicted vector are long-term reference pictures and POC values of the reference pictures are same.

In addition, the following condition check is added on combined merge candidate derivation process.

–	AddPicId( RefPicListL0[ refIdxL0l0Cand ] ) != AddPicId( RefPicListL1[ refIdxL1l1Cand ] )

The cross-checker would like to comment that AddPicId(pic) is a hook for Ver2 decoder and the behavior of the function will change in Ver2 spec.

Note1: The cross-checker confirmed the proponent that a typo in the following sentence in 6.1.2 in the first revision document should be fixed as follows

- Prediction between temporal short-term motion vector and temporal long-term motion vector is enabled disabled.

Note2: The cross-checker confirmed the proponent that the table in 5.1.3.3 in the first revision document should be fixed as follows. However, it is out of this cross-check scope.

| Scenario 1 | Scenario 2 | Scenario 3 | Scenario 4 | Scenario 5 | Scenario 6 | Scenario 7 |
| --- | --- | --- | --- | --- | --- | --- |
| ST Scale | No LT scale | No ST pred. for inter-view | Multiple Inter-view | No LT pred. for inter-view | No LT pred. for ST | No pred. between LTs |
| Y | Y | Y | N | N | NY | N |

# Patent rights declaration(s)

Sony Corporation does not have current or pending patent rights relating to the technology described in this contribution.