| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0524 |
| --- | --- |

| Title: | Mental cross-check of JCTVC-J0122: AHG10: Hooks related to motion for the 3DV extension of HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jill Boyce 433 Hackensack Ave Hackensack, NJ 07601 USA | Tel: Email: | jill@vidyo.com |
| Source: | Vidyo |  |  |

_____________________________

# Abstract

This document provides a mental cross-check a portion of the contribution JCTVC-J0122 AHG10: Hooks related to motion for the 3DV extension of HEVC.

The checked contribution proposes 5 solutions to a problem identified for re-using the base HEVC motion vector decoding process for the 3D extension, related to scaling of prediction MVs in inter-view coding.  The first 3 proposed solutions are cross-checked in this document, which have been reviewed and understood by this author.

# Discussion

The proposed solutions are split into 2 categories. The first 3 proposals, which are in the first category, are summarized by the cross-check author below.

In the first category, no changes to the base specification are required, and the solutions apply only to the 3D extension.

- Solution 1: In the extension, use long term reference pictures for all inter-view coded pictures to avoid MV scaling from being applied

- Solution 2: In the extension, a new picture marking process is introduced, so that a picture that is indicated in the bitstream as being a short-term reference picture is marked in the decoder as being a long-term reference picture, and then after decoding is re-marked with its original status

- Solution 3: In the extension, the POC used in the decoding process for inter-view pictures is changed to a new, unused value, and then after decoding is re-assigned its original value so that different inter-view pictures can have different POC values when used for inter-view prediction

In the second category, changes to the base specification are proposed.  Those proposed solutions were not cross-checked in this document.