| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0528 |
| --- | --- |

| Title: | Mental cross-check of JCTVC-J0270: HEVC VUI Parameters with Extension Hooks |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jill Boyce 433 Hackensack Ave Hackensack, NJ 07601 USA | Tel: Email: | jill@vidyo.com |
| Source: | Vidyo |  |  |

_____________________________

# Abstract

This document provides a mental cross-check a portion of the contribution JCTVC- J0270: HEVC VUI Parameters with Extension Hooks.  The checked contribution has reviewed and understood by this author.

The checked contribution proposes extensions to the VUI for temporal scalability.

# Discussion

Two options are presented to duplicate some of the syntax elements in the VUI for temporal layers.

In both options, a vui_num_temporal_layers_minus1 syntax element is sent, to indicate how many temporal layers will have VUI parameters present, which need not be equal to the sps_max_temporal_layers_minus1.

In both options, the following syntax elements or syntax functions are duplicated for each of the indicated temporal layers:   nal_hrd_parameters_present_flag, nal_hrd_parameters( ),   vcl_hrd_parameters_present_flag,  low_delay_hrd_flag, 	sub_pic_cpb_params_present_flag, and num_units_in_sub_tick.

In option 1, the field_seq_flag, timing_info_present_flag, num_units_in_tick,   time_scale, and fixed_pic_rate_flag are replicated for each temporal layer, while in option 2, those values are not duplicated but are sent once and apply to all temporal layers.

The proposed semantics are not fully mature.