| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0530 |
| --- | --- |

| Title: | AHG13: Crosscheck for Signalling of long-term reference pictures in the slice header (JCTVC-J0115) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Crosscheck |  |  |
| Author(s) or Contact(s): | Karsten Suehring Fraunhofer HHI | Email: | Karsten.Suehring@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Introduction

The document and provided source code have been studied and analysed. Du to time constraints the results have been checked based on the provided software.

# Results

## Motivation

The examples in section 1 provide a comprehensive explanation why the long term RPS is not independent from buffer state. They illustrate that in case of picture loss some the situation may arise that it is not clear which of the pictures in the DPB needs to be marked as long-term picture.

## Solution

The solution has been verified to work with different types of random access. The proposal uses POC full differences to identify long-term pictures, which works in situations where decoding starts at CRA or BLA pictures and the absolute POC MSB values are not known.

The overall process of long-term picture identification is simplified by removing the conditional operations on delta_poc_msb_present_flag.

## Draft Text

The draft text in the contribution is in good shape. Minor improvements in the semantics section have been suggested to the authors and will be addressed in a revision.

The long-term RPS decoding process is shortened and simplified.

## Simulation results

The provided software has been checked to implement the described method. The du to the late availability of the software only subset of the results have been checked. No issues have been found in these checks.