| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0066 |
| --- | --- |

| Title: | Chroma CFB cleanup |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Pieter Kapsenberg (Intel Corporation) CH7-427 5000 W Chandler Blvd Chandler AZ, 85226 | Tel: Email: | +1 480 552 5481 pieter.kapsenberg |
| Source: | Intel Corp |  |  |

_____________________________

# Abstract

This contribution claims that the context increment for cbf_cb and cbf_cr can exceed the maximum value intended. A solution is proposed that clamps the context increment to 2.

# Introduction

The context increment for the chroma CBF flags is given by the transform tree depth, trafoDepth. Currently in the text and in HM three context states are available for each slice, suggesting a context increment range of 0..2. In the extreme case of a 64x64 CU with a deep RQT the following applies:

| RQT size luma | RQT size chroma | trafoDepth |
| --- | --- | --- |
| 64x64 | 32x32 | 0 |
| 32x32 | 16x16 | 1 |
| 16x16 | 8x8 | 2 |
| 8x8 | 4x4 | 3 |

When the chroma TU is 4x4, a context increment of 3 is needed, but this is not available. Current HM software uses a context initialized with the CNU constant 154.

Note that the common conditions specify max_transform_hierarchy_depth_intra = 2, so this situation can only occur in a 64x64 CU that is predicted in intra mode and has pred_mode NxN, because then the intraSplitFlag == 1 allows for trafoDepth 3. However, this does not appear to ever happen, the encoder instead splits the CU, hiding the issue.

# Proposed solution: Set a max ctxInc of 2

To match HM, an additional context state with increment 3 could be added, however this is undesirable. Luma CBF only has 2 separate contexts, it seems unnecessary to have 4 contexts for chroma.

By setting ctxInc = Min(2,trafoDepth), the current context init table does not have to change, and as an added bonus it will also work for future extensions incorporating 4:2:2 and 4:4:4 chroma formats.

# Patent rights declaration(s)

Intel Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Draft Text changes for Solution

Changes in Table 9-39 highlighted :

| cbf_cb, cbf_cr | Error: Reference source not found | 0 | Min(2,trafoDepth) | na | na | na | na |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | Min(2,trafoDepth) | na | na | na | na |
|  |  | 6 | Min(2,trafoDepth) | na | na | na | na |