| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0535 |
| --- | --- |

| Title: | Mental cross-check of JCTVC-J0184 on transform skip |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Hui Yong Kim, Sung-Chang Lim *Kyung Yong Kim ETRI, 218 Gajeong-ro, Yuseong-gu, Daejeon, 305-700, Korea *Media Lab., Kyung Hee University, 1, Seochun, Kiheung, Youngin, Gyonggi, 446-701, Korea | Tel: Email: | +82-42-860-5069 {hykim5, sclim}@etri.re.kr *kimky@khu.ac.kr |
| Source: | ETRI and Kyung Hee University |  |  |

_____________________________

# Abstract

This document reports a mental cross-check of JCTVC-J0184, “On transform skip.” The contribution document was mentally examined and understood by the cross-checker.

# Introduction

The contribution claims the following two modifications on transform skip mode.

- Part 1: Transmission of transform_skip_enable_flag in PPS or Slice Header rather than in SPS

- Part 2: Applying flat quantization matrices for TransformSkip TUs

# Part 1: Transmission of transform_skip_enable_flag in PPS or Slice Header rather than in SPS

The proponents claim that PPS or Slice level control of transform skip enable/disable is necessary for low cost mode decision. The solution proposed is to signal transform_skip_enabled_flag in PPS or Slice header instead of in SPS.

# Part 2: Applying flat quantization matrices for TransformSkip TUs

The proponents claim that flat quantization matrices shall be applied for transform skip mode TBs. They provide three alternative methods for achieving it. Solution 1 does not apply quantization matrix for transform skipped TB. Solution 2 explicitly signals flat scaling factor for transform skipped TBs in scaling_list() syntax structure. Solution 3 reuses DC scaling factor of quantization matix for the flat scaling factor for transform skipped TBs.

As a cross-checker, we support the idea of applying a flat scaling factor for transform skipped TBs. So we believe one of the proposed solutions should be adopted in the DIS version of HM. We also believe that solution 1 is most attractive since it is the simplest solution for solving the claimed problem.