| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0546 |
| --- | --- |

| Title: | AHG9/10: vps_id in slice header (partial re-proposal of JCTVC-I0524) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 7180 08000 miska.hannuksela@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Abstract

One aspect of contribution was JCTVC-I0524 was a proposal to include an identifier (ref_vps_id) of the active video parameter set (VPS) into the slice header of RAP pictures. The asserted benefit is that media-aware network elements can conclude the active VPS more easily than with the WD7 syntax that reportedly requires two indirection steps, i.e. resolving the active picture parameter set and the active sequence parameter set. When compared to JCTVC-I0524, this contribution provides further details as well as options for the detailed syntax and semantics.

# Proposal

In the following four options of the syntax and semantics are proposed. They differ in whether the reference VPS ID is present only in the first slice of a RAP picture (option 1) or not (option 2). They also differ in whether the reference VPS ID is ue(v)-coded (option A) or u(v)-coded (option B), where the length v is proposed to be 4.

## Option 1A: 
ref_vps_id present in the first slice of a RAP picture, 
ref_vps_id ue(v)-coded:

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| if( RapPicFlag && first_slice_in_pic_flag ) |  |
| ref_vps_id | ue(v) |
| pic_parameter_set_id | ue(v) |

ref_vps_id shall be equal to the value of video_parameter_set_id of the active video parameter set.

## Option 1B: 
ref_vps_id present in the first slice of a RAP picture, 
ref_vps_id u(4)-coded:

#### 7.4.2.1	Video parameter set RBSP semantics

…

video_parameter_set_id identifies a video parameter set. The value of video_parameter_set_id shall be in the range of 0 to 1415, inclusive.

…

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| if( RapPicFlag && first_slice_in_pic_flag ) |  |
| ref_vps_id | u(4) |
| pic_parameter_set_id | ue(v) |

ref_vps_id shall be equal to the value of video_parameter_set_id of the active video parameter set. ref_vps_value shall be in the range of 0 to 14, inclusive.

## Option 2A: 
ref_vps_id_plus1 present in all slices of a RAP picture, 
ref_vps_id_plus1 ue(v)-coded:

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| if( RapPicFlag ) |  |
| ref_vps_id_plus1 | ue(v) |
| pic_parameter_set_id | ue(v) |

ref_vps_id_plus1 – 1 shall be equal to the value of video_parameter_set_id of the active video parameter set. ref_vps_value_plus1 shall be in the range of 1 to 15, inclusive.

## Option 2B: 
ref_vps_id_plus1 present in all slices of a RAP picture, 
ref_vps_id_plus1 u(4)-coded:

#### 7.4.2.1	Video parameter set RBSP semantics

…

video_parameter_set_id identifies a video parameter set. The value of video_parameter_set_id shall be in the range of 0 to 1415, inclusive.

…

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| if( RapPicFlag ) |  |
| ref_vps_id_plus1 | u(4) |
| pic_parameter_set_id | ue(v) |

ref_vps_id_plus1 – 1 shall be equal to the value of video_parameter_set_id of the active video parameter set. ref_vps_value_plus1 shall be in the range of 1 to 15, inclusive.

# Patent rights declaration(s)

Nokia Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).