| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0548 |
| --- | --- |

| Title: | AHG10: VUI and HRD syntax designs agreed by the BoG on VPS and NUH |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Munsi Haque 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4099 munsi.haque@am.sony.com |
|  | Ye-Kui Wang 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-651-8345 yekuiw@qualcomm.com |
|  | Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 7180 08000 miska.hannuksela@nokia.com |
| Source: | Sony Electronics Inc., Qualcomm Incorporated, Nokia Corporation |  |  |

_____________________________

# Abstract

This contribution documents the agreed syntax designs for VUI and HRD syntaxes. The HRD syntax is claimed applicable to both the HEVC base specification for temporal scalability and for potential future HEVC extensions. The VUI parameters syntax is changed accordingly. If HRD parameters remain as part of VUI, the syntax structure hrd_parameters( ) is present in the VUI syntax. Otherwise, the syntax structure hrd_parameters( ) is not present in the VUI syntax.

- HRD parameters syntax and semantics

| hrd_parameters( commonInfPresentFlag, MaxNumSubLayersMinus1 ) { | Descriptor |
| --- | --- |
| if( commonInfPresentFlag ) { |  |
| timing_info_present_flag | u(1) |
| if( timing_info_present_flag ) { |  |
| num_units_in_tick | u(32) |
| time_scale | u(32) |
| fixed_pic_rate_flag | u(1) |
| } |  |
| nal_hrd_parameters_present_flag | u(1) |
| vcl_hrd_parameters_present_flag | u(1) |
| if( nal_hrd_parameters_present_flag ¦ ¦ vcl_hrd_parameters_present_flag ){ |  |
| low_delay_hrd_flag | u(1) |
| sub_pic_cpb_params_present_flag | u(1) |
| if( sub_pic_cpb_params_present_flag ) |  |
| num_units_in_sub_tick | u(32) |
| bit_rate_scale | u(4) |
| cpb_size_scale | u(4) |
| cbr_flag | u(1) |
| initial_cpb_removal_delay_length_minus1 | u(5) |
| cpb_removal_delay_length_minus1 | u(5) |
| dpb_output_delay_length_minus1 | u(5) |
| time_offset_length | u(5) |
| } |  |
| } |  |
| if( nal_hrd_parameters_present_flag ¦ ¦ vcl_hrd_parameters_present_flag ) |  |
| for( i = 0; i <= MaxNumSubLayersMinus1; i++ ) { |  |
| if( nal_hrd_parameters_present_flag ) |  |
| hrd_parameters_sub_layer( i ) |  |
| if( vcl_hrd_parameters_present_flag ) |  |
| hrd_parameters_sub_layer( i ) |  |
| } |  |
| } |  |

| hrd_parameters_sub_layer( tId ) { | Descriptor |
| --- | --- |
| cpb_cnt_minus1 | ue(v) |
| for( SchedSelIdx = 0; SchedSelIdx <= cpb_cnt_minus1; SchedSelIdx++ ) { |  |
| bit_rate_value_minus1[ SchedSelIdx ] | ue(v) |
| cpb_size_value_minus1[ SchedSelIdx ] | ue(v) |
| } |  |
| } |  |

The semantics of each syntax elements in the hrd_parameters( ) syntax structure or the hrd_parameters_sub_layer( ) syntax structure is the same as that of the syntax element with the same name in the latest HEVC draft specification. Syntax elements in the hrd_parameters( ) syntax structure apply to the representation of the sub-layer with temporal_id equal to vps_max_sub_layers_minus1. Syntax elements in the hrd_parameters_sub_layer( tId ) syntax structure apply to the representation of the sub-layer with temporal_id equal to tId.

- VUI parameters syntax and semantics

In the following, it is assumed that HRD parameters remain as part of VUI, hence the syntax structure hrd_parameters( ) is present in the VUI syntax.

| vui_parameters( ) { | Descriptor |
| --- | --- |
| aspect_ratio_info_present_flag | u(1) |
| if( aspect_ratio_info_present_flag ) { |  |
| aspect_ratio_idc | u(8) |
| if( aspect_ratio_idc = = Extended_SAR ) { |  |
| sar_width | u(16) |
| sar_height | u(16) |
| } |  |
| } |  |
| overscan_info_present_flag | u(1) |
| if( overscan_info_present_flag ) |  |
| overscan_appropriate_flag | u(1) |
| video_signal_type_present_flag | u(1) |
| if( video_signal_type_present_flag ) { |  |
| video_format | u(3) |
| video_full_range_flag | u(1) |
| colour_description_present_flag | u(1) |
| if( colour_description_present_flag ) { |  |
| colour_primaries | u(8) |
| transfer_characteristics | u(8) |
| matrix_coefficients | u(8) |
| } |  |
| } |  |
| chroma_loc_info_present_flag | u(1) |
| if( chroma_loc_info_present_flag ) { |  |
| chroma_sample_loc_type_top_field | ue(v) |
| chroma_sample_loc_type_bottom_field | ue(v) |
| } |  |
| neutral_chroma_indication_flag | u(1) |
| field_seq_flag | u(1) |
| hrd_parameters( 1, sps_max_sub_layers_minus1 ) |  |
| timing_info_present_flag | u(1) |
| if( timing_info_present_flag ) { |  |
| num_units_in_tick | u(32) |
| time_scale | u(32) |
| fixed_pic_rate_flag | u(1) |
| } |  |
| nal_hrd_parameters_present_flag | u(1) |
| if( nal_hrd_parameters_present_flag ) |  |
| hrd_parameters( ) |  |
| vcl_hrd_parameters_present_flag | u(1) |
| if( vcl_hrd_parameters_present_flag ) |  |
| hrd_parameters( ) |  |
| if( nal_hrd_parameters_present_flag ¦ ¦ vcl_hrd_parameters_present_flag ) |  |
| low_delay_hrd_flag | u(1) |
| sub_pic_cpb_params_present_flag | u(1) |
| if( sub_pic_cpb_params_present_flag ) |  |
| num_units_in_sub_tick | u(32) |
| } |  |
| bitstream_restriction_flag | u(1) |
| if( bitstream_restriction_flag ) { |  |
| tiles_fixed_structure_flag | u(1) |
| motion_vectors_over_pic_boundaries_flag | u(1) |
| max_bytes_per_pic_denom | ue(v) |
| max_bits_per_mincu_denom | ue(v) |
| log2_max_mv_length_horizontal | ue(v) |
| log2_max_mv_length_vertical | ue(v) |
| } |  |
| } |  |

Semantics of the removed syntax elements are removed, and semantics of remaining syntax elements remain unchanged.

# Patent rights declaration(s)

Sony Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Nokia Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).