| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0548r1 |
| --- | --- |

| Title: | AHG10: VUI and HRD syntax designs agreed by the BoG on VPS and NUH |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Munsi Haque 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4099 munsi.haque@am.sony.com |
|  | Ye-Kui Wang 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-651-8345 yekuiw@qualcomm.com |
|  | Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 7180 08000 miska.hannuksela@nokia.com |
| Source: | Sony Electronics Inc., Qualcomm Incorporated, Nokia Corporation |  |  |

_____________________________

# Abstract

This contribution documents the designs for VUI and HRD syntaxes agreed by the BoG on VPS and NUH. The HRD syntax is claimed applicable to both the HEVC base specification for temporal scalability and for potential future HEVC extensions. The VUI parameters syntax is changed accordingly. If HRD parameters remain as part of VUI, the syntax structure hrd_parameters( ) is present in the VUI syntax. Otherwise, the syntax structure hrd_parameters( ) is not present in the VUI syntax.

Revision 1 of this document include changes made during the discussion of the original version of this document in Track A, on July 15, 2012. In other words, Revisio 1 of this document documents the designs of VUI and HRD syntaxes agreed by Track A. Changes relative to the original version are all marked.

- HRD parameters syntax and semantics

| hrd_parameters( commonInfPresentFlag, MaxNumSubLayersMinus1 ) { | Descriptor |
| --- | --- |
| if( commonInfPresentFlag ) { |  |
| timing_info_present_flag | u(1) |
| if( timing_info_present_flag ) { |  |
| num_units_in_tick | u(32) |
| time_scale | u(32) |
| } |  |
| nal_hrd_parameters_present_flag | u(1) |
| vcl_hrd_parameters_present_flag | u(1) |
| if( nal_hrd_parameters_present_flag ¦ ¦ vcl_hrd_parameters_present_flag ){ |  |
| sub_pic_cpb_params_present_flag | u(1) |
| if( sub_pic_cpb_params_present_flag ) |  |
| tick_divisor_minus2 | u(8) |
| bit_rate_scale | u(4) |
| cpb_size_scale | u(4) |
| initial_cpb_removal_delay_length_minus1 | u(5) |
| cpb_removal_delay_length_minus1 | u(5) |
| dpb_output_delay_length_minus1 | u(5) |
| } |  |
| } |  |
| for( i = 0; i <= MaxNumSubLayersMinus1; i++ ) { |  |
| fixed_pic_rate_flag[ i ] | u(1) |
| if( fixed_pic_rate_flag[ i ] ) |  |
| pic_duration_in_tc_minus1[ i ] | ue(v) |
| low_delay_hrd_flag[ i ] | u(1) |
| cpb_cnt_minus1[ i ] | ue(v) |
| if( nal_hrd_parameters_present_flag ) |  |
| hrd_parameters_sub_layer( i ) |  |
| if( vcl_hrd_parameters_present_flag ) |  |
| hrd_parameters_sub_layer( i ) |  |
| } |  |
| } |  |

| hrd_parameters_sub_layer( tId ) { | Descriptor |
| --- | --- |
| for( SchedSelIdx = 0; SchedSelIdx <= cpb_cnt_minus1[ i ]; SchedSelIdx++ ) { |  |
| bit_rate_value_minus1[ SchedSelIdx ] | ue(v) |
| cpb_size_value_minus1[ SchedSelIdx ] | ue(v) |
| cbr_flag[ SchedSelIdx ] | u(1) |
| } |  |
| } |  |

The semantics of each syntax elements in the hrd_parameters( ) syntax structure or the hrd_parameters_sub_layer( ) syntax structure is the same as that of the syntax element with the same name in the latest HEVC draft specification. Syntax elements in the hrd_parameters( ) syntax structure apply to the representation of the sub-layer with temporal_id equal to vps_max_sub_layers_minus1. Syntax elements in the hrd_parameters_sub_layer( tId ) syntax structure apply to the representation of the sub-layer with temporal_id equal to tId.

The semantics of new syntax elements or syntax elements for which the syntax element names have been changed, including addition of the suffix "[ i ]", are as follows.

tick_divisor_minus2 is used to specify the sub-picture clock tick. A sub-picture clock tick is the minimum interval of time that can be represented in the coded data when sub_pic_cpb_params_present_flag is equal to 1.

fixed_pic_rate_flag[ i ] equal to 1 indicates that, when HighestTemporalId is equal to i, the temporal distance between the HRD output times of any two consecutive pictures in output order is constrained as follows. fixed_pic_rate_flag[ i ] equal to 0 indicates that no such constraints apply to the temporal distance between the HRD output times of any two consecutive pictures in output order.

When fixed_pic_rate_flag[ i ] is not present, it is inferred to be equal to 0.

When HighestTemporalId is equal to i and fixed_pic_rate_flag[ i ] is equal to 1 for a coded video sequence containing picture n, the value computed for to,dpb( n ) as specified in Equation Error: Reference source not found shall be equal to tc * ( pic_duration_in_tcs_minus1[ i ] + 1 ), wherein tc is as specified in Equation C-1 (using the value of tc for the coded video sequence containing picture n) when one or more of the following conditions are true for the following picture nn that is specified for use in Equation Error: Reference source not found:

–	picture nn is in the same coded video sequence as picture n.

–	picture nn is in a different coded video sequence and fixed_pic_rate_flag[ i ] is equal to 1 in the coded video sequence containing picture nn, the value of num_units_in_tick  time_scale is the same for both coded video sequences, and the value of pic_duration_in_tc_minus1[ i] is the same for both coded video sequences.

pic_duration_in_tc_minus1[ i ] plus 1 specifies, when HighestTemporalId is equal to i, the temporal distance, in clock ticks, between the HRD output times of any two consecutive pictures in output order in the coded video sequence. The value of pic_duration_in_tc_minus1[ i ] shall be in the range of 0 to 2047, inclusive.

low_delay_hrd_flag[ i ] specifies the HRD operational mode, when HighestTemporalId is equal to i, as specified in Annex Error: Reference source not found. When fixed_pic_rate_flag[ i ] is equal to 1, low_delay_hrd_flag[ i ] shall be equal to 0. When low_delay_hrd_flag[ i ] is not present, its value is inferred to be equal to 1 − fixed_pic_rate_flag[ i ].

NOTE 11 – When low_delay_hrd_flag[ i ] is equal to 1, "big pictures" that violate the nominal CPB removal times due to the number of bits used by an access unit are permitted. It is expected, but not required, that such "big pictures" occur only occasionally.

cpb_cnt_minus1[ i ] plus 1 specifies the number of alternative CPB specifications in the bitstream of the  coded video sequence when HighestTemporalId is equal to i. The value of cpb_cnt_minus1[ i ] shall be in the range of 0 to 31, inclusive. When low_delay_hrd_flag[ i ] is equal to 1, cpb_cnt_minus1[ i ] shall be equal to 0. When cpb_cnt_minus1[ i ] is not present, it is inferred to be equal to 0.

- VUI parameters syntax and semantics

In the following, it is assumed that HRD parameters remain as part of VUI, hence the syntax structure hrd_parameters( ) is present in the VUI syntax.

| vui_parameters( ) { | Descriptor |
| --- | --- |
| aspect_ratio_info_present_flag | u(1) |
| if( aspect_ratio_info_present_flag ) { |  |
| aspect_ratio_idc | u(8) |
| if( aspect_ratio_idc = = Extended_SAR ) { |  |
| sar_width | u(16) |
| sar_height | u(16) |
| } |  |
| } |  |
| overscan_info_present_flag | u(1) |
| if( overscan_info_present_flag ) |  |
| overscan_appropriate_flag | u(1) |
| video_signal_type_present_flag | u(1) |
| if( video_signal_type_present_flag ) { |  |
| video_format | u(3) |
| video_full_range_flag | u(1) |
| colour_description_present_flag | u(1) |
| if( colour_description_present_flag ) { |  |
| colour_primaries | u(8) |
| transfer_characteristics | u(8) |
| matrix_coefficients | u(8) |
| } |  |
| } |  |
| chroma_loc_info_present_flag | u(1) |
| if( chroma_loc_info_present_flag ) { |  |
| chroma_sample_loc_type_top_field | ue(v) |
| chroma_sample_loc_type_bottom_field | ue(v) |
| } |  |
| neutral_chroma_indication_flag | u(1) |
| field_seq_flag | u(1) |
| hrd_parameters( 1, sps_max_sub_layers_minus1 ) |  |
| timing_info_present_flag | u(1) |
| if( timing_info_present_flag ) { |  |
| num_units_in_tick | u(32) |
| time_scale | u(32) |
| fixed_pic_rate_flag | u(1) |
| } |  |
| nal_hrd_parameters_present_flag | u(1) |
| if( nal_hrd_parameters_present_flag ) |  |
| hrd_parameters( ) |  |
| vcl_hrd_parameters_present_flag | u(1) |
| if( vcl_hrd_parameters_present_flag ) |  |
| hrd_parameters( ) |  |
| if( nal_hrd_parameters_present_flag ¦ ¦ vcl_hrd_parameters_present_flag ) |  |
| low_delay_hrd_flag | u(1) |
| sub_pic_cpb_params_present_flag | u(1) |
| if( sub_pic_cpb_params_present_flag ) |  |
| num_units_in_sub_tick | u(32) |
| } |  |
| bitstream_restriction_flag | u(1) |
| if( bitstream_restriction_flag ) { |  |
| tiles_fixed_structure_flag | u(1) |
| motion_vectors_over_pic_boundaries_flag | u(1) |
| max_bytes_per_pic_denom | ue(v) |
| max_bits_per_mincu_denom | ue(v) |
| log2_max_mv_length_horizontal | ue(v) |
| log2_max_mv_length_vertical | ue(v) |
| } |  |
| } |  |

Semantics of the removed syntax elements are removed, and semantics of remaining syntax elements remain unchanged.

- HRD

- C.1	General

…

The variable tc is derived as follows and is called a clock tick:

tc = num_units_in_tick  time_scale		(C-1)

The variable tc_sub is derived as follows and is called a sub-picture clock tick:

tc_sub = tc  ( tick_divisor_minus2 + 2 )		(C-1)

…

# Patent rights declaration(s)

Sony Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Nokia Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).