| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11 – 20 July 2012 | Document: JCTVC-J0549 |
| --- | --- |

| Title: | NAL unit types for non-reference pictures within the same temporal sub-layer |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rickard Sjöberg Jonatan Samuelsson Ericsson Research Färögatan 6 16480 Stockholm TK Tan Junya Takiue 3-6 Hikarinooka, Yokosuka-shi Kanagawa, 239 8536 Japan | Tel: Email: | +46 761153563 rickard.sjoberg@ericsson.com jonatan.samuelsson@ericsson.com +65 9799 3861 tktan.p@rd.nttdocomo.co.jp takiue@nttdocomo.co.jp |
| Source: | NTT DOCOMO, Inc. Ericsson |  |  |

_____________________________

# Abstract

This is the combination of proposal J0231 and J0250. It contains the description of three new NAL unit types used for indicating non-reference pictures within the same temporal layer.

The three types are non-reference coded slice, non-reference TLA and non-reference GTLA.

# Introduction

This contribution proposes to remove nal_ref_flag and duplicate nal_unit_types for coded slice, TLA and GTLA. The existing nal_unit_types for coded slice, TLA and GTLA are used to indicate reference pictures within the same temporal sub-layer. The three new nal_unit_types for coded slice, TLA and GTLA are used to indicate non-reference pictures within the same temporal sub-layer.

# Proposed text

3.72 non-reference picture: A picture coded with nal_ref_flag equal to 0. A non-reference picture is not used for inter prediction of any other pictures.

3.98 reference picture: A picture with nal_ref_flag equal to 1. A reference picture that contains samples that may be used for inter prediction in the decoding process of subsequent pictures in decoding order.

nal_ref_flag equal to 1 specifies that the content of the NAL unit contains a sequence parameter set, a picture parameter set, an adaptation parameter set or a slice of a reference picture.

nal_ref_flag equal to 0 for a NAL unit containing a slice indicates that the slice is part of a non-reference picture.

nal_ref_flag shall be equal to 1 for video parameter set, sequence parameter set, picture parameter set or adaptation parameter set NAL units. When nal_ref_flag is equal to 0 for one VCL NAL unit of a particular picture, it shall be equal to 0 for all VCL NAL units of the particular picture.

nal_ref_flag shall be equal to 1 for NAL units with nal_unit_type equal to 4, 5, 6, 7, or 8.

nal_ref_flag shall be equal to 0 for all NAL units having nal_unit_type equal to 29, 30, or 31.

Table 7-1 – NAL unit type codes and NAL unit type classes

| nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- |
| 0 | Unspecified | non-VCL |
| 1 | Coded slice of a non-RAP, non-TFD and non-TLA picture slice_layer_rbsp( ) | VCL |
| 2 | Coded slice of a TFD picture slice_layer_rbsp( ) | VCL |
| 3 | Coded slice of a TLA picture slice_layer_rbsp( ) | VCL |
| 4, 5 | Coded slice of a CRA picture slice_layer_rbsp( ) | VCL |
| 6, 7 | Coded slice of a BLA picture slice_layer_rbsp( ) | VCL |
| 8 | Coded slice of an IDR picture slice_layer_rbsp( ) | VCL |
| 9 | Coded slice of a GTLA picture slice_layer_rbsp( ) | VCL |
| 10 | Coded slice of a non-reference non-RAP, non-TFD and non-TLA picture slice_layer_rbsp( ) | VCL |
| 11 | Coded slice of a non-reference TLA picture slice_layer_rbsp( ) | VCL |
| 12 | Coded slice of a non-reference GTLA picture slice_layer_rbsp( ) | VCL |
| 13..24 | Reserved | n/a |
| 25 | Video parameter set video_parameter_set_rbsp( ) | non-VCL |
| 26 | Sequence parameter set seq_parameter_set_rbsp( ) | non-VCL |
| 27 | Picture parameter set pic_parameter_set_rbsp( ) | non-VCL |
| 28 | Adaptation parameter set aps_rbsp( ) | non-VCL |
| 29 | Access unit delimiter access_unit_delimiter_rbsp( ) | non-VCL |
| 30 | Filler data filler_data_rbsp( ) | non-VCL |
| 31 | Supplemental enhancement information (SEI) sei_rbsp( ) | non-VCL |
| 32..47 | Reserved | n/a |
| 48..63 | Unspecified | non-VCL |

- NOTE 4 - When the value of nal_unit_type is equal to 10, 11 or 12 the content of the NAL unit contains a slice of a picture that is not included in the RefPicSetStCurrBefore, RefPicSetStCurrAfter and RefPicSetLtCurr of a picture of the same temporal layer. A picture with nal_unit_type equal to 10, 11 or 12 may be discarded without affecting the decidability of that temporal layer.

##### 7.4.1.2.2 Order of access units and association to coded video sequences

The values of picture order count for the coded pictures in consecutive access units in decoding order containing non reference pictures shall be increasing.

## 8.1 General

After all slices of the current picture have been decoded, if it is a reference picture the decoded reference the picture is marked as "used for short-term reference", otherwise (it is a non-reference picture) the decoded picture is marked as "unused for reference", as specified in subclause 8.3.5.

##### 7.4.1.2.4 Detection of the first VCL NAL unit of a coded picture

–	nal_ref_flag differs in value with one of the nal_ref_flag values being equal to 0.

### 8.3.2 Decoding process for reference picture set

It is a requirement of bitstream conformance that the reference picture set is restricted as follows:

- There shall be no reference picture with nal_unit_type equal to 10, 11 or 12 and temporal_id equal to that of the current picture included in RefPicSetStCurrBefore, RefPicSetStCurrAfter and RefPicSetLtCurr.

### 8.3.5 Marking of the current picture after decoding

This process is invoked after all slices of the current picture have been decoded.

–	If nal_ref_flag of the current picture is equal to 0, the current picture is marked as "unused for reference".

–	Otherwise, tThe current picture is marked as "used for short-term reference".

### C.5.3 Picture decoding, marking and storage

If tThe current decoded picture is a reference picture, it is marked as "used for reference", otherwise (the current decoded picture is a non-reference picture), it is marked as "unused for reference".

# Patent rights declaration(s)

NTT DOCOMO, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).