| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0550 |
| --- | --- |

| Title: | BoG report on VPS and NAL unit header |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jill Boyce | Tel: Email: | jill@vidyo.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

The BoG met on 13 July to review and/or revisit the contributions in Agenda items 5.12.1 NAL unit header and 5.12.5 Video parameter set (VPS) and sequence parameter set (SPS).  The group also reviewed contribution JCTVC-J0272 which was not assigned, but considered to be relevant.

The attachment to the v2 version of this document describes the syntax and semantics capturing the Track B adoptions based upon the BoG recommendations.

The attachment to the v3 version of this document removes the SEI message syntax, which is included in the r1 version of J0261.

# Recommendations

The BoG recommends the following:

JCTVC-J0074: BoG recommends to reserve 6 NAL unit type values as VCL NAL units, and the proposed sub-bitstream extraction process (proposals #1 and #2).

JCTVC-J0261 or JCTVC-J0546: BoG recommends to either create an SEI message to signal the active vps_id, or add the vps_id to the slice header as a fixed length field in RAP pictures.

JCTVC-J0548 (based upon JCTVC-J0270 and JCTVC-J0272): BoG recommends an extension to the HRD parameters in VUI which duplicates some syntax elements for each temporal sub-layer.

JCTVC-J0549 (combination of JCTVC-J0231 and JCTVC-J0250): BoG recommends to create 3 new NUT values (duplicating TLA, GTLA, and coded slice) to indicate that the slice is not used as a reference for another picture of the same temporal sub-layer.

BoG output doc (draws from JCTVC-J0075, JCTVC-J0112, JCTVC-J0113, JCTVC-J0114, JCTVC-J0196, JCTVC-J0245, and JCTVC-J0257): BoG recommends

- remove the following duplicated syntax elements from the SPS which are already present in the VPS: max_dec_pic_buffering, num_reorder_pics, max_latency_increase, temporal_id_nesting_flag, max_temporal_layers_minus1

- move profile (profile_idc, profile_space, profile_compatability_flags, constraint_flags) from the SPS to the VPS

- in VPS, optionally send profile for each lower temporal sub-layer

- send max level in VPS.  level_idc still sent in SPS, may be lower than VPS max level

- in VPS, optionally send max level for each lower temporal sub-layer

- in SPS, optionally send level for each lower temporal sub-layer, may be lower than corresponding VPS max level

- in VPS, used fixed length coding for the syntax elements at the beginning of the VPS

- in VPS, add a byte pointer following fixed length syntax elements and before first ue(v) coded syntax elements

- in NAL unit header, remove nal_ref_flag, and allocate bit to reserved bits.  Reorder syntax elements in NUH so that all 6 reserved bits are contiguous

- move emulation prevention in NAL unit header to temporal_id, from the reserved bits intended for use as layer_id.  temporal_id is renamed temporal_id_plus1.

- rename the max_layer_minus1 in VPS base spec as a reserved field, so that specific semantics for additional layers aren’t necessary

BoG encourages further study on moving the SPS VUI HRD parameters to the VPS.  A problem as identified that there are no clear HRD performance specified when bitstreams with enhancement layers are fed to a decoder conforming to the base specification.  BoG recommends to revisit JCTVC-J0562 as a potential solution.

# 2. Document Review and Discussion Notes

## 2.1 Document review of 5.12.5 Video parameter set (VPS) and sequence parameter set (SPS)

## JCTVC-J0074 AHG10 Hooks for Scalable Coding: Sequence Parameter Set Design [M. M. Hannuksela (Nokia)]

BoG recommends to reserve 6 NAL unit type values as VCL NAL units, and the proposed sub-bitstream extraction process.

JCTVC-J0459 AHG10: Mental cross-check of JCTVC-J0074 [Y.-K. Wang (Qualcomm)] [late]

JCTVC-J0075 AHG10 Hooks for Scalable Coding: Video Parameter Set Design	[M. M. Hannuksela (Nokia)]

JCTVC-J0460 AHG10: Mental cross-check of JCTVC-J0075 [Y.-K. Wang (Qualcomm)] [late]

JCTVC-J0114 AHG10: On video parameter set [Y. Chen, Y.-K. Wang (Qualcomm)]

JCTVC-J0502 AHG10: Mental cross-check of JCTVC-J0114 (Video parameter set HEVC base specification) [M. M. Hannuksela (Nokia)]

JCTVC-J0124 AHG10: On video parameter set for HEVC extensions [Y. Chen, Y.-K. Wang (Qualcomm)]

JCTVC-J0196 AHG10: Proposed modification to video parameter set [K. Sugimoto, S. Sekiguchi (Mitsubishi)]

This contribution proposes modifications to the specification of video parameter set, which was adopted at the last meeting. The video parameter set provides a normative way for encapsulation of multiple sub-bitstreams those constitutes a single coded video sequence. Here, the term “sub-bitstream” represents temporal layer within the context of current HEVC base specification, however, it also assumes support of (spatial) enhancement layers that refer to HEVC base layer stream for their decoding. As discussed in the requirement on scalable coding extension to the current HEVC spec, base layer for such scalable extension could be other coding standard such as AVC. Considering the requirement such that encoder producing bitstream conforming to HEVC spec shall output consistent syntax element of video parameter set, this document addresses the issue of video parameter set and proposes solution for that.

Not clear if action is required now.

It was remarked that J0432 is similar.

BoG recommends to rename the max_layer_minus1 as a reserved field, so that specific semantics aren’t necessary.

JCTVC-J0438 AHG10: Cross-check of JCTVC-J0196 [J. Xu (Microsoft)] [late]

JCTVC-J0257 AHG9/AHG10: Design of the Video Parameter Set [R. Skupin, V. George, T. Schierl]

This document proposes a design for the Video Parameter Set (VPS) that facilitates dealing with a video sequence on the application layer. We argue for the signalization of profile and level requirements in the VPS and for low level requirements, which are limited by level constraints, to be signaled as either u(n) or u(v). Furthermore, we explain the benefit of signaling coding dependencies and scalability information in the VPS and the advantages of using the parameter set extension mechanism for non-base layer signalization. Additionally, we propose to use an SEI message for indication of layer removal.

No redundant signaling between VPS and SPS

No entropy coding in VPS

Add a scalability_type syntax element in VPS.  It was suggested that a reserved field could be used in the base specification.

The syntax for the layer_id_plus1_dim_len could be placed after the vps extension flag, and no action would be required for the base specification.

Question about the interaction with the sub-bitstream extraction process.

It was remarked that it would be helpful in capabilities extension to include some syntax elements in the VPS.

There was some interest in fixed length coding instead of entropy coding in the VPS, at least at the beginning.  It was remarked that the VPS activation also requires entropy decoding.

JCTVC-J0484 AHG9/AHG10: Mental cross-check of JCTVC-J0257: Design of the Video Parameter Set [J. Boyce (Vidyo)] [late]

JCTVC-J0261 AHG9: Signaling of VPS Activation [T. C. Thang (UoA), J. W. Kang, H. Lee, J. Lee, J. S. Choi (ETRI)]

Video parameter set (VPS) can be used to describe essential information of “scalable layers” of a bitstream. However, to know which VPS is active, an extractor must parse slice headers, related PPS and SPS as well. This is obviously a big burden for any extractor. In this contribution, some simple mechanisms for activation signaling of video parameter set using additional messages are proposed.

3 options for placing the vps_id

- AUD

- new SEI message

- add to the recovery_point SEI message

ue(v) syntax elements proposed, perhaps u(v) better

Contribution JCTVC-I0524 had proposed vps_id required to be present in the slice header of RAP pictures, which is a 4th option.

Want to avoid start code emulation, perhaps could send vps_id_plus1

Miska to late-register a new contribution (J0546) based on I0524.

BoG recommends to adopt either option #2 or J0546.

JCTVC-J0481 Mental crosscheck for JCTVC-J0261 Signaling of VPS Activation [Hendry, B. Jeon (LG)] [late]

JCTVC-J0270 HEVC VUI Parameters with Extension Hooks  [Munsi Haque, Kazushi Sato, Ali Tabatabai, Teruhiko Suzuki (Sony)]

This contribution presents updates of the current HEVC VUI Parameters Syntax structure to consider the Temporal Scalability extension. These parameters can also be extended to meet future HEVC extensions.

Some parameters in the VUI are duplicated for each temporal layer.  Two options in the contribution about which parameters are duplicated.

J0114 also proposed putting some VUI information in VPS, and included support for temporal scalability.

J0074 also discusses HRD parameters per temporal layer via SPS per temporal layer

Option 2 was preferred by the group, which duplicated fewer of the values.

vui_num_temporal_layers, vui_temporal_id not needed in syntax

Which parameters should be replicated?

hrd parameters should not be allowed to be present for some temporal layers but not all.  Should be for all temporal sub-layers or none.  Move the loop after a presence flag, and not have the presence flags inside the loop.  Separate presence flag for NAL and VCL HRD.

Don’t need to duplicate low_delay_hrd_flag, sub_pic_cpb_params_present_flag, num_units_in_sub_tick

Agreement what should be duplicated per temporal layer and not.  Somewhat dependent on the decision of the location of some VUI parameters in the VPS.

BoG recommends a VUI definition based on the above, to be described in JCTVC-J0548, written by Munsi, YK, and Miska.

JCTVC-J0528 Mental cross-check of JCTVC-J0270: HEVC VUI Parameters with Extension Hooks [J. Boyce (Vidyo)] [late]

JCTVC-J0487 Scalable Video Coding Signalling in VPS [A. Luthra (Motorola Mobility)] [late]

Not presented.  Contributor not available.  Considered by the group to be relevant for the extension, but not to impact the base specification.

JCTVC-J0272 Simplifications of HRD parameters for Temporal Scalability [Munsi Haque, Kazushi Sato, Ali Tabatabai, Teruhiko Suzuki (Sony)]

This document considers the HEVC VUI Parameters Syntax structure with extension hooks for Temporal Scalability. In this modified VUI syntax structure, there are two HRD parameters syntax function calls that exist for each temporal layer.  For multiple temporal layers, such HRD parameters syntax could have redundant parameters. So this is analyzed to find out some common and shared parameters among them so that the size of the overall VUI parameters syntax structure can be reduced.

Group prefers the following:

cpr_flag shouldn’t be duplicated for all temporal sub-layers

Should be duplicated for each temporal sub-layer: cpb_cnt_minus1, bit_rate_value_minus1, cpb_size_value_minus1

Same syntax elements present for both vlc_hrd_parameters( ) and nal_hrd_parameters( )

BoG recommends an HRD parameters definition as captured above, to be described in JCTVC-J0548.

JCTVC-J0534 Mental Cross-check for JCTVC-J0272 - Simplifications of HRD parameters for Temporal Scalability [S. Deshpande (Sharp)] [late]

## 2.2 General VPS Questions and discussion

Is the base spec required to parse and use the VPS?  (Is there any replication of syntax elements between VPS and SPS in base spec?  )

Benefits of requiring the VPS in base spec:

- Clean design for now & extension, for HEVC specification as well as system specifications

- Reduces need for repetition of the same information

- MANE could use VPS and not need to use SPS(s).

- Identification of newly activated VPS will be easier than identification of newly activated SPSs, especially with repeated vps_id

- Could use VPS in capabilities exchange and application specs (e.g. RTP)

Cons of requiring the VPS in the base spec:

- Additional parameter set to parse and store

BoG recommends that there may be syntax elements in the VPS that are not present in the SPS, which are required for decoding.

What are candidates to be included in VPS in base spec?

  - profile and levels (including level per temporal sub-layer)

    - Send profile & max level in VPS

      - Anticipated in future extension that profile & max level will be sent individually for each layer

    - Optionally send temporal sub-layer profile & optionally send max level in VPS for each lower temporal sub-layer (can send level w/o sending profile)

    - Send level in SPS, may be lower than the max level sent in the VPS

    - Optionally send temporal sub-layer level in SPS for each lower temporal sub-layer

BoG agreement on profile and level as indicated above.

  - representation format

  - HRD

BoG agreement in principle to move HRD from VUI in SPS to VPS, as long as there is a mechanism to indicate the highest layer present in bitstream (including temporal sub-layers).  Revisit once text is available.

  - non-HRD VUI

  - RPS

Entropy coding in VPS?

Most essential items to be used by MANE to be fixed length coded, and at beginning of the VPS

Order of syntax elements in VPS:

vps_id u(n)

max temporal layers u(3)

reserved u(5) [for max layers]

profile

max level

temporal_id_nesting_flag

sub-layer profile & max level

byte pointer

loop over dec_pic_buffering, reorder, latency

HRD

extension_flag

BoG recommends VPS syntax as described above.  YK to draft text, which may be an output doc of the BoG.  BoG to meet on Saturday to review before reporting back to track.

## 2.3 Discussion on NAL unit header

J0063, J0231, J0250, J0112, J0113, and J0174 were all suggested to be related. Only the first four of these were discussed in the preceding AHG9 meeting.

Big questions on NAL unit header:

1, Remove nal_ref_flag?

yes

2. Any need for additional priority within a temporal sub-layer?

3. Should we add 5 (= 3 + 2) NUT values to indicate non-reference picture instead of nal_ref_flag?

4. Should there be a means to indicate that a pic is not used as a reference for its own temporal sub-layer?

NUT values already assigned

6 RAPs, TLA, GTLA, TFD, DLP, coded slice, 4 parameter sets, AUD, SEI, filler  = 18

21 if we assign 3 more

out of 48 possible (give 16 for systems), leaving 27 if we do this, or 30 if we don’t

suggestion to apply this only of coded slice, TLA, and GTLA NUT values, not the others.

BoG recommends to create 3 new NUT values (TLA, GTLA, and coded slice) to indicate that the slice is not used as a reference for another picture of the same temporal sub-layer, to be described in JCTVC-J0549.

5. If nal_ref_flag is removed, how should the bit be used?

- to increase the bits for layer_id/reserved bits?

- used for multi-standard support?

- used for nal unit type?

BoG agrees on  #1, should reorder syntax elements to make all reserved bits contiguous

| forbidden_zero_bit | f(1) |
| --- | --- |
| nal_unit_type | u(6) |
| reserved_zero_6bits | u(6) |
| temporal_id_plus1 | u(3) |

BoG recommends changing NAL unit header design as specified above.

6. Should the emulation prevention value be moved from reserved_one_5bits/layer_id to temporal_id?  (Would reduce number of possible temporal values from 8 to 7.)

Yes.  BoG recommends to do move emulation prevention from reserved_one_5bits/layer_id to temporal_id.  reserved_one_5bits is renamed reserved_zero_5bits, and temporal_id is renamed temporal_id_plus1.  (JCTVC-J0112)

7. Should temporal_id be combined with reserved_one_5bits to form an 8 bit layer_id value?

Not right now.

## 2.4 Document review of 5.12.1 NAL unit header (9)

JCTVC-J0063 AHG9: Syntax for NAL Packet Priority [Eun-Seok Ryu, Yan Ye, Yuwen He, Yong He (InterDigital)]

(This was initially reviewed in the AHG9 meeting, where discussion of this was chaired by Y.-K. Wang and G. Sullivan.)

Pictures in the same temporal level in a hierarchical-B structure can have different influence on error propagation and on decoded video quality. Currently in HEVC draft 7, the NAL unit header does not indicate packet priority within the same temporal layer. This contribution proposes two syntax options to indicate such priority of a NAL unit.

R. Sjoberg expressed a basic understanding of the proposal prior to the availability of a more formal indication of a cross check.

It was remarked that the prioritization may depend on the loss concealment method.

It was noted that the nal_ref_flag is basically always currently equal to 1 except at the highest temporal layer.

"Method 1" would change nal_ref_flag to be a nal_priority_flag indicating relative priority within a temporal level.

"Method 2" would provide a priority_id in the AUD, which could carry more bits than the nal_ref_flag.

It was noted that there would be no normative purpose for the proposed priority_id – that it is just metadata – and could be an SEI message.

Some participants expressed some skepticism about the value of the proposed priority_id.

Further discussed in BoG.

JCTVC-J0522 Mental crosscheck of JCTVC-J0063: Syntax for NAL Packet Priority [R. Sjöberg (Ericsson)] [late]

JCTVC-J0231 On nal_ref_flag [T. K. Tan, Junya Takiue (NTT Docomo)]

(This was initially reviewed in the AHG9 meeting, where discussion of this was chaired by Y.-K. Wang and G. Sullivan.)

This contribution seeks to clarify the purpose of the nal_ref_flag.  The nal_ref_flag does not seem to have any use in the decoding process apart from the final marking of the picture as reference or non-reference picture.

This contribution proposes to remove the nal_ref_flag syntax element from the NAL unit header and create 3 new NAL unit types for coded slices that can be either used for reference or not.

J0463 is reportedly a (late) cross-check.

It was suggested that an alternative would be to use the maximum temporal later value as a non-reference picture indication, rather than using the NUT or nal_ref_flag.

This (or something like it) seemed promising.

Further discussed in BoG.

JCTVC-J0250 Indication of non-reference pictures [R. Sjöberg, J. Samuelsson (Ericsson)]

(This was initially reviewed in the AHG9 meeting, where discussion of this was chaired by Y.-K. Wang and G. Sullivan.)

This contribution proposes to change the semantics of nal_ref_flag so that in addition to indicating non-reference pictures of the highest layer, it is also capable of indicating whether a picture in any layer is a non-reference picture of its own layer. This contribution claims that this makes it possible to indicate whether a picture is a reference picture or not in a sub-stream where the highest layer has been removed and thus indicate whether that picture can safely be removed from the sub-stream without affecting the decoding of the remaining pictures in the sub-bitstream.

It was commented that this would make it more difficult for a "middle box" to identify pictures that can be dropped. Unless the highest temporal ID in the bitstream is known, the non-reference pictures cannot be dropped.

Further discussed in BoG.

JCTVC-J0520 Mental cross-check of JCTVC-J0250 [Yan Ye] [late]

JCTVC-J0112 AHG9: Various comments on HEVC draft 7 [Y.-K. Wang (Qualcomm)]

(This was initially reviewed in the AHG9 meeting, where discussion of this was chaired by G. Sullivan.)

This document proposes the following:

- Removal of nal_ref_flag, and push the saved bit to reserved_one_5bits to make it to reserved_one_6bits; this proposal is suggested to be ignored if the proposal in JCTVC-J0113 is adopted

- Change of temporal_id to temporal_id_plus1 (The DPB size in level definitions only support hierarchical coding structures with GOP size up to 16 (see document JCTVC-J0111 for analyses of DPB size requirements for different GOP sizes), i.e., typically up to 5 temporal layers are supported.) However, it was commented that there may be a desire in the future to use deeper temporal nesting.

- Change of the value 0 of nal_unit_type from "Unspecified" to "Reserved".

- An alleged editorial fix to the semantics of the extension syntax elements in VPS, SPS, PPS, APS and slice data. This was not agreed. However, it was agreed that some further clarification of the intended tolerance of decoders for reserved values would be desirable.

- Extending of the extension mechanism to all types of NAL units. The participants did not support this part of the proposal.

- To support SEI NAL units that may follow the first VCL NAL unit in the same access unit

Further discussed in BoG.

JCTVC-J0469 AHG9: Mental cross-check of JCTVC-J0112 [T. C. Thang (UoA), Hendry (LG)] [late]

JCTVC-J0113 AHG10: High-level syntax hook for HEVC multi-standard extensions [Y. Chen, Y.-K. Wang (Qualcomm)]

This is a follow-up proposal of JCTVC-I0355. A prior document G149 was also suggested as relevant background information.

In JCTVC-I0355, a high-level syntax hook for HEVC multi-standard scalable or 3DV extensions wherein the base layer or view is AVC compatible is proposed. It proposed to use the following principles for the syntax design:

- For the NAL unit header length in the multi-standard scalable for 3DV extensions to be the same as in the existing HEVC design;

- That there should be sufficient NAL unit types to be used by HEVC and its potential future extensions, ideally the same as in the existing HEVC design;

- For the AVC NAL units to be distinguishable from the NAL unit header itself.

A comment was given in response to JCTVC-I0355 that an AVC decoder would not be able to distinguish between an AVC NAL unit and an HEVC NAL unit.

In this proposal, an improved design is proposed to solve the above issue while the above three design principles are still followed.

The proposed change to the NAL unit header is to remove nal_ref_flag and re-arrange the syntax elements in the HEVC NAL unit header.

The proposal uses the NUT range from 16 to 31 from the AVC perspective.

A participant questioned the need to multiplex AVC within HEVC within the bitstream level – suggesting to depend on the system level to provide that capability.

Cross-checked in J0492.

Further discussed in BoG.

JCTVC-J0492 AHG10: Mental cross-check of JCTVC-J0113 [M. M. Hannuksela (Nokia)] [late]

JCTVC-J0174 AHG 9 / AHG 10: On NAL unit header [T. Thang (UoA), J. Kang, H. Lee, J. Lee (ETRI), Hendry, B. Jeon (LG)]

This contribution discusses two items. First, it is suggested that the functionality of nal_ref_flag might be redundant as described in JCTVC-I0251 and JCTVC-I0355 so that the removal of the flag should be considered. To cover the flag functionality to differentiate reference and non-reference pictures, it is suggested to add a constraint to the semantics of temporal_id such that temporal_id of NAL units that contain slices of a non-reference picture must not be equal to 0.

It was commented that we could have a non-reference slice NUT.

Second, it is asserted that if future extensions of HEVC also use the current fixed 2 bytes NAL unit header size, there is only 5 bits, which is the reserved_one_5bits, available to be used to describe layer identification. The contributor suggested that this might be too small when considering that the extension might cover not only scalability extension but also multiview extension. Furthermore, it might not be necessary to treat temporal identification from identification of other scalability / view types in the extensions of HEVC. Therefore, it is proposed to combine reserved_one_5bits and temporal_id and change its name to layer_id.

Further discussed in BoG.

JCTVC-J0464 AHG9: Mental cross-check of JCTVC-J0174 [Y.-K. Wang (Qualcomm)] [late]

JCTVC-J0463 AHG9: Mental cross-check of JCTVC-J0231 [Y.-K. Wang (Qualcomm)] [late]

JCTVC-J0432 On NAL Unit Header and Video Parameter Set Design [B. Choi, J. Kim, J. Park (Samsung)] [late]

The NAL unit header and VPS design are proposed for HEVC 3D/scalable extension with consideration of hybrid 3D/scalable coding design with non-HEVC video coding standards. This proposal is originally prepared for JCT2 (HEVC 3D extension). However, it is also proposed in JCT because it contains some syntax change in the HEVC draft.

This proposal did not actually propose a change of the NUH of the base spec.

The VPS aspect of the contribution proposes to have an indicator of the type of standard associated with each layer. It was commented that this is a more elaborate scheme than what is expressed in our requirements and that it is something that could be done later if appropriate. Further study of this is encouraged.

JCTVC-J0239 AHG10 - Selective inter-layer prediction signaling for scalable extension [J. Xu (Microsoft)]

Proposes a flag to be included in the NAL unit header to indicate whether the current picture (layer representation) is used by layer representations from higher layers, similar as the discard_flag in the AVC based SVC, and change the 5-bit reserved_one_5bits to be 4-bit reserved_one_4bits.

Karsten: If a base layer is encoded first, and then add enhancement layers later.

Stephan: Instead of using one flag, we could use NAL unit types, equivalent to use of a fraction of bit. We should not reduce the 5 bits for layer_id for future extensions.

Further study was encouraged.

[YK to add more detailed notes.]

JCTVC-J0428 AHG10: Mental cross-check of Selective inter-layer prediction signalling (JCTVC-J0239) [K. Sugimoto, S. Sekiguchi (Mitsubishi)] [late]