| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0551 |
| --- | --- |

| Title: | Mental cross-check of JCTVC-J0344 (Refinement of random access point support) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 7180 08000 miska.hannuksela@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Abstract

This document provides a mental cross-check for JCTVC-J0344-v2.

# Mental cross-check

It is claimed that the cross-checker basically understands what is being proposed in JCTVC-J0344-v2. The cross-checker would like to summarize the proposal in JCTVC-J0344-v2 as follows:

The contribution proposes:

- An allocation NAL unit type values to CRA, BLA, and IDR pictures, some of which are additionally characterized by whether or not they (may) have leading pictures and whether or not these leading pictures can be TFD pictures.

- A constraint on the decoding order of leading pictures: “Among all coded pictures that follow a RAP picture in decoding order, all coded pictures that precede the RAP picture in output order shall precede, in decoding order, any coded picture that follows the RAP picture in output order.”