| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11 – 20 July 2012 | Document: JCTVC-J0XXX |
| --- | --- |

| Title: | Mental cross-check of Repeat picture output and HRD management (JCTVC-J0444) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Lowell Winger 97 Randall Drive, Waterloo, Ont, CAN | Tel: Email: | +1 519 725 9797 x304 lowell.winger@magnumsemi.com |
| Source: | Magnum Semiconductor |  |  |

_____________________________

# Abstract

This document reports a mental cross-check of the JCTVC-J0444, “Repeat picture output and HRD management” The contribution document was examined and understood by the cross-checker, and it was confirmed that the proposed draft text was consistent with the proposal.

# Understanding of the problem statement

Applications require support for CVSes that may correspond to sources with respective picture rates lower than the fixed picture rate intended for output, such as when the source corresponds to 24 Hz film content and the fixed picture rate is 60 Hz. Further, to minimize or eliminate the number of blank output pictures across transitions from one CVS to another CVS,  the no_output_of_prior_pics_flag is not equal to one. In such case, there is need to specify when a picture in the DPB is to be repeated for output and the associated HRD operation.

# Understanding of the proposal

Three alternatives are proposed:

- If fixed_pic_rate_flag = 1, pic_output_flag is not present and inferred equal to 1. Each decoded picture is output at least once, except when PicOutputFlag = 0 for a TFD picture associated with either the preceding BLA picture or the CRA picture that is the first coded picture in the bitstream. pic_out_idx is proposed as a u(4) but it could be u(3) if deemed three bits is deemed adequate.

- The syntax and semantics of pic_output_flag are independent of fixed_pic_rate_flag. repeat_pic_output_flag is present only if fixed_pic_rate_flag = 1 and  pic_output_flag = 1 since a picture that is not output would not require an output repetition specification. pic_out_idx is proposed as a u(4) but it could be u(3) if deemed three bits is deemed adequate.

- repeat_pic_output_flag is present if fixed_pic_rate_flag = 1 and does not depend on the syntax or semantics of pic_output_flag. pic_out_idx is proposed as a u(4) but it could be u(3) if deemed three bits is deemed adequate.

Interpretation of pic_out_idx is as per a table specified in the contribution.

# Draft text check

The proposed draft text just adding one bit for the indicator, and one sentence describing the semantics of the proposed indicator. It was confirmed that the description is simple and clear, and it is consistent with the proposal.

# References

- Arturo A. Rodriguez, A. K Katti, and H-Y Hwang, “Repeat picture output and HRD management”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-J0444, Stockholm, SE, July 2012