| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0556 |
| --- | --- |

| Title: | AHG6: Cross Check of JCTVC-J0286 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Geert Van der Auwera 5775 Morehouse Drive San Diego, CA 92121 | Email: | geertv@qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Summary

This cross check reports on the JCTVC-J0286 proposal. The BD-rate results for Algorithm 2 are found to match those reported in the proposal. In general, the proposal description presents the technical aspects, however, section 3.4 of the proposal does not contain the details on the boundary strength modification that is found in the source code.

# Algorithm 2 Description

The Algorithm 2 is identical to the algorithm described in the main part with one difference. In Algorithm 2, there are no changes in calculation of Bs value, which stays the same as in HM7.0. Instead, the tc_offset for intra blocks is increased from 2 to 4.

# Objective Results

Objective coding results have been verified for Algorithm 2 described in JCTVC-J0286. BD-rate results are obtained according to the common test conditions with HM7.0 as the reference. The encoding and decoding times are measured in a cluster computing environment, which reduces timing accuracy.  The BD-rates match those reported by the proponent. In addition the bit rates and PSNR values (QP 32, 37, 41) for additional sequences “Riverbed”, “WestWindEasy”, “DucksTakeOff” have been cross checked and found to match the reported values.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.4% | 0.6% | 0.6% | 0.8% | 0.9% | 0.8% |
| Class B | 0.3% | 0.7% | 0.7% | 0.8% | 0.7% | 0.7% |
| Class C | 0.3% | 0.8% | 0.8% | 0.5% | 1.0% | 1.0% |
| Class D | 0.3% | 0.8% | 0.8% | 0.4% | 0.9% | 1.0% |
| Class E | 0.1% | 0.3% | 0.0% | 0.4% | 0.5% | 0.2% |
| Class F | 0.1% | 0.8% | 0.9% | 0.2% | 1.0% | 1.2% |
| Overall | 0.3% | 0.7% | 0.7% | 0.5% | 0.8% | 0.8% |
|  | 0.3% | 0.7% | 0.7% | 0.5% | 0.9% | 0.8% |
| Enc Time[%] | 104% |  |  | 104% |  |  |
| Dec Time[%] | 100% |  |  | 95% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.4% | 0.4% | 1.0% | 0.6% |
| Class B | 0.1% | 0.9% | 0.7% | 0.4% | 0.8% | 0.7% |
| Class C | 0.1% | 0.9% | 0.8% | 0.2% | 0.9% | 0.7% |
| Class D | 0.1% | 1.1% | 0.8% | 0.1% | 0.6% | 0.5% |
| Class E |  |  |  |  |  |  |
| Class F | 0.1% | 1.2% | 1.1% | 0.2% | 1.5% | 1.3% |
| Overall | 0.1% | 0.8% | 0.8% | 0.3% | 1.0% | 0.8% |
|  | 0.1% | 0.8% | 0.8% | 0.3% | 1.0% | 0.8% |
| Enc Time[%] | 104% |  |  | 103% |  |  |
| Dec Time[%] | 99% |  |  | 94% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.3% | 0.2% | 0.5% | 0.5% | 0.4% |
| Class C | 0.1% | 0.4% | 0.3% | 0.5% | 0.2% | 0.3% |
| Class D | 0.0% | 0.6% | 0.4% | 0.1% | 0.6% | 0.2% |
| Class E | 0.3% | -0.1% | 0.3% | 2.0% | 0.6% | 0.8% |
| Class F | 0.0% | 0.0% | 1.4% | 0.4% | 1.1% | 1.3% |
| Overall | 0.1% | 0.3% | 0.5% | 0.6% | 0.6% | 0.6% |
|  | 0.1% | 0.3% | 0.5% | 0.6% | 0.7% | 0.7% |
| Enc Time[%] | 105% |  |  | 101% |  |  |
| Dec Time[%] | 107% |  |  | 101% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.3% | 0.4% | -0.2% | 0.1% | 0.5% | 0.1% |
| Class C | 0.0% | -0.1% | 0.2% | 0.3% | 0.3% | 0.3% |
| Class D | 0.0% | 0.6% | 0.7% | 0.1% | 0.1% | -0.4% |
| Class E | -0.1% | -0.9% | 0.7% | 0.7% | 0.8% | 0.9% |
| Class F | 0.0% | 0.0% | 0.5% | 0.2% | 1.1% | 0.3% |
| Overall | -0.1% | 0.1% | 0.3% | 0.3% | 0.5% | 0.2% |
|  | -0.1% | 0.0% | 0.3% | 0.3% | 0.5% | 0.4% |
| Enc Time[%] | 104% |  |  | 102% |  |  |
| Dec Time[%] | 105% |  |  | 102% |  |  |