| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0558r1 |
| --- | --- |

| Title: | JCT-VC BoG report: High-level parallel processing |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Michael Horowitz 1055 W. Hastings Street Vancouver, BC V6E 2E9 Canada | Tel: Email: | +1 778 731-3432 x323 michael@ebriskvideo.com |
| Source: | eBrisk Video, Inc. |  |  |

_____________________________

# Abstract

This document contains meeting notes for the BoG on high-level parallelism and includes recommendations to the JCT-VC on the topics related to high-level parallelism.

# Introduction

These notes are organized into four sections: general, tiles, wavefront parallel processing, and entry point signalling. The following text color coding conventions are applied:

- Red text indicates the result of the BoG discussion

- Blue text indicates additional conditions that should be considered in connection with the BoG discussion result

- Yellow highlighting indicates a BoG recommendation to the JCT-VC.

# Input documents

This section contains notes on input documents covered by the high-level parallelism BoG. The following four sub-sections reflect the four categories into which the input documents have been organized in the JCT-VC meeting notes.

## General

JCTVC-J0123 AHG4: On tiles and wavefronts [M. Coban, Y.-K. Wang (Qualcomm)]

Abstract

There were discussions that the syntax design itself should not disallow the combination of tiles and WPP, and whether the combination is allowed should be a profile issue. This document proposes a syntax that allows for combination of parallelism tools (tiles, WPP, and entropy slices), while whether different combinations are allowed can be either restricted through semantics or through profile definitions. For the combination of tiles and WPP, a restricted co-existence is proposed, by mandating every tile to start a new slice when both of these two parallelism tools co-exist in a picture. However, the document does not intend to propose to change the fact that in the Main profile (currently the only profile defined) combination of tiles and WPP is disallowed.

If tiles and wavefronts coexist then add constraint to start each tile with a new slice.

Replace tiles_or_entropy_coding_sync_idc with parallelism_idc

Comment: suggestion to split the parallelism_idc into separate flags

Comment: Proposal is reasonable IF more than one high-level parallelism tool is active simultaneously.  There is concern that recommending J0123 invites the possibility of simultaneous high-level parallel tools being active (c.f., JCTVC-338).  The BoG recommends that this topic be discussed in Track A.

Comment: should we consider not constraining tiles to start with a new slice since we do not have a clear use case that would use the combination.

Question: how would one implement a codec with both tiles and WPP simultaneously without the restriction?

Comment: There is a contribution that talks to that point (J0322)

Comment: separate 2 proposals in this contribution.

Note: proponent provided text for constraint to start each tile with a new slice if WPP and tiles are both active. The group thought it looked fine.

JCTVC-J0486 Mental cross-check of On tiles and wavefront parallel processing (JCTVC-J0123) [M. Horowitz (eBrisk)] [late]

Proposal makes sense.

JCTVC-J0249 Decoder parallelism indication [J. Samuelsson, R. Sjöberg, M. Westerlund, B. Burman]

Abstract

This document proposes a parameter; Threads to be used for indicating how many parallel decoding threads a bitstream is encoded for. The parameter also represents an approximation of the speed-up that can be achieved when exploiting the parallel decoding possibilities to its maximum. The parameter is proposed to be represented by the syntax element threads_idc in the sequence parameter set (SPS), after profile_idc and level_idc, and any value of threads_idc greater than 0 imposes additional constraints on the bitstream.

The proposed parameter is intended to be used together with profile and level to indicate the capabilities of a decoder: “For profile P1, when Threads is greater than or equal to T1, I can decode all bitstreams up to and including level L1”. The proposed parameter is also intended to be used as a declaration from an encoder of the level of parallelism in the bitstreams it produces.

Proposal to add one syntax element, threads_idc (to SPS), to indicate that the bitstream has been encoded using parallelism tools.

Proponents hypothesizes that processing bottleneck is determined by the number of luma samples.

Comment: this is not necessarily true. For some architectures number of bins is more relevant.

Comment: this was anticipated and an additional constraint was shown (from back up slide).  That additional constraint is not being proposed.

Consequently, the proponents propose constraint on the number of luma samples per tile and for WPP there is a different constraint.

Proposal does not contemplate case where more than one high-level parallelism tool is active at the same time.

Comment: HW decoders have to handle worst case so the threads_idc is not helpful (several comments to this effect)

Comment: A HW decoder could claim, for example, conformance to level 4.3 or if threads_idc >=4 it can decode level 5.2.

Comment: One expert liked the spirit of the proposal but would like to look at details.  Might be particularly useful if capabilities can be negotiated.

Comment: The proposal could complicate interoperability particularly when capabilities are not negotiated

Question: would this be better suited in a VUI message?

Comment: Perhaps the problem of supporting decoder parallelism is best considered outside the JCT-VC.  1) Negotiate capabilities 2) mandate certain configurations of parallelism tools through bodies that focus on particular application spaces (e.g. DVB)

Comment: the proposed syntax element does not affect decoder behavior and perhaps should not be included in the bitstream.

Interesting ideas and in particular inclusion in the VUI might be an interesting thing to consider.

JCTVC-J0479 Mental cross-check of JCTVC-J0249: Decoder parallelism indication [J. Boyce (Vidyo)] [late] [miss]  Has been uploaded now.  Well written and other comments are captured above.

JCTVC-J0284 Tiles and Wavefront Parallel Processing Restrictions For The Main Profile S. Worrall, A. Wise

Abstract

This contribution proposes some restrictions for the Main Profile of HEVC. The author argues that the proposed restrictions will reduce the conformance checking burden for implementers, and make parallel decoding easier to achieve. It is proposed that two different sets of restrictions be included in the Main Profile for Wavefront Parallel Processing (WPP) and tiles respectively. The proposed restrictions are independent of level, although restricting the use of parallel processing tools to levels greater than 3.1 is recommended. A spreadsheet has been provided to demonstrate the tile configurations specified by the recommended constraints.

Contribution proposes additional restrictions on the parallel processing tools (tiles and WPP)

For tiles the restrictions proposed are: tiles shall be used for levels 3.1, uniform_spacing_flag = 1, column width constraints to constraint the width to 384 (or something close without going under) when tiles are used.

Draft text was presented.

Fewer configurations make conformance testing easier

Comment: a universal decoder could not advantage of the proposed min num tiles, med num tiles and max num tiles

Comment: having a constraint on the maximum number of tiles would be useful

Comment: removal of non-uniform tile spacing is desirable for some decoder architectures

The tile dimension constraints agreed upon in this BoG’s notes under J0235 satisfies the intent of this contribution except for the mandating of the uniform_spacing_flag == 1.

There is no consensus to recommend adoption of uniform_spacing_flag == 1 constraint

## Tiles

JCTVC-J0039 AHG4/AHG9: Syntax for restricting slices and tiles [C.-W. Hsu, C.-Y. Tsai, Y.-W. Huang, Y.-C. Chang, C.-M. Wang, C.-Y. Cheng, S. Lei (MediaTek)]

Abstract

In the current HEVC, both slice and tile coding structures can coexist with normative constraints on the encoder side. The encoder constraints require that all slices within a tile shall be complete and all tiles within a slice shall be complete. However, a practical decoder may still have to deal with a bitstream that unintentionally violates the constraints. In this contribution, the conditions to signal end_of_slice_flag are changed to better guide an encoder to follow the constraints. With such changes, an unintentional violation of the constraints can be avoided. The proposal is as follows: When tiles are enabled, for slices contained in a tile, the end_of_slice_flag of the last slice granularity unit (LCU or CU) of the tile is not explicitly coded and is inferred as true if the slice starts in the middle of the tile; for tiles contained in a slice, the end_of_slice_flag syntax elements are coded as usual in the first tile, but in each of the rest tiles, only the end_of_slice_flag of the last slice granularity unit of the tile is explicitly coded while those non-last slice granularity units are not explicitly coded and are inferred as false. Simulation results reportedly show no bit rate increase and unchanged run time for the proposed syntax changes in comparison with HM-7.0-dev.

Conditions to signal end_of_slice_flag are changed to better guide an encoder to follow the normative constraints on tile and slice combinations.

Idea is to decrease the probability of an encoder manufacturer making a mistake.

Question: is wording sufficient to reduce the probability of a mistake or is a syntax change (as proposed) a better way to go?

Comment: as a result of this proposal there is a special check for parsing the inferred flags.

Comment: decoder can detect the kind of error this proposal seeks to protect against without this proposal

Comment: The proposed syntax changes will make it particularly difficult if the constraints that this proposal addresses change at some time in the future.

Interesting idea but there was no consensus to recommend adoption

JCTVC-J0471 Crosscheck of Syntax for restricting slices and tiles (JCTVC-J0039) [M. Coban, W.-J. Chien (Qualcomm)] [late]  Contribution checked out well.

JCTVC-J0042 AHG4/AHG9: Syntax for restricting tile numbers and tile sizes [C.-Y. Tsai, C.-W. Hsu, Y.-W. Huang, Y.-C. Chang, C.-M. Wang, C.-Y. Cheng, S. Lei (MediaTek)]

Abstract

In the current HEVC, an encoder shall obey a normative constraint on tile width. The encoder constraint requires that tile width shall be equal to or greater than 384 pixels. However, a practical decoder may still have to deal with a bitstream that unintentionally violates this constraint. It is proposed to modify the syntax elements of tile width and number of tile columns to better guide an encoder to follow the constraint. With such changes, only legal tile widths have corresponding codewords to code them. Thus, an unintentional violation of the constraint can be avoided.

Idea is to decrease the probability of an encoder manufacturer making a mistake.

Replace column_width[] syntax element with column_width_minus_min_ctb_num[]

Additional syntax element to prevent non-uniform tile case from causing

Comment: 384 luma sample width may change

Comment: we could make it a variable (perhaps profile dependent)

Comment: a decoder could tell very early (by parsing PPS) whether there will be a conformance problem making this contribution potentially less valuable.

Comment: knowing the conformance problem early may not be helpful since a decoder may have to decode this kind of non-conformant bitstream.

Comment: proposed coding of syntax element as tu() would make it the only tu() coding in PPS (or for any non CABAC encoded syntax element)

Related contributions J0235, J0284

Things have changed slightly as a result of our recommended adoption of J0235

BoG recommends that the proponents:

- Replace tu(v) with ue(v)

- Make changes to reflect J0235

  - Add constraints for minimum tile height (64 luma samples)

  - Change the numbers to reflect new minimum tile width (256 luma samples)

For revisit in Track A

JCTVC-J0412 Cross-check of “AHG4/AHG9: Syntax modifications for tile width constraint” (JCTVC-J0042) [M. Horowitz, S. Xu (eBrisk)] [late]

JCTVC-J0085 AHG9: On number of tile rows limit [M. Zhou(TI)]

Abstract

In HM7.0 tile column width is restricted to be larger than or equal to 384, but there is no restriction on tile row height. In the worst case there will be 340 tiles in a 1080p frame (i.e. 1920*1088/384/16). Such a high number of tiles is unlikely required by real-world applications, but imposes a significant burden on real-time decoder design due to tile transition overheads and CABAC reset at end of each tile. It is recommended to impose restriction on tile row height as follows: a conforming bitstream shall satisfy  num_tile_rows_minus1 ≤ (pic_height_in_luma_samples + 96)/192  - 1. For a 1080p frame this additional constraint effectively reduces the worst case tile number from 340 to 30.

Proposes to impose a minimum tile size of 384 x 192.

Proposed minimum tile size used to derive tile size constraints

JCTVC-J0289 On parallel partitions in profiles [A. Wells, C. Fogg (Ambarella & Harmonic)]

Abstract

This proposal requests that among the final generic HEVC bitstream restrictions, the smallest tile dimensions permitted are: 512 pixels wide (rather than the current Annex A limit of 384 pixels) by 256 pixels tall (height is not currently restricted).  It is desirable that tiles be permitted to encompass the full size of the picture in the vertical and/or horizontal dimensions.  In particular, the authors advocate that the final restrictions should permit an encoder to optionally partition a 4K @ 60 fps (Level 5.2) sequence into an arrangement of 4 approximately uniformly spaced tile columns with no tile rows (i.e. 4x1 geometry). Should mandatory parallelism be imposed on bitstreams (max tile size), then it is requested that the total pixels per tile not exceed 2,228,224 (the current MaxLumaFS values for Level 4.2) for any level.  Finally, if Main Profile is to have any parallel partition tools at all, then tiles should be included (as per current HEVC draft), or both tiles and wavefront tools be placed into a parallel partition profile though used mutually exclusively within a sequence (as per current HEVC draft). The authors recognize that the market place will implement both these parallel partition schemes anyway, thus creating not just another “But VLC will play it!” scenario, rather, high volume semiconductor decoders will be obligated as well.

Proposes to impose a minimum tile size of 512 x 256

Proposed minimum tile size used to derive tile size constraints

JCTVC-J0088 AHG4: Enable parallel decoding with tiles [M. Zhou (TI)]

Abstract

Real-time UHD decoding can exceed the capability of a single core decoder. To enable parallel decoding on multi-core platforms, it is proposed to mandate evenly divided sub-pictures (tiles) for high levels to guarantee pixel-rate balancing among cores when sub-pictures are processed in parallel. The key points of proposal are: 1) A picture is divided into a number of sub-pictures of equal size (in units of LCUs); 2) Sub-pictures are independent, only in-loop filters are allowed to cross the sub-picture boundaries; 3) Tiles, slices, entropy slices and WPP are contained in sub-pictures and cannot cross sub-picture boundaries; 4) The sub-picture partitioning information is signaled with tile syntax. If sub-pictures are mandated, tiles have to be uniformly spaced in both directions (i.e. option 1 same degree of parallelism for encoder and decoder) or uniformly spaced in vertical direction only (i.e. option 2 to allow more parallelism on the encoder side than on the decoder side). 5) Sub-picture entries in bitstream are signaled in the APS. The proposed change guarantees pixel-rate balancing on multi-core platforms and allows building a multi-core decoder by simply replicating the single core decoder without need of increasing the line buffer size.

The BoG recommends that this contribution be considered during the profiles and levels discussion in the plenary

JCTVC-J0206 AHG 4: Asynchronous Tile Output [Hendry, B. Jeon (LG)]

Abstract

In current HEVC specification, when tile is used, coded data from each tile shall be written to output bitstream in the sequential order according to tile identification in increasing order from the first tile to the last one. It is assessed that, for parallel encoders, such design may cause delay in writing coded data into output bitstream since encoding of tile with lower tile_id is not always completed earlier due to possible processing speed of encoder resources or different texture complexity of regions in an input picture.

In this contribution proposes Asynchronous Tile Output (ATO). ATO allows encoding process that finishes faster / earlier writing its coded data into output bitstream without having to wait for process of lower tile_id to be written first so that delay can be reduced / avoided. To accommodate the proposed ATO with minimum changes to current specification, in this contribution, there are two technical options:

Option 1: When tile is used, signal tile_id (i.e., offset_tile_id [i]) for entry_point_offset [i].

Option 2: Extend option 1 with one flag in PPS to control ATO, that is, if the flag is PPS is not set, the ATO is not enabled and offset_tile_id [i] is not necessary to be signalled.

Experiment results show that the effect of the signalling offset_tile_id [i] to BD-rate performance is 0% for intra only, 0.1% for random access and low delay configurations.

Motivation for asynchronous tile output is to avoid the use of an intermediate buffer by not having to reorder the data at the encoder

Idea is to put tile ID into entry point signalling

Question: if a picture contains more than one slice is the slice decoding order preserved.  Answer is yes.

Comment: the benefit may not be worth the change

Comment: the change is minimal

Question: What about deblocking?

Comment: if a decoder wishes to deblock on the fly then the decoder would have to wait for all the bits in a slice to arrive (in the worst case)

Comment: all tiles in a slice would typically arrive at the same time (in the same packet)

Question: If the motivation is not to use an intermediate buffer (for reordering) do we achieve our goal since isn’t and intermediate buffer required to build slice header

Comment: there is a flag to deactivate feature

Comment: some encoders may use the feature

Comment: there are a couple of experts in support and a few not in support

Interesting idea but there was no consensus to recommend adoption

JCTVC-J0423 Crosscheck of Asynchronous Tile Output (JCTVC-J0206) [G. Clare, F. Henry (Orange Labs)] [late]

The code matched the proponent’s claim and the results matched.

JCTVC-J0209 AHG 4: Constraint for slice and tile [Hendry, J. Kim, B. Jeon (LG)]

Abstract

This contribution identifies a possible decoding issue when one or more slice(s) contains multiple tiles, specifically, when there is a gap in tile_id of tiles that belong to the same slice. When such condition happens, decoder might misinterpret position of the next LCU it will process which may lead to incorrect reconstruction.

To deal with the issue, it is proposed to add a new constraint governing slice and tile such that tiles that exist in a slice shall be sequential in raster scan order and such as there must be no discontinuity in tile_id.

Proposes adding clarifying text “Tiles that exist in a slice shall be sequential in raster scan order and such as there must be no discontinuity in tile_id” to reduce the probability that an encoder manufacturer will create a non-conformant bitstream.

The BoG suggests that the current specification for coding tree block ordering within a picture when more than 1 tile is present can be clarified by specifying the scan order of tiles.  We suggest adding a definition that describes how tiles are scanned within a picture.

JCTVC-J0485 Mental cross-check of AHG 4: Constraint for slice and tile (JCTVC-J0209) [M. Horowitz (eBrisk)] [late]

The reviewer believes the proposed constraint already exists in the WD, however additional clarification may be beneficial.  For example, the definition of a slice might be changed from:

“slice: An integer number of coding tree blocks ordered consecutively in the raster scan. The division of each picture into slices is a partitioning. The coding tree block addresses are derived from the first coding tree block address in a slice (as represented in the slice header).”

to

“slice: An integer number of coding tree blocks ordered consecutively in the tile scan. The division of each picture into slices is a partitioning. The coding tree block addresses are derived from the first coding tree block address in a slice (as represented in the slice header).”

JCTVC-J0235 On tile size restrictions in HEVC [A. Fuldseth (Cisco)]

Abstract

In this contribution the effect on compression performance when using multiple tiles for encoding is analyzed. Using the HM software, the BDR loss for different number of tiles per frame is measured. It is asserted that for many real-time multi-core encoder implementations, the benefit from using more compute power through having a relatively large number of cores work in parallel is likely to be higher than the loss due to dependency breaks at tile boundaries. On the other hand, it has also been argued that a large number of tiles per frame represent an implementation burden for single-core decoder designs. As a consequence, the current HEVC draft text mandates a minimum tile width of 384 luma samples. A desire to mandate a minimum tile height is also anticipated. This contribution proposes changes to the HEVC draft text to better satisfy the needs of both multi-core encoder and single-core decoder implementations. In particular, it is proposed to make the number of tiles a decoder capability parameter, where the maximum number of tiles vertically and horizontally applies only to the maximum resolution at each level. This is achieved by amending table A-1 to include a maximum number of tiles horizontally and vertically. This approach allows for the added flexibility for an encoder to use the same number of tiles independent of resolution and at the same time allows the decoder to express support for a maximum number of tiles though claiming conformance to a specific level.

One point made is that the gain in available CPU resources that more tiles more than compensates the losses due to having more tiles

More tiles useful for load balancing

Proposal: level dependent limits on maximum number of vertical tiles and maximum number of horizontal tiles  -- this would replace the 384 luma sample minimum tile width constraint (and obviate the need for a horizontal minimum tile size.

Comment: In AVC it is common to express restrictions according to level

Comment: proposal present clear motivation

Comment: for HW designs it helps to limit worst c

Comment: Allows smaller resolution video to use tiles

Comment: it is still desirable for some architectures to have a minimum tile width because of column buffer memory access patterns (narrow tiles can lead to memory bursts)

Comment: It may be desirable to have a minimum tile width and height specified in addition to the proposed constraints (presumably these constraints will be smaller than they would otherwise be without the proposal)

Table generated using:

maxHorTiles = floor(width/384)

maxVerTiles = floor(height/192)

Comments:

Rounded upwards to 1 for level 1 & 2

Table A-10-1 does not specify width and height. Aspect ratios implied by table A-10-4 are used.

The BoG recommends replacing the 384 luma sample minimum tile width constraint with the following table and minimum tile width and height constraints

| Level | Max luma pixel rate MaxLumaPR (samples/sec) | Max luma picture size MaxLumaFS (samples) | Max bit rate MaxBR (1000 bits/s) | Min Compression Ratio MinCR | MaxDpbSize (picture storage buffers) | Max CPB size (1000 bits) | Max # of tile rows MaxTileRows | Max # of tile columns MaxTileColumns |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 552,960 | 36,864 | 128 | 2 | 6 | 350 | 1 | 1 |
| 2 | 3,686,400 | 122,880 | 1,000 | 2 | 6 | 1,000 | 1 | 1 |
| 2.1 | 7,372,800 | 245,760 | 3,000 | 2 | 6 | 6,000 | 1 | 1 |
| 3 | 13,762,560 | 458,752 | 5,000 | 2 | 6 | 5,000 | 2 | 2 |
| 3.1 | 33,177,600 | 983,040 | 9,000 | 2 | 6 | 9,000 | 3 | 3 |
| 4 | 62,668,800 | 2,088,960 | 15,000 | 4 | 6 | 15,000 | 5 | 5 |
| 4.1 | 62,668,800 | 2,088,960 | 30,000 | 4 | 6 | 30,000 | 5 | 5 |
| 4.2 | 133,693,440 | 2,228,224 | 30,000 | 4 | 6 | 30,000 | 5 | 5 |
| 4.3 | 133,693,440 | 2,228,224 | 50,000 | 4 | 6 | 50,000 | 5 | 5 |
| 5 | 267,386,880 | 8,912,896 | 50,000 | 6 | 6 | 50,000 | 11 | 10 |
| 5.1 | 267,386,880 | 8,912,896 | 100,000 | 8 | 6 | 100,000 | 11 | 10 |
| 5.2 | 534,773,760 | 8,912,896 | 150,000 | 8 | 6 | 150,000 | 11 | 10 |
| 6 | 1,002,700,800 | 33,423,360 | 300,000 | 8 | 6 | 300,000 | 22 | 20 |
| 6.1 | 2,005,401,600 | 33,423,360 | 500,000 | 8 | 6 | 500,000 | 22 | 20 |
| 6.2 | 4,010,803,200 | 33,423,360 | 800,000 | 6 | 6 | 800,000 | 22 | 20 |

Note: except for the values in the right-most two rows the values in this table are exemplary.

Minimum tile width constraint:  256 luma samples

Minimum tile height constraint:  64 luma samples

JCTVC-J0383 A cross-check report for JCTVC-J0235 on tile size restriction in HEVC [K. Misra, K. Seung-Hwan, A. Segall (Sharp)] [late] Confirmed software and results.  Proposal makes sense.

## Wavefront parallel process (WPP)

JCTVC-J0032 An HEVC transcoder converting non-parallel bitstreams to/from WPP [G. Clare, F. Henry (Orange Labs)]

Abstract

This contribution presents a transcoder that can convert an HEVC bitstream from non-parallel to WPP, and the converse. This is achieved by plugging the output of the parsing process of HM7.0 into the input of an entropy coder. The process is lossless (the decoded videos are identical) and reversible (bitstreams transcoded twice are bit-level identical). Furthermore, producing a WPP bitstream by transcoding a non parallel one rather than from the direct source produces +0.1% (I), +0.1% (RA) and +0.3% (LDB) performance degradation measure in BD-rate. The same behavior is observed when converting a WPP bitstream to a non-parallel. The software is provided to interested JCTVC experts.

Informational document

Motivation is to show that non-parallel streams can be made parallel

Transcoding to and from WPP format, losses are small, reconstructions are identical

Software is available upon request

JCTVC-J0040 AHG4/AHG9: Syntax for restricting slices and WPP [C.-W. Hsu, C.-Y. Tsai, Y.-W. Huang, Y.-C. Chang, C.-M. Wang, C.-Y. Cheng, S. Lei (MediaTek)]

Abstract

In the current HEVC, both slice and WPP coding structures can coexist with a normative constraint on the encoder side. The encoder constraint requires that a slice must end in a LCU row if the slice starts in the middle of the LCU row. However, a practical decoder may still have to deal with a bitstream that unintentionally violates the constraint. In this contribution, the conditions to signal end_of_slice_flag are changed to better guide an encoder to follow the constraint. The proposal is as follows: When WPP is enabled, the end_of_slice_flag of the last slice granularity unit (LCU or CU) of a LCU row is not explicitly coded and is inferred as true if the slice starts in the middle of the LCU row. Simulation results reportedly show no bit rate increases and unchanged run time for the proposed syntax changes in comparison with HM-7.0-dev.

Contribution is in same spirit of JCTVC-J0039 (i.e., to decrease the probability of an encoder manufacturer making a mistake).

Comment: it is likely that similar comments to J0039 apply

Comment: This proposal has only one constraint compared with two constraints in J0039 – making this simpler.

Comment: for J0039 of the biggest concerns was that the proposed changes in syntax and semantics would lock in a particular constraint.

Comment: A similar concern exists in this case to

Interesting idea but there was no consensus to recommend adoption

JCTVC-J0472 Crosscheck of Syntax for restricting slices and WPP (JCTVC-J0040) [M. Coban, W.-J. Chien (Qualcomm)] [late]

JCTVC-J0279 CU skip and split CABAC context modification for flexible wavefront parallel parsing [M. Coban, W.-J. Chien, M. Karczewicz (Qualcomm)]

Abstract

This contribution proposes removal of CU skip and split CABAC dependency on the upper CTB row for enabling parallel parsing of wavefront substreams with CTB row level synchronization instead of more restrictive CTB level synchronization. It is reported that the proposed context modeling CU skip and split flag results in 0.02%, 0.05%, 0.09% BD-rate loss for AI-Main, RA-Main, LD-Main configurations, respectively, and 0.02%, 0.06%, 0.14% BD-rate loss for AI-HE10, RA-HE10, LD-HE10 configurations, respectively.

Considered in Track B earlier.

JCTVC-J0425 AHG4: Improving parallelization efficiency of WPP using Overlapped Wavefront [M. Alvarez-Mesa, V. George, T. Schierl (HHI), C. C. Chi, B. Juurlink (TU Berlin)] [late]

Abstract

In this document we present Overlapped Wavefront (OWF) execution that improves the parallelization efficiency of WPP.  For straightforward implementation OWF would require a restriction on the downward motion vector size. The required motion vector restriction can be applied to the specification of HEVC levels as it has been done before for H.264/AVC. We implemented the OWF approach on a multithreaded decoder and evaluated it on a parallel machine.

Informational document to improve WPP parallelization

Idea is to move threads as they become available to encode the next picture.  One catch, the reference has to be available.  Proposal notes that restricting the vertical MV length can guarantee the reference is available for the next picture.

Contribution shows speed up vs. WPP without overlapped wavefront.  Gains are due to roll-on / roll-off delay of WPP without overlapped wavefront.

## Entry point signalling

JCTVC-J0033 WPP entry points simplification [G. Clare, F. Henry (Orange Labs)]

Abstract

This contribution presents a simplified method to indicate entry point offsets for WPP substreams. Due to historical reasons in HM6.0 and earlier versions, combining WPP and slices sometimes produced empty substreams with corresponding zero-length offsets, due to the interleaved aspects of WPP substreams. Now that WPP substreams correspond to one line of CTBs, it becomes possible to modify the design of entry points so that there are no empty substreams and zero offsets anymore. The proposed modification produces a rate distortion performance of -3.4%, -3.3%, -2.4%, -2.4% (AI, RA, LB, LP) when tested with 1500 byte slices, against HM 7.0.

Simplification opportunity arises as a result of the last meeting’s simplification of WPP to restrict one substream per LCU row.

Gains are only for the case when a picture contains more than one slice

Minor semantic changes, no syntax changes proposed

Comment: like in spirit

Consensus is to recommend adoption in spirit, but semantics need to be further clarified and revisited

JCTVC-J0162 Crosscheck for JCTVC-J0033 [Hendry, B. Jeon (LG)]

The results matched and the software is as described in the proposal.  The proposal is effectively a cleanup for what was decided in Geneva in April.

JCTVC-J0081 AHG4: Signaling sub-stream entries in APS for parallel decoding [M. Zhou (TI)]

Abstract

In the current HM7.0 design, sub-stream entries are signaled in slice header only to facilitate single sub-stream switching of decoding parallelized streams with single core decoders. No entry points are available in the bitstream to assist parallel decoding on multiple core platforms. This contribution advocates signaling of sub-stream entry points in the APS as well to facilitate parallel decoding. Each sub-stream entry point in the APS shall be a parallel decoding entry point; the number of sub-stream entry points signaled in APS is not necessarily equal to the number of WPPs, tiles or slices in the picture referring to the APS.

The BoG recommends that this contribution be considered during the profiles and levels discussion in the plenary

JCTVC-J0299 AHG4: On entry point coding [S.H. Kim, K. Misra, A. Segall (Sharp)]

Abstract

Two modifications to the coding of entry point offsets are reported.  These modifications are asserted to provide some gains in coding efficiency for scenarios where the number of bytes between entry points is small.  The first modification changes the binarization process for coding entry point offsets; the second allows signaling a “minimum entry point value” and then signaling the entry point information relative to this minimum value.  Results show the proposed scheme changes BD-rate about -0.50% on average in Class E sequence with 24 entry points per frame.

Modify binarization, and use a predictor and code delta offset

Comment: saves bits with little complexity – I like it

Question: would proponent provide the break down in gain between using minimum offset and quotient remainder?

No results for WPP were reported

Comment: current scheme is simple enough

No consensus to adopt.

JCTVC-J0475 Cross-check of JCTVC-J0299 - On entry point coding [A. Fuldseth (Cisco)] [late] [miss]

Simulations still running, but all results that are complete match.

JCTVC-J0322 AHG4: Entry point signaling for wavefront substreams within tiles [K. Misra, K. Seung-Hwan, A. Segall (Sharp)]

Abstract

The draft standard does not allow for the combined use of wavefront parallel processing and tiles in any future profile.  In the event that the JCTVC decides to allow future, combined signalling, this contribution proposes syntax and semantics to clarify the signalling of entry points when both tiles and wavefront parallel processing are enabled.  The proposed syntax accounts for current constraints that each coding tree block (CTB) row within a tile represents a separate wavefront substream.

Comment: if it is agreed that we should have syntax supporting both tiles and WPP, we should consider harmonizing this proposal with J0123.

BoG recommends that the proponents of J0123 and J0322 harmonize their proposals to present in Track A during the review of the BoG’s report in the event that Track A decides to allow syntax supporting simultaneous use of tiles and WPP.