| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0561 |
| --- | --- |

| Title: | Mental cross-check of JCTVC-J0283 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Cross-check |  |  |
| Author(s) or Contact(s): | T. Davies | Email: | thdavies@cisco.com |
| Source: | Cisco Systems |  |  |

_____________________________

# Abstract

This document reports a mental cross-check of JCTVC-J0283, “Field coding and MaxDpbSize constraints” The contribution document was examined and understood by the cross-checker, and it was confirmed that the proposed draft text was consistent with the proposal.

# Problem statement

JCTVC-J0283 [1] proposes to allow the DPB size in picture units to be increased by a factor of 2 when the picture size is less than or equal to half of the maximum size permitted in a level. This is achieved by defining a value MaxDpbSizeFS determined in the level table A-10-1 in the HEVC draft text [2], which is the number of pictures allowed in the DPB in units of the largest possible picture. Then  MaxDpbSize is determined by:

MaxDpbSize = Floor(3 - 2 * PicSizeLuma/MaxLumaFS) * MaxDpbSizeFS

The motivation for this change is to support a larger number of reference pictures for field coding of interlace material.

# Comments

It is noted that the formula above limits the multiple of MaxDpbSizeFS to 2. If MaxDpbSizeFS < 9 this means that the maximum number of reference pictures supported is still fewer than in AVC [3].

# References

[1]	JCTVC-J0283, “Field Coding and MaxDpbSize constraints”, D.Flynn, A. Gabriellini, M. Mrak and M. Naccari (BBC), 10th Meeting of JCT-VC, Stockholm, SE, 11 – 20 July 2012

[2]	JCTVC-I1003, “High Efficiency Video Coding (HEVC) text specification draft 7”, B. Bross, W.-J. Han, G. J. Sullivan, J.-R. Ohm, T. Wiegand (Editors), JCTVC 9th Meeting, Geneva 27 April – 7 May 2012

[3]	ITU-T Rec. H.264 | ISO/IEC 14496-10 Advanced Video Coding

# Patent rights declaration(s)

Cisco Systems may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).