| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11 – 20 July 2012 | Document: JCTVC-J0562 |
| --- | --- |

| Title: | HRD parameters in VPS |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Qualcomm Incorporated Miska M. Hannuksela Nokia Corporation |  |  |
| Source: | Qualcomm Incorporated, Nokia Corporation |  |  |

_____________________________

# Abstract

The contribution proposes moving HRD parameters from SPS to VPS and text for the HRD (HEVC Annex C) specifying which HRD parameters are used for conformance checking.

# Introduction

A problem was identified in JCTVC-J0074 that when the conformance of a bitstream containing scalable layers is checked using HEVC v1 standard or an HEVC v1 decoder decodes a bitstream containing scalable layers, the HRD parameters of the “highest” layer present in the bitstream should be used. However, HEVC draft 7 lacks the signalling of HRD parameters of higher layers. In other words, there is no clear HRD operation specified when bitstreams with enhancement layers are fed to a decoder conforming to the base specification.

The BoG on VPS and NAL unit header acknowledged the above-mentioned problem (see the BoG report JCTVC-J0550). A solution for the problem had been provided in JCTVC-J0074, but as different assumptions on VPS/SPS design had been made in JCTVC-J0074 from those the BoG ended up to recommend, the solution of JCTVC-J0074 was aligned to the BoG recommendation of VPS/SPS design in this contribution. The BoG recommended to consider this contribution in one of the JCTVC meeting tracks or plenary.

The green-highlighted parts edited on top of the BoG recommendation (“JCTVC-J0550 BoG output text.docx”), while the yellow highlights and red strikethroughs are inherited from the BoG recommendation and merely indicate the changes proposed by the BoG compared to the HEVC draft 7.

# Video parameter set RBSP syntax and semantics

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| video_parameter_set_id | u(4) |
| temporal_id_nesting_flag | u(1) |
| reserved_zero_2bits | u(2) |
| reserved_zero_6bits | u(6) |
| vps_max_sub_layers_minus1 | u(3) |
| profile_level( 1, vps_max_sub_layers_minus1 ) |  |
| next_essential_info_byte_offset | u(12) |
| for( i = 0; i <= vps_max_sub_layers_minus1; i++ ) { |  |
| max_dec_pic_buffering[ i ] | ue(v) |
| max_num_reorder_pics[ i ] | ue(v) |
| max_latency_increase[ i ] | ue(v) |
| } |  |
| num_hrd_parameters | ue(v) |
| for( i = 0; i < num_hrd_parameters; i++ ) { |  |
| if( i > 0 ) |  |
| op_point( i ) |  |
| hrd_parameters( i = = 0, vps_max_sub_layers_minus1 ) |  |
| } |  |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

video_parameter_set_id identifies a video parameter set. The value of video_parameter_set_id shall be in the range of 0 to 15, inclusive.

vps_max_sub_layers_minus1 plus 1 specifies the maximum number of temporal sub-layers that may be present in the bitstream. The value of vps_max_sub_layers_minus1 shall be in the range of 0 to 6, inclusive.

[Ed.(YK): Add a definition of (temporal) sub-layer in Clause 3.]

vps_temporal_id_nesting_flag specifies whether inter prediction is additionally restricted for the coded video sequence.

Dependent on vps_temporal_id_nesting_flag, the following applies.

–	If vps_temporal_id_nesting_flag is equal to 0, additional constraints may not be obeyed.

–	Otherwise (vps_temporal_id_nesting_flag is equal to 1), the following applies.

–	For each access unit auA with TemporalId equal to tIdA, an access unit auB with TemporalId equal to tIdB that is less than or equal to tIdA shall not be referenced by inter prediction when there exists an access unit auC with TemporalId equal to tIdC that is less than tIdB, which follows the access unit auB and precedes the access unit auA in decoding order.

NOTE 1 – The syntax element vps_temporal_id_nesting_flag is used to indicate that temporal up-switching, i.e., switching from decoding of up to a specific TemporalId tIdN to decoding up to a TemporalId tIdM that is greater than tIdN, is always possible.

reserved_zero_6bits shall be equal to 0 in bitstreams conforming to this Recommendation | International Standard. Other values for reserved_zero_6bits are reserved for future use by ITU-T | ISO/IEC. Decoders should ignore values of reserved_zero_6bits not equal to 0 in a video parameter set NAL unit.

NOTE 2 – It is anticipated that in future scalable and/or 3DV extensions of this specification, this field specifies the maximum number of layers that may be present in the coded video sequence, wherein a layer may e.g. be a spatial scalable layer, a quality scalable layer, a texture view or a depth view.

reserved_zero_2bits shall be equal to 0 in bitstreams conforming to this Recommendation | International Standard. Other values for reserved_zero_2bits are reserved for future use by ITU-T | ISO/IEC. Decoders should ignore values of reserved_zero_2bits not equal to 0 in a video parameter set NAL unit.

next_essential_info_byte_offset specifies the byte offset of the next set of profile and/or level information and other fixed-length coded information in the VPS NAL unit, starting from the beginning of the NAL unit. Decoders conforming to this Recommendation | International Standard shall ignore the value of next_essential_info_byte_offset.

NOTE 3 – In a future extension of this specification, e.g. a scalable coding extension or a 3DV extension, VPS information for non-base layer or view may be included in the VPS NAL unit, after the VPS information for the base layer or view. It is anticipated that the VPS information for non-base layer or view also starts from fixed-length coded profile, level and other information that are essential for session negotiation and/or capability exchange. The byte offset specified by next_essential_info_byte_offset would then help to locate and access that essential information in the VPS NAL unit without the need of entropy decoding, which may not be equipped with some network entities that may desire to access only all the information in the VPS that is essential for session negotiation and/or capability exchange.

- max_dec_pic_buffering[ i ] specifies the required size of the decoded picture buffer in units of picture storage buffers for the bitstream subset as specified in subclause Error: Reference source not found with i as input. The coded video sequence shall not require a decoded picture buffer with size of more than Max( 1, max_dec_pic_buffering[ i ] ) picture storage buffers to enable the output of decoded pictures at the output times specified by dpb_output_delay of the picture timing SEI messages. The value of max_dec_pic_buffering[ i ] shall be in the range of 0 to MaxDpbSize (as specified in subclause Error: Reference source not found), inclusive.

- max_num_reorder_pics[ i ] indicates the maximum allowed number of pictures preceding any picture in decoding order and succeeding that picture in output order for the bitstream subset as specified in subclause Error: Reference source not found with i as input. The value of max_num_reorder_pics[ i ] shall be in the range of 0 to max_dec_pic_buffering[ i ], inclusive.

- max_latency_increase[ i ] not equal to 0 is used to compute the value of MaxLatencyPictures[ i ] as specified by setting MaxLatencyPictures[ i ] to max_num_reorder_pics[ i ] + max_latency_increase[ i ] for the bitstream subset as specified in subclause Error: Reference source not found with i as input. When max_latency_increase[ i ] is not equal to 0, the value of MaxLatencyPictures[ i ] specifies the maximum number of pictures that can precede any picture in the coded video sequence in output order and follow that picture in decoding order for the bitstream subset as specified in subclause Error: Reference source not found with i as input. When max_latency_increase[ i ] is equal to 0, no corresponding limit is expressed. The value of max_latency_increase[ i ] shall be in the range of 0 to 232 − 1, inclusive.

num_hrd_parameters specifies the number of hrd_parameters( ) syntax structures present in the video parameter set RBSP.

vps_extension_flag equal to 0 specifies that no vps_extension_data_flag syntax elements are present in the video parameter set RBSP syntax structure. vps_extension_flag shall be equal to 0 in bitstreams conforming to this Recommendation | International Standard. The value of 1 for vps_extension_flag is reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all data that follow the value 1 for vps_extension_flag in a video parameter set NAL unit.

vps_extension_data_flag may have any value. It shall not affect the conformance to profiles specified in this Recommendation | International Standard.

## Profile and level syntax and semantics

| profile_level( ProfilePresentFlag, MaxNumSubLayersMinus1 ) { |  |
| --- | --- |
| if( ProfilePresentFlag ) { |  |
| profile_space | u(3) |
| profile_idc | u(5) |
| for( i = 0; i < 32; i++ ) |  |
| profile_compatability_flag[ i ] | u(1) |
| constraint_flags | u(16) |
| } |  |
| level_idc | u(8) |
| for( i = 0; i < MaxNumSubLayersMinus1; i++ ) { |  |
| sub_layer_profile_present_flag[ i ] | u(1) |
| sub_layer_level_present_flag[ i ] | u(1) |
| if( ProfilePresentFlag && sub_layer_profile_present_flag[ i ] ) { |  |
| sub_layer_profile_space[ i ] | u(3) |
| sub_layer_profile_idc[ i ] | u(5) |
| for( j = 0; j < 32; j++ ) |  |
| sub_layer_profile_compatability_flag[ i ][ j ] | u(1) |
| sub_layer_constraint_flags[ i ] | u(16) |
| } |  |
| if( sub_layer_level_present_flag[ i ] ) |  |
| sub_layer_level_idc[ i ] | u(8) |
| } |  |
| } |  |

profile_space specifies the context for the interpretation of profile_idc and profile_combatibility_flag[ i ] for all possible values of i. The value of profile_space shall be equal to 0 in bitstreams conforming to this Recommendation | International Standard. Other values for profile_space are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the coded video sequence if profile_space is not equal to 0.

When profile_space is equal to 0, profile_idc indicates the profile to which the coded video sequence conforms.

[Ed. (DS): We might prefer not to use the profile_idc value zero, or reserve it to mean "no profile signalled, bitstream is unconstrained"; this gives us one spare bit in the profile compatibility flags array.]

When profile_space is equal to 0, profile_compatability_flag[ i ] equal to 1 indicates that the coded video sequence conforms to the profile indicated by profile_idc equal to i. When profile_space is equal to 0, profile_compatability_flag[ profile_idc ] shall be equal to 1.

constraint_flags shall be equal to 0 in bitstreams conforming to this Recommendation | International Standard. Other values for for constraint_flags are reserved for future use by ITU-T | ISO/IEC. Decoders should ignore the value of constraint_flags.

level_idc indicates the maximum level to which the coded video sequence conforms when the profile_level( ) syntax structure is included in a video parameter set. level_idc indicates the level to which the coded video sequence conforms when the profile_level( ) syntax structure is included in a sequence parameter set.

NOTE 1 – A greater value of level_idc indicates a higher level. The maximum level signalled in the video parameter set for a coded video sequence may be higher than the level signalled in the sequence parameter set for the same coded video sequence.

NOTE 2 – The level_idc definitions need to be independent of profile_idc, as a decoder may not recognize the the profile_idc value but instead recognize, and be able to decode, one or more of the profiles indicated by the profile compatibility flags.

NOTE 3 – For bitstreams compatible with more than one profile, the value of profile_idc should be set to the "best viewed as" profile.

NOTE 4 – The constraint_flags can be used to indicate the respect of further constraints on the bitstream (e.g. that a selected tool, permitted by the profiles signalled, is nonetheless not used). These flags are ideally profile independent, but unlike level_idc, it is allowed to be contextual on profile_idc.

sub_layer_profile_present_flag[ i ] equal to 1 specifies that profile information may be present in the profile_level( ) syntax structure for the representation of the sub-layer with TemporalId equal to i. sub_layer_profile_present_flag[ i ] equal to 0 specifies that profile information is not present in the profile_level( ) syntax structure for the representations of the sub-layer with TemporalId equal to i.

[Ed.(YK): Add a definition of (temporal) sub-layer representation or representation of (temporal) sub-layer in Clause 3 to consist of all NAL units with TemporalId not greater than the TemporalId of the sub-layer.]

sub_layer_level_present_flag[ i ] equal to 1 specifies that level information is present in the profile_level( ) syntax structure for the representation of the sub-layer with TemporalId equal to i. sub_layer_level_present_flag[ i ] equal to 0 specifies that level information is not present in the profile_level( ) syntax structure for the representation of the sub-layer with TemporalId equal to i.

sub_layer_profile_space[ i ], sub_layer_profile_idc[ i ] , sub_layer_profile_compatability_flag[ i ][ j ] , sub_layer_constraint_flags[ i ] , and sub_layer_level_idc[ i ] have the semantics of profile_space, profile_idc, profile_compatability_flag[ j ], constraint_flags, and level_idc, respectively, but apply to the representation of the sub-layer with TemporalId equal to i.

## Operating point syntax and semantics

| op_point( opIdx ) { | Descriptor |
| --- | --- |
| op_num_layer_id_values_minus1[ opIdx ] | ue(v) |
| for( i = 0; i <= op_num_layer_id_values_minus1; i++ ) |  |
| op_layer_id[ opIdx ][ i ] | u(6) |
| } |  |

op_num_layer_id_values_minus1[ opIdx ] specifies a number of reserved_zero_6bits values included in the operating point identified by opIdx.

op_num_layer_id_values_minus1[ 0 ] is inferred to be equal to 0.

op_layer_id[ opIdx ][ i ] specifies a reserved_zero_6bits value included in the operating point identified by opIdx. No value of op_layer_id[ opIdx ][ i ] shall be equal to op_layer_id[ opIdx ][ j ] when i is not equal to j and both i and j are in the range of 0 to op_num_layer_id_values_minus1, inclusive.

op_layer_id[ 0 ][ 0 ] is inferred to be equal to 0.

# SPS syntax and semantics

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| video_parameter_set_id | u(4) |
| sps_max_sub_layers_minus1 | u(3) |
| reserved_zero_bit | u(1) |
| profile_level( 0, sps_max_sub_layers_minus1 ) |  |
| profile_space | u(3) |
| profile_idc | u(5) |
| constraint_flags | u(16) |
| level_idc | u(8) |
| for( i = 0; i < 32; i++ ) |  |
| profile_compatability_flag[ i ] | u(1) |
| seq_parameter_set_id | ue(v) |
| video_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| sps_max_temporal_layers_minus1 | u(3) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| pic_cropping_flag | u(1) |
| if( pic_cropping_flag ) { |  |
| pic_crop_left_offset | ue(v) |
| pic_crop_right_offset | ue(v) |
| pic_crop_top_offset | ue(v) |
| pic_crop_bottom_offset | ue(v) |
| } |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| [Ed. (BB): chroma bit depth present in HM software but not used further ] |  |
| pcm_enabled_flag | u(1) |
| if( pcm_enabled_flag ) { |  |
| pcm_sample_bit_depth_luma_minus1 | u(4) |
| pcm_sample_bit_depth_chroma_minus1 | u(4) |
| } |  |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| for( i = 0; i <= sps_max_temporal_layers_minus1; i++ ) { |  |
| sps_max_dec_pic_buffering[ i ] | ue(v) |
| sps_num_reorder_pics[ i ] | ue(v) |
| sps_max_latency_increase[ i ] | ue(v) |
| } |  |
| restricted_ref_pic_lists_flag | u(1) |
| if( restricted_ref_pic_lists_flag ) |  |
| lists_modification_present_flag | u(1) |
| log2_min_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_coding_block_size | ue(v) |
| log2_min_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_transform_block_size | ue(v) |
| if( pcm_enabled_flag ) { |  |
| log2_min_pcm_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_pcm_coding_block_size | ue(v) |
| } |  |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| scaling_list_enable_flag | u(1) |
| if( scaling_list_enable_flag ) { |  |
| sps_scaling_list_data_present_flag | u(1) |
| if( sps_scaling_list_data_present_flag ) |  |
| scaling_list_param( ) |  |
| } |  |
| chroma_pred_from_luma_enabled_flag | u(1) |
| transform_skip_enabled_flag | u(1) |
| seq_loop_filter_across_slices_enabled_flag | u(1) |
| asymmetric_motion_partitions_enabled_flag | u(1) |
| nsrqt_enabled_flag | u(1) |
| sample_adaptive_offset_enabled_flag | u(1) |
| adaptive_loop_filter_enabled_flag | u(1) |
| if( pcm_enabled_flag ) |  |
| pcm_loop_filter_disable_flag | u(1) |
| sps_temporal_id_nesting_flag | u(1) |
| [Ed. (BB): x y padding syntax missing here, present in HM software ] |  |
| num_short_term_ref_pic_sets | ue(v) |
| for( i = 0; i < num_short_term_ref_pic_sets; i++) |  |
| short_term_ref_pic_set( i ) |  |
| long_term_ref_pics_present_flag | u(1) |
| sps_temporal_mvp_enable_flag | u(1) |
| vui_parameters_present_flag | u(1) |
| if( vui_parameters_present_flag ) |  |
| vui_parameters( ) |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

video_parameter_set_id identifies a video parameter set. The value of video_parameter_set_id shall be in the range of 0 to 15, inclusive.

sps_max_sub_layers_minus1 plus 1 specifies the maximum number of temporal sub-layers that may be present in the bitstream. The value of sps_max_sub_layers_minus1 shall be in the range of 0 to 6, inclusive.

reserved_zero_bit shall be equal to 0 in bitstreams conforming to this Recommendation | International Standard. The value 1 for reserved_zero_bit is reserved for future use by ITU-T | ISO/IEC. Decoders should ignore values of reserved_zero_bit not equal to 0 in a video parameter set NAL unit.

Semantics of the removed syntax elements are removed, and semantics of remaining syntax elements remain unchanged.

# VUI and HRD parameters syntax and semantics

See document JCTVC-J0548.

## HRD parameters syntax

| hrd_parameters( ) { | Descriptor |
| --- | --- |
| cpb_cnt_minus1 | ue(v) |
| bit_rate_scale | u(4) |
| cpb_size_scale | u(4) |
| for( SchedSelIdx = 0; SchedSelIdx <= cpb_cnt_minus1; SchedSelIdx++ ) { |  |
| bit_rate_value_minus1[ SchedSelIdx ] | ue(v) |
| cpb_size_value_minus1[ SchedSelIdx ] | ue(v) |
| cbr_flag[ SchedSelIdx ] | u(1) |
| } |  |
| initial_cpb_removal_delay_length_minus1 | u(5) |
| cpb_removal_delay_length_minus1 | u(5) |
| dpb_output_delay_length_minus1 | u(5) |
| time_offset_length | u(5) |
| } |  |

# Decoding process

## General [Ed. (GJS): Create similar subclauses for other places where there is "orphaned" text that is at the parent level of a section that contains subclauses.]

Outputs of this process are decoded samples of the current picture (sometimes referred to by the variable CurrPic).

Depending on the value of chroma_format_idc, the number of sample arrays of the current picture is as follows.

–	If chroma_format_idc is equal to 0, the current picture consists of 1 sample array SL.

–	Otherwise (chroma_format_idc is not equal to 0), the current picture consists of 3 sample arrays SL, SCb, SCr.

This clause describes the decoding process, given syntax elements and upper-case variables from clause Error: Reference source not found.

The decoding process is specified such that all decoders shall produce numerically identical results. Any decoding process that produces identical results to the process described here conforms to the decoding process requirements of this Recommendation | International Standard.

The variable HighestTemporalId that identifies the highest sub-layer is specified by external means not specified in this Specification, or, when not specified by external means, HighestTemporalId is set to vps_max_sub_layers_minus1.

Each picture referred to in this clause is a complete coded picture.

Depending on the value of separate_colour_plane_flag, the decoding process is structured as follows.

–	If separate_colour_plane_flag is equal to 0, the decoding process is invoked a single time with the current picture being the output.

–	Otherwise (separate_colour_plane_flag is equal to 1), the decoding process is invoked three times. Inputs to the decoding process are all NAL units of the coded picture with identical value of colour_plane_id. The decoding process of NAL units with a particular value of colour_plane_id is specified as if only a coded video sequence with monochrome colour format with that particular value of colour_plane_id would be present in the bitstream. The output of each of the three decoding processes is assigned to the 3 sample arrays of the current picture with the NAL units with colour_plane_id equal to 0 being assigned to SL, the NAL units with colour_plane_id equal to 1 being assigned to SCb, and the NAL units with colour_plane_id equal to 2 being assigned to SCr.

NOTE – The variable ChromaArrayType is derived as 0 when separate_colour_plane_flag is equal to 1 and chroma_format_idc is equal to 3. In the decoding process, the value of this variable is evaluated resulting in operations identical to that of monochrome pictures with chroma_format_idc being equal to 0.

The decoding process operates as follows:

- The decoding of NAL units is specified in subclause Error: Reference source not found.

- The processes in subclause Error: Reference source not found specify decoding processes using syntax elements in the slice layer and above:

–	Variables and functions relating to picture order count are derived in subclause Error: Reference source not found (only needed to be invoked for one slice of a picture).

–	The decoding process for reference picture set in subclause Error: Reference source not found is invoked, wherein reference pictures may be marked as "unused for reference" (only needed to be invoked for one slice of a picture).

–	PicOutputFlag is set as follows:

–	If the first coded picture in the bitstream is a CRA picture, and the current picture is a TFD picture associated with the CRA picture, or if the previous RAP picture preceding the current picture in decoding order is a BLA picture and the current picture is a TFD picture associated with the BLA picture, PicOutputFlag is set equal to 0 and the decoding process for generating unavailable reference pictures specified in subclause Error: Reference source not found is invoked (only needed to be invoked for one slice of a picture).

–	Otherwise, PicOutputFlag is set equal to pic_output_flag.

–	At the beginning of the decoding process for each P or B slice, the decoding process for reference picture lists construction specified in subclause Error: Reference source not found is invoked for derivation of reference picture list 0 (RefPicList0), and when decoding a B slice, reference picture list 1 (RefPicList1).

–	After all slices of the current picture have been decoded, if it is a reference picture the decoded reference picture is marked as "used for short-term reference", otherwise (it is a non-reference picture) the decoded picture is marked as "unused for reference", as specified in subclause Error: Reference source not found.

- The processes in subclauses Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, and Error: Reference source not found specify decoding processes using syntax elements in the coding tree unit layer and above.

- C.1	General

…

Two types of HRD parameter sets (NAL HRD parameters and VCL HRD parameters) are used. The HRD parameter sets are signalled through video usability information as specified in subclauses Error: Reference source not found and Error: Reference source not found, which is part of the sequence parameter set syntax structure.

The HRD parameters sets for bitstream conformance are selected as follows: The variable layerIdList is a list of values for reserved_zero_6bits, which are present in the bitstream. The hrd_parameters( ) structure for which the set of values specified by op_layer_id[ opIdx ][ i ] for i in the range of 0 to op_num_layer_id_values_minus1[ opIdx ], inclusive, is identical to layerIdList is used for bitstream conformance. Within that hrd_parameters( ) structure, the hrd_parameters_sub_layer( HighestTemporalId ) structure or structures are used for bitstream conformance.

…

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Nokia Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).