| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, Sweden, 11– 20 July 2012 | Document: JCTVC-J0563r2 |
| --- | --- |

| Title: | Additional test data for SAO contributions considered for adoption |  |  |
| --- | --- | --- | --- |
| Status: | Report |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | E. Alshina #416 Matean 3-dong, Yeongtong-gu, Suwon-si, Gyeonggi-do, 443-742, Korea | Tel: Email: | +82 10 3026 1305 elena_a.alshina@samsung.com |
| Source: | Samsung Electronics Ltd. |  |  |

_____________________________

# Abstract

This is report about performance analysis of joint integration for adoption candidates: JCTVC-J0041, 0043, 0046, 0268, 0045, 0355, 0044, 0139 identified during SAO discussion in JCTVC 10th .

# Introduction

Following contributions were recommended for adoption during discussion of SAO in JCTVC 10th meeting.

Simplifications

  - JCTVC-J0041

    - # ctxs reduction for SAO merge syntax

  - JCTVC-J0043

    - By-pass coding for SAO magnitudes

  - JCTVC-J0046==J0054

    - Removal of condition check in left band position coding

  - JCTVC-J0268

    - SAO type coding with only 1 ctx bin

Coding efficiency

  - JCTVC-J0355

    - Single merge flags for 3 color components

Implementation friendly design

  - JCTVC-J0045

    - SAO type is shared between Cb&Cr

Encoder search improvements

  - JCTVC-Joo44

    - Extension of JCTVC-I0184

  - JCTVC-J0139

    - Use non-de-blocked pixels for SAO parameter s estimation

# Simplifications

The main issue is addressed in contribution in “simplifications” category us reduction of context coded bins.

It was requested to study performance of changes suggested in these 4 contributions implemented jointly. Sony, MediaTek and Samsung worked on this code and test results provided by Sony and MediaTek. They run independent simulation and their results match.

Full size simulation is finished by time of presenting 2nd revision this document. Performance test is summarized in Table 1.

Table 1. Performance of joint implementation of JCTVC-J0041, 0043, 0046 and 0268 vs HM7.0.

| 4 together | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0,1% | 0,0% | -0,1% | 0,1% | 0,0% | -0,1% |
| Class B | 0,1% | -0,1% | -0,2% | 0,1% | -0,1% | -0,2% |
| Class C | 0,1% | -0,2% | -0,3% | 0,1% | -0,1% | -0,1% |
| Class D | 0,1% | -0,4% | -0,4% | 0,1% | -0,2% | -0,2% |
| Class E | 0,2% | -0,3% | -0,3% | 0,2% | -0,2% | -0,4% |
| Overall | 0,1% | -0,2% | -0,3% | 0,1% | -0,1% | -0,2% |
| Class F | 0,1% | -0,3% | -0,3% | 0,1% | -0,1% | -0,1% |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0,2% | -0,5% | -0,5% | 0,1% | -0,1% | -0,2% |
| Class B | 0,0% | -0,2% | -0,5% | 0,0% | -0,3% | -0,4% |
| Class C | -0,1% | -0,3% | -0,4% | 0,0% | -0,2% | -0,2% |
| Class D | -0,2% | -0,6% | -0,7% | -0,2% | -0,3% | -0,4% |
| Class E |  |  |  |  |  |  |
| Overall | 0,0% | -0,4% | -0,5% | 0,0% | -0,2% | -0,3% |
| Class F | 0,0% | -0,3% | -0,4% | 0,0% | -0,2% | -0,3% |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0,0% | -0,1% | -0,5% | 0,0% | -0,2% | -0,3% |
| Class C | -0,1% | -0,4% | -0,4% | 0,0% | -0,1% | 0,1% |
| Class D | -0,2% | -0,2% | -0,1% | 0,0% | 0,1% | 0,1% |
| Class E | -0,3% | -0,4% | -0,8% | 0,0% | -0,5% | -0,8% |
| Overall | -0,1% | -0,3% | -0,4% | 0,0% | -0,2% | -0,2% |
| Class F | -0,2% | -0,8% | -0,5% | 0,1% | 0,0% | 0,8% |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0,2% | -0,1% | -0,6% | 0,0% | -0,3% | -0,6% |
| Class C | 0,0% | -0,6% | -0,4% | 0,1% | -0,2% | 0,0% |
| Class D | -0,1% | -0,3% | -0,6% | -0,1% | -0,3% | -0,9% |
| Class E | 0,1% | -0,1% | -0,3% | -0,1% | 0,6% | -0,6% |
| Overall | 0,0% | -0,3% | -0,5% | 0,0% | -0,1% | -0,5% |
| Class F | -0,4% | -1,2% | -2,1% | 0,1% | 0,0% | 0,8% |
| Avg. | 0,0% | -0,2% | -0,4% |  |  |  |

Jointly these contributions reduce 99% of SAO ctx coded bins in the worst case (HE10 test). For MAIN test case the context coded bins reduction is 95% (worst case).

Additionally 6 context models are removed from HEVC design by suggested simplification.

# Performance improvement and implementation friendly modifications.

JCTVC-J0045 proposes to share SAO type between Cb and Cr component in order to simplify memory access during SAO process and harmonize SAO design with other HEVC part (intra prediction direction, MV, interpolation filter, ALF shape are shared between 2 Chroma components).

It was requested to test J0045 on top of J0355 which forces to merge flags to be shared between 3 color components.

Joint integration was done by Canon, Motorola and Samsung. Results of full frame simulation are presented in Table 2.

Table 2. Joint implementation of JCTVC-J0355 and J0045 vs HM7.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0,0% | 0,3% | 0,2% | 0,0% | 0,2% | 0,5% |
| Class B | 0,0% | 0,2% | 0,3% | 0,0% | 0,2% | 0,4% |
| Class C | 0,0% | 0,2% | 0,2% | 0,0% | 0,2% | 0,4% |
| Class D | 0,0% | 0,0% | 0,1% | 0,0% | 0,0% | 0,1% |
| Class E | -0,1% | 0,0% | 0,0% | -0,1% | 0,1% | 0,2% |
| Overall | 0,0% | 0,1% | 0,2% | 0,0% | 0,2% | 0,3% |
| Class F | 0,0% | 0,3% | 0,3% | 0,0% | 0,3% | 0,4% |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0,2% | 0,3% | -0,2% | -0,1% | 0,2% | 0,4% |
| Class B | -0,2% | 0,3% | -0,1% | -0,2% | 0,3% | 0,3% |
| Class C | -0,2% | 0,1% | 0,1% | -0,2% | 0,2% | 0,2% |
| Class D | -0,4% | 0,1% | 0,1% | -0,4% | 0,0% | 0,0% |
| Class E |  |  |  |  |  |  |
| Overall | -0,3% | 0,2% | 0,0% | -0,2% | 0,2% | 0,2% |
| Class F | -0,2% | 0,1% | 0,3% | -0,2% | 0,1% | 0,2% |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0,1% | 0,0% | -0,5% | -0,2% | 0,1% | 0,3% |
| Class C | -0,2% | 0,1% | 0,0% | -0,2% | 0,2% | 0,3% |
| Class D | -0,4% | 0,4% | 0,5% | -0,3% | 0,4% | 0,3% |
| Class E | -0,3% | -0,6% | -2,1% | -0,3% | -0,4% | -0,3% |
| Overall | -0,2% | 0,0% | -0,4% | -0,2% | 0,1% | 0,2% |
| Class F | -0,5% | -0,8% | -0,4% | -0,3% | -0,3% | 0,1% |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0,6% | -0,3% | -1,3% | -0,3% | 0,2% | 0,0% |
| Class C | -0,3% | -0,2% | -0,5% | -0,2% | 0,1% | 0,3% |
| Class D | -0,4% | 0,4% | 0,6% | -0,3% | 0,0% | -0,1% |
| Class E | -0,7% | -2,1% | -2,6% | -0,7% | -0,1% | -0,3% |
| Overall | -0,5% | -0,5% | -0,9% | -0,3% | 0,1% | 0,0% |
| Class F | -0,9% | -2,1% | -2,5% | -0,4% | -0,3% | 0,1% |
| Avg. | -0,2% | 0,0% | -0,1% |  |  |  |

It is confirmed that together both contributions improve HEVC performance by 0.2% in term of Luma BD-rate without loss in Chroma.

It was requested to demonstrate test results for J0045 on top of J0355. They are reported in Table 3.

Table 3. Anchor is JCTVC-J0355, modification is JCTVC-J0355&JCTVC-J0045.

| J0355&0045 | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0,0% | 0,0% | 0,0% | 0,0% | 0,1% | 0,2% |
| Class B | 0,0% | 0,0% | 0,1% | 0,0% | 0,0% | 0,1% |
| Class C | 0,0% | 0,0% | 0,1% | 0,0% | 0,0% | 0,1% |
| Class D | 0,0% | -0,1% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class E | 0,0% | 0,0% | -0,1% | 0,0% | 0,1% | 0,1% |
| Overall | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,1% |
| Class F | 0,0% | 0,0% | 0,1% | 0,0% | -0,1% | 0,0% |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0,1% | 0,6% | 0,0% | 0,0% | -0,1% | 0,0% |
| Class B | 0,0% | 0,3% | 0,1% | 0,0% | 0,1% | 0,2% |
| Class C | -0,1% | 0,2% | 0,2% | 0,0% | 0,1% | 0,1% |
| Class D | 0,0% | 0,2% | 0,3% | -0,1% | 0,1% | 0,2% |
| Class E |  |  |  |  |  |  |
| Overall | -0,1% | 0,3% | 0,1% | 0,0% | 0,1% | 0,1% |
| Class F | -0,1% | 0,0% | 0,3% | 0,0% | 0,0% | 0,0% |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0,0% | 0,3% | 0,0% | 0,0% | 0,0% | 0,1% |
| Class C | -0,1% | 0,2% | 0,1% | -0,1% | 0,2% | 0,1% |
| Class D | -0,1% | 0,7% | -0,3% | 0,0% | -0,1% | 0,2% |
| Class E | 0,0% | 0,7% | -0,4% | 0,0% | 0,8% | 0,3% |
| Overall | -0,1% | 0,4% | -0,1% | -0,1% | 0,2% | 0,2% |
| Class F | -0,2% | -0,1% | -0,2% | -0,2% | -0,7% | -0,3% |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0,1% | 0,1% | -0,7% | -0,1% | 0,2% | 0,2% |
| Class C | -0,1% | 0,1% | -0,2% | -0,1% | 0,1% | 0,0% |
| Class D | -0,1% | 0,4% | 0,2% | 0,0% | 0,1% | 0,3% |
| Class E | -0,2% | 0,0% | -0,6% | -0,2% | 0,3% | -0,2% |
| Overall | -0,1% | 0,2% | -0,3% | -0,1% | 0,2% | 0,1% |
| Class F | -0,2% | -0,7% | -1,7% | -0,2% | -0,7% | -0,3% |
| Avg. | -0,1% | 0,2% | 0,0% |  |  |  |

This test confirms that sharing of SAO type between Cb and Cr additionally to sharing merge flags for all 3 color components doesn’t harm performance and improves Luma BD-rate 0.1%.

Additional simulation is running for LCU size 32x32 test. This test shows higher gain from JCTVC-J0355&JCTVC-J0045 (as it was expected).

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0,0% | 0,0% | -0,1% | -0,1% | 0,0% | 0,0% |
| Class B | -0,1% | -0,1% | -0,1% | -0,1% | 0,0% | 0,0% |
| Class C | -0,1% | 0,1% | 0,1% | 0,0% | 0,0% | 0,0% |
| Class D | -0,1% | -0,1% | -0,1% | -0,1% | 0,0% | 0,0% |
| Class E | -0,1% | -0,1% | -0,1% | -0,1% | -0,1% | -0,1% |
| Overall | -0,1% | 0,0% | 0,0% | -0,1% | 0,0% | 0,0% |
| Class F | -0,1% | 0,1% | 0,3% | -0,1% | 0,0% | 0,1% |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0,2% | 0,2% | -0,1% | -0,2% | -0,1% | -0,1% |
| Class B | -0,2% | 0,0% | -0,4% | -0,2% | -0,1% | -0,1% |
| Class C | -0,2% | 0,0% | 0,0% | -0,2% | 0,0% | 0,1% |
| Class D | -0,2% | -0,1% | -0,1% | -0,2% | -0,2% | -0,2% |
| Class E |  |  |  |  |  |  |
| Overall | -0,2% | 0,0% | -0,2% | -0,2% | -0,1% | -0,1% |
| Class F | -0,4% | -0,1% | 0,1% | -0,3% | -0,1% | 0,1% |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0,3% | -0,1% | -0,4% | -0,3% | -0,2% | 0,1% |
| Class C | -0,2% | 0,1% | -0,1% | -0,2% | 0,0% | -0,2% |
| Class D | -0,2% | 0,4% | -0,1% | -0,2% | -0,2% | -0,5% |
| Class E | -1,0% | -1,7% | -1,9% | -1,1% | -1,1% | -0,8% |
| Overall | -0,4% | -0,2% | -0,5% | -0,4% | -0,3% | -0,3% |
| Class F | -0,6% | -0,5% | -0,7% | -0,7% | 0,5% | 0,0% |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0,8% | -0,3% | -0,3% | -0,4% | -0,3% | -0,6% |
| Class C | -0,5% | 0,0% | -0,2% | -0,4% | 0,0% | -0,1% |
| Class D | -0,4% | -0,2% | -0,9% | -0,3% | 0,1% | -0,1% |
| Class E | -1,3% | -1,8% | -2,5% | -1,0% | -0,9% | -0,1% |
| Overall | -0,7% | -0,5% | -0,8% | -0,5% | -0,2% | -0,2% |
| Class F | -0,7% | -0,4% | -0,2% | -0,9% | -0,1% | -1,0% |
| Avg. | -0,3% | -0,2% | -0,3% |  |  |  |

Canon and Motorola provided these test data.

# Encoder improvements.

JCTVC-J0044 and JCTVC-J0139 modify SAO encoder search process and so improve performance. In order to check possible interaction they were implemented jointly and tested by TI.

Results are summarized in Table 4.

Table 4. Joint implementation of encoder modification.

JCTVC-J0139 vs HM7.0:

|  | Main |  |  | HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| AI | 0.0% | -0.5% | -0.6% | 0.0% | -0.2% | -0.3% |
| RA | 0.0% | -0.5% | -0.6% | 0.0% | -0.3% | -0.4% |
| LDB | -0.1% | -0.9% | -1.3% | -0.1% | -0.5% | -0.7% |
| LDP | -0.3% | -1.2% | -1.2% | -0.1% | -0.5% | -0.9% |
| Ave | -0.1% | -0.8% | -0.9% | -0.1% | -0.4% | -0.6% |

JCTVC-J0044 vs HM7.0:

|  | Main |  |  | HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| AI | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RA | -0.3% | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% |
| LDB | -0.1% | 0.0% | 0.1% | -0.1% | -0.1% | 0.1% |
| LDP | -0.3% | 0.1% | 0.1% | -0.1% | 0.1% | -0.3% |
| Ave | -0.2% | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% |

JCTVC-J0044+J0139 vs HM7.0:

|  | Main |  |  | HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| AI | 0.0% | -0.5% | -0.6% | 0.0% | -0.2% | -0.3% |
| RA | -0.4% | -0.6% | -0.8% | -0.2% | -0.4% | -0.5% |
| LDB | -0.2% | -0.9% | -1.2% | -0.2% | -0.4% | -0.5% |
| LDP | -0.6% | -1.2% | -1.3% | -0.3% | -0.6% | -0.8% |
| Ave | -0.3% | -0.8% | -1.0% | -0.2% | -0.4% | -0.5% |

These test confirm that HEVC performance can be improved by 0.3% in terms of Luma BD-rate with encoder only modification.

# Specification text

Specification text for all tested contributions was created, sent to HEVC editors and attached to this contribution. Additionally SAO related changes from adopted JCTV-J0087 and JCTVC-J0544 are integrated together.

# Conclusions

With suggested for adoption SAO simplifications the number of SAO context coded bins in the worst case is reduced 99%. Number of context models used in SAO is 2 with suggested modification (instead 8 in HM7.0). Performance impact in term of BD-rate is

0.0% (Y), -0.2% (U), -0.4% (V) (no change for Luma and gain for Chroma).

Sharing of SAO type between Cb and Cr provides 0.1% Luma BD-rate improvement on top of sharing SAO merge flags between 3 color components (JCTVC-J0355). Jointly these changes improve HEVC performance by 0.2% in terms of Luma BD-rate.

Encoder modifications improve HEVC performance by

0.3% (Luma) and 0.7% (Chroma).