| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11 –20 July 2012 | Document: JCTVC-J0565 |
| --- | --- |

| Title: | AHG6: Report of viewing results for comparison between Main LDB and Main LDP with ALF studied in JCTVC-J0049 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Tomoo Yamakage | Tel: Email: | tomoo.yamakage@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

Results for an informal subjective viewing for comparison between Main LDB without ALF and Main LDP with ALF are reported. Evaluated method of ALF is JCTVC-J0390 Software package 2.  This report is also the subjective picture quality inspection of JCTVC-J0049.

All Class E sequences with Qp = 32 and 37 are used for evaluation.  One out of six test points showed that Main LDB (without ALF) is better than Main LDP with ALF.  Another test point showed that Main LDP with ALF is better than Main LDB (without ALF).  Remaining four test points showed that there is no difference in terms of picture quality.

# Viewing conditions

Viewing conditions are as follows:

- 14 July 2012 at room 359, 08:30-09:55

- Up to three people per session

- Monitor: Samsung LE46B65

- PC: Intel i7 2.67GHz, raid of 4 hard drives 8TB total and 8GB RAM, running on Windows 7Some monster PC (Windows 7 – 64 bit)

- Qp = 32, 37

- Main LDB (without ALF) and Main LDP with ALF are compared

- A-B-vote method is used with 5-grade scale as follows:

B: A is worse than B  (A is displayed first, and B followed.)

b: A is slightly worse than B  (A is displayed first, and B followed.)

0: Equal

a: A is slightly better than B  (A is displayed first, and B followed.)

A: A is better than B  (A is displayed first, and B followed.)

Note: “A” is presented first, and “B” is presented second.

- 23 viewers

- Viewing order is randomized

# Viewing results

The following table shows the file size of each bitstream:

Two out of six test points exceeded 2% tolerance for Main LDP with ALF.

The following graph shows the results for 23 viewers with 95% confidence intervals:

LDP with ALF is better

LDB without ALF is better

One out of six test points (FourPeople, Qp=37) showed that Main LDB (without ALF) is better than Main LDP with ALF.  Another test point (Johnny, Qp=32) showed that Main LDP with ALF is better than Main LDB (without ALF).  Remaining four test points showed that there is no difference in terms of picture quality.  In conclusion, those two coding configuration provides similar subjective picture quality.

Since this comparison is discussed in JCTVC-J0049, it is suggested to review the complexity and the objective gain trade-off presented in JCTVC-J0049.

# Patent rights declaration(s)

TOSHIBA Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).