| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0567 |
| --- | --- |

| Title: | BoG Report: Report of subjective test on extended adaptability range of deblocking filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Teruhiko Suzuki 2-10-1, Osaki, Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2740 teruhikos@jp.sony.com |
| Source: | BoG on subjective test on extended adaptability range of deblocking |  |  |

_____________________________

# Introduction

This contribution reports the results of subjective test conducted at Stockholm meeting to compare 4 proposals, JCTVC-J0090, JCTVC-J0096, JCTVC-J0181 and JCTVC-J0286.

# Proposals to be tested

JCTVC-J0090 AHG6: Transform Dependent Deblocking [G. Van der Auwera, R. Joshi, M. Karczewicz (Qualcomm)]

Tested method signals specific deblocking adjustment parameters to control the deblocking strength in case that at least one of two adjacent video blocks P and Q is included in a TU of maximum size. The strongest β threshold curve (beta-prop-d16) is used with increased β values outside of the QP range of the common test conditions (>41), but within the QP range of the current HM7 curve.

JCTVC-J0096 Suppression of blocking artifacts at 32x32 transform boundaries [D.-K. Kwon, M. Budagavi (TI)]

Algorithm 1 in JCTVC-J0096 was tested. In addition to beta_offset_div2 and tc_offset_div2, it is proposed to signal tu32_beta_offset_div2 and tu32_tc_offset_div2 in PPS or slice header. It is also proposed that these new syntax elements are applied only to 32x32 transform boundaries while the existing syntax elements beta_offset_div2 and tc_offset_div2 are applied to other block boundaries than 32x32 transform boundaries.

JCTVC-J0181 AHG6: On deblocking filter parameters [S. Lu, O. Nakagami, M. Ikeda, T. Suzuki (Sony)]

Proposal 1 in JCTVC-J0181 was tested. This proposed to replace the β values with Table2-2 as in JCTVC-J0181. The changes are highlighted in red. The value is determined by the principle that the changing rate of β should have similar trend with the changing rate of tc to reflect quantization change. The β-QP relation with proposed β is shown in Figure 2-1 in JCTVC-J0181.

Since the changes lie at QP bigger than 41, there is no impact on common test conditions.

JCTVC-J0286 AHG6: Adaptive deblocking filtering [A. Norkin (Ericsson)]

Algorithm 1 from J0286 was tested. The algorithm includes limiting intra-boundary smoothing for DC, vertical and horizontal intra-modes to block sizes below 32x32, modification of strong deblocking filter and its decisions, and increasing Bs value on 32x32 intra blocks boundaries from 2 to 3.

No deblocking parameters are sent, deblocking is operating in the default mode on all test sequences.

Table 1 Summary of tested proposals

|  |  | Anchor | JCTVC-J0090 | JCTVC-J0096 | JCTVC-J0181 | JCTVC-J0286 |
| --- | --- | --- | --- | --- | --- | --- |
| Deblocking changes | Beta curve change | No | Yes (beta-prop4-d16) | No | Yes (Prop1) | No |
|  | Strong filter decision change | No | Yes (Max TU) | Yes (32x32) | No | Yes |
|  | Strong filter change | No | No | No | No | Yes |
|  | Boundary strength change | No | No | No | No | Yes (64x64 and 32x32 CU) |
|  | New syntax elements (deblock parameters) | No | LoopFilterOffset MaxTUEnabled LoopFilterBetaOffset MaxTU LoopFilterTcOffset MaxTUDiv2 | tu32_beta_offset_div2 tu32_tc_offset_div2 | No | No |
| Intra coding changes | Intra prediction change | No | No | No | No | Yes (limit intra boundary smoothing for DC/ vertical/horizontal) |

# Subjective test

## Test conditions

Test sequences: Riverbed, DucksTakeOff, WestWindEasy, ChinaSpeed

QP: 32, 37

Anchor: LD-B Main + LoopFilterBetaOffset_div2 = 13 and LoopFilterTcOffset_div2 = 13

Test scenario: Low delay B Main

Add HM-7.0 common test condition as one proposal

All test sequences, proposals(including HM-7.0 common test condition), anchors are randomized.

Table 2 offset parameters used in the test

|  | Anchor | HM-7.0 | JCTVC-J0090 | JCTVC-J0096 | JCTVC-J0181 | JCTVC-J0286 |
| --- | --- | --- | --- | --- | --- | --- |
| DeblockingFilterControlPresent | 1 | 0 | 1 | 1 | 1 | 0 |
| LoopFilterBetaOffset_div2 | 13 | 0 | 0 | 0 | 13 | - |
| LoopFilterTcOffset_div2 | 13 | 0 | 0 | 0 | 13 | - |
| tu32_beta_offset_div2 | - | - | - | 13 | - | - |
| tu32_tc_offset_div2 | - | - | - | 13 | - | - |
| LoopFilterOffsetMaxTUEnabled | - | - | 1 | - | - | - |
| LoopFilterBetaOffsetMaxTU | - | - | 26 | - | - | - |
| LoopFilterTcOffsetMaxTUDiv2 | - | - | 13 | - | - | - |

## Test Method

As suggested by JCTVC main meeting, A&B preference test was selected.

A&B Preference TEST

The Test Cell was done of:

- The message “A” on the screen for 1 second (white letter on black background or, better, black letters on mid gray background);

- Play the Anchor video clip for a given QP and coding condition;

- The message “B” on the screen for 1 second (white letter on black background or, better, black letters on mid gray background);

- Play the Test video clip for the same QP and coding condition;

- The message “A” on the screen for 1 second (white letter on black background or, better, black letters on mid gray background);

- Play the Anchor video clip for a given QP and coding condition;

- The message “B” on the screen for 1 second (white letter on black background or, better, black letters on mid gray background);

- Play the Test video clip for the same QP and coding condition;

- The message “VOTE” on the screen for 5 second (white letter on black background or, better, black letters on mid gray background);

A is reference, then vote the quality of B

The “VOTE” is (5 score, -2: A>>B -1:A>B 0:A=B +1:A<B +2:A<<B)

2 QP * (4 proposals + HM7 (CTC)) * 4 sequences = 40 tests

For each test sequences are displayed as shown in Figure 1. In the “Vote” period, each user votes either “A” or “B”. It takes 50 sec for each test. So total time for the test is:

50 sec * (40 tests) = 2000 sec = 33.3 min

In this test, “A” and “B” are either anchor or proposal. The order of tests is shuffled randomly to make fair comparisons.

Figure 1: 1 test case

Viewing sessions have been held at “Room 359” room on July 13, 2012. Totally 22 non-proponent viewers were participated to the viewing session. The BoG would like to thank experts who participated in viewing.

# Problems during test

During the third session of the subjective test, it was found by PSNR check that the sequence ChinaSpeed at QP32 of proposal J0181 was used instead of ChinaSpeedQP37 of the same proposal (i.e. the same sequence at QP32 was used for both QP32 and QP37). The sequence at QP37 was then replaced with the correct one and the test continued. Therefore, the results for the first three sessions of the subjective test for sequence ChinaSpeed37 for proposal J0181 were not used when calculating the average score and confidence intervals.

# Test Result

# Annex: Bitstream data

|  |  | Riverbed | DucksTakeOff | WestWindEasy | ChinaSpeed |
| --- | --- | --- | --- | --- | --- |
| QP32 | HM7 (CTC) | 5859.47 (+0.12%) | 18542.10 (-0.15%) | 3753.22 (0.33%) | 1334.09 (-2.1%) |
|  | Anchor | 5852.75 (0 %) | 18569.44 (0%) | 3740.90 (0%) | 1362.40 (0%) |
|  | JCTVC-J0090 | 5852.45 (-0.01%) | 18689.27 (+1.2%) | 3747.06 (+0.16%) | 1351.55 (-0.80%) |
|  | JCTVC-J0096 | 5854.84 (0.03%) | 18568.74 (-0.004%) | 3746.51 (+0.15%) | 1339.04 (-1.71%) |
|  | JCTVC-J0181 | 5850.69 (-0.04%) | 18686.73 (+0.63%) | 3750.51 (+0.26%) | 1424.84(+4.6%) |
|  | JCTVC-J0286 | 5861.332(0.15%) | 18541.3 (-0.15%) | 3751.095 (0.27% ) | 1333.655 ( -2.11%) |
| QP37 | HM7 (CTC) | 2855.32 (+0.14%) | 7588.07 (-0.03%) | 1672.06 (+0.36%) | 604.54 (-0.53%) |
|  | Anchor | 2851.39 (0%) | 7590.24 (0%) | 1666.02 (0%) | 607.76 (0%) |
|  | JCTVC-J0090 | 2852.32 (+0.03%) | 7614.43 (+0.32%)))) as calculated from the result of 500 frames. | 1667.93 (+0.11%) | 609.89 (+0.35%) |
|  | JCTVC-J0096 | 2853.23 (+0.06%) | 7591.43 (+0.02%) | 1672.31 (+0.38%) | 605.92 (-0.30%) |
|  | JCTVC-J0181 | 2846.61 (-0.17%) | 7605.85 (+0.21%) | 1663.60 (-0.15%) | 615.95(+1.3%) |
|  | JCTVC-J0286 | 2853.658 (0.08% ) | 7582.997(-0.10% ) | 1667.71 (0.10%) | 603.9451 ( -0.63% ) |

Table 3: Bitrate (kbps)

Note: Bitrate of ChinaSpeed was calculated from the result of 500 frames.