- Annex C

Hypothetical reference decoder

(This annex forms an integral part of this Recommendation | International Standard)

  - General

This annex specifies the hypothetical reference decoder (HRD) and its use to check bitstream and decoder conformance.

Two types of bitstreams are subject to HRD conformance checking for this Recommendation | International Standard. The first such type of bitstream, called Type I bitstream, is a NAL unit stream containing only the VCL NAL units and filler data NAL units for all access units in the bitstream. The second type of bitstream, called a Type II bitstream, contains, in addition to the VCL NAL units and filler data NAL units for all access units in the bitstream, at least one of the following:

–	additional non-VCL NAL units other than filler data NAL units,

–	all leading_zero_8bits, zero_byte, start_code_prefix_one_3bytes, and trailing_zero_8bits syntax elements that form a byte stream from the NAL unit stream (as specified in Annex Error: Reference source not found).

Figure C-1 shows the types of bitstream conformance points checked by the HRD.

Figure C-1 – Structure of byte streams and NAL unit streams for HRD conformance checks [Ed. (GJS): Fix figure numbering in annex(es)]

The syntax elements of non-VCL NAL units (or their default values for some of the syntax elements), required for the HRD, are specified in the semantic subclauses of clause Error: Reference source not found, Annexes B and Error: Reference source not found.

Two types of HRD parameter sets (NAL HRD parameters and VCL HRD parameters) are used. The HRD parameter sets are signalled through video usability information as specified in subclauses Error: Reference source not found and Error: Reference source not found, which is part of the sequence parameter set syntax structure.

All sequence parameter sets and picture parameter sets referred to in the VCL NAL units, and corresponding buffering period and picture timing SEI messages shall be conveyed to the HRD, in a timely manner, either in the bitstream (by non-VCL NAL units), or by other means not specified in this Recommendation | International Standard.

In Annexes A, B, and Error: Reference source not found, the specification for "presence" of non-VCL NAL units is also satisfied when those NAL units (or just some of them) are conveyed to decoders (or to the HRD) by other means not specified by this Recommendation | International Standard. For the purpose of counting bits, only the appropriate bits that are actually present in the bitstream are counted.

NOTE 3 – As an example, synchronization of a non-VCL NAL unit, conveyed by means other than presence in the bitstream, with the NAL units that are present in the bitstream, can be achieved by indicating two points in the bitstream, between which the non-VCL NAL unit would have been present in the bitstream, had the encoder decided to convey it in the bitstream.

When the content of a non-VCL NAL unit is conveyed for the application by some means other than presence within the bitstream, the representation of the content of the non-VCL NAL unit is not required to use the same syntax specified in this annex.

NOTE 4 – When HRD information is contained within the bitstream, it is possible to verify the conformance of a bitstream to the requirements of this subclause based solely on information contained in the bitstream. When the HRD information is not present in the bitstream, as is the case for all "stand-alone" Type I bitstreams, conformance can only be verified when the HRD data is supplied by some other means not specified in this Recommendation | International Standard.

The HRD contains a coded picture buffer (CPB), an instantaneous decoding process, a decoded picture buffer (DPB), and output cropping as shown in Figure C-2.

Figure C-2 – HRD buffer model

[Ed. (JS): Remove "fields" from figure. (DF) Rename "frames" to "pictures"? (GJS): Yes, and I believe our current model now always stores the current picture, so the bypass around the DPB should be removed. (YK): And, "Access units" -> "decoding units"; "Reference fields or frames" -> "reference pictures"; "Fields of frames" (first instance) -> "decoded decoding units"; "Fields of frames" (second instance) -> "decoded pictures"; "Output cropped fields of frames" -> "Output cropped pictures"]

The CPB size (number of bits) is CpbSize[ SchedSelIdx ]. The DPB size (number of picture storage buffers) for temporal layer X is sps_max_dec_pic_buffering[ X ] for each X in the range of 0 to sps_max_temporal_layers_minus1, inclusive. [Ed. (GJS): Think about using MaxDpbSize versus max_dec_pic_buffering (various places in document).]

The variable SubPicCpbPreferredFlag is either specified by external means, or when not specified by external means, set to 0.

The variable SubPicCpbFlag is derived as follows:

SubPicCpbFlag = SubPicCpbPreferredFlag  &&  sub_pic_cpb_params_present_flag	(C-1)

If SubPicCpbFlag is equal to 0, the CPB operates at access unit level and each decoding unit is an access unit. Otherwise the CPB operates at sub-picture level and each decoding unit is a subset of an access unit.

The HRD operates as follows. Data associated with decoding units that flow into the CPB according to a specified arrival schedule are delivered by the HSS. The data associated with each decoding unit are removed and decoded instantaneously by the instantaneous decoding process at CPB removal times. Each decoded picture is placed in the DPB. A decoded picture is removed from the DPB at the later of the DPB output time or the time that it becomes no longer needed for inter-prediction reference.

The operation of the CPB is specified in subclause A.2. The instantaneous decoder operation is specified in clauses Error: Reference source not found-Error: Reference source not found. The operation of the DPB is specified in subclause A.3. The output cropping is specified in subclause A.3.2.

HSS and HRD information concerning the number of enumerated delivery schedules and their associated bit rates and buffer sizes is specified in subclauses Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, and Error: Reference source not found. The HRD is initialised as specified by the buffering period SEI message as specified in subclauses B.1.1 and B.2.1. The removal timing of decoding units from the CPB and output timing of decoded pictures from the DPB are specified in the picture timing SEI message as specified in subclauses B.1.2 and B.2.1. All timing information relating to a specific decoding unit shall arrive prior to the CPB removal time of the decoding unit.

The HRD is used to check conformance of bitstreams and decoders as specified in subclauses A.4 and a, respectively.

NOTE 7 – While conformance is guaranteed under the assumption that all frame-rates and clocks used to generate the bitstream match exactly the values signalled in the bitstream, in a real system each of these may vary from the signalled or specified value.

All the arithmetic in this annex is done with real values, so that no rounding errors can propagate. For example, the number of bits in a CPB just prior to or after removal of a decoding unitis not necessarily an integer.

The variable tc is derived as follows and is called a clock tick:

tc = num_units_in_tick  time_scale		(C-1)

The variable tc_sub is derived as follows and is called a sub-picture clock tick:

tc_sub = num_units_in_sub_tick  time_scale		(C-1)

The following is specified for expressing the constraints in this annex:

–	Let access unit n be the n-th access unit in decoding order with the first access unit being access unit 0.

–	Let picture n be the coded picture or the decoded picture of access unit n.

–	Let decoding unit m be the m-th decoding unit in decoding order with the first decoding unit being decoding unit 0.

  - Operation of coded picture buffer (CPB)

The specifications in this subclause apply independently to each set of CPB parameters that is present and to both the Type I and Type II conformance points shown in Figure C-1.

    - Timing of bitstream arrival

The HRD may be initialised at any one of the buffering period SEI messages. Prior to initialisation, the CPB is empty.

NOTE – After initialisation, the HRD is not initialised again by subsequent buffering period SEI messages.

- Each access unit is referred to as access unit n, where the number n identifies the particular access unit. The access unit that is associated with the buffering period SEI message that initialises the CPB is referred to as access unit 0. The value of n is incremented by 1 for each subsequent access unit in decoding order.

- Each decoding unit is referred to as decoding unit m, where the number m identifies the particular decoding unit. The first decoding unit in decoding order in access unit 0 is referred to as decoding unit 0. The value of m is incremented by 1 for each subsequent decoding unit in decoding order.

The variables InitCpbRemovalDelay[ SchedSelIdx ] and InitCpbRemovalDelayOffset[ SchedSelIdx ] are set as follows.

–	If either of the following conditions is true:, InitCpbRemovalDelay[ SchedSelIdx ] and InitCpbRemovalDelayOffset[ SchedSelIdx ] are set to the values of the corresponding initial_alt_cpb_removal_delay[ SchedSelIdx ] and initial_alt_cpb_removal_delay_offset[ SchedSelIdx ], respectively, of the associated buffering period SEI message:

–	Access unit 0 is a CRA or BLA access unit for which the coded picture has nal_unit_type equal to 5 or 7, and the value of rap_cpb_params_present_flag of the associated buffering period SEI message is equal to 1;

–	SubPicCpbFlag is equal to 1.

Otherwise, InitCpbRemovalDelay[ SchedSelIdx ] and InitCpbRemovalDelayOffset[ SchedSelIdx ] are set to the values of the corresponding initial_cpb_removal_delay[ SchedSelIdx ] and initial_cpb_removal_delay_offset[ SchedSelIdx ], respectively, of the associated buffering period SEI message.

The time at which the first bit of decoding unit n begins to enter the CPB is referred to as the initial arrival time tai( m ).

The initial arrival time of decoding units is derived as follows.

–	If the decoding unit is decoding unit 0, tai( 0 ) = 0,

–	Otherwise (the decoding unit is decoding unit m with m > 0), the following applies.

–	If cbr_flag[ SchedSelIdx ] is equal to 1, the initial arrival time for decoding unit m, is equal to the final arrival time (which is derived below) of decoding unit m − 1, i.e.,

tai( m ) = taf( m − 1 )		(C-2)

–	Otherwise (cbr_flag[ SchedSelIdx ] is equal to 0), the initial arrival time for decoding unit m is derived by

tai( m ) = Max( taf( m − 1 ), tai,earliest( m ) )		(C-3)

where tai,earliest( m ) is derived as follows.

–	If decoding unit m is not the first decoding unit of a subsequent buffering period, tai,earliest( m ) is derived as

tai,earliest( m ) = tr,n( m ) − ( InitCpbRemovalDelay[ SchedSelIdx ] +
	InitCpbRemovalDelayOffset[ SchedSelIdx ] )  90000	(C-4)

with tr,n( m ) being the nominal removal time of decoding unit m from the CPB as specified in subclause Error: Reference source not found.

–	Otherwise (decoding unit m is the first decoding unit of a subsequent buffering period), tai,earliest( m ) is derived as

tai,earliest( m ) = tr,n( m ) − ( InitCpbRemovalDelay[ SchedSelIdx ]  90000 )	(C-5)

The final arrival time for decoding unit m is derived by

taf( m ) = tai( m ) + b( m )  BitRate[ SchedSelIdx ]		(C-6)

- where b( m ) is the size in bits of decoding unit m, counting the bits of the VCL NAL units and the filler data NAL units for the Type I conformance point or all bits of the Type II bitstream for the Type II conformance point, where the Type I and Type II conformance points are as shown in Figure C-1.

The values of SchedSelIdx, BitRate[ SchedSelIdx ], and CpbSize[ SchedSelIdx ] are constrained as follows.

–	If the content of the active sequence parameter sets for the access unit containing decoding unit m and the previous access unit differ, the HSS selects a value SchedSelIdx1 of SchedSelIdx from among the values of SchedSelIdx provided in the active sequence parameter set for the access unit containing decoding unit m that results in a BitRate[ SchedSelIdx1 ] or CpbSize[ SchedSelIdx1 ] for the access unit containing decoding unit m. The value of BitRate[ SchedSelIdx1 ] or CpbSize[ SchedSelIdx1 ] may differ from the value of BitRate[ SchedSelIdx0 ] or CpbSize[ SchedSelIdx0 ] for the value SchedSelIdx0 of SchedSelIdx that was in use for the previous access unit.

–	Otherwise, the HSS continues to operate with the previous values of SchedSelIdx, BitRate[ SchedSelIdx ] and CpbSize[ SchedSelIdx ].

When the HSS selects values of BitRate[ SchedSelIdx ] or CpbSize[ SchedSelIdx ] that differ from those of the previous access unit, the following applies.

–	the variable BitRate[ SchedSelIdx ] comes into effect at time tai( m )

–	the variable CpbSize[ SchedSelIdx ] comes into effect as follows.

–	If the new value of CpbSize[ SchedSelIdx ] exceeds the old CPB size, it comes into effect at time tai( m ),

–	Otherwise, the new value of CpbSize[ SchedSelIdx ] comes into effect at the CPB removal time of the last decoding unit of the access unit containing decoding unit m.

When SubPicCpbFlag is equal to 1, the initial CPB arrival time of access unit n tai( n ) is set to the initial CPB arrival time of the first decoding unit in access unit n, and the final CPB arrival time of access unit n taf( n ) is set to the final CPB arrival time of the last decoding unit in access unit n.

NOTE – When SubPicCpbFlag is equal to 0, each decoding unit is an access unit, hence the initial and final CPB arrival times of access unit n are the initial and final CPB arrival times of decoding unit n.

    - Timing of decoding unit removal and decoding of decoding unit

If SubPicCpbFlag is equal to 0, the variable CpbRemovalDelay( m ) is set to the value of cpb_removal_delay in the picture timing SEI message associated with the access unit that is decoding unit m, and the variable Tc is set to tc. Otherwise the variable CpbRemovalDelay( m ) is set to the value of du_cpb_removal_delay[ i ] for decoding unit m in the picture timing SEI message associated with the access unit that contains decoding unit m, and the variable Tc is set to tc_sub.

When a decoding unit m is the decoding unit with n equal to 0 (the first decoding unit of the access unit that initialises the HRD), the nominal removal time of the decoding unit from the CPB is specified by

tr,n( 0 ) = InitCpbRemovalDelay[ SchedSelIdx ]  90000	(C-7)

When a decoding unit m is the first decoding unit of the first access unit of a buffering period that does not initialise the HRD, the nominal removal time of the decoding unit from the CPB is specified by

tr,n( m ) = tr,n( mb ) + Tc * CpbRemovalDelay( m )		(C-8)

where tr,n( mb ) is the nominal removal time of the first decoding unit of the previous buffering period.

When a decoding unit m is the first decoding unit of a buffering period, mb is set equal to m at the removal time tr,n( m ) of the decoding unit m.

The nominal removal time tr,n( m ) of a decoding unit m that is not the first decoding unit of a buffering period is given by

tr,n( m ) = tr,n( mb ) + Tc * CpbRemovalDelay( m )

When a decoding unit m is the first decoding unit of a buffering period, mb is set equal to m at the removal time tr,n( m ) of the decoding unit m.

The nominal removal time tr,n( m ) of a decoding unit m that is not the first decoding unit of a buffering period is given by

tr,n( m ) = tr,n( mb ) + Tc * CpbRemovalDelay( m )		(C-9)

where tr,n( mb ) is the nominal removal time of the first decoding unit of the current buffering period.

The removal time of decoding unit m is specified as follows.

–	If low_delay_hrd_flag is equal to 0 or tr,n( m ) >= taf( m ), the removal time of decoding unit m is specified by

tr( m ) = tr,n( m )		(C-10)

- Otherwise (low_delay_hrd_flag is equal to 1 and tr,n( m ) < taf( m )), and sub_pic_cpb_params_present_flag is equal to 1 and the CPB is operating at the sub-picture level, the removal time of decoding unit m when it is not the last decoding unit of the access unit is specified by

tr( m ) = taf( m )	(C-11)

- Otherwise (low_delay_hrd_flag is equal to 1 and tr,n( m ) < taf( m )), and sub_pic_cpb_params_present_flag is equal to 1 and the CPB is operating at the sub-picture level, the removal time of decoding unit m when it is the last decoding unit of the access unit is specified by

tr( m ) = tr,n( m )+max( (tc_sub*Ceil(( taf( m) − tr,n( m))tc_sub)) , (tc*Ceil(( taf( n ) − tr,n( n ) )  tc )))

Otherwise (low_delay_hrd_flag is equal to 1 and tr,n( n ) < taf( n )), and sub_pic_cpb_params_present_flag is equal to 1 and the CPB is operating at the access unit level the removal time of access unit n is specified by

tr( n ) = tr,n( n )+max( (tc_sub*Ceil( ( taf( m ) − tr,n( m))tc_sub)) , (tc*Ceil(( taf( n ) − tr,n( n ) )  tc )))

Otherwise (low_delay_hrd_flag is equal to 1 and tr,n( m ) < taf( m )), the removal time of decoding unit m is specified by

tr( m ) = tr,n( m ) + Tc * Ceil( ( taf( m ) − tr,n( m ) ) c )	(C-12)

NOTE 1 – The latter case indicates that the size of decoding unit m, b( m ), is so large that it prevents removal at the nominal removal time.

When SubPicCpbFlag is equal to 1, the nominal CPB removal time of access unit n tr,n( n ) is set to the nominal CPB removal time of the last decoding unit in access unit n, the CPB removal time of access unit n tr( n ) is set to the CPB removal time of the last decoding unit in access unit n.

NOTE 2 – When SubPicCpbFlag is equal to 0, each decoding unit is an access unit, hence the nominal CPB removal time and the CPB revmoal time of access unit n are the nominal CPB removal time and the CPB removal time of decoding unit n.

At CPB removal time of decoding unit m, the decoding unit is instantaneously decoded.

  - Operation of the decoded picture buffer (DPB)

The decoded picture buffer contains picture storage buffers. Each of the picture storage buffers may contain a decoded picture that is marked as "used for reference" or is held for future output. Prior to initialization, the DPB is empty (the DPB fullness is set to zero). The following steps of the subclauses of this subclause happen in the sequence as listed below.

    - Removal of pictures from the DPB

The removal of pictures from the DPB before decoding of the current picture (but after parsing the slice header of the first slice of the current picture) happens instantaneously at the CPB removal time of the first decoding unit of access unit n (containing the current picture) and proceeds as follows.

The decoding process for reference picture set as specified in subclause Error: Reference source not found is invoked.

If the current picture is an IDR or a BLA picture, the following applies:

- When the IDR or BLA picture is not the first IDR or BLA picture decoded and the value of pic_width_in_luma_samples or pic_height_in_luma_samples or sps_max_dec_pic_buffering[ i ] for any possible value of i derived from the active sequence parameter set is different from the value of pic_width_in_luma_samples or pic_height_in_luma_samples or sps_max_dec_pic_buffering[ i ] derived from the sequence parameter set that was active for the preceding picture, respectively, no_output_of_prior_pics_flag is inferred to be equal to 1 by the HRD, regardless of the actual value of no_output_of_prior_pics_flag.

NOTE 1 – Decoder implementations should try to handle picture or DPB size changes more gracefully than the HRD in regard to changes in pic_width_in_luma_samples, pic_height_in_luma_samples, or sps_max_dec_pic_buffering[ i ].

- When no_output_of_prior_pics_flag is equal to 1 or is inferred to be equal to 1, all picture storage buffers in the DPB are emptied without output of the pictures they contain, and DPB fullness is set to 0.

All pictures k in the DPB, for which all of the following conditions are true, are removed from the DPB:

–	picture k is marked as "unused for reference",

–	picture k has PicOutputFlag equal to 0 or its DPB output time is less than or equal to the CPB removal time of the first decoding unit (denoted as decoding unit m) of the current picture n; i.e., to,dpb( k ) <= tr( m )

When a picture is removed from the DPB, the DPB fullness is decremented by one.

    - Picture output

The following happens instantaneously at the CPB removal time of access unit n, tr( n ).

Picture n is considered as decoded after the last decoding unit of the picture is decoded.

When picture n has PicOutputFlag equal to 1, its DPB output time to,dpb( n ) is derived by

to,dpb( n ) = tr( n ) + tc * dpb_output_delay( n )		(C-13)

where dpb_output_delay( n ) is the value of dpb_output_delay specified in the picture timing SEI message associated with access unit n.

The output of the current picture is specified as follows.

–	If PicOutputFlag is equal to 1 and to,dpb( n ) = tr( n ), the current picture is output.

–	Otherwise, if PicOutputFlag is equal to 0, the current picture is not output, but will be stored in the DPB as specified in subclause A.3.3.

–	Otherwise (PicOutputFlag is equal to 1 and to,dpb( n ) > tr( n ) ), the current picture is output later and will be stored in the DPB (as specified in subclause A.3.3) and is output at time to,dpb( n ) unless indicated not to be output by the decoding or inference of no_output_of_prior_pics_flag equal to 1 at a time that precedes to,dpb( n ).

When output, the picture shall be cropped, using the cropping rectangle specified in the active sequence parameter set.

When picture n is a picture that is output and is not the last picture of the bitstream that is output, the value of Dto,dpb( n ) is defined as:

Dto,dpb( n ) = to,dpb( nn ) − to,dpb( n )		(C-14)

where nn indicates the picture that follows after picture n in output order and has PicOutputFlag equal to 1.

    - Current decoded picture marking and storage

The following happens instantaneously at the CPB removal time of access unit n, tr( n ).

The current decoded picture is stored in the DPB in an empty picture storage buffer, and the DPB fullness is incremented by one. If the current picture is a reference picture, it is marked as "used for reference", otherwise it is marked as "unused for reference".

  - Bitstream conformance

A bitstream of coded data conforming to this Recommendation | International Standard shall fulfil the following requirements.

It is a requirement of bitstream conformance that the bitstream shall be constructed according to the syntax, semantics, and constraints specified in this Recommendation | International Standard outside of this annex.

It is a requirement of bitstream conformance that the first coded picture in a bitstream shall be a RAP picture, i.e., an IDR picture or a CRA picture or a BLA picture.

It is a requirement of bitstream conformance that any TFD picture associated with a particular RAP picture shall follow in output order any another RAP picture preceding the particular RAP picture in decoding order.

NOTE 1 – This constraint guarantees decodability of a TFD picture if its associated RAP picture is a CRA picture and if that CRA picture is not the first coded picture in the bitstream.

It is a requirement of bitstream conformance that any coded picture that follows a RAP picture in decoding order and uses inter prediction either from any picture that precedes the RAP picture in decoding order or from another TFD picture associated with the RAP picture shall be a TFD picture associated with the RAP picture.

NOTE 2 – Any coded picture that follows a RAP picture in decoding order and uses inter prediction either from any picture that precedes the RAP picture in decoding order or from another TFD picture associated with the RAP picture must precede the RAP in output order.

For any current picture being decoded, let the variables maxPicOrderCnt and minPicOrderCnt be set equal to the maximum and the mininum, respectively, of the PicOrderCntVal values for the current picture and all pictures in the DPB that are currently marked as "used for short-term reference" or "needed for output", and let the variable prevRefPic be the previous reference picture in decoding order that has temporal_id equal to or less than the temporal_id of the current picture. It is a requirement of bitstream conformance that the value of maxPicOrderCnt – minPicOrderCnt shall be less than MaxPicOrderCntLsb / 2, and that the absolute difference between the PicOrderCntVal values for the current picture and for prevRefPic shall be less than MaxPicOrderCntLsb / 2.

[Ed. (YK): Rewording of "needed for output" needed, as a picture may only be marked as "needed for output" when the output order DPB operation is applied.]

The bitstream is tested by the HRD for conformance as specified below:

For Type I bitstreams, the number of tests carried out is equal to cpb_cnt_minus1 + 1 where cpb_cnt_minus1 is either the syntax element of hrd_parameters( ) following the vcl_hrd_parameters_present_flag or is determined by the application by other means not specified in this Recommendation | International Standard. One test is carried out for each bit rate and CPB size combination specified by hrd_parameters( ) following the vcl_hrd_parameters_present_flag. Each of these tests is conducted at the Type I conformance point shown in Figure C-1.

For Type II bitstreams there are two sets of tests. The number of tests of the first set is equal to cpb_cnt_minus1 + 1 where cpb_cnt_minus1 is either the syntax element of hrd_parameters( ) following the vcl_hrd_parameters_present_flag or is determined by the application by other means not specified in this Recommendation | International Standard. One test is carried out for each bit rate and CPB size combination. Each of these tests is conducted at the Type I conformance point shown in Figure C-1. For these tests, only VCL and filler data NAL units are counted for the input bit rate and CPB storage.

The number of tests of the second set, for Type II bitstreams, is equal to cpb_cnt_minus1 + 1 where cpb_cnt_minus1 is either the syntax element of hrd_parameters( ) following the nal_hrd_parameters_present_flag or is determined by the application by other means not specified in this Recommendation | International Standard. One test is carried out for each bit rate and CPB size combination specified by hrd_parameters( ) following the nal_hrd_parameters_present_flag. Each of these tests is conducted at the Type II conformance point shown in Figure C-1. For these tests, all NAL units (of a Type II NAL unit stream) or all bytes (of a byte stream) are counted for the input bit rate and CPB storage.

NOTE 3 – NAL HRD parameters established by a value of SchedSelIdx for the Type II conformance point shown in Figure C-1 are sufficient to also establish VCL HRD conformance for the Type I conformance point shown in Figure C-1 for the same values of InitCpbRemovalDelay[ SchedSelIdx ], BitRate[ SchedSelIdx ], and CpbSize[ SchedSelIdx ] for the VBR case (cbr_flag[ SchedSelIdx ] equal to 0). This is because the data flow into the Type I conformance point is a subset of the data flow into the Type II conformance point and because, for the VBR case, the CPB is allowed to become empty and stay empty until the time a next picture is scheduled to begin to arrive. For example, when decoding a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found using the decoding process specified in clauses Error: Reference source not found-Error: Reference source not found, when NAL HRD parameters are provided for the Type II conformance point that not only fall within the bounds set for NAL HRD parameters for profile conformance in item Error: Reference source not found of subclause Error: Reference source not found but also fall within the bounds set for VCL HRD parameters for profile conformance in item Error: Reference source not found of subclause Error: Reference source not found, conformance of the VCL HRD for the Type I conformance point is also assured to fall within the bounds of item Error: Reference source not found of subclause Error: Reference source not found.

It is a requirement of bitstream conformance that all of the following conditions shall be fulfilled for each of the tests:

- For each access unit n, with n>0, associated with a buffering period SEI message, with tg,90( n ) specified by

tg,90( n ) = 90000 * ( tr,n( n ) − taf( n − 1 ) )		(C-15)

the value of InitCpbRemovalDelay[ SchedSelIdx ] shall be constrained as follows.

–	If cbr_flag[ SchedSelIdx ] is equal to 0,

InitCpbRemovalDelay[ SchedSelIdx ] <= Ceil( tg,90( n ) )	(C-16)

–	Otherwise (cbr_flag[ SchedSelIdx ] is equal to 1),

Floor( tg,90( n ) ) <= 	InitCpbRemovalDelay[ SchedSelIdx ] <= Ceil( tg,90( n ) )	(C-17)

- NOTE 4 – The exact number of bits in the CPB at the removal time of each picture may depend on which buffering period SEI message is selected to initialise the HRD. Encoders must take this into account to ensure that all specified constraints must be obeyed regardless of which buffering period SEI message is selected to initialise the HRD, as the HRD may be initialised at any one of the buffering period SEI messages.

- A CPB overflow is specified as the condition in which the total number of bits in the CPB is larger than the CPB size. The CPB shall never overflow.

- A CPB underflow is specified as the condition in which the nominal CPB removal time of decoding unit m tr,n( m ) is less than the final CPB arrival time of decoding unit m taf( m ) for any value of m. When low_delay_hrd_flag is equal to 0, the CPB shall never underflow.

- A CPB underflow at access unit n shall occur when low_delay_hrd_flag is equal to 1 and SubPicCpbFlag is equal to 1 and a CPB underflow at decoding unit m in access unit n occurs.  (Needs editorial improvement)

- The nominal removal times of pictures from the CPB (starting from the second picture in decoding order), shall satisfy the constraints on tr,n( n ) and tr( n ) expressed in subclauses Error: Reference source not found through Error: Reference source not found.

- After the decoding process for reference picture set as specified in subclause Error: Reference source not found has been invoked, the number of decoded pictures in the DPB, not including the current picture, with temporal_id lower than or equal to the temporal_id of the current picture, shall be less than or equal to sps_max_dec_pic_buffering[ temporal_id ].

- All reference pictures shall be present in the DPB when needed for prediction. Each picture shall be present in the DPB at its DPB output time unless it is removed from the DPB before its output time by one of the processes specified in subclause A.3.

- The value of to,dpb( n ) as given by Equation C-14, which is the difference between the output time of a picture and that of the first picture following it in output order and having pic_output_flag equal to 1, shall satisfy the constraint expressed in subclause Error: Reference source not found for the profile and level specified in the bitstream using the decoding process specified in clauses Error: Reference source not found-Error: Reference source not found.

- If sub_pic_cpb_params_present_flag is 1 then it is requirement of the bitstream conformance that following constraint is obeyed when signaling the values for cpb_removal_delay and du_cpb_removal_delay.

cpb_removal_delay*tc = =du_cpb_removal_delay[num_deocding_units_minus1]* tc_sub

  - Decoder conformance

A decoder conforming to this Recommendation | International Standard shall fulfil the following requirements.

A decoder claiming conformance to a specific profile and level shall be able to decode successfully all conforming bitstreams specified for decoder conformance in subclause A.4, provided that all sequence parameter sets and picture parameter sets referred to in the VCL NAL units, and appropriate buffering period and picture timing SEI messages are conveyed to the decoder, in a timely manner, either in the bitstream (by non-VCL NAL units), or by external means not specified by this Recommendation | International Standard.

There are two types of conformance that can be claimed by a decoder: output timing conformance and output order conformance.

To check conformance of a decoder, test bitstreams conforming to the claimed profile and level, as specified by subclause A.4 are delivered by a hypothetical stream scheduler (HSS) both to the HRD and to the decoder under test (DUT). All pictures output by the HRD shall also be output by the DUT and, for each picture output by the HRD, the values of all samples that are output by the DUT for the corresponding picture shall be equal to the values of the samples output by the HRD.

For output timing decoder conformance, the HSS operates as described above, with delivery schedules selected only from the subset of values of SchedSelIdx for which the bit rate and CPB size are restricted as specified in Annex Error: Reference source not found for the specified profile and level, or with "interpolated" delivery schedules as specified below for which the bit rate and CPB size are restricted as specified in Annex Error: Reference source not found. The same delivery schedule is used for both the HRD and DUT.

When the HRD parameters and the buffering period SEI messages are present with cpb_cnt_minus1 greater than 0, the decoder shall be capable of decoding the bitstream as delivered from the HSS operating using an "interpolated" delivery schedule specified as having peak bit rate r, CPB size c( r ), and initial CPB removal delay ( f( r )r ) as follows:

 = ( r − BitRate[ SchedSelIdx − 1 ] )  ( BitRate[ SchedSelIdx ] − BitRate[ SchedSelIdx − 1 ] ),	(C-18)

c( r ) =  * CpbSize[ SchedSelIdx ] + (1 −  * CpbSize[ SchedSelIdx−1 ],	(C-19)

f( r ) = InitCpbRemovalDelay[ SchedSelIdx ] * BitRate[ SchedSelIdx ] + 
		( 1 −  InitCpbRemovalDelay[ SchedSelIdx − 1 ] * BitRate[ SchedSelIdx − 1 ]	(C-20)

for any SchedSelIdx > 0 and r such that BitRate[ SchedSelIdx − 1 ] <= r <= BitRate[ SchedSelIdx ] such that r and c( r ) are within the limits as specified in Annex Error: Reference source not found for the maximum bit rate and buffer size for the specified profile and level.

NOTE 1 – InitCpbRemovalDelay[ SchedSelIdx ] can be different from one buffering period to another and have to be re-calculated.

For output timing decoder conformance, an HRD as described above is used and the timing (relative to the delivery time of the first bit) of picture output is the same for both HRD and the DUT up to a fixed delay.

For output order decoder conformance, the HSS delivers the bitstream to the DUT "by demand" from the DUT, meaning that the HSS delivers bits (in decoding order) only when the DUT requires more bits to proceed with its processing.

NOTE 2 – This means that for this test, the coded picture buffer of the DUT could be as small as the size of the largest access unit.

A modified HRD as described below is used, and the HSS delivers the bitstream to the HRD by one of the schedules specified in the bitstream such that the bit rate and CPB size are restricted as specified in Annex Error: Reference source not found. The order of pictures output shall be the same for both HRD and the DUT.

For output order decoder conformance, the HRD CPB size is equal to CpbSize[ SchedSelIdx ] for the selected schedule and the DPB size is equal to MaxDpbSize. Removal time from the CPB for the HRD is equal to final bit arrival time and decoding is immediate. The operation of the DPB of this HRD is described below.

    - Operation of the output order DPB

The decoded picture buffer contains picture storage buffers. Each of the picture storage buffers contains a decoded picture that is marked as "used for reference" or is held for future output. At HRD initialization, the DPB is empty. The following steps happen in the order as listed below.

    - Removal of pictures from the DPB

The removal of pictures from the DPB before decoding of the current picture (but after parsing the slice header of the first slice of the current picture) happens instantaneously when the first decoding unit of the access unit containing the current picture is removed from the CPB and proceeds as follows.

The decoding process for reference picture set as specified in subclause Error: Reference source not found is invoked.

–	If the current picture is an IDR or a BLA picture, the following applies.

- When the IDR or BLA picture is not the first IDR or BLA picture decoded and the value of pic_width_in_luma_samples or pic_height_in_luma_samples or sps_max_dec_pic_buffering[ i ] for any possible value of i derived from the active sequence parameter set is different from the value of pic_width_in_luma_samples or pic_height_in_luma_samples or sps_max_dec_pic_buffering[ i ] derived from the sequence parameter set that was active for the preceding picture, respectively, no_output_of_prior_pics_flag is inferred to be equal to 1 by the HRD, regardless of the actual value of no_output_of_prior_pics_flag.

NOTE – Decoder implementations should try to handle picture or DPB size changes more gracefully than the HRD in regard to changes in pic_width_in_luma_samples, pic_height_in_luma_samples or sps_max_dec_pic_buffering[ i ].

- When no_output_of_prior_pics_flag is equal to 1 or is inferred to be equal to 1, all picture storage buffers in the DPB are emptied without output of the pictures they contain.

–	Otherwise (the current picture is not an IDR or a BLA picture), picture storage buffers containing a picture which are marked as "not needed for output" and "unused for reference" are emptied (without output). When one or more of the following conditions are true, the "bumping" process specified in subclause A.4.2.1 is invoked repeatedly until there is an empty picture storage buffer to store the current decoded picture.

- The number of pictures in the DPB that are marked as "needed for output" is greater than sps_num_reorder_pics[ temporal_id ],

- The number of pictures in the DPB with temporal_id lower than or equal to the temporal_id of the current picture is equal to sps_max_dec_pic_buffering[ temporal_id ].

When the current picture is an IDR or a BLA picture for which no_output_of_prior_pics_flag is not equal to 1 and is not inferred to be equal to 1, the following two steps are performed.

1.	Picture storage buffers containing a picture that is marked as "not needed for output" and "unused for reference" are emptied (without output).

2.	All non-empty picture storage buffers in the DPB are emptied by repeatedly invoking the "bumping" process specified in subclause A.4.2.1.

      - "Bumping" process

The "bumping" process is invoked in the following cases.

–	The current picture is an IDR or a BLA picture and no_output_of_prior_pics_flag is not equal to 1 and is not inferred to be equal to 1, as specified in subclause A.4.2.

–	The number of pictures in the DPB that are marked "needed for output" is greater than sps_num_reorder_pics[ temporal_id ], as specified in subclause A.4.2.

–	The number of pictures in the DPB with temporal_id lower than or equal to the temporal_id of the current picture is equal to sps_max_dec_pic_buffering[ temporal_id ], as specified in subclause A.4.2.

The "bumping" process consists of the following ordered steps:

- The picture that is first for output is selected as the one having the smallest value of PicOrderCntVal of all pictures in the DPB marked as "needed for output".

- The picture is cropped, using the cropping rectangle specified in the active sequence parameter set for the picture, the cropped picture is output, and the picture is marked as "not needed for output".

- If the picture storage buffer that included the picture that was cropped and output contains a picture marked as "unused for reference", the picture storage buffer is emptied.

    - Picture decoding, marking and storage

The following happens instantaneously when the last decoding unit of access unit n containing the current picture is removed from the CPB.

The current picture is considered as decoded after the last decoding unit of the picture is decoded. The current decoded picture is stored in an empty picture storage buffer in the DPB, and the following applies.

–	If the current decoded picture has PicOutputFlag equal to 1, it is marked as "needed for output".

–	Otherwise (the current decoded picture has PicOutputFlag equal to 0), it is marked as "not needed for output".

If the current decoded picture is a reference picture, it is marked as "used for reference", otherwise (the current decoded picture is a non-reference picture), it is marked as "unused for reference".

- Annex D

Supplemental enhancement information

(This annex forms an integral part of this Recommendation | International Standard)

This annex specifies syntax and semantics for SEI message payloads.

SEI messages assist in processes related to decoding, display or other purposes. However, SEI messages are not required for constructing the luma or chroma samples by the decoding process. Conforming decoders are not required to process this information for output order conformance to this Recommendation | International Standard (see Annex A for the specification of conformance). Some SEI message information is required to check bitstream conformance and for output timing decoder conformance.

In Annex Error: Reference source not found, specification for presence of SEI messages are also satisfied when those messages (or some subset of them) are conveyed to decoders (or to the HRD) by other means not specified by this Recommendation | International Standard. When present in the bitstream, SEI messages shall obey the syntax and semantics specified in subclauses Error: Reference source not found and this annex. When the content of an SEI message is conveyed for the application by some means other than presence within the bitstream, the representation of the content of the SEI message is not required to use the same syntax specified in this annex. For the purpose of counting bits, only the appropriate bits that are actually present in the bitstream are counted.

  - SEI payload syntax

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| if( payloadType = = 0 ) |  |
| buffering_period( payloadSize ) |  |
| else if( payloadType = = 1 ) |  |
| pic_timing( payloadSize ) |  |
| else if( payloadType = = 2 ) |  |
| pan_scan_rect( payloadSize ) |  |
| else if( payloadType = = 3 ) |  |
| filler_payload( payloadSize ) |  |
| else if( payloadType = = 4 ) |  |
| user_data_registered_itu_t_t35( payloadSize ) |  |
| else if( payloadType = = 5 ) |  |
| user_data_unregistered( payloadSize ) |  |
| else if( payloadType = = 6 ) |  |
| recovery_point( payloadSize ) |  |
| else if( payloadType = = 9 ) |  |
| scene_info( payloadSize ) |  |
| else if( payloadType = = 15 ) |  |
| full_frame_snapshot( payloadSize ) |  |
| else if( payloadType = = 16 ) |  |
| progressive_refinement_segment_start( payloadSize ) |  |
| else if( payloadType = = 17 ) |  |
| progressive_refinement_segment_end( payloadSize ) |  |
| else if( payloadType = = 19 ) |  |
| film_grain_characteristics( payloadSize ) |  |
| else if( payloadType = = 20 ) |  |
| deblocking_filter_display_preference( payloadSize ) |  |
| else if( payloadType = = 22 ) |  |
| post_filter_hint( payloadSize ) |  |
| else if( payloadType = = 23 ) |  |
| tone_mapping_info( payloadSize ) |  |
| else if( payloadType = = 45 ) |  |
| frame_packing_arrangement( payloadSize ) |  |
| else if( payloadType = = 47 ) |  |
| display_orientation( payloadSize ) |  |
| else if( payloadType = = 128 ) |  |
| sop_description( payloadSize ) |  |
| else if( payloadType = = 129 ) |  |
| field_indication( payloadSize ) |  |
| else if( payloadType = = 130) |  |
| decoded_picture_hash( payloadSize ) |  |
| else |  |
| reserved_sei_message( payloadSize ) |  |
| if( !byte_aligned( ) ) { |  |
| bit_equal_to_one /* equal to 1 */ | f(1) |
| while( !byte_aligned( ) ) |  |
| bit_equal_to_zero /* equal to 0 */ | f(1) |
| } |  |
| } |  |

    - Buffering period SEI message syntax

| buffering_period( payloadSize ) { | Descriptor |
| --- | --- |
| seq_parameter_set_id | ue(v) |
| if( !sub_pic_cpb_params_present_flag ) |  |
| rap_cpb_params_present_flag | u(1) |
| if( NalHrdBpPresentFlag ) { |  |
| for( SchedSelIdx = 0; SchedSelIdx <= cpb_cnt_minus1; SchedSelIdx++ ) { |  |
| initial_cpb_removal_delay[ SchedSelIdx ] | u(v) |
| initial_cpb_removal_delay_offset[ SchedSelIdx ] | u(v) |
| if( sub_pic_cpb_params_present_flag ¦ ¦ rap_cpb_params_present_flag ) { |  |
| initial_alt_cpb_removal_delay[ SchedSelIdx ] | u(v) |
| initial_alt_cpb_removal_delay_offset[ SchedSelIdx ] | u(v) |
| } |  |
| } |  |
| } |  |
| if( VclHrdBpPresentFlag ) { |  |
| for( SchedSelIdx = 0; SchedSelIdx <= cpb_cnt_minus1; SchedSelIdx++ ) { |  |
| initial_cpb_removal_delay[ SchedSelIdx ] | u(v) |
| initial_cpb_removal_delay_offset[ SchedSelIdx ] | u(v) |
| if( sub_pic_cpb_params_present_flag ¦ ¦ rap_cpb_params_present_flag) { |  |
| initial_alt_cpb_removal_delay[ SchedSelIdx ] | u(v) |
| initial_alt_cpb_removal_delay_offset[ SchedSelIdx ] | u(v) |
| } |  |
| } |  |
| } |  |
| } |  |

    - Picture timing SEI message syntax

| pic_timing( payloadSize ) { | Descriptor |
| --- | --- |
| if( CpbDpbDelaysPresentFlag ) { |  |
| cpb_removal_delay | u(v) |
| dpb_output_delay | u(v) |
| if( sub_pic_cpb_params_present_flag ) { |  |
| num_decoding_units_minus2 | ue(v) |
| for( i = 0; i <= num_decoding_units_minus2; i++ ) { |  |
| num_nalus_in_du_minus1[ i ] | ue(v) |
| du_cpb_removal_delay[ i ] | u(v) |
| } |  |
| } |  |
| } |  |
| } |  |

    - Pan-scan rectangle SEI message syntax

The syntax table is specifed in subclause D.1.3 of Rec. ITU-T H.264 | ISO/IEC 14496-10.

    - Filler payload SEI message syntax

The syntax table is specifed in subclause D.1.4 of Rec. ITU-T H.264 | ISO/IEC 14496-10.

    - User data registered by Rec. ITU-T T.35 SEI message syntax

The syntax table is specifed in subclause D.1.5 of Rec. ITU-T H.264 | ISO/IEC 14496-10.

    - User data unregistered SEI message syntax

The syntax table is specifed in subclause D.1.6 of Rec. ITU-T H.264 | ISO/IEC 14496-10.

    - Recovery point SEI message syntax

| recovery_point( payloadSize ) { | Descriptor |
| --- | --- |
| recovery_poc_cnt | ue(v) |
| exact_match_flag | u(1) |
| broken_link_flag | u(1) |
| } |  |

[Ed. (YK): The broken_link_flag is not useful anymore after the introduction of BLA pictures, and therefore should be removed from the recovery point SEI message.]

    - Scene information SEI message syntax

The syntax table is specifed in subclause D.1.10 of Rec. ITU-T H.264 | ISO/IEC 14496-10.

    - Full-frame snapshot SEI message syntax

The syntax table is specifed in subclause D.1.16 of Rec. ITU-T H.264 | ISO/IEC 14496-10.

    - Progressive refinement segment start SEI message syntax

The syntax table is specifed in subclause D.1.17 of Rec. ITU-T H.264 | ISO/IEC 14496-10.

    - Progressive refinement segment end SEI message syntax

The syntax table is specifed in subclause D.1.18 of Rec. ITU-T H.264 | ISO/IEC 14496-10.

    - Film grain characteristics SEI message syntax

The syntax table is specifed in subclause D.1.20 of Rec. ITU-T H.264 | ISO/IEC 14496-10.

    - Deblocking filter display preference SEI message syntax

The syntax table is specifed in subclause D.1.21 of Rec. ITU-T H.264 | ISO/IEC 14496-10.

    - Post-filter hint SEI message syntax

The syntax table is specifed in subclause D.1.23 of Rec. ITU-T H.264 | ISO/IEC 14496-10.

    - Tone mapping information SEI message syntax

The syntax table is specifed in subclause D.1.24 of Rec. ITU-T H.264 | ISO/IEC 14496-10.

    - Frame packing arrangement SEI message syntax

The syntax table is specifed in subclause D.1.25 of Rec. ITU-T H.264 | ISO/IEC 14496-10.

    - Display orientation SEI message syntax

| display_orientation( payloadSize ) { | Descriptor |
| --- | --- |
| display_orientation_cancel_flag | u(1) |
| if( !display_orientation_cancel_flag ) { |  |
| hor_flip | u(1) |
| ver_flip | u(1) |
| anticlockwise_rotation | u(16) |
| display_orientation_repetition_period | ue(v) |
| display_orientation_extension_flag | u(1) |
| } |  |
| } |  |

    - SOP description SEI message syntax

| sop_description( payloadSize ) { | Descriptor |
| --- | --- |
| sps_id | ue(v) |
| num_pics_in_sop_minus1 | ue(v) |
| for( i = 0; i <= num_pics_in_sop_minus1; i++ ) { |  |
| sop_desc_nal_ref_flag[ i ] | u(1) |
| sop_desc_temporal_id[ i ] | u(3) |
| st_rps_idx[ i ] | ue(v) |
| if( i > 0 ) |  |
| poc_delta[ i ] | se(v) |
| } |  |
| } |  |

    - Field indication SEI message syntax

| field_indication( payloadSize ) { | Descriptor |
| --- | --- |
| field_pic_flag | u(1) |
| progressive_source_flag | u(1) |
| duplicate_flag | u(1) |
| if( field_pic_flag ) |  |
| bottom_field_flag | u(1) |
| else if( !progressive_source_flag ) |  |
| top_field_first_flag | u(1) |
| else |  |
| reserved_zero_1bit /* equal to 0 */ | u(1) |
| reserved_zero_4bits /* equal to 0 */ | u(4) |
| } |  |

    - Decoded picture hash SEI message syntax

| decoded_picture_hash( payloadSize ) { | Descriptor |
| --- | --- |
| hash_type | u(8) |
| for( cIdx = 0; cIdx < (chroma_format_idc = = 0) ? 1 : 3; cIdx++ ) |  |
| if( hash_type = = 0 ) |  |
| for( i = 0; i < 16; i++) |  |
| picture_md5[ cIdx ][ i ] | b(8) |
| else if( hash_type = = 1 ) |  |
| picture_crc[ cIdx ] | u(16) |
| else if( hash_type = = 2 ) |  |
| picture_checksum[ cIdx ] | u(32) |
| } |  |

    - Reserved SEI message syntax

| reserved_sei_message( payloadSize ) { | Descriptor |
| --- | --- |
| for( i = 0; i < payloadSize; i++ ) |  |
| reserved_sei_message_payload_byte | b(8) |
| } |  |

  - SEI payload semantics

    - Buffering period SEI message semantics

When NalHrdBpPresentFlag or VclHrdBpPresentFlag are equal to 1, a buffering period SEI message can be associated with any access unit in the bitstream, and a buffering period SEI message shall be associated with each RAP access unit, and with each access unit associated with a recovery point SEI message.

NOTE – For some applications, the frequent presence of a buffering period SEI message may be desirable.

A buffering period is specified as the set of access units between two instances of the buffering period SEI message in decoding order.

seq_parameter_set_id specifies the sequence parameter set that contains the sequence HRD attributes. The value of seq_parameter_set_id shall be equal to the value of seq_parameter_set_id in the picture parameter set referenced by the primary coded picture associated with the buffering period SEI message. The value of seq_parameter_set_id shall be in the range of 0 to 31, inclusive.

rap_cpb_params_present_flag equal to 1 specifies the presence of the initial_alt_cpb_removal_delay[ SchedSelIdx ] and initial_alt_cpb_removal_delay_offset[ SchedSelIdx ] syntax elements. When not present, the value of rap_cpb_params_present_flag is inferred to be equal to 0. When the associated picture is neither a CRA picture nor a BLA picture, the value of rap_cpb_params_present_flag shall be equal to 0.

initial_cpb_removal_delay[ SchedSelIdx ] and initial_alt_cpb_removal_delay[ SchedSelIdx ] specify the initial CPB removal delays for the SchedSelIdx-th CPB. The syntax elements have a length in bits given by initial_cpb_removal_delay_length_minus1 + 1, and are in units of a 90 kHz clock. The values of the syntax elements shall not be equal to 0 and shall not exceed 90000 * ( CpbSize[ SchedSelIdx ]  BitRate[ SchedSelIdx ] ), the time-equivalent of the CPB size in 90 kHz clock units.

initial_cpb_removal_delay_offset[ SchedSelIdx ] and initial_alt_cpb_removal_delay_offset[ SchedSelIdx ] are used for the SchedSelIdx-th CPB to specify the initial delivery time of coded data units to the CPB. The syntax elements have a length in bits given by initial_cpb_removal_delay_length_minus1 + 1 and are in units of a 90 kHz clock. These syntax elements are not used by decoders and may be needed only for the delivery scheduler (HSS) specified in Annex A.

Over the entire coded video sequence, the sum of initial_cpb_removal_delay[ SchedSelIdx ] and initial_cpb_removal_delay_offset[ SchedSelIdx ] shall be constant for each value of SchedSelIdx, and the sum of initial_alt_cpb_removal_delay[ SchedSelIdx ] and initial_alt_cpb_removal_delay_offset[ SchedSelIdx ] shall be constant for each value of SchedSelIdx.

    - Picture timing SEI message semantics

NOTE 1 – The syntax of the picture timing SEI message is dependent on the content of the sequence parameter set that is active for the coded picture associated with the picture timing SEI message. However, unless the picture timing SEI message of an IDR or BLA access unit is preceded by a buffering period SEI message within the same access unit, the activation of the associated sequence parameter set (and, for IDR or BLA pictures that are not the first picture in the bitstream, the determination that the coded picture is an IDR picture or a BLA picture) does not occur until the decoding of the first coded slice NAL unit of the coded picture. Since the coded slice NAL unit of the coded picture follows the picture timing SEI message in NAL unit order, there may be cases in which it is necessary for a decoder to store the RBSP containing the picture timing SEI message until determining the parameters of the sequence parameter that will be active for the coded picture, and then perform the parsing of the picture timing SEI message.

The presence of picture timing SEI message in the bitstream is specified as follows.

–	If CpbDpbDelaysPresentFlag is equal to 1, one picture timing SEI message shall be present in every access unit of the coded video sequence.

–	Otherwise (CpbDpbDelaysPresentFlag is equal to 0), no picture timing SEI messages shall be present in any access unit of the coded video sequence.

cpb_removal_delay specifies how many clock ticks to wait after removal from the CPB of the access unit associated with the most recent buffering period SEI message in a preceding access unit before removing from the buffer the access unit data associated with the picture timing SEI message. This value is also used to calculate an earliest possible time of arrival of access unit data into the CPB for the HSS. The syntax element is a fixed length code whose length in bits is given by cpb_removal_delay_length_minus1 + 1. The cpb_removal_delay is the remainder of a modulo 2(cpb_removal_delay_length_minus1 + 1) counter.

NOTE 2 – The value of cpb_removal_delay_length_minus1 that determines the length (in bits) of the syntax element cpb_removal_delay is the value of cpb_removal_delay_length_minus1 coded in the sequence parameter set that is active for the primary coded picture associated with the picture timing SEI message, although cpb_removal_delay specifies a number of clock ticks relative to the removal time of the preceding access unit containing a buffering period SEI message, which may be an access unit of a different coded video sequence.

dpb_output_delay is used to compute the DPB output time of the picture. It specifies how many clock ticks to wait after removal of the last decoding unit in an access unit from the CPB before the decoded picture is output from the DPB.

NOTE 3 – A picture is not removed from the DPB at its output time when it is still marked as "used for short-term reference" or "used for long-term reference".

NOTE 4 – Only one dpb_output_delay is specified for a decoded picture.

The length of the syntax element dpb_output_delay is given in bits by dpb_output_delay_length_minus1 + 1. When sps_max_dec_pic_buffering[ max_temporal_layers_minus1 ] is equal to 0, dpb_output_delay shall be equal to 0.

The output time derived from the dpb_output_delay of any picture that is output from an output timing conforming decoder shall precede the output time derived from the dpb_output_delay of all pictures in any subsequent coded video sequence in decoding order.

The picture output order established by the values of this syntax element shall be the same order as established by the values of PicOrderCntVal.

For pictures that are not output by the "bumping" process because they precede, in decoding order, an IDR or BLA picture with no_output_of_prior_pics_flag equal to 1 or inferred to be equal to 1, the output times derived from dpb_output_delay shall be increasing with increasing value of PicOrderCntVal relative to all pictures within the same coded video sequence.

num_decoding_units_minus2  plus 2 specifies the number of decoding units in the access unit the picture timing SEI message is associated with. The value of num_decoding_units_minus2 shall be in the range of 0 to PicWidthInCtbs * PicHeightInCtbs – 2, inclusive.

num_nalus_in_du_minus1[ i ] plus 1 specifies the number of NAL units in the i-th decoding unit of the access unit the picture timing SEI message is associated with. The value of num_nalus_in_du_minus1[ i ] shall be in the range of 0 to PicWidthInCtbs * PicHeightInCtbs – 1, inclusive.

The first decoding unit of the access unit consists of the first num_nalus_in_du_minus1[ 0 ] + 1 consecutive NAL units in decoding order in the access unit. The i-th (with i greater than 0) decoding unit of the access unit consists of the num_nalus_in_du_minus1[ i ] + 1 consecutive NAL units immediately following the last NAL unit in the previous decoding unit of the access unit, in decoding order. There shall be at least one VCL NAL unit in each decoding unit. All non-VCL NAL units associated with a VCL NAL unit shall be included in the same decoding unit.

du_cpb_removal_delay[ i ] specifies how many sub-picture clock ticks to wait after removal from the CPB of the first decoding unit in the access unit associated with the most recent buffering period SEI message in a preceding access unit before removing from the CPB the i-th decoding unit in the access unit associated with the picture timing SEI message. This value is also used to calculate an earliest possible time of arrival of decoding unit data into the CPB for the HSS, as specified in Annex A. The syntax element is a fixed length code whose length in bits is given by cpb_removal_delay_length_minus1 + 1. The du_cpb_removal_delay[ i ] is the remainder of a modulo 2(cpb_removal_delay_length_minus1 + 1) counter.

NOTE 5 – The value of cpb_removal_delay_length_minus1 that determines the length (in bits) of the syntax element du_cpb_removal_delay[ i ] is the value of cpb_removal_delay_length_minus1 coded in the sequence parameter set that is active for the coded picture associated with the picture timing SEI message, although du_cpb_removal_delay[ i ] specifies a number of sub-picture clock ticks relative to the removal time of the first decoding unit in the preceding access unit containing a buffering period SEI message, which may be an access unit of a different coded video sequence.

    - Pan-scan rectangle SEI message semantics

The semantics specifed in subclause D.2.3 of ITU-T Rec. H.264 | ISO/IEC 14496-10 apply.