| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 10th Meeting: Stockholm, Sweden, 11 July – 20 July, 2012 | Document: JCTVC-J0569 WG11 Number: m26294 |
| --- | --- |

| Title: | Sub-picture Based CPB Removal Timing |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande 5750 Pacific Rim Blvd, Camas, WA 98607, USA. Kimihiko Kazui 4-1-1 Kamikodanaka, Nakahara-ku 211-8588 Kawasaki JAPAN | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com +81-44-754-2639 kazui.kimihiko@jp.fujitsu.com |
| Source: | Sharp Labs of America, Inc., FUJITSU LABORATORIES LTD. |  |  |

_____________________________

# Abstract

When the sub-picture level CPB removal delay parameters are present the CPB may operate at access unit level or sub-picture level. This document proposes a bitstream conformance requirement, which guarantees that the operation of sub picture based CPB operating at sub-picture level and CPB operating at access unit level results in the same timing of decoding unit removal.

Additionally when sub-picture level CPB removal delay parameters are present this document proposes a change in the removal time of decoding unit when low_delay_hrd_flag is 1 and tr,n( m ) < taf( m ).

The proposed changes are included in the attachment of this document.

# Introduction

This proposal merges related aspects of the documents JCTVC-J0136 [2] and JCTVC-J0306 [3].

# Proposal

HEVC Draft 7 [1] includes the sub-picture based CPB HRD operation and related syntax and semantics.

We describe proposed changes to the sub-picture based CPB operation.

- When the sub-picture level CPB removal delay parameters are present the CPB may operate at access unit level or sub-picture level. This document proposes a bitstream conformance requirement, which guarantees that the operation of sub picture based CPB operating at sub-picture level and CPB operating at access unit level results in the same timing of decoding unit removal.

- When sub-picture level CPB removal delay parameters are present this document proposes a change in the removal time of decoding unit when low_delay_hrd_flag is 1 and tr,n( m ) < taf( m ).

# Patent rights declaration(s)

Sharp may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

FUJITSU LIMITED may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] B. Bros, W-J. Han, J-R. Ohm, G. J. Sullivan, and T. Wiegand, “High efficiency video coding (HEVC) text specification draft 7,” JCTVC-I1003_dK, Geneva, May 2012.

[2] K. Kazui, J. Koyama, S. Shimada, A. Nakagwa, “AHG9: Improvement of HRD for sub-picture based operation,” JCTVC-J0136, Stockholm, July 2012.

[3] S. Deshpande, “On Sub-picture Based CPB,” JCTVC-J0306, Stockholm, July 2012