| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0573r1 |
| --- | --- |

| Title: | Tiles for the Main profile |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | J. Boyce (Vidyo) A. Fuldseth (Cisco) M. Horowitz (eBrisk) R. James (Altera) J. Sampedro (Polycom) A. Segall (Sharp) J. Sievers (LifeSize) A. Wells (Ambarella) Y. Ye (InterDigital) | Email: | michael@ebriskvideo.com |
| Source: | Altera, Ambarella, Cisco, eBrisk, InterDigital, LifeSize, Polycom, Sharp, Vidyo |  |  |

_____________________________

# Abstract

The coding tool called tiles have been a part of the Main profile since the Main profile was defined at the JCT-VC meeting in San Jose in February 2012. We believe that tiles is an important tool for a variety of applications and support its continued inclusion in the Main profile.

# Introduction

The coding tool called tiles have been a part of the Main profile since the Main profile was defined at the 8th JCT-VC meeting in San Jose, California in February 2012. We believe that tiles is an important tool for a variety of applications and support its continued inclusion in the Main profile.

While tiles have been reported to increase the complexity of single core decoder applications, it has been asserted that some mainstream applications cannot cost-effectively manufacture high quality HEVC-based video implementations without tiles. For example, there exist software-based encoders for low delay video applications on multi-core processors that partition the picture into tiles and each tile is processed by single core. While it would be possible to use slices (in lieu of tiles) in a similar manner, the coding efficiency losses resulting from slice header overhead, losses due to the less compact shape of slices (compared with tiles) and losses due to slice header and packetization overhead make this choice undesirable [1]. Similarly, issues with inter-core communication requirements, execution stalls propagating through the processing pipeline, roll-on/roll-off delay, and difficulty MTU size matching make wavefront parallel processing an unpractical choice for this application.

In addition, tiles has been shown to significantly improve HEVC motion estimation performance for hardware implementations that have a limited amount of on-chip memory [1, 2].

With attention to decoder parallelism, it has been proposed that mandating tiles for resolutions of 1080p and higher would facilitate cost effective design for HEVC decoders for very high-resolution video. For example, a previously verified 1080p HEVC decoder design may be replicated to achieve a 4k decoder with a minimum of additional verification work, thereby lowering development cost and accelerating time to market.

# References

[1] A. Fuldseth, M. Horowitz, A. Segall, M. Zhou, “Tiles”, JCTVC-F335, 6th JCT-VC Meeting, Torino, July, 2011.

[2] M. Zhou, “Sub-picture based raster scanning coding order for HEVC UHD video coding”, JCTVC-B062, 2nd JCT-VC Meeting, Geneva, July, 2010.