| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0575 m26325 |
| --- | --- |

| Title: | On WPP support in the Main profile |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Gordon Clare (Orange Labs), Félix Henry (Orange Labs), Thanos Leontaris (Apple) Stewart Worral (Aspex) Jérôme Vieron (Ateme) Thomas Schierl (HHI) Mickaël Raulet (INSA) Byeongmoon Jeon (LG) Lowell Winger (Magnum Semiconductor) Didier Siron (STMicroelectronics) Pierre Andrivon (Technicolor) Didier Nicholson (Thales Communications and Security) Xavier Ducloux (Thomson Video Networks) | Tel: Email: | Orange +33 2 99 12 45 58 gordon.clare@orange-ftgroup.com felix.henry@orange-ftgroup.com +1-408-862-4495 aleon@apple.com +44 1494 558121 stewart.worrall@aspex-semi.com +33 684637454 j.vieron@ateme.com thomas.schierl@hhi.fraunhofer.de mickael.Raulet@insa-rennes.fr bm.jeon@lge.com lowell.Winger@magnumsemi.com didier.siron@st.com pierre.Andrivon@technicolor.com didier.nicholson@thalesgroup.com xavier.ducloux@thomson-networks.com |
| Source: | Orange Labs, Apple, Aspex, Ateme, HHI, INSA, LG, Magnum Semiconductor , STMicroelectronics, Technicolor, Thales Communications and Security, Thomson Video Networks |  |  |

_____________________________

# Abstract

This informative contribution recalls a number of asserted advantages of WPP for high level parallel processing in HEVC. The authors stress the interest of maintaining this tool in the Main profile.

# Discussion

WPP [1] has been introduced in the standard during the 6th JCTVC meeting and included in the Main profile during the 9th JCTVC meeting. WPP is used for high level parallel encoding or decoding of HEVC bitstreams.

WPP has a significant number of advantages compared to other high-level parallel processing approaches:

- At similar levels of parallelism, WPP has the highest rate-distortion performance [2][3]

- WPP does not break dependencies, and therefore does not produce any visible boundary artifacts [2]

- WPP does not change the usual CTB raster scan order

- The very structure of WPP makes it straightforward to implement in both hardware or software, since all processing steps can be performed at the CTB processing level [4]

- When WPP is activated, this immediately provides a large level of parallelism (up to the number of CTB rows) without requiring negotiating between the encoder and the decoder. [1]

- A non-parallel friendly bitstream can be easily converted to/from WPP, and this has been shown by a software implementation provided to JCTVC experts [6]

- It has been shown by two independent implementations [1][7] that multithreaded implementations of the HEVC decoder can reach significant acceleration factors, and it was also shown that a simple non-normative improvement of WPP allows it to reach state-of-the-art acceleration factors even with large number of cores (x9 with 10 cores on 1600p images and above).

The authors would like to stress that these attractive features are achieved with extremely low impact on decoder complexity, as only a single context copy per line is required to enable them all.

Consequently, the authors support the continued inclusion of WPP in the Main profile.

# Bibliography

[1] JCTVC-F274/m20694 “Wavefront Parallel Processing for HEVC Encoding and Decoding” C. Gordon, F. Henry, S. Pateux, 6th JCTVC meeting, Torino, july 2011

[2] JCTVC-I0198/m24437, “On Tiles and Wavefront tools for parallelism”, J.-M. Thiesse, J. Viéron, 9th JCTVC meeting, Geneva, may 2012

[3]JCTVC-I0153/m24392 “AHG8: Inclusion Of Parallel Processing Schemes In The Main Profile”, S. Worrall, , 9th JCTVC meeting, Geneva, may 2012

[4] JCTVC-I520, “Parallel Scalability and Efficiency of WPP and Tiles”, M. Alvarez-Mesa, V. George, C. C. Chi, B. Juurlink, T. Schierl, , 9th JCTVC meeting, Geneva, may 2012

[5] JCTVC-I0153/m24392, “AHG8: Inclusion Of Parallel Processing Schemes In The Main Profile”, S. Worrall, 9th JCTVC meeting, Geneva, may 2012

[6] JCTVC-J0032 “An HEVC transcoder converting non-parallel bitstreams to/from WPP”, G. Clare, F. Henry, 9th JCTVC meeting, Geneva, may 2012

[7] JCTVC-J0425/m25785 “AHG4: Improving parallelization efficiency of WPP using Overlapped Wavefront”, M. Alvarez-Mesa, V. George, C. C. Chi, B. Juurlink, T. Schierl, , 10th JCTVC meeting, Stockholm, july 2012