| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0576 |
| --- | --- |

| Title: | VPS syntax for scalable and 3D extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce 433 Hackensack Ave Hackensack, NJ 07601 USA | Tel: Email: | jill@vidyo.com |
| Source: | Vidyo, Inc. |  |  |

_____________________________

# Abstract

VPS syntax for the scalable and 3D extensions is proposed.  The proposed syntax is based upon previous contribution JCTVC-I0230.  In the proposed VPS extension, depending upon the extension type, a mapping is provided between layer_id in the NAL unit header, and scalability parameters (dependency_id, quality_id), and/or view scalability parameters (view_id, depth_flag, view_order_idx). In order to accommodate possible future extensions, with backwards compatible layers, additional layer attributes may be signaled for each layer, based upon presence flags.

# Proposed syntax

The proposed VPS syntax for the extension is listed below.

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| video_parameter_set_id | u(4) |
| vps_temporal_id_nesting_flag | u(1) |
| reserved_zero_2bits | u(2) |
| max_num_layers_minus1 //reserved_zero_6bits | u(6) |
| vps_max_sub_layers_minus1 | u(3) |
| profile_level( 1, vps_max_sub_layers_minus1 ) |  |
| next_essential_info_byte_offset //reserved_zero_12bits | u(12) |
| for( i = 0; i <= vps_max_sub_layers_minus1; i++ ) { |  |
| vps_max_dec_pic_buffering[ i ] | ue(v) |
| vps_max_num_reorder_pics[ i ] | ue(v) |
| vps_max_latency_increase[ i ] | ue(v) |
| } |  |
| num_hrd_parameters | ue(v) |
| for( i = 0; i < num_hrd_parameters; i++ ) { |  |
| if( i > 0 ) |  |
| op_point( i ) |  |
| hrd_parameters( i = = 0, vps_max_sub_layers_minus1 ) |  |
| } |  |
| bit_equal_to_one /* equal to 1 */ | u(1) |
| if( max_num_layers_minus1 > 0) { |  |
| extension_type | u(6) |
| if (extension_type == 0 ¦¦ extension_type == 2) |  |
| ViewScalabilityParamsPresent = 1 |  |
| if (extension_type == 1 ¦¦ extension_type == 2) |  |
| ScalabilityParamsPresent = 1 |  |
| if (extension_type > 2) { |  |
| ViewScalabilityParamsPresent = view_scalability_params_present_flag | u(1) |
| ScalabilityParamsPresent = scalability_params_present_flag | u(1) |
| } |  |
| for( j = 0; j < 4; j++ ) |  |
| layer_attribute_present_flag[ j ] | u(1) |
| for( i = 1; i < max_num_layers_minus1; i++ ) { |  |
| if (ViewScalabilityParamsPresent) { |  |
| view_id[ i ] | u(8) |
| depth_flag[ i ] | u(1) |
| view_order_idx[ i ] | u(8) |
| } |  |
| if (ScalabilityParamsPresent) { |  |
| dependency_id[ i ] | u(8) |
| quality_id[ i ] | u(8) |
| } |  |
| for( j = 0; j < 4; j++ ) |  |
| if (layer_attribute_present_flag[ j ]) |  |
| layer_attribute[ i ][ j ]) | u(8) |
| } |  |
| } |  |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

…

# Patent rights declaration(s)

(Remove these parenthetical remarks before submitting: NOTE – Activities in the JCT-VC and contributions to the JCT-VC are subject to the common patent policy for ITU-T/ITU-R/ISO/IEC. A statement of that policy can be found at
http://www.itu.int/ITU-T/dbase/patent/patent-policy.html, with further information available at http://www.itu.int/ITU-T/ipr/index.html and in the ISO/IEC Directives. The form to be used for the formal reporting of patent rights to ITU-T/ITU-R/ISO/IEC can be found at http://www.itu.int/ITU-T/ipr/index.html. Contributions to the JCT-VC proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The provided informal notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form. This obligation to provide an informal notice is supplemental to, and does not replace, any existing obligations of parties with technology included in a final or draft standard to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC. Two examples of such an informal IPR notification statement for a contribution are provided below.)

Vidyo, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).