| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0578 |
| --- | --- |

| Title: | Information on interpolation filters and ALF |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | In Suk Chong, Marta Karczewicz Qualcomm 5775 Morehouse Dr, San Diego, CA 92121, USA | Tel: Email: | In Suk Chong +1-858-658-5673 {ichong, martak} @qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In the current Main profile, motion compensation/estimation requires most of the complexity in the HEVC decoder/encoder, respectively. In the current HEVC design, 8/7 tap interpolation filter is used for motion compensation. 6 tap interpolation filter was tested in the Main profile to test the trade off betweeen complexity and performance; 6tap interpolation has 33% less DRAM access (or bandwidth) and 8 less multiplications. We also tested ALF on top of 6 tap interpolation filter because the additional complexity of ALF is no (or minor) DRAM access and 9 multiplications, which may lead to less complexity than current Main profile. For the sequences with 1080p or higher resolution, 6 tap interpolation filter has 0.5%/1.3% (RA/LB) loss or 0.3%/0.6% (RA/LB) loss without BQTerrace, while 6 tap interpolation with ALF has 3.3%/1.8%(RA/LB) gain compared to current Main profile.

# Introduction

In the current Main profile, motion compensation/estimation requires most of the complexity in the HEVC decoder/encoder, respectively. For example, in [1], motion compensation requires 49% of the decoder time in ARM based implementation. Now 8/7 tap interpolation filter is used in motion compensation of the HEVC design. If we consider 6 tap interpolation filter (in Table 1) instead of current 8/7 tap interpolation filter, there is significant complexity reduction: 33% less DRAM access (or bandwidth) and 8 less multiplications. Considering the fact that DRAM access requires a lot of power implication (i.e., 250 times more than SRAM access [2]), it is not ignorable.

const Short TComInterpolationFilter::m_lumaFilter[4][NTAPS_LUMA] =

{

{  0, 0,   0, 64,  0,   0, 0,  0 },

{  0, 2,  -8, 56, 18,  -6, 2,  0 },

{  0, 2, -10, 40, 40, -10, 2,  0 },

{  0, 2,  -6, 18, 56,  -8, 2,  0 },

};

Table 1. 6 tap DCTIF interpolation filter used

Keeping that in mind, the additional complexity of ALF is no (or minor) DRAM access and 9 multiplications. Therefore, in this informational proposal, we teseted 6 tap interpolation filter as well as 6 tap interpolation filter + ALF, which may have less complexity than current Main profile.

# Simulation results and conclusion

Experiments were conducted for Main-RA/LB conditions. For the sequences with 1080p or higher resolution, 6 tap interpolation filter has 0.5%/1.3% (RA/LB) loss or 0.3%/0.6% (RA/LB) loss without BQTerrace, while 6 tap interpolation + ALF has 3.3%/1.8%(RA/LB) gain compared to current Main profile.

|  | Random Access Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.3% | -0.5% | -0.3% |
| Class B | 0.8% | 0.1% | 0.1% |
| Class C | 1.9% | 0.7% | 0.6% |
| Class D | 4.1% | 1.8% | 1.8% |
| Class E |  |  |  |
| Class F | 0.3% | 0.1% | 0.1% |
| Overall | 1.5% | 0.4% | 0.5% |
|  | 1.5% | 0.4% | 0.5% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | #NUM! |  |  |
|  | Low delay B Main |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 1.3% | 0.1% | 0.0% |
| Class C | 2.3% | 0.7% | 0.6% |
| Class D | 4.0% | 2.1% | 2.1% |
| Class E | 1.3% | 0.8% | 0.5% |
| Class F | 0.6% | 0.0% | 0.2% |
| Overall | 1.9% | 0.7% | 0.7% |
|  | 1.9% | 0.7% | 0.6% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | #NUM! |  |  |

Table 1. Results with 6 tap

|  | Random Access Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.3% | -0.5% | -0.3% |
| Class B | 0.3% | 0.1% | 0.0% |
| Class C | 1.9% | 0.7% | 0.6% |
| Class D | 4.1% | 1.8% | 1.8% |
| Class E |  |  |  |
| Overall | 1.6% | 0.5% | 0.5% |
|  | 1.6% | 0.5% | 0.5% |
| Class F | 0.3% | 0.1% | 0.1% |
| Enc Time[%] | 104% |  |  |
| Dec Time[%] | #NUM! |  |  |
|  | Low delay B Main |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.6% | 0.1% | 0.1% |
| Class C | 2.3% | 0.7% | 0.6% |
| Class D | 4.0% | 2.1% | 2.1% |
| Class E | 1.3% | 0.8% | 0.5% |
| Overall | 2.0% | 0.9% | 0.8% |
|  | 2.0% | 0.9% | 0.8% |
| Class F | 0.6% | 0.0% | 0.2% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | #NUM! |  |  |

Table 2. Results with 6 tap (BQTerrace)

|  | Random Access Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -4.3% | -3.5% | -4.3% |
| Class B | -2.4% | -2.9% | -1.6% |
| Class C | -0.7% | -1.2% | -1.5% |
| Class D | -0.6% | -0.1% | -0.3% |
| Class E |  |  |  |
| Class F | -0.8% | -1.1% | -1.1% |
| Overall | -1.8% | -1.8% | -1.7% |
|  | -1.8% | -1.8% | -1.8% |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | #NUM! |  |  |
|  | Low delay B Main |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -1.8% | -3.0% | -2.4% |
| Class C | -0.9% | -2.3% | -2.0% |
| Class D | 0.2% | 0.3% | -0.8% |
| Class E | -2.0% | -6.1% | -6.7% |
| Class F | -2.3% | -2.5% | -2.1% |
| Overall | -1.3% | -2.6% | -2.6% |
|  | -1.3% | -2.5% | -2.6% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | #NUM! |  |  |

Table 3. Results with 6 tap and ALF

# References

[1] F. Bossen, “On software complexity: decoding 720p content on a tablet”, JCTVC-J0128

[2] C.-Y. Chen, C.-Y. Tsai, Y.-C. Chang, C.-Y. Cheng, F.-C. Yeh, H.-W. Li, T.-D. Chuang, Y.-W. Huang, S. Lei, “AHG6: Comparison between ALF and bi-prediction MC in low-delay conditions”,JCTVC-J0049