| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0579-r1 |
| --- | --- |

| Title: | BoG report on miscellaneous limits |  |  |
| --- | --- | --- | --- |
| Status: | [Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Keiichi Chono Minhua Zhou | Email: | chono@ct.jp.nec.com zhou@ti.com |
| Source: | BoG on miscellaneous limits |  |  |

_____________________________

Recommendations

BoG recommends that two unrelated topics, “2 table level definitions” and “max bitrates” in JCTVC-J0151, are discussed in the main session.

BoG recommends adopting the 16-bit range constraint for both horizontal and vertical MVs. (JCTVC-J0225 and JCTVC-J0335.)

BoG recommends considering the DPB-size reduction (JCTVC-J01519) in the main session.

BoG recommends discussing the following constraints in the main session:

- Num_ref_idx constraint (JCTVC-J0151)

- Bounds on syntax elements excluding MV and DMV (JCTVC-J0335 and JCTVC-J0225)

- Slice/WPP/Tile number limit for specify the maximum number of CABAC reset operations  (JCTVC-J0082)

  - Issue for a Tile case had been addressed by the other BoG.

  - Discuss the necessity of a constraint for a WPP case.

  - Discuss a unified constraint for slice and WPP in terms of maximum number of CABAC reset operations.

- APS limit

- Bit/bin constraint at LCU level (JCTVC-J0059, JCTVC-J0080, and JCTVC-J0151).

- J0059: Propose to have a limit in terms of either of bits or bins

- J0082: Propose to have a limit in terms of bits

- J0151: Propose to have no level limit

# Document review and discussion notes

JCTVC-J0151 Study on HEVC profiles and levels [T. Suzuki, M. Haque, A. Tabatabai (Sony)]

Proposes to change the current definition of profile and level.

- DPB size (decrease, but perhaps support more pictures when pic size gets small)

- Num_ref_idx (decrease limit on ref pic list length – e.g. 8)

- 2 table level definitions (i.e. "tiers")

- Max bitrates

- No constraint on max bit/bin at LCU level

[DPB size discussion]

The DPB-size proposal is to reduce the physical DBP size from 6 to 5. (The physical DPB size equal to 6, for example, means that 6 times of the max picture size in each Level.) JCTVC-J0163 offers additional test results related to this proposal and shows that the physical DBP size of 5 enables the hierarchical GOP 8 in common test conditions. JCTVC-J0417 is the cross-check of JCTVC-J0163.

Non-proponent asked the relationship with other proposals discussed in the main session on the number of reference pictures. The proponent clarified that the purpose of the proposal is to reduce the physical DBP size not to change the number of reference pictures.

The cross-checker expressed a favor of the physical DBP size reduction.

BoG recommends considering the DPB-size reduction in the main session.

[Num_ref_idx limit]

The proposal is to reduce the maximum values of num_ref_idx_lx_default_active_minus1 in PPS and num_ref_idx_lx_active_minus1 in Slice header. It is proposed change the current maximum value of 15 to 7.

Question: There was discussion in the main session that the number of reference pictures is not be more than 8. What is the intent of the proposal?

Purpose is to reduce the maitainnance of reference picture pointers (i.e., reduce the number of mappings between physical reference picures and reference picture indices).

Some experts expressed a favour of reducing the maximum numbers but other experts expressed a favour of keeping the current maximum numbers.

BoG recommends discussing the num_ref_idx limits in the main session.

[2 table level definitions and Max bitrate]

These items were identified as non “miscellaneous limits”. It was agreed that these items are discussed in the main session.

[No constraint on max bit/bin at LCU level]

It was agreed that the discussion is deferred until the review of the related other proposals.

JCTVC-J0335 Explicit Specification of Bounds on Syntax Elements [B. Heng, T. Hellman, W. Wan (Broadcom)]

The contribution proposes to specify the ranges of some syntax elements and MV ranges by changing the semantics. JCTVC-J0133 raises the same issue on the range of RQT related syntax elements. JCTVC-J0225 raises the related issues on the MV range. MV range issues was discussed in MV range category.

Text was provided and is being reviewed by the editors.

The discussion notes are appended the most right column in the following table.

“Suggestion” and “Revisit” items should be discussed in the main session.

| Item | Syntax Element | Type | Min Value | Max Value | Proposed Min | Proposed Max | Notes | BoG Discussion Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | max_transform_ hierarchy_depth_inter | ue(v) | 0 | ?? | 0 | Log2MaxCtbSize – Log2MinTrafoSize |  | No objection. |
| 2 | max_transform_ hierarchy_depth_intra | ue(v) | 0 | ?? | 0 | Log2MaxCtbSize – Log2MinTrafoSize |  | No objection. |
| 3 | cb_qp_offset | se(v) | ?? | ?? | -12 | 12 | Consistent with AVC | This aspect was adopted as in J0342. |
| 4 | cr_qp_offset | se(v) | ?? | ?? | -12 | 12 | Consistent with AVC | This aspect was adopted as in J0342. |
| 5 | column_width | ue(v) | ?? | ?? | 1 | Indirectly bound by requiring sum of column widths adds up to picture width |  | No objection on the idea. Suggestions: - Consider the use of “column_width _minus1”. - Consider the use of u(v). |
| 6 | row_height | ue(v) | ?? | ?? | 1 | Indirectly bound by requiring sum of row heights adds up to picture height |  | No objection on the idea. Suggestions: - Consider the use of “column_width _minus1”. - Consider the use of u(v). |
| 7 | beta_offset_div2 | se(v) | ?? | ?? | -6 | 6 | Consistent with AVC | No objection. |
| 8 | tc_offset_div2 | se(v) | ?? | ?? | -6 | 6 | Consistent with AVC | No objection. |
| 9 | scaling_list_pred_ matrix_id_delta | ue(v) | 0 | ?? | 0 | Indirectly bound by requiring RefMatrixID >= 0 |  | No objection. It was suggested to refer the text for the corrected description. The correction was confirmed during the BoG meeting. |
| 10 | scaling_list_dc_ coef_minus8 | se(v) | ?? | ?? | -8 | 247 | Prevents DC from going outside 0-255 range | Revisit (Suzuki-san checks this.) Comments: What is the original intention of the range (8-255 is intended)? Suggestions: - Consider the use of ue(v) instead of se(v). |
| 11 | scaling_list_ delta_coef | se(v) | ?? | ?? | -128 | 127 | Consistent with AVC | Revisit (Suzuki-san checks this.) Comments: (1) What is the appropriate range? (2) Need to check whether the maximum value is equal to 255. (3) Need to check whether the reconstructed value is computed with modulo. |
| 12 | alf_start_ second_filter | ue(v) | ?? | ?? | 1 | 15 | Suggest to remove. Unclear whether using only two filters needs special signaling | No objection on the idea. Suggestions: - Discuss the necessity of this syntax element. - If needed, specify the range. |
| 13 | five_minus_max_ num_merge_cand | ue(v) | 0 | 5? | 0 | 4 | 5 implies zero merge candidates which is unclear | The issue had been solved. |
| 14 | offset_len _minus1 | ue(v) | ?? | ?? | 0 | 31 |  | No objection It was confirmed that 32-bit (4GB) is sufficient to signal the offset even in 8Kx4K. |
| 15 | delta_idx _minus1 | ue(v) | 0 | ?? | 0 | Indirectly bound by requiring RIdx >= 0 |  | Revisit Suggestions: - The minimum value is equal to 0. - The Maximum value of Ridx should be checked and specified. (The admissible reconstructed value is up to 15; it should be less than or equal to active_num_ref_idx. |
| 16 | abs_mvd _minus2 | EGk(v) | 0 | ?? | 0 | Indirectly bound by requiring -both mvd_x and mvd_y be in the range [-215, 215-1] | Also need to bound motion vector (mvLX and mvLY) to range [-215, 215-1] | See the notes of J0225. Opinions: - 16—bit for MV and 17 -bit DMV. - 15—bit for MV and 16 -bit DMV. - No constraint on DMV. - 16-bits for both DMV and MV. |

[MV range limit]

JCTVC-J0225 Restrictions to the maximum motion vector range [Alistair Goudie (Imagination Technologies)]

The contribution proposes both non-normative encoder restriction and normative clipping operation in MV derivation processes.

Quenstion1: 16-bit is sufficient for horizontal MV?

- Sufficient. Keep 16-bit.

Quenstion2: Do we consider a constraint based on picture width?

- No action is needed.

  - It was remarked that w/o constraint, decoder can handle it by using a clipping.

Question3: 16-bit is needed for horizontal MV?

- Keep 16-bit.

  - It was remarked that 13-bit is sufficient for vertical MVs.

  - It was remarked that 16-bit is needed.

  - It was remarked that both 13-bit and 16-bit do not change decoder requirements.

Question4: Do we need a normative clipping operation in MV derivation processes?

- No consensus.

  - It was remarked that there is clipping in MV scaling process. Having a clipping in MV derivation process would be consistent with the MV scaling process.

  - It was remarked that additional clipping is burden for decoders and unnecessary. The problem can be handled by non-normative ways.

Summary of BoG recommendation on the MV range issues:

- Specify 16-bit range for both horizontal and vertical MVs. (The author of JCTVC-J0335 will provide the text.)

[Slice/WPP/Tile number limit]

JCTVC-J0080 On number of slices per picture limit [M. Zhou (TI)

In HM7.0 it is a CABAC reset mandated at the end of each LCU row when WPP is in use and the end of each tile when tiles are enabled. Due to CABAC reset, a WPP or a tile has approximately same high-level decoder overhead as an entropy slice. However, in the current specification of number of slices per picture limit, only regular slices, dependent slices and entropy slices are counted. Therefore, it is recommended to include both number of WPPs and number of tiles into the number of slices per picture limit. Note that this contribution does not request to increase the current limit of maximum number of slices allowed for a picture at different levels.

The contribution proposes to include both number of WPPs and number of tiles into the number of slices per picture limit in order to limit the maximum number of CABAC reset.

It was noted by the proponent that there was interaction with Tile # constraint suggested by BoG on high-level parallel processing and the issue for a Tile case had been addressed. The remaining issue is for a WPP case. It was agreed that the issue is discussed in the main session because of the absence of WPP experts.

Discussion notes.

- Issue: For example. 68 times of CABAC-resets are required for each picture of a 1080 video sequence when WPP and 16x16 LCU are used, whereas 65 times of CABAC-resets are specified by the maximum limit in slice numbers. Since it is expected that the complexities of CABAC reset operations for WPP and slices are comparable, some limit should be defined for the WPP case.

- The proponent requests to discusses the following items in the main session: (1) The necessity of some constraint for WPP. (2) Discuss a unified constraint for slice and WPP in terms of maximum number of CABAC reset operations.

[APS limit]

There is no contribution on APS limit. The issue (the maximum number of APS IDs) was raised by Minhua during BoG on high level syntax. It was agreed that the issue is discussed in the main session because of the absence of APS experts.

Discussion notes.

- It was confirmed by Yu-Wen that slices within a picture is not allowed to change APS IDs in the current spec.

- It was noted that 64 APSs take about 10KB.

- One expert suggested 16 or 32 would be a reasonable limit.

- It was noted that the HM uses up to 8 APSs.

- One expert suggests 16 (2.5KByte) or 32 (5KByte).

- Software uses APSs up to 8.

[LCU bit/bin limit]
There was no time to review the related contributions. JCTVC-J0442 summarizes the related input contributions as follows:

- J0059: Propose to have a limit in terms of either of bits or bins (Text is also proposed)

- J0082: Propose to have a limit in terms of bits (Text is also proposed)

- J0151: Propose to have no level limit

JCTVC-J0442 Summary of discussion and input contributions on constraint on number of bits/bins per LCU

JCTVC-J0059 Suggested constraint on number of bits/bins per LCU [K. Chono (NEC)]

JCTVC-J0082 On number of bits per LCU limit [M. Zhou, V. Sze (TI)]