| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J1101_r2 |
| --- | --- |

| Title: | Description of Core Experiment 1: Deblocking filter |  |  |
| --- | --- | --- | --- |
| Status: | Output document of JCT-VC |  |  |
| Purpose: | CE description |  |  |
| Author(s) or Contact(s): | Andrey Norkin (Primary) Teruhiko Suzuki Madhukar Budagavi Geert Van der Auwera | Email: | andrey.norkin@ericsson.com teruhikos@jp.sony.com madhukar@ti.com geertv@qualcomm.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document is a description of Core Experiment 1 on Deblocking filter.

# Introduction

During the 10th JCT-VC meeting in Stockholm, a subjective viewing of the deblocking proposals has been conducted [1]. Based on the results of the subjective test, it has been decided to further study separate components of proposals JCTVC-J0181 [2] and JCTVC-J0286 [3] and their combinations in the CE. It has been proposed to study performance of the proposals on critical sequences (i.e. showing blocking artifacts in the default operating mode) as well as studying their performance on sequences from the common test conditions [6]. The proposal JCTVC-J0181 is extending the current adaptability range of the deblocking filter and proposal JCTVC-J0286 is addressing block artifacts removal in the default mode of operation.

# Participants

| Nr. | Name | Company | Email | Remarks |
| --- | --- | --- | --- | --- |
| 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 20 21 22 23 24 25 26 | Andrey Norkin (primary coordinator) Teruhiko Suzuki (coordinator) Masaru Ikeda Geert Van der Auwera (coordinator) Madhukar Budagavi (coordinator) Ohji Nakagami Shuo Lu Ankur Saxena Do-Kyoung Kwon Polin Lai Frank Bossen Elena Alshina Peisong Chen T.K. Tan Chih-Wei Hsu Jicheng An Yu-Wen Huang Matthias Narroschke Semih Esenlik Hui Yong Kim Kemal Ugur Sung-Chang Lim Kazuo Sugimoto Minhua Zhou Gaëlle Martin-Cocher Dake He | Ericsson Sony Sony Qualcomm Texas Instruments Sony Sony Samsung TI Samsung NTT Docomo Samsung Broadcom NTT Docomo MediaTek MediaTek MediaTek Panasonic Panasonic ETRI Nokia ETRI Mitsubishi TI RIM RIM | andrey.norkin@ericsson.com teruhikos@jp.sony.com masaru.ikeda@jp.sony.com geertv@qualcomm.com madhukar@ti.com ohji.nakagami@jp.sony.com shuo.lu@jp.sony.com asaxena@sta.samsung.com d-kwon@ti.com wlai@sta.samsung.com bossen@docomoinnovations.com elena.alshina@gmail.com elena_a.alshina@samsung.com peisongc@broadcom.com tktan@m-sph.com cw.hsu@mediatek.com jicheng.an@mediatek.com yuwen.huang@mediatek.com Matthias.Narroschke@eu.panasonic.com Semih.Esenlik@eu.panasonic.com hykim5@etri.re.kr Kemal.Ugur@nokia.com sclim@etri.re.kr Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp zhou@ti.com gmartincocher@rim.com dhe@rim.com | Proponent Proponent Proponent Proponent Proponent Proponent |

# Experimental Conditions

  - Software

HM 8.0 shall be used (provided on August 8, 2012).

  - Test Sequences, Bit Rates and Coding Conditions

The recommended sequences, configurations and test cases described in JCTVC-J1100 will be used. Results for all mandatory test cases and for HE10 LD-P and Main LD-P configurations shall be provided. Providing results for additional sequences is encouraged but not mandated.

Results for additional test sequences shall also be provided for LD main and RA main configurations. The additional sequences are "Riverbed", "West Winds Easy", "Ducks Take Off", "Red Kayak".

T. Suzuki will make the sequences available so that the identical versions were used.

  - Evaluation of CE Results

Results of the CE will be evaluated on the basis of BD-measures, complexity, and subjective quality. Complexity assessment is performed on the base of decoding complexity measurement.

### Complexity assessment

Both HW and SW complexity of proposed tools for both encoder and decoder will be evaluated.

To measure software run time, HM8.0 software with the anchor configuration and the proposal implemented on the HM8.0 software shall be used. To measure decoding time, the decoding shall be executed on the same machine without output of decoded YUV data. The computational time must be measured for each test sequence and test case for both anchor and proposal. Relative computational time calculated against the anchor must be presented.

### Subjective tests at the meeting

An informal subjective viewing by experts during the Shanghai meeting should be performed and results provided during the meeting in order to ensure tested proposals do not cause degradation of subjective quality on common test condition sequences and improve subjective quality on the additional sequences. Test sequences should be determined before the beginning of cross-check. Proponents are not allowed to participate in the test. It is recommended that the number of test subjects is over 20.

The following test procedure will be used. ABAB test is used. The same sequence and the anchor are shown one after another. The order of proposals is randomized for every test and sequence and identities of the proposals are hidden. Test subjects are asked to rate each proposal on a scale from -2 to 2. The results are later calculated as well as 95-percent confidence intervals for every proposal and test sequence.

### ABAB test with anchor as HM8.0 + signalled maximum tC / beta offsets

Proposals use signalling of deblocking parameters to modify tC and beta thresholds. The deblocking parameters are preferably set at the same level (maximum) for all test sequences. Type 1 sequences as defined in Section 3.3.7 are used.

### ABAB test with anchor as HM8.0 (common test conditions)

Proposals do not use modification of deblocking parameters. Type 1 and Type 2 sequences sequences as defined in Section 3.3.7 are used.

### ABAB test with anchor as HM8.0 + signalled maximum tC / beta offsets + max TU size restricted to 16x16

Proposals use signalling of deblocking parameters to modify tC and beta thresholds. The deblocking parameters are preferably set at the same level (maximum) for all test sequences and the maximum TU size is set to 16x16. Type 1 sequences as defined in Section 3.3.7 are used.

### ABAB test with anchor as HM8.0 (common test conditions) + max TU size restricted to 16x16

Proposals do not use modification of deblocking parameters. Type 1 and Type 2 sequences as defined in Section 3.3.7are used.

### Sequences to be used in subjective test

Type 1 sequences: These sequences will be used to demonstrate improvement in visual quality: "Riverbed", "West Winds Easy", "Ducks Take Off 25fps", "China Speed", "Red Kayak". Additional sequences may be added before the cross-check if the content can demonstrate blocking artifacts.

QP values: 32, 37, 41

Anchor1: HM-8.0 with max beta_offset and tc_offset .

Anchor2: HM-8.0 (CTC)

The sequences are available at the following URL: http://sdrv.ms/MPDyGd

Type 2 sequences: These sequences will be used to demonstrate that there is no visual degradation in common test conditions: "Kimono", "Cactus", "BasketballDrillText", "BQMall", and "Jonny". The selection of sequences may be modified if the content shows visual quality degradation from deblocking.

QP values:, 32, 37 (revisit).

Anchor: HM-8.0 (CTC).

Note 1: Two anchors shall be used.

HM8.0 shall be used as the anchor.

HM8.0 with tc_offset_div2 = 6 and beta_offset_div2 = 6 shall be used as the second anchor.

Note 2: Cross-checking includes informal viewing of proposals. All cross-checkers should report the informal viewing results (without scoring) and their equipment (e.g., monitor, etc). If a room, equipment, and time-slot for informal viewing are available at the Shanghai meeting, informal viewing will be conducted. The test sequences, QP values, and coding conditions for the potential informal viewing at the Shanghai meeting are chosen from the sequences, configurations and test cases described in JCTVC-J1100 and additional sequences described in this document. The proponents should be ready to provide the requested sequences. The selection should be conducted by CE1 participants including non-proponents.

Note 3: The proponents are encouraged to provide additional results for separate tools (groups of tools) in their proposals if the proposal consists of several independent modifications of the HM software. The possibility to turn on and off these groups of tools should be provided in the software. This specifically applies to adjustments of the thresholds and tables as well as to modifications to the encoder control and implementations, which could also be applied to improve the anchor. The cross-checkers should have a possibility to assess, which part of the coding gain comes from the thresholds/tables adjustment or from modifications to the encoder control or implementations. The cross-checkers are encouraged to perform the subjective viewing of the separate tools/combinations of tools in order to ensure that removing or adding certain tools from the proposal does not degrade the subjective quality.

# Tools to test

- a 	Beta table modification (JCTVC-J0181, S. Lu, O. Nakagami, M. Ikeda, T. Suzuki (Sony))

It is proposed to replace the β values with Table 2-2 [2]. The changes are highlighted in red. The value is determined by the principle that the changing rate of β should have similar trend with the changing rate of tc to reflect quantization change. Since the changes lie at QP bigger than 41, there is no impact on common test conditions (provided the QP values in CTC do not change).

## AHG6: Reduction of block artifacts in HEVC for large blocks (JCTVC-J0286, A. Norkin (Ericsson))

The proposal consists of three main parts.

### b.1 	Removing intra-boundary smoothing for larger block sizes

It is proposed to remove the intra-boundary smoothing (applied in HEVC for DC, vertical and horizontal prediction modes) from the larger intra-block sizes (in the proposal, 32x32) [3]. This reportedly enables to avoid forming irregularities on left and/or top boundaries of intra-blocks.

### b.2.1 	Modification of strong filter decisions

The strong filter decisions are modified to enable application of the HEVC strong filter on inclined surfaces (current HEVC strong filter is applied to the flat surfaces) [3].

### b.2.2 	Modification of strong filter

Modification of strong filter. The strong filter is modified to match strong filter decisions from b.2.2 so that it avoids modifying an inclined surface when applied to it [3].

### b.3.1 	Boundary strength modification for CU

The derivation of the boundary strength is modified in a way that for the edges that have boundary strength Bs = 2, Bs can be increased to 3 in case of CU larger or equal to 32x32 when the block boundary is on the 32x32 filtering grid [3].

### b.3.2 	Tc_offset modification

Alternatively to b.3.1, the tc_offset for intra-block boundaries is increased from 2 to 4 (which would result in the same effect as in the previous paragraph but for all intra-block boundaries) [3].

## AHG6: Reduction of block artifacts in HEVC for large blocks (JCTVC-J0090, G. Van der Auwera, R. Joshi, M. Karczewicz (Qualcomm))

### b.3.3 	Boundary strength modification for max TU

Alternatively to b.3.2 and b.3.1, Bs is increased from 2 to 3 at the intra TU boundary when one of the TUs has a maximum TU size. The procedure for determining TU boundary for the maximum TU size is described in [4].

# Space of possible test combinations

This section represents space of possible variants to be tested.

## Beta table modification

- Modified beta table

- Current HM beta table

## Restricting intra-boundary smoothing

- Intra-boundary smoothing is enabled for all block sizes

- Intra-boundary smoothing is enabled for block sizes below certain size (e.g. 32x32 or 16x16)

- Intra-boundary smoothing is disabled for all block sizes

## Strong filter and its decisions

- Current strong filter and decisions

- Proposed strong filter and decisions

- Possible additional tests: 1. new strong filter decision + old strong filter, 2. old strong filter decision + new strong filter

## Bs modification

- Bs modification for 32x32 sample grid, on 32x32 CU and 64x64 CU.

- Bs modification for TU boundary at TU of maximum size

- tc_offset is increased with the same value for all intra block boundaries

# Combinations tested in CE

The following tests for combinations of tools are performed in the CE

Table 1 Combinations tested in CE

| Test No (proponent) | BetaTab a | Strong Filter &Decisions b.2.1 & b.2.2 | Bs Modification b.3.1 | Bs = 2 => tc_offset = X b.3.2 | Bs Mod. b.3.3 | IntraSmoothing Restriction b.1 |
| --- | --- | --- | --- | --- | --- | --- |
| 1 (Sony) | x |  |  |  |  |  |
| 2 (Ericsson) |  | x | x |  |  | x |
| 3 (Sony) | x | x | x |  |  | x |
| 4 (Ericsson) |  | x |  | x |  | x |
| 5 (Sony) | x | x |  | x |  | x |
| 6 (Sony) |  |  |  |  |  | x |
| 7 (Ericsson) |  | x | x |  |  |  |
| 8 (Ericsson) |  | x |  |  | x | x |
| 9 (Ericsson) | x | x |  |  | x | x |

Table 2 Cross-verification assignment

| Test/proponent | Proponent | Cross-checker(s) |
| --- | --- | --- |
| - Beta Table Modification | Sony | Qualcomm |
| - New Strong Filter + Decisions + Bs Modification + Intra Smoothing Restricted | Ericsson | Samsung |
| - Beta Table Modification + New Strong Filter + Decisions + Bs Modification + Intra Smoothing Restricted | Sony | Ericsson |
| - New Strong Filter + Decisions + Intra Smoothing Restricted + Intra Tc_offset = X | Ericsson | Broadcom |
| - Beta Table Modification + New Strong Filter + Decisions + Intra Smoothing Restricted + Intra Tc_offset = X | Sony | TI |
| - Intra Smoothing Restricted | Sony | Samsung |
| - New Strong Filter + Decisions + Bs Modification | Ericsson | Sony |
| - New Strong Filter + Decisions + Bs Modification (b.3.3) + Intra Smoothing Restricted | Ericsson | TI, Qualcomm |
| - Beta Table Modification + New Strong Filter + Decisions + Bs Modification (b.3.3) + Intra Smoothing Restricted | Ericsson | TI, Qualcomm |

# Time-line and Responsibilities

2012-Aug-10:	CE1 description finalized and uploaded.

2012-Aug-10:	Additional sequences uploaded.

2012-Aug-13:	HM8.0 software is available (by plan).

2012-Aug-27:	Cross-verification begins, proponents provide software, draft of contribution text and results to cross-verifiers(s) and CE1 coordinators (2 weeks after planned availability of HM8.0).

2012-Sept-10:	Cross-verifiers report results to CE participants (2 weeks from cross-check start)

2012-Oct-01:	Contribution documents uploaded, the document number and title are reported to coordinators.

2012-Oct-05:	Summary report uploaded

# References

- T. Suzuki, "BoG Report: Report of subjective test on extended adaptability range of deblocking filter", JCTVC-J0567, Stockholm, Sweden, July 2012.

- S. Lu, O. Nakagami, M. Ikeda, T. Suzuki, "AHG6: On deblocking filter parameters", JCTVC-J0181, Stockholm, Sweden, July 2012.

- A. Norkin, "AHG6: Reduction of block artifacts in HEVC for large blocks", JCTVC-J0286, Stockholm, Sweden, July 2012.

- G. Van der Auwera, R. Joshi, M. Karczewicz, "AHG6: Transform Dependent Deblocking Strength", JCTVC-J0090, Stockholm, Sweden, July 2012.

- "High Efficiency Video Coding (HEVC) text specification draft 7", JCTVC-I1003, Geneva, CH, May 2012.

- F. Bossen, "Common HM test conditions and software reference configurations", JCTVC-I1100, Geneva, CH, May 2012.