| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0009 |
| --- | --- |

| Title: | JCT-VC AHG report: High-level syntax (AHG9) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang 5775 Morehouse Dr San Diego, CA 92121 USA Gary J. Sullivan One Microsoft Way Redmond WA 98052-6399 USA | Tel: Email: | 1-858-651-8345 yekuiw@qti.qualcomm.com +1-425-703-5308 garysull@microsoft.com |
| Source: | High-level syntax AHG (AHG9) |  |  |

_____________________________

# Abstract

This report summarizes the activities of the high-level syntax ad hoc group (AHG9) between the 10th and 11th JCT-VC meetings, and lists the related input documents submitted to this meeting according to their topics.

A face-to-face AHG meeting was held from 9:00 to 18:00 on October 9, 2012, at the same meeting venue as the 11th JCT-VC meeting. Meeting minutes and AHG recommendations made during the face-to-face meeting are included in the report.

# Mandates

- Study NAL unit header, video parameter set, sequence parameter set, picture parameter set, and slice header syntax designs.

- Study SEI messages and VUI syntax designs, including checking and fixing texts for SEI messages currently included by referring to the AVC specification.

- Study the hypothetical reference decoder (HRD) syntax and operations and the related text for bitstream conformance and decoder conformance.

- Work towards simplification and general minor cleanup of the high-level syntax.

- Assist in software development and text drafting for the high-level syntax in the HEVC design.

# Reflector discussions

There was a discussion on the relationship of profile, tier and level information included in the active VPS and the active SPS for the base layer.

Currently, it is stated in a note in HEVC draft 8 that the maximum level signalled in the video parameter set may be higher than the level signalled for a coded video sequence in the sequence parameter set. There was a suggestion made by Mathias Wien to explicitly require the profile_tier_level() syntax structures present in both the VPS and the SPS to be identical for a coded video sequence, and a note might be added to explain the envisioned usage in the potential scalable and/or 3DV extensions.

There were some followup discussions between Jill Boyce, Ye-Kui Wang, and Ajay Luthra on this syntax hierarchy relationship. Luthra and Wang generally discussed the relationship between system-layer and video-layer functionalities in various system environments. No conclusion was made.

The AHG requests the JCT-VC to consider and resolve the above issue at this meeting.

# Related contributions

Input documents related to the mandates of this AHG and registered by October 6 are allocated to different topics as follows, wherein the numbers indicates the numbers of documents of the corresponding topic categories (cross-check documents were not counted in these numbers, and altogether there are 89 related input documents):

- NAL unit header (3)

- Random access and adaptation (6)

- Slices and slice header parameters (17)

  - Slices (7)

  - Picture order count (POC) (3)

  - Reference picture set (RPS) (3)

  - Reference picture list (RPL) (4)

- Parameter sets in HEVC version 1 (6)

- High-level syntax clean-ups (9)

- High-level parallelism (7)

- Hypothetical reference decoder (4)

- VUI and SEI (16)

  - VUI (1)

  - Frame-packed video (4)

  - Region-of-interest (ROI) signalling (4)

  - Misc. SEI messages (7)

- Parameter sets for extensions (11)

- Profile and level (6)

  - Profile (2)

  - Level constraints (4)

- Interlaced coding high-level (3)

- Transform skipping high-level (1)

The list of document is as follows, sorted first according to agenda item and second by JCT-VC number.

| JCT-VC number | Category | Agenda item | Title | Authors |
| --- | --- | --- | --- | --- |
| JCTVC-K0121 | NUH | A.I.01 | AHG9: NAL unit header design for support of multi-standard extensions | Y. Chen, Y.-K. Wang (Qualcomm) |
| JCTVC-K0177 | NUH | A.I.01 | AHG9: NAL unit header with layer ID partitioning | B. Choi, J. Park (Samsung), Y.-K. Wang, Y. Chen (Qualcomm) |
| JCTVC-K0210 | NUH | A.I.01 | AHG9: NAL unit header design for base spec | B. Choi, T. Lee, C. Kim, J. Park (Samsung) |
| JCTVC-K0122 | RA & adaptation | A.I.02 | AHG9: On CRA and BLA pictures | Y.-K. Wang (Qualcomm) |
| JCTVC-K0159 | RA & adaptation | A.I.02 | AHG 9: On referenced and non-referenced DLP and TFD NAL units | Hendry, Y. Jeon, B. Jeon (LG) |
| JCTVC-K0164 | RA & adaptation | A.I.02 | AHG 9: On non-referenced TSA and STSA NAL units | Hendry, B. Jeon (LG) |
| JCTVC-K0166 | RA & adaptation | A.I.02 | AHG 9: On random access point NAL units | Hendry, B. Jeon (LG) |
| JCTVC-K0208 | RA & adaptation | A.I.02 | AHG9: On random access point pictures | B. Choi, T. Lee, Y. Park, J. Park (Samsung) |
| JCTVC-K0219 | RA & adaptation | A.I.02 | On Random Access Point Picture Signaling | S. Deshpande (Sharp) |
| JCTVC-K0113 | Slices | A.I.03.1 | The general structure and syntax of the slice extension | David Singer, Thanos Leontaris, Alexis Tourapis |
| JCTVC-K0114 | Slices | A.I.03.1 | Specific Uses of the Slice Extension | David Singer, Thanos Leontaris, Alexis Tourapis |
| JCTVC-K0167 | Slices | A.I.03.1 | AHG 9: On dependent slice | Hendry, B. Jeon (LG) |
| JCTVC-K0168 | Slices | A.I.03.1 | AHG 9: Short Slice Header | Hendry, B. Jeon (LG) |
| JCTVC-K0184 | Slices | A.I.03.1 | AHG9: On dependent slices syntax | S. Esenlik, M. Narroschke, T. Wedi (Panasonic) |
| JCTVC-K0288 | Slice | A.I.03.1 | Comments on Entropy Slices | W. Wan, B. Heng (Broadcom) |
| JCTVC-K0314 | Slice | A.I.03.1 | Comments on Dependent Slices | W. Wan, B. Heng, P. Chen (Broadcom) |
| JCTVC-K0143 | POC | A.I.03.2 | On handling of minimum POC range at decoder | K. Kazui, J. Koyama, S. Shimada, A. Nakagawa (Fujitsu) |
| JCTVC-K0154 | POC | A.I.03.2 | Simplification of PicOrderCntMsb calculation and specification | C. Auyeung, J. Xu, A. Tabatabai |
| JCTVC-K0227 | POC | A.I.03.2 | On POC Signalling | Xiaosong(Steve) Zhou, Dazhong Zhang, Yunfei Zheng, Jae Hoon Kim (Apple) |
| JCTVC-K0123 | RPS | A.I.03.3 | AHG9: Reference picture set clean-ups | A. K. Ramasubramonian, Y.-K. Wang (Qualcomm) |
| JCTVC-K0222 | RPS | A.I.03.3 | On Reference Picture Set Extension | S. Deshpande (Sharp) |
| JCTVC-K0235 | RPS | A.I.03.3 | Single inclusion of long-term reference pictures in RPS | J. Samuelsson, R. Sjöberg (Ericsson) |
| JCTVC-K0124 | RPL | A.I.03.4 | AHG9: Reference picture list modification with truncation | A. K. Ramasubramonian, Y. Chen (Qualcomm) |
| JCTVC-K0170 | RPL | A.I.03.4 | AHG 9: On restricted reference picture list | Hendry, Y.Jeon, B.Jeon (LG) |
| JCTVC-K0224 | RPL | A.I.03.4 | Syntax cleanup for reference picture list modification signalling | G. J. Sullivan, S. Kanumuri (Microsoft) |
| JCTVC-K0255 | RPL | A.I.03.4 | Cleanup of reference picture list modification | T. Yamamoto, T. Tsukuba, T. Ikai (Sharp) |
| JCTVC-K0125 | PS in V1 | A.I.04 | AHG9: On video parameter set | Y.-K. Wang, Y. Chen (Qualcomm) |
| JCTVC-K0173 | PS in V1 | A.I.04 | On VPS | K Sato (Sony) |
| JCTVC-K0180 | PS in V1 | A.I.04 | AHG9: Operation points in VPS and nesting SEI | M. M. Hannuksela (Nokia) |
| JCTVC-K0209 | PS in V1 | A.I.04 | AHG9: VPS and SPS design | B. Choi, T. Lee, Y. Cho, J. Park (Samsung) |
| JCTVC-K0223 | PS in V1 | A.I.04 | On Video Parameter Set | S. Deshpande (Sharp) |
| JCTVC-K0232 | PS in V1 | A.I.04 | On support of mixed video sequences in High Level Syntaxes | M. Haque, K. Sato, A. Tabatabai |
| JCTVC-K0120 | HLS clean-ups | A.I.05 | AHG9: High-level syntax clean-ups | Y.-K. Wang (Qualcomm) |
| JCTVC-K0136 | HLS clean-ups | A.I.05 | AHG9: High-level syntax cleanups | C.-Y. Chen, C.-Y. Tsai, C.-W. Hsu, Y.-W. Huang, S. Lei (MediaTek) |
| JCTVC-K0174 | HLS clean-ups | A.I.05 | On PPS | K Sato (Sony) |
| JCTVC-K0213 | HLS clean-ups | A.I.05 | AHG9: On Tickets 637, 766, and, 777 | K. Chono (NEC) |
| JCTVC-K0217 | HLS clean-ups | A.I.05 | Proposed Changes on Coding Tree Unit Syntax and Sequence parameter set RBSP syntax | Xue Fang (Motorola Mobility) |
| JCTVC-K0251 | HLS clean-ups | A.I.05 | Modification of slice_temporal_mvp_enable_flag | Yue Yu, Jian Lou, Limin Wang (Motorola Mobility) |
| JCTVC-K0254 | HLS clean-ups | A.I.05 | AHG9: Parsing profile and level information of temporal sub-layers | T. Tsukuba, T. Yamamoto (Sharp), S. Deshpande (Sharp Labs of America) |
| JCTVC-K0256 | HLS clean-ups | A.I.05 | Cleanup of Slice Header | Yue Yu, Jian Lou, Limin Wang(Motorola Mobility) |
| JCTVC-K0257 | HLS clean-ups | A.I.05 | Modification of Enabling Weighted Prediction | Yue Yu, Jian Lou, Limin Wang(Motorola Mobility) |
| JCTVC-K0106 | AHG4: Tiles | A.I.06 | On Tile Processing Order | P. Kapsenberg (Intel), W. Zhang (Intel) |
| JCTVC-K0108 | AHG4: SEI | A.I.06 | On parallel decoding with SEI message containing Reference Dependency Tree | W. W. Ro, M. Kim, D. Kim, K. Kim (Yonsei Univ.), C. Kim, Y. Park, K. Choi (Samsung) |
| JCTVC-K0135 | AHG4: WPP | A.I.06 | AHG4: Independent parsing for WPP substreams | T.-D. Chuang, C.-W. Hsu, Y.-W. Huang, S. Lei (MediaTek) |
| JCTVC-K0182 | AHG4: Tiles & WPP | A.I.06 | CABAC termination for the end of tile/WPP | K. Terada, H. Sasai (Panasonic) |
| JCTVC-K0183 | AHG4: Tiles & WPP | A.I.06 | AHG4: Dependent slices restriction | S. Esenlik, M. Narroschke, T. Wedi (Panasonic) |
| JCTVC-K0200 | AHG4: SEI | A.I.06 | AHG4: Sub-stream entry points SEI message | M. Zhou (TI) |
| JCTVC-K0236 | AHG4: SEI | A.I.06 | Decoder parallelism indication | J. Samuelsson, R. Sjöberg, M. Westerlund, B. Burman, P. Fröjdh (Ericsson) |
| JCTVC-K0126 | HRD | A.I.07 | AHG9: On HRD and related general issues | Y.-K. Wang (Qualcomm) |
| JCTVC-K0140 | HRD | A.I.07 | AHG9: Simple HEVC stream editing | K. Kazui, J. Koyama, S. Shimada, A. Nakagawa (Fujitsu) |
| JCTVC-K0214 | HRD | A.I.07 | Early bumping | R. Sjöberg, J. Samuelsson (Ericsson) |
| JCTVC-K0221 | HRD | A.I.07 | On Sub-picture based HRD Buffering | S. Deshpande (Sharp) |
| JCTVC-K0263 | VUI | A.I.08.1 | Sample scale factor in VUI | Arturo Rodriguez |
| JCTVC-K0105 | Frame packing | A.I.08.2 | FPA SEI message modification | O. Nakagami (Sony), T. Suzuki (Sony), G. Ballocca (Sisvel Technology), V. Giovara (Sisvel Technology), P. Sunna (RAI), M. Arena (RAI) |
| JCTVC-K0116 | Frame packing | A.I.08.2 | 2D compatible frame packing stereo 3D video | Xiaofeng YANG, Peiyu YUE, Yuanyuan ZHANG |
| JCTVC-K0119 | Frame packing | A.I.08.2 | AHG9: Indication of frame-packed or interlaced video | Y.-K. Wang (Qualcomm) |
| JCTVC-K0240 | Frame packing | A.I.08.2 | Frame packing arrangement SEI for 4:4:4 content in 4:2:0 bitstreams | Y. Wu, S. Kanumuri, S. Sadhwani, L. Zhu, S. Sankuratri, G. J. Sullivan, B. A. Kumar (Microsoft) |
| JCTVC-K0128 | ROI | A.I.08.3 | AHG9: Signalling of regions of interest and gradual decoding refresh | Y.-K. Wang (Qualcomm) |
| JCTVC-K0218 | ROI | A.I.08.3 | Tile-based region-of-interest signalling with sub-picture SEI messages | Robert Skupin, Valeri George, Thomas Schierl |
| JCTVC-K0247 | ROI | A.I.08.3 | Region of interest (ROI) SEI message | G. Dedeoglu, M. Budagavi (TI) |
| JCTVC-K0248 | ROI | A.I.08.3 | ROI tile sections | Yan Ye, Yuwen He, Yong He (InterDigital Communications), Michael Horowitz (eBrisk Video) |
| JCTVC-K0117 | SEI | A.I.08.4 | Parameter ID change information SEI for bitstream splicing | Xiaofeng YANG, Peiyu YUE, Yuanyuan ZHANG |
| JCTVC-K0127 | SEI | A.I.08.4 | AHG9: Signalling of bitstream and elementary stream properties | Y.-K. Wang, A. K. Ramasubramonian (Qualcomm) |
| JCTVC-K0131 | SEI | A.I.08.4 | Deblocking filter display preference SEI message | Alexis Tourapis, Athanasios Leontaris |
| JCTVC-K0141 | SEI | A.I.08.4 | AHG9: Low delay display hint SEI | K. Kazui, J. Koyama, S. Shimada, A. Nakagawa (Fujitsu) |
| JCTVC-K0142 | SEI | A.I.08.4 | AHG9: Modification of SEIs specified in AVC | K. Kazui, J. Koyama, S. Shimada, A. Nakagawa (Fujitsu) |
| JCTVC-K0205 | SEI | A.I.08.4 | TL0 index SEI message | J. Boyce (Vidyo) |
| JCTVC-K0262 | SEI | A.I.08.4 | AHG9: NAL Packet Priority SEI Message | Eun-Seok Ryu, Yan Ye, Yuwen He, Yong He(InterDigital Communications) |
| JCTVC-K0169 | PS ext. | A.I.09 | AHG 9: On layer ID partition in VPS | Hendry, B.Jeon (LG) |
| JCTVC-K0204 | PS ext. | A.I.09 | VPS syntax for scalable and 3D extensions | J. Boyce (Vidyo) |
| JCTVC-K0206 | PS ext. | A.I.09 | VPS support for out-of-band signaling and hybrid codec scalability | J. Boyce, D. Hong, W. Jang, S. Wenger (Vidyo), A. Luthra (Motorola) |
| JCTVC-K0228 | PS ext. | A.I.09 | Proposal to Video Parameter Set and its Extension | T. C. Thang (UoA), J. W. Kang (ETRI), H. Lee, J. Lee, J. S. Choi |
| JCTVC-K0231 | PS ext. | A.I.09 | VPS syntax re-ordering for easy access of extension parameters | M. Haque, K. Sato |
| JCTVC-K0233 | PS ext. | A.I.09 | VPS Extension updates in Strawman Design Approach 1 using a modified version of Scalability Mapping Table | M. Haque, K. Sato, A. Tabatabai |
| JCTVC-K0234 | PS ext. | A.I.09 | VUI Extension | M. Haque, K. Sato |
| JCTVC-K0274 | PS ext. | A.I.09 | VPS extension design | M. M. Hannuksela (Nokia) |
| JCTVC-K0276 | PS ext. | A.I.09 | Parameter set design for HEVC extensions | Thomas Rusert (Ericsson) |
| JCTVC-K0277 | PS ext. | A.I.09 | Inter-layer SPS prediction for HEVC extensions | Thomas Rusert (Ericsson) |
| JCTVC-K0283 | PS ext. | A.I.09 | Video Parameter Set extension design | Robert Skupin, Valeri George, Thomas Schierl |
| JCTVC-K0285 | Profile & Level | A.I.10.1 | In Support Of A Still Frame Profile of HEVC v1 | W. Dai, M. Krishnan, P. Topiwala (FastVDO), |
| JCTVC-K0287 | Profile & Level | A.I.10.1 | CANNB comments on HEVC and HEVC profiles | G. Martin-Cocher |
| JCTVC-K0176 | Profile & Level | A.I.10.2 | AHG9: On LCU bit size limit | K. Chono (NEC) |
| JCTVC-K0201 | Profile & Level | A.I.10.2 | AHG9: on number of slices constraint | M. Zhou(TI) |
| JCTVC-K0202 | Profile & Level | A.I.10.2 | AHG9: on number of tiles constraint | M. Zhou(TI) |
| JCTVC-K0225 | Profile & Level | A.I.10.2 | On DPB Extension | L. Kerofsky, S. Deshpande (Sharp) |
| JCTVC-K0146 | Interlaced - high level | A.I.11 | Comments on Coding and Display Formats | W. Wan, B. Heng, W. Ahmad (Broadcom) |
| JCTVC-K0160 | Interlaced - high level | A.I.11 | field indication metadata | C. Fogg, P. Haskell, O. Bar-Nir, A. Wells, D. Le Gall |
| JCTVC-K0226 | Interlaced - high level | A.I.11 | Confidence level and mixed-content indicators for field indication SEI message | G. J. Sullivan, Y. Wu (Microsoft) |
| JCTVC-K0203 | Transform skipping - high level | A.I.12 | AHG9: A bug fix for scaling list signalling in SPS when transform skipping is enabled | A. Osamoto, M. Zhou(TI) |

# Meeting minutes and recommendations made during the face-to-face AHG meeting

The AHG held a meeting from 0900‒1800 on Tuesday 9 October 2012 at the same venue as the 11th JCT-VC meeting that was to begin on the following day. The AHG meeting was chaired by Gary Sullivan and Ye-Kui Wang. Due to lack of time, the AHG only reviewed contributions in the following four topic areas:

- NAL unit header (3 contributions)

- Random access and adaptation (6 contributions)

- Slices (6 of the 8 contributions; 2 contributions in this category were not reviewed due to the absence of presenters)

- Misc. SEI messages (1 of the 7 contributions)

Detailed notes of the discussion in each of these areas are found in the following sections. Certain key aspects are highlighted with yellow highlighting for the review and attention of the JCT-VC.

## NAL unit header

JCTVC-K0121 AHG9: NAL unit header design for support of multi-standard extensions [Y. Chen, Y.-K. Wang (Qualcomm)]

This is a follow-up proposal of JCTVC-I0355 and JCTVC-J0113, for support an HEVC extension where the base layer/view is AVC and the enhancement layers/views are HEVC. In both of these proposals, NAL unit header syntax changes were proposed so that distinguishing of AVC and HEVC NAL units by checking the NAL unit header becomes possible. Both the approaches in JCTVC-I0355 and JCTVC-J0113 require a bit to be reserved at the HEVC NAL unit header.

In this proposal, it is proposed that the order of syntax elements to be changed to support the multi-standard functionality, which is a "should" requirement for the scalable enhancement of HEVC and was also supported in 3DV discussions.

It was remarked that this could be addressed at the systems level, rather than trying to create bitstreams that are mixed at the NAL unit level.

K0206 was mentioned as being relevant.

The specific suggestion is to switch the order of the nal_unit_type and nuh_reserved_zero_6bits syntax elements in the NUH. Then, in a mixed bitstream, the value of the one of the bits of nuh_reserved_zero_6bits (which, conceptually, is the layer ID) would always be set to 1.

It was commented that it could be desirable to detect HEVC vs. AVC even without enabling the mixed-bitstream construction.

Actual operation with reception of a mixed bitstream fed directly to an existing AVC decoder (without a demux).

The HLS AHG suggested to revisit after K0206 discussion.

JCTVC-K0177 AHG9: NAL unit header with layer ID partitioning [B. Choi, J. Park (Samsung), Y.-K. Wang, Y. Chen (Qualcomm)]

At the 10th JCT-VC meeting, two approaches for NAL unit header and video parameter set for HEVC extension were discussed, and their “straw man” designs were described in JCTVC-J1007. This contribution proposes two alternative NAL unit header designs in the same category as approach 2 in JCTVC-J1007. The proposed design partitions a layer_id into specific scalability dimension identifiers by parsing only NAL unit headers.

For the "base" (v1) specification, the only proposed changes are 1) the NUH of the VPS, and in one variation of the proposal, 2) having the size of the temporal_id depend on the NUH of the VPS.

The proposal avoids needing a mapping table in the VPS to identify a layer type, by instead creating a partitioning of the layer id bits in the NUH into separate fields for different types of layering.

The partitioning approach would be somewhat simpler to use, while the mapping table approach would be somewhat more flexible – somewhat more "future proof".

It was noted that the proposal enables a non-fixed partitioning of temporal ID bits, which has both advantages and disadvantages. In that regard, it adds some flexibility. However, it was questioned whether we would really need more than 6 bits of layer ID range.

When reviewed in the HLS AHG, there did not seem to be strong support for this proposal.

JCTVC-K0210 AHG9: NAL unit header design for base spec [B. Choi, T. Lee, C. Kim, J. Park (Samsung)]

In the latest HEVC WD, 6 bits are reserved for layer_id in the NAL unit header. However, even if layer_id should be signalled in NAL unit headers in an HEVC extension, it was suggested that this does not mean that the bits for layer_id should be reserved and be fixed to a pre-defined value in the HEVC base spec. If a way to distinguish base layer NAL units from enhancement layer NAL units is present, the reserved bits can be used for other purposes than layer_id in HEVC base spec. This document proposes two alternative NAL unit header designs for the distinguishing base layer NAL units and non-base layer NAL units. Some part of this contribution overlaps the design proposed in JCTVC-K0177.

One proposed approach is to use a bit to identify whether the current layer is the base layer or not. When the layer is identified as the base layer, more bits would become available for use in the base layer. Another aspect of the proposal merges the temporal sub-layer ID into other layering identification bits.

Another proposed approach is to increase the number of bits for NUT and use different NUTs for enhancement layers (including temporal layers) than the values of the NUTs used for the base layer.

In each variant, there would be a distinction drawn between the syntax of the base layer NUT and the syntax of enhancement layer NUTs, and then some syntax elements in the NUH of the base layer that would become available for use for other purposes.

One proposed use of the additional available syntax in the base layer would be an inter-layer prediction (ILP) flag, indicating whether the base layer is used by enhancement layers or not. Others include a picture output flag, and a no-output-of-prior-pictures flag.

There was some interest in adding an ILP flag. This should be further discussed, and potentially added – e.g. in the slice header if not the NUH.

When reviewed in the HLS AHG, there did not seem to be strong support for this proposal (other than in having an ILP flag).

## Random access and adaptation

JCTVC-K0122 AHG9: On CRA and BLA pictures [Y.-K. Wang (Qualcomm)]

This document proposes the use of an external-means-determined flag, UseAltCpbParamsFlag, to specify which set of initial CPB removal delay and offset parameters is used, for BLA pictures with nal_unit_type equal to BLA_W_LP and CRA pictures. The proposed flag is also used to specify which of the BLA NAL unit types is used when a CRA picture is indicated to be handled as a BLA picture.

In the current draft, the NUT of AU number 0 determines which parameters apply. Also, in the current draft, there is an external means flag, HandleCraAsBlaFlag, that is used.

There was support expressed for the proposal in the HLS AHG discussion. Pending close study, the HLS AHG supported the proposal (at least in concept).

JCTVC-K0159 AHG 9: On referenced and non-referenced DLP and TFD NAL units [Hendry, Y. Jeon, B. Jeon (LG)]

In the 10th JCTVC meeting, it was agreed to remove nal_ref_flag from NAL unit header and divide NAL unit types for coded slice of a non-TSA, non-STSA trailing picture, coded slice of a TSA picture, and coded slice of an STSA picture into referenced and non-referenced NAL unit types to replace the functionality that was provided by the removed flag. This contribution proposes to also divide NAL unit types for leading pictures (i.e., DLP and TFD NAL unit types) into referenced and non-referenced types. The rationale for this is that leading pictures actually behave similar to trailing pictures in the decoding process when random access / splicing are not done.

It was commented that we would ordinarily not expect many leading pictures per temporal layer, so this modification might not be necessary. The temporal ID already provides some ability to indicate "droppable" pictures within the leading pictures category.

Note that one of the items in K0208 is the same as this proposal.

It was remarked that this topic was already discussed at the previous meeting, and the new contributions are not really adding to that. The question is whether it is really worth using two NUTs for this indication.

In the HLS AHG discussion, there was a mixture of support and skepticism regarding this proposal. Further discussion was suggested.

JCTVC-K0164 AHG 9: On non-referenced TSA and STSA NAL units [Hendry, B. Jeon (LG)]

In the 10th JCTVC meeting, it was agreed to adopt STSA NAL units and also to differentiate TSA and STSA NAL unit types into referenced and non-referenced NAL units (i.e., TSA_R, TSA_N, STSA_R, and STSA_N). Since both TSA and STSA NAL units can be referenced (i.e., TSA_R and STSA_R) and non-referenced (i.e., TSA_N and STSA_N), it was suggested that there is a possibility that for the TSA_N and / or STSA_N are removed from the bitstream by a system / network that work simply by checking whether or not a NAL unit is needed for reference.

It was remarked that there may be value in retaining the non-reference TSA and STSA types, and that an encoder would only use those types if those pictures were reasonably discardable (e.g. if there were also some reference TSA or STSA pictures in the bitstream as well). Having the non-reference types may allow the decoder to recover somewhat earlier.

However, it was remarked that using another temporal ID for the non-reference TSA/STSA pictures, it could provide a similar functionality. This was questioned in regard to trying to have a fixed frame rate in each temporal sub-layer.

There was not a consensus in the HLS AHG on this, so further discussion was suggested.

Another aspect discussed in the contribution was whether it is reasonable to allow an STSA picture in a layer when there was not a TSA or STSA the next lower layer. In some instances, this seemed reasonable, so no action was suggested to be taken on this aspect by the HLS AHG.

JCTVC-K0166 AHG 9: On random access point NAL units [Hendry, B. Jeon (LG)]

This contribution proposes merging the IDR and BLA NUTs, asserting that they have similar functionality. It was noted that dropping IDR was previously proposed (at least twice). The contribution seemed to essentially repeat that proposal. Skepticism was expressed about this by some participants, saying that some systems may want to rely on the distinctness of IDR functionality.

No action was recommended by the HLS AHG on this aspect.

Another aspect of the proposal was to merge the NUTs for RAP pictures and use the current temporal ID bit field to distinguish between the RAP types instead of using the NUT. This would save 5 NUT values. It was remarked that K0219 includes a proposed similar approach.

There were mixed opinions about this in the HLS AHG, due to the conditional interpretation of the temporal ID bit field from a bitstream extraction perspective, so further discussion was suggested. The temporal ID would need to be inferred as zero when the NUT indicates a RAP picture.

The HLS AHG recommended further discussion of this (together with K0219).

JCTVC-K0208 AHG9: On random access point pictures [B. Choi, T. Lee, Y. Park, J. Park (Samsung)]

In this contribution, several syntax changes on random access point pictures are proposed.

- CRA and BLA NAL units and pictures: 1) Inclusion of new NAL unit and picture types for CRA with TFD and DLP pictures, 2) Removal of BLA_W_DLP NAL unit and picture type. This seemed unnecessary in HLS AHG discussion.

- IDR NAL unit and picture: Combining two IDR types to an IDR type. This was not supported in the HLS AHG discussion, as the current design simplifies the detection of the simplest stream access point type (e.g. as used in DASH and file format specs).

- TSA and STSA NAL units and pictures: Inclusion of TSA&TFD picture and STSA&TFD picture– proposing to add two NUTs. This was originally proposed in J0107, but having the extra distinction was not supported and there seemed to be no clear need to change that in the HLS AHG discussion.

- Reference/non-reference NAL units and pictures: 1) Inclusion of DLP and TFD pictures referenced or non-referenced, 2) Inclusion of nuh_ref_flag in NAL unit header. The same as proposed in K0159 – see notes in that section.

- TFD NAL unit and picture: Inclusion of nuh_discard_flag – a proposal that depends on JCTVC-K0210. See discussion of K0210.

- DLP picture: Enabling inter-prediction from DLP pictures to the following pictures. The coding efficiency benefit did not seem sufficient to justify this change.

- Moving no_output_of_prior_pics_flag into the NUH – a proposal that depends on JCTVC-K0210. See discussion of K0210.

JCTVC-K0219 On Random Access Point Picture Signaling [S. Deshpande (Sharp)]

In HEVC when a RAP picture is signalled the syntax element nuh_temporal_id_plus1 is required to be signalled such that TemporalId shall be equal to 0. This document proposes different alternatives to use this unused syntax element in the case of RAP pictures. It is asserted that this is beneficial and does not sacrifice complexity.

- Approach 1 in this proposal is essentially the same as the RAP type NUT merging proposed in K0166.

- Approach 2 concerned back-to-back IDR pictures, and proposed sending a RAP ID (similar to IDR pic ID in AVC) for RAP pictures.

- Approach 3 reserves the current temporal ID bits for future use when the picture is a RAP picture.

See notes in section on K0219.

An additional separate aspect of the document proposes signalling the pic_order_cnt_lsb syntax element for IDR pictures. Note that we no longer have IDR pic ID. The pic_order_cnt_lsb for an IDR would be interpreted in the same manner as for a BLA.

It was noted that K0166 is related to this.

It was asked whether it should be required for back-to-back IDR pictures to have different values of pic_order_cnt_lsb. There would be both advantages and disadvantages to such a requirement.

It was remarked that there is an SEI message proposal K0205 that is related, and there are system-level uses for this – e.g. for AVC/SVC.

It was remarked that this may not really provide any capability that is not otherwise available, other than providing some identifier. In the HLS AHG, there did not seem to be strong interest in this aspect.

## Slices

JCTVC-K0113 The general structure and syntax of the slice extension [David Singer, Thanos Leontaris, Alexis Tourapis (Apple)]

No presenter was available in HLS AHG meeting to present this contribution.

JCTVC-K0114 Specific Uses of the Slice Extension [David Singer, Thanos Leontaris, Alexis Tourapis (Apple)]

No presenter was available in HLS AHG meeting to present this contribution.

JCTVC-K0167 AHG 9: On dependent slice [Hendry, B. Jeon (LG)]

This contribution proposes, for a dependent slice, to send the POC LSB and the CTU address of the slice on which it depends.

Without something like this, detection of losses at the header level would be difficult (absent a system-level functionality for this). However, it was questioned whether the ability to perform the detection at the header level (without decoding the preceding slice) is very useful for a dependent slice.

In the HLS AHG, the opinions on this seemed somewhat mixed and there was no strong support expressed by non-proponents.

JCTVC-K0168 AHG 9: Short Slice Header [Hendry, B. Jeon (LG)]

In the current HEVC specification, all syntax elements of a slice header must present even when they do not change from slice to slice within the same picture, unless the slice is a dependent slice. It was asserted that in environment where error resiliency is not critical, duplicating syntax elements in slice headers is undesirable. Previous related proposals have included JCTVC-I0070 and JCTVC-J0109.

This contribution proposes a short slice header that is indicated by a ‘short_slice_header_flag’. When the flag is set, the slice header contains only syntax elements that change from the previous regular slice header with the flag not set and syntax elements that are not present would be copied from previous regular slice header.

The slice header contents are proposed to not be allowed to change within a picture when using this scheme.

A participant commented that this somewhat defeats the purpose of separating a picture into slices.

In the HLS AHG, the general view seemed to be that similar ideas had been proposed before, and that it seemed unwise to adopt such a scheme at this point.

JCTVC-K0284 Cross-check of JCTVC-K0168 on Short Slice Header [Félix Henry, Gordon Clare (Orange Labs)]

JCTVC-K0184 AHG9: On dependent slices syntax [S. Esenlik, M. Narroschke, T. Wedi (Panasonic)]

The dependent slices introduce an additional dimension of dependency between the VCL NAL units, where a dependent slice is not decodable if the preceding VCL NAL unit is lost. It was asserted to be desirable to take this dependency into account in routing operations for the purposes of more efficient bitstream adaptation and managing of the downlink bandwidth. In the document it is proposed to change the signalling orders of the dependent_slice_flag and dependent_slice_enabled_flag in the corresponding syntax structures in order to enable routers to access and utilize the slice dependency indication.

In the HLS AHG, there was some support expressed for taking action on this topic. However, offline study and further discussion seemed necessary.

JCTVC-K0288 Comments on Entropy Slices [W. Wan, B. Heng (Broadcom)] [late]

The current HEVC draft text specifies the concept of entropy slices but they are not included in the Main Profile and seem to be the only feature that has this status.  This contribution notes that entropy slices do not break a number of cross-slice dependencies in CABAC decoding state (e.g. for intra mode identification and/or because of context selection dependencies). As such, it is not possible to independently CABAC decode entropy slices. Because they are asserted to no longer accomplish their intended purpose, this contribution recommends removing entropy slices from the HEVC specification.

K0120 also proposes removal of entropy slices from the draft standard.

The HLS AHG recommended removal of entropy slices from the draft standard.

JCTVC-K0314 Comments on Dependent Slices [W. Wan, B. Heng, P. Chen (Broadcom)] [late]

This contribution notes an issue with the specification of dependent slices in the current HEVC WD text.  Specifically, it is unclear in which instances the term “slice” is intended to refer to both ("normal") slices and dependent slices, and in which instances “slice” refers only to (normal) slices. Two areas of the current HM software (QP prediction and loop filtering) are discussed that are asserted to have contradicting behavior for dependent slices. It is proposed that the “slice” term be clarified and used consistently with respect to dependent slices and suggested that cross-slice prediction is always allowed for dependent slices.

It was remarked that "dependent slice" may not be a good name, as the data is really part of the same "slice".

Conceptually, a "dependent slice" should be considered part of the same slice.

The HLS AHG agreed that these aspects should be clarified. Offline study with review of the identified issues was recommended.

## Misc. SEI messages

JCTVC-K0205 TL0 index SEI message [J. Boyce, D. Hong, W. Jang(Vidyo)]

It is proposed to include a TL0_index SEI message in the HEVC base specification, for improved error resiliency when temporal scalability is used. The proposed SEI message is similar to that used in the tl0_dep_rep_index SEI message in SVC, with revised syntax and semantics in order to better align with the HEVC base specification design.

It was noted that this is related to K0166 and K0219.

It was remarked that in Daegu we actually expressed an intent to have the TL0_index SEI message in the HEVC design, based on D082 – but did not follow through on that thus far.

The proposed normative increment-by-one behaviour was discussed – in relation to both syntax elements of the proposed SEI message. It was suggested that some operations, such as removing a non-reference picture or splicing bitstreams, would violate this constraint, so the normative nature of the constraint was questioned. Suggestions included eliminating the normative constraint on rap_idx behaviour and making the tl0_idx increment in the same way that frame_num did in AVC (i.e. increment relative to the previous TL0 picture that was not a non-reference picture for its temporal layer and was not a leading picture).

It was remarked that the RPS also enables picture loss detection.

The HLS AHG suggested to revisit this contribution after offline work to consider these issues.

# Further recommendations

Besides the recommendations made during the face-to-face AHG meeting, the high-level syntax AHG further recommends reviewing all related input contributions.