| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0021 |
| --- | --- |

| Title: | CE1: Summary report of core experiment on deblocking filtering |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Andrey Norkin Teruhiko Suzuki Madhukar Budagavi Geert Van der Auwera | Email: | andrey.norkin@ericsson.com teruhikos@jp.sony.com madhukar@ti.com geertv@qualcomm.com |
| Source: | CE coordinators |  |  |

_____________________________

# Introduction

This contribution is a summary report of Core Experiment 1 on deblocking filtering [1]. Based on the results of the subjective test, it has been decided to further study separate components of proposals JCTVC-J0181 [3] and JCTVC-J0286 [4] and their combinations in the CE1 as well as the TU based decisions from JCTVC-J0189 and JCTVC-J0096 [6]. It has been decided to study performance of the proposals on critical sequences (i.e. those showing blocking artifacts in the default operating mode) as well as studying the performance on sequences from the common test conditions [8] in order to ensure there is no degradation of subjective quality. All the tests are based on HM8.0. The experiments are performed according to the common test conditions provided in JCTVC-J1100. Additionally, all the proposals provide results for LD-P configurations and the results on additional sequences from CE1.

# Document list

## Summary document (this document)

- JCTVC-K0021 [CE coordinators] CE1: Summary report of core experiment on deblocking filtering

## Proposals

- JCTVC-K0149,  S. Lu, O. Nakagami, T. Suzuki (Sony),  CE1: On deblocking filter

- JCTVC-K0186,  A. Norkin (Ericsson),  CE1: Reduction of block artifacts in HEVC

## Cross-verification documents

- JCTVC-K0151, S. Lu, O. Nakagami (Sony), CE1:Cross check of test7 in CE1

- JCTVC-K0187, A. Norkin (Ericsson), CE1: Cross-check of Test 3 (K-149, Sony) by Ericsson

- JCTVC-K0212, W. Wan, P. Chen(Broadcom), CE1: Cross-check of Test 4

- JCTVC-K0243, G. Van der Auwera (Qualcomm), CE1: Cross-check of JCTVC-K0149 and JCTVC-K0186

- JCTVC-K0266, E. Alshina, CE1: Cross-check for test 2

- JCTVC-K0267, E. Alshina, CE1: Cross-check for test 6

- JCTVC-K0281, M. Budagavi, D.-K. Kwon (TI), 	CE1: Cross-check of Test 8, Test 9, Test 4, Test 5.

# Description of tested tools

Combinations of the following tools have been tested in the CE according to the CE description [1].

Tool a. Beta table modification

JCTVC-J0181, S. Lu, O. Nakagami, M. Ikeda, T. Suzuki (Sony)

It is proposed to replace the β values with Table 2-2 [3]. The value is determined by the principle that the changing rate of β should have similar trend with the changing rate of tc to reflect quantization change. Since the changes lie at QP bigger than 41, there is no impact on common test conditions (provided the QP values in CTC do not change).

The tool is used by sending the deblocking beta_offset to the decoder, so that modified part of the beta table is used.

Figure 1: β-tc-QP relation: blue line is tc, red line is β in HM8.0, green line is proposed β

The proposal consists of three main parts.

## Tool b.1 	Removing intra-boundary smoothing for larger block sizes (32x32 and 16x16)

## JCTVC-J0286, A. Norkin (Ericsson)

It is proposed to remove the intra-boundary smoothing (applied in HEVC for DC, vertical and horizontal prediction modes) from the larger intra-block sizes (32x32 and possibly 16x16). This reportedly avoids  forming irregularities on left and/or top boundaries of intra-blocks.

Figure 2 DC prediction mode, block 32x32, intra-filtering along a block boundary creates a singularity in prediction.

## Tool b.2.1 	Modification of strong filter decisions

Two conditions in strong filter decision are modified to enable application of the HEVC strong filter on inclined surfaces (current HEVC strong filter is applied to the flat surfaces).

| (a) HM8.0 strong filter decisions | (b) Proposed strong filter decisions |
| --- | --- |

Figure 3 HM8.0 strong filter decisions (a) vs proposed strong filter decisions (b)

2*dpqi < (β>>2)			          			//same as in HM8.0

(|p3i - p0i| + |q0i – q3i|) < (β>>2) 		// conditions that signal is flat

((| p3,i − 2*p2,i + p1,i | + | q3,i − 2*q2,i + q1,i |) <<1) < (β>>2)  		// condition that signal is flat or a ramp

|p0i – q0i| < ((5*tC + 1)>>1)  (tC <<4)		// the threshold is increased

## Tool b.2.2 	Modification of strong filter

The strong filter is modified to match strong filter decisions from b.2.2 so that it avoids modifying an inclined surface when applied to it

| - Effect of HEVC strong filter applied to a ramp | - Effect of proposed strong filter applied to a ramp |
| --- | --- |

Figure 4 Comparison of modification of ramp by HEVC filer and the proposed filter. The original signal is shown in blue, the signal after modification by a filter is shown in red. The HEVC filter (a) modifies the ramp stronger than the proposed filter (b).

p0’ = Clip3( p0−tc0, p0+tc0, ( p2 + 2*p1 + 2*p0 + 2*q0 + q1 + 4 ) >> 3 )   // same filter as in HM8.0
q0’ = Clip3( q0−tc0, q0+tc0, ( p1 + 2*p0 + 2*q0 + 2*q1 + q2 + 4 ) >> 3 )   // same filter as in HM8.0

p1’ = Clip3( p1−2*tc, p1+2*tc, ( p2 + p1 + p0 + q0 + 2 ) >> 2 )
p2’ = Clip3( p2−2*tc, p2+2*tc, ( 2*p3 + 3*p2 + p1 + p0 + q0 + 4 ) >> 3 ) 
q1’ = Clip3( q1−2*tc, q1+2*tc, ( p0 + q0 + q1 + q2 + 2 ) >> 2 )
q2’= Clip3( q2−2*tc, q2+2*tc, ( p0 + q0 + q1 + 3*q2 + 2*q3 + 4 ) >> 3 )

p1' = Clip3(p1-tc1, p1+tc1, ((p3 + 2*p2+ 2*p1 + p0 + 2*q0 + 4)>>3))          
q1'   = Clip3(q1-tc1, q1+tc1, ((q3 + 2*q2+ 2*q1 +   q0+2*p0 + 4)>>3))
p2' = Clip3(p2-tc3, p2+tc3, ((3*p3 + 3*p2 + p1 + q0 + 4 )>>3))
q2' = Clip3(q2-tc3, q2+tc3, ((p0 + q1 + 3*q2 + 3*q3 +4 )>>3))

## Tool b.3.1 	 Modification of boundary strengh (Bs) derivation for CU32x32

The derivation of the boundary strength BS is modified in a way that for the edges that have boundary strength Bs = 2, Bs can be increased to 3 in case of CU larger or equal to 32x32 when the block boundary is on the 32x32 filtering grid.

## Tool b.3.2 	Increase value of intra_tc_offset

Alternatively to b.3.1, the intra_tc_offset is increased from 2 to 3 to increase filtering of intra-blocks.

Q = Clip3( 0, 53, QPC + 2 3*(bS − 1) + (tc_offset_div2 << 1 ) )

## Tool b.3.2 	Modification of boundary strengh (Bs) derivation for max TU

J0090, G. Van der Auwera, R. Joshi, M. Karczewicz (Qualcomm)

J0096, D.-K. Kwon, M. Budagavi (TI)

Alternatively to b.3.2 and b.3.1, Bs is increased from 2 to 3 at the intra TU boundary when one of the TUs has a maximum TU size. The procedure for determining TU boundary for the maximum TU size is described in

# Tested combinations and cross-verification assignment

Table 1 Combinations tested in CE

| Test No | Document/proponent (tester) | BetaTab a | Boundary Smoothing Restriction b.1 | Strong Filter &Decisions b.2.1 & b.2.2 | Modific Bs CU 32x32 b.3.1 | Intra tc_offset =3 b.3.2 | Mod. BS maxTU b.3.3 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | K0149 (Sony) | x |  |  |  |  |  |
| 2 | K0186 (Ericsson) |  | 32 | x | x |  |  |
| 3 | K0149 (Sony) | x | 32 | x | x |  |  |
| 4 | K0186 (Ericsson) |  | 32 | x |  | x |  |
| 5 | K0149 (Sony) | x | 32 | x |  | x |  |
| 6 | K0149 (Sony) |  | 32 |  |  |  |  |
| 7 | K0186 (Ericsson) |  |  | x | x |  |  |
| 8 | K0186 (Ericsson) |  | 32 and 16 | x |  |  | x |
| 9 | K0186 (Ericsson) | x | 32 and 16 | x |  |  | x |

Table 2 Cross-verification assignment

| Test | Proponent | Cross-checker(s) |
| --- | --- | --- |
|  | K0149 (Sony) | K0243 (Qualcomm) |
|  | K0186 (Ericsson) | K0266 (Samsung) |
|  | K0149 (Sony) | K0187 (Ericsson) |
|  | K0186 (Ericsson) | K0212 (Broadcom) |
|  | K0149 (Sony) | K028, (TI) |
|  | K0149 (Sony) | K0267 (Samsung) |
|  | K0186 (Ericsson) | K0151 (Sony) |
|  | K0186 (Ericsson) | K0281 (TI), K0243 (Qualcomm) |
|  | K0186 (Ericsson) | K0281 (TI), K0243 (Qualcomm) |

# Objective performance on common test conditions

Based on common test conditions Error: Reference source not found, results for all mandatory test cases and two additional LD-P configurations shall be provided for CE1 proposals. The following evaluation criteria are used:

- BD-rate relative to the anchor(s)

- Complexity (decoding time measurement)

In the tables below, the following abbreviations are used:

AI – all intra

RA – random access

LD B – low delay (B)

LD P – low delay (P)

In Table 3 - Table 6 the second summary of Excel template was used, which includes the type F sequences and LD P configurations. For the gray-shadowed rows, no simulations were done since the performance on the common test conditions is identical to the performance of the test above (the difference between the combinations is in modifying the deblocking beta table outside CTC). Therefore, the same numbers from the test above are used. Test 1 is identical on CTC to HM8.0, Test 3 is identical to Test 2, Test 5 to Test 4 and Test 9 to Test 8.

Table 3 BD-Rate (Y)

| Test/document | Main |  |  |  | HE 10-bit |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LD B | LD P | AI | RA | LD B | LD P |
| Test 1 K0149 (Sony) | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Test 2 K0186 (Ericsson) | 0.1 | 0.0 | 0.1 | -0.1 | 0.2 | 0.1 | 0.5 | -0.1 |
| Test 3 K0149 (Sony) | 0.1 | 0.0 | 0.1 | -0.1 | 0.2 | 0.1 | 0.5 | -0.1 |
| Test 4 K0186 (Ericsson) | 0.1 | 0.0 | 0.1 | -0.1 | 0.3 | 0.1 | 0.5 | 0.0 |
| Test 5 K0149 (Sony) | 0.1 | 0.0 | 0.1 | -0.1 | 0.3 | 0.1 | 0.5 | 0.0 |
| Test 6 K0149 (Sony) | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Test 7 K0186 (Ericsson) | 0.1 | 0.0 | 0.1 | -0.1 | 0.2 | 0.1 | 0.5 | 0.0 |
| Test 8 K0186 (Ericsson) | 0.1 | 0.0 | 0.1 | -0.1 | 0.3 | 0.1 | 0.5 | 0.0 |
| Test 9 K0186 (Ericsson) | 0.1 | 0.0 | 0.1 | -0.1 | 0.3 | 0.1 | 0.5 | 0.0 |

Table 4 BD-Rate (U)

| Test/document | Main |  |  |  | HE 10-bit |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LD B | LD P | AI | RA | LD B | LD P |
| Test 1 K0149 (Sony) | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Test 2 K0186 (Ericsson) | 0.1 | 0.0 | 0.3 | 0.3 | 0.1 | 0.1 | 0.3 | 0.0 |
| Test 3 K0149 (Sony) | 0.1 | 0.0 | 0.3 | 0.3 | 0.1 | 0.1 | 0.3 | 0.0 |
| Test 4 K0186 (Ericsson) | 0.0 | 0.1 | 0.2 | 0.3 | 0.0 | 0.2 | 0.5 | 0.2 |
| Test 5 K0149 (Sony) | 0.0 | 0.1 | 0.2 | 0.3 | 0.0 | 0.2 | 0.5 | 0.2 |
| Test 6 K0149 (Sony) | 0.1 | -0.1 | 0.1 | 0.3 | 0.1 | 0.0 | -0.1 | -0.2 |
| Test 7 K0186 (Ericsson) | 0.0 | 0.0 | 0.2 | 0.3 | 0.0 | 0.0 | 0.5 | 0.1 |
| Test 8 K0186 (Ericsson) | 0.2 | 0.0 | 0.3 | 0.3 | 0.2 | 0.2 | 0.6 | 0.2 |
| Test 9 K0186 (Ericsson) | 0.2 | 0.0 | 0.3 | 0.3 | 0.2 | 0.2 | 0.6 | 0.2 |

Table 5 BD-Rate (V)

| Test/document | Main |  |  |  | HE 10-bit |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LD B | LD P | AI | RA | LD B | LD P |
| Test 1 K0149 (Sony) | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Test 2 K0186 (Ericsson) | 0.1 | 0.1 | 0.3 | 0.2 | 0.1 | 0.0 | 0.2 | 0.0 |
| Test 3 K0149 (Sony) | 0.1 | 0.1 | 0.3 | 0.2 | 0.1 | 0.0 | 0.2 | 0.0 |
| Test 4 K0186 (Ericsson) | 0.0 | 0.2 | 0.3 | 0.0 | 0.1 | 0.0 | 0.2 | 0.0 |
| Test 5 K0149 (Sony) | 0.0 | 0.2 | 0.3 | 0.0 | 0.1 | 0.0 | 0.2 | 0.0 |
| Test 6 K0149 (Sony) | 0.1 | 0.1 | 0.0 | 0.2 | 0.1 | -0.1 | -0.1 | -0.1 |
| Test 7 K0186 (Ericsson) | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 | -0.1 | 0.1 | -0.2 |
| Test 8 K0186 (Ericsson) | 0.2 | 0.1 | 0.1 | 0.0 | 0.2 | 0.0 | 0.1 | -0.2 |
| Test 9 K0186 (Ericsson) | 0.2 | 0.1 | 0.1 | 0.0 | 0.2 | 0.0 | 0.1 | -0.2 |

## Complexity – relative decoding time against the anchor

Table 6 Summary of relative decoding time (%) (against HM4.0 anchor)

| Test/document | Main |  |  |  | HE 10-bit |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LD B | LD P | AI | RA | LD B | LD P |
| Test 1 K0149 (Sony) | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% |
| Test 2 K0186 (Ericsson) | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% |
| Test 3 K0149 (Sony) | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% |
| Test 4 K0186 (Ericsson) | 100% | 100% | 99% | 100% | 100% | 100% | 100% | 100% |
| Test 5 K0149 (Sony) | 100% | 100% | 99% | 100% | 100% | 100% | 100% | 100% |
| Test 6 K0149 (Sony) |  |  |  |  |  |  |  |  |
| Test 7 K0186 (Ericsson) | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% |
| Test 8 K0186 (Ericsson) | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% |
| Test 9 K0186 (Ericsson) | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% |

# Comments and subjective quality results from cross-checkers

The subjective performance has been evaluated on two types of sequences.

Type 1 sequences are used to demonstrate improvement in visual quality: "Riverbed", "West Winds Easy", "Ducks Take Off 25fps", "China Speed", "Red Kayak".

The sequences are available at the following URL: http://sdrv.ms/MPDyGd

Type 2 sequences: These sequences are used to demonstrate that there is no visual degradation in common test conditions: "Kimono", "Cactus", "BasketballDrillText", "BQMall", and "Jonny". The selection of sequences may be modified if the content shows visual quality degradation from deblocking.

JCTVC-K0266, E. Alshina (Samsung), CE1: Cross-check for test 2

Objective results and source code:

The objective results for CE1 test 1 (JCTVC-K0149), and CE1 tests 8 and 9 (JCTVC-K0186) are reported and it is confirmed that there is a match with those reported in the respective proposals.

BD-rate has been measured for Test 2 vs the anchor for Type 1 sequences using QP: 27,32,37,41. Test 2 has shown objective performance improvement over HM8.0 (-0.4% on average).

Subjective quality improvements on Type 1 sequences:

Tests 2 has been evaluated on Type 1 sequences at QP 32, 37, 41. It has been observed that in 67% cases evaluated visually the verified algorithm reduces the amount and strength of block artefacts. In 13% cases verified algorithm performs equally well with HM8.0 de-blocking filter (i.e. the anchor is already good enough). In remaining 20% tests both anchor and verified algorithm have block artefacts.

Tests 4 has also been evaluated. It has been found that Test 4 provides similar subjective quality improvement to Test 2, while requiring less modifications to HM8.0 (no modification to Bs derivation is done). Test 4 has been observed to produce better subjective quality than Test 2 on some sequences (e.g. China Speed).

Test 2 vs HM8.0

| Sequence/test case | QP point | Comparison with Anchor | Better than anchor? |
| --- | --- | --- | --- |
| DucksTakeOff, 25fps, RA-main | 32 | Can see moderate strength block artefacts for both anchor and verified algorithm. Picture quality for both anchor and modification is lower than acceptable. | no |
|  | 37 | Can see moderate strength block artefacts for verified algorithm; but for anchor artefacts are stronger. Picture quality for anchor is unacceptable. | yes |
|  | 41 | Can see moderate strength block artefacts for verified algorithm; but for anchor artefacts are stronger. Picture quality for anchor is unacceptable. | yes |
| RedKayak,570fr, RA-main | 32 | Can see moderate strength block artefacts for verified algorithm; but for anchor artefacts are stronger. Main artefacts appear for frames #460…500. Picture quality for anchor is unacceptable. | yes |
|  | 37 | Can see moderate strength block artefacts for verified algorithm; but for anchor artefacts are stronger. . Main artefacts appear for frames #460…500. Picture quality for anchor is unacceptable. | yes |
|  | 41 | Picture quality is unacceptable for both anchor and modification. | no |
| Riverbed, LDB-main | 32 | Can see moderate strength block artefacts in anchor; verified algorithm provides acceptable picture quality. | yes |
|  | 37 | Can see moderate strength block artefacts for verified algorithm; but for anchor artefacts are stronger. Picture quality for anchor is unacceptable. | yes |
|  | 41 | Picture quality is unacceptable for both anchor and modification. | no |
| WestWindEasy, LDB-main | 32 | Picture quality is nice for both anchor and modification. | no |
|  | 37 | Picture quality for modification is acceptable. Picture quality for modification is lower than acceptable. | yes |
|  | 41 | Can see moderate strength block artefacts for verified algorithm; but for anchor artefacts are stronger. Picture quality for anchor is unacceptable. | yes |
| ChinaSpeed, LDB-main | 32 | Picture quality is good for both anchor and modification. | no |
|  | 37 | Can see moderate strength block artefacts for verified algorithm; but for anchor artefacts are stronger. Main artefacts appear on dark area. Picture quality for anchor is unacceptable. | yes |
|  | 41 | Can see moderate strength block artefacts for verified algorithm; but for anchor artefacts are stronger. Main artefacts appear on dark area. Picture quality for anchor is unacceptable. | yes |

For RiverBed test sequence block artefacts are visible for HM8.0 under common test conditions even for QP=32. CE1-Test 6 doesn’t help visual quality improvement, but CE1-Test2 and CE1-Test4 make blockiness smaller.

Overall impression about next video sequence (RedKayak) tested in CE1 is following: block artefacts are very strong (especially at the last part of this sequence), verified algorithm makes them less visible, but visual quality still very bad.

For RiverBed test sequence block artefacts are visible for HM8.0 under common test conditions even for QP=32. CE1-Test 6 doesn’t help visual quality improvement, but CE1-test2 and CE1-test4 make blockingness smaller.

For WestWindEasy test sequence block artefacts are visible for HM8.0 under common test conditions for QP=37 and 41. Again there is no visual quality improvement from CE1-test 6, but both CE1-test2 and CE1-test4 make blockiness much less visible.

For class F sequence ChinaSpeed block artefacts are very strong on dark area of the picture. Both CE1-tes2 and CE1-test 4 algorithms reduce the strength of block artefacts, but block boundary still visible. According to the cross-checker, CE1-test4 provides the best visual quality for this particular case.

Check for degradation of subjective quality on Type 2 sequences:

Test 2 and Test 4 has been evaluated. No degradation of subjective quality was observed on Type 2 sequences.

JCTVC-K0187, A. Norkin (Ericsson), CE1: Cross-check of Test 3 (K-149, Sony) by Ericsson

Objective results and source code:

The cross-checker confirms the objective results. It is confirmed that source code exactly reflects the proposal. BD-rate matches with what provided by the proponent.

Subjective quality improvements on Type 1 sequences:

Test 3 has been evaluated on Type 1 sequences with tc_offset_div2 and beta_offset_div 2 set equal to 6.

Test 3 with max offsets reportedly performs better than HM8.0 with max offsets (e.g. on West Winds Easy and ChinaSpeed sequences).

Test 3 with max offsets was compared to Test 2 with max offset in order to evaluate the effect of modified beta table, when the maximum deblocking offsets are used. It has been found that the performance of Test 3 with max offsets is in general similar to the performance of Test 2 on the observed sequences. The difference between Test 2 and Test 3 on the evaluated sequences is visible but it is difficult to conclude, which of the tests has better quality when watched in motion. When the prediction is different, different parts of video sequence seem better or worse relative to each other.

Check for degradation of subjective quality on Type 2 sequences:

The test has not included comparison of the CTC sequences, since Test 3 without the offsets on common test conditions QPs is equivalent to Test 2.

JCTVC-K0212, W. Wan, P. Chen (Broadcom), CE1: Cross-check of Test 4

Objective results and source code:

This contribution reports cross-check results of CE1: deblocking filter test 4. The experiment results generated by the cross-checker match those provided by the proponent.

Subjective quality improvements on Type 1 sequences:

Check for degradation of subjective quality on Type 2 sequences:

JCTVC-K0267, E. Alshina, CE1: Cross-check for test 6

Objective results and source code:

Objective results match with proponent’s data.

BD-rate has been measured for Test 2 vs the anchor for Type 1 sequences using QP: 27,32,37,41. According to these test data verified algorithm shows noise level objective performance for additional test sequences and QP=27,32,37,41.

Subjective quality improvements on Type 1 sequences:

Test 6 in de-blocking filter CE is verified in this report. Subjective quality was investigated during this cross-check. Cross-checker has found no critical effect on visual quality from disabling Intra boundary smoothing for 32x32 PUs. In general, no difference has been found between anchor and CE1-test 6.

Check for degradation of subjective quality on Type 2 sequences:

JCTVC-K0151, S. Lu, O. Nakagami (Sony), CE1: Cross check of test 7 in CE1

Objective results and source code:

The cross-checker confirms the objective results. It is confirmed that source code exactly reflects the proposal. BD-rate matches with what provided by the proposer too.

Subjective performance on Type 1 sequences:

Test 7 has been evaluated at QP 37 and 41 on Type 1 sequences. It has been observed that Test 7 can reduce the block noises slightly in some of the observed sequences, but the improvement is not good enough.

For sequence Riverbed, WestWindEasy, and DucksTakeOff (QP 37 and 41) slightly better subjective quality is observed, but does not reach satisfactory level.

For sequence RedKayk (first 300 frames) and F.ChinaSpeed (QP 37 and 41), similar results with the HM8.0 anchor are observed.

The sequences-wise results are summarized in the following.

| sequence | condition | Test7 vs. HM8.0, TU32x32 |
| --- | --- | --- |
| riverbed | LB-Main, QP37 | Better than anchor But still blocky noises remains |
| WestWindEasy | LB-Main, QP37 | Better than anchor at the moving grasses part, but at the background, the blocky noises remains. |
| F.ChinaSpeed | LB-Main, QP37 | Same as anchor |
| RedKayak | RA-Main, QP37 | Only the first 300 frames are evaluated, and have same quality as anchor |
| DucksTakeOff | RA-Main, QP37 | For still pictures, blocky noises are reduced. But for video, the effect is not obvious |
| riverbed | LB-Main, QP41 | Slightly better than anchor But still blocky noises remains Quality is not satisfactory |
| WestWindEasy | LB-Main, QP41 | Better than anchor at the moving grasses part, but at the background, blocky noises remains. |
| F.ChinaSpeed | LB-Main, QP41 | Same as anchor |
| RedKayak | RA-Main, QP41 | Only the first 300 frames are evaluated, and have same quality as anchor |
| DucksTakeOff | RA-Main, QP41 | Slightly better than anchor But still blocky noises remains especially in the later part of the sequence(only water surface part) Quality is not satisfactory |

Check for degradation of subjective quality (on Type 2 sequences):

JCTVC-K0281, M. Budagavi, D.-K. Kwon (TI), 	CE1: Cross-check of Test 8, Test 9, Test 4, Test 5

Objective results and source code:

CE1 Test 8, Test 9, Test 4 and Test 5 have been evaluated. It is reported that there is a match to the BD-Rate results generated by the proponents.

Subjective performance on Type 1 sequences:

Test 8 and Test 4 have been subjectively evaluated. The quality improvements have mainly been observed in Ducks Take Off (RA and LD-B) and Riverbed sequences. It has been observed that some blocking artifacts still remain in the end of the Ducks Take Off sequence coded in LD-B.

Similar results have been observed for Test 8 with limiting the maximum transform size to 16x16.

Sequence-wise comments for Test 8 vs anchor are shown in the tables below. In these tables, “No visible improvement” means “hard to see difference clearly” or “negligible improvement” or “one of viewers could not see difference”. “No” in “Better than anchor” columns means “hard to see difference clearly” or “negligible improvement” or “one of viewers cannot see difference”.

| Test 8 vs HM-8.0, Max TU size = 32x32 |  |  |  |
| --- | --- | --- | --- |
|  | QP | Comments | Better than anchor (HM-8.0) |
| Ducks Takeoff, 25 fps, LB | 37 | Anchor contains visible block edge artifacts. For test, block edge artifacts are less visible in the beginning of the sequence but quite a few remain visible at end of sequence. | Some improvement |
| Ducks Takeoff, 25 fps, RA | 37 | Anchor contains visible block edge artifacts. For test, most of the block edge artifacts get smoothed | Yes |
| RedKayak, LB | 37 | Both anchor and test have significant blockiness. | No |
| RedKayak, RA | 37 | Both anchor and test have significant blockiness. | No |
| Riverbed | 37 | Anchor contains visible blocking artifacts. For test, a lot of the block edge artifacts get smoothed. However blockiness still exists at left part of video sequence. | Yes |
| WestWindEasy | 37 | Block edges are smoothed in test in first few frames. But blocking still remains very noticable in the center of the video through out the sequence. | No |
| ChinaSpeed | 37 | Blockiness in dark sky region in top center of the sequence for both anchor and test. (NB: In HM-7.0 version of the software, test showed better quality than anchor for Chinaspeed) | No |

| Test 4 v/s HM-8.0, Max TU size = 32x32 |  |  |  |
| --- | --- | --- | --- |
|  | QP | Comments | Better than anchor (HM-8.0) |
| Ducks Takeoff, 25 fps, LB | 37 | Anchor contains visible block edge artifacts. For test, block edge artifacts are less visible in the beginning of the sequence but quite a few remain visible at end of sequence. | Some improvement |
| Ducks Takeoff, 25 fps, RA | 37 | Anchor contains visible block edge artifacts. For test, most of the block edge artifacts get smoothed | Yes |
| RedKayak, LB | 37 | Both anchor and test have significant blockiness. | No |
| RedKayak, RA | 37 | Both anchor and test have significant blockiness. | No |
| Riverbed | 37 | Anchor contains visible blocking artifacts. For test, a lot of the block edge artifacts get smoothed. However blockiness still exists at left part of video sequence. | Yes |
| WestWindEasy | 37 | Block edges are smoothed in test in first few frames. But blocking still remains very noticable in the center of the video through out the sequence. | No |
| ChinaSpeed | 37 | Blockiness in dark sky region in top center of the sequence for both anchor and test. | No |

Similar results (to the tables above) are reported for Test 8.

Test 5 with maximum deblocking offsets was compared against Test 4 without the deblocking offsets. Test 9 with offsets was compared to Test 8 without the offsets. Quality improvements have been observed on Ducks Take Off (Low delay B) and slightly on Riverbed sequences. ChinaSpeed shows slightly higher flicker in sky region in Tests 5 and 9 compared to Test 8 and Test 4. Comment (CE coordinator): Comparing test 5 with the maximum offsets against test 4 without offsets does not make it clear whether the further improvement in quality is because of the modified beta table or just because of using the offsets).

Test 5 and Test 9 (with max-offsets) show slight improvement in video quality over HM-8.0 (with max-offsets) on Riverbed.

Check for degradation of subjective quality (on Type 2 sequences):

Test 4 and Test 8 video quality at QP=32 and QP=37 was compared against HM8.0 on all Type 2 sequences. No significant changes in quality were observed.

JCTVC-K0243, G. Van der Auwera (Qualcomm), CE1: Cross-check of JCTVC-K0149 and JCTVC-K0186

Objective results and source code:

The objective results for CE1 test 1 (JCTVC-K0149), and CE1 tests 8 and 9 (JCTVC-K0186) are reported and it is confirmed that there is a match with those reported in the respective proposals.

Subjective quality improvements on Type 1 sequences:

The cross-checker reports that Test 8 does not adequately remove the large block artifacts on two sequences. The problem of large block artifacts is not solved in the ‘Ducks Take Off’ and ‘Red Kayak’ sequences. Further investigation by the cross-checker revealed that the same problem exists with CE1 tests 2, 4, and 7. It is clear to the cross-checker that the sharp edges of the large block artifacts in Ducks Take Off are not reduced by any of the test configurations and that the large block artifacts in Red Kayak are still visible in all test configurations.

Tests 1 and 9 have been evaluated too. Cross-checking indicates that large block artifacts are removed in more cases and that the edges are somewhat smoother than the HM8.0 anchor (maximum deblocking offsets chosen in both cases). However, in some sequences such ‘Red Kayak’, large block artifacts are still visible. Oversmoothing is also observed, because maximum deblocking offsets are applied to all edges.

Example frames from “Ducks Take Off” sequence have been coded (LD-B, QP=37) with HM8.0, ‘test 1’, ‘test 9’, respectively. Maximum deblocking offsets are applied. The sharp block artifact edges are smoothed in all examples. ‘Test 1’ and ‘test 9’ are reported to be somewhat blurrier than the anchor.

Example frames from the “Red Kayak” sequence were coded (LD-B, QP=32) with HM8.0, ‘test 1’, ‘test 9’, respectively. Maximum deblocking offsets were applied. Large block artifact edges are reported in all examples. ‘Test 1’ and ‘test 9’ examples are somewhat blurrier than the anchor.

Check for degradation of subjective quality on Type 2 sequences:

Subjective quality degradation has been reported for Kimono sequence. The cross-checker clearly sees that the CE1 tests blur Kimono’s face and the roof compared with the anchor.

# Conclusions

In CE1, the proposals have been evaluated based on the common test conditions. Additionally, all the proposals were required to provide results for LD-P configurations. The results are summarized based on BD-rates and decoding time. The subjective and objective performance has been evaluated on Type 1 and Type 2 sequences and is reported by the cross-checkers.

# References

- A. Norkin, T. Suzuki, M. Budagavi, G. Van der Auwera, "Description of Core Experiment 1: Deblocking filter", JCTVC-J1101, Stockholm, SE, July 2012.

- T. Suzuki, "BoG Report: Report of subjective test on extended adaptability range of deblocking filter", JCTVC-J0567, Stockholm, Sweden, July 2012.

- S. Lu, O. Nakagami, M. Ikeda, T. Suzuki, "AHG6: On deblocking filter parameters", JCTVC-J0181, Stockholm, Sweden, July 2012.

- A. Norkin, "AHG6: Reduction of block artifacts in HEVC for large blocks", JCTVC-J0286, Stockholm, Sweden, July 2012.

- G. Van der Auwera, R. Joshi, M. Karczewicz, "AHG6: Transform Dependent Deblocking Strength", JCTVC-J0090, Stockholm, Sweden, July 2012.

- D.-K. Kwon, M. Budagavi, " Suppression of blocking artifacts at 32x32 transform boundaries", JCTVC-J0096, Stockholm, Sweden, July 2012

- "High Efficiency Video Coding (HEVC) text specification draft 7", JCTVC-J1003, Geneva, CH, May 2012.

- F. Bossen, "Common HM test conditions and software reference configurations", JCTVC-J1100, Geneva, CH, May 2012.