| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 11th Meeting: Shanghai, CN, 10-19 Oct., 2012 | Document: JCTVC-K0032 |
| --- | --- |

| Title: | Description of scalable video coding technology proposal by SHARP (proposal 2) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomoyuki Yamamoto, Yukinobu Yasugi, Hisao Kumai, Maki Takahashi 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba, JAPAN | Tel: Email: | +81 43 299 8696 yamamoto.tomoyuki@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution presents SHARP propoal in response to Call for Proposal on Scalable Video Coding Extension of HEVC. One significant characteristics of the proposed codec is that it only uses base-layer reconstruction data as inter-layer information. Thus it is capable of producing scalable bitstream in which base layer is encoded with non-HEVC codec such as MPEG-2 and AVC. The luma enhancement layer bd-rate reductions for HEVC 2x scalability are 34.1% and 26.5% for intra only and random access configuration respectively compared to HEVC higher layer anchor. For HEVC 1.5x scalability, they are 53.7% and 44.3%. For Hybrid 2.0x scalability, in which AVC base layer is used, the bd-rate reduction is 23.0% compared to HEVC higher layer anchor, and 49.4% compared to AVC higher layer anchor. For Hybrid 1.5x scalability, the one is 37.3% compared to HEVC, and 61.9% compared to AVC.

# Contents

Abstract	1

Contents	1

1	Introduction	3

2	Algorithm description	3

2.1	CTU/TU/PU partitioning	3

2.2	Up-sampling filters	4

2.2.1	UP-sampling filter characteristics	4

2.2.2	Adaptive filter prior to inter-layer prediction	4

2.3	Inter-layer prediction	4

2.3.1	Inter-layer prediction of CU/TU/PU partitions	5

2.3.2	Inter-layer residual prediction	5

2.3.3	Inter-layer motion vector prediction	5

2.3.4	Inter-layer mode prediction	5

2.3.5	Inter-layer texture prediction (ILTP)	5

2.3.6	Hybrid intra and inter-layer prediction (HIILP)	5

2.4	Inter-frame prediction	5

2.5	Intra-frame prediction	5

2.6	In-loop filtering	6

2.6.1	De-blocking filter	6

2.6.2	SAO	6

2.6.3	Other	6

2.7	Spatial transforms	6

2.8	Quantization	6

2.9	Motion/mode/parameter coding	6

2.9.1	Merge/skip	6

2.9.2	Motion vector coding	6

2.9.3	Intra prediction mode coding	6

2.9.4	SAO parameter coding	6

2.10	Entropy coding of residual / transform coefficients	6

2.11	High-layer syntax	6

3	Syntax and semantics description	6

3.1	Syntax and semantics for intra and inter-layer mode signaling	7

3.2	Syntax and semantics for adaptive filter prior to inter-layer prediction	7

4	Compression performance discussion	8

4.1	Category 1 (HEVC base layer)	8

4.1.1	Spatial scalability	8

4.1.2	Intra-only spatial scalability	9

4.1.3	SNR scalability	10

4.1.4	Overall	10

4.2	Category 2 (AVC base layer)	10

4.2.1	Spatial scalability	10

4.2.2	Overall	11

5	Complexity analysis	12

5.1	Encoding time and measurement methodology	12

5.2	Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM	13

5.3	Description of computing platform used to determine encoding and decoding times reported in sections 5.1 and 5.2	14

5.4	Expected memory usage of encoder	14

5.5	Expected memory usage of decoder	14

5.6	Complexity characteristics of encoder motion estimation and partitioning selection in enhancement layer(s)	14

5.7	Complexity characteristics of decoder motion compensation in enhancement layer(s)	14

5.8	Complexity characteristics of encoder intra-frame prediction type and partitioning selection in enhancement layer(s)	15

5.9	Complexity characteristics of decoder intra-frame prediction operation in enhancement layer(s)	15

5.10	Complexity characteristics of upsampling filters and transforms specific in enhancement layer(s)	15

5.11	Complexity characteristics of quantization and inverse quantization in enhancement layer(s)	15

5.12	Complexity characteristics of encoder entropy coding operation in enhancement layer(s)	15

5.13	Complexity characteristics of decoder entropy decoding operation in enhancement layer(s)	15

5.14	Degree of capability for encoder parallel processing	15

5.15	Degree of capability for decoder parallel processing	15

6	Software implementation description	15

7	Highlighted aspects discussion	16

7.1	Support of base layer bitstream other than HEVC	16

8	Closing remarks	16

9	Patent rights declaration(s)	17

References	17

# Introduction

This document describes a SHARP proposal in response to Call for Proposal on Scalable Video Coding Extension of HEVC [1]. The proposed codec is designed so that it supports the scenario where base layer is encoded with non-HEVC codec. In order to provide such functionality, the proposed codec has the characteristic that only the reconstructed pixel data of base layer can be used as inter-layer information.

The proposed codec is developed by enhancing HEVC reference software (HM-6.1.1) [2]. Figure 1 illustrates the decoder structure. Additional coding tools include up-sampler applied to base layer reconstruction, adaptive filter prior to inter-layer prediction, inter-layer texture prediction, and hybrid intra and inter-layer texture prediction. Details of those tools are introduced in the following section.

Figure 1: Decoder structure

Indicate the category/categories the proposal covers (tick box )

 	Category 1 (HEVC base layer) spatial scalability

 	Category 1 (HEVC base layer) intra-only spatial scalability

 	Category 1 (HEVC base layer) SNR scalability

 	Category 2 (AVC base layer) spatial scalability

 	The proposal obeys the constraints under section 5 of the CfP (if box is not ticked, explain cases where constraints are violated)

# Algorithm description

The details of components of the proposed codec are described in this section. Those which are not described in this section are the same as the corresponding parts of the base design (HM-6.1.1).

## CTU/TU/PU partitioning

Basically the same as HM-6.1.1, including CTB size, and max. and min. CU/TU/PU sizes. Asymmetric motion partition (AMP) implemented in HM-6.1.1 is enabled.

## Up-sampling filters

As shown in Figure 1, up-sampling filter is applied to the base layer reconstruction picture to make the picture resolution the same as that of the enhancement layer. After up-sampling, an adaptive filter is applied to make base-layer reference picture which is used in inter-layer texture prediction.

### UP-sampling filter characteristics

The up-sampling filter is a separate FIR filter. The number of taps is eight for luma and four for chroma. The coefficients are as follows;

{-1, 4, -11, 40, 40, -11, 4, -1} for luma (2x cases),

{-4, 36, 36, -4} for chroma (2x),

{-1, 4, -9, 26, 52,-12,5,-1} for luma (1.5x),

{-5,25,51,-7} for chroma (1.5x).

### Adaptive filter prior to inter-layer prediction

Adaptive Filter (AF) is applied to the up-sampled base-layer reconstruction. In AF, 5x5 square shape filter characterized by 13 coefficients is used to each pixel. Figure 2 shows the layout of coefficients in the filtering area. Filtering can be turned on and off in a unit called AF unit. The AF units are represented by quad-tree structure, of which max depth is equal to 3. In other words, the minimum AF unit size is 1/8 of picture both in width and height.

| c0 | c1 | c2 | c3 | c4 |
| --- | --- | --- | --- | --- |
| c5 | c6 | c7 | c8 | c9 |
| c10 | c11 | c12 | c11 | c10 |
| c9 | c8 | c7 | c6 | c5 |
| c4 | c3 | c2 | c1 | c0 |

Figure 2: Filter shape of adaptive filter

In AF, split flags for AF unit, the coefficients, and filter-enabling flags are signaled in the first slice header of a picture without any inter-frame prediction. Those flags are signaled at each color component independently. Please also refer to syntax section (3.2).

In the encoding process, the filter coefficients are calculated for each color component from the original enhancement-layer image and the up-sampled base-layer picture. After calculating coefficients, the filter-enabling flags are determined on AF unit basis by comparing SAD values before and after applying the filter. Finally, the filter coefficients are re-calculated from the pixels of only enabled AF units.

## Inter-layer prediction

Only reconstructed pixel data is used for inter-layer prediction. Inter-layer texture prediction (ILTP) and Hybrid intra and inter-layer prediction (HIILP) are introduced. In intra CU, ILTP and HIILP can be selected in addition to original intra modes (i.e. DC, Planar, and Angular modes). A new flag, inter_layer_pred_flag, is added to intra CU syntax. Signaling of prediction modes in intra CU is summarized in Table 1. Whereas IntraPredMode is signaled per prediction block, inter_layer_pred_flag is signaled per CU. A flag, inter_layer_pred_flag, is encoded using CABAC with one single context.

Table 1: Signaling of prediction modes in intra CU

| inter_layer_pred_flag | IntraPredMode | Associated prediction mode |
| --- | --- | --- |
| 0 | 0 | Planar |
| 0 | 1 | DC |
| 0 | 2..34 | Angular |
| 1 | 1 | ILTP |
| 1 | 0, 2..34 | HIILP |

### Inter-layer prediction of CU/TU/PU partitions

No partition information is referred from base layer.

### Inter-layer residual prediction

No partition information is referred from base layer.

### Inter-layer motion vector prediction

No motion information is referred from base layer.

### Inter-layer mode prediction

No mode information is referred from base layer.

### Inter-layer texture prediction (ILTP)

Inter-layer texture prediction (ILTP) is a prediction mode where base layer reference picture is directly used as prediction image. The pixel values of the reference region on base layer reference picture may be filtered with Adaptive Filter. The reference region is collocated to the target prediction block thus additional information, such as the displacement vector, is not needed.

### Hybrid intra and inter-layer prediction (HIILP)

Hybrid intra and inter-layer prediction (HIILP) is a prediction mode where the prediction image created by either Planar or Angular mode is modified based on the average pixel values of the reference region on base layer reference picture. The modification is done by 4x4 unit basis. Conceptually, the original prediction image is modified so that the DC component of each 4x4 unit is replaced by the DC component of the corresponding 4x4 unit on base layer reference picture. Specifically, the prediction image of HIILP mode, P(x,y), is derived by the following equation.

P(x,y) = Clip3(0, 255, Po(x,y) - Avg4x4(Po, x,y) + Avg4x4(R, x,y))

Where Po (x,y) is pixel values of prediction image by either Planar or Angular modes. The mode used to derive Po (x,y) is determined based on IntraPredMode value. Avg4x4(Po, x,y) is the average pixel value of 4x4 region on prediction image Po. The top-left pixel position of the 4x4 region is ((x>>2)<<2, (y>>2)<<2). Similarly, Avg4x4(R, x, y) is the average pixel value of 4x4 region on base layer reference picture R.

## Inter-frame prediction

The same as HM-6.1.1.

## Intra-frame prediction

The same as HM-6.1.1.

## In-loop filtering

### De-blocking filter

The same as HM-6.1.1.

### SAO

The same as HM-6.1.1.

### Other

Adaptive loop filter implemented in HM-6.1.1 is enabled.

## Spatial transforms

The same as HM-6.1.1.

## Quantization

The same as HM-6.1.1 including RDOQ.

## Motion/mode/parameter coding

The same as HM-6.1.1.

### Merge/skip

The same as HM-6.1.1.

### Motion vector coding

The same as HM-6.1.1.

### Intra prediction mode coding

To introduce inter-layer prediction mode, a new flag, inter_layer_pred_flag, is added to intra CU syntax  (2.3.5 and 3.1). Syntax and decoding process for conventional intra prediction modes (i.e. DC, Planar, and Angular) are the same as HM-6.1.1.

## Entropy coding of residual / transform coefficients

The same as HM-6.1.1.

## High-layer syntax

New NAL unit type is used to contain VLC data of enhancement layer.

# Syntax and semantics description

Following syntax are different from HEVC. Rewriting is not possible.

## Syntax and semantics for intra and inter-layer mode signaling

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| ... |  |
| if( slice_type != I ) |  |
| skip_flag[ x0 ][ y0 ] | ae(v) |
| if( skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, log2CbSize ) |  |
| else { |  |
| nCbS = ( 1 << log2CbSize ) |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if ( PredMode[ x0 ][ y0 ] = = MODE_INTRA ) |  |
| inter_layer_pred_flag[ x0 ][ y0 ] | ae(v) |
| if( PredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ log2CbSize = = Log2MinCbSize ) |  |
| part_mode | ae(v) |
| ... |  |
| } |  |
| } |  |

inter_layer_pred_flag[ x0 ][ y0 ] equal to 1 specifies base-layer reference picture is referred in order to derive prediction image for each prediction block included in the current coding unit. inter_layer_pred_flag[ x0 ][ y0 ] equal to 0 specifies base-layer reference picture is not referred in order to derive prediction image for any prediction block included in the current coding unit. When inter_layer_pred_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

## Syntax and semantics for adaptive filter prior to inter-layer prediction

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| ... |  |
| if (first_slice_in_pic_flag) |  |
| { |  |
| base_layer_filter_flag_qtree(0, 0, 0) |  |
| if (base_layer_filter_split_flag[ 0 ][ 0 ][ 0 ] ¦¦ base_layer_filter_flag[ 0 ][ 0 ]) |  |
| for (cIdx=0; cIdx<=2; cIdx++) |  |
| for ( i=0; i<12; i++) |  |
| base_layer_filter_coeff[ cIdx ][ i ] | se(v) |
| } |  |
| ... |  |
| } |  |

| base_layer_filter_flag_qtree(depth, x, y) { | Descriptor |
| --- | --- |
| MaxDepth = 3 |  |
| if ( depth < MaxDepth ) |  |
| base_layer_filter_split_flag[ depth ][ x ][ y ] | u(1) |
| if ( base_layer_filter_split_flag[ depth ][ x ][ y ] ) { |  |
| subSize = ( 1 << (MaxDepth - (depth+1)) ) |  |
| base_layer_filter_flag_qtree(depth+1, x, y) |  |
| base_layer_filter_flag_qtree(depth+1, x+subSize, y) |  |
| base_layer_filter_flag_qtree(depth+1, x, y+subSize) |  |
| base_layer_filter_flag_qtree(depth+1, x+subSize, y+subSize) |  |
| } |  |
| else |  |
| base_layer_filter_flag[ x ][ y ] | u(1) |
| } |  |

base_layer_filter_split_flag[ depth ][ x ][ y ] specifies whether a base-layer filter unit is split into base-layer filter units with half horizontal and vertical size. The array indices x, y specify the location ( x, y ) relative to the top-left base-layer filter unit in picture in unit of minimum AF unit. The array index depth specify the depth of the node in quad tree to which a considered base-layer filter unit belongs. When base_layer_filter_split_flag[ depth ][ x ][ y ] is not present, its value is inferred to be equal to 0.

base_layer_filter_flag[ x ][ y ] equal to 1 specifies base-layer filter is applied for a unit identified by the array index x, y. base_layer_filter_flag[ x ][ y ] equal to 0 specifies base-layer filter is not applied for a unit identified by x, y. When base_layer_filter_flag[ x ][ y ] is not present, its value is inferred to be equal to 0.

base_layer_filter_coeff[ cIdx ][ i ] specifies the base-layer filter coefficients for cIdx color component. The association between the array index i and coefficient position are shown in Figure 2.

# Compression performance discussion

## Category 1 (HEVC base layer)

### Spatial scalability

Table 4.1.1.1 EL-only actual BD-rate (spatial scalability)

|  | Random Access HEVC 2x |  |  | Random Access HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -28.8% | -16.4% | -16.4% |  |  |  |
| Class B | -24.2% | -15.7% | -13.2% | -44.3% | -36.8% | -34.2% |
| Overall | -26.5% | -16.0% | -14.8% | -44.3% | -36.8% | -34.2% |
|  | -26.5% | -16.2% | -14.9% | -44.5% | -36.9% | -34.5% |

Table 4.1.1.2 EL-only target BD-rate (spatial scalability)

|  | Random Access HEVC 2x |  |  | Random Access HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -28.3% | -15.8% | -15.8% |  |  |  |
| Class B | -23.9% | -15.3% | -12.7% | -44.1% | -36.5% | -33.9% |
| Overall | -26.1% | -15.6% | -14.3% | -44.1% | -36.5% | -33.9% |
|  | -26.1% | -15.7% | -14.4% | -44.2% | -36.6% | -34.3% |

Table 4.1.1.3 EL+BL actual BD-rate (spatial scalability)

|  | Random Access HEVC 2x |  |  | Random Access HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -18.8% | -8.4% | -8.4% |  |  |  |
| Class B | -15.8% | -8.5% | -6.3% | -25.4% | -19.0% | -16.9% |
| Overall | -17.3% | -8.4% | -7.3% | -25.4% | -19.0% | -16.9% |
|  | -17.3% | -8.5% | -7.4% | -25.4% | -19.0% | -16.9% |

Table 4.1.1.4 EL+BL target BD-rate (spatial scalability)

|  | Random Access HEVC 2x |  |  | Random Access HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -18.4% | -8.0% | -8.0% |  |  |  |
| Class B | -15.5% | -8.2% | -6.0% | -25.2% | -18.8% | -16.8% |
| Overall | -17.0% | -8.1% | -7.0% | -25.2% | -18.8% | -16.8% |
|  | -17.0% | -8.2% | -7.1% | -25.2% | -18.8% | -16.7% |

### Intra-only spatial scalability

Table 4.1.2.1 EL-only actual BD-rate (spatial scalability)

|  | All Intra HEVC 2x |  |  | All Intra HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -37.5% | -31.2% | -32.5% |  |  |  |
| Class B | -30.7% | -23.2% | -23.7% | -53.7% | -49.8% | -50.5% |
| Overall | -34.1% | -27.2% | -28.1% | -53.7% | -49.8% | -50.5% |
|  | -34.1% | -27.1% | -27.9% | -53.9% | -50.4% | -51.4% |

Table 4.1.2.2 EL-only target BD-rate (spatial scalability)

|  | All Intra HEVC 2x |  |  | All Intra HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -37.2% | -31.0% | -32.2% |  |  |  |
| Class B | -30.4% | -22.9% | -23.4% | -53.5% | -49.6% | -50.3% |
| Overall | -33.8% | -26.9% | -27.8% | -53.5% | -49.6% | -50.3% |
|  | -33.9% | -26.8% | -27.6% | -53.7% | -50.3% | -51.2% |

Table 4.1.2.3 EL+BL actual BD-rate (spatial scalability)

|  | All Intra HEVC 2x |  |  | All Intra HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -25.7% | -20.5% | -21.6% |  |  |  |
| Class B | -20.9% | -14.5% | -15.0% | -31.8% | -28.5% | -29.2% |
| Overall | -23.3% | -17.5% | -18.3% | -31.8% | -28.5% | -29.2% |
|  | -23.3% | -17.4% | -18.2% | -31.8% | -28.6% | -29.4% |

Table 4.1.2.4 EL+BL target BD-rate (spatial scalability)

|  | All Intra HEVC 2x |  |  | All Intra HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -25.5% | -20.3% | -21.5% |  |  |  |
| Class B | -20.7% | -14.3% | -14.8% | -31.7% | -28.4% | -29.1% |
| Overall | -23.1% | -17.3% | -18.1% | -31.7% | -28.4% | -29.1% |
|  | -23.1% | -17.2% | -18.0% | -31.7% | -28.5% | -29.2% |

### SNR scalability

### Overall

Figure 4.1.4.1 summarizes luma BD-rate for Category 1.

a) Spatial scalability

b) Intra only scalability

Figure 4.1.4.1 Summary of luma BD-rate (Category 1)

## Category 2 (AVC base layer)

### Spatial scalability

Table 4.2.1.1 EL-only actual BD-rate (AVC base layer)

|  | Hybrid vs HEVC 2x |  |  | Hybrid vs HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -25.4% | -13.0% | -13.1% |  |  |  |
| Class B | -20.6% | -12.3% | -9.7% | -37.3% | -29.0% | -26.0% |
| Overall | -23.0% | -12.6% | -11.4% | -37.3% | -29.0% | -26.0% |
|  | -23.0% | -12.7% | -11.5% | -37.4% | -29.2% | -26.3% |

|  | Hybrid vs AVC 2x |  |  | Hybrid vs AVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -47.3% | -36.2% | -37.2% |  |  |  |
| Class B | -51.5% | -47.0% | -45.1% | -62.1% | -58.1% | -56.1% |
| Overall | -49.4% | -41.5% | -41.2% | -61.9% | -58.0% | -56.0% |
|  | -49.4% | -41.6% | -41.2% | -62.1% | -58.1% | -56.1% |

Table 4.2.1.2 EL-only target BD-rate (AVC base layer)

|  | Hybrid vs HEVC 2x |  |  | Hybrid vs HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -24.9% | -12.4% | -12.6% |  |  |  |
| Class B | -20.3% | -11.9% | -9.3% | -37.0% | -28.6% | -25.6% |
| Overall | -22.6% | -12.2% | -10.9% | -37.0% | -28.6% | -25.6% |
|  | -22.6% | -12.2% | -11.0% | -37.1% | -28.8% | -26.0% |

|  | Hybrid vs AVC 2x |  |  | Hybrid vs AVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -47.0% | -35.8% | -36.8% |  |  |  |
| Class B | -51.3% | -46.7% | -44.9% | -61.8% | -57.8% | -55.7% |
| Overall | -49.1% | -41.2% | -40.8% | -61.8% | -57.8% | -55.7% |
|  | -49.2% | -41.3% | -40.9% | -61.9% | -57.9% | -55.9% |

Table 4.2.1.3 EL+BL actual BD-rate (AVC base layer)

|  | Hybrid vs HEVC 2x |  |  | Hybrid vs HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -15.5% | -5.4% | -5.5% |  |  |  |
| Class B | -12.4% | -5.5% | -3.3% | -18.9% | -12.4% | -10.1% |
| Overall | -13.9% | -5.4% | -4.4% | -18.9% | -12.4% | -10.1% |
|  | -13.9% | -5.5% | -4.4% | -19.0% | -12.4% | -10.1% |

|  | Hybrid vs AVC 2x |  |  | Hybrid vs AVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -33.0% | -22.7% | -23.6% |  |  |  |
| Class B | -38.7% | -34.1% | -32.4% | -41.3% | -36.9% | -35.0% |
| Overall | -35.8% | -28.4% | -28.0% | -41.3% | -36.9% | -35.0% |
|  | -35.9% | -28.4% | -28.0% | -41.3% | -37.0% | -35.0% |

Table 4.2.1.4 EL+BL target BD-rate (AVC base layer)

|  | Hybrid vs HEVC 2x |  |  | Hybrid vs HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -15.2% | -5.0% | -5.1% |  |  |  |
| Class B | -12.2% | -5.3% | -3.1% | -18.8% | -12.2% | -9.9% |
| Overall | -13.7% | -5.1% | -4.1% | -18.8% | -12.2% | -9.9% |
|  | -13.7% | -5.2% | -4.1% | -18.8% | -12.2% | -9.9% |

|  | Hybrid vs AVC 2x |  |  | Hybrid vs AVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -32.7% | -22.5% | -23.4% |  |  |  |
| Class B | -38.6% | -33.9% | -32.2% | -41.2% | -36.8% | -34.9% |
| Overall | -35.6% | -28.2% | -27.8% | -41.2% | -36.8% | -34.9% |
|  | -35.7% | -28.2% | -27.8% | -41.2% | -36.9% | -34.9% |

### Overall

Figure 4.2.2.1 summarizes luma BD-rate for Category 2.

a) Hybrid vs HEVC

b) Hybrid vs AVC

Figure 4.2.2.1 Summary of luma BD-rate (Category 2)

# Complexity analysis

## Encoding time and measurement methodology

Encoding time was taken from encoder logs (the line of “Total Time”). All encoding jobs were executed on a PC cluster environment. Table 5.1.1 and 5.1.2 show the comparison of encoding time between proposal and anchor. In the tables, the encoding time of the proposed method were taken from the value encoded with the lowest QP values, whereas the encoding time of the anchor method were taken from the value encoded with QP 22.

The increment of the encoding time in the proposed method is about 2 % and it’s mainly due to the enabling AMP and ALF and the introduction of new prediction modes with inter-layer information, ILTP and HIILP.

Table 5.1.1 Encoding time comparison (All intra)

| All Intra | anchor[s] | proposed [s] |  |  | ratio to anchor |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | HEVC 2x | HEVC 1.5x |  | HEVC 2x | HEVC 1.5x |  |
| Traffic QP 22 | 13566.9 | 24280.4 |  |  | 1.79 |  |  |
| PeopleOnStreet QP 22 | 10606.0 | 26218.5 |  |  | 2.47 |  |  |
| Kimono QP 22 | 3715.6 | 9760.6 | 9891.9 |  | 2.63 | 2.66 |  |
| ParkScene QP 22 | 4467.0 | 10902.8 | 10968.9 |  | 2.44 | 2.46 |  |
| Cactus QP 22 | 9697.0 | 25519.0 | 23425.3 |  | 2.63 | 2.42 |  |
| BasketballDrive QP 22 | 8874.3 | 21799.9 | 21553.1 |  | 2.46 | 2.43 |  |
| BQTerrace QP 22 | 12251.8 | 28198.2 | 28204.2 |  | 2.30 | 2.30 |  |
| GEO MEAN | 8211.7 | 19475.5 | 17289.8 |  | 2.37 | 2.11 |  |

Table 5.1.2 Encoding time comparison (Random access)

| Random access | anchor[s] | proposed [s] |  |  | ratio to anchor |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | HEVC 2x | HEVC 1.5x | Hybrid1.5x | HEVC 2x | HEVC 1.5x | Hybrid1.5x |
| Traffic QP 22 | 24416.1 | 36856.2 |  |  | 1.51 |  |  |
| PeopleOnStreet QP 22 | 35540.3 | 58558.2 |  |  | 1.65 |  |  |
| Kimono QP 22 | 11943.4 | 18413.7 | 20249.1 | 18421.9 | 1.54 | 1.70 | 1.54 |
| ParkScene QP 22 | 10868.5 | 16685.5 | 16905.2 | 16708.2 | 1.54 | 1.56 | 1.54 |
| Cactus QP 22 | 26153.0 | 42122.4 | 41631.1 | 41623.4 | 1.61 | 1.59 | 1.59 |
| BasketballDrive QP 22 | 31846.5 | 46260.2 | 45094.6 | 45126.7 | 1.45 | 1.42 | 1.42 |
| BQTerrace QP 22 | 34230.4 | 59568.7 | 62396.9 | 58813.7 | 1.74 | 1.82 | 1.72 |
| GEO MEAN | 22808.6 | 35907.7 | 33160.9 | 32085.0 | 1.57 | 1.45 | 1.41 |

## Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM

Decoding time was taken from decoder logs (the line of “Total Time”). Table 5.2.1 and 5.2.2 show the comparison of decoding time between the proposed method and anchor. All decoding times in the tables were measured on the same node of a PC cluster. The decoding time of the lowest QP values is used as in the comparison of the decoding time.

Decoding time of the proposed method is longer than anchor. Construction of base-layer reference picture (up-sampling and adaptive filtering), inter-layer prediction (ILTP and HIILP), AMP, and ALF are considered to make this difference. Note that the use of different QP value and the different amount of coefficients data also may contribute to the difference.

Table 5.2.1 Decoding time comparison (All intra)

| All Intra | anchor[s] | proposed [s] |  |  | ratio to anchor |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | HEVC 2x | HEVC 1.5x |  | HEVC 2x | HEVC 1.5x |  |
| Traffic QP 22 | 202.8 | 233.9 |  |  | 1.15 |  |  |
| PeopleOnStreet QP 22 | 125.0 | 270.0 |  |  | 2.16 |  |  |
| Kimono QP 22 | 36.0 | 84.1 | 93.3 |  | 2.34 | 2.59 |  |
| ParkScene QP 22 | 55.0 | 97.9 | 116.7 |  | 1.78 | 2.12 |  |
| Cactus QP 22 | 114.2 | 207.5 | 218.6 |  | 1.82 | 1.91 |  |
| BasketballDrive QP 22 | 102.0 | 205.4 | 219.1 |  | 2.01 | 2.15 |  |
| BQTerrace QP 22 | 154.1 | 264.6 | 279.3 |  | 1.72 | 1.81 |  |
| GEO MEAN | 98.5 | 178.9 | 170.9 |  | 1.82 | 1.73 |  |

Table 5.2.2 Decoding time comparison (Random access)

| Random access | anchor[s] | proposed [s] |  |  | ratio to anchor |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | HEVC 2x | HEVC 1.5x | Hybrid1.5x | HEVC 2x | HEVC 1.5x | Hybrid1.5x |
| Traffic QP 22 | 53.0 | 179.6 |  |  | 3.39 |  |  |
| PeopleOnStreet QP 22 | 85.4 | 222.8 |  |  | 2.61 |  |  |
| Kimono QP 22 | 22.9 | 76.7 | 86.4 | 86.6 | 3.34 | 3.77 | 3.78 |
| ParkScene QP 22 | 25.2 | 77.2 | 86.5 | 87.2 | 3.07 | 3.44 | 3.46 |
| Cactus QP 22 | 52.6 | 166.5 | 185.3 | 186.5 | 3.17 | 3.53 | 3.55 |
| BasketballDrive QP 22 | 58.9 | 168.8 | 187.7 | 188.3 | 2.87 | 3.19 | 3.20 |
| BQTerrace QP 22 | 82.6 | 224.1 | 245.0 | 246.8 | 2.71 | 2.97 | 2.99 |
| GEO MEAN | 48.9 | 147.1 | 144.8 | 145.6 | 3.01 | 2.96 | 2.98 |

## Description of computing platform used to determine encoding and decoding times reported in sections 5.1 and 5.2

We use a PC cluster environment for encoding and decoding time evaluation. The specifications of nodes are as follows.

CPU: Intel Xeon 5667 (3.07GHz quad core)

Memory: 24GB

OS: ubuntu(lucid) 64b

Compiler: GCC 4.4.3 (-O3)

## Expected memory usage of encoder

In addition to the frame buffers required in HM-6.1.1, the following frame buffers are needed:

1. Frame buffers to store base-layer reconstruction for reordering. The number of frame buffers needed depends on GOP structures of both base and enhancement layers. The size of each frame buffer is the resolution of base layer.

2. Frame buffer to store base-layer reference picture. The size of the frame buffer is the resolution of enhancement layer.

## Expected memory usage of decoder

The same additional frame buffers as in encoder are required.

## Complexity characteristics of encoder motion estimation and partitioning selection in enhancement layer(s)

The complexity is equivalent to that of HM-6.1.1 since available inter frame prediction modes as well as motion search algorithm are the same as HM-6.1.1.

## Complexity characteristics of decoder motion compensation in enhancement layer(s)

The complexity is equivalent to HM-6.1.1.

## Complexity characteristics of encoder intra-frame prediction type and partitioning selection in enhancement layer(s)

On prediction type selection, the number of types to be checked is almost doubled compared to HM-6.1.1. In addition to the tests with inter_layer_pred_flag equal to 0, which corresponds to the conventional intra-frame prediction tests, the tests with inter_layer_pred_flag equal to 1 are conducted. Specifically, 1 modes for ILTP and 34 modes for HIILP are tested.

On partitioning selection, the complexity is competitive to HM-6.1.1.

## Complexity characteristics of decoder intra-frame prediction operation in enhancement layer(s)

On ILTP, prediction image is just copying collocated base-layer reference picture samples. Note that up-sampling and adaptive filtering may be needed to construct base-layer reference picture.

On HIILP, it requires additional DC adjustment operation in addition to the conventional HEVC intra prediction. It is more complex compared to HEVC intra prediction, but additional operations are less complex compared to inter frame prediction.

## Complexity characteristics of upsampling filters and transforms specific in enhancement layer(s)

Up-sampling filter is fixed 8 tap filter for luma, and 4 tap for chroma. As an adaptive filter prior to inter-layer prediction, 5x5 tap 2D filter is applied to create base-layer reference picture.

On transforms, the complexity is equivalent to HM-6.1.1.

## Complexity characteristics of quantization and inverse quantization in enhancement layer(s)

The complexity is equivalent to HM-6.1.1.

## Complexity characteristics of encoder entropy coding operation in enhancement layer(s)

The complexity is equivalent to that of HM-6.1.1. The additional computing process to encode additional syntax elements are negligible.

## Complexity characteristics of decoder entropy decoding operation in enhancement layer(s)

The complexity is equivalent to that of HM-6.1.1. The additional computing process to parse additional syntax elements are negligible.

## Degree of capability for encoder parallel processing

Similar parallel processing capability compared to HM-6.1.1 in condition that base-layer reconstruction is provided timely.

## Degree of capability for decoder parallel processing

Similar parallel processing capability compared to HM-6.1.1 in condition that base-layer reconstruction is provided timely.

# Software implementation description

The software is written in C++.

# Highlighted aspects discussion

## Support of base layer bitstream other than HEVC

The proposed codec supports combination of different coding standard with scalability. MPEG-2 base layer, AVC base layer, and other types of base layers could be used. In the proposed codec, this is achieved by Scalability using Base Layer Reconstructed pixel only mode (BLR mode). The basic idea of BLR mode is to use only base layer reconstructed pixel values as the information from base layer. Other data such as motion and coefficient data are not referenced. An example high level architecture of the proposed decoder is shown in Figure 7.1.1. More discussions on BLR mode can be found in MPEG contribution m25749 [3].

One potential application of BLR mode is hybrid TV, which signals enhancement layer bitstream encoded by HEVC scalable extension on top of existing base layer bitstream encoded by existing standard such as MPEG-2 video.

Figure 7.1.2 shows one use cases of hybrid TV. In this use case, the original input video is 4K2K (4K) video. It is designed to provide 4K video services using existing broadcast HD video as the base layer (BL). The BL input is generated by down sampling the input picture (4K => HD). The BL bitstream is transmitted to decoder via broadcast network, such as terrestrial, cable, and satellite without changes of existing broadcast system. In contrast, the enhancement layer (EL) bitstream is transmitted via internet. This EL bitstream can be signaled in lower bitrate compared to single 4K HEVC bitstream.

Figure 7.1.1: Scalable decoder block diagram

Figure 7.1.2: 4K2K Hybrid TV use case

# Closing remarks

This contribution presents SHARP proposal in response to Call for Proposal on Scalable Video Coding Extension of HEVC. One significant characteristics of the proposed codec is that it only uses base-layer reconstruction data as inter-layer information. With relatively simple codec architecture, it is reported that the proposed codec achieves significant gains compared to HEVC higher layer anchor while being capable of supporting non-HEVC base-layer such as AVC and MPEG-2. It is recommended to develop scalable video coding technology based on the proposed codec in consideration of the support of non-HEVC base layer.

# Patent rights declaration(s)

SHARP Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] “Joint Call for Proposals on Scalable Video Coding Extensions of High Efficiency Video Coding (HEVC)”

[2] HEVC reference software (HM-6.1.1)

[3] H. Kumai, T. Yamamoto, A. Segall, M. Takahashi, Y. Yasugi, and S. Watanabe, “Proposals for HEVC scalability Extension,” ISO/IEC JTC1/SC29/WG11 MPEG2012/m25749, July 2012, Stockholm, Sweden