| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 11th Meeting: Shanghai, CN, 10-19 Oct., 2012 | Document: JCTVC-K0034_r1 |
| --- | --- |

| Title: | Description of scalable video coding technology proposal by InterDigital Communications |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Dong, Yong He, Yuwen He, George McClellan, Eun-Seok Ryu, Xiaoyu Xiu, Yan Ye 9710 Scranton R-D, #250 San Diego, CA 92121 USA | Tel: Email: | +1-858-210-4803 yan.ye@interdigital.com |
| Source: | InterDigital Communications, LLC |  |  |

_____________________________

# Abstract

This proposal is InterDigital Communications’ response to the Call for Proposals (CfP) on scalable video coding extensions of HEVC, jointly issued by ITU-T SG16 Q.6 (VCEG) and ISO/IEC JTC1/SC29/WG11 (MPEG). The goal of this proposal is to provide a scalable video compression technology which has significantly higher compression capability than a simulcast solution which simply compresses the two or more layers of video inputs separately. To achieve this goal, a number of new algorithmic tools are proposed, primarily focusing on improving inter layer prediction at the picture level. The picture level inter layer prediction tools include selectable upsampling filters, intra mode dependent directional filters, motion field mapping, enhancement layer skipped slice, and adaptive reference picture placement. The inter layer reference is then used together with the temporal reference pictures to predict the enhancement layer picture. The enhancement layer encoder and decoder are kept primarily the same as the single layer HEVC codec, with only one additional in-loop filter, called edge enhancement filter, applied at the picture level, whereas block level operations in the enhancement layer are kept exactly the same as the single layer HEVC codec. The proposed video codec achieves approximately {34%, 55%, 25%, 45%, 33%} bit rate savings in the enhancement layer for {AI 2x, AI 1.5x, RA 2x, RA 1.5x, RA SNR} scalability case in category 1, and {24%, 41%} bit rate savings for {RA 2x, RA 1.5x} scalability case in category 2. The average decoding time for the proposed system was measured to be between about 1.2 and 3.0 times that of the corresponding anchors for category 1, and between about 2.6 and 4.6 times that of the corresponding anchors for category 2.

# Contents

Abstract	1

Contents	2

Glossary of terms used in this proposal	4

1	Introduction	5

2	Algorithm description	7

2.1	CTU/TU/PU partitioning	7

2.2	Up-sampling filters	7

2.2.1	Default upsampling filters	7

2.2.2	Selectable upsampling filters (SUF)	7

2.2.3	Intra mode dependent directional filters (IMDDF)	9

2.3	Inter-layer prediction	12

2.3.1	Inter-layer prediction of CU/TU/PU partitions	12

2.3.2	Inter-layer residual prediction	12

2.3.3	Inter-layer motion vector prediction: motion field mapping (MFM)	12

2.3.4	Inter-layer mode prediction	13

2.3.5	Other: Enhancement Layer Skipped Slice (ELSkip)	13

2.3.6	Other: Adaptive Reference Picture Placement (ARPP)	13

2.4	Inter-frame prediction	14

2.5	Intra-frame prediction	14

2.6	In-loop filtering	14

2.6.1	De-blocking filter	14

2.6.2	SAO	14

2.6.3	Other: Edge Enhancement Filter (EEF)	14

2.7	Spatial transforms	16

2.8	Quantization	16

2.9	Motion/mode/parameter coding	16

2.9.1	Merge/skip	16

2.9.2	Motion vector coding	16

2.9.3	Intra prediction mode coding	16

2.9.4	SAO parameter coding	17

2.9.5	Other: EEF parameter coding	17

2.10	Entropy coding of residual / transform coefficients	17

2.11	High-layer syntax	17

2.12	Picture level multiple pass encoding and early termination	17

2.13	AVC to HEVC scalability	18

3	Syntax and semantics description	18

3.1	Syntax	18

3.1.1	NAL unit header syntax change	18

3.1.2	Sequence parameter set syntax change	18

3.1.3	Adaptation parameter set syntax	18

3.1.4	Edge enhancement filter parameter syntax	19

3.2	Semantics	19

3.2.1	NAL unit header semantics	19

3.2.2	Sequence parameter set semantics	20

3.2.3	Adaptation parameter set semantics	20

3.2.4	Edge enhancement filter parameter semantics	20

4	Compression performance discussion	20

4.1	Category 1 (HEVC base layer)	20

4.1.1	Spatial scalability	20

4.1.2	Intra-only spatial scalability	21

4.1.3	SNR scalability	21

4.1.4	Overall	21

4.2	Category 2 (AVC base layer)	21

4.2.1	Spatial scalability	21

4.2.2	Overall	22

5	Complexity analysis	22

5.1	Encoding time and measurement methodology	22

5.2	Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM	23

5.3	Description of computing platform used to determine encoding and decoding times reported in sections 5.1 and 5.2	24

5.4	Expected memory usage of encoder	25

5.5	Expected memory usage of decoder	25

5.6	Complexity characteristics of encoder motion estimation and partitioning selection in enhancement layer(s)	25

5.7	Complexity characteristics of decoder motion compensation in enhancement layer(s)	26

5.8	Complexity characteristics of encoder intra-frame prediction type and partitioning selection in enhancement layer(s)	26

5.9	Complexity characteristics of decoder intra-frame prediction operation in enhancement layer(s)	26

5.10	Complexity characteristics of upsampling filters and transforms specific in enhancement layer(s)	26

5.11	Complexity characteristics of quantization and inverse quantization in enhancement layer(s)	26

5.12	Complexity characteristics of encoder entropy coding operation in enhancement layer(s)	26

5.13	Complexity characteristics of decoder entropy decoding operation in enhancement layer(s)	26

5.14	Degree of capability for encoder parallel processing	27

5.15	Degree of capability for decoder parallel processing	27

6	Software implementation description	27

7	Highlighted aspects discussion	27

8	Closing remarks	27

9	Patent rights declaration(s)	27

References	28

# Glossary of terms used in this proposal

- ARPP		adaptive reference picture placement

- EEF		edge enhancement filter

- ELSkip		enhancement layer skipped slice

- ILR		inter layer reference

- IMDDF		intra mode dependent directional filters

- MFM		motion field mapping

- SUF		selectable upsampling filters

# Introduction

As the first phase of HEVC standard [1] comes close to finalization, a Call for Proposals on the scalable extensions of HEVC was recently issued jointly by ITU-T SG16 Q.6 (VCEG) and ISO/IEC JTC1/SC29/WG11 (MPEG) [2]. This proposal is InterDigital Communications’ response to the CfP. It aims to provide a scalable video coding system with significantly higher compression capability than the comparable simulcast coding system. To achieve this compression efficiency goal without over-burdening implementation of the scalable system, the coding technologies in the proposed system are focused on picture level operations. Among these, five picture level coding technologies are proposed to improve inter layer prediction; these include selectable upsampling filters (SUF), intra mode dependent directional filters (IMDDF), motion field mapping (MFM), enhancement layer skipped slice (ELSkip), and adaptive reference picture placement (ARPP). Within the enhancement layer, changes are kept at minimal: except for the introduction of one additional in-loop filter, called edge enhancement filter (EEF), which operates at the picture level, all block level operations in the enhancement layer are kept the same as a single layer HEVC codec. With a design philosophy of providing as much scalable coding efficiency as possible re-using the single layer HEVC codec, the proposed scalable system is aimed at providing a good and relatively simple starting point for the next phase of HEVC scalable extensions.

Figure 1. Block diagram of the proposed scalable system

Figure 2. Prediction structure of the proposed scalable system

Figure 1 shows the block diagram of the proposed system at the encoder side. Figure 2 shows the prediction structure employed at the enhancement layer. The picture level inter layer processing block in Figure 1 generates the inter layer reference picture based on the base layer reconstruction, which is then used as an additional reference picture to predict the corresponding enhancement layer picture. In the proposed system, except for one additional in-loop filter called EEF, the enhancement layer encoder block level operations remain identical to those of a single layer HEVC encoder. The new coding tools in the proposed scheme are listed below:

- Selectable upsampling filters (SUF): the proposed system supports 4 upsampling filters in addition to the default upsampling filters. These additional filters have different filter characteristics that are suitable for different types of video signals. SUF filter characteristics are discussed in 2.2.1. The best filter is selected at the picture level through multiple pass encoding, as discussed in 2.12.

- Intra mode dependent directional filters (IMDDF): when the base layer blocks are coded using intra prediction, its prediction mode is used to select an appropriate directional filter accordingly. The directional filters are discussed in 2.2.3. In our implementation, IMDDF is applied only when the base layer picture is an intra picture. In this case, IMDDF is selected together with SUF using multiple pass encoding according to 2.12.

- Motion field mapping (MFM): in order to enable temporal motion vector prediction (TMVP) in the enhancement layer using the inter layer reference picture, the motion field associated with the inter layer reference picture is generated by mapping the motion field of the corresponding base layer picture. Discussion of MFM is in 2.3.3.

- Enhancement layer skipped slice (ELSkip): the entire enhancement layer picture can be directly copied from the inter layer reference picture, and no additional coding of the enhancement layer picture is performed. Discussion of ELSkip is in 2.3.5.

- Adaptive reference picture placement (ARPP): the inter layer reference is placed in the enhancement layer reference lists according to the prediction signal quality it provides in comparison with the other temporal reference pictures. Discussion of ARPP is in 2.3.6.

- Edge enhancement filtering (EEF): an additional in-loop filter is used in the enhancement layer codec. This in-loop filter is focused on improving the signal quality in the edge and high texture areas. Discussion of EEF is in 2.6.3.

Compared with the simulcast solution that encodes the multiple layer inputs independently, the proposed scalable system on average saves 25% and 45% of the enhancement layer bit rates for 2x and 1.5x spatial ratios, respectively, for random access configuration. In all-intra configuration, the enhancement layer bit rate savings are 34% and 55% for 2x and 1.5x ratios, respectively. In SNR scalability, the enhancement layer bit rate savings are 33% on average. The proposed system also supports scalable coding with base layer coded in H.264/AVC, and the enhancement layer bit rate savings in this case are 24% and 41% for 2x and 1.5x, respectively.

Indicate the category/categories the proposal covers (tick box )

 	Category 1 (HEVC base layer) spatial scalability

 	Category 1 (HEVC base layer) intra-only spatial scalability

	Category 1 (HEVC base layer) SNR scalability

 	Category 2 (AVC base layer) spatial scalability

 	The proposal obeys the constraints under section 5 of the CfP (if box is not ticked, explain cases where constraints are violated)

# Algorithm description

## CTU/TU/PU partitioning

Same as single layer HEVC.

## Up-sampling filters

The proposed system includes a total of 3 types of upsampling filters: the default filters, the SUF, and the IMDDF. They are discussed in detail in the following sections.

### Default upsampling filters

The default upsampling filters in the proposed system that generate the inter layer reference pictures for 1.5X and 2X spatial scalability have the following features:

- Derived from Gaussian windowed Sinc function.

- 2-D separable.

- For pixels in the ILR picture that correspond to pixels at integer pixel locations in the base layer picture, they are directly copied from the base layer without filtering.

- For pixels in the ILR picture that correspond to pixels at factional pixel locations in the based layer picture, they are filtered using 8-tap filters for the luma component and 4-tap filters for the chroma components.

- The precision of all the upsampling filter coefficients is 8-bit, and filtering is realized by 16-bit implementation.

The default upsampling filter coefficients are provided in Table 1.

Table 1. The default upsampling filter coefficients

| Luma | 2X |  | [-3 | 8 | -21 | 80 | 80 | -21 | 8 | -3]/128 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.5X |  | [-5 | 11 | -24 | 107 | 53 | -18 | 8 | -4]/128 |
|  |  |  | [-4 | 8 | -18 | 53 | 107 | -24 | 11 | -5]/128 |
| Chroma | 2X | Vertical | [-17 | 100 | 56 | -11]/128 |  |  |  |  |
|  |  |  | [-3 | 15 | 126 | -10]/128 |  |  |  |  |
|  |  | Horizontal | [-15 | 79 | 79 | -15]/128 |  |  |  |  |
|  | 1.5X | Vertical | [-20 | 122 | 38 | -12]/128 |  |  |  |  |
|  |  |  | [-20 | 71 | 101 | -24]/128 |  |  |  |  |
|  |  |  | [-4 | 12 | 128 | -8]/128 |  |  |  |  |
|  |  | Horizontal | [-23 | 113 | 54 | -16]/128 |  |  |  |  |
|  |  |  | [-16 | 54 | 113 | -23]/128 |  |  |  |  |

### Selectable upsampling filters (SUF)

In addition to the default upsampling filters, four selectable upsampling filters for each of the 1.5X and 2X cases are supported in the proposed system. These are used to provide a wider range of filter frequency response characteristics. They have the following characteristics:

- They are 2-D non-separable.

- Each SUF consists of a set of phase filters, and each phase filter has sizes 7×7, 7×6, 6×7, or 6×6, depending on to which phase the filter applies.

- All the filter coefficients can be represented by 8-bit integers.

Figure 3 and Figure 4 show the impulse responses of the four SUFs for 1.5X and 2X upsampling, respectively. SUF filters are applied to luma samples only. Together with the default upsampling filters, one of the SUF filters can be selected at the picture level. Indication for which filters are selected is signaled in the APS. The detailed syntax and semantics are introduced in Section 3.

Figure 3 Impulse responses of the four SUFs for 2X upsampling

Figure 4 Impulse responses of the four SUFs for 1.5X upsampling

### Intra mode dependent directional filters (IMDDF)

Figure 5 shows all the intra prediction modes supported in HEVC. For intra coded blocks, their prediction modes are highly correlated to the direction of the textures/edges in the block. In the case of scalable coding, when the corresponding base layer blocks are intra coded, the directionality information provided by their prediction modes can be used to improve the inter layer prediction process. IMDDF is specifically designed to take advantage of this “free” information offered by the base layer to improve enhancement layer coding.

Figure 5 Intra prediction modes in HEVC

As show in Figure 6, there are 4 and 9 relative phases for 2x and 1.5x ratios, respectively. IMDDF consists of a set of poly-phase filters that correspond to each prediction mode. As examples, Figure 7 and Figure 8 show the impulse responses of the IMDDF filters for the horizontal, vertical, and diagonal modes for 2x and for 1.5x, respectively. IMDDF filter sizes for 2x ratio and for 1.5x ratio are given in Table 2 and Table 3, respectively.

- 2x ratio 					(b) 1.5x ratio

Figure 6 Upsampling filter phases for luma samples

- 2x IMDDF for horizontal mode (mode 10)                  (b) 2x IMDDF for vertical (mode 26)

(c)  2x IMDDF 135⁰ diagonal (mode 2)                                (d) 2x IMDDF: 45⁰ diagonal (mode 18)

Figure 7 Examples of IMDDF impulse responses for 2x scaling ratio

- 1.5x IMDDF for horizontal mode (mode 10)                  (b) 1.5x IMDDF for vertical (mode 26)

(c)  1.5x IMDDF 135⁰ diagonal (mode 2)                                (d) 1.5x IMDDF: 45⁰ diagonal (mode 18)

Figure 8 Examples of IMDDF impulse responses for 1.5x scaling ratio

Table 2 IMDDF filter size for 2x ratio

| Filter phase | Filter size (H×V) |
| --- | --- |
| 0 | 7×7 |
| 1 | 8×7 |
| 2 | 7×8 |
| 3 | 8×8 |

Table 3 IMDDF filter size for 1.5x ratio

| Filter phase | Filter size (H×V) |
| --- | --- |
| 0 | 7×7 |
| 1 | 8×7 |
| 2 | 8×7 |
| 3 | 7×8 |
| 4 | 8×8 |
| 5 | 8×8 |
| 6 | 7×8 |
| 7 | 8×8 |
| 8 | 8×8 |

Currently, IMDDF is only applied when the corresponding base layer picture is intra coded. When such is the case, IMDDF is considered together with the SUF filters, and the best filter is selected at the picture level. IMDDF is only applied to the luma samples.

## Inter-layer prediction

### Inter-layer prediction of CU/TU/PU partitions

Not applied.

### Inter-layer residual prediction

Not applied.

### Inter-layer motion vector prediction: motion field mapping (MFM)

Inter-layer motion vector prediction is carried out at the picture level. Specifically, picture level processing of the base layer pictures in DPB in Figure 1 also includes mapping the base layer motion to form motion field for the inter layer reference picture. This allows the inter layer reference to be used as a collocated picture for enhancement layer TMVP.

Figure 9. Example relationship between the SPUs of upsampled base layer and the original base layer

In our software implementation, motion field mapping is performed based on units of Smallest PUs (SPUs). The compressed MV field from the base layer is used. Taking spatial scalability with 1.5x up-sampling ratio as an example, Figure 9 shows the relationship between the SPUs of the inter layer reference picture (blocks depicted with red solid lines) and the SPUs of the reconstructed base layer picture (blocks depicted with blue dashed lines). As shown in Figure 9, depending on the up-sampling ratio and the SPU’s coordinates in the picture, each SPU in the inter-layer reference picture could correspond to various numbers and proportions of the base layer SPUs. The proposed motion field mapping (MFM) method estimates the motion field for each of the inter layer SPUs, including its reference picture index and its MV, using  motion information from their corresponding base layer SPUs.

#### Reference picture index mapping

To create its motion field, it is assumed that the inter-layer picture has the same temporal prediction structure as its corresponding base layer picture. As in Figure 2, denote the base layer picture as BL and the inter layer reference picture as ILR, the ILR’s virtual reference picture lists are generated by copying the same entries from BL’s corresponding reference picture list entries.

When an inter layer SPU is covered by just one base layer SPU, for example, block “u0” in Figure 9, its reference index is copied from the base layer SPU. When an inter layer SPU is covered by more than one base layer SPUs, for example, block “u4” in Figure 9, the majority rule is applied to assign reference picture index to the inter layer SPU; specifically, the inter layer SPU will inherit the reference picture index used most frequently by its corresponding base layer SPUs. To break any possible ties, the reference index representing a reference picture with the shortest POC distance to the current picture will be selected.

#### Motion vector mapping

To estimate MVs of inter-layer reference pictures, an area-based weighted average scheme is applied.  The reference index of the inter layer SPU, denoted as refILR, is decided according to Section 2.3.3.1. The motion vector  for the inter layer SPU, denoted as MVILR, is decided by applying weighted average to the corresponding base layer SPUs that have the same reference index as refILR, using (1),

MVILR=NSi⋅MViBLSi, 	B=j:refj=refILR			(1)

where MViBL is the MV of the corresponding base layer SPUs, N is the upsampling ratio (for example, N = 1.5 or 2), and B is the subset of base layer SPUs with reference index equal to refILR. The x and y component of the motion vector are calculated separately using (1).

### Inter-layer mode prediction

Not applied.

### Other: Enhancement Layer Skipped Slice (ELSkip)

Enhancement layer skipped slice is introduced to the proposed system, based on the observation that the inter layer reference picture, after being processed with the proposed inter layer processing techniques (such as SUF and IMDDF), can be used directly as the enhancement layer reconstruction and can sometimes offer better rate distortion performance. When such is the case, only inter layer processing parameters, used to generate the inter layer reference, need to be sent at picture (or slice) level, and no additional coding of the blocks in the enhancement layer picture is performed, which significantly reduces decoding complexity as well.

In the proposed system, the encoder uses R-D cost to determine whether to choose the ELSkip mode for the enhancement layer picture. The percentage of ELSkip slices depends on the video content, the correlation between the layers, as well as on the relatively rate allocation between the base and the enhancement layers. Figure 10 compares the rate distortion performance of the proposed scalable system with that of single layer HEVC, for Kimono in random access 1.5x configuration. Fixed QP values are used in the simulation, with QPenh – QPbase = 4, QPbase = {22, 26, 30, 34}. As shown, the proposed scalable system is as efficient as the single layer HEVC codec in these examples; a closer look at the encoder statistics reveals that, in these examples, the ELSkip mode is selected for as much as 80% of the enhancement layer pictures.

Figure 10 Example of the proposed scalable system achieving the same R-D performance as single layer HEVC

In the proposed system, the ELSkip slice is signaled using a new nal_unit_type, as discussed in Section 3. Currently the slice header of an ELSkip slice is still signaled in the proposed system, mostly to ensure correct DPB operation using RPS. An additional consideration for ELSkip is its combination with MFM. Specifically, the ELSkip slice not only inherits the sample values of the inter layer reference (that is, by directly copying them), it also inherits the mapped motion field of the inter layer reference, as discussed in 2.3.3. This allows any ELSkip slice to be used as a collocated picture for TMVP of other future pictures.

### Other: Adaptive Reference Picture Placement (ARPP)

After applying inter layer processing as in Figure 1, the inter layer reference picture is added as an additional reference picture to predict the enhancement layer video. By default, the inter layer reference picture is placed at the end of the reference picture lists. Compared to the temporal reference pictures in the enhancement layer, the inter layer reference picture sometimes can provide better prediction; for example, when the temporal distance from temporal reference pictures is relatively long, and/or when the base layer picture is coded using lower quantization parameters. In other cases, however, temporal reference pictures provide better prediction than the inter layer reference; for example, when the temporal distance in the enhancement layer is short, and/or when the base layer is coded with higher quantization parameters. Therefore, in this proposal, an adaptive reference picture placement (ARPP) algorithm is introduced to decide the appropriate order of the temporal and inter-layer reference pictures in the reference lists.

Picture level multi-pass encoder is used to decide the order of the reference pictures in the reference lists.  Specifically, accumulated SATD values for each of the reference pictures (including temporal and inter layer) from the previous coding passes are stored. Then, their order in the reference lists is decided by sorting from the lowest motion distortion to the highest motion distortion.

## Inter-frame prediction

Same as single layer HEVC.

## Intra-frame prediction

Same as single layer HEVC.

## In-loop filtering

### De-blocking filter

Same as single layer HEVC.

### SAO

Same as single layer HEVC.

### Other: Edge Enhancement Filter (EEF)

The high frequency components in the original video signal are the most prone to being lost due to quantization. This information loss often manifests itself as loss of detail in the reconstructed video, especially around edges and texture rich areas. An example is shown in   , where sub-figure (a) shows the original luma signal, and sub-figure (b) shows the overlay of two binary maps: the first one showing pixels with large reconstruction error (in green) and the second one showing edge pixels detected by Canny edge detector (in pink). Pixels with both characteristics (that is, edge pixels with large reconstruction errors) are shown in orange.    (c) is a zoomed-in version of (b). A significant percentage of orange pixels are observed, which means a lot of pixels with large reconstruction errors represent edges in the original video. Based on this observation, an edge enhancement filter (EEF) is applied to the enhancement layer codec to improve signal fidelity around edges in the reconstructed picture.

As shown in Figure 12, the EEF process is carried out in 3 steps. In the first step, edge detection using Canny edge detector is applied to obtain the edge map and direction of edge pixels, Gx,Gy,θ. Then, the detected edge pixels are first classified into 8 direction groups as shown in Figure 13, according to the detected edge direction θ. Then, edge pixels within each of the 8 direction groups are further classified into two sub-groups, according to whether their second order derivative along the gradient direction are positive or negative, as in Figure 14. The sign of the second order derivative for the edge pixel n is calculated using (2), where p(n) is the value of pixel n, and pixels (n-1) and (n+1) are the left and right neighboring pixels of pixel n along the gradient direction.

sign(n)=−1 if p(n)<(p(n−1)+p(n+1))2+1 if p(n)>(p(n−1)+p(n+1))2		(2)

In this way, EEF classifies the edge pixels into a total of 16 groups. For each group, an offset is trained and signaled to the decoder. These offset values are used to correct the distortion around edges caused by quantization, and to restore the fidelity to the original video signal in these areas.

EEF is applied in the enhancement layer codec as an additional in-loop filter, following SAO, as shown Figure 15.

The EEF parameters for each edge group are signaled in APS. The detail is discussed in 2.9.5.

(a)                                                                                                   (b)

(c)

Figure 11 (a) the original enhancement layer luma signal; (b) overlay of reconstruction error map and detected edge map: pixels with large reconstruction error are in green, edge pixels are in pink, and the orange pixels are detected edge pixels with large errors; (c) a cropped region in (b)

Figure 12 Three steps in EEF

Figure 13 Edge directions

(a)                                         (b)

Figure 14 Classification of edge pixels within an edge direction group based on second order derivative

Figure 15 Block diagram of in-loop filtering with EEF

## Spatial transforms

Same as single layer HEVC.

## Quantization

Same as single layer HEVC.

## Motion/mode/parameter coding

### Merge/skip

Same as single layer HEVC.

### Motion vector coding

Same as single layer HEVC.

### Intra prediction mode coding

Same as single layer HEVC.

### SAO parameter coding

Same as single layer HEVC.

### Other: EEF parameter coding

As in Figure 15, EEF is applied as an additional in-loop filter following SAO in the enhancement layer. When enabled, EEF needs to signal 1 offset per edge group, or equivalently, 16 offset values per picture. These EEF parameters are carried in the APS. The detailed syntax and semantics are discussed in Section 3.

## Entropy coding of residual / transform coefficients

Same as single layer HEVC.

## High-layer syntax

High level syntax remains mostly the same as single layer HEVC, except 1) a nuh_layer_id replaces the reserved_zero_6bits in NAL unit header, 2) a new nal_unit_type is assigned to signal ELSkip slice. These will be discussed in further detail in Section 3.

## Picture level multiple pass encoding and early termination

In the proposed encoder, picture level multiple pass encoding is performed to choose appropriate inter layer processing parameters. Figure 16 shows the encoding flow. The encoder performs up to 3 coding passes, and except for the first coding pass, early termination is applied to the two subsequent coding passes. Early termination uses coding statistics from previous passes to decide whether a subsequent coding pass is likely to provide any benefits. For example, to determine whether to perform a second pass using a different ILR processed by different filters such as SUF or IMDDF, the reduction in distortion achieved by the filters is weighed against the additional bit overhead they incur. To determine whether to perform the ARPP coding pass, the reordered reference picture lists are examined. If they are mostly the same as the default order, then the ARPP coding pass is not performed.

The decision on whether to select the ELSkip mode does not require another coding pass; instead, the encoder only needs to calculate and compare the R-D cost of the ELSkip mode with the best R-D cost of previous coding passes.

Figure 16. Picture-level multi-pass encoding with early termination

## AVC to HEVC scalability

AVC to HEVC scalability (category 2) is carried out using the same system architecture as in Figure 1. In the current implementation, only sample values from reconstructed AVC base layer pictures are used; the motion and mode information from base layer bistream is not used. Therefore, only a subset of the 6 tools introduced above are applicable to AVC to HEVC scalability, as shown in Table 4.

Table 4. Applicability of the proposed tools to AVC to HEVC scalability

| Tool name | Applicability (yes/no) |
| --- | --- |
| SUF | Yes |
| IMDDF | No |
| MFM | No |
| ARPP | Yes |
| ELSkip | Yes |
| EEF | Yes |

# Syntax and semantics description

## Syntax

### NAL unit header syntax change

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_unit_type | u(6) |
| nuh_reserved_zero_6bits | u(6) |
| nuh_layer_id | u(6) |
| nuh_temporal_id_plus1 | u(3) |
| } |  |

### Sequence parameter set syntax change

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_idc | u(8) |
| ...... |  |
| IMDDF_enabled_flag | u(1) |
| suf_enabled_flag | u(1) |
| ...... |  |

### Adaptation parameter set syntax

The APS, previously supported in single layer HEVC but later removed, provides a very convenient means for carrying the inter layer processing parameters. It is used in the proposed system to signal these processing parameters.

| aps_rbsp( ) { | Descriptor |
| --- | --- |
| aps_id | ue(v) |
| if( layer_id > 0 ) { |  |
| if( NumUpsampleMethod >1 ) { |  |
| upsample_method | u(2) |
| if (upsample_method == SUF) |  |
| suf_filter_idx | u(2) |
| } |  |
| eef_enabled_flag | u(1) |
| if( eef_enabled_flag ) |  |
| eef_ param( ) |  |
| } |  |
| aps_extension_flag | u(1) |
| if( aps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| aps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

### Edge enhancement filter parameter syntax

| eef_param( ) { | Descriptor |
| --- | --- |
| for( cmp = 0; cmp < chroma_format_idc == 0 ? 1:3; cmp++ ) { |  |
| eef_comp_flag[cmp] | u(1) |
| if (eef_comp_flag[cmp] ) { |  |
| for( idx = 0; idx < 16; idx++ ) |  |
| eef_filter_coeff[ cmp ][idx] | se(v) |
| } |  |
| } |  |
| } |  |

## Semantics

### NAL unit header semantics

nuh_layer_id indicates the video layer to which the current NAL unit belongs.

Table 5 NAL unit type codes and NAL unit type classes

| nal_unit_type | Name of nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| 0 | UNSPEC0 | Unspecified | non-VCL |
|  | … |  |  |
| 15 | SKIP_SLICE | Coded slice of a skipped slice | VCL |
| 16..20 | RSV_VCL16.. RSV_VCL20 | Reserved | VCL |
|  | … |  |  |

### Sequence parameter set semantics

IMDDF_enabled_flag equal to 1 indicates intra mode dependent directional filtering is used in inter layer reference processing. IMDDF_enabled_flag equal to 0 indicates intra mode dependent directional filtering is not used in inter layer reference processing.

suf_enabled_flag equal to 1 indicates selectable upsampling filter is used in inter layer reference processing. suf_enabled_flag equal to 0 indicates selectable upsampling filter is not used in inter layer reference processing.

NumUpsampleMethod is set to be equal to (1+ IMDDF_enabled_flag+suf_enabled_flag) if the two layers have different spatial resolutions, and is set to be equal to 1 if the two layers have the same spatial resolution.

### Adaptation parameter set semantics

upsample_method_idc indicates the upsampling method used to process the inter layer reference picture and to decode the associated slices. The mapping between upsample_method_idc and the upsampling filters is shown in Table 6. When upsample_method_idc is not present, it is inferred to be equal to 0.

Table 6. Indication of upsampling method

| Index | Name |
| --- | --- |
| 0 | DEFAULT |
| 1 | SUF |
| 2 | IMDDF |

suf_filter_idx specifies the SUF filter index used to process the inter layer reference picture and to decode the associated slices, when upsample_method_idc is set to be equal to SUF. When suf_filter_idx is not present, it is inferred to be equal to 0.

eef_enabled_flag equal to 1 specifies that the edge enhancement filter process is applied to the reconstructed picture after the sample adaptive offset process.

### Edge enhancement filter parameter semantics

eef_comp_flag[ cmp ] equal to 1 specifies that the edge enhancement filter process is applied to the color component cmp.

eef_filter_coef [ cmp ][ idx ] specifies the 16 filter offset coefficients used in the edge enhancement process for color component cmp.

# Compression performance discussion

The compression performance is measured using BD rate as required by the CfP. For all of the 688 data points in all categories, the actual rates meet the CfP requirement that they are lower but not more than 10% lower than the corresponding target rates (most are no more than 5% below the targets). In this section, compression performance of the proposed system will be discussed using data in the “EL-only actual rate” category of the accompanying CfP template spreadsheet. Readers are referred to the accompanying spreadsheet for further detail.

## Category 1 (HEVC base layer)

### Spatial scalability

Table 7 shows the average BD rate reduction of the proposed system for spatial scalability using random access setting. Compared to HEVC anchors, BD rates of -25.3% and -44.7% are achieved for the luma component for 2x and for 1.5x, respectively.

Table 7. Average BD rate reduction of the proposed system for spatial scalability

|  | Random Access HEVC 2x |  |  | Random Access HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -27.5% | -8.7% | -8.9% |  |  |  |
| Class B | -23.1% | -9.4% | -6.2% | -44.7% | -33.5% | -30.8% |
| Overall | -25.3% | -9.0% | -7.5% | -44.7% | -33.5% | -30.8% |
|  | -25.3% | -9.3% | -7.9% | -44.9% | -33.8% | -31.3% |

### Intra-only spatial scalability

Table 8 shows the average BD rate reduction of the proposed system for spatial scalability using intra only configuration. Compared to the HEVC anchors, BD rates of -34.3% and -54.7% are achieved for the luma component for 2x and for 1.5x, respectively.

Table 8. Average BD rate reduction of the proposed system for intra-only spatial scalability

|  | All Intra HEVC 2x |  |  | All Intra HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -37.7% | -26.7% | -27.5% |  |  |  |
| Class B | -31.0% | -19.4% | -19.2% | -54.7% | -47.6% | -47.3% |
| Overall | -34.3% | -23.0% | -23.4% | -54.7% | -47.6% | -47.3% |
|  | -34.4% | -23.0% | -23.4% | -54.9% | -49.8% | -51.0% |

### SNR scalability

Table 9 shows the average BD rate reduction of the proposed system for SNR scalability using random access configuration. Compared to HEVC anchors, average BD rate reduction of -33.0% is achieved for the luma component.

Table 9. Average BD rate reduction of the proposed system for SNR scalability

|  | Random Access HEVC SNR |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A+ | -35.1% | -19.6% | -18.6% |
| Class B | -30.9% | -14.3% | -6.5% |
| Overall | -33.0% | -16.9% | -12.5% |
|  | -33.0% | -17.3% | -13.2% |

### Overall

As expected, the performance (i.e., BD rate reduction) of the proposed system varies depending on the scaling ratio between the two layers. For 2x ratio, average gains of 25% and 34% are achieved for random access and intra only configurations, respectively. For 1.5x ratio, average gains of 45% and 55% are achieved for random access and intra only configurations, respectively. For SNR scalability, average gain of 33% is achieved.

## Category 2 (AVC base layer)

### Spatial scalability

Table 10 and Table 11 show the average BD rate reduction of the proposed system for 2x and 1.5x spatial scalability with AVC base layer, when compared to HEVC- and AVC-coded higher layer anchors, respectively. Compared to the HEVC higher layer anchors, BD rates of -24.2% and -40.8% are achieved for 2x and for 1.5x, respectively. Compared to the AVC higher layer anchors, BD rates of -50.1% and -64.0% are achieved for 2x and for 1.5x, respectively.

Table 10. Average BD rate reduction of the proposed system for spatial scalability using AVC base layer, compared to HEVC anchors

|  | Hybrid vs HEVC 2x |  |  | Hybrid vs HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -26.5% | -7.8% | -8.3% |  |  |  |
| Class B | -21.9% | -8.3% | -4.5% | -40.8% | -30.7% | -27.8% |
| Overall | -24.2% | -8.0% | -6.4% | -40.8% | -30.7% | -27.8% |
|  | -24.2% | -8.2% | -6.6% | -41.0% | -30.9% | -28.1% |

Table 11. Average BD rate reduction of the proposed system for spatial scalability using AVC base layer, compared to AVC anchors

|  | Hybrid vs AVC 2x |  |  | Hybrid vs AVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -48.0% | -31.9% | -33.5% |  |  |  |
| Class B | -52.2% | -45.1% | -42.6% | -64.1% | -58.8% | -56.5% |
| Overall | -50.1% | -38.4% | -38.0% | -64.0% | -58.7% | -56.3% |
|  | -50.1% | -38.5% | -38.0% | -64.1% | -58.8% | -56.5% |

### Overall

Overall, the proposed scalable system is 15% and 22% more efficient than HEVC coded enhancement layer for 2x and 1.5x scaling ratios. Because the current software implementation only makes use of the sample values from the reconstructed AVC base layer, but not the mode and motion information, additional gains are expected when the mode and motion information from base layer bitstreams is used, for example, to enable MFM and IMDDF.

# Complexity analysis

## Encoding time and measurement methodology

Table 12 shows the average encoding time for each sequence under each configuration. The encoding time is measured using the Linux cluster as specified in Table 14.

Table 12. Average encoding time for each sequence under different configurations

| Configuration | Scalability | Test sequences | Average encoding time (hours) |
| --- | --- | --- | --- |
| AI | 2x | Traffic PeopleOnStreet Kimono ParkScene Cactus BasketballDrive BQTerrace | 14.65 14.54 3.64 6.16 15.58 13.62 21.58 |
|  | 1.5x | Kimono ParkScene Cactus BasketballDrive BQTerrace | 3.61 2.96 8.08 9.82 18.94 |
| RA | 2x | Traffic PeopleOnStreet Kimono ParkScene Cactus BasketballDrive BQTerrace | 17.78 22.92 10.84 8.91 17.70 24.49 25.11 |
|  | 1.5x | Kimono ParkScene Cactus BasketballDrive BQTerrace | 12.96 9.08 18.78 19.17 22.15 |
|  | SNR | Traffic PeopleOnStreet Kimono ParkScene Cactus BasketballDrive BQTerrace | 22.99 31.85 10.88 9.78 20.22 26.91 25.58 |
| AVC base | 2x | Traffic PeopleOnStreet Kimono ParkScene Cactus BasketballDrive BQTerrace | 17.01 20.62 12.21 9.11 18.46 23.77 27.07 |
|  | 1.5x | Kimono ParkScene Cactus BasketballDrive BQTerrace | 9.81 8.10 14.07 16.72 17.19 |

Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM

Table 13 compares the decoding times of the proposed decoder and the anchor (HM6.1) decoder, for each sequence under each configuration. The decoding times are measured with the following considerations:

- For category 1 bitstreams, decoding time of the anchors includes decoding of both lower and higher layers. YUV output is enabled for the enhancement layer only.

- For category 2 bitstreams, decoding time of the anchors includes decoding of the higher layer only. YUV output is enabled.

The decoding times are measured on the Windows server platform as specified in Table 15. For category 1 cases, the average decoding time for the proposed system was measured to be between about 1.2 and 3.1 times that of the corresponding anchors; for category 2 cases, the average decoding time for the proposed system was measured to be between about 2.6 and 4.6 times that of the corresponding anchors. The increased decoding time ratio for category 2 cases is primarily because additional I/O time needed for the proposed decoder to read from reconstructed base layer YUV files.

Table 13. Average decoding time of the proposed system in comparison with the anchor system

| Configuration | Scalability | Test sequences | Ave decoding time of proposed (seconds) | Ave decoding time of anchor (seconds) | Ratio |
| --- | --- | --- | --- | --- | --- |
| AI | 2x | Traffic | 303 | 258 | 1.2 |
|  |  | PeopleOnStreet | 342 | 223 | 1.5 |
|  |  | Kimono | 147 | 53 | 2.8 |
|  |  | ParkScene | 134 | 69 | 1.9 |
|  |  | Cactus | 266 | 143 | 1.9 |
|  |  | BasketballDrive | 274 | 128 | 2.1 |
|  |  | BQTerrace | 428 | 181 | 2.4 |
|  | 1.5x | Kimono | 154 | 76 | 2.0 |
|  |  | ParkScene | 143 | 107 | 1.3 |
|  |  | Cactus | 271 | 212 | 1.3 |
|  |  | BasketballDrive | 289 | 178 | 1.6 |
|  |  | BQTerrace | 443 | 249 | 1.8 |
| RA | 2x | Traffic | 245 | 88 | 2.8 |
|  |  | PeopleOnStreet | 285 | 113 | 2.5 |
|  |  | Kimono | 115 | 39 | 2.9 |
|  |  | ParkScene | 109 | 40 | 2.7 |
|  |  | Cactus | 185 | 93 | 2.0 |
|  |  | BasketballDrive | 276 | 98 | 2.8 |
|  |  | BQTerrace | 336 | 112 | 3.0 |
|  | 1.5x | Kimono | 127 | 53 | 2.4 |
|  |  | ParkScene | 116 | 61 | 1.9 |
|  |  | Cactus | 227 | 118 | 1.9 |
|  |  | BasketballDrive | 292 | 134 | 2.2 |
|  |  | BQTerrace | 331 | 148 | 2.2 |
|  | SNR | Traffic | 244 | 149 | 1.6 |
|  |  | PeopleOnStreet | 324 | 179 | 1.8 |
|  |  | Kimono | 113 | 64 | 1.8 |
|  |  | ParkScene | 121 | 66 | 1.8 |
|  |  | Cactus | 191 | 157 | 1.2 |
|  |  | BasketballDrive | 262 | 159 | 1.6 |
|  |  | BQTerrace | 298 | 191 | 1.6 |
| AVC base layer | 2x | Traffic | 264 | 74 | 3.6 |
|  |  | PeopleOnStreet | 299 | 91 | 3.3 |
|  |  | Kimono | 146 | 32 | 4.6 |
|  |  | ParkScene | 125 | 34 | 3.7 |
|  |  | Cactus | 208 | 81 | 2.6 |
|  |  | BasketballDrive | 259 | 83 | 3.1 |
|  |  | BQTerrace | 368 | 97 | 3.8 |
|  | 1.5x | Kimono | 132 | 32 | 4.1 |
|  |  | ParkScene | 125 | 34 | 3.7 |
|  |  | Cactus | 210 | 81 | 2.6 |
|  |  | BasketballDrive | 304 | 83 | 3.7 |
|  |  | BQTerrace | 311 | 97 | 3.2 |

## Description of computing platform used to determine encoding and decoding times reported in sections 5.1 and

Two types of computing platforms are used to determine the encoding and the decoding times reported above and in the detailed accompanying spreadsheet, the Linux cluster as specified in Table 14 and the Windows server in Table 15. The Linux grid is used to generate the encoded bitstreams; as in Table 14, it consists of two categories of CPUs of slightly varying performance (<10% difference). The Windows server is used to compare the decoding times of the anchors with those of the proposed system.

Table 14. Technical specifications of the Linux cluster used to generate InterDigital’s CfP response bistreams

| CPU | Intel Xeon E5-2650 / Clock speed 2.00GHz Intel Xeon X5675 / Clock speed 3.07GHz |
| --- | --- |
| Memory for each core | 2 GB |
| HDD characteristics | -Hard Drive Capacity: 300GB -Interface: Serial Attached SCSI -External Data Transfer Rate: 6.0 GB/sec -Rotational Speed: 10,000 rpm |
| Compiler | GCC version 4.4.5 |
| Operating System (OS) | 64 bit Linux |

Table 15. Technical specifications of the Windows server used to compare decoding times of the anchor bitstreams and InterDigital’s CfP response bitstreams

| CPU | Intel Xeon X5675 / Clock speed 3.07GHz |
| --- | --- |
| Memory for each core | 2 GB |
| HDD characteristics | -Hard Drive Capacity: 500GB -Interface: Serial Attached SCSI |
| Compiler | Visual studio 2010 |
| Operating System (OS) | 64 bit Windows Server 2003 |

## Expected memory usage of encoder

Table 16 shows the expected memory usage of the proposed encoder. The proposed encoder uses between 3.5GB (for class A SNR scalability) and 720MB (for class B all intra 2x scalability) of memory.

Table 16. Expected memory usage of the proposed scalable encoder for each sequence class and configuration

| Configuration | Scalability | Class | Average encoder memory usage |
| --- | --- | --- | --- |
| AI | 2x | Class A | 2.50 GB |
|  |  | Class B | 720 MB |
|  | 1.5x | Class B | 750 MB |
| RA | 2x | Class A | 1.65 GB |
|  |  | Class B | 1.50 GB |
|  | 1.5x | Class B | 1.68 GB |
|  | SNR | Class A | 3.50 GB |
|  |  | Class B | 2.00 GB |

## Expected memory usage of decoder

Table 17 shows the expected memory usage of the proposed decoder. The proposed decoder uses between 2.3GB (for class A SNR scalability) and 595MB (for class B all intra 2x scalability) of memory.

Table 17. Expected memory usage of the proposed scalable decoder for each sequence class and configuration

| Configuration | Scalability | Class | Average decoder memory usage |
| --- | --- | --- | --- |
| AI | 2x | Class A | 2.08 GB |
|  |  | Class B | 595 MB |
|  | 1.5x | Class B | 613 MB |
| RA | 2x | Class A | 2.15 GB |
|  |  | Class B | 610 MB |
|  | 1.5x | Class B | 630 MB |
|  | SNR | Class A | 2.30 GB |
|  |  | Class B | 635 MB |

## Complexity characteristics of encoder motion estimation and partitioning selection in enhancement layer(s)

Compared to single layer anchor encoding, motion estimation at the enhancement layer considers one additional reference picture (the inter layer reference picture). Partitioning selection logic in the enhancement layer is unchanged from HM6.1.

## Complexity characteristics of decoder motion compensation in enhancement layer(s)

Same as HM6.1, except that one additional reference picture needs to be stored for the enhancement layer.

## Complexity characteristics of encoder intra-frame prediction type and partitioning selection in enhancement layer(s)

Same as HM6.1.

## Complexity characteristics of decoder intra-frame prediction operation in enhancement layer(s)

Same as HM6.1.

## Complexity characteristics of upsampling filters and transforms specific in enhancement layer(s)

The proposed system uses three types of upsampling filters, the default filter, the SUF, and the IMDDF (for base layer intra coded pictures only), for the luma component. The number of arithmetic operations (including addition, multiplication, shift, etc) for each filter type for the luma component is shown in Table 18.  Only the default filters are used for the chroma components; the default chroma filter tap lengths are shown in Table 1.

Table 18. Number of arithmetic operations for each filter phase (defined in Figure 6) for the luma component

|  | Phase | Default | SUF | IMDDF |
| --- | --- | --- | --- | --- |
| 2x | 0 | 0 | 99 | 99 |
|  | 1 | 17 | 85 | 113 |
|  | 2 | 17 | 85 | 113 |
|  | 3 | 34 | 73 | 129 |
| 1.5x | 0 | 0 | 99 | 99 |
|  | 1 | 17 | 85 | 113 |
|  | 2 | 17 | 85 | 113 |
|  | 3 | 17 | 85 | 113 |
|  | 4 | 34 | 73 | 129 |
|  | 5 | 34 | 73 | 129 |
|  | 6 | 17 | 85 | 113 |
|  | 7 | 34 | 85 | 129 |
|  | 8 | 34 | 73 | 129 |

Both the default filter and the SUF can be implemented using 16-bit fixed-point arithmetic. At the time when simulations were launched to obtain the bitstreams for this response to CfP, the IMDDF filters contained in that software version requires precision beyond16-bit fixed-point arithmetic. The IMDDF filters have since been simplified to also use 16-bit fixed point implementation, with no observable performance loss on average.

Transforms are the same as in single layer HEVC codec.

## Complexity characteristics of quantization and inverse quantization in enhancement layer(s)

Same as HM6.1.

## Complexity characteristics of encoder entropy coding operation in enhancement layer(s)

Same as HM6.1.

## Complexity characteristics of decoder entropy decoding operation in enhancement layer(s)

Same as HM6.1.

## Degree of capability for encoder parallel processing

Multi-threaded programming can be used at high level to parallelize base layer encoding, enhancement layer encoding, and inter layer reference picture processing. Within the enhancement layer, the degree of capability for encoder parallel processing remains substantially similar to that of a single layer HEVC encoder, as block level operations of the proposed system are the same as those of a single layer HEVC codec. Inter layer reference picture processing can be further accelerated by region-based parallel processing.

## Degree of capability for decoder parallel processing

The scalable decoder is designed for multi-threaded programming, which at high level parallelizes base layer decoding, enhancement layer decoding, and inter layer reference picture processing with shared buffers. An initial version of a multi-threaded decoder has been implemented for concept verification. Within the enhancement layer, the degree of capability for decoder parallel processing remains substantially similar to that of a single layer HEVC decoder, as block level operations of the proposed system are the same as those of a single layer HEVC codec.

# Software implementation description

Software implementation of the proposed scalable system is built upon HM6.1. The software implementation is written following the HM software implementation guidelines. In addition, the software implementation includes a VC10 build on Windows used specifically to check memory problems (such as memory leaks) in the software.

# Highlighted aspects discussion

As discussed, this response to the CfP follows one important guiding principle, which is to re-use the single layer HEVC coding technologies to the fullest extent possible. As such, block level operations at the enhancement layer in the proposed system are kept unchanged from HEVC, and focus is placed on using picture level processing technologies to improve inter layer prediction and scalable coding efficiency. To reiterate, the following picture level processing tools are introduced in this proposal:

- SUF allows one of 4 upsampling filters to be used in place of the default upsampling filters.

- IMDDF uses the existing directionality information provided by the intra prediction modes of the base layer blocks to select appropriate directional filters.

- MFM forms the motion field for the inter layer reference picture by mapping from the base layer motion field, thus allowing enhancement layer TMVP to use the inter layer reference picture as collocated picture.

- ELSkip copies the entire enhancement layer slice (picture) from the inter layer reference picture when it is beneficial to do so.

- ARPP places the inter layer reference adaptively according to how it compares with the enhancement layer temporal reference pictures.

- EEF is an additional in-loop filter used in the enhancement layer codec to improve signal quality in the edge and high texture areas.

# Closing remarks

All aspects of the proposed scalable system, including detailed algorithm descriptions, performance results and complexity analysis, have been discussed. The proposed system is capable of scalable coding in all of the categories and configurations laid out in the CfP document. On average, the proposed system achieves {34%, 55%, 25%, 45%, 33%} bit rate savings in the enhancement layer for {AI 2x, AI 1.5x, RA 2x, RA 1.5x, RA SNR} cases in category 1 of the CfP, and {24%, 41%} bit rate savings for {RA 2x, RA 1.5x} cases in category 2 of the CfP.

In conclusion, sufficient information about the proposed system has been provided to demonstrate the validity of the scalable coding tools. It is in our opinion that, because of its focus on picture level processing only, the proposed scalable system can provide a good starting point for the collaborative phase of HEVC scalable extensions.

# Patent rights declaration(s)

InterDigital Communications, LLC may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- B. Bross, W-J. Han, J-R. Ohm, G. J. Sullivan, High Efficiency Video Coding (HEVC) Text Specification Draft 8. Document no JCTVC-J1003. July 2012.

- ISO/IEC JTC-1/SC29/WG11 w12957, Joint Call for Proposals on Scalable Video Coding Extensions of High Efficiency Video Coding (HEVC), July 2012.