| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 11th Meeting: Shanghai, CN, 10-19 Oct., 2012 | Document: JCTVC-K0039 |
| --- | --- |

| Title: | Description of scalable video coding technology proposal by Intel |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Jen Chiu 2200 Mission College Blvd. Santa Clara CA USA 95054 Wenhao Zhang 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 Lidong Xu 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 Yu Han 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 Zhipin Deng 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 Xiaoxia Cai 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 Hong Jiang 2200 Mission College Blvd. Santa Clara CA USA 95054 | Tel: Email: Tel: Email: Tel: Email: Tel: Email: Tel: Email: Tel: Email: Tel: Email: | 408-765-9494 yi-jen.chiu@intel.com 86-10-82171657 wenhao.zhang@intel.com 86-10-82171425 lidong.xu@intel.com 86-10-82171388 yu.han@intel.com 86-10-82171372 Zhipin.deng@intel.com 86-10-82171674 xiaoxia.cai@intel.com 916-386-0457 hong.h.jiang@intel.com |
| Source: | Intel Corp. |  |  |

_____________________________

# Abstract

This contribution presents a scalable video codec proposal from Intel in response to the Joint Call for Proposals on Scalable Video Coding Extensions of High Efficiency Video Coding issued by ISO/IEC JTC1/SC 29/WG 11 (MPEG) and ITU-T SG16 Q.6 (VCEG). This proposal is implemented in a multi-layered coding structure based on HM6.1 Reference software of HEVC coder, and the test result is provided for the scenario of SNR Scalability specified by the Call for Proposal. It contains a collection of inter-layer coding tools to improve the coding efficiency: inter-layer pixel prediction, inter-layer motion prediction, inter-layer residual refinement, inter-layer pattern/mode prediction and inter-layer refining filter. Together with the existing coding tool in HM 6.1, these inter-layer coding tools report the reduced-bit-rate bitstream under the RA_HEVC_SNR test configuration of Call for Proposal. The proposed tools achieve 36.3% BD-bitrate reduction on average on the EL-only actual rate testing case, 21.6% of BD-bitrate reduction on the EL-only target rate testing case, 20.5% of BD-bitrate reduction on the EL+BL actual rate testing case and 11.8% of BD-bitrate reduction on the EL+BL target rate testing case.

# Contents

Abstract	1

Contents	2

1	Introduction	4

2	Algorithm description	4

2.1	CTU/TU/PU partitioning	5

2.2	Up-sampling filters	5

2.3	Inter-layer prediction	5

2.3.1	Inter-layer pixel prediction	5

2.3.2	Inter-layer residual refinement	6

2.3.3	Inter-layer motion vector prediction	8

2.3.4	Inter-layer pattern/mode prediction	9

2.3.5	Inter-layer refine filtering	10

2.4	Inter-frame prediction	11

2.5	Intra-frame prediction	11

2.6	In-loop filtering	11

2.6.1	De-blocking filter	11

2.6.2	SAO	11

2.6.3	Other	11

2.7	Spatial transforms	11

2.8	Quantization	11

2.9	Motion/mode/parameter coding	11

2.9.1	Merge/skip	12

2.9.2	Motion vector coding	12

2.9.3	Intra prediction mode coding	12

2.9.4	SAO parameter coding	12

2.9.5	Other	12

2.10	Entropy coding of residual / transform coefficients	12

2.11	High-layer syntax	12

2.12	Additional algorithmic description discussion topic #1	12

2.13	Additional algorithmic description discussion topic #2	12

2.14	…	12

3	Syntax and semantics description	12

3.1	Proposed NAL unit header syntax and semantics	12

3.2	Proposed SPS syntax and semantics	13

3.3	Proposed CU syntax and semantics	14

3.4	Proposed PU syntax and semantics	17

4	Compression performance discussion	18

4.1	Category 1 (HEVC base layer)	18

4.1.1	Spatial scalability	18

4.1.2	Intra-only spatial scalability	18

4.1.3	SNR scalability	19

4.1.4	Overall	20

4.2	Category 2 (AVC base layer)	20

4.2.1	Spatial scalability	20

4.2.2	Overall	20

5	Complexity analysis	20

5.1	Encoding time and measurement methodology	20

5.2	Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM	21

5.3	Description of computing platform used to determine encoding and decoding times reported in sections 5.1 and 5.2	21

5.4	Expected memory usage of encoder	21

5.5	Expected memory usage of decoder	21

5.6	Complexity characteristics of encoder motion estimation and partitioning selection in enhancement layer(s)	21

5.7	Complexity characteristics of decoder motion compensation in enhancement layer(s)	22

5.8	Complexity characteristics of encoder intra-frame prediction type and partitioning selection in enhancement layer(s)	22

5.9	Complexity characteristics of decoder intra-frame prediction operation in enhancement layer(s)	22

5.10	Complexity characteristics of upsampling filters and transforms specific in enhancement layer(s)	22

5.11	Complexity characteristics of quantization and inverse quantization in enhancement layer(s)	22

5.12	Complexity characteristics of encoder entropy coding operation in enhancement layer(s)	22

5.13	Complexity characteristics of decoder entropy decoding operation in enhancement layer(s)	22

5.14	Degree of capability for encoder parallel processing	22

5.15	Degree of capability for decoder parallel processing	22

5.16	Additional complexity discussion topic #1	22

5.17	Additional complexity discussion topic #2	22

5.18	…	23

6	Software implementation description	23

7	Highlighted aspects discussion	23

8	Closing remarks	23

9	Reference	23

10	Patent rights declaration(s)	23

# Introduction

Indicate the category/categories the proposal covers (tick box )

 	Category 1 (HEVC base layer) spatial scalability

 	Category 1 (HEVC base layer) intra-only spatial scalability

 	Category 1 (HEVC base layer) SNR scalability

 	Category 2 (AVC base layer) spatial scalability

 	The proposal obeys the constraints under section 5 of the CfP (if box is not ticked, explain cases where constraints are violated)

This contribution describes Intel’s proposal in response to the Joint Call for Proposals (CfP) on Scalable Video Coding (SVC) Extensions of High Efficiency Video Coding (HEVC) issued by ISO/IEC JTC1/SC 29/WG 11 (MPEG) and ITU-T SG16 Q.6 (VCEG) on July 2012 [1]. A SVC system is a multi-layer structure and it typically consists of one Base Layer (BL) and multiple Enhancement Layers (ELs). BL video is formed based upon the information from the base layer, and then EL video can utilize the information from the same layer and from the layers below the current layer. For simplicity, this contribution presents the case which has only two layers: one BL and one EL.

The SVC codec in this contribution provides the test result for SNR Scalability which specified in the CfP. It is implemented in a multi-layered coding structure based on HM6.1 Reference software of HEVC coder; that is, the BL video bitstream is encoded with the existing coding tools in HM6.1. A couple of inter-layer coding tools are developed to remove the redundancy between layers and to improve the compression performance of the EL video bitstream. The proposed inter-layer coding tools are listed as follows,

Inter-layer pixel prediction: the reconstructed pixel samples of lower layers are used as the predictive signal when coding video of the upper layers.

Inter-layer motion prediction: the motion parameters derived from the lower layers are used as the additional motion candidates when coding video of the upper layers.

Inter-layer residual refinement: the residual of the lower layer is used to as the refinement signal to reduce the magnitude of intra/inter prediction error signal when coding video of upper layers.

Inter-layer pattern/mode prediction: the quadtree splitting pattern and coding modes derived from lower layer are applied when coding video the upper layers.

Inter-layer refining filter: the resulting signal of inter-layer pixel prediction is refined by a filtering process to further improve the predictive efficiency.

Under the RA_HEVC_SNR configuration provided in the CfP, the proposed tools achieve 36.3% BD-bitrate reduction on average on the EL-only actual rate testing case, 21.6% of BD-bitrate reduction on the EL-only target rate testing case, 20.5% of BD-bitrate reduction on the EL+BL actual rate testing case and 11.8% of BD-bitrate reduction on the EL+BL target rate testing case.

# Algorithm description

Figure 1 shows a block diagram flow of the proposed SVC encoder structure of a BL and an EL. The BL video is coded based on HM 6.1 coding tools. The EL video is coded by the combination of HM6.1 coding tools and the new inter-layer coding tools in this contribution.

Figure 1 – Block diagram of proposed SVC encoder

The white boxes indicate the critical modules of the HEVC coding tools in the SVC flow. For example, HEVC’s intra/inter prediction modules utilize the spatial/temporal correlation within each layer to encode video in BL and in EL respectively. The gray boxes indicate the proposed inter-layer coding modules to remove redundancy between BL and EL.

The proposed inter-layer coding tools in this contribution are block based, and they apply to CU, PU or TU level of EL picture. The first step to process a block in EL is to look for the information of the co-located blocks in BL according to the relative coordinate location reflected by the spatial resolution ratio between BL video and EL video. The syntax, semantics and process schemes of CU, PU and TU are defined based on HEVC standard to describe the SVC coding structure and to add the proposed inter-layer coding tools.

## CTU/TU/PU partitioning

The CTU/TU/PU partition schemes are same as HM6.1.

## Up-sampling filters

This contribution applies to the SNR scalability scenario, in which the EL pictures have same spatial resolution as the size of BL pictures. Up-sampling filtering is not applied in SNR scalability scenario.

## Inter-layer prediction

### Inter-layer pixel prediction

Inter-layer pixel prediction utilizes the reconstructed sample pixels of BL to produce the predictive signal to encode the collocated pixels in EL. It applies at CU level by a signal flag, denoted as inter_layer_skip_flag, to be added to CU syntax table to indicate if the current CU to apply inter-layer pixel prediction or not. When inter_layer_skip_flag is equal to 1, there is a flag, denoted as inter_layer_direct_flag, to indicate if residual signal exists or not for current CU. With these two flags, two new coding modes are proposed for CU coding of EL video:

ILSkip mode: inter-layer pixel prediction is used without any residual transmitted;

ILDirect mode: inter-layer pixel prediction is used with residual to be decoded.

The CU syntax table and semantics related to the inter-layer pixel prediction are described in section 3.3. The CU decoding flow chart with inter-layer pixel prediction is shown in Figure 2.

Figure 2 - Flowchart of CU decoding process with inter-layer pixel prediction

The inter-layer pixel prediction can be applied when the collocated BL blocks are coded as the inter mode or as intra mode. The multi-loop decoding process is required when the collocated BL blocks are coded as inter mode.

The predictive signal generated by inter-layer pixel prediction can be further modified by a refining filter, which is specified in section 2.3.5.

### Inter-layer residual refinement

The residual value of BL can be used to refine the residual of the intra/inter prediction for coding the video in EL. The magnitude of predicting error of EL can be can be reduced by adding the residual in BL to the prediction signal in EL. With the inter-layer residual refinement at EL encoder, the residual refined by the residual of BL will be transformed, quantized and entropy coded into bitstream. The integration of inter-layer residual refinement is dependent on the coding mode of the CU. When the CU in EL is coded by using intra modes, this contribution applies inter-layer residual refinement on CU level with a flag, denoted as inter_layer_residual_refine_for_intra_flag, to be added in syntax table to indciate if current intra CU uses inter-layer residual refinement or not. The CU syntax table and semantics related to the inter-layer residual refinement are described in section 3.3. Figure 3 provides the flowchart of intra CU decoding process with inter-layer residual refinement to be enabled in a SVC structure.

Figure 3 - Flowchart of intra CU decoding process with inter-layer residual refinement

On the other hand, if the CU in EL is coded by using inter modes, this contribution applies inter-layer residual refinement on PU level with a flag, denoted as inter_layer_residual_refine_for_inter_flag, to be added in syntax table to indciate if current inter PU uses inter-layer residual refinement or not. The PU syntax table and semantics related to the inter-layer pixel prediction are described in section 3.4. Figure 4 provides the flowchart of inter PU decoding process with inter-layer residual refinement to be enabled in a SVC structure.

Figure 4 – Flowchart of inter PU decoding process with inter-layer residual refinement

The availability of inter-layer residual refinement is conditional to the coding modes of the collocated BL blocks. When current EL PU is inter coded, this contribution only applies inter-layer residual refinement when the collocated blocks in BL are all coded in inter coding mode. When current EL PU is intra coded, this contribution only applies inter-layer residual refinement when the collocated blocks in BL are all coded in intra coding mode. If the collocated blocks in BL are mixed by inter and intra coded, the inter-layer residual refinement is not applied to the EL PU.

### Inter-layer motion vector prediction

We observe that there is a correlation for the motion patterns of the video object between BL video and EL video. With this in mind, we can utilize the motion information of BL as the prediction signal to encode for the video in EL. The motion parameters of BL such as the inter prediction direction, reference index and motion vectors, can be used as the additional candidate information to encoder for the inter coding for EL video. For Merge mode and AMVP mode, the motion information of BL, which aligns to the right-bottom corner of current PU in EL, is extracted and is placed as the first entry in the candidate list. With this motion information from the co-located BL added to the set of the motion information from EL, the maximum number of motion candidates is increased from 5 to 6 for Merge PUs and is increased from 2 to 3 for AMVP PUs. Figure 5 and Figure 6 pictorially illustrate the motion information candidates to use for the coding of video in EL.

Figure 5 - Merge candidates in coding video in EL

As shown in Figure 5, Merge candidate list is set up with the following four steps,

Step 1: Check motion parameter of A from the collocated block in BL;

Step 2: Check motion parameters of spatial neighbors B, C, D and E. If one of them is unavailable, check spatial neighbor F;

Step 3: Check motion parameter of temporal neighbor G;

Step 4: Process the candidates in steps 1-3 by pruning the duplicates to limit the list size equal to 6.

Figure 6 - AMVP candidates in coding video in EL

As shown in Figure 6, AMVP candidate list is set up with the following five steps,

Step 1: Check motion parameter of A from the collocated block in BL;

Step 2: Check motion parameter of the first left spatial neighbors in the order of B0, B1;

Step 3: Check motion parameter of the first top spatial neighbors in the order of C0, C1 and C2;

Step 4: Check motion parameter of temporal neighbor D;

Step 5: Process the candidates in steps 1-4 by pruning the duplicates to limit the list size equal to 3.

### Inter-layer pattern/mode prediction

Inter-layer pattern/mode prediction utilizes the splitting pattern and prediction information of BL to encode video in EL. To encode a CU in EL, the information of sub-CU quadtree structure, PU partitions and intra/inter prediction modes can be derived from collocated blocks in BL when all of the collocated BL blocks belong to a single CU. With the quadtree pattern projection from BL to EL, the partition sizes and prediction modes of each node CU in BL can be copied to the collocated CU in EL. Inter-layer pattern prediction transmits only the transform parameter and residual for the target CU in EL. Inter-layer pattern/mode prediction is an additional CU coding mode to code video in EL. A CU level control flag, denoted as inter_layer_pattern_mode_pred_flag, is added in CU syntax table to indicate if current CU uses inter-layer pattern/mode prediction or not. With this flag, a new coding mode called ILPMPred mode, is proposed for CU coding of EL video. The CU syntax table and semantics related to the inter-layer pattern/mode prediction are described in section 3.3. Figure 7 illustrates an example of inter-layer pattern/mode prediction.

Figure 7 - An example of inter-layer pattern/mode prediction

### Inter-layer refine filtering

With the inter-layer pixel prediction to produce the predicted pixel samples based on the reconstructed pixels from BL as described in section 2.3.1, the predicted pixel samples can be further processed by an inter-layer refining filter. This refining filter can improve the accuracy of inter-layer prediction to achieve better compression performance for video in EL. This contribution applies a 13-tap 2D filter on each predicted pixel sample value. The refining filter has the diamond shape as shown in Figure 8,

.

Figure 8 – Filter window of inter-layer refining filter

In Figure 8, P0 to P12 indicate the predicted pixel samples before filtering. Assume P0’ is the refined value of the center pixel P0, the filtering process can be expressed as the following equation,

P0’ = C0* P0 + C1* P1 + … + C12* P12 	(1)

where C0 to C12 are the filter coefficients. The filter coefficients are produced by encoder, and are adaptively refreshed to decoder according to the characteristics of the input video. This contribution uses picture-level refreshing of the filter by coding the coefficients in the first slice header of each picture in EL video.

In order to reduce the overhead of transmitting the coefficients, the refining filter is implemented in symmetry. Specifically, the filter taps whose positions are with respect to center with symmetry share the same coefficients, as described in following equations,

C0

C1 = C2;

C3 = C4;

Filter coefficients include:	C5 = C6;	(2)

C7 = C8;

C9 = C10;

C11 = C12.

With symmetry, the number of filter coefficients is reduced from 13 to 7. In this contribution, two sets of refining filter are used by Luma component and Chroma components, respectively. The total number of coefficients for each picture of EL video is equal to 14.

## Inter-frame prediction

The primary operation of inter-frame prediction is same as HM6.1.

## Intra-frame prediction

The primary operation of intra-frame prediction is same as HM6.1.

## In-loop filtering

### De-blocking filter

The primary operation of de-blocking filter is same as HM6.1.

### SAO

The primary operation of SAO is same as HM6.1.

### Other

As specified in section 2.3.5, this contribution applies a new refining filter to improve the performance of inter-layer prediction.

## Spatial transforms

The primary operation of transform schemes is same as HM6.1.

## Quantization

The primary operation of quantization schemes is same as HM6.1.

## Motion/mode/parameter coding

Ad described n section 2.3.3, for merge/skip PUs in EL video, the maximum number of motion candidates is increased from 5 to 6. For AMVP PUs in EL video, the maximum number of motion candidates is increased from 2 to 3.

### Merge/skip

The primary operation of merge/skip mode is same as HM 6.1.

### Motion vector coding

The primary operation of motion vector coding is same as HM 6.1.

### Intra prediction mode coding

The primary operation of intra prediction mode coding is same as HM 6.1.

### SAO parameter coding

The primary operation of SAO parameter coding is same as HM 6.1.

### Other

## Entropy coding of residual / transform coefficients

The primary operation Entropy coding is same as HM6.1.

## High-layer syntax

The primary syntax of high layer syntax follows HM 6.1.

## Additional algorithmic description discussion topic #1

## Additional algorithmic description discussion topic #2

## …

# Syntax and semantics description

As described in section 2, a collection of inter-layer coding algorithms are proposed in addition to the existing HEVC coding tools to improve the compression performance of EL videos. In this section, the syntax and semantics are modified based on the latest HEVC spec. JCTVC-K0030 [2].

## Proposed NAL unit header syntax and semantics

This section is related to 7.3.1.2 and 7.4.1.2 of JCTVC-K0030, which specify the NAL header syntax and semantics of HEVC.

In this contribution, the NAL header is extended by adding layer index. With the layer id information in NAL header, substreams with specific scalability can be extracted without parsing the RBSP data structure in the NAL unit. The proposed NAL header syntax table and semantics are shown in the following table,

Table 1 Proposed NAL header syntax table

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_unit_type | u(6) |
| reserved_zero_bits | u(2) |
| nuh_layer_id | u(4) |
| nuh_temporal_id_plus1 | u(3) |
| } |  |

nuh_layer_id specifies the scalable layer id of the RBSP data structure contained in the NAL unit. The variables  SpatialScalableId and SnrScalableId are derived as:

SpatialScalableId = nuh_layer_id >>2

SnrScalableId = nuh_layer_id&0x03

## Proposed SPS syntax and semantics

This section is related to 7.3.2.2 and 7.4.2.2 of JCTVC-K0030, which specify the sequence parameter set (SPS) syntax and semantics of HEVC.

In this contribution, SPS syntax is extended by adding several flags to signal if the inter-layer coding tools described in section 2.3 are used when coding the EL video. The proposed NAL header table and semantics are shown in the following table,

Table 2 Proposed SPS syntax

| seq_parameter_set_scalable_extension( ) { | Descriptor |
| --- | --- |
| /* share same syntax elements of 7.3.2.2 of JCTVC-K0030 */ |  |
| /* … */ |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) { |  |
| inter_layer_pixel_prediction_enable_flag | u(1) |
| if ( inter_layer_pixel_prediction_enable_flag ) { |  |
| inter_layer_refine_filter_enable_flag | u(1) |
| } |  |
| inter_layer_residual_refinement_enable_flag | u(1) |
| inter_layer_motion_prediction_enable_flag | u(1) |
| inter_layer_pattern_mode_prediction_enable_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

inter_layer_pixel_prediction_enabled_flag equal to 1 specifies the inter-layer pixel prediction may be used in coding tree blocks; inter_layer_pixel_prediction_enabled_flag equal to 0 specifies that asymmetric motion partitions cannot be used in coding tree blocks.

inter_layer_refine_filter_enabled_flag equal to 1 specifies the inter-layer refine filter is applied to the inter-layer pixel prediction process.

inter_layer_residual_refinement_enabled_flag equal to 1 specifies the inter-layer resdual refinementn may be used in coding tree blocks; inter_layer_pixel_prediction_enabled_flag equal to 0 specifies that asymmetric motion partitions cannot be used in coding tree blocks.

inter_layer_motion_prediction_enabled_flag equal to 1 specifies the inter-layer motion prediction may be used in coding tree blocks; inter_layer_pixel_prediction_enabled_flag equal to 0 specifies that asymmetric motion partitions cannot be used in coding tree blocks.

inter_layer_pattern_mode_prediction_enabled_flag equal to 1 specifies the inter-layer pixel prediction may be used in coding tree blocks; inter_layer_pixel_prediction_enabled_flag equal to 0 specifies that asymmetric motion partitions cannot be used in coding tree blocks.

## Proposed CU syntax and semantics

This section is related to 7.3.8.6 and 7.4.8.6 of JCTVC-K0030, which specify the CU syntax and semantics of HEVC.

As illustrated in section 2.3.1 and 2.3.4 of this contribution, three new coding modes for the CU in EL video are proposed: ILSkip, ILDirect and ILPMPred. Syntax and semantics corresponding to the three coding modes are shown in Table 3.

In addition, the proposed codec applies inter-layer residual refinement, which is described in section 2.3.2, to intra codec CU in EL video. The syntax and semantics are also presented in Table 3.

Table 3 Proposed coding unit syntax

| coding_unit_scalable_extension ( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| inter_layer_skip_flag[ x0 ][ y0 ] | ae(v) |
| if ( inter_layer_skip_flag[ x0 ][ y0 ] ) { |  |
| inter_layer_direct_flag[ x0 ][ y0 ] | ae(v) |
| if ( inter_layer_direct_flag[ x0 ][ y0 ] ) |  |
| transform_tree( x0, y0 x0, y0, log2CbSize, 0, 0 ) |  |
| } else { |  |
| if( transquant_bypass_enable_flag ) { |  |
| cu_transquant_bypass_flag | ae(v) |
| } |  |
| if( slice_type != I ) |  |
| skip_flag[ x0 ][ y0 ] | ae(v) |
| if( skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, log2CbSize ) |  |
| else { |  |
| inter_layer_pattern_mode_pred_flag[ x0 ][ y0 ] | ae(v) |
| if ( inter_layer_pattern_mode_pred_flag[ x0 ][ y0 ] ) { |  |
| coding_quadtree_ilpm_pred( x0, y0, log2CbSize, ctDepth ) |  |
| } else { |  |
| nCbS = ( 1 << log2CbSize ) |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( PredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ log2CbSize = = Log2MinCbSizeY ) |  |
| part_mode | ae(v) |
| if( PredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && log2CbSize >= Log2MinIpcmCbSizeY && log2CbSize <= Log2MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| if( pcm_flag[ x0 ][ y0 ] ) { |  |
| num_subsequent_pcm | tu(3) |
| NumPCMBlock = num_subsequent_pcm + 1 |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( x0, y0, log2CbSize ) |  |
| } else { |  |
| pbOffset = ( PartMode = = PART_NxN ) ? ( nCbS / 2 ) : 0 [Ed. (TK) Does this create an infinite loop when pbOffset = 0?] |  |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) { |  |
| prev_intra_luma_pred_flag[ x0 + i ][ y0+ j ] | ae(v) |
| } |  |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) { |  |
| if( prev_intra_luma_pred_flag[ x0 + i ][ y0+ j ] ) |  |
| mpm_idx[ x0 + i ][ y0+ j ] | ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 + i ][ y0+ j ] | ae(v) |
| } |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| inter_layer_residual_refine_for_intra_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { |  |
| if( PartMode = = PART_2Nx2N ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else if( PartMode = = PART_2NxN ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS, nCbS / 2 ) |  |
| } else if( PartMode = = PART_Nx2N ) { |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS ) |  |
| } else if( PartMode = = PART_2NxnU ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 4 ), nCbS, nCbS *3 / 4 ) |  |
| } else if( PartMode = = PART_2NxnD ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS *3 / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS * 3 / 4 ), nCbS, nCbS / 4 ) |  |
| } else if( PartMode = = PART_nLx2N ) { |  |
| prediction_unit( x0, y0, nCbS /4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 4 ), y0, nCbS *3 / 4, nCbS) |  |
| } else if( PartMode = = PART_nRx2N ) { |  |
| prediction_unit( x0, y0, nCbS *3 / 4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS * 3 / 4 ), y0, nCbS / 4, nCbS ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS / 2) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| } |  |
| } |  |
| if( !pcm_flag[ x0 ][ y0 ] ) { |  |
| if( PredMode[ x0 ][ y0 ] != MODE_INTRA && !(PartMode = = PART_2Nx2N && merge_flag[x0][y0]) ) |  |
| no_residual_syntax_flag | ae(v) |
| if( !no_residual_syntax_flag ) { |  |
| MaxTrafoDepth = ( PredMode[ x0 ][ y0 ] = = MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : max_transform_hierarchy_depth_inter ) |  |
| transform_tree( x0, y0 x0, y0, log2CbSize, 0, 0 ) |  |
| } |  |
| } /* else of inter_layer_pattern_mode_pred_flag[ x0 ][ y0 ] */ |  |
| } |  |
| } /* else of inter_layer_skip_flag[ x0 ][ y0 ] */ |  |
| } |  |

inter_layer_skip_flag[ x0 ][ y0 ] equal to 1 specifies that inter-layer pixel prediction is applied to the current coding unit. inter_layer_skip_flag[ x0 ][ y0 ] equal to 0 specifies that inter-layer pixel prediction is not applied to the current coding unit.

inter_layer_direct_flag[ x0 ][ y0 ] equal to 1 specifies that transform_tree() syntax is present for the current inter-layer pixel predicted coding unit. inter_layer_direct_flag[ x0 ][ y0 ] equal to 0 specifies that transform_tree() syntax is not present for the current inter-layer pixel predicted coding unit.

inter_layer_pattern_mode_pred_flag[ x0 ][ y0 ] equal to 1 specifies that coding_quadtree_ilpm_pred() syntax is present for the current inter-layer pixel predicted coding unit. inter_layer_pattern_mode_pred_flag [ x0 ][ y0 ] equal to 0 specifies that coding_quadtree_ilpm_pred() syntax is not present for the current inter-layer pixel predicted coding unit.

inter_layer_residual_refine_for_intra_flag[ x0 ][ y0 ] equal to 1 specifies that inter layer residual refinement is applied to the current coding unit. inter_layer_residual_refine_for_intra_flag[ x0 ][ y0 ] equal to 0 specifies that inter layer residual refinement is not applied to the current coding unit.

The coding_quadtree_ilpm_pred() syntax table corresponds to the sub-CU structure of ILPMPred mode, which is derived based on 7.3.8.4 of JCTVC-K0030 as following table shows,

Table 4 Proposed coding tree syntax for ILPMPred mode

| coding_quadtree_ilpm_pred( x0, y0, log2CbSize, ctDepth ) { | Descriptor |
| --- | --- |
| if( ( cu_qp_delta_enabled_flag && log2CbSize >= Log2MinCuQpDeltaSize ) { |  |
| IsCuQpDeltaCoded = 0 |  |
| CuQpDelta = 0 |  |
| } |  |
| if( split_cu_flag[ x0 ][ y0 ] ) { |  |
| x1 = x0 + ( ( 1 << log2CbSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2CbSize ) >> 1 ) |  |
| coding_quadtree_ilpm_pred( x0, y0, log2CbSize − 1, ctDepth + 1 ) |  |
| if( x1 < pic_width_in_luma_samples ) |  |
| coding_quadtree_ilpm_pred( x1, y0, log2CbSize − 1, ctDepth + 1 ) |  |
| if( y1 < pic_height_in_luma_samples ) |  |
| coding_quadtree_ilpm_pred( x0, y1, log2CbSize − 1, ctDepth + 1 ) |  |
| if( x1 < pic_width_in_luma_samples && y1 < pic_height_in_luma_samples ) |  |
| coding_quadtree_ilpm_pred( x1, y1, log2CbSize − 1, ctDepth + 1 ) |  |
| } else { |  |
| coding_unit_ilpm_pred( x0, y0, log2CbSize ) |  |
| } |  |
| } |  |

The coding_unit_ilpm_pred() syntax table corresponds to the leaf CU synatx in ILPMPred mode, which is derived based on 7.3.8.6 of JCTVC-K0030 as following table,

Table 5 Proposed coding unit syntax for ILPMPred mode

| coding_unit_ilpm_pred( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( slice_type == I ¦¦ !skip_flag[ x0 ][ y0 ] ) { |  |
| if( PredMode[ x0 ][ y0 ] != MODE_INTRA && !(PartMode = = PART_2Nx2N && merge_flag[x0][y0]) ) |  |
| no_residual_syntax_flag | ae(v) |
| if( !no_residual_syntax_flag ) { |  |
| MaxTrafoDepth = ( PredMode[ x0 ][ y0 ] = = MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : max_transform_hierarchy_depth_inter ) |  |
| transform_tree( x0, y0 x0, y0, log2CbSize, 0, 0 ) |  |
| } |  |
| } |  |
| } |  |

## Proposed PU syntax and semantics

This section is related to 7.3.8.7 and 7.4.8.7 of JCTVC-K0030, which specify the PU syntax and semantics of HEVC.

As illustrated in section 2.3.2 of this contribution, the proposed codec applies inter-layer residual refinement to each inter-coded PU in EL video. The corresponding syntax and semantics are shown as follows,

Table 6 Proposed prediction unit syntax

| prediction_unit_scalable_extension( x0, y0, nPbW, nPbH ) { | Descriptor |
| --- | --- |
| inter_layer_residual_refine_for_inter_flag[ x0 ][ y0 ] | ae(v) |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { /* MODE_INTER */ |  |
| merge_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( slice_type = = B ) |  |
| inter_pred_idc[ x0 ][ y0 ] | ae(v) |
| if( inter_pred_idc[ x0 ][ y0 ] != Pred_L1 ) { |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) |  |
| ref_idx_l0[ x0 ][ y0 ] | ae(v) |
| mvd_coding( x0, y0, 0 ) |  |
| mvp_l0_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( inter_pred_idc[ x0 ][ y0 ] != Pred_L0 ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) |  |
| ref_idx_l1[ x0 ][ y0 ] | ae(v) |
| if( mvd_l1_zero_flag && inter_pred_idc[ x0 ][ y0 ] = = Pred_BI) { |  |
| MvdL1[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL1[ x0 ][ y0 ][ 1 ] = 0 |  |
| } else |  |
| mvd_coding( x0, y0, 1 ) |  |
| mvp_l1_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

inter_layer_residual_refine_for_inter_flag[ x0 ][ y0 ] equal to 1 specifies that inter layer residual refinement is applied to the current prediction unit. inter_layer_residual_refine_for_inter_flag[ x0 ][ y0 ] equal to 0 specifies that inter layer residual refinement is not applied to the current prediction unit.

# Compression performance discussion

## Category 1 (HEVC base layer)

### Spatial scalability

### Intra-only spatial scalability

### SNR scalability

The compression performance is measured under the conditions provided by the CfP. Given the base layer bitstreams and the RA_HEVC_SNR configuration, sequences of Class A+ and Class B are tested. The average BD-bitrate are shown in Table 7. It is reported the proposed codec achieves 36.3% of BD-bitrate reduction on the EL-only actual rate testing case, 21.6% of BD-bitrate reduction on the EL-only target rate testing case, 20.5% of BD-bitrate reduction on the EL+BL actual rate testing case and 11.8% of BD-bitrate reduction on the EL+BL target rate testing case.

Table 7 Average BR-bitrate reduction of SNR scalability

|  |  | Random Access HEVC SNR |  |  |
| --- | --- | --- | --- | --- |
|  |  | Y | U | V |
| EL-only actual rate | Class A+ | -40.2% | -34.0% | -33.1% |
|  | Class B | -32.5% | -27.4% | -25.4% |
|  | Overall | -36.3% | -30.7% | -29.2% |
|  |  | -36.5% | -32.3% | -30.9% |
| EL-only target rate | Class A+ | -20.9% | -15.4% | -14.6% |
|  | Class B | -22.3% | -18.0% | -16.2% |
|  | Overall | -21.6% | -16.7% | -15.4% |
|  |  | -21.8% | -17.8% | -16.6% |
| EL+BL actual rate | Class A+ | -22.6% | -18.1% | -17.5% |
|  | Class B | -18.4% | -14.5% | -12.9% |
|  | Overall | -20.5% | -16.3% | -15.2% |
|  |  | -20.6% | -16.4% | -15.3% |
| EL+BL actual rate | Class A+ | -11.1% | -6.7% | -6.1% |
|  | Class B | -12.6% | -9.2% | -7.6% |
|  | Overall | -11.8% | -8.0% | -6.9% |
|  |  | -11.9% | -8.4% | -7.4% |

The detailed information for each testing case is reported in Table 8 to . The per-sequence bit rate and PSNR results are provided in the accompanied spreadsheet.

Table 8 Average BR-bitrate reduction of SNR scalability in EL-only actual rate testing case

| EL-only actual rate | Random Access HEVC SNR |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Traffic | -34.3% | -32.2% | -29.5% |
| PeopleOnStreet | -46.0% | -35.7% | -36.8% |
| Kimono | -39.5% | -35.1% | -34.1% |
| ParkScene | -33.3% | -30.9% | -29.3% |
| Cactus | -32.9% | -29.8% | -27.0% |
| BasketballDrive | -34.8% | -27.9% | -26.7% |
| BQTerrace | -22.1% | -13.4% | -9.7% |
| Delta -8 | -9.8% | -9.6% | -9.0% |
| Delta -6 | -19.7% | -16.7% | -15.7% |
| Delta -4 | -45.2% | -38.1% | -36.1% |
| Delta -2 | -84.8% | -84.1% | -83.8% |
| Average | -36.3% | -30.7% | -29.2% |

Table 9 Average BR-bitrate reduction of SNR scalability in EL-only target rate testing case

| EL-only target rate | Random Access HEVC SNR |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Traffic | -23.1% | -21.4% | -19.0% |
| PeopleOnStreet | -18.8% | -9.4% | -10.2% |
| Kimono | -20.8% | -17.4% | -16.8% |
| ParkScene | -23.2% | -21.2% | -20.1% |
| Cactus | -21.6% | -19.6% | -16.6% |
| BasketballDrive | -20.1% | -14.1% | -13.1% |
| BQTerrace | -25.6% | -17.9% | -14.5% |
| Delta -8 | -7.6% | -7.4% | -6.7% |
| Delta -6 | -14.8% | -11.6% | -10.6% |
| Delta -4 | -24.2% | -15.8% | -13.6% |
| Delta -2 | -57.6% | -57.0% | -56.4% |
| Average | -21.6% | -16.7% | -15.4% |

Table 10 Average BR-bitrate reduction of SNR scalability in EL+BL actual rate testing case

| EL+BL actual rate | Random Access HEVC SNR |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Traffic | -19.4% | -17.9% | -15.8% |
| PeopleOnStreet | -25.9% | -18.3% | -19.1% |
| Kimono | -22.1% | -19.1% | -18.5% |
| ParkScene | -18.7% | -16.9% | -15.8% |
| Cactus | -18.4% | -16.4% | -14.0% |
| BasketballDrive | -19.4% | -14.3% | -13.4% |
| BQTerrace | -13.4% | -5.9% | -2.8% |
| Delta -8 | -7.2% | -7.0% | -6.4% |
| Delta -6 | -12.5% | -9.4% | -8.4% |
| Delta -4 | -23.8% | -15.3% | -13.0% |
| Delta -2 | -39.8% | -38.9% | -38.1% |
| Average | -20.5% | -16.3% | -15.2% |

Table 11 Average BR-bitrate reduction of SNR scalability in EL+BL target rate testing case

| EL+BL target rate | Random Access HEVC SNR |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Traffic | -12.8% | -11.6% | -9.6% |
| PeopleOnStreet | -9.3% | -1.9% | -2.6% |
| Kimono | -10.6% | -8.1% | -7.6% |
| ParkScene | -12.8% | -11.3% | -10.4% |
| Cactus | -12.0% | -10.6% | -8.0% |
| BasketballDrive | -10.8% | -5.9% | -5.1% |
| BQTerrace | -16.8% | -10.0% | -7.1% |
| Delta -8 | -5.7% | -5.5% | -4.8% |
| Delta -6 | -9.3% | -6.1% | -5.1% |
| Delta -4 | -10.7% | -1.4% | 0.9% |
| Delta -2 | -24.4% | -23.7% | -22.7% |
| Average | -11.8% | -8.0% | -6.9% |

### Overall

## Category 2 (AVC base layer)

### Spatial scalability

### Overall

# Complexity analysis

## Encoding time and measurement methodology

The encoding run time is measured on the platform specified in section 5.3. Run time of the proposed codec is collected by encoding the sequences under the RA_HEVC_SNR configuration of CfP. According to the recommendation of the CfP, the run time for the HM6.1 encoding for the second lowest QP (QP = 22) at the EL is provided as anchor. In addition, the average encoding run time of the simulcast BL and EL is also collected. The comparison of encoding run time is shown in the following table,

Table 12 Encoding run time comparison

|  | Anchor Enc T [s] |  |  | Scalable Enc T [s] |
| --- | --- | --- | --- | --- |
|  | QP = 22 | Average of Simulcast BL QP = 26, 30, 34, 38 | Average of Simulcast EL QP = BL QP - [8, 6, 4, 2] |  |
| Traffic | 17003 | 13935 | 15384 | 35948 |
| PeopleOnStreet | 23130 | 17703 | 20237 | 42214 |
| Kimono | 9125 | 7134 | 8057 | 16316 |
| ParkScene | 7563 | 6201 | 6880 | 14406 |
| Cactus | 16391 | 12168 | 14260 | 28878 |
| BasketballDrive | 18195 | 14576 | 16427 | 33196 |
| BQTerrace | 22590 | 16046 | 19186 | 39035 |
| Average | 16285 | 12538 | 14347 | 29999 |

## Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM

The decoding time is measured on the platform specified in section 5.3. The YUV output is enabled during simulations. The bitstreams that contain one BL video substream and one EL video substream are decoded by the proposed codec and the run time is collected. According to the recommendation of the CfP, the run time for the HM6.1 decoding for the second lowest QP (QP = 22) at the higher layer is provided as anchor. In addition, the average decoding time of the simulcast BL and EL is also collected. The comparison of decoding run time is shown in the following table,

Table 13 Decoding run time comparison

|  | Anchor Dec T [s] |  |  | Scalable Dec T [s] |
| --- | --- | --- | --- | --- |
|  | QP = 22 | Average of Simulcast BL QP = 26, 30, 34, 38 | Average of Simulcast EL QP = BL QP - [8, 6, 4, 2] |  |
| Traffic | 61 | 47 | 55 | 123 |
| PeopleOnStreet | 96 | 68 | 79 | 156 |
| Kimono | 29 | 24 | 27 | 57 |
| ParkScene | 30 | 24 | 27 | 58 |
| Cactus | 58 | 41 | 50 | 110 |
| BasketballDrive | 65 | 52 | 59 | 126 |
| BQTerrace | 92 | 54 | 72 | 151 |
| Average | 62 | 44 | 53 | 111 |

## Description of computing platform used to determine encoding and decoding times reported in sections 5.1 and

The configurations of the software computing platform used to determine the encoding and decoding time are listed as follows,

Table 14 Computing configuration for encoding/decoding time determination

| CPU | Intel Core i7-3770K@ 3.5GHz (8 cores) |
| --- | --- |
| Memory | 16GB RAM |
| OS | Window 7 Enterprise (64bit) |
| Compiler | Microsoft Visual C++ 2008 |
| Executable | 64bit |
| Encoding | Run 2 tasks simultaneously |
| Decoding | Run 2 tasks simultaneously |

## Expected memory usage of encoder

The approximate maximum memory allocation of the proposed encoder software is measured in the implementation software. It was reported the expected memory used to encoding Class A+ sequences is about 5.8GB, and the expected memory used to encoding Class B sequences is about 1.6GB. The memory access of inter-layer residual refinement and inter-layer refining filter are expected to increase memory usage of the codec.

## Expected memory usage of decoder

The approximate maximum memory allocation of the proposed decoder software is measured. It was reported the expected memory used to decoding Class A+ sequences is about 2.5GB, and the expected memory used to decoding Class B sequences is about 900MB. The memory access of inter-layer residual refinement and inter-layer refining filter are expected to increase memory usage of the codec.

## Complexity characteristics of encoder motion estimation and partitioning selection in enhancement layer(s)

The motion estimation is identical to HM6.1.

For each CU in EL video, encoder needs to check three coding modes, such as ILSkip mode, ILDirect mode and ILPMPred mode, as described in section 2.3.1 and 2.3.4. For each PU in EL video, encoder checks if the inter-layer residual refinement should be used, as described in section 2.3.2. These added checking processes increase the complexity of the EL video encoder.

## Complexity characteristics of decoder motion compensation in enhancement layer(s)

The primary operation of motion compensation is identical to HM6.1.

## Complexity characteristics of encoder intra-frame prediction type and partitioning selection in enhancement layer(s)

The primary operation of intra prediction types is identical to HM6.1.

For each CU in EL video, encoder checks three coding modes, such as ILSkip mode, ILDirect mode and ILPMPred mode, as described in section 2.3.1 and 2.3.4. For each PU in EL video, encoder checks if the inter-layer residual refinement should be used, as described in section 2.3.2. All of these newly added checking processes increase the complexity of the EL video encoder.

## Complexity characteristics of decoder intra-frame prediction operation in enhancement layer(s)

The primary operation of intra prediction operations is identical to HM6.1.

## Complexity characteristics of upsampling filters and transforms specific in enhancement layer(s)

This contribution doesn’t apply upsampling filters in EL, and the transform schemes are identical to HM6.1.

## Complexity characteristics of quantization and inverse quantization in enhancement layer(s)

The quantization schemes are identical to HM6.1.

## Complexity characteristics of encoder entropy coding operation in enhancement layer(s)

The entropy encoding schemes are similar to HM6.1.

## Complexity characteristics of decoder entropy decoding operation in enhancement layer(s)

The entropy decoding schemes are similar to HM6.1.

## Degree of capability for encoder parallel processing

The parallel encoding capability is similar to HM6.1.

## Degree of capability for decoder parallel processing

The parallel decoding capability is similar to HM6.1.

## Additional complexity discussion topic #1

## Additional complexity discussion topic #2

## …

# Software implementation description

In this contribution, the proposed algorithms are implemented in C++ and in single-thread. The software is based on HM6.1. No specific optimization of the software implementation is explored in this contribution.

# Highlighted aspects discussion

# Closing remarks

This contribution presents Intel’s response to Joint Call for Proposals (CfP) on Joint Call for Proposals on Scalable Video Coding Extensions of High Efficiency Video Coding. It achieves SNR scalability by introducing a number of inter-layer coding algorithms, including inter-layer pixel prediction, inter-layer motion prediction, inter-layer residual refinement, inter-layer pattern/mode prediction and inter-layer refining filter. On average, the proposed codec shows 36.3% performance improvement under the RA_HEVC_SNR testing configuration.

# Reference

[1] Joint Call for Proposals on Scalable Video Coding Extensions of High Efficiency Video Coding (HEVC), ISO/IEC JTC 1/SC 29/WG 11 and ITU-T SG 16 WP 3, Stockholm, Sweden, July 2012.

[2] Proposed Editorial Improvements for High efficiency video coding (HEVC) Text Specification Draft 8, version 3, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-K0030, Shanghai, China, October 2012.

# Patent rights declaration(s)

Intel Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).