| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0040 |
| --- | --- |

| Title: | Syntax and semantics of scalable video coding extension to HEVC proposal by Nokia |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Oguz Bici, Done Bugdayci, Alireza Aminlou, Antti Hallapuro, Miska Hannuksela, Jani Lainema, Kemal Ugur Visiokatu 3, Tampere Finland | Email: | ext-oguz.bici@nokia.com; ext-done.bugdayci@nokia.com; ext-alireza.aminlou@nokia.com; miska.hannuksela@nokia.com; antti.hallapuro@nokia.com; jani.lainema@nokia.com; kemal.ugur@nokia.com; |
| Source: | Nokia |  |  |

_____________________________

# Syntax

Below are the syntax and semantics for the changed tools in HEVC. The changes are shown here with respect to HEVC draft standard version d07.

## CTU/TU/PU partitioning

coding_unit( x0, y0, log2CbSize ) {

…

if( PredMode[ x0 ][ y0 ] = = MODE_INTRA ) {

if(slice_type = = I && log2CbSize = = Log2MaxCbSize )

intra_skip_flag

if( intra_skip_flag )

return

if( PartMode = = PART_2Nx2N && pcm_enabled_flag && log2CbSize >= Log2MinIPCMCUSize && log2CbSize <= Log2MaxIPCMCUSize )

pcm_flag

if( pcm_flag ) {

num_subsequent_pcm

NumPCMBlock = num_subsequent_pcm + 1

while( !byte_aligned( ) )

pcm_alignment_zero_bit

pcm_sample( x0, y0, log2CbSize )

} else {

pbOffset = ( PartMode = = PART_NxN ) ? ( nCbS / 2 ) : 0

for( j = 0; j <= pbOffset; j = j + pbOffset )

for( i = 0; i <= pbOffset; i = i + pbOffset ) {

if( slice_type = = I )

pred_base_flag[ x0 + i ][ y0+ j ]

if ( ! pred_base_flag[ x0 + i ][ y0+ j ] )

Prev_intra_luma_pred_flag[ x0 + i ][ y0+ j ]

}

for( j = 0; j <= pbOffset; j = j + pbOffset )

for( i = 0; i <= pbOffset; i = i + pbOffset ) {

if ( ! pred_base_flag[ x0 + i ][ y0+ j ] ){

if( prev_intra_luma_pred_flag[ x0 + i ][ y0+ j ] )

mpm_idx[ x0 + i ][ y0+ j ]

else

rem_intra_luma_pred_mode[ x0 + i ][ y0+ j ]

}

}

if ( ! pred_base_flag[ x0 + i ][ y0+ j ] )

intra_chroma_pred_mode[ x0 ][ y0 ]

}

…

if( !pcm_flag ) {

if( PredMode[ x0 ][ y0 ] != MODE_INTRA && !(PartMode = = PART_2Nx2N && merge_flag[x0][y0]) && !(PartMode = = PART_2Nx2N && pred_base_flag [x0][y0]))

no_residual_syntax_flag

if( !no_residual_syntax_flag ) {

MaxTrafoDepth = ( PredMode[ x0 ][ y0 ] = = MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : max_transform_hierarchy_depth_inter )

transform_tree( x0, y0 x0, y0, log2CbSize, 0, 0 )

}

}

}

}

prediction_unit( x0, y0, nPbW, nPbH ) {

pred_base_flag[ x0 + i ][ y0+ j ]

if ( ! pred_base_flag[ x0 + i ][ y0+ j ] ){

…

}

}

intra_skip_flag when set to 1 indicates that the coding unit contains one prediction unit of type 2Nx2N, the prediction performed by copying the co-located reconstructed samples of the upsampled base layer and the residual decoding is omitted. When intra_skip_flag is not present, it is inferred to be 0.

pred_base_flag [ x0 ][ y0 ] specifies whether the associated prediction unit performs the prediction by copying the co-located reconstructed samples of the upsampled base layer. When pred_base_flag[x0][y0] is not present, it is inferred to be 0.

## Mode-dependent enhancement of intra prediction

8.4.4.2.6 Specification of Intra_DC (1) prediction mode

Inputs to this process are:

–	neighbouring samples p[ x ][ y ], with x, y = −1..2*nT−1,

–	upsampled base layer samples b[ x ][ y ], with x, y = 0..nT−1,

–	a variable nT specifying the transform block size,

–	a variable cIdx specifying the chroma component of the current block.

…

The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nT−1, are derived as the following ordered steps:

- When decoding base layer representation, variable dcVal is derived as:

dcVal = x'=0nS−1p[x'][−1]+y'=0nS−1p[−1][ y']+nS>>(k+1)
where k=Log2(nT)

When decoding enhancement layer representation, variable dcVal is derived as:

dcVal = y'=0nS−1x'=0nS−1b[x'][y']+(1<<(2⋅k−1))>>2⋅k
where k=Log2(nT)

8.4.4.2.6 Specification of Intra_Angular (2..34) prediction mode

Inputs to this process are:

–	intra prediction mode intraPredMode,

–	neighbouring samples p[ x ][ y ], with x, y = −1..2*nT−1,

–	upsampled base layer samples b[ x ][ y ], with x, y = −1..nT−1,

–	a variable nT specifying the transform block size,

–	a variable cIdx specifying the chroma component of the current block.

[Insert between steps 2.b and 2.c:]

c.    The following applies to the enhancement layer decoding, depending on the value of intraPredMode.

- If intraPredMode is equal to 10, the predSamples[ x ][ y ]  are derived by

predSamples[ x ][ y ] = Clip ( ref[ y+1 ] + b[ x ][ y ] – b[ x ][ -1 ] )

- Otherwise if intraPredMode is equal to 26, the predSamples[ x ][ y ]  are derived by

predSamples[ x ][ y ] = Clip ( ref[ x+1 ] + b[ x ][ y ] – b[ -1 ][ y ] )

- Otherwise, value of dcDelta is calculated as follows:

dcDelta = ( sum( b[ x ][ y ] ) + sum( predSamples[ x ][ y ] ) ) / (nT*nT)

and the values of predSamples[ x ][ y ] are updated as follows:

predSamples[ x ][ y ] = Clip ( predSamples[ x ][ y ] + dcDelta )

## Merge/skip and AMVP

8.5.2.1.1 Derivation process for luma motion vectors for merge mode

…

The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, and the prediction utilization flags predFlagL0 and predFlagL1 are derived as specified by the following ordered steps:

- The derivation process for merging candidates from neighboring prediction unit partitions in subclause 8.5.2.1.2 is invoked with the luma coding block location ( xC, yC ), the coding block size nCS, the luma prediction block location ( xP, yP ), the variable singleMCLFlag, the width and the height of the luma prediction block nPbW and nPbH and the partition index partIdx as inputs and the output is assigned to the availability flags availableFlagN, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N and the motion vectors mvL0N and mvL1N with N being replaced by A0, A1, B0, B1 or B2.

- The derivation process for merging candidates from corresponding base layer block in subclause 8.5.2.1.10 is invoked with the luma coding block location ( xC, yC ), the coding block size nCS, the luma prediction block location ( xP, yP ), the variable singleMCLFlag, the width and the height of the luma prediction block nPbW and nPbH and the partition index partIdx as inputs and the output is assigned to the availability flag availableFlagBase, the reference indices refIdxL0Base and refIdxL1Base, and the motion vectors mvL0Base and mvL1Base.

- The reference index for temporal merging candidate refIdxLX (with X being 0 or 1) is set equal to 0.

- The derivation process for temporal luma motion vector prediction in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), the width and the height of the luma prediction block nPbW and nPbH, and refIdxLX as the inputs and with the output being the availability flag availableFlagLXCol and the temporal motion vector mvLXCol. The variables availableFlagCol and predFlagLXCol (with X being 0 or 1, respectively) are derived as specified below.

availableFlagCol = availableFlagL0Col | | availableFlagL1Col	(8-1)

predFlagLXCol = availableFlagLXCol	(8-2)

- The merging candidate list, mergeCandList, is constructed as follows.

  - Base, if availableFlagBase is equal to 1

  - A1, if availableFlagA1 is equal to 1

  - B1, if availableFlagB1 is equal to 1

  - B0, if availableFlagB0 is equal to 1

  - A0, if availableFlagA0 is equal to 1

  - B2, if availableFlagB2 is equal to 1

  - Col, if availableFlagCol is equal to 1

- The variable numMergeCand and numOrigMergeCand are set to the number of merging candidates in the mergeCandList.

…

        - Derivation process for spatial merging candidates

Inputs to this process are

- a luma location ( xC, yC ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable nCS specifying the size of the current luma coding block,

- a luma location ( xP, yP ) specifying the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- a variable singleMCLFlag,

- variables specifying the width and the height of the luma prediction block, nPbW and nPbH,

- a variable availableFlagBase specifying the availability of the base layer prediction candidate,

- a variable partIdx specifying the index of the current prediction unit within the current coding unit.

…

–	If one or more of the following conditions are true with X being replaced by 0 and 1, the availableFlagN is set equal to 0, both components mvLXN are set equal to 0, refIdxLXN is set equal to −1 and predFlagLXN is set equal to 0.

- (xP >> (log2_parallel_merge_level_minus2 + 2)) is equal to (xN >> (log2_parallel_merge_level_minus2 + 2)) and (yP >> (log2_parallel_merge_level_minus2 + 2)) is equal to (yN >> (log2_parallel_merge_level_minus2 + 2)).

- N is equal to A0 and availableFlagBase + availableFlagA1 + availableFlagB0 + availableFlagB1 is equal to 4.

- N is equal to B2 and availableFlagA0 + availableFlagA1 + availableFlagB0 + availableFlagB1 is equal to 4 and  availableFlagBase is equal to 0.

- N is equal to B2 and availableFlagA0 + availableFlagA1 + availableFlagB0 + availableFlagB1 is equal to 3 and availableFlagBase is equal to 1.

- availableN is equal to FALSE

...

        - Derivation process for luma motion vector prediction

…

The motion vector predictor mvpLX is derived in the following ordered steps.

1.	The derivation process for motion vector predictor candidates from neighboring prediction unit partitions in subclause 8.5.2.1.6 is invoked with the luma coding block location ( xC, yC ), the coding block size nCS, the luma prediction block location ( xP, yP ), the width and the height of the luma prediction block nPbW and nPbH, refIdxLX (with X being 0 or 1, respectively), and the partition index partIdx as inputs and the availability flags availableFlagLXN and the motion vectors mvLXN with N being replaced by A, B as the output.

2.	The derivation process for motion vector predictor candidates from corresponding base layer block in subclause 8.5.2.1.9 is invoked with the luma coding block location ( xC, yC ), the coding block size nCS, the luma prediction block location ( xP, yP ), the width and the height of the luma prediction block nPbW and nPbH, refIdxLX (with X being 0 or 1, respectively), and the partition index partIdx as inputs and the availability flags availableFlagLXBase and the motion vectors mvLXBase as the output.

3.	If availableFlagLXBase and one of the availableFlagLXA and availableFlagLXB are equal to 1 or availableFlagLXBase is equal to 0 and both availableFlagLXA and availableFlagLXB are equal to 1 and mvLXA is not equal to mvLXB; If both availableFlagLXA and availableFlagLXB are equal to 1 and mvLXA is not equal to mvLXB, availableFlagLXCol is set equal to 0, otherwise, the derivation process for temporal luma motion vector prediction in subclause 8.5.2.1.7 is invoked with luma location ( xP, yP ), the width and the height of the luma prediction block nPbW and nPbH, and refIdxLX (with X being 0 or 1, respectively) as the inputs and with the output being the availability flag availableFlagLXCol and the temporal motion vector predictor mvLXCol.

4.	The motion vector predictor candidate list, mvpListLX, is constructed as follows.

1.	mvLXBase, if availableFlagLXBase is equal to 1

2.	mvLXA, if availableFlagLXA is equal to 1

3.	mvLXB, if availableFlagLXB is equal to 1

4.	mvLXCol, if availableFlagLXCol is equal to 1

1.	mvLXA, if availableFlagLXA is equal to 1

2.	mvLXB, if availableFlagLXB is equal to 1

3.	mvLXCol, if availableFlagLXCol is equal to 1

...

8.5.2.1.9 Derivation process for motion vector predictor candidates from corresponding base layer block

Inputs to this process are

- a luma location ( xC, yC ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable nCS specifying the size of the current luma coding block,

- a luma location ( xP, yP ) specifying the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- variables specifying the width and the height of the luma prediction block, nPbW and nPbH,

- the reference index of the current prediction unit partition refIdxLX (with X being 0 or 1),

- a variable scalFactor specifying the spatial scalability factor,

- a variable partIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

- the motion vectors mvLXBase of the corresponding base layer block,

- the availability flags availableFlagLXBase of the corresponding base layer block.

The variable isScaledFlagLX with X being 0 or 1 is set equal to 0.

The motion vector mvLXBase and the availability flag availableFlagLXBase are derived in the following ordered steps:

- The sample location (xBase, yBase) is set equal to ((xP + nPbW/2)/scalFactor, (yP + nPbH/2)/scalFactor).

- The availability flag availableFlagLXBase is set equal to 0 and the both components of mvLXBase are set equal to 0.

- The availability derivation process for a corresponding base layer block is invoked with the luma location ( xP, yP ), the width and the height of the luma prediction block nPbW and nPbH, the luma location ( xBase, yBase ) is set to ((xP + nPbW/2)/scalFactor, (yP + nPbH/2)/scalFactor) and the partition index partIdx as inputs and the output is assigned to the base layer prediction block availability flag availableBase.

- When availableBase is equal to TRUE, the variable isScaledFlagLX is set equal to 1.

- Using scalability factor     td and tb are initialized to smallest integers where td/tb is equal to scalability factor (For example in case of dyadic scalability td = 2 and tb = 1)

- For ( xBase, yBase ) the following applies repeatedly until availableFlagLXBase is equal to 1:

- When availableBase is equal to TRUE, PredModeBase[ xBase][ yBase] is not equal to MODE_INTRA and availableFlagLXBase is equal to 0, the following applies.

- If, PredFlagLXBase[ xBase ][ yBase ] is equal to 1 and the reference index refIdxLXBase[ xBase ][ yBase ] is equal to the reference index of the current prediction unit refIdxLX, availableFlagLXBase is set equal to 1 and the following assignments are made.

mvLXBase = MvLXBase[ xBase][ yBase]  / scalFactor

refIdxBase = RefIdxLXBase[ xBase ][ yBase ]

- Otherwise, if PredFlagLYBase[ xBase ][ yBase ] (with Y = !X) is equal to 1 and PicOrderCnt( RefPicListYBase[ refIdxLYBase[ xBase ][ yBase ] ] ) is equal to PicOrderCnt( RefPicListX[ refIdxLX ] ), availableFlagLXBase is set equal to 1 and the following assignments are made.

mvLXBase = MvLYBase[ xBase][ yBase] / scalFactor

refIdxBase = to RefIdxLYBase[ xBase ][ yBase ]

- When availableFlagLXBase is equal to 0, for ( xBase, yBase ) the following applies repeatedly until availableFlagLXBase is equal to 1:

- When availableBase is equal to TRUE, PredModeBase[ xBase][ yBase ] is not equal to MODE_INTRA and availableFlagLXBase is equal to 0, the following applies.

- If PredFlagLXBase[ xBase ][ yBase ] is equal to 1, availableFlagLXBase is set equal to 1 and the following assignments are made.

mvLXBase = MvLXBase[ xBase ][ yBase ] / scalFactor 			
          refIdxBase = RefIdxLXBase[ xBase ][ yBase ]	)
          refPicListBase = RefPicListXBase

Otherwise, if PredFlagLY[ xBase ][ yBase ] (with Y = !X) is equal to 1, availableFlagLXBase is set equal to 1 and the following assignments are made.

mvLXBase = MvLYBase[ xBase ][ yBase ]/scalFactor 	
refIdxBase = to RefIdxLYBase[ xBase ][ yBase ]					
refPicListBase = RefPicListYBase

- When availableFlagLXBase is equal to 1, and both refPicListBase[ refIdxBase ] and RefPicListX[ refIdxLX ] are short-term reference pictures, mvLXBase is derived as specified below.

tx = ( 16384 + ( Abs( td ) >> 1 ) ) / td	(8-3)

distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 ) >> 6 )

mvLXBase = Clip3( −32768, 32767, Sign2( distScaleFactor * MvLXBase[ xBase ][ yBase ]) * 			( (Abs( distScaleFactor * MvLXBase[ xBase ][ yBase ]) + 127 ) >> 8 ) )

where td and tb are derived as

td = Clip3( −128, 127, PicOrderCntVal – PicOrderCnt( refPicListBase[ refIdxBase] ) )

tb = Clip3( −128, 127, PicOrderCntVal – PicOrderCnt( RefPicListX[ refIdxLX ] ) )

8.5.2.1.10 Derivation process for base layer merging candidates

Inputs to this process are

–	a luma location ( xC, yC ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

–	a variable nCS specifying the size of the current luma coding block,

–	a luma location ( xP, yP ) specifying the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

–	variables specifying the width and the height of the luma prediction block, nPbW and nPbH,

–	the reference index of the current prediction unit partition refIdxLX (with X being 0 or 1),

–	a variable scalFactor specifying the spatial scalability factor,

–	a variable partIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process with X being replaced by 0 or 1 are

–	the availability flag availableFlagBase of the corresponding base block,

–	the reference indices refIdxLXBase of the corresponding base block,

–	the motion vectors mvLXBase of the corresponding base block.

The reference index of the base layer motion vector cannot be directly used for enhancement layer as it might refer to a different picture in enhancement layer. Let’s indicate the reference index which will be used for the enhancement layer motion as refIdxLXEnh. The refIdxLXEnh is derived by

refIdxLXEnh = refMapTable[LX][refIdxLXBase]

Let’s also denote the mapping from reference picture index to corresponding POC for base layer with POC = Ref2POCBase[LX][refIdx]  and the same mapping for enhancement layer is denoted by POC = Ref2POCEnh[LX][refIdx]. The refMapTable is initialized once per slice. This initialization can happen by various means:

- The values of refMapTable can be signaled in the bitstream to the decoder.

- The values of the refMapTable can be derived using the corresponding POC values of the reference pictures in enhancement and base layer reference pictures. This derivation could happen by:

  - Searching the reference list for both enhancement and base layer and deriving the refMapTable where the POC value of reference picture at index refIdxBase in base reference picture list corresponds to the same POC value of reference picture at index refIdxEnh in enhancement reference picture list.

  - Furthermore, this searching could take into account the corresponding weighted prediction parameters of each reference picture in the reference picture list.

  - If the aforementioned process could not find a mapping satisfying the above criteria the corresponding entry in the refMapTable could be set to 0.

For the derivation of availableFlagBase with width and the height of the luma prediction block being nPbW and nPbH and (xBase, yBase ) being ( (xP + nPbW/2)/scalFactor, (yP+ nPbH)/scalFactor ) the following applies.

–	If corresponding base layer block including (xBase, yBase) location prediction mode is equal to MODE_INTRA availableFlagBase is set to 0.

–	For either of the reference picture lists, if the POC value of the corresponding base layer reference picture for listX is different than POC value of the enhancement layer reference picture, availableFlagBase is set to 0.

–	Otherwise, availableFlagBase is set equal to 1 and the following assignments are made.

refIdxLXBase = RefIdxLXBase[ xBase][ yBase ]

refIdxLXEnh  = refMapTable[LX][refIdxLXBase]

refIdxLXBase   = refIdxLXEnh

mvLXBase = (MvLXBase[ xBase ][ yBase]/scaleFactor)

## Intra prediction mode coding

prediction_unit( x0, y0, log2CbSize ) {

if( skip_flag[ x0 ][ y0 ] ) {

if( MaxNumMergeCand > 1 )

merge_idx[ x0 ][ y0 ]

} else if( PredMode = = MODE_INTRA ) {

if( PartMode = = PART_2Nx2N && pcm_enabled_flag && log2CbSize >= Log2MinIPCMCUSize && log2CbSize <= Log2MaxIPCMCUSize )

pcm_flag

if( pcm_flag ) {

num_subsequent_pcm

NumPCMBlock = num_subsequent_pcm + 1

while( !byte_aligned( ) )

pcm_alignment_zero_bit

pcm_sample( x0, y0, log2CbSize )

} else {

prev_intra_luma_pred_flag[ x0 ][ y0 ]

if( prev_intra_luma_pred_flag[ x0 ][ y0 ] )

mpm_idx[ x0 ][ y0 ]

else

rem_intra_luma_pred_mode[ x0 ][ y0 ]

luma_intra_pred_equal_base_flag[x0 + i][y0 + j ]

if( !luma_intra_pred_equal_base_flag [ x0 + i ][ y0+ j ] ) {

luma_intra_pred_diff_sign_flag[ x0 + i ][ y0 + j ]

luma_intra_pred_diff_abs_equal1 _flag[ x0 + i ][ y0 + j

if( luma_intra_pred_diff_abs_level_equal1_flag [ x0 + i ][ y0+ j ] ) {

luma_intra_pred_diff_abs_equal2_flag[ x0 + i ][ y0 + j ]

if( luma_intra_pred_diff_abs_level_equal2_flag [ x0 + i ][ y0+ j ] )

luma_intra_pred_diff_abs_remaining[ x0 + i ][ y0 + j ]

}

intra_chroma_pred_mode[ x0 ][ y0 ]

}

} else { /* MODE_INTER */

merge_flag[ x0 ][ y0 ]

if( merge_flag[ x0 ][ y0 ] ) {

if( MaxNumMergeCand > 1 )

merge_idx[ x0 ][ y0 ]

} else {

if( slice_type = = B )

inter_pred_idc[ x0 ][ y0 ]

if( inter_pred_idc[ x0 ][ y0 ] != Pred_L1 ) {

if( num_ref_idx_l0_active_minus1 > 0 )

ref_idx_l0[ x0 ][ y0 ]

mvd_coding(mvd_l0[ x0 ][ y0 ][ 0 ], mvd_l0[ x0 ][ y0 ][ 1 ])

mvp_l0_flag[ x0 ][ y0 ]

}

if( inter_pred_idc[ x0 ][ y0 ] != Pred_L0 ) {

if( num_ref_idx_l1_active_minus1 > 0 )

ref_idx_l1[ x0 ][ y0 ]

if( mvd_l1_zero_flag && inter_pred_idc[ x0 ][ y0 ] = = Pred_BI) {

mvd_l1[ x0 ][ y0 ][ 0 ] = 0

mvd_l1[ x0 ][ y0 ][ 1 ] = 0

} else

mvd_coding( mvd_l1[ x0 ][ y0 ][ 0 ], mvd_l1[ x0 ][ y0 ][ 1 ] )

mvp_l1_flag[ x0 ][ y0 ]

}

}

}

}

luma_intra_pred_equal_base_flag[x0 + i][y0 + j ], luma_intra_pred_diff_sign_flag[x0 + i][y0 + j ], luma_intra_pred_diff_abs_equal1_flag[x0 + i][y0 + j ], luma_intra_pred_diff_abs_equal2_flag[x0 + i][y0 + j ] and luma_intra_pred_diff_abs_remaining[x0 + i][y0 + j ] specify the intra prediction mode for luma samples. Intra prediction mode is derived according to subclause 8.4.2

…

IntraPredMode[ xB ][ yB ] is derived by applying the following procedure:

- If luma_intra_pred_equal_base_flag[ xB ][ yB ] is equal to 1 then IntraPredMode[ xB ][ yB ] is equal to IntraPredModeBase[ xBase ][ yBase ] and it is set accordingly.

- Otherwise IntraPredMode[ xB ][ yB ] is derived as follows:

  - Sign of the difference between the intra layer prediction modes of corresponding base layer block and current block is determined as follows:

    - If luma_intra_pred_diff_sign_flag[ xB ][ yB ] is equal to 1 then a variable indicating the sign of the difference called signOfDiff  is set to 1.

    - Otherwise, signOfDiff is set to -1.

  - If luma_intra_pred_diff_abs_equal1_flag[ xB ][ yB ] is equal to 1 then a variable indicating the absolute value of the difference called magnOfDiff is set to 1.

  - Otherwise,

    - If luma_intra_pred_diff_abs_equal2_flag[ xB ][ yB ] is equal to 1 then magnOfDiff is set to 2.

    - Otherwise magnOffDiff is set to ( luma_intra_pred_diff_abs_remaining[ xB ][ yB ] + 3 ).

  - IntraPredMode[ xB ][ yB ] is set to ( IntraPredModeBase[ xBase ][ yBase ] - (signOfDiff * magnOfDiff ) ).

## Base-layer enhanced motion compensation prediction (BEMCP)

prediction_unit( x0, y0, nPbW, nPbH ) {

if( skip_flag[ x0 ][ y0 ] ) {

if( MaxNumMergeCand > 1 )

merge_idx[ x0 ][ y0 ]

} else { /* MODE_INTER */

merge_flag[ x0 ][ y0 ]

if( merge_flag[ x0 ][ y0 ] ) {

if( MaxNumMergeCand > 1 )

merge_idx[ x0 ][ y0 ]

} else {

if( slice_type = = B )

inter_pred_idc[ x0 ][ y0 ]

if( inter_pred_idc[ x0 ][ y0 ] != Pred_L1 ) {

if( num_ref_idx_l0_active_minus1 > 0 )

ref_idx_l0[ x0 ][ y0 ]

mvd_coding( x0, y0, 0 )

mvp_l0_flag[ x0 ][ y0 ]

}

if( inter_pred_idc[ x0 ][ y0 ] != Pred_L0 ) {

if( num_ref_idx_l1_active_minus1 > 0 )

ref_idx_l1[ x0 ][ y0 ]

if( mvd_l1_zero_flag && inter_pred_idc[ x0 ][ y0 ] = = Pred_BI) {

MvdL1[ x0 ][ y0 ][ 0 ] = 0

MvdL1[ x0 ][ y0 ][ 1 ] = 0

} else

mvd_coding( x0, y0, 1 )

mvp_l1_flag[ x0 ][ y0 ]

}

}

}

base_enh_mcp[ x0 ][ y0 ]

}

base_enh_mcp [ x0 ][ y0 ] specifies the type of base enhanced motion compensated prediction (BEMCP) to apply for the prediction unit. It can have three values: 0, 1 or 2. 0 indicates normal motion compensation, 1 indicates base enhance motion compensation with full base layer component added to the enhancement layer prediction and 2 indicates base enhance motion compensation with half of the base layer component added to the enhancement layer prediction. When base_enh_mcp[x0][y0] is not present, it is inferred to be 0.

[New section 8.5.2.3]

8.5.2.3	Decoding process for inter prediction samples in enhancement layersInputs to this process are:

–	a luma location ( xC, yC ) specifying the top-left sample of the current luma coding block relative to the top left luma sample of the current picture,

–	a luma location ( xB, yB ) specifying the top-left sample of the current luma prediction block relative to the top-left sample of the current luma coding block,

–	a variable nCS specifying the size of the current luma coding block,

–	variables specifying the width and the height of the luma prediction block, nPbW and nPbH,

–	luma motion vectors mvL0 and mvL1,

–	chroma motion vectors mvCL0 and mvCL1,

–	reference indices refIdxL0 and refIdxL1,

–	prediction list utilization flags, predFlagL0 and predFlagL1,

–	upsampled base layer sample arrays bL, bCb and bCr.

Outputs of this process are:

–	a (nCSL)x(nCSL) array predSamplesL of luma prediction samples, where nCSL is derived as specified in 8.5.2.2,

–	a (nCSC)x(nCSC) array predSamplesCb of chroma prediction samples for the component Cb, where nCSC is derived as specified in 8.5.2.2,

–	a (nCSC)x(nCSC) array predSamplesCr of chroma residual samples for the component Cr, where nCSC is derived as specified in 8.5.2.2.

The decoding process of section 8.5.2.2 is invoked to create preliminary enhancement layer prediction sample arrays predEnhL, predEnhCb and predEnhCb. If base_enh_mcp [ xC ][ yC ] is larger than 0, base layer prediction sample arrays predBaseL, predBaseCb and predBaseCb are also created by invoking the decoding process of 8.5.2.2 but substituting the reference picture sample arrays refPicLXL, refPicLXCb, and refPicLXCr with upsampled base layer reference pictures corresponding to the reference index refIdxLX in 8.5.2.2.1.

Depending on the value of base_enh_mcp [ xC ][ yC ] the following applies.

- If base_enh_mcp [ xC ][ yC ] is equal to 0:

predSamplesL[ x ][ y ] = predEnhL[ x ][ y ]

predSamplesCb[ x ][ y ] = predEnhCb[ x ][ y ]

predSamplesCr[ x ][ y ] = predEnhCr[ x ][ y ]

- Otherwise if base_enh_mcp [ xC ][ yC ] is equal to 1:

predSamplesL[ x ][ y ] = Clip ( predEnhL[ x ][ y ] + bL[ x ][ y ] - predEnhL[ x ][ y ] )

predSamplesCb[ x ][ y ] = Clip ( predEnhCb[ x ][ y ] + bCb[ x ][ y ] – predEnhCb[ x ][ y ] )

predSamplesCr[ x ][ y ] = Clip ( predEnhCr[ x ][ y ] + bCr[ x ][ y ] – predEnhCr[ x ][ y ] )

- Otherwise:

predSamplesL[ x ][ y ] = Clip ( predEnhL[ x ][ y ] + (bL[ x ][ y ] - predEnhL[ x ][ y ]) >> 1 )

predSamplesCb[ x ][ y ] = Clip ( predEnhCb[ x ][ y ] + (bCb[ x ][ y ] – predEnhCb[ x ][ y ]) >> 1)

predSamplesCr[ x ][ y ] = Clip ( predEnhCr[ x ][ y ] + (bCr[ x ][ y ] – predEnhCr[ x ][ y ]) >> 1 )

## High-level syntax

### Layer identifier and layer dependencies

reserved_zero_6bits in the NAL unit header is renamed to layer_id:

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_unit_type | u(6) |
| layer_idnuh_reserved_zero_6bits | u(6) |
| nuh_temporal_id_plus1 | u(3) |
| } |  |

Operation points are indicated in HEVC v1 VPS syntax as proposed in JVTVC-K0180 and indexed. The indexed operation points are used in the syntax below for mapping a combination of a profile and level to an operation point.

The following syntax structure is extracted from VPS as its own syntax structure that can be “called” multiple times:

| dpb_params( maxSubLayers) { | Descriptor |
| --- | --- |
| for( i = 0; i < maxSubLayers; i++ ) { |  |
| vps_max_dec_pic_buffering[ i ] | ue(v) |
| vps_max_num_reorder_pics[ i ] | ue(v) |
| vps_max_latency_increase[ i ] | ue(v) |
| } |  |
| } |  |

The VPS extension is specified as follows:

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| num_dimensions_minus1 | ue(v) |
| for( i = 0; i <= num_dimensions_minus1; i++ ) { |  |
| dimension_type[ i ] | u(4) |
| dimension_len[ i ] | u(8) |
| } |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| vps_layer_id[ i ] | u(v) |
| nuh_layer_id[ i ] | u(6) |
| num_direct_ref_layers[ i ] | u(6) |
| for( j = 0; j < num_direct_ref_layers[ i ]; j++ ) |  |
| ref_layer_id[ i ][ j ] | u(6) |
| } |  |
| num_profile_level | ue(v) |
| for( i = 1; i <= num_profile_level + 1; i++ ) { |  |
| prfl_lvl_num_op_points_minus1[ i ] | ue(v) |
| for( j = 0; j <= prfl_lvl_num_op_points_minus1[ i ]; j ++ ) |  |
| prfl_lvl_op_point_idx[ i ][ j ] | ue(v) |
| profile_tier_level( 1, vps_max_sub_layers_minus1 ) |  |
| } |  |
| num_dpb_params_minus1 | ue(v) |
| for( i = 1; i <= num_dpb_params_minus1; i++ ) { |  |
| dpb_params_num_layers_minus1[ i ] | ue(v) |
| for( j = 0; j <= dpb_params_num_layers_minus1; j++ ) |  |
| dpb_params_layer_id[ i ][ j ] | u(6) |
| dpb_params( vps_max_syb_layers_minus1 + 1 ) |  |
| } |  |
| alt_dec_order_flag | u(1) |
| if( alt_dec_order_flag ) { |  |
| num_dpb_params_alt | ue(v) |
| for( i = 0; i < num_dpb_params_alt; i++ ) { |  |
| dpb_params_alt_num_layers_minus1[ i ] | ue(v) |
| for( j = 0; j <= dpb_params_alt_num_layers_minus1; j++ ) |  |
| dpb_params_alt_layer_id[ i ][ j ] | u(6) |
| dpb_params( vps_max_syb_layers_minus1 + 1 ) |  |
| } |  |
| } |  |

vps_extension_byte_alignment_reserved_one_bit shall be equal to 1.

num_dimensions_minus1 plus 1 specifies the number of dimension types.

dimension_type[ i ] specifies the i-th scalability dimension type, as specified in the following table:

| dimension_type | dimension_id[ dimension_type ] |
| --- | --- |
| 0 | view order idx |
| 1 | depth flag |
| 2 | dependency ID |
| 3 | quality ID |
| 4..15 | reserved |

The value of dimension_type[ i ] shall be in the range of 0 to 3, inclusive, in bitstreams conforming to this Recommendation | International Standard. Other values for dimension_type[ i ] are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore values of dimension_type[ i ] that are not in the range of 0 to 3, inclusive.

dimension_len[ i ] specifies the number of bits of the i-th scalability dimension identifier within the combined VPS layer identifier.

vps_layer_id[ i ] specifies the values of DimensionId[ dimType ] for values of dimType in the range of 0 to num_dimensions_minus1, inclusive, as follows:

vpsLayerId = vps_layer_id[ i ]
for( j = num_dimensions_minus1; j >= 0; j-- ) {
	DimensionId[ j ] = vpsLayerId & ( ( 1 << dimension_len[ j ] ) – 1 )
	vpsLayerId = vpsLayerId >> dimension_len[ j ]
}

NOTE: In other words, vps_layer_id[ i ] is a bit-field of identifiers dimension_id[ dimension_type[ num_dimensions_minus1 ] ] .. dimension_id[ dimension_type[ 0 ] ] from most significant bit to least significant bit, where dimension_len[ i ] indicates the length of each dimension_id field in the bit-field.

nuh_layer_id[ i ] specifies that when the value of layer_id in the NAL unit header is equal to nuh_layer_id[ i ], values of DimensionId[ i ] are in effect in the decoding process.

num_direct_ref_layers[ i ] specifies the number of layers the i-th layer directly depends on.

ref_layer_id[ i ][ j ] identifies the j-th layer the i-th layer directly depends on.

ref_layer_id[ i ][ j ] identifies the j-th layer the i-th layer directly depends on.

num_profile_level specifies the number of profile_tier_level( ) syntax structures present in the VPS extension.

prfl_lvl_num_op_points_minus1[ i ] specifies the number of operation points for which the i-th profile_tier_level( ) syntax structure present in the VPS extension applies to.

prfl_lvl_op_point_idx[ i ][ j ], where j is in the range 0 to prfl_lvl_num_op_points_minus1[ i ], inclusive, specifies the operation points for which the i-th profile_tier_level( ) syntax structure present in the VPS extension applies to.

num_dpb_params_minus1 specifies the number of dpb_params( ) syntax structures present in the VPS for the primary decoding order. num_dpb_params_minus1 equal to 0 specifies that dpb_params( ) syntax structure for layer_id equal to 0 (as specified in video_parameter_set_rbsp syntax structure) applies to all layers.

dpb_params_num_layers_minus1[ i ] + 1 specifies the number of layers for which the i-th dpb_params( ) syntax structures present in the VPS for the primary decoding order applies to.

dpb_params_layer_id[ i ][ j ], where j is in the range of 0 to dpb_params_num_layers_minus1[ i ], specifies the layer_id values of the layers for which the i-th dpb_params( ) syntax structures present in the VPS for the primary decoding order applies to.

alt_dec_order_flag equal to 1 specifies that syntax elements related to an alternative decoding order are present.

num_dpb_params_alt specifies the number of dpb_params( ) syntax structures present in the VPS for the alternative decoding order.

dpb_params_alt_num_layers_minus1[ i ] + 1 specifies the number of layers for which the i-th dpb_params( ) syntax structures present in the VPS for the alternative decoding order applies to.

dpb_params_alt_layer_id[ i ][ j ], where j is in the range of 0 to dpb_params_alt_num_layers_minus1[ i ], specifies the layer_id values of the layers for which the i-th dpb_params( ) syntax structures present in the VPS for the alternative decoding order applies to.

### Slice extension

The slice extension is used for two purposes:

- inter_layer_reference_flag

- Alternative decoding order for uneven GOP lengths

The proposed slice layer syntax structure is as follows:

| slice_layer_rbsp( ) { | Descriptor |
| --- | --- |
| slice_header( ) |  |
| slice_data( ) |  |
| if( slice_extension_present_flag ) { |  |
| slice_extension_flag | u(1) |
| if( slice_extension_flag ) { |  |
| inter_layer_reference_flag | u(1) |
| if( alt_dec_order_flag ) { |  |
| num_foll | ue(v) |
| for( i = 0; i < num_foll; i++ ) { |  |
| foll_layer_id[ i ] | u(6) |
| foll_pic_order_cnt_lsb[ i ] | u(v) |
| } |  |
| alt_dec_order_rps_sps_flag | u(1) |
| if( alt_dec_order_rps_sps_flag ) |  |
| alt_dec_order_rps_idx | u(v) |
| else |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets + 1 ) |  |
| } |  |
| } |  |
| slice_extension2_flag | u(1) |
| if( slice_extension2_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| slice_extension_data_flag | u(1) |
| } |  |
| rbsp_slice_trailing_bits( ) |  |
| } |  |
| } |  |

slice_extension_flag specifies the presence of the defined slice extension syntax elements.

inter_layer_reference_flag equal to 1 specifies that the current picture may be used as reference for inter-layer prediction. inter_layer_reference_flag equal to 0 specifies that the current picture is not used as reference for inter-layer prediction.

num_foll specifies the number of component pictures constrained as follows. Let nextPic be the next component picture in decoding order that follows the current component picture. Let follPicSet be a set of component pictures that follow the current picture in the alternative decoding order and precede nextPic in the alternative decoding order. num_foll is equal to the number of component pictures in follPicSet.

foll_layer_id[ i ] is equal to the LayerId of the i-th component picture in follPicSet.

foll_pic_order_cnt_lsb[ i ] is equal to the pic_order_cnt_lsb of the i-th component picture in follPicSet.

alt_dec_order_rps_sps_flag equal to 0 specifies that a short-term reference picture set syntax structure is included in the slice extension and specifies additional component pictures included in RefPicSetStFoll. alt_dec_order_rps_sps_flag equal to 1 specifies that alt_dec_order_rps_idx is present.

alt_dec_order_rps_idx specifies the index of the short-term RPS that includes additional component pictures included in RefPicSetStFoll.

slice_extension2_flag equal to 0 specifies that no slice_extension_data_flag syntax elements are present in the slice layer RBSP syntax structure.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).