| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 11th Meeting: Shanghai, CN, 10-19 Oct., 2012 | Document: JCTVC-K0042 |
| --- | --- |

| Title: | Description of scalable video coding technology proposal by Fraunhofer HHI (Configuration A) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | H. Schwarz, C. Bartnik, P. Helle, T. Hinz, A. Khairat, H. Kirchhoffer, H. Laksman, D. Marpe, M. Siekmann, J. Stegemann, K. Sühring, T. Wiegand Einsteinufer 37 10587 Berlin Germany | Tel: Email: | +49-30-31002-226 heiko.schwarz@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This document describes the Fraunhofer HHI proposal as response to the Call for Scalable Video Coding Extensions of HEVC. The proposed coding scheme represents an extension of HEVC. It includes additional concepts for inter-layer prediction using already decoded or reconstructed base layer data. The proposal uses the same coding tools for both spatial and SNR scalability as well as for both AVC and HEVC base layers.

The proposal described in this document is similar to the proposal in JCTVC-K0043. The only difference is that the proposal described in this document does not include an adaptive loop filter for enhancement layer, while the proposal in document JCTVC-K0043 uses the adaptive loop filter as specified in version 6 of the HEVC draft. The bitstreams for both proposals have been generated using the same software.

# Contents

Abstract	1

Contents	1

1	Introduction	4

2	Algorithm description	4

2.1	CTU/TU/PU partitioning	4

2.2	Up-sampling filters	5

2.3	Inter-layer prediction	6

2.3.1	Inter-layer prediction of CU/TU/PU partitions	6

2.3.2	Inter-layer residual prediction	7

2.3.3	Inter-layer motion vector prediction	7

2.3.4	Inter-layer mode prediction	9

2.3.5	Inter-layer intra prediction	9

2.4	Inter-frame prediction	9

2.4.1	Conventional inter prediction (BL_INTER_STD)	10

2.4.2	Inter prediction using base layer residual samples (BL_INTER_RESI)	11

2.4.3	Inter prediction using difference pictures (BL_INTER_RECO)	12

2.5	Intra-frame prediction	13

2.5.1	Conventional intra prediction (BL_INTRA_STD)	13

2.5.2	Prediction using reconstructed base layer samples (BL_RECO_CPY)	13

2.5.3	Prediction using reconstructed and filtered base layer samples (BL_RECO_CPY_FILT)	14

2.5.4	Prediction using reconstructed base layer samples and spatial intra prediction using a difference signal (BL_INTRA_RECO)	14

2.5.5	Weighted prediction using reconstructed base layer samples and spatial intra prediction (BL_INTRA_COMB)	15

2.6	In-loop filtering	16

2.6.1	De-blocking filter	16

2.6.2	SAO	16

2.6.3	Other	16

2.7	Spatial transforms	16

2.8	Quantization	17

2.9	Motion/mode/parameter coding	17

2.9.1	Merge/skip	17

2.9.2	Motion vector coding	17

2.9.3	Intra prediction mode coding	17

2.9.4	SAO parameter coding	17

2.10	Entropy coding of residual / transform coefficients	17

2.11	High-layer syntax	18

2.12	Encoder control	18

3	Syntax and semantics description	19

3.1	Syntax	19

3.1.1	NAL unit syntax	19

3.1.2	Sequence parameter set RBSP syntax	19

3.1.3	Coding unit syntax	22

3.1.4	Base layer type syntax	23

3.1.5	Base copy transform tree syntax	24

3.1.6	Residual coding syntax	25

3.2	Semantics	27

3.2.1	NAL unit semantics	27

3.2.2	Sequence parameter set RBSP semantics	27

3.2.3	Coding unit semantics	28

3.2.4	Base layer type syntax	28

3.2.5	Base copy transform tree semantics	29

3.2.6	Residual coding semantics	29

3.3	Context modeling	30

3.3.1	Context modeling for flags indicating the used base layer prediction mode	30

3.3.2	Context modeling the syntax element adap_scan_idx	30

3.3.3	Context derivation for last significant position	30

3.3.4	Context derivation for sigGroupFlag	30

4	Compression performance discussion	30

4.1	Category 1 (HEVC base layer)	30

4.1.1	Spatial scalability	30

4.1.2	Intra-only spatial scalability	31

4.1.3	SNR scalability	31

4.1.4	Overall	31

4.2	Category 2 (AVC base layer)	31

4.2.1	Spatial scalability	31

4.2.2	Overall	31

5	Complexity analysis	31

5.1	Encoding time and measurement methodology	31

5.2	Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM	32

5.3	Description of computing platform used to determine encoding and decoding times reported in sections 5.1 and 5.2	32

5.4	Expected memory usage of encoder	32

5.5	Expected memory usage of decoder	33

5.6	Complexity characteristics of encoder motion estimation and partitioning selection in enhancement layer(s)	33

5.7	Complexity characteristics of decoder motion compensation in enhancement layer(s)	34

5.8	Complexity characteristics of encoder intra-frame prediction type and partitioning selection in enhancement layer(s)	34

5.9	Complexity characteristics of decoder intra-frame prediction operation in enhancement layer(s)	35

5.10	Complexity characteristics of upsampling filters and transforms specific in enhancement layer(s)	35

5.11	Complexity characteristics of quantization and inverse quantization in enhancement layer(s)	36

5.12	Complexity characteristics of encoder entropy coding operation in enhancement layer(s)	36

5.13	Complexity characteristics of decoder entropy decoding operation in enhancement layer(s)	36

5.14	Degree of capability for encoder parallel processing	36

5.15	Degree of capability for decoder parallel processing	36

6	Software implementation description	36

7	Highlighted aspects discussion	37

8	Closing remarks	37

9	Patent rights declaration(s)	37

# Introduction

The document describes an extension of HEVC for enabling spatial scalable and quality scalable video coding with an AVC or HEVC base layer. The same coding tools are used for all configurations.

The proposal covers the following categories:

 	Category 1 (HEVC base layer) spatial scalability

 	Category 1 (HEVC base layer) intra-only spatial scalability

 	Category 1 (HEVC base layer) SNR scalability

 	Category 2 (AVC base layer) spatial scalability

 	The proposal obeys the constraints under section 5 of the CfP (if box is not ticked, explain cases where constraints are violated)

# Algorithm description

A simplified block diagram of the proposed scalable extension of HEVC is depicted in Figure 1. For coding a spatial or quality enhancement layer, basically the same concepts as for single layer HEVC are used. A picture is partitioned into coding units, for which a prediction mode can be chosen. Inter-coded CUs are further partitioned into prediction units that are associated with a set of motion parameters. The residual for a CU is coded using a partitioning in transform units.

In a simple configuration, the coding architecture allows simulcasting the different spatial resolutions or quality levels. However, in order to improve the coding efficiency relative to simulcast, several inter-layer prediction techniques have been included in the design.

Figure 1: Simplified block diagram of the proposed scalable extension of HEVC.

The proposed inter-layer prediction techniques and coding modes are described in the following subsections.

## CTU/TU/PU partitioning

For coding the enhancement layers, the same partitioning concepts as in single-layer HEVC are used.

An enhancement layer picture is partitioned into coding tree blocks (CTBs). The size of the CTBs can be chosen by the encoder. A luma CTB covers a square picture area of NxN samples of the luma components and the associated chroma CTBs cover each a square picture area of (N/2)x(N/2) samples of each of the two chroma components. The value of N is transmitted inside the bitstream and can be 16, 32, or 64. The luma CTB and the two associated chroma CTBs, together with the associated syntax, form a coding tree unit (CTU).

The luma and chroma CTBs can be further partitioned into multiple coding blocks (CBs). The CTU contains a quadtree syntax that allows for splitting into square locks of variable size. The size of a CB can range from the same size as the CTB to a minimum size (8x8 luma samples or larger) that is specified by a syntax element transmitted to the decoder. The luma CB and the chroma CBs, together with the associated syntax, form a coding unit (CU).

For each CU, the prediction mode is signaled to the decoder. In single-layer HEVC, the prediction mode can be intra or inter. For enhancement layers, the prediction mode (intra or inter) is associated with additional data that indicate the usage of the conventional HEVC prediction modes or the usage of additional inter-layer prediction tools. If the additional data indicate the usage of inter-layer prediction, the intra or inter prediction process of single-layer HEVC is modified in a way that base layer data are used in the intra or inter prediction process. Detailed information about the additional enhancement layer prediction modes are given in sections 2.3.4 and 2.3.5.

For the purpose of intra prediction, the same partitioning as specified in HEVC is used. For the purpose of inter prediction, the partitioning of a coding unit into 1, 2, or 4 prediction units can be done in the same way as for single-layer HEVC. The partitioning patterns are depicted in Figure 2.

In summary, the partitioning into coding units and prediction units has not been changed relative to single-layer HEVC.

Figure 2: Possible partitionings of a coding unit into prediction units.

For the purpose of transform coding the prediction residual, a CU is partitioned into so called TUs, each of which represents a separate block to be transformed and quantized. The partitioning of a CU in transform units has not been changed relative to single-layer HEVC.

## Up-sampling filters

Various up-sampling filters are supported for spatial scalability modes. The base layer reconstructed pictures are up-sampled using 8-tap FIR filters for luma and bilinear filters for chroma, whereas, the base layer residual signals are up-sampled using bilinear filters for both luma and chroma.

The upsampling filters are 2-D separable, i.e., 1-D filters operate horizontally and vertically over the entire picture. The filter coefficients used for the phase offsets required for the investigated scalability ratios are listed in the following table:

| Phase offset | Filter coefficients |
| --- | --- |
| 0 | No filter |
| 1/3 | [ -1, 4, -11, 52, 26, -8, 3, -1]/64 |
| 1/2 | [-1, 4, -11, 40, 40, -11, 4, -1]/64 (Same as HEVC half-pel interpolation filter) |

The upsampling filter for phase offsets of 1/2 is the same as the half-sample motion-compensated interpolation filter specified in HEVC.

The luma sample at origin (top-left corner) of an enhancement layer picture is assumed to be phase aligned to the luma sample at origin of the corresponding base layer picture and the upsampling filters are selected sample-wise according to the phase offsets of other enhancement layer samples, determined to support a specific spatial scalability factor (e.g. 2x, 1.5x, etc). Other than the filters listed in the table above, phase offsets in a resolution of 1/16th of a sample are also supported in the upsampling stage.

An extended set of filters with approximately 1/16th phase offsets is also supported for arbitrary upsampling ratios, as shown in the following table:

| Phase offset | Filter coefficients |
| --- | --- |
| 0/16 | [ 0, 0, 0, 64, 0, 0, 0, 0]/64 |
| 1/16 | [ 0, 1, -3, 63, 4, -2, 1, 0]/64 |
| 2/16 | [ -1, 3, -7, 63, 8, -3, 1, 0]/64 |
| 3/16 | [ -1, 3, -8, 60, 14, -6, 3, -1]/64 |
| 4/16 | [ -1, 4, -10, 57, 19, -7, 3, -1]/64 |
| 5/16 | [ -1, 4, -11, 52, 26, -8, 3, -1]/64 |
| 6/16 | [ -1, 4, -11, 49, 30, -10, 4, -1]/64 |
| 7/16 | [ -1, 4, -11, 45, 34, -10, 4, -1]/64 |
| 8/16 | [ -1, 4, -11, 40, 40, -11, 4, -1]/64 |

## Inter-layer prediction

The proposal includes concepts for inter-layer prediction of the CU/PU partitioning, the coding mode, the motion parameters, the residual, and reconstructed samples. These inter-layer prediction methods are described in the following subsections.

### Inter-layer prediction of CU/TU/PU partitions

In P- or B-slices, for each quad tree level in the picture partitioning equal or lower (meaning towards smaller block sizes) to the CTU level, it can be signaled using the bl_copy_params_flag inside the bit stream, that the square block at the particular level completely infers the further partitioning into CUs and PUs and associated coding mode information from the base layer without any further signaling. However, partitioning of the inferred CUs into TUs is not inferred but signaled in the same manner as for a CU in HEVC.

If the bl_copy_params_flag indicates that the partitioning and the coding parameters for a current CU are inferred from the base layer, the derivation is done as follows:

Each sample position in the enhancement layer pel is mapped onto a co-located sample position pbl in the base layer by the following equations:

Thereby, the unit one represents the distance between two adjacent base layer sample positions. The result is a mapping of 4x4 blocks in the enhancement layer onto 4x4 blocks in the base layer for any offset in sampling rates between the enhancement and base layers.

The inferred partitioning is then derived recursively by joining 4x4 enhancement layer blocks mapped to 4x4 base layer blocks with identical coding parameters (i.e., motion parameters or intra modes) into PUs and CUs. The process continues until no further joining is possible. Further joining is impossible when a joined PU would contain multiple different coding parameters or all 4x4 blocks inside the block are already joined into one PU.

The motion parameters that are associated with the enhancement layer Pus are directly derived from the co-located base layer PUs. The number of motion hypotheses and the reference frame indices are directly copied; the motion vectors are scaled according to the resolution ration between base and enhancement layer.

For coding units with bl_copy_params_flag equal to 1, no further motion parameters are included in the bitstream.

The upsampling of mode information and motion parameters from the base layer proceeds as follows:

For each finest partition in an enhancement layer CTU a corresponding co-located position in the base layer is determined according to ratio of enhancement layer picture and base layer picture width and the ratio of enhancement layer picture height and base layer picture height. The obtained base layer position is used to access the base layer data partition and copy the data into the enhancement layer partition. The partition size is initially set to the minimum supported PU size.

After all data partitions in the enhancement layer are filled with data from the base layer the partition size and depth of sub-partitions in the CTU are determined by a recursive combination process starting with finest granularity by comparing syntax elements. This process operates as follows.

If in a specific partition the four sub-partitions do not have the same prediction modes, then the combination process is terminated for this partition. Otherwise, depending on the prediction mode, intra prediction data or motion data are compared. If the compared data are equal for all sub-partitions, the depth is decreased and the combination process is done on the next smaller depth. Otherwise, if the information differs depending on the prediction mode, the depth is fixed and no further combination is done for the considered depth. In case the prediction mode is inter, the partition size is determined by comparing the sub-partition's motion parameters.

The process of mode and motion parameter upsampling always yields CU and PU partitionings that are conforming to the HEVC specification.

### Inter-layer residual prediction

Inter-layer residual prediction can be used in combination with motion-compensated prediction. The corresponding prediction mode is denoted as BL_INTER_RESI and is described in detail in section 2.4.2.

### Inter-layer motion vector prediction

This section describes the usage of the already coded motion information of the base layer to improve motion vector prediction of the enhancement layer. For employing base layer motion parameters for improving the motion parameter coding, the decoding process for motion vectors and the decoding process for motion parameters for merged blocks has been modified. The syntax for signaling the motion parameters has not been changes.

#### Modifications of motion vector prediction

As of HM6.0 the advanced motion vector prediction uses two spatial and one temporal motion vector predictor that are used as possible candidates for motion vector prediction. In this proposal an additional inter-layer motion vector predictor may be derived extending the candidate list by one item. The collocated base layer prediction block, which covers the sample position C0 depicted in Figure 3, is used to derive the motion data for the possible candidate.

Figure 3: Collocated base layer prediction block indicating the center positions.

If the identified base layer block contains motion information for the considered reference picture list and reference picture index, the corresponding motion vector is inserted at the first position in the candidate list of the current prediction block in the enhancement layer. The inserted motion vector is scaled according to the resolution ratio. If the candidate is added to the list or if there is no motion data available the list construction and motion compensation is pursued subsequently as in single-layer HEVC.

#### Modifications of motion parameter derivation for merged blocks

A second inter-layer motion vector predictor may be utilized by the block merging algorithm. At the beginning of the merge list construction the collocated base layer prediction block that covers the sample position C0 is used to infer a possible set of motion parameters as a candidate to the top of the list. However, the derived set of motion parameters are only inserted to list if the merge flag of the collocated block in the base layer is equal to 0. Otherwise the set of motion parameters is not considered as a candidate and the list construction is continued in the same manner as in single-layer HEVC. For spatial scalability, the base layer motion vectors are scaled according to the resolution ratio. The total length of the merge candidate list is not extended and remains the same. After the candidate list construction is finished, a re-ordering process may be invoked depending on the collocated base layer prediction block that covers the top left sample position inside the current prediction block. If the merge flag of this block equals to 1 the origin O of the motion vector predictor in the base layer is retrieved. Figure 4 depicts the origins of the spatial and temporal candidates that may occur. The candidate list of the enhancement layer is now checked if a merge candidate exists which is from the corresponding origin. If a candidate is found it is moved to the top of the list and addressed by the lowest index. The order of the remaining entries in the candidate list is retained and their indices are increased by one.

Figure 4: Possible sample locations of the origins O of the motion vector predictor of the merge candidates.

### Inter-layer mode prediction

Coding modes are predicted from base layer information if the syntax element bl_copy_params_flag is equal to 1. In this case, for an enhancement layer 4x4 block mapped to a base layer 4x4 block (as described in 2.3.1) which is inter predicted, the corresponding motion parameters are copied to the enhancement layer 4x4 block and will be used for motion compensation. The spatial displacement components of the motion parameters are rounded to the nearest available value in the enhancement layer sampling grid.

What kind of reference pictures are used in the motion compensation for any inter prediction performed is signaled once for the whole CU for which the partitioning and the coding parameters are inferred.

For an enhancement layer 4x4 block mapped to a base layer 4x4 block which is intra predicted, the enhancement layer 4x4 block is also intra predicted, but using the BL_RECO_COPY mode reconstruction mode described in section 2.5.2, i.e. by using the reconstructed and (upsampled) base layer signal for prediction.

### Inter-layer intra prediction

Various added enhancement layer modes use reconstructed base layer samples for improving the prediction in the enhancement layer. These modes include the BL_INTER_RECO mode described in section 2.4.3, the BL_RECO_CPY mode described in section 2.5.2, the BL_RECO_CPY_FILT mode described in section 2.5.3, the BL_INTRA_RECO mode described in section 2.5.4, and the BL_INTRA_COMB mode described in section 2.5.5.

## Inter-frame prediction

This section describes the provided methods to obtain an inter prediction signal for enhancement layer coding/prediction units. In addition to the conventional motion-compensated prediction as specified in HEVC, additional enhancement layer inter prediction modes are provided, which use, in addition to reconstructed samples of already coded enhancement layer pictures, base layer samples for generating the prediction signal for a coding unit. The used inter prediction mode is signaled by the variable BLType (see section 3.2.4). If BLType is equal to BL_INTER_STD, the conventional motion-compensated prediction of single-layer HEVC is used; otherwise, one of the inter-layer inter prediction modes described below is used.

For the provided methods of inter-layer inter prediction, the reconstructed samples of the base layer or the reconstructed residual samples of the base layer are used. If the resolution of the base layer is different from the resolution of the enhancement layer, the reconstructed base layer samples or reconstructed base layer residual samples are upsampled according to the resolution ratio. The upsampling is described in section 2.2.

The provided inter-layer inter prediction modes differ in the base layer signals that are accessed from already reconstructed base layers and already reconstructed enhancement layers.

The following variables are used in the description of the following subsections:

t0 – time index of current picture

t1 – time index of an already reconstructed picture

EL – enhancement layer

BL – base layer

EL(t0) – current enhancement layer picture to be coded

EL_reco – enhancement layer reconstruction

BL_reco – base layer reconstruction

BL_resi – base layer residual signal, i.e. inverse transform of base layer transform coefficients

EL_diff – difference between enhancement layer reconstruction and 
                (upsampled) base layer reconstruction

The used signals are illustrated in Figure 5.

Figure 5: Illustration of picture buffers in base layer and enhancement layer.

### Conventional inter prediction (BL_INTER_STD)

When BLType is equal to BL_INTER_STD, the conventional motion-compensated prediction as specified in single-layer HEVC is used. The transmitted or inferred motion vectors point to a reference area in already reconstructed enhancement layer pictures. Base layer pictures are not used in this mode.

In short, the prediction for enhancement layer samples can be written as:

prediction = MCP_filter( EL_reco(t1) ).

The conventional motion-compensated prediction is illustrated in Figure 6.

Figure 6: BL_INTER_STD mode using HEVC motion compensation technique that accesses already reconstructed enhancement layer reference pictures.

### Inter prediction using base layer residual samples (BL_INTER_RESI)

The prediction mode described in the following is used when BLType is equal to BL_INTER_RESI.

In the BL_INTER_RESI mode the base layer residual signal (inverse transform of base layer transform coefficients) is added to the motion-compensated enhancement layer prediction signal to obtain the final prediction for the current enhancement layer PUs. For the enhancement layer motion compensation, it uses the same inter prediction technique as in HEVC. The motion vectors point to areas of already reconstructed enhancement layer pictures. The base layer reconstructed signal is not used in this mode, but only the base layer residuals. The region in the base layer picture co-located with the considered region in enhancement layer picture is identified and the residual signal is taken from the identified base layer region. In case of spatial scalability, the base layer residual is upsampled using a bilinear filter to the resolution of the enhancement layer before the summation.

In short, the prediction for enhancement layer samples can be written as:

prediction = filter( BL_resi(t0) ) + MCP_filter( EL_reco(t1) ).

It is also possible that 2 hypotheses of the enhancement layer reconstruction signals are used, e.g.

prediction = filter( BL_resi(t0) ) + MCP_filter1( EL_reco(t1) ) + MCP_filter2( EL_reco(t2) ).

The formation of the prediction signal in the BL_INTER_RESI mode is illustrated in Figure 7.

Figure 7: BL_INTER_RESI mode combines the base layer residual signal with the motion compensated enhancement layer signal.

### Inter prediction using difference pictures (BL_INTER_RECO)

The prediction mode described in the following is used when BLType is equal to BL_INTER_RECO.

In the BL_INTER_RECO mode, the base layer reconstructed signal is added to a motion-compensated enhancement layer difference signal to obtain the final prediction for the current enhancement layer PUs. The enhancement layer difference signal can be defined as:

- the difference between enhancement layer reconstruction and (upsampled) base layer reconstruction, or

- difference between enhancement layer reconstruction before or between loop filtering stages (like Deblocking, SAO, ALF) and (upsampled) base layer reconstruction.

The proposed software supports these variants of the enhancement layer difference and the submitted bitstream uses the difference between enhancement layer reconstruction after deblocking (and before SAO) and the (upsampled) base layer reconstruction.

For the enhancement layer motion compensation, the same inter prediction technique as in single-layer HEVC is used. The motion vectors point to areas of an enhancement layer difference signals in this case. Base layer residual signal is not used in this mode, but only the reconstructed samples. The region in the base layer picture co-located with the considered region in enhancement layer picture is identified and the reconstruction signal is taken from the identified base layer region. In case of spatial scalability, the base layer reconstruction is upsampled using an 8-tap filter to the resolution of the enhancement layer before the summation.

In short, the prediction for enhancement layer samples can be written as:

prediction = filter( BL_reco(t0) ) + MCP_filter( EL_diff(t1) ).

It is also possible that 2 hypotheses of the enhancement layer difference signals are used, e.g.

prediction = filter( BL_resi(t0) ) + MCP_filter1( EL_diff(t1) ) + MCP_filter2( EL_diff(t2) ).

The formation of the prediction signal in the BL_INTER_RECO mode is illustrated in Figure 8.

Figure 8: BL_INTER_RECO mode combines the base layer reconstruction signal with the motion compensated enhancement layer difference signal.

## Intra-frame prediction

This section describes the provided methods to obtain an intra prediction signal for enhancement layer coding units. In addition to the spatial intra prediction as specified in HEVC, additional enhancement layer intra prediction modes are provided, which use, in addition to reconstructed samples of the current enhancement layer picture, base layer samples for generating the prediction signal for a coding unit. The used intra prediction mode is signaled by the variable BLType (see section 3.2.4). If BLType is equal to BL_INTRA_STD, the conventional intra prediction of single-layer HEVC is used; otherwise, one of the inter-layer intra prediction modes described below is used.

For the provided methods of inter-layer intra prediction, the reconstructed samples of the base layer are used. If the resolution of the base layer is different from the resolution of the enhancement layer, the reconstructed base layer samples are upsampled according to the resolution ratio. The upsampling is described in section 2.2.

### Conventional intra prediction (BL_INTRA_STD)

When BLType is equal to BL_INTRA_STD, the conventional spatial intra prediction as specified in single-layer HEVC is used.

### Prediction using reconstructed base layer samples (BL_RECO_CPY)

The prediction mode described in the following is used when BLType is equal to BL_RECO_CPY.

The prediction signal for the BL_RECO_CPY mode is obtained as follows:

- If the resolution of the base layer signal is the same as the resolution of the enhancement layer signal, the prediction signal is formed by the co-located reconstructed samples of the base layer.

- Otherwise (the resolution of the base layer signal is less than the resolution of the enhancement layer signal), the prediction signal is formed by upsampling the reconstructed samples of the co-located area in the base layer.

The final reconstruction signal is obtained by adding a residual signal to the prediction signal. The residual signal is coded by transform coding.

The BL_RECO_CPY mode is available for all slices types. In P and B slices this mode is implicitly used when bl_copy_params_flag is equal to 1 and the current coding unit completely or partially covers an intra partition of the base layer. For the each of the co-located intra partitions, the BL_RECO_CPY mode is used to reconstruct the corresponding partition of the enhancement layer.

The mode can be used at any CU depth.

If this mode is signaled no further information is transmitted in the bitstream for specifying the prediction.

### Prediction using reconstructed and filtered base layer samples (BL_RECO_CPY_FILT)

The prediction mode described in the following is used when BLType is equal to BL_RECO_CPY_FILT.

The prediction signal for the BL_RECO_CPY_FILT mode is obtained as follows:

- A first prediction signal for the current block extended by one sample at each side is obtained as described in section 2.5.2.

- The final prediction signal is generated by filtering the first prediction signal with the filter (1,2,1)/4 in both horizontal and vertical direction.

The BL_RECO_CPY_FILT mode is available for all slices types.

The mode can be used at any CU depth.

If this mode is signaled no further information is transmitted in the bitstream for specifying the prediction.

### Prediction using reconstructed base layer samples and spatial intra prediction using a difference signal (BL_INTRA_RECO)

The prediction mode described in the following is used when BLType is equal to BL_INTRA_RECO.

The prediction signal for the BL_INTRA_RECO mode is constructed by the sum of two intermediate prediction signals, as illustrated in Figure 9.

The first prediction signal is derived in the same way as the prediction signal for the BL_RECO_CPY mode described in section 2.5.2. It is built by copying or upsampling the reconstructed base layer samples of the co-located area in the base layer.

The second prediction signal is derived by spatial intra prediction using an intermediate difference signal for neighboring samples of already reconstructed blocks. The intermediate difference signal is calculated as pixel-wise difference between the reconstructed enhancement layer signal and the, in case of spatial scalability, upsampled reconstructed base layer signal of the co-located area. To this difference signal the value of 128 is added and the signal is clipped to the range [0; 255] before it is used for spatial intra prediction. With this intermediate difference signal a spatial intra prediction as specified in HEVC is applied to obtain a spatially predicted difference signal. After the prediction the value 128 is subtracted from the intermediate predicted difference signal.

The final prediction signal is sample-wise added to the upsampled or copied reconstructed base layer signal to obtain the final prediction signal.

Figure 9: Generation of an intra prediction signal by a sum of an (upsampled/filtered) base layer reconstruction signal (BL Reco) and a spatial intra prediction that uses a difference signal (EH Diff) of already coding neighboring blocks, where the difference signal (EH Diff) for the already coded blocks is generated by subtracting an (upsampled/filtered) base layer reconstruction signal (BL Reco) from a reconstructed enhancement layer signal (EH Reco).

The intra prediction modes that are used for spatial intra prediction of the difference signal are coded using the conventional HEVC syntax.

### Weighted prediction using reconstructed base layer samples and spatial intra prediction (BL_INTRA_COMB)

The prediction mode described in the following is used when BLType is equal to BL_INTRA_COMB.

The prediction signal for the BL_INTRA_COMB mode is constructed by a weighted sum of two intermediate prediction signals, where different weights are used for different frequency components, as schematically illustrated in Figure 10.

The first prediction signal is derived in the same way as the prediction signal for the BL_RECO_CPY mode described in section 2.5.2. It is built by copying or upsampling the reconstructed base layer samples of the co-located area in the base layer.

The second prediction signal is derived by conventional spatial intra prediction using reconstructed samples of already reconstructed blocks.

Both intermediate prediction signals are forward transformed using a transform of the corresponding block size. In the next step the obtained transform coefficients are combined using position dependent weights. The combined signal is then inverse transformed to obtain the final prediction signal. If the prediction block size exceeds 32x32, the block is split into blocks of 32x32 and for each of the 32x32 the combination of the prediction signals in the transform domain is done separately.

Figure 10: Generation of an intra prediction signal by a frequency-weighted sum of an (upsampled/filtered) base layer reconstruction signal (BL Reco) and a spatial intra prediction that uses reconstructed enhancement layer samples (EH Reco) of already coding neighboring blocks.

The intra prediction modes that are used for spatial intra prediction of the difference signal are coded using the conventional HEVC syntax.

## In-loop filtering

### De-blocking filter

For this proposal, the same de-blocking filter as specified in HEVC is applied except for one modification, which is described in the following. If none of the adjacent block is spatially intra predicted, but at least one of the adjacent blocks is inter-layer intra predicted (using the BL_RECO_CPY or the BL_RECO_CPY_FILT mode), the boundary strength is modified according to the following condition:

- If the considered boundary is a transform block boundary and at least one of the two adjacent blocks contains residual data, the boundary strength is set to 1.

- Otherwise the boundary strength is set to 0.

### SAO

The Sample Adaptive Offset filter as specified in HEVC is applied to the enhancement layer signal after the de-blocking filter. The SAO filter can also be switched off as indicated in the syntax; but for all generated bitstreams, the SAO filter has been applied.

### Other

Beside the deblocking filter and the SAO filter, no further in-loop filtering has been used. This is the only difference to the proposal described in document JCTVC-K0043, in which an additional adaptive loop filter as specified in version 6 of the HEVC draft has been applied to the reconstructed enhancement layer signal.

## Spatial transforms

The proposal uses the same transforms as defined in HEVC for block sizes of 4x4 up to 32x32, independent of the type of base layer (HEVC, H.264/AVC).

## Quantization

The proposal uses the same quantization process as defined in HEVC, independent of the type of base layer (HEVC, H.264/AVC).

## Motion/mode/parameter coding

### Merge/skip

The coding of the merge flag and the merge index in this proposal is the same as in single-layer HEVC. Differences for the construction of the candidate list for merged blocks are described in section 2.3.3.2.

### Motion vector coding

The motion vector coding is the similar to the motion vector coding in single-layer HEVC. Differences for constructing the motion vector candidate lists are described in section 2.3.3.1. The length of the candidate list is increased by one and the coding of the motion vector predictor index is accommodated. This is achieved by increasing the maximum length of the longest possible truncated unary code by one.

### Intra prediction mode coding

A modified most probable intra prediction mode list is used at the enhancement layer.

The list is modified as follows: If the base layer block at the co-located position corresponding to the current enhancement block's top-left sample position is intra coded, and the base layer's intra prediction mode for this block represents an angular intra prediction mode, and the derived left intra prediction mode does not represent an angular intra prediction mode, the intra prediction mode derived from the left enhancement layer block is replaced with the base layer intra prediction mode. Otherwise, if the intra prediction mode derived from the above enhancement layer block does not represent an angular intra prediction mode, it is replaced with the base layer intra prediction mode.

The above described modification is inserted between the steps 4 and 5 of the “Derivation process for luma intra prediction mode” specified in subclause 8.1.4 of the HEVC draft. With this modified list, the intra prediction coding is done as specified in the HEVC draft.

### SAO parameter coding

Parameters for the SAO filter in the enhancement layer are coded in exactly the same manner as specified in HEVC.

## Entropy coding of residual / transform coefficients

Compared to HEVC, only the coding of transformation coefficients in the enhancement layer is modified.

Vertical and horizontal scan patterns are introduced also for 16x16 and 32x32 transformation blocks. These new scans follow a simple scheme for subdividing coefficient positions into subgroups: Starting from DC position, each 16 consecutive positions in scan order form a subgroup, which leads to subgroups of the size 16x1 for horizontal scans, and 1x16 for vertical scans, respectively. Thus, in the case of 16x16 TUs each row represents a subgroup in horizontal scan, and each column represents a subgroup in vertical scan, respectively. This concept is depicted in Figure 11.

Figure 11: Vertical and horizontal scan for 16x16 transformation units.

The newly introduced scan patterns can be only selected for luminance signal and for transform units with a last significant coefficient position outside the first 4x4 sub-block. In those cases, the actual selected scan pattern is chosen during the encoding process to be either diagonal, vertical, or horizontal, and it is signaled inside the bitstream. The encoder decision is done by estimating the cost for each scan, by using a scan dependent rate distortion optimized quantization, and selecting that scan pattern with minimum cost. The context model derivation for coding significant coefficient subgroups and the last significant position is slightly modified: A different set of context models is used for those TUs with a signaled scan pattern. Furthermore, in those cases, the coding of the vertical last significant position depends on the horizontal last position. If the horizontal last position exceeds three, a separate set of context models is used for coding the vertical position.

For signaling the selected scan pattern two context models are introduced.

First, it is signaled whether diagonal scan is selected, or not. If not, it is signaled whether the more likely scan pattern is selected. If the last horizontal position exceeds the vertical position, horizontal scan is set as more likely, if not, vertical scan is set to be more likely.

The context model initialization is updated for the encoding/decoding of the enhancement layer. Thereby the basic algorithm, mapping an initial value and a QP to an initial context model state, is not changed. The enhancement layer uses a new set of initialization values.

## High-layer syntax

The contribution does not contain any new high level syntax concepts compared to HEVC.

For HEVC base layers it is possible to use a single bitstream file for base and enhancement layers, as well as separate bitstream files. For H.264/AVC base layers, the enhancement layer is always coded in a separate bitstream file.

For distinguishing base and enhancement layers in a single HEVC scalable bitstream the reserved flags in the first byte of the NAL unit header have been used to code a layer identifier (see section 3.1.1). These fields were intended for that purpose at the time of base layer bitstream generation. More advanced hooks for extensions have been added to the HEVC draft and software later, but could not be used with the given base layer files.

The changes for layer identification are expected to be modified to conform to the HEVC base specification in a later standardization phase.

Enhancement layers contain their own SPS, PPS (and APSs). The SPS has been extended to allow switching on/off the different enhancement layer tools.

## Encoder control

For coding the enhancement layers, exactly the same prediction structure as for the provided base layers has been used. The intra pictures are inserted at the same positions as the intra pictures in the base layer. In the enhancement layer, the same type of random access is provided as in the base layer.

The encoder decision in the enhancement layer is based on the same Lagrange coder control technique as implemented in the HEVC reference software. The motion search, mode selection, and quantization are based on the same cost functions.

A minor change has been made to the derivation of the Lagrange parameters. The Lagrange parameter for the enhancement layer does not only depend on the enhancement layer quantization parameter, but also on the base layer quantization parameter.

For hitting the target rates, the quantization is change once in coding order (if it is changed at all).

# Syntax and semantics description

In subsection 3.1, the syntax modifications and additions are described relative to the HEVC draft 6, which has been used as basis for the developed software and the provided HEVC anchor bitstreams.

Syntax diagrams that are not listed in this section have not been modified. In the listed syntax diagrams, the modifications relative to the HEVC draft 6 are highlighted in red; non-highlighted parts are the same as in the HEVC draft 6.

The semantics for modified or added syntax elements are described in subsection 3.2

## Syntax

### NAL unit syntax

| nal_unit( NumBytesInNALunit ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_ref_flag | u(1) |
| nal_unit_type | u(6) |
| NumBytesInRBSP = 0 |  |
| temporal_id | u(3) |
| layer_id | u(4) |
| reserved_one_bit | u(1) |
| reserved_one_5bits | u(5) |
| for( i = nalUnitHeaderBytes; i < NumBytesInNALunit; i++ ) { |  |
| if( i + 2 < NumBytesInNALunit && next_bits( 24 ) = = 0x000003 ) { |  |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| i += 2 |  |
| emulation_prevention_three_byte /* equal to 0x03 */ | f(8) |
| } else |  |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| } |  |
| } |  |

### Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_idc | u(8) |
| reserved_zero_8bits /* equal to 0 */ | u(8) |
| level_idc | u(8) |
| seq_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| max_temporal_layers_minus1 | u(3) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| pic_cropping_flag | u(1) |
| if( pic_cropping_flag ) { |  |
| pic_crop_left_offset | ue(v) |
| pic_crop_right_offset | ue(v) |
| pic_crop_top_offset | ue(v) |
| pic_crop_bottom_offset | ue(v) |
| } |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| pcm_enabled_flag | u(1) |
| if( pcm_enabled_flag ) { |  |
| pcm_bit_depth_luma_minus1 | u(4) |
| pcm_bit_depth_chroma_minus1 | u(4) |
| } |  |
| qpprime_y_zero_transquant_bypass_flag | u(1) |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| for( i = 0; i <= max_temporal_layers_minus1; i++ ) { |  |
| max_dec_pic_buffering[ i ] | ue(v) |
| num_reorder_pics[ i ] | ue(v) |
| max_latency_increase[ i ] | ue(v) |
| } |  |
| restricted_ref_pic_lists_flag | u(1) |
| if( restricted_ref_pic_lists_flag ) |  |
| lists_modification_present_flag | u(1) |
| log2_min_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_coding_block_size | ue(v) |
| log2_min_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_transform_block_size | ue(v) |
| if( pcm_enabled_flag ) { |  |
| log2_min_pcm_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_pcm_coding_block_size | ue(v) |
| } |  |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| scaling_list_enable_flag |  |
| chroma_pred_from_luma_enabled_flag | u(1) |
| deblocking_filter_in_aps_enabled_flag | u(1) |
| seq_loop_filter_across_slices_enabled_flag | u(1) |
| asymmetric_motion_partitions_enabled_flag | u(1) |
| non_square_quadtree_enabled_flag | u(1) |
| sample_adaptive_offset_enabled_flag | u(1) |
| adaptive_loop_filter_enabled_flag | u(1) |
| if( adaptive_loop_filter_enabled_flag ) |  |
| alf_coef_in_slice_flag | u(1) |
| if( pcm_enabled_flag ) |  |
| pcm_loop_filter_disable_flag | u(1) |
| temporal_id_nesting_flag | u(1) |
| if( log2_min_coding_block_size_minus3 = = 0 ) |  |
| inter_4x4_enabled_flag | u(1) |
| num_short_term_ref_pic_sets | ue(v) |
| for( i = 0; i < num_short_term_ref_pic_sets; i++) |  |
| short_term_ref_pic_set( i ) |  |
| long_term_ref_pics_present_flag | u(1) |
| tiles_or_entropy_coding_sync_idc | u(2) |
| if( tiles_or_entropy_coding_sync_idc = = 1 ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_spacing_flag | u(1) |
| if( !uniform_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| column_width[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height[ i ] | ue(v) |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| } |  |
| vui_parameters_present_flag | u(1) |
| if( vui_parameters_present_flag ) |  |
| vui_parameters( ) |  |
| if( layer_id > 0 ) { |  |
| sps_scalable_extension_flag | u(1) |
| if( sps_scalable_extension_flag ) { |  |
| sps_enable_bl_motion_flag | u(1) |
| sps_enable_inter_base_flag | u(1) |
| sps_enable_intra_base_flag | u(1) |
| sps_enable_bl_reco_flag | u(1) |
| sps_enable_bl_resi_flag | u(1) |
| sps_enable_bl_ipred_flage | u(1) |
| sps_enable_bl_reco_copy_flag | u(1) |
| sps_enable_bl_intra_comb_flag | u(1) |
| } |  |
| } |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

### Coding unit syntax

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |  |
| --- | --- | --- |
| CurrCbAddrTS = MinCbAddrZS[ x0 >> Log2MinCbSize ][ y0 >> Log2MinCbSize ] |  |  |
| if( slice_type != I ) |  |  |
| skip_flag[ x0 ][ y0 ] | ae(v) |  |
| if( skip_flag[ x0 ][ y0 ] ) { |  |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |  |
| if( layer_id > 0 ) |  |  |
| bl_type_coding( x0, y0, MODE_INTER ) |  |  |
| } |  |  |
| else if( layer_id > 0 && ( ( slice_type = = I && sps_enable_intra_base_flag = = 1) ¦ ¦ ( slice_type != I && sps_enable_inter_base_flag = = 1) ) ) { |  |  |
| bl_copy_params_flag[ x0 ][ y0 ] | ae(v) |  |
| if( bl_copy_params_flag[ x0 ][ y0 ] ) { |  |  |
| if( slice_type = = I ) { |  |  |
| bl_type_coding( x0, y0, MODE_INTRA_BASE ) |  |  |
| transform_tree( x0, y0, x0, y0, log2CbSize, log2CbSize, log2CbSize, 0, 0 ) |  |  |
| } |  |  |
| else { |  |  |
| bl_type_coding( x0, y0, MODE_INTER_BASE ) |  |  |
| basecopy_trafo_tree( x0, y0, x0, y0, log2CbSize, log2CbSize, log2CbSize, 0, 0 ) |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| else { |  |  |
| if( slice_type != I ¦ ¦ log2CbSize = = Log2MinCbSize ) |  |  |
| { |  |  |
| if( slice_type != I ) |  |  |
| pred_mode_flag | ae(v) |  |
| if( PredMode != MODE_INTRA ¦ ¦ log2CbSize = = Log2MinCbSize ) |  |  |
| part_mode | ae(v) |  |
| } |  |  |
| x1 = x0 + ( ( 1 << log2CbSize ) >> 1 ) |  |  |
| y1 = y0 + ( ( 1 << log2CbSize ) >> 1 ) |  |  |
| x2 = x1 − ( ( 1 << log2CbSize ) >> 2 ) |  |  |
| y2 = y1 − ( ( 1 << log2CbSize ) >> 2 ) |  |  |
| x3 = x1 + ( ( 1 << log2CbSize ) >> 2 ) |  |  |
| y3 = y1 + ( ( 1 << log2CbSize ) >> 2 ) |  |  |
| if( PartMode = = PART_2Nx2N ) |  |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |  |
| else if( PartMode = = PART_2NxN ) { |  |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |  |
| prediction_unit( x0, y1 , log2CbSize ) |  |  |
| } else if( PartMode = = PART_Nx2N ) { |  |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |  |
| prediction_unit( x1, y0 , log2CbSize ) |  |  |
| } else if( PartMode = = PART_2NxnU ) { |  |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |  |
| prediction_unit( x0, y2 , log2CbSize ) |  |  |
| } else if( PartMode = = PART_2NxnD ) { |  |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |  |
| prediction_unit( x0, y3 , log2CbSize ) |  |  |
| } else if( PartMode = = PART_nLx2N ) { |  |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |  |
| prediction_unit( x2, y0 , log2CbSize ) |  |  |
| } else if( PartMode = = PART_nRx2N ) { |  |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |  |
| prediction_unit( x3, y0 , log2CbSize ) |  |  |
| } else { /* PART_NxN */ |  |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |  |
| prediction_unit( x1, y0 , log2CbSize ) |  |  |
| prediction_unit( x0, y1 , log2CbSize ) |  |  |
| prediction_unit( x1, y1 , log2CbSize ) |  |  |
| } |  |  |
| if( layer_id > 0 ) |  |  |
| bl_type_coding( x0, y0, PredMode ) |  |  |
| if( !pcm_flag ) |  |  |
| transform_tree( x0, y0, x0, y0, log2CbSize, log2CbSize, log2CbSize, 0, 0 ) |  |  |
| } |  |  |
| } |  |  |

### Base layer type syntax

| bl_type_coding( x0, y0, bl_type_mode_control ) { | Descriptor |
| --- | --- |
| if( bl_type_mode_control = = MODE_INTRA ) { |  |
| if( sps_enable_bl_reco_flag = = 1 ) |  |
| bl_type_reco_flag | ae(v) |
| if( bl_type_reco_flag = = 0 ) |  |
| BLType[ x0 ][ y0 ] = BL_INTRA_STD |  |
| else { |  |
| if( sps_enable_bl_intra_comb_flag = = 1 ) |  |
| bl_type_intra_comb_flag | ae(v) |
| if( bl_type_intra_comb_flag = = 1 ) |  |
| BLType[ x0 ][ y0 ] = BL_INTRA_COMB |  |
| else |  |
| BLType[ x0 ][ y0 ] = BL_INTRA_RECO |  |
| } |  |
| } |  |
| else if( bl_type_mode_control = = MODE_INTRA_BASE ) { |  |
| if( sps_enable_bl_reco_copy_flag = = 1 ) |  |
| bl_type_reco_cpy_flag | ae(v) |
| if( bl_type_reco_cpy_flag = = 1 ) |  |
| BLType[ x0 ][ y0 ] = BL_RECO_CPY |  |
| else |  |
| BLType[ x0 ][ y0 ] = BL_RECO_CPY_FILT |  |
| } |  |
| else { |  |
| if( sps_enable_bl_resi_flag = = 1 ¦ ¦ bl_type_mode_control = = MODE_INTER_BASE ) |  |
| bl_type_resi_flag | ae(v) |
| BLTypeFound = 0 |  |
| if( bl_type_resi_flag = = 0 ) |  |
| BLType[ x0 ][ y0 ] = BL_INTER_STD |  |
| else { |  |
| if( sps_enable_bl_reco_flag = = 1 ¦ ¦ bl_type_mode_control = = MODE_INTER_BASE ) { |  |
| bl_type_reco_flag | ae(v) |
| if( bl_type_reco_flag = = 1 ) { |  |
| BLType[ x0 ][ y0 ] = BL_INTER_RECO |  |
| bl_type_found = true |  |
| } |  |
| } |  |
| if( BLTypeFound = = 0 && ( sps_enable_bl_reco_copy_flag = = 1 ¦ ¦ bl_type_mode_control = = MODE_INTER_BASE ){ |  |
| bl_type_reco_copy_flag | ae(v) |
| if( bl_type_reco_copy_flag = = 1 ¦ ¦ bl_type_mode_control = = MODE_INTER_BASE ) { |  |
| BLType[ x0 ][ y0 ] = BL_RECO_CPY |  |
| BLTypeFound = 1 |  |
| } |  |
| } |  |
| if ( BLTypeFound = = 0 && sps_enable_bl_reco_copy_flag = = 1 ){ |  |
| bl_type_reco_copy_filt_flag | ae(v) |
| if( bl_type_reco_copy_filt_flag = = 1 ) { |  |
| BLType[ x0 ][ y0 ] = BL_RECO_CPY_FILT |  |
| BLTypeFound = true |  |
| } |  |
| if ( BLTypeFound = = false ) |  |
| BLType[ x0 ][ y0 ] = BL_INTER_RESI |  |
| } |  |
| } |  |
| } |  |

### Base copy transform tree syntax

| basecopy_trafo_tree( x0, y0, xC, yC, log2CbSize, log2TrafoW, log2TrafoH, trafoDepth, blkIdx ) { | Descriptor |
| --- | --- |
| if( BaseCuDepth[ x0 ] [ y0 ] > trafoDepth ) { |  |
| x1 = x0 + ( ( 1 << log2TrafoWidth ) >> 1 ) |  |
| y1 = y0 |  |
| x2 = x0 |  |
| y2 = y0 + ( ( 1 << log2TrafoHeight ) >> 1 ) |  |
| x3 = x1 |  |
| y3 = y2 |  |
| basecopy_trafo_tree ( x0, y0, x0, y0, log2CbSize, log2TrafoW − 1, log2TrafoH − 1, trafoDepth + 1, 0 ) |  |
| basecopy_trafo_tree ( x1, y1, x0, y0, log2CbSize, log2TrafoW − 1, log2TrafoH − 1, trafoDepth + 1, 1 ) |  |
| basecopy_trafo_tree ( x2, y2, x0, y0, log2CbSize, log2TrafoW − 1, log2TrafoH − 1, trafoDepth + 1, 2 ) |  |
| basecopy_trafo_tree ( x3, y3, x0, y0, log2CbSize, log2TrafoW − 1, log2TrafoH − 1, trafoDepth + 1, 3 ) |  |
| } |  |
| transform_tree( x0, y0, x0, y0, log2CbSize, log2TrafoW, log2TrafoH, trafoDepth, 0 ) |  |
| } |  |

### Residual coding syntax

| residual_coding( x0, y0, log2TrafoWidth, log2TrafoHeight, scanIdx, cIdx ) { | Descriptor |
| --- | --- |
| last_significant_coeff_x_prefix | ae(v) |
| last_significant_coeff_y_prefix | ae(v) |
| if( last_significant_coeff_x_prefix > 3 ) |  |
| last_significant_coeff_x_suffix | ae(v) |
| if( last_significant_coeff_y_prefix > 3 ) |  |
| last_significant_coeff_y_suffix | ae(v) |
| if( layer_id > 0 && log2TrafoWidth > 3 && cIdx = = 0 && ( LastSignificantCoeffX >= 4 ¦ ¦ LastSignificantCoeffY >= 4 ) ) { |  |
| adap_scan_idx | ae(v) |
| scanIdx = AdaptiveScanType[adap_scan_idx][ LastSignificantCoeffX][ LastSignificantCoeffY] |  |
| } |  |
| numCoeff = 0 |  |
| do { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ numCoeff ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ numCoeff ][ 1 ] |  |
| numCoeff++ |  |
| } while( ( xC != LastSignificantCoeffX ) ¦ ¦ ( yC != LastSignificantCoeffY ) ) |  |
| numLastSubset = (numCoeff − 1) >> 4 |  |
| for( i = numLastSubset; i >= 0; i− − ) { |  |
| offset = i << 4 |  |
| if( scanIdx = = 1 && ( ( log2TrafoWidth = = 3 && log2TrafoHeight = = 3 ) ¦ ¦ ( log2TrafoWidth = = 4 && log2TrafoHeight = = 4 ) ) { |  |
| xCG = 0 |  |
| yCG = i |  |
| } else if( scanIdx = = 2 && ( ( log2TrafoWidth = = 3 && log2TrafoHeight = = 3 ) ¦ ¦ ( log2TrafoWidth = = 4 && log2TrafoHeight = = 4 ) ) { |  |
| xCG = i |  |
| yCG = 0 |  |
| } else if( scanIdx != 0 && log2TrafoWidth = = 5 && log2TrafoHeight = = 5 ) { |  |
| xCG = i & 1 |  |
| yCG = i >> 1 |  |
| } else { |  |
| xCG = ScanOrder[ log2TrafoWidth − 2 ][ log2TrafoHeight − 2 ][ scanIdx ][ i ][ 0 ] |  |
| yCG = ScanOrder[ log2TrafoWidth − 2 ][ log2TrafoHeight − 2 ][ scanIdx ][ i ][ 1 ] |  |
| } |  |
| implicitNonZeroCoeff = 0 |  |
| if( (i < numLastSubset) && (i > 0) ) { |  |
| significant_coeff_group_flag[ xCG ][ yCG ] | ae(v) |
| implicitNonZeroCoeff = 1 |  |
| } |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( (n + offset) < (numCoeff − 1) && significant_coeff_group_flag[ xCG ][ yCG ] && ( n > 0 ¦ ¦ implicitNonZeroCoeff = = 0 ) ) { |  |
| significant_coeff_flag[ xC ][ yC ] | ae(v) |
| if( significant_coeff_flag[ xC ][ yC ] = = 1 ) |  |
| implicitNonZeroCoeff = 0 |  |
| } |  |
| } |  |
| firstNZPosInCG = 16 |  |
| lastNZPosInCG = −1 |  |
| numSigCoeff = 0 |  |
| firstGreater1CoeffIdx = −1 |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| if( numSigCoeff < 8 ) { |  |
| coeff_abs_level_greater1_flag[ n ] | ae(v) |
| numSigCoeff++ |  |
| if( coeff_abs_level_greater1_flag[ n ] && firstGreater1CoeffIdx = = −1 ) |  |
| firstGreater1CoeffIdx = n |  |
| } |  |
| if( lastNZPosInCG = = −1) |  |
| lastNZPosInCG = n |  |
| firstNZPosInCG = n |  |
| } |  |
| } |  |
| signHidden = ( lastNZPosInCG – firstNZPosInCG >= sign_hiding_threshold) ? 1 : 0 |  |
| if( firstGreater1CoeffIdx != −1 ) |  |
| coeff_abs_level_greater2_flag[ firstGreater1CoeffIdx] | ae(v) |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] && (!sign_data_hiding_flag ¦ ¦ !signHidden ¦ ¦ n != firstNZPosInCG) ) |  |
| coeff_sign_flag[ n ] | ae(v) |
| } |  |
| numSigCoeff = 0 |  |
| sumAbs = 0 |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| baseLevel = 1 + coeff_abs_level_greater1_flag[ n ] + coeff_abs_level_greater2_flag[ n ] |  |
| if( baseLevel = = ( ( numSigCoeff < 8 ) ? ( (n = = firstGreater1CoeffIdx) ? 3 : 2 ) : 1 ) ) |  |
| coeff_abs_level_remaining[ n ] | ae(v) |
| transCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( coeff_abs_level_remaining[ n ] + baseLevel ) * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| if( sign_data_hiding_flag && signHidden ) { |  |
| sumAbs += ( coeff_abs_level_remaining[ n ] + baseLevel ) |  |
| if( n = = firstNZPosInCG && (sumAbs%2 = = 1) ) |  |
| transCoeffLevel[x0][y0][cIdx][xC][yC] = − transCoeffLevel[x0][y0][cIdx][xC][yC] |  |
| } |  |
| numSigCoeff++ |  |
| } else |  |
| transCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = 0 |  |
| } |  |
| } |  |
| } |  |

## Semantics

### NAL unit semantics

layer_id specified a layer identifier for the NAL unit. layer_id equal to 0 specifies that the NAL unit is a NAL unit of the backwards compatible HEVC base layer. layer_id greater than 1 specifies that the NAL unit is a NAL unit of an enhancement layer with a layer identifier equal to layer_id.

It is a requirement of bitstream conformance that coded slice NAL units with a particular value of layer_id shall not refer to parameter sets with a different value of layer_id. SEI messages with a particular value of layer_id are associated with the enhancement layer with the same value of layer_id.

reserved_one_bit shall be equal to 1. Other values of reserved_one_bit may be specified in the future by ITU-T | ISO/IEC. Decoders shall ignore (i.e. remove from the bitstream and discard) NAL units with values of reserved_one_bit not equal to 1.

### Sequence parameter set RBSP semantics

sps_scalable_extension_flag equal to 1 specifies that the syntax elements specifying the used inter-layer prediction techniques are present in the sequence parameter set RBSP syntax structure. sps_scalable_extension_flag equal to 0 specifies that the syntax elements specifying the used inter-layer prediction techniques are not present in the sequence parameter set RBSP syntax structure.

When sps_scalable_extension_flag is not present, it shall be inferred to be equal to 0.

sps_enable_bl_motion_flag specifies whether the enhancement layer modifications for the derivation of the merge candidate lists and the motion vector predictor candidate lists are used. sps_enable_bl_motion_flag equal to 1 specifies that the modifications are used; sps_enable_bl_motion_flag equal to 0 specifies that the modifications are not used.

When sps_enable_bl_motion_flag is not present, it shall be inferred to be equal to 0.

sps_enable_inter_base_flag equal to 1 specifies that the syntax element bl_copy_params_flag is present in the coding unit syntax of P and B slices. sps_enable_inter_base_flag equal to 0 specifies that the syntax element bl_copy_params_flag is not present in the coding unit syntax of P and B slices.

When sps_enable_inter_base_flag is not present, it shall be inferred to be equal to 0.

sps_enable_intra_base_flag equal to 1 specifies that the syntax element bl_copy_params_flag is present in the coding unit syntax of I slices. sps_enable_intra_base_flag equal to 0 specifies that the syntax element bl_copy_params_flag is not present in the coding unit syntax of I slices.

When sps_enable_intra_base_flag is not present, it shall be inferred to be equal to 0.

sps_enable_bl_reco_flag equal to 1 specifies that the syntax element bl_type_reco_flag is present in the coding unit syntax for coding units with bl_copy_params_flag equal to 0 in P and B slices. sps_enable_bl_reco_flag equal to 0 specifies that the syntax element bl_type_reco_flag is not present in the coding unit syntax for coding units with bl_copy_params_flag equal to 0 in P and B slices.

When sps_enable_bl_reco_flag is not present, it shall be inferred to be equal to 0.

sps_enable_bl_resi_flag equal to 1 specifies that the syntax element bl_type_resi_flag is present in the coding unit syntax for coding units with bl_copy_params_flag equal to 0 in P and B slices. sps_enable_bl_resi_flag equal to 0 specifies that the syntax element bl_type_resi_flag is not present in the coding unit syntax for coding units with bl_copy_params_flag equal to 0 in P and B slices.

When sps_enable_bl_resi_flag is not present, it shall be inferred to be equal to 0.

sps_enable_bl_ipred_flag specifies whether the enhancement layer modifications for the derivation of intra prediction modes are used. sps_enable_bl_ipred_flag equal to 1 specifies that the modifications are used; sps_enable_bl_ipred_flag equal to 0 specifies that the modifications are not used.

When sps_enable_bl_ipred_flag is not present, it shall be inferred to be equal to 0.

sps_enable_bl_reco_copy_flag equal to 1 specifies that the syntax elements bl_type_reco_copy_flag and bl_type_reco_copy_filt_flag are present in the coding unit syntax. sps_enable_bl_reco_copy_flag equal to 0 specifies that the syntax elements bl_type_reco_copy_flag and bl_type_reco_copy_filt_flag are not present in the coding unit syntax.

When sps_enable_bl_reco_copy_flag is not present, it shall be inferred to be equal to 0.

sps_enable_bl_intra_comb_flag equal to 1 specifies that the syntax element bl_type_intra_comb_flag is present in the coding unit syntax of intra coding units. sps_enable_bl_intra_comb_flag equal to 0 specifies that the syntax element bl_type_intra_comb_flag is not present in the coding unit syntax of inra coding units.

When sps_enable_bl_intra_comb_flag is not present, it shall be inferred to be equal to 0.

### Coding unit semantics

bl_copy_params_flag equal to 1 specifies that the partitioning and motion parameters for coding an enhancement layer CU are inferred from the syntax elements of the base layer. bl_copy_params_flag equal to 0 specifies that the partitioning and motion parameters are not inferred from the syntax elements of the base layer.

The variable BLType is derived based on the syntax elements in the syntax structure bl_type_coding( ).

### Base layer type syntax

bl_type_reco_flag, bl_type_intra_comb_flag, bl_type_reco_cpy_flag, bl_type_resi_flag, and bl_type_reco_cpy_filt_flag specify the intra or inter reconstruction mode of the coding unit. These flags are used to derive the variable BLType[ x0 ][ y0 ], which can have the values BL_INTRA_STD, BL_RECO_CPY, BL_RECO_CPY_FILT, BL_INTRA_RECO, BL_INTRA_COMB, BL_INTER_STD, BL_INTER_RESI, BL_INTER_RECO.

BLType[ x0 ][ y0 ] equal to BL_INTRA_STD specifies that the coding unit is predicted by spatial intra prediction. This mode is the same as the spatial intra prediction in single-layer HEVC.

BLType[ x0 ][ y0 ] equal to BL_RECO_CPY specifies that the prediction signal for the coding unit is generated by upsampling, or copying in case of quality scalability, the reconstructed base layer signal for the co-located area in the base layer.

BLType[ x0 ][ y0 ] equal to BL_RECO_CPY_FILT specifies that the prediction signal for the coding unit is generated by upsampling, or copying in case of quality scalability, and additionally filtering the reconstructed base layer signal for the co-located area in the base layer.

BLType[ x0 ][ y0 ] equal to BL_INTRA_RECO specifies that the prediction signal for the coding unit is generated as follows. A first prediction signal is generated by upsampling, or copying in case of quality scalability, the reconstructed base layer signal for the co-located area in the base layer. A second prediction signal is generated by spatial intra prediction using a difference signal of the neighboring samples of already reconstructed blocks. The difference signal for the neighboring blocks is obtained by subtracting the corresponding upsampled/copied base layer signal from the reconstructed enhancement layer signal (before deblocking and loop filtering). Beside the usage of a difference signal for intra prediction, the same intra prediction as in single-layer HEVC is used. The two prediction signals are added together forming the final prediction signal for the coding unit. The intra prediction modes are transmitted as side information as in single-layer HEVC.

BLType[ x0 ][ y0 ] equal to BL_INTRA_COMB specifies that the prediction signal for the coding unit is generated as follows. A first prediction signal is generated by upsampling, or copying in case of quality scalability, the reconstructed base layer signal for the co-located area in the base layer. A second prediction signal is generated by conventional spatial intra prediction using the reconstructed neighboring samples of already reconstructed blocks. Both prediction signals are transformed, the transform coefficients are scaled using defined scaling matrices, and thereafter added in the transform domain. The final prediction signal is obtained by inverse transforming the obtained transform coefficients. The intra prediction modes are transmitted as side information as in single-layer HEVC.

BLType[ x0 ][ y0 ] equal to BL_INTER_STD specifies that the coding unit is predicted by motion-compensated prediction. This mode is the same as the inter prediction mode in single-layer HEVC.

BLType[ x0 ][ y0 ] equal to BL_INTER_RESI specifies that the coding unit is predicted by motion-compensated prediction as in single-layer HEVC. For forming the final prediction signal for the coding unit, a second prediction signal is generated by upsampling, or copying in case of quality scalability, the reconstructed base layer residual signal for the co-located area in the base layer. The final prediction signal for the coding unit is obtained by adding the upsampled/copied base layer residual signal to the motion-compensated prediction signal.

BLType[ x0 ][ y0 ] equal to BL_INTER_RECO specifies that the prediction signal for the coding unit is obtained as follows. The first part of the prediction signal is obtained by upsampling, or copying in case of quality scalability, the reconstructed base layer signal for the co-located area in the base layer. The second part is obtained by motion-compensated prediction using a difference signal for the corresponding reference picture. The difference signal for the used reference picture is obtained by subtracting the upsampled base layer from the reconstructed enhancement layer signal after deblocking. Both prediction signals are added to form the final prediction signal for the coding unit.

### Base copy transform tree semantics

The partitioning into transform units is coded in the same way as for single-layer HEVC. If the partitioning of a coding unit into smaller coding units is inferred from the base layer, the inferred partitioning is considered in the same way as it was coded.

### Residual coding semantics

adap_scan_idx indicates the scan pattern used for parsing transform coefficient levels. It is used only for luma 16x16 and 32x32 transform sizes. In HEVC HM6.0 the scan pattern for 16x16 and 32x32 was fixed to diagonal. It is now extended to include horizontal and vertical scans.

Depending on the horizontal and vertical coordinates of the last significant position, a most probable scan is derived and is used to map the value of adap_scan_idx to diagonal, horizontal or vertical scans.

## Context modeling

### Context modeling for flags indicating the used base layer prediction mode

For each of the newly introduced flags bl_copy_params_flag, bl_type_reco_flag, bl_type_intra_comb_flag, bl_type_reco_cpy_flag, bl_type_resi_flag, and bl_type_reco_cpy_filt_flag a separate context model is used.

### Context modeling the syntax element adap_scan_idx

Two context models are used for the newly introduced syntax element adap_scan_idx, one for each of the two bins.

### Context derivation for last significant position

Inputs to this process are log2TrafoWidth, layer_id, cIdx.

Output of the process is adaScanLastCtxOffset.

The variable adaScanLastCtxOffset is initialized to 0.

When log2TrafoWidth is greater than 3 and layer_id is greater than 0 and cIdx indicates luma, then adaScanLastCtxOffset is set to 2. The offset for context used to indicate last position in X direction is set to adaScanLastCtxOffset. The offset for context used to indicate last position in Y direction is set to adaScanLastCtxOffset if LastSignificantCoeffX is less than or equal to 3 and adaScanLastCtxOffset + 1 if LastSignificantCoeffX is greater than 3.

The context derivation for chroma is same as in single-layer HEVC.

### Context derivation for sigGroupFlag

Inputs to this process are log2TrafoWidth, layer_id, cIdx, LastSignificantCoeffX and LastSignificantCoeffY.

Output of the process is adapScanGrpCtxOffset.

The variable adapScanGrpCtxOffset is initialized to 0.

When log2TrafoWidth is greater than 3 and layer_id is greater than 0 and cIdx indicates luma and last position is greater than 3 in either horizontal or vertical direction, then adapScanGrpCtxOffset is set to 2.

Context derivation for chroma is same as in single-layer HEVC.

# Compression performance discussion

## Category 1 (HEVC base layer)

### Spatial scalability

Detailed results are contained in the accompanying Excel document.

On average, bit rate savings for the enhancement layer of 31.5% and 48.9% have been obtained for spatial scalability ratios of 2.0 and 1.5, respectively. Overall bit rate savings relative to simulcast of 21.8% and 29.9% for the spatial scalability ratios of 2.0 and 1.5, respectively, have been measured.

### Intra-only spatial scalability

Detailed results are contained in the accompanying Excel document.

On average, bit rate savings for the enhancement layer of 35.8% and 54.5% have been obtained for spatial scalability ratios of 2.0 and 1.5, respectively. Overall bit rate savings relative to simulcast of 24.9% and 32.6% for the spatial scalability ratios of 2.0 and 1.5, respectively, have been measured.

### SNR scalability

Detailed results are contained in the accompanying Excel document.

On average, bit rate savings for the enhancement layer of 39.3% have been obtained. Overall bit rate savings relative to simulcast of 26.4% have been measured.

### Overall

Results for spatial scalability with random access, intra-only spatial scalability, and SNR scalability have been provided. The provided results indicate that substantial bit rate savings relative to simulcasting different resolutions or quality levels can be obtained with the proposed scalable extension of HEVC.

## Category 2 (AVC base layer)

### Spatial scalability

Detailed results are contained in the accompanying Excel document.

When comparing with HEVC enhancement layers, on average, bit rate savings for the enhancement layer of 30.0% and 44.9% have been obtained for spatial scalability ratios of 2.0 and 1.5, respectively. Overall bit rate savings relative to simulcast with an HEVC enhancement layer of 20.1% and 26.1% for the spatial scalability ratios of 2.0 and 1.5, respectively, have been measured.

When comparing with AVC enhancement layers, on average, bit rate savings for the enhancement layer of 53.8% and 66.3% have been obtained for spatial scalability ratios of 2.0 and 1.5, respectively. Overall bit rate savings relative to simulcast with an AVC enhancement layer of 40.5% and 46.5% for the spatial scalability ratios of 2.0 and 1.5, respectively, have been measured.

### Overall

Results for spatial scalability with random access and different resolution ratios have been provided. The provided results indicate that substantial bit rate savings relative to simulcasting different resolutions or quality levels can be obtained with the proposed scalable extension of HEVC.

# Complexity analysis

## Encoding time and measurement methodology

The encodings have been run on a non-homogenous cluster (see section Error: Reference source not found).

The times have been measured inside the encoder software using the same timer functionality as in the HEVC HM reference software. The times are complete software run times including all file input/output and decoding of the base layer.

Generation of MD5 checksum SEI messages (one per layer as in HM 6.1) has been enabled in the bitstream generation.

The measured encoding times are summarized in the accompanying Excel document.

## Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM

Decodings have been run on a homogenous section of the cluster (see section Error: Reference source not found).

The times have been measured inside the decoder software using the same timer functionality as in the HEVC HM reference software. The times are complete software run times including all file input/output and decoding of the base layer.

Verification of MD5 checksum SEI messages (one per layer as in HM 6.2) has been enabled for decoding the bitstreams.

The measured decoding times are summarized in the accompanying Excel document.

## Description of computing platform used to determine encoding and decoding times reported in sections 5.1 and 5.2

The encodings have been run on a non-homogenous Linux (Ubuntu 10.10) cluster using 64-bit binaries (gcc 4.4.4) on the following types of CPUs:

- Intel Xeon X5670  @ 2.93GHz

- Intel Xeon X5570  @ 2.93GHz

- Intel Xeon X5675  @ 3.07GHz

- Intel Xeon E5-2690 0 @ 2.90GHz

The nodes contain two to four CPUs with four to eight cores each. On all cluster nodes Turbo mode and Hyperthreading functionalities have been disabled to minimize the effects between jobs on the same node. Due to shared I/O channels such effects cannot be completely eliminated, but with the overall long run times of encodings, they should be rather minor. Larger differences are expected from the different CPU types.

All nodes use server grade hard disks of different sizes and from different manufacturers.

Decodings have been run on a homogenous section of the cluster using Intel XEON X5675 CPUs @ 3.07GHz.

## Expected memory usage of encoder

In addition to the functionality of an HEVC encoder, the proposed codec uses additional tools specific to scalable video coding as described in the sections above. Some of these tools require additional memory for different representations of the reference pictures, or for referencing the base layer. Due to the multi-loop structure of the codec, the prediction process of an enhancement layer bit stream can make use of the reconstructed base layer pictures. This requires a conventional (H.264/AVC or HEVC) base layer encoder (or decoder when using an already coded base layer) with its own specific memory requirements. In the following, these memory requirements are discussed in reference to a typical encoder implementation. The decoder specific memory requirements are described in the next section.

In spatial scalability the reconstructed base layer representation needs to be spatially upsampled for inter-layer intra prediction. A fast encoder implementation might choose to store the upsampled base layer picture or the area covered by the current CTB, although on-the-fly upsampling would be possible at the cost of increased processing power.

One of the proposed coding tools employs motion-compensated prediction using the difference signal between the enhancement layer reconstruction of a reference picture before applying SAO and ALF and the (potentially upsampled) base layer reconstruction of the same reference picture. Thus either the unfiltered enhancement layer picture or the difference signal (which might be preferable for encoder search) needs to be stored for each enhancement layer reference picture in the DPB, when this mode is enabled.

This additional picture buffer could be completely avoided by using the reconstructed reference picture after SAO (and potentially ALF).

Another scalability tool utilizes the upsampled residual signal of the current base layer for inter-layer inter prediction. Thus, one further picture buffer is required to store this signal. An encoder implementation may employ on-the-fly upsampling such that a picture buffer with base layer dimensions is sufficient.

For using syntax elements of the base layer, in a tightly coupled environment only minimal additional memory is required. For SNR scalability the syntax elements can be directly accessed at the co-location in the base layer. For spatial scalability the syntax elements need to be converted and scaled at CTU level to allow a prediction of the block partitioning.

When base and enhancement layer encoder are not tightly coupled an additional shared memory for accessing the base layer syntax elements might be necessary.

In summary, the memory requirements of the proposed codec consist of the following three parts:

- Single layer encoder/decoder memory (standard HEVC or AVC) for base layer

- Single layer encoder/decoder memory (standard HEVC) for the enhancement layer

- Memory for SVC-specific tools on the enhancement layer:

  - One extra picture buffer for each reference picture of the enhancement layer

  - One single picture buffer of base layer size for inter-layer inter prediction

## Expected memory usage of decoder

Due to multi-loop coding a complete base layer decoder (H.264/AVC or HEVC) with its memory requirement needs to be available also on the decoder side.

The additional memory requirement of the scalable enhancement layer decoder is similar to the encoder. The same new prediction signals are required as for encoding, but a real-world implementation would more likely choose on-the-fly filtering whenever possible, because in contrast to the encoder no search is performed.

However, the reference implementation chooses per-picture filtering for alignment with the encoder.

## Complexity characteristics of encoder motion estimation and partitioning selection in enhancement layer(s)

The actual motion estimation process of HM has not been changed for enhancement layer motion estimation.

The use of the BL_INTER_RESI and BL_INTER_RECO modes requires two additional searches:

In BL_INTER_RESI mode, a simple 2-tap filter is used to upsample the base layer residual signal and subtract it from the original to create the block that is matched with the reconstructed reference pictures. For sub-pel positions the original 8-tap HEVC interpolation filters are used. This search has similar complexity to the HM single-layer search.

In BL_INTER_RECO mode, 8-tap filters are used to upsample the reconstructed base layer signal and subtract from the original to create the block that is matched with a difference signal in the enhancement layer. For sub-pel positions, simple 2-tap interpolation filters are used. Due to the smaller filter length on the search area, this search has a lower complexity than the HM single-layer search.

## Complexity characteristics of decoder motion compensation in enhancement layer(s)

Motion compensation in SNR enhancement layers is expected to have a similar complexity compared to HEVC. For spatial enhancement layers, in some modes additional upsampling filters need to be used.

In SNR scalability, the BL_INTER_RESI mode requires an 8-tap filter for sub-pel positions which is similar to single layer HEVC. The BL_INTER_RECO mode has even less complexity because it uses only a 2-tap filter for sub-pel positions.

In spatial scalability, both BL_INTER_RESI and BL_INTER_RECO use one 8-tap and one 2-tap filter in the motion compensation process which is only a small increase compared to single layer HEVC.

## Complexity characteristics of encoder intra-frame prediction type and partitioning selection in enhancement layer(s)

Compared to HEVC, the provided encoder uses a set of additional prediction modes. The modes are at first approximately evaluated by analyzing the prediction algorithms.

The analysis disregards upsampling or prefiltering which might have to be applied in order to access base layer information.

The BL_RECO_COPY mode is the least complex mode. The mode simply accesses the reconstructed base layer signal as prediction signal, then a single RDOQ-based residual coding is evaluated. In terms of additional source code, the mode uses only already implemented functions.

The BL_INTRA_RECO mode is fairly comparable to the conventional intra prediction mode as used with HEVC in terms of complexity. Instead of using the reconstructed signal, the difference signal for spatial intra prediction is used. For RDOQ-based residual coding a modified “original signal” is used, which is the difference of the original signal and the base layer reconstructed signal, thus the intra prediction mode testing is reused for this mode. The final reconstruction signal is obtained after the prediction mode decision and RDOQ was finished. Any additional operations required can be assumed as negligible.

In terms of additional source code, this mode was completely integrated into the existing intra mode. Beside of switching pointers for the input and original signals no additional source code was added. Adding the 2nd prediction signal from the base layer is only done once, after the intra prediction direction was determined.

The BL_INTRA_COMB mode decision is a little more complex compared to the HEVC-intra mode decision. Because additional transforms with intermediate prediction signals are required. At the beginning of the prediction mode decision process a forward transform (or low pass filtering) is applied to the base-layer's reconstruction signal. This transformed signal can be stored and reused during the decision process. While looping over the prediction directions the intra prediction signal is obtained from the reconstructed enhancement layer signal. The predicted signal is then forward-transformed and coefficient-wise weighted with the stored transformed base layer signal. The obtained combined signal is inverse transformed to obtain the final prediction signal. This prediction signal is used for RDOQ-cost determination.

Since the RDOQ-complexity compared to the complexity of the transforms strongly depends on the block size as well as on the used QP, it is hard to compare the complexities of the HEVC intra mode against the proposed mode.

Given the case that transforming is the most complex part of the intra prediction mode decision than the maximum complexity of the provided mode as it is implemented is less than two times compared to the HEVC intra mode.

However, it is also possible to reduce the complexity to HEVC intra mode complexity by not inverse transforming the prediction signal and calculating the residual signal prior to quantization in the transform domain. In this case only the weighting of transform coefficients adds negligible complexity within the intra prediction loop.

In the currently implemented version this mode is integrated in the existing HEVC intra mode. The additional source code is limited to the weighting transform coefficients in the transform domain, otherwise only already available transform functions are used.

In summary, in terms of extra source code, the provided intra modes are just an extension of the original HEVC intra mode, where mostly existing methods are called. In terms of calculation time, the completely tested set of intra modes approximately adds additional complexity of about three times the original intra mode, but can be reduced to about two times the complexity.

## Complexity characteristics of decoder intra-frame prediction operation in enhancement layer(s)

Compared to HEVC the provided decoder uses a set of additional prediction modes to obtain the best intra coding mode in terms of RD-performance.

The modes are at first approximately evaluated by analyzing the prediction algorithms.

This analysis disregards upsampling or prefiltering which might have to be applied in order to access base layer information. In terms of calculation time, the upsampling processes is also ignored in the complexity approximation.

However, at decoder side, it is more likely to implement an on-the-fly upsampling, which is triggered only if required to obtain a prediction signal from the base layer. In this case, it would be more intuitive to add the upsampling cost to the mode.

Also at the decoder the BL_RECO_CPY mode does not required much additional code. The base layer is accessed at the reconstruction signal to obtain a prediction signal which is optionally improved with a residual signal. Altogether this mode is comparable to an Inter prediction mode with one directional prediction with zero displacement.

The BL_INTRA_RECO mode is slightly more complex than the conventional HEVC intra prediction mode, because of the additional difference signal required for prediction. In the current implementation, this signal is calculated right after reconstructing a block. So it is accessible without additional cost. A probably more advanced approach would calculated this signal only if requested by this mode. Additionally the second reconstruction signal from the base layer is required to form the prediction signal, which requires an additional block access and a sample-wise addition.

The BL_INTRA_COMB mode is the most complex mode at the decoder, because it requires two additional forward transforms or a double prediction block filtering to obtain the prediction signal for the block. In case forward transforms are used, the final reconstruction signal can be obtained by adding the scaled transform coefficients to the weighted prediction block in the transform domain. Finally an inverse transform is applied to obtain the reconstruction signal. In the current implementation an intermediate step is performed using the inverse transform in order to obtain the prediction signal in the spatial domain. The scaled coefficients are inverse transformed separately and the final prediction block is obtained by combining the prediction and residual signal in the spatial domain. The current implementation is therefore more complex than necessary, but it therefore can be used with only small extension of the existing intra decoding scheme.

## Complexity characteristics of upsampling filters and transforms specific in enhancement layer(s)

Some important parameters influencing the complexity of upsampling filters are the number of filter taps, bit-depth of filter coefficients, separable or non-separable structure, etc.

The upsampling filters for the base layer reconstruction and base layer residual are both 2D separable. The upsampling filters for base layer reconstruction have 8-taps and use 6-bit filter coefficients, making it similar to motion compensation filters in HEVC. The upsampling filters for base layer residual are bilinear filters.

The transforms for coding enhancement layer residuals are the same as in HEVC.

## Complexity characteristics of quantization and inverse quantization in enhancement layer(s)

Quantization and inverse quantization of residual blocks in the enhancement layer is done in the same way as in HEVC and has thus the same complexity characteristics.

## Complexity characteristics of encoder entropy coding operation in enhancement layer(s)

The complexity of the encoder entropy coding operation in enhancement layer(s) is slightly increased in terms of runtime. This is due to the increased number of rate distortion optimized quantization calls for estimating the costs of different scan patterns in the case of 16x16 and 32x32 luminance transformation blocks. However, as the cost estimating processes for different scan patterns are independent of each other, they can be done parallel and thus, decrease the runtime overhead.

## Complexity characteristics of decoder entropy decoding operation in enhancement layer(s)

The entropy decoding complexity of the enhancement layer is comparable to the entropy decoding process of single-layer HEVC, as the introduced decoding process of specific scan patterns is negligible itself and done even for a subset of 16x16 and 32x32 TUs only.

## Degree of capability for encoder parallel processing

Since the enhancement layer encoding process is similar to the HEVC encoding process except for the additional tools, the same techniques for parallel processing as in HEVC can be applied. Furthermore, the enhancement layer encoding depends on the base layer signal, which must be available. More precisely, for the encoding of a particular enhancement layer block, the collocated base layer block (or blocks) must already be encoded. Therefore, encoding of the enhancement layer can start if sufficient blocks of the base layer are already encoded and in this way, partly parallel encoding of base and enhancement layer can be accomplished.

## Degree of capability for decoder parallel processing

For decoding the enhancement layers, a similar degree of parallel processing as for single-layer HEVC is provided.

# Software implementation description

The enhancement layer software is based on HM-6.1. The changes in the software, which include additional modes are done in a way that the changes of the existing software are minimized. In order to support H.264/AVC base layers, a modified decoder taken from the JMVC software was integrated into the software, which also includes a mapping from H.264/AVC symbols to HEVC symbols. The H.264/AVC decoder is available at encoder and decoder side. The decoder is designed such that multiple layer decoders simultaneously decode depending layers in a frame-wise order.

The encoder is capable of either decoding an HEVC or H.264/AVC base layer bitstream and encoding the HEVC enhancement layer or encoding HEVC base and enhancement layers simultaneously.

The provided enhancement layer features are configurable using configuration file parameters at the encoder. The configuration is transmitted via a bitmask in the SPS, thus, the decoder doesn't need to be rebuild in order to get a matching decoder configuration.

At the outer API, the software is structure more modular than the existing HEVC version. Thus the integration of upsampling functionality and the inter-layer interaction is provided in the encoder and decoder applications, which strongly increases the readability of the code.

Another change worth to mention is the adaption of the enhancement layer coding order in case an H.264/AVC base layer is used. This on-the-fly adaptation analyses the base layer and reconfigures the processing order, reference picture list etc. on the fly. The modification in the code are encapsulated in functions to ease the identification of those changes.

To give the enhancement layer access to appropriated base layer information an intermediate picture buffer is used which provides the upsampled base layer prediction signals as well as the upsampled syntax elements stored in a CU data structure. This buffer is linked to the CU data of the enhancement layer to enable full accesses to the upsampled base layer data wherever CU data of the enhancement layer are available. Access to base layer reconstruction and residual signals can be done by determining the current enhancement block position and CU address and directly access the prediction signal with these two variables.

To obtain the upsampled signals, a conversion filter class was implemented in a new file. The upsampling method for the syntax elements was integrated in the hosting the PicSym and CUData classes.

All spatial intra prediction modes are integrated in the existing intra prediction mode functions with only small adaptations. The same applies for inter prediction modes, where a base layer signal is used to support the prediction.

# Highlighted aspects discussion

The scalable extension describes in this document has the following properties:

- It uses the same concept of CUs/PUs/TUs.

- The added inter-layer prediction mechanisms re-use existing functionality as motion-compensated prediction, spatial intra prediction, and transform coding to a large extend.

- The same coding tools are used for spatial and quality scalability.

- The same coding tools are used for scalable representations with AVC and HEVC base layers.

# Closing remarks

In this document, a scalable extension of HEVC is proposed. For coding the enhancement layers, the basic decoding processes as specified in HEVC are re-used with additional inter-layer prediction concepts. The same coding tools are used for spatial and quality scalability, and for enhancement layers for AVC and HEVC base layers. The simulation results indicate that substantial bit rate savings relative to simulcast are achievable with the proposed techniques.

# Patent rights declaration(s)

Fraunhofer HHI may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).