| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 11th Meeting: Shanghai, CN, 10-19 Oct., 2012 | Document: JCTVC-K0045 |
| --- | --- |

| Title: | Description of low complexity scalable video coding technology proposal by Vidyo and Samsung |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal, Information |  |  |
| Author(s) or Contact(s): | Jill Boyce, Danny Hong, Wonkap Jang, Adeel Abbas, Srinath Reddy, Ken McCann, Jeong Hoon Park, JaeHyun Kim, Chanyul Kim, Jung-Hye Min, Elena Alshina, Alexander Alshin, Il-Koo Kim, Tammy Lee, Byeongdoo Choi, YinJi Piao, SeungSoo Jeong, Sunil Lee, Yongjin Cho, Jeong Yong Choi | Email: | jill@vidyo.com, danny@vidyo.com, wonkap@vidyo.com, adeel@vidyo.com, sreddy@vidyo.com, ken@zetacast.com, jeonghoon@samsung.com, jhgim@samsung.com, dionism@samsung.com, jh643.min@samsung.com, elena_a.alshina@samsung.com, alexander_b.alshin@samsung.com, ilkoo.kim@samsung.com, tammy.lee@samsung.com, b.d.choi@samsung.com, yj1003.piao@samsung.com, ss00.jeong@samsung.com, sunil.lee@samsung.com, yongjin9.cho@samsung.com, eldragon.choi@samsung.com |
| Source: | Vidyo, Inc. and Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This proposal is the response to the Call for Proposals on Scalable Video Coding Extensions of High Efficiency Video Coding (HEVC), jointly produced by Vidyo and Samsung.  It includes responses to all categories included in the CfP: spatial scalability, intra-only scalability, SNR scalability, and coded scalability. Comparing the scalable enhancement layer to the simulcast high resolution anchor, the proposal reports average luma BD-rate gains of 30.3% for RA HEVC 2x spatial scalability, 47.1% for RA HEVC 1.5x spatial scalability, 35.4% for All Intra HEVC 2x spatial scalability, 54.1% for All Intra HEVC 1.5x spatial scalability, 36.8% for HEVC SNR scalability.  For hybrid codec scalability with a base AVC layer, the proposal reports average BD-rate gains vs HEVC simulcast high-resolution anchors of 28.9% for 2x, and 43.2% for 1.5x.  New coding tools are proposed for the enhancement layer, taking advantage of the texture, the motion information, and the intra prediction modes of the base layer. Other tools include modified deblocking filter, additional motion compensation filter, and inter-layer SAO. A higher complexity trade-off point for this coding framework is presented in the companion proposal described in JCTVC-K0044.

# Contents

Abstract	1

Contents	1

1	Introduction	2

2	Algorithm description	2

2.1	CTU/TU/PU partitioning	2

2.2	Up-sampling filters	2

2.3	Inter-layer prediction	2

2.3.1	Inter-layer prediction of CU/TU/PU partitions	2

2.3.2	Inter-layer residual prediction	2

2.3.3	Inter-layer motion vector prediction	2

2.3.4	Inter-layer mode prediction	2

2.3.5	Other	2

2.4	Inter-frame prediction	2

2.5	Intra-frame prediction	2

2.6	In-loop filtering	2

2.6.1	De-blocking filter	2

2.6.2	SAO	2

2.6.3	Other	2

2.7	Spatial transforms	2

2.8	Quantization	2

2.9	Motion/mode/parameter coding	2

2.9.1	Merge/skip	2

2.9.2	Motion vector coding	2

2.9.3	Intra prediction mode coding	2

2.9.4	SAO parameter coding	2

2.9.5	Other	2

2.10	Entropy coding of residual / transform coefficients	2

2.11	High-layer syntax	2

2.12	Additional algorithmic description discussion topic #1	2

2.13	Additional algorithmic description discussion topic #2	2

2.14	…	2

3	Syntax and semantics description	2

4	Compression performance discussion	2

4.1	Category 1 (HEVC base layer)	2

4.1.1	Spatial scalability	2

4.1.2	Intra-only spatial scalability	2

4.1.3	SNR scalability	2

4.1.4	Overall	2

4.2	Category 2 (AVC base layer)	2

4.2.1	Spatial scalability	2

4.2.2	Overall	2

5	Complexity analysis	2

5.1	Encoding time and measurement methodology	2

5.2	Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM	2

5.3	Description of computing platform used to determine encoding and decoding times reported in sections 5.1 and 5.2	2

5.4	Expected memory usage of encoder	2

5.5	Expected memory usage of decoder	2

5.6	Complexity characteristics of encoder motion estimation and partitioning selection in enhancement layer(s)	2

5.7	Complexity characteristics of decoder motion compensation in enhancement layer(s)	2

5.8	Complexity characteristics of encoder intra-frame prediction type and partitioning selection in enhancement layer(s)	2

5.9	Complexity characteristics of decoder intra-frame prediction operation in enhancement layer(s)	2

5.10	Complexity characteristics of upsampling filters and transforms specific in enhancement layer(s)	2

5.11	Complexity characteristics of quantization and inverse quantization in enhancement layer(s)	2

5.12	Complexity characteristics of encoder entropy coding operation in enhancement layer(s)	2

5.13	Complexity characteristics of decoder entropy decoding operation in enhancement layer(s)	2

5.14	Degree of capability for encoder parallel processing	2

5.15	Degree of capability for decoder parallel processing	2

5.16	Additional complexity discussion topic #1	2

5.17	Additional complexity discussion topic #2	2

5.18	…	2

6	Software implementation description	2

7	Highlighted aspects discussion	2

8	Closing remarks	2

9	Patent rights declaration(s)	2

# Introduction

This proposal is the response to the scalable coding extension Call for Proposals by Vidyo and Samsung.  The proposal covers all categories, and obeys all of the constraints.

 	Category 1 (HEVC base layer) spatial scalability

 	Category 1 (HEVC base layer) intra-only spatial scalability

 	Category 1 (HEVC base layer) SNR scalability

 	Category 2 (AVC base layer) spatial scalability

 	The proposal obeys the constraints under section 5 of the CfP (if box is not ticked, explain cases where constraints are violated)

# Algorithm description

The following tools are added to the HEVC base specification for scalability support

- Intra_BL mode

  - Intra_BL Skip mode

- Inter_BL mode

- Difference mode

    - Bilinear MC filter for difference mode

    - Deblocking filter consideration of difference mode

- Inter-layer spatial MPM prediction

- Inter-layer motion prediction

- Inter-layer SAO

The base layer is referred to as BL, and the enhancement layer as EL.  For the response to the call, only two layers are used, but the proposed tools could be used for more scalable layers.

## CTU/TU/PU partitioning

The same basic CTU/TU/PU partitioning structure from the current HEVC spec is used.

A CU may be coded in Intra_BL or Inter_BL mode, for which the PU size is inferred to be 2Nx2N.   If the CU is not coded in Intra_BL or Inter_BL mode, PU signaling is performed as in the current HEVC specification.

## Up-sampling filters

The up-sampling filter is implemented as a separable 2D FIR filter in the same way as that for the fractional motion compensation. The filter coefficients were generated by using the same algorithm for the motion compensation interpolation (DCT-IF, refer to [1]) in the current HEVC specification. As shown in Tables 1 and 2, Luma and Chroma samples are up-scaled by 8-tap and 4-tap filters, respectively. The phase shift caused by the up-sampling process is dependent on the resolution ratio between a BL and an EL.

Table 1 – 8-tap DCT-IF coefficients for general luma interpolation

| Position | Filter coefficients |
| --- | --- |
| 1/12 | { -1, 1, -4, 63, 6, -2, 1, 0 } |
| 2/12 | { -1, 3, -7, 61, 12, -5, 2, -1} |
| 3/12 | { -1, 4, -10, 58, 17, -5, 1, 0 } |
| 4/12 | { 1, 4, -11, 52, 26, -8, 3, -1} |
| 5/12 | { -1, 5, -12, 47, 32, -9, 3, -1 } |
| 6/12 | { -1, 4, -11, 40, 40, -11, 4, -1} |

Table 2 – 4-tap DCT-IF coefficients for general chroma interpolation

| Position | Filter coefficients |
| --- | --- |
| 1/12 | { -2, 62, 5, -1} |
| 2/12 | { -4, 59, 11, -2 } |
| 3/12 | { -4, 54, 16, -2 } |
| 4/12 | { -5, 50, 22,-3 } |
| 5/12 | {-5, 43, 30, -4 } |
| 6/12 | {-4, 36, 36, -4 } |

## Inter-layer prediction

The EL coding takes advantage of prediction from BL texture, motion information (motion vectors, reference picture indices, etc…), and intra prediction mode.  Double-loop decoding is required.  Several CU coding modes are introduced, some of which may be combined, to take advantage of inter-layer prediction, including Intra_BL mode, Inter_BL mode, and difference mode.  In the proposed encoder, a RD-optimized decision is made for the various possible CU modes.

Intra_BL mode has similar functionality to that mode in SVC, where a block of reconstructed BL pixels are scaled to the EL CU resolution and used as the predictor for the 2Nx2N PU of the CU.  One difference to the mode in SVC is that this mode can be used for any CU in the EL even though the corresponding BL block is not coded in intra.  This difference is allowed due to double-loop decoding.

Inter_BL mode is introduced where the EL CU derives its motion information from corresponding BL.

When coding the EL, if a CU is not coded in Intra_BL mode, then it can be coded either in “pixel coding mode” or “difference coding mode”, signaled with a flag added to the CU – diff_coding_flag.  In pixel coding mode, the input pixels of the input picture are simply coded as in the current non-scalable HEVC design; however, in difference coding mode, the prediction tools of the non-scalable HEVC (i.e., intra prediction and motion compensation) is combined with inter-layer textual prediction to derive the predicted block of pixels.  In pixel coding mode, intra and inter prediction is done only from spatially or temporally neighboring (coded) pixel values of the EL, while in difference coding mode, intra and inter prediction also incorporate the (coded) pixel values of the BL.

Figure 1 – Non-scalable HEVC encoder

Figure 2 – Proposed scalable HEVC encoder

Enhancement layer intra CU coding

EL CU can be coded using the same spatial intra directional prediction as in the non-scalable HEVC design.  Additionally, inter-layer prediction is applied using either Intra_BL or difference coding mode.

When the CU is coded in Intra_BL mode, no spatial intra directional prediction is performed, and only inter-layer pixel prediction is used.  In this mode, the corresponding BL block of reconstructed pixels are scaled to the EL CU resolution and used as the predictor for the 2Nx2N PU of the CU.  The resulting residual samples of the CU are coded following non-scalable HEVC’s inter residual sample coding process with its prediction mode set to intra DC mode by default.

When the CU is coded in Intra_BL mode, due to the efficiency of the mode, the residual coefficients often get quantized to zero.  As a result, a special mode of Intra_BL – Intra_BL Skip mode – is introduced.  In this case the CU is implied to have no significant coefficient.

When the CU is not coded in Intra_BL mode, spatial intra directional prediction and inter-layer prediction can be combined by coding the CU in difference coding mode.  In this mode, spatial intra directional prediction is performed using previously coded difference values, resulting in a predicted block of difference values, and the scaled BL’s reconstructed pixels that correspond to the current pixels of the CU are added.

Below is a figure of the non-scalable HEVC intra prediction (IPred) module where the inputs are the current block of pixels and previously coded pixels and the outputs are the predicted pixels for the current block and the intra prediction info (e.g., the spatial intra directional mode).  The IPred module consists of Spatial Pred module that derives predicted blocks using the intra prediction modes, and Mode Decision module that chooses the “best” mode.

Figure 3 – Non-scalable HEVC intra prediction (IPred) module

The EL IPred module can be implemented by applying some modifications to the above described IPred module.  The EL IPred module is shown in the below figure, where depending on the diff_coding_flag, the input to the Spatial Pred module can be either previously coded pixels or difference values.  In the latter case (in difference coding mode), the previously coded difference values are derived by subtracting from the previously coded pixels the corresponding scaled BL’s reconstructed pixels.  Additionally, in difference coding mode, the output of the Spatial Pred module (predicted block of difference values) must be combined with the scaled BL’s reconstructed pixels that correspond to the current block of pixels being coded.

The EL IPred module also has an option to use the scaled BL’s reconstructed pixels that correspond to the current block of pixels being coded – signaled by intra_bl_flag or intra_bl_skip_flag.  When intra_bl_skip_flag is set (indicating Intra_BL Skip mode), no additional residual samples are coded for the CU.

Figure 4 – Proposed scalable HEVC intra prediction (EL IPred) module

Note that for the EL IPred module there is additional scaled BL IPred info as an input to the Mode Decision module.  This information is used for inter-layer spatial MPM prediction where the most probable spatial prediction mode is derived based on the corresponding BL’s coded mode, resulting in, on the average, less bits to code the IPred info for the EL PU.  When determining the corresponding BL PU (or a block in the case of mixed-codec scalability), there is no required correlation between the CU or PU sizes for the BL and the EL.  The EL simply selects the position corresponding to the center of the EL PU, and the BL PU that includes the corresponding sample (after properly scaling) is used.  The BL’s information is currently stored in the granularity of 4x4 blocks; however, if memory is an issue, the future version can be modified to store the information in the granularity of 8x8 blocks.

Enhancement layer inter CU coding

EL CU can be coded using the same inter prediction tools as in the non-scalable HEVC design.  Additionally, inter-layer prediction is applied using Inter_BL and/or difference coding mode.

When the CU is coded in Inter_BL mode, all motion information for each 8x8 block of the CU is derived from the corresponding BL block’s motion information.  For example, if the CU is a 32x32 block, then each of the sixteen 8x8 blocks of the CU gets its own motion information derived from the corresponding BL. When determining the corresponding BL motion information, the BL PU (or a block in case of mixed-codec scalability) that includes the corresponding sample (after proper scaling) of the left-top corner of each 8x8 block is selected.  Hence, each 8x8 block of the CU may get motion compensated blocks from different locations. If the corresponding BL motion information of an 8x8 block is not available, the 8x8 block’s motion is set by the previous 8x8 block’s motion in z-scan order or it is set to zero motion if it is the first 8x8 block inside the CU. Inter-BL mode is applied to not only pixel-coding mode but also difference-coding mode.

When the CU is not coded in Skip mode, inter prediction and inter-layer prediction can be combined by coding the CU in difference coding mode.  In this mode, the motion compensated blocks consist of difference values, and the scaled BL’s reconstructed pixels that correspond to the current pixels of the CU are added.

Figure 5 shows the non-scalable HEVC ME/MC module where the inputs are the current block of pixels and reference pictures from the RBP and the outputs are the predicted (motion compensated) pixels for the current block and the motion info (e.g., motion vectors and reference picture indices).  The ME/MC module consists of MC module that derives predicted blocks using the motion information, and Mode Decision module that chooses the “best” mode.

Figure 5 – Non-scalable HEVC ME/MC module

The EL ME/MC module can be implemented by applying some modifications to the above described ME/MC module.  The EL ME/MC module is shown in the below figure, where depending on the diff_coding_flag, the input to the MC module can be either pixels of the reference pictures or difference values of the reference pictures.  In the latter case (in difference coding mode), the difference values are derived by subtracting from the reference picture pixels the corresponding scaled BL’s reconstructed pixels.  Additionally, in difference coding mode, the output of the MC module (predicted block of difference values) must be combined with the scaled BL’s reconstructed pixels that correspond to the current block of pixels being coded.

The input to the EL ME/MC module can be either pixels of reference pictures or difference values of reference pictures. In difference coding mode, the MC needs the interpolation of residual texture information where residual texture is generated by subtracting the corresponding scaled BL’s reconstructed pixels from the reference picture pixels. Similarly to the up-sampling of residuals for the inter-layer residual prediction in SVC, the bilinear interpolation filter is used for the ME/MC in the difference coding mode while for the ME/MC in the pixel coding mode, the interpolation filter in the current HEVC specification is used.

Figure 6 – Proposed scalable HEVC ME/MC (EL ME/MC) module

Note that for the EL ME/MC module there is additional scaled BL motion info as an input to the Mode Decision module.  This information is used for inter-layer motion prediction.  The scaled BL motion info can be used to derive motion information of EL CU – signaled by inter_bl_flag – resulting in Inter_BL mode.

When the current CU is not coded in Inter_BL mode, for each PU, the BL’s motion information is added as a motion prediction candidate to the candidate list.  In the current implementation, the BL’s motion information is added in the last position in the list; however, placement of the BL’s information can be controlled in slice header in the future version.  When determining the corresponding BL PU (or a block in the case of mixed-codec scalability), there is no required correlation between the CU or PU sizes for the BL and the EL.  The EL simply selects the position corresponding to the center of the EL PU, and the BL PU that includes the corresponding sample (after properly scaling) is used.  The BL’s information is currently stored in the granularity of 4x4 blocks; however, if memory is an issue, the future version can be modified to store the information in the granularity of 8x8 blocks.

### Inter-layer prediction of CU/TU/PU partitions

In the proposed scalable codec, inter-layer prediction of CU/TU/PU partitions is not employed.

### Inter-layer residual prediction

Double loop decoding is required in this proposal.  Inter-layer residual prediction, such as that done in SVC, is not used.  Instead inter-layer texture prediction is used for both intra and inter coding of CU.

### Inter-layer motion vector prediction

As described in more detail in section 2.3, when the current CU is not coded in Inter_BL mode, for each PU, the BL’s motion information is added as a motion prediction candidate to the candidate list, where the BL’s motion information is retrieved from a BL PU corresponding to the center of the EL PU. The BL’s motion information is used for both merge mode and motion vector predictor candidates.

### Inter-layer mode prediction

Inter-layer mode prediction is applied for deriving EL PU’s most probable modes (MPMs) for (non-Intra_BL) intra coding.  The current non-scalable HEVC design’s derivation process for MPMs yields three distinct MPMs in the order of likelihood to be chosen.  Here, the three MPMs are referred to as the target layer MPMs.  For the EL PU, if the corresponding BL PU is also coded in intra mode, then the BL’s coded intra prediction mode can also be used as an MPM for the EL PU: the BL’s coded intra prediction mode is referred to as the base layer MPM.  If the base layer MPM exists, then it is used as the very first MPM (the most-likely to be chosen MPM) and the first two target layer MPMs that is different from the base layer MPM is used as the second and third MPMs, respectively.

### Other

Inter-layer SAO

With inter-layer SAO, SAO process specified in the current non-scalable HEVC specification is applied to up-sampled BL reconstructed pictures to minimize the difference between up-sampled BL reconstructed pictures and original EL input pictures. The output of inter-layer SAO that improves up-sampled signal is used for difference coding mode or Intra_BL mode. For this reason, inter-layer SAO works as an inter-layer prediction tool. In the current implementation, inter-layer SAO parameters are signaled per slice basis as in the HM 6.1, where both of edge offset (EO) and band offset (BO) are applied differently from the in-loop SAO that applies either EO or BO. The overall structure of the inter-layer SAO is depicted in Figure 7, and Figure 8 illustrates how in-loop and inter-layer SAOs are applied in the proposed scalable codec. The in-loop SAO for both BL and EL and the inter-layer SAO share a sample processing module and temporal buffers.

Figure 7 – Inter layer SAO operating sturcture

Figure 8 – SAO application in the proposed scalable codec

## Inter-frame prediction

Detailed description is available in Section 2.3.

## Intra-frame prediction

Detailed description is available in Section 2.3.

## In-loop filtering

### De-blocking filter

As in the current non-scalable HEVC specification, the deblocking filter process is applied to all PU and TU edges according to the partition type and the transform depth of each coding unit. However, when the CU is coded in Inter_BL mode, the prediction unit boundary is derived as 8x8 block even though its partition type is SIZE_2Nx2N.

An additional criteria is added to the de-blocking filter’s block strength calculation to consider difference coding mode vs. pixel coding mode, as highlighted in Figure 9.

Figure 9 – Proposed de-blocking filter strength calculation

### SAO

The SAO is incorporated in the EL coding process in the same way as in the current non-scalable HEVC specification.

### Other

## Spatial transforms

Spatial transforms are unchanged from the current non-scalable HEVC specification.

## Quantization

Quantization is unchanged from the current non-scalable HEVC specification.

## Motion/mode/parameter coding

### Merge/skip

Intra_BL Skip mode has been introduced in the EL coding, whose detailed description can be found in Section 2.3. The syntax and semantics of Intra_BL Skip mode are provided in Section 3.

### Motion vector coding

As described in section 2.3.3, an additional motion vector predictor candidate is added, from the co-located base layer PU.  Otherwise, motion vector coding is unchanged from the current HEVC specification.

### Intra prediction mode coding

Intra prediction mode coding is unchanged from the current non-scalable HEVC specification

### SAO parameter coding

SAO parameter coding is unchanged from the current non-scalable HEVC specification, both for the added in-loop SAO, as well as the normal SAO.

### Other

## Entropy coding of residual / transform coefficients

Entropy coding of residual/transform coefficients is unchanged from the current non-scalable HEVC specification.

## High-layer syntax

The high-layer syntax of the proposal is compatible with use of the VPS and the options presented in JCTVC-J1007, but have not been implemented, because the software is based upon HM 6.1.  The high-layer syntax is based upon JCTVC-H1003_dK, which best corresponds to the HM 6.1 software.

The reserved field, reserved_one_5bits, in the NAL unit header is used for layer_id.  (For the most recent version of the specification, the corresponding syntax element would be nuh_reserved_zero_6bits).   The layer_id value is used to distinguish between the base layer and the enhancement layer.

# Syntax and semantics description

Proposed modifications to the CU syntax are highlighted in the table below.

## Coding unit syntax

| coding_unit( x0, y0, log2CbSize, RefLayerId ) { | Descriptor |
| --- | --- |
| CurrCbAddrTS = MinCbAddrZS[ x0 >> Log2MinCbSize ][ y0 >> Log2MinCbSize ] |  |
| if( slice_type != I ) |  |
| skip_flag[ x0 ][ y0 ] | ae(v) |
| if( skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| else { |  |
| if( RefLayerId != -1 ) { // a dependent layer exists |  |
| intra_bl_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( !intra_bl_skip_flag[ x0 ][ y0 ] ) { |  |
| intra_bl_flag[ x0 ][ y0 ] | ae(v) |
| if( !intra_bl_flag[ x0 ][ y0 ] ) { |  |
| if( slice_type != I ) |  |
| inter_bl_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| if( !intra_bl_flag [ x0 ][ y0 ] && !inter_bl_flag[ x0 ][ y0 ] ) { |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( PredMode != MODE_INTRA ¦ ¦ log2CbSize = = Log2MinCbSize ) |  |
| part_mode | ae(v) |
| x1 = x0 + ( ( 1 << log2CbSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2CbSize ) >> 1 ) |  |
| x2 = x1 − ( ( 1 << log2CbSize ) >> 2 ) |  |
| y2 = y1 − ( ( 1 << log2CbSize ) >> 2 ) |  |
| x3 = x1 + ( ( 1 << log2CbSize ) >> 2 ) |  |
| y3 = y1 + ( ( 1 << log2CbSize ) >> 2 ) |  |
| if( PartMode = = PART_2Nx2N ) |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| else if( PartMode = = PART_2NxN ) { |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x0, y1 , log2CbSize ) |  |
| } else if( PartMode = = PART_Nx2N ) { |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x1, y0 , log2CbSize ) |  |
| } else if( PartMode = = PART_2NxnU ) { |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x0, y2 , log2CbSize ) |  |
| } else if( PartMode = = PART_2NxnD ) { |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x0, y3 , log2CbSize ) |  |
| } else if( PartMode = = PART_nLx2N ) { |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x2, y0 , log2CbSize ) |  |
| } else if( PartMode = = PART_nRx2N ) { |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x3, y0 , log2CbSize ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x1, y0 , log2CbSize ) |  |
| prediction_unit( x0, y1 , log2CbSize ) |  |
| prediction_unit( x1, y1 , log2CbSize ) |  |
| } |  |
| } !intra_bl_flag[ x0 ][ y0 ] && !inter_bl_flag[ x0 ][ y0 ] |  |
| if( RefLayerId != -1 && !intra_bl_flag[ x0 ][ y0 ] ) |  |
| diff_coding_flag[ x0 ][ y0 ] | ae(v) |
| if( !intra_bl_skip_flag[ x0 ][ y0 ] ) { |  |
| if( !pcm_flag ) |  |
| transform_tree( x0, y0, x0, y0, log2CbSize, log2CbSize, log2CbSize, 0, 0 ) |  |
| } // !intra_bl_skip_flag[ x0 ][ y0 ] |  |
| } |  |
| } |  |

### Transform tree syntax

| transform_tree( x0, y0, xC, yC, log2CbSize, log2TrafoWidth, log2TrafoHeight, trafoDepth, blkIdx ) { | Descriptor |
| --- | --- |
| if( trafoDepth = = 0 && IntraSplitFlag = = 0 && PredMode != MODE_INTRA && !(PartMode = = PART_2Nx2N && merge_flag[x0][y0] && !diff_coding_flag[x0][y0]) && ! intra_bl_flag[x0][y0]) |  |
| no_residual_data_flag | ae(v) |
| if( !no_residual_data_flag ) { |  |
| log2TrafoSize = ( log2TrafoWidth + log2TrafoHeight ) >> 1 |  |
| intraSplitFlag = ( IntraSplitFlag && trafoDepth = = 0 ? 1 : 0 ) |  |
| interSplitFlag = ( max_transform_hierarchy_depth_inter = =0 && PredMode = = MODE_INTER && PartMode != PART_2Nx2N && trafoDepth = = 0 ) |  |
| maxDepth = ( PredMode = = MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : max_transform_hierarchy_depth_inter + InterSplitFlag ) |  |
| xBase = x0 − ( x0 & ( 1 << log2TrafoWidth ) ) |  |
| yBase = y0 − ( y0 & ( 1 << log2TrafoHeight ) ) |  |
| if( log2TrafoSize <= Log2MaxTrafoSize && log2TrafoSize > Log2MinTrafoSize && trafoDepth < maxDepth && !intraSplitFlag && !interSplitFlag ) |  |
| split_transform_flag[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| if( log2TrafoSize <= Log2MaxTrafoSize ) { |  |
| firstChromaCbf = ( log2TrafoSize = = Log2MaxTrafoSize ¦ ¦ trafoDepth = = 0 ) ? 1 : 0 |  |
| if( firstChromaCbf ¦ ¦ log2TrafoSize > Log2MinTrafoSize ) { |  |
| if( firstChromaCbf ¦ ¦ cbf_cb[ xBase ][ yBase ][ trafoDepth − 1 ] ) { |  |
| readCbf = TRUE |  |
| if( blkIdx = = 3 && log2TrafoSize < Log2MaxTrafoSize ) |  |
| readCbf = cbf_cb[ xBase ][ yBase ][ trafoDepth ] ¦ ¦ cbf_cb[ xBase + ( 1 << log2TrafoWidth ) ][ yBase ][ trafoDepth ] ¦ ¦ cbf_cb[ xBase ][ yBase + ( 1 << log2TrafoHeight ) ][ trafoDepth ] |  |
| if( !readCbf ) |  |
| cbf_cb[ x0 ][ y0 ][ trafoDepth ] = 1 |  |
| else |  |
| cbf_cb[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| } |  |
| if( firstChromaCbf ¦ ¦ cbf_cr[ xBase ][ yBase ][ trafoDepth − 1 ] ) { |  |
| readCbf = TRUE |  |
| if( blkIdx = = 3 && log2TrafoSize < Log2MaxTrafoSize ) |  |
| readCbf = cbf_cr[ xBase ][ yBase ][ trafoDepth ] ¦ ¦ cbf_cr[ xBase + ( 1 << log2TrafoWidth ) ][ yBase ][ trafoDepth ] ¦ ¦ cbf_cr[ xBase ][ yBase + ( 1 << log2TrafoHeight ) ][ trafoDepth ] |  |
| if( !readCbf ) |  |
| cbf_cr[ x0 ][ y0 ][ trafoDepth ] = 1 |  |
| else |  |
| cbf_cr[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| } |  |
| } |  |
| } |  |
| if( split_transform_flag[ x0 ][ y0 ][ trafoDepth ] ) { |  |
| if( InterTUSplitDirection = = 2 ) { |  |
| x1 = x0 + ( ( 1 << log2TrafoWidth ) >> 1 ) |  |
| y1 = y0 |  |
| x2 = x0 |  |
| y2 = y0 + ( ( 1 << log2TrafoHeight ) >> 1 ) |  |
| x3 = x1 |  |
| y3 = y2 |  |
| } else { |  |
| x1 = x0 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y1 = y0 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| x2 = x1 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y2 = y1 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| x3 = x2 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y3 = y2 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| log2TrafoHeight = log2TrafoHeight + 2 * InterTUSplitDirection − 1 |  |
| log2TrafoWidth = log2TrafoWidth − 2 * InterTUSplitDirection + 1 |  |
| } |  |
| transform_tree( x0, y0, x0, y0, log2CbSize, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 0 ) |  |
| transform_tree( x1, y1, x0, y0, log2CbSize, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 1 ) |  |
| transform_tree( x2, y2, x0, y0, log2CbSize, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 2 ) |  |
| transform_tree( x3, y3, x0, y0, log2CbSize, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 3 ) |  |
| } else { |  |
| if( PredMode = = MODE_INTRA ¦ ¦ trafoDepth != 0 ¦ ¦ cbf_cb[ x0 ][ y0 ][ trafoDepth ] ¦ ¦ cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) { |  |
| readCbf = TRUE |  |
| if( blkIdx = = 3 && PredMode != MODE_INTRA && ( ( log2CbSize <= Log2MaxTrafoSize+1 ) ¦ ¦ ( log2TrafoSize < Log2MaxTrafoSize ) ) |  |
| readCbf = cbf_luma[ xBase ][ yBase ][ trafoDepth ] ¦ ¦ cbf_luma[ xBase + ( 1 << log2TrafoWidth ) ][ yBase ][ trafoDepth ] ¦ ¦ cbf_luma[ xBase ][ yBase + ( 1 << log2TrafoHeight ) ][ trafoDepth ] ¦ ¦ cbf_cb[ xBase ][ yBase ][ trafoDepth − 1] ¦ ¦ cbf_cr[ xBase ][ yBase ][ trafoDepth − 1] |  |
| if( !readCbf ) |  |
| cbf_luma[ x0 ][ y0 ][ trafoDepth ] = 1 |  |
| else |  |
| cbf_luma[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| } |  |
| transform_unit (x0, y0, xC, yC, log2TrafoWidth, log2TrafoHeight, trafoDepth, blkIdx) |  |
| } |  |
| } |  |
| } |  |

## Coding unit semantics

intra_bl_skip_flag[ x0 ][ y0 ] equal to 1 specifies prediction_unit( ) and transform_tree( ) are not present for the current coding unit. The current CU’s PU information is derived from the reference layer with RefLayerId.  In this case, there is no coded residue and also intra_bl_flag and  diff_coding_flag are not present; intra_bl_flag is implied to be equal to 1. intra_bl_skip_flag[ x0 ][ y0 ] equal to 0 specifies that intra_bl_flag is present and  prediction_unit( ), transform_tree( ), and diff_coding_flag may be present for the current coding unit

intra_bl_flag[ x0 ][ y0 ] equal to 1 specifies prediction_unit( ) is not present for the current coding unit, and is determined using the reference layer with RefLayerId. Additionally, inter_bl_flag and diff_coding_flag are not present.  intra_bl_flag[ x0 ][ y0 ] equal to 0 specifies prediction_unit( ), inter_bl_flag, and diff_coding_flag may be present for the current coding unit.

inter_bl_flag[ x0 ][ y0 ] equal to 1 specifies prediction_unit( ) is not present for the current coding unit, and is determined using the reference layer with RefLayerId.  inter_bl_flag[ x0 ][ y0 ] equal to 0 specifies prediction_unit( ) may be present for the current coding unit.

diff_coding_flag[ x0 ][ y0 ]  equal to 1 specifies that the current coding unit is coded in difference coding mode, as specified in subclause X.X.  diff_coding_flag[ x0 ][ y0 ] equal to 0 specifies that the current coding unit is not coded in difference coding mode.

# Compression performance discussion

Test video sequences used for the performance evaluation of the proposed scalable video codec are summarized in Table 3.

The performance is reported with 4 BD rates of EL-only actual and target bit rates, and EL+BL actual and target bit rates. As the performance anchor, the performance of the non-scalable HEVC encodings is used; single layer for EL-only comparison, and simulcast for EL+BL comparison.

Table 3. Test video sequences

|  |  | Category 1 |  |  |  |  | Category 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Spatial |  | Intra-only |  | SNR | Spatial |  |
|  |  | 2x | 1.5x | 2x | 1.5x |  | 2x | 1.5x |
| Class A+ | PeopleOnStreet (3840x2160) |  | X |  | X |  |  | X |
|  | Traffic (3840x2048) |  | X |  | X |  |  | X |
| Class B | BasketballDrive |  |  |  |  |  |  |  |
|  | BQTerrace |  |  |  |  |  |  |  |
|  | Cactus |  |  |  |  |  |  |  |
|  | Kimono |  |  |  |  |  |  |  |
|  | ParkScene |  |  |  |  |  |  |  |

## Category 1 (HEVC base layer)

### Spatial scalability

As specified in the CfP document, 2 down-scale factors of 1.5 and 2 in each spatial direction are tested. We directly followed coding conditions for the evaluation of the category 1 spatial scalability performance evaluation. In Tables 4 and 5, the EL encoding performance of the proposed scalable codec at the actual and the target bit rates is provided with respect to the non-scalable HEVC anchor. In case of 2x scalability, the average gain of 29.95%, 22.9%, and 22.6% were achieved for Y, U, and V components, respectively. With the 1.5x scalability test, the proposed scalable codec achieved 47.0%, 44.25%, and 40.45% for Y, U, and V components, respectively.

Table 4. Performance of EL-only actual rate

|  | Random Access HEVC 2x |  |  | Random Access HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -33.3% | -25.0% | -26.0% |  |  |  |
| Class B | -27.3% | -21.2% | -19.7% | -47.1% | -47.1% | -40.6% |
| Overall | -30.3% | -23.1% | -22.8% | -47.1% | -47.1% | -40.6% |

Table 5. Performance of EL-only target rate

|  | Random Access HEVC 2x |  |  | Random Access HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -32.7% | -24.3% | -25.3% |  |  |  |
| Class B | -27.1% | -21.0% | -19.4% | -46.9% | -41.4% | -40.3% |
| Overall | -29.9% | -22.7% | -22.4% | -46.9% | -41.4% | -40.3% |

Tables 6 and 7 summarize the performance gain of scalable bitstream (BL+EL) with respect to the simulcast anchor. Average gains of 19.5%, 14.25%, and 14.0% for Y, U, and V components are achieved in case of 2x scalability, and 28.2%, 23.15% and 22.2% of Y, U, V gains are observed with 1.5x scalability.

Table 6. Performance of EL+BL actual rate

|  | Random Access HEVC 2x |  |  | Random Access HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -22.7% | -15.6% | -16.4% |  |  |  |
| Class B | -18.5% | -13.2% | -11.9% | -28.3% | -23.2 | -22.3 |
| Overall | -20.6% | -14.4% | -14.2% | -28.3% | -23.2 | -22.3 |

Table 7. Performance of EL+BL target rate

|  | Random Access HEVC 2x |  |  | Random Access HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -22.3% | -15.1% | -15.9% |  |  |  |
| Class B | -18.4% | -13.1% | -11.8% | -28.1% | -23.1% | -22.1% |
| Overall | -20.3% | -14.1% | -13.8% | -28.1% | -23.1% | -22.1% |

### Intra-only spatial scalability

This configuration examines the intra frame compression performance of the proposed scalable video codec with 2 downscaling factors of 1.5 and 2. The coding gains are summarized in Tables 8, 9, 10, and 11, where the first 2 are the EL-only results, and the latter 2 are the results of scalable bitstream (BL+EL). With the 2x scalability EL-only comparison, 35.35%, 33.25%, and 33.55% of average coding gains are reported for Y, U, and V components, respectively. In case of 1.5x scalability EL-only comparision, 54.1%, 52.25%, and 52.4% of Y, U, and V coding gains are observed.

Table 8. Performance of EL-only actual rate

|  | All Intra HEVC 2x |  |  | All Intra HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -39.1% | -37.2% | -38.0% |  |  |  |
| Class B | -31.6% | -29.4% | -29.2% | -54.1% | -52.3% | -52.4% |
| Overall | -35.4% | -33.3% | -33.6% | -54.1% | -52.3% | -52.4% |

Table 9. Performance of EL-only target rate

|  | All Intra HEVC 2x |  |  | All Intra HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -39.1% | -37.1% | -37.9% |  |  |  |
| Class B | -31.6% | -29.3% | -29.2% | -54.1% | -52.2% | -52.4% |
| Overall | -35.3% | -33.2% | -33.5% | -54.1% | -52.2% | -52.4% |

In case of EL+BL performance comparison, the observed average gains for Y, U, and V components are 24.5%, 22.5%, and 22.8% in case of 2x scalability, and 32.25%, 30.65%, and 30.9% for 1.5x scalability.

Table 10. Performance of EL+BL actual rate

|  | All Intra HEVC 2x |  |  | All Intra HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -27.2% | -25.2% | -26.0% |  |  |  |
| Class B | -21.8% | -19.8% | -19.7% | -32.3% | -30.7% | -30.9% |
| Overall | -24.5% | -22.5% | -22.8% | -32.3% | -30.7% | -30.9% |

Table 11. Performance of EL+BL target rate

|  | All Intra HEVC 2x |  |  | All Intra HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -27.2% | -25.2% | -25.9% |  |  |  |
| Class B | -21.7% | -19.8% | -19.7% | -32.2% | -30.6% | -30.9% |
| Overall | -24.5% | -22.5% | -22.8% | -32.2% | -30.6% | -30.9% |

### SNR scalability

With this test, the performance of SNR scalability is examined, where the resolution ratio between a BL and an EL is 1. In table 12, EL-only gains are summarized, where average gains of 36.6%, 29.8% and 28.3% are observed for Y, U and V components, respectively.

Table 12. Performance of EL-only rate

|  | Actual rate |  |  | Target rate |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -38.7% | -32.5% | -32.5% | -38.1% | -31.8% | -21.9% |
| Class B | -35.0% | -27.6% | -24.5% | -34.8% | -27.3% | -24.3% |
| Overall | -36.8% | -30.0% | -28.5% | -36.4% | -29.6% | -28.1% |

In case of EL+BL performance comparison, the observed average gains for Y, U, and V components are 24.05%, 18.3%, and 17.05%.

Table 13. Performance of EL+BL rate

|  | Actual rate |  |  | Target rate |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -25.3% | -20.02 | -20.3 | -24.9% | -19.8% | -19.9% |
| Class B | -23.1% | -16.7% | -14.1% | -22.9% | -16.5% | -13.9% |
| Overall | -24.2% | -18.4% | -17.2% | -23.9% | -18.2% | -16.9% |

### Overall

With the category 1 test, the scalable bitstream achieves average Y, U, and V gains of 40.6%, 36.49% and 35.46% in case of EL-only rate comparison, and 25.89%, 21.77% and 21.39% in case of EL+BL rate comparison.

## Category 2 (AVC base layer)

With the category 2 tests, the performance of the coding standard scalability is examined, where BLs are encoded with H.264/AVC standard.

### Spatial scalability

Similarly to the category 1 spatial scalability in Section 4.1.1, 2 downscaling factors of 1.5 and 2 are considered and RA from the common test condition is applied. In the EL-only comparison, the proposed scalable codec achieves average Y, U, and V gains of 28.75%, 21.55%, and 21.0% for 2x scalability and 43.1%, 37.9%, and 36.05% for 1.5x scalability as summarized in Tables 14 and 15 .

Table 14. Performance of EL-only actual rate with respect to HEVC anchor

|  | Hybrid vs HEVC 2x |  |  | Hybrid vs HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -32.3% | -23.6% | -24.5% |  |  |  |
| Class B | -25.5% | -19.9% | -17.9% | -43.2% | -38.0% | -36.2% |
| Overall | -28.9% | -21.7% | -21.2% | -43.2% | -38.0% | -36.2% |

Table 15. Performance of EL-only target rate with respect to HEVC anchor

|  | Hybrid vs HEVC 2x |  |  | Hybrid vs HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -31.9% | -23.1% | -24.1% |  |  |  |
| Class B | -25.3% | -19.6% | -17.6% | -43.0% | -37.8% | -35.9% |
| Overall | -28.6% | -21.4% | -20.8% | -43.0% | -37.8% | -35.9% |

In case of EL+BL rate performance comparison, the observed average gains for Y, U, and V components are 19.1%, 12.8%, and 12.4% in case of 2x scalability, and 24.65%, 20.1%, and 18.55% for 1.5x scalability.

Table 16. Performance of EL+BL actual rate with respect to HEVC anchor

|  | Hybrid vs HEVC 2x |  |  | Hybrid vs HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -21.6% | -14.0% | -14.8% |  |  |  |
| Class B | -16.7% | -11.9% | -10.2% | -24.7% | -20.2% | -18.6% |
| Overall | -19.2% | -12.9% | -12.5% | -24.7% | -20.2% | -18.6% |

Table 17. Performance of EL+BL target rate with respect to HEVC anchor

|  | Random Access HEVC 2x |  |  | Random Access HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -21.4% | -13.7% | -14.5% |  |  |  |
| Class B | -16.5% | -11.7% | -10.0% | -24.6% | -20.0% | -18.5% |
| Overall | -19.0% | -12.7% | -12.3% | -24.6% | -20.0% | -18.5% |

### Overall

The average gains of coding standard scalability is identical to what is reported in 4.2.1 because only RA cases are tested.

# Complexity analysis

## Encoding time and measurement methodology

For encoding time measurement, a clustered system with multiple CPUs and RAMs was used. Each CPU has at most similar clock-speed with AMD Opteron 2.6GHz. The 32 bit executable is generated by GCC 4.12. The execution time is measured on RedHat Linux using intrinsic API functions of Linux.

As specified in the CfP document, the encoding time at the second lowest QP is measured.

With the current implementation, the encoding complexity of the proposed scalable codec appears to be excessively higher than that of the HM 6.1 anchor. However, it has to be noted that the test system was overloaded with lots of simulations, and thus, the measured time may not reflect the actual encoding time of the proposed scalable video codec.

Table 18. Encoding time measurement result of the HM 6.1 anchor

| Class | Resolution | Sequence | Spatial scalability |  |  |  | SNR scalability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | RA HEVC |  | AI HEVC |  | RA HEVC |  |
|  |  |  | [s] | [hr] | [s] | [hr] | [s] | [hr] |
| A+ | 3840x2048 | Traffic | 26871.64 | 7.46 | 9218.81 | 2.56 | 29338.69 | 8.15 |
|  | 3840x2160 | PeopleOnStreet | 37398.48 | 10.39 | 10000.44 | 2.78 | 41382.43 | 11.50 |
| B | 1920x1080 | Kimono | 12855.80 | 3.57 | 3554.03 | 0.99 | 13208.29 | 3.67 |
|  |  | ParkScene | 11912.20 | 3.31 | 4216.42 | 1.17 | 12377.12 | 3.44 |
|  |  | Cactus | 27720.10 | 7.70 | 9085.57 | 2.52 | 31600.89 | 8.78 |
|  |  | BasketballDrive | 31269.95 | 8.69 | 8281.43 | 2.30 | 33020.80 | 9.17 |
|  |  | BQTerrace | 36395.72 | 10.11 | 11503.32 | 3.20 | 41295.75 | 11.47 |
|  |  | Average | 24196.05 | 6.72 | 7383.02 | 2.05 | 26234.55 | 7.29 |

Table 19. Encoding time measurement result of the proposed scalable extension

| Class | Resolution | Sequence | Spatial scalability |  |  |  | SNR scalability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | RA HEVC |  | AI HEVC |  | RA HEVC |  |
|  |  |  | [s] | [hr] | [s] | [hr] | [s] | [hr] |
| A+ | 3840x2048 | Traffic | 74240.19 | 20.62 | 25633.73 | 7.12 | 101528.57 | 28.20 |
|  | 3840x2160 | PeopleOnStreet | 102332.12 | 28.43 | 27886.34 | 7.75 | 137963.73 | 38.32 |
| B | 1920x1080 | Kimono | 36905.35 | 10.25 | 10345.59 | 2.87 | 46693.41 | 12.97 |
|  |  | ParkScene | 34534.30 | 9.59 | 11709.94 | 3.25 | 44384.55 | 12.33 |
|  |  | Cactus | 75664.98 | 21.02 | 25614.97 | 7.12 | 102450.57 | 28.46 |
|  |  | BasketballDrive | 84120.92 | 23.37 | 23341.60 | 6.48 | 109737.54 | 30.48 |
|  |  | BQTerrace | 100141.60 | 27.82 | 31647.36 | 8.79 | 140249.12 | 38.96 |
|  |  | Average | 67173.91 | 18.66 | 20718.27 | 5.76 | 89436.70 | 24.84 |

## Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM

For decoding time measurement, a clustered system with multiple CPUs and RAMs was used. Each CPU has at most similar clock-speed with AMD Opteron 2.6GHz. The 64 bit executable is generated by GCC 4.12. The execution time is measured on RedHat Linux using intrinsic API functions of Linux.

Similarly to the encoding time measurement, the decoding time at the second lowest QP was recorded. As we stated in Section 5.1, the time measurement could be biased due to the overload of the test system.

Table 20. Decoding time measurement result of the HM 6.1 anchor (sec)

| Class | Resolution | Sequence | Spatial scalability |  | SNR scalability |
| --- | --- | --- | --- | --- | --- |
|  |  |  | RA HEVC | AI HEVC | RA HEVC |
| A+ | 3840x2048 | Traffic | 91.87 | 138.68 | 73.95 |
|  | 3840x2160 | PeopleOnStreet | 98.16 | 148.76 | 108.45 |
| B | 1920x1080 | Kimono | 31.53 | 42.93 | 32.77 |
|  |  | ParkScene | 32.57 | 61.76 | 34.06 |
|  |  | Cactus | 82.32 | 130.07 | 77.64 |
|  |  | BasketballDrive | 89.89 | 109.81 | 77.87 |
|  |  | BQTerrace | 110.38 | 163.84 | 118.31 |
|  |  | Average | 69.15 | 103.59 | 67.53 |

Table 21. Encoding time measurement result of the proposed scalable extension (sec)

| Class | Resolution | Sequence | Spatial scalability |  | SNR scalability |
| --- | --- | --- | --- | --- | --- |
|  |  |  | RA HEVC | AI HEVC | RA HEVC |
| A+ | 3840x2048 | Traffic | 153.06 | 216.61 | 191.17 |
|  | 3840x2160 | PeopleOnStreet | 193.61 | 233.55 | 240.52 |
| B | 1920x1080 | Kimono | 68.60 | 81.44 | 89.33 |
|  |  | ParkScene | 70.40 | 96.65 | 87.93 |
|  |  | Cactus | 137.95 | 201.79 | 160.47 |
|  |  | BasketballDrive | 150.45 | 192.47 | 189.66 |
|  |  | BQTerrace | 187.32 | 255.62 | 243.01 |
|  |  | Average | 127.79 | 169.10 | 159.89 |

## Description of computing platform used to determine encoding and decoding times reported in sections 5.1 and 5.2

The encoding and decoding times are measure on the same machine as described in Sections 5.1 and 5.2.

## Expected memory usage of encoder

As described in Section 2.3, difference coding mode requires both coded pixels of the EL and also the coded pixels of the BL.  In this mode, the BL’s coded pixels are scaled and used to modify the inputs and outputs of the Spatial Pred module in Figure 4 and also the MC module in Figure 6.  These inputs and outputs can be generated on the fly and hence no additional memory is required.

To simplify the code in the current software implementation, three additional YUV buffers of the EL picture resolution are added for each picture being coded and stored in the DPB.  The first YUV buffer is used to store, before coding the EL, the scaled version of the BL’s reconstructed picture.  The second YUV buffer is used to store the reconstructed difference value picture that is derived by subtracting the pixel values of the first YUV buffer from the pixel values of the reconstructed picture.  This replaces the need to generate the difference values for the inputs of both the Spatial Pred and MC modules in Figures 4 and 6.  Note that these additional YUV buffers are added for each coded picture in the DPB, and as a result, they are readily available for the MC module.  Lastly, the third YUV buffer is used to store the difference values that correspond to the difference between the input pixel values and the pixel values of the first YUV buffer.  The input difference values are used to speed up the mode decision process by calculating the cost based on the error between the input difference values and predicted difference values for difference coding mode.

For inter-layer prediction of coding information such as intra prediction mode and motion information, the BL’s corresponding coded information can be scaled (upsampled) and used on the fly.  However, for simplification of the current software implementation, the scaled BL’s coded information is stored in the EL for each coded picture in DPB.

Note that some temporal buffers which do not have major impacts were not considered.  Table 22 lists the additional memory usage in the current software implementation.  Again, the three additional YUV buffers and the buffer containing scaled BL’s coded information are not necessary as all the values stored in the buffers can be generated on the fly.

Table 22- Expected memory usage of encoder

| Tools | Extra required memory size | Extra memory size used in the software implementation |
| --- | --- | --- |
| diff_coding_mode | all information can be generated on the fly | 3 YUV buffers for each coded picture in the DPB |
| inter-layer prediction of intra prediction mode and motion info | all information can be generated on the fly | a buffer containing the scaled BL’s coded information for each coded picture in the DPB |

## Expected memory usage of decoder

As described in the above section, difference coding mode requires both coded pixels of the EL and also the coded pixels of the BL, from which the predicted block can be generated on the fly.  Hence, the BL’s YUV buffers for the decoded pictures in the DPB are needed.  Additionally, for inter-layer prediction of coding information such as intra prediction mode and motion information, the BL’s coded information is needed for the current EL picture.  Note that since the proposed scalability design follows double-loop decoding, all these BL information should be readily available form the decoding process of the BL.  The additional memory usage is from the memory needed for decoding the BL.

For the current software implementation, since both the encoder and decoder share the common structure for coded picture, the decoder also has three additional YUV buffers for each coded picture in the DPB.  Additionally, the BL’s coded information is scaled (upsampled) and stored in the EL for each coded picture in the DPB..

Note that some temporal buffers which do not have major impacts were not considered.  Table 23 lists the additional memory usage in the current software implementation.  Again, the three additional YUV buffers and the buffer containing scaled BL’s coded information are not necessary as all the values stored in the buffers can be generated on the fly.

| Table 23- Expected memory usage of decoderTools | Required memory size | Memory size used in software implementation |
| --- | --- | --- |
| decoding BL | memory needed for decoding the BL | memory needed for decoding the BL |
| diff_coding_mode | all information can be generated on the fly | 3 YUV buffers for each coded picture in the DPB |
| inter-layer prediction of intra prediction mode and motion info | all information can be generated on the fly | A buffer containing the scaled BL’s coded information for each coded picture in the DPB |

## Complexity characteristics of encoder motion estimation and partitioning selection in enhancement layer(s)

In the current software implementation, the encoder goes through the ME process twice: one in pixel coding mode and another in difference coding mode. While this implementation greatly increases the complexity of the encoder, there are many possible ways to simplify the process such that ME for difference coding mode is not carried out for every block. One such simplification would be to apply the ME process once in pixel coding mode, and if the best mode is not Skip/Merge, then to try difference coding mode with the same partition and motion information.

The Inter_BL mode just adds one additional cost checking for each CU, hence adds negligible complexity overhead to the overall ME process.

## Complexity characteristics of decoder motion compensation in enhancement layer(s)

Due to double-loop decoding of the proposed design, the BL must always be additionally decoded when decoding the EL.

When a CU in the EL is coded in pixel coding mode, the decoding process is the same as that of HM6.1.

When a CU is coded in difference coding mode, a scaling process of the BL’s decoded pixels and addition of the scaled BL pixels to the predicted block of difference values are additionally needed.  Note that in this mode, if sub-pel interpolation is needed for motion compensation, then a less complex filter (as described in Section 2.3) is applied compared to the interpolation filter used for pixel coding mode.

## Complexity characteristics of encoder intra-frame prediction type and partitioning selection in enhancement layer(s)

For each EL CU, encoding with three different prediction modes, pixel_coding_mode, difference_ mode, and Intra_BL mode, are performed and the best prediction mode in terms of the Lagrangian rate distortion cost is selected.

While the input buffers for pixel_coding_mode and difference_mode are different, they share the same encoding mechanism of angular predictions from the non-scalable HEVC intra prediction. For each prediction unit, the best direction from 35 pre-defined directions is chosen based on following encoding algorithm. From 35 directions, the N (8 for 4x4 and 8x8, 3 for 16x16 and 32x32) best candidates are selected by fast mode decision process which uses the absolute sum of Hadamard transformed residual. At most 3 more directions from left, above, and corresponding BL PU are added to the final candidate list for full RD comparison with actual residual coding cost.

Encoding process of Intra_BL mode does not involve mode decision process from the set of candidates as in angular predictions. Scaled reconstruction pixels from BL are simply copied and used as prediction. RD cost of two coding modes, Intra_BL skip mode and normal Intra_BL, are compared and the final mode with better coding efficiency is selected.

## Complexity characteristics of decoder intra-frame prediction operation in enhancement layer(s)

Due to double-loop decoding of the proposed design, the BL must always be additionally decoded when decoding the EL.

When a CU in the EL is coded in pixel coding mode, the decoding process is the same as that of HM6.1.

When a CU is coded in Intra_BL mode, a scaling process of the BL’s decoded pixels is needed.  However, the derivation process for intra predicted block of pixels is not need as the scaled BL’s decoded pixels are used as the predictor for the CU.

When a CU is coded in difference coding mode, a scaling process of the BL’s decoded pixels and addition of the scaled BL pixels to the predicted block of difference values are additionally needed.

## Complexity characteristics of upsampling filters and transforms specific in enhancement layer(s)

Since spatial transforms are unchanged from the current non-scalable HEVC specification, their complexity characteristics remain the same.

The complexity of up-sampling is equivalent to that of the fractional sample interpolation process in the current non-scalable HEVC specification.

## Complexity characteristics of quantization and inverse quantization in enhancement layer(s)

Because quantization is unchanged from the current non-scalable HEVC specification, the complexity characteristics of quantization and inverse quantization remains the same.

## Complexity characteristics of encoder entropy coding operation in enhancement layer(s)

Because entropy coding is unchanged from the current non-scalable HEVC specification, the complexity characteristics of encoder entropy coding operation remain the same.

## Complexity characteristics of decoder entropy decoding operation in enhancement layer(s)

Because entropy coding is unchanged from the current non-scalable HEVC specification, the complexity characteristics of decoder entropy decoding operation remain the same.

## Degree of capability for encoder parallel processing

Slice/tile level encoder parallel processing is supported, similar to that available in base HEVC.  Additionally, enhancement layer encoding of a particular picture may be performed in parallel to base layer encoding of a subsequent coded picture.

## Degree of capability for decoder parallel processing

Slice/tile level encoder parallel processing is supported, similar to that available in base HEVC.  Additionally, enhancement layer decoding of a particular picture may possibly be performed in parallel to base layer decoding of a subsequent coded picture, depending on picture coding dependencies.

# Software implementation description

The software was based upon the HM6.1 software, and is written in C++.

The encoder can operate in either of two different modes.  In the first mode, the encoder encodes both the base and enhancement layers together.  In the second mode, which was used for the CfP response, the encoder encodes only the enhancement layer, making use of a pre-encoded base layer.  In this second mode, a decoder has been modified to output meta-data (modes, motion vectors, reference indices, etc.).  This modified decoder is run to output the base layer decoded pictures and the meta data, which the enhancement layer encoder uses as an input.

The decoder, similar to the encoder, can operate in one of the two modes described above.  In the first mode, the decoder decodes the scalable bitstream that contains coded information of both the BL and EL.  In the second mode, the input bitstream contains only the EL coded data, and previously decoded BL pictures and meta data are used as additional inputs.

# Highlighted aspects discussion

While the CfP response only utilizes two layers, the proposed techniques could also be used with additional layers.

# Closing remarks

This document provides details of the implementation, complexity and performance of a low complexity proposal produced in response to the Call for Proposals on Scalable Video Coding Extensions of HEVC.  Compared to the simulcast high resolution anchor, the proposal reports average luma BD-rate gains of 30.3% for RA HEVC 2x spatial scalability, 47.1% for RA HEVC 1.5x spatial scalability, 35.4% for All Intra HEVC 2x spatial scalability, 54.1% for All Intra HEVC 1.5x spatial scalability, and 36.8% for HEVC SNR scalability. A higher efficiency trade-off point for this coding framework is presented in a companion proposal [2].

In conclusion, we believe that the proposed coding framework is a strong candidate to be used as the basis of the Test Model for further work on Scalable Video Coding Extensions of HEVC.

# References

- E. Alshina and A. Alshin, “CE3: DCT derived interpolation filter test by Samsung,” JCTVC-F247, Torino, Italy, Jul. 2011

- (Vidyo), K. McCann, J. H. Park, J. Kim, C. Kim, J.-H. Min, E. Alshina, A. Alshin, I.-K. Kim, T. Lee, B. Choi, Y. Piao, S. Jeong, S. Lee, Y. Cho, J. Y. Choi (Samsung)  J. Boyce, D. Hong, W. Jang, A. Abbas, S. Reddy, “Description of high efficiency scalable video coding technology proposal by Vidyo and Samsung”, JCTVC-K0045, Shanghai, China, Oct. 2012

# Patent rights declaration(s)

Vidyo, Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Samsung Electronics Co., Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).