| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 11th Meeting: Shanghai, CN, 10-19 Oct., 2012 | Document: JCTVC-K0046 |
| --- | --- |

| Title: | Description of scalable video coding technology proposal by Huawei Technologies |  |  |
| --- | --- | --- | --- |
| Status: | [Input Document to JCT-VC] |  |  |
| Purpose: | [Proposal] |  |  |
| Author(s) or Contact(s): | Haitao Yang Jinwen Zan Xianghui Wei Wen Gao Haoping Yu Huawei Technologies Co., Ltd Li Li Jinlei Zhang Bin Li Houqiang Li Univ. of Science and Technology of China Lei Feng Univ. of Southern California | Email: haitao.yang@huawei.com Email: jinwen.zan@huawei.com Email: xianghui.wei@huawei.com Email: wen.gao@huawei.com Email: haoping.yu@huawei.com Email: lilimao@mail.ustc.edu.cn Email: jlzhang@mail.ustc.edu.cn Email: yhlibin@mail.ustc.edu.cn Email: lihq@ustc.edu.cn Email: leif@usc.edu |  |
| Source: | [Huawei, USTC] |  |  |

_____________________________

# Abstract

This contribution proposes a video coding solution that will add spatial scalability feature to the design of HEVC. The proposed solution uses HEVC to encode the base-layer and consists of inter-layer texture prediction, inter-layer mode prediction, inter-layer texture skip mode, etc., and these tools are developed jointly by Huawei and University of Science and Technology of China. In average, the proposed HEVC scalable solution can achieve 27.5% BD rate saving when comparing the enhancement layer only bitstreams vs the simulcast HEVC SVC CfP Category 1 (HEVC base layer) anchor bitstreams, and 18.2% BD rate saving when comparing the combined scalable base and enhancement layer bistreams vs. the same set of simulcast HEVC anchor bitstreams.

# Contents

Abstract	1

Contents	1

1	Introduction	3

2	Algorithm description	3

2.1	Up-sampling filters	3

2.2	Inter-layer prediction	4

2.2.1	Inter-layer texture prediction	4

2.2.2	Inter-layer texture prediction skip	4

2.2.3	Inter-layer motion vector prediction	6

2.3	In-loop filtering	7

2.3.1	De-blocking filter	7

3	Syntax and semantics description	8

4	Compression performance discussion	11

4.1	Category 1 (HEVC base layer)	11

4.1.1	Spatial scalability	11

5	Complexity analysis	12

5.1	Encoding time and measurement methodology	12

5.2	Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM	12

5.3	Description of computing platform used to determine encoding and decoding times reported in sections 5.1 and 5.2	12

5.4	Expected memory usage of encoder	13

5.5	Expected memory usage of decoder	13

5.6	Complexity characteristics of encoder motion estimation and partitioning selection in enhancement layer(s)	13

5.7	Complexity characteristics of decoder motion compensation in enhancement layer(s)	13

5.8	Complexity characteristics of encoder intra-frame prediction type and partitioning selection in enhancement layer(s)	13

5.9	Complexity characteristics of decoder intra-frame prediction operation in enhancement layer(s)	13

5.10	Complexity characteristics of upsampling filters and transforms specific in enhancement layer(s)	13

5.11	Complexity characteristics of quantization and inverse quantization in enhancement layer(s)	14

5.12	Complexity characteristics of encoder entropy coding operation in enhancement layer(s)	14

5.13	Complexity characteristics of decoder entropy decoding operation in enhancement layer(s)	14

5.14	Degree of capability for encoder parallel processing	14

5.15	Degree of capability for decoder parallel processing	14

6	Software implementation description	14

7	Closing remarks	14

8	Patent rights declaration(s)	15

# Introduction

(This contribution addresses  Category 1 (HEVC base layer) spatial scalability)

The proposed HEVC scalable coding solution was developed in response to the recent JCTVC Joint Call for Proposals on Scalable Video Coding Extensions of High Efficiency Video Coding (HEVC). In particular, this proposal is targeted at spatial scalability in Category 1 (HEVC base layer) and consists of tools such as inter-layer texture prediction, inter-layer mode prediction, inter-layer texture skip mode, etc. The detailed descriptions of these methods as well as the algorithm used in the design of up-sampling filters are given in Section 2. Section 3 provides syntax and semantics changes resulted from these new tools. The coding performance discussion is given in Section 4, and all of the test results presented in this contribution came from the simulations that were carried out strictly using the requirements and test conditions given in the CfP document. Complexity analysis is given in Section 5, which is followed by Section 6 Software Implementation Description. As in Closing Remarks, we suggest the proposed spatial scalable coding tools be included in the new HM-SVC reference software for further study and testing.

# Algorithm description

## Up-sampling filters

DCT based interpolation method, DCT-IF, has been adopted in the current draft specification of HEVC. In this proposal, the same idea is used in the design of the inter-layer up-sampling filters, and the filter coefficients are calculated based on the DCT and inverse DCT transforms defined in (1) and (2) below.

(1)

(2)

where M=4 and 2 for luma and chroma, repeoctively; and  = i/6, i=0, 1, 2,…5.

In particular, 1/6 sub-pixel sample step is used in up-sampling both luma and chroma components in base-layer. Each luma filter has 8 taps, and each chroma filter has 4 taps. All filter coefficients are scaled by 64 and they are given in Table 1 and 2.

Table 1　Up-sampling filter coefficients for luma

|  | 8-tap |
| --- | --- |
| integer | { 0, 0, 0, 64, 0, 0, 0, 0 } |
| 1/6 | { -1, 3, -7, 61, 12, -5, 2, -1 } |
| 2/6 | { -1, 4, -11, 52, 26, -8, 3, -1 } |
| 3/6 | { -1, 4, -11, 40, 40, -11, 4, -1 } |
| 4/6 | { -1, 3, -8, 26, 52, -11, 4, -1 } |
| 5/6 | { -1, 2, -5, 12, 61, -7, 3, -1 } |

Table 2　Up-sampling filter coefficients for chroma

|  | 4-tap |
| --- | --- |
| integer | { 0, 64, 0, 0} |
| 1/6 | { -4, 59, 11, -2} |
| 2/6 | { -5, 50, 22, -3} |
| 3/6 | { -4, 36, 36, -4} |
| 4/6 | { -3, 22, 50, -5} |
| 5/6 | { -2, 11, 59, -4} |

In this proposal, horizontal up-sampling is performed first, which is followed by vertical up-sampling. While over sampling the base-layer pixels by a factor of 6, the output pixels are only generated for the locations at Int( k x Up-sampling-Ratio /6), where k is the index of the up-sampled pixels, and Up-sampling-Ratio is either 2 or 3/2 for 2x or 1.5x spatial scalability cases, respectively.

## Inter-layer prediction

### Inter-layer texture prediction

By using the up-sampling filtering technique described above, the reconstructed base layer, after up-sampling, is used as the prediction of the enhancement layer CU. This coding mode, called the inter-layer texture prediction mode, applies to CUs in both inter and intra pictures in the enhancement layer, with partition size 2Nx2N. A flag, named interlayer_texture_prediction_flag is inserted after the skip flag to indicate whether this mode is selected.

### Inter-layer texture prediction skip

As described above, the reconstructed base layer can be up-sampled and used as a prediction in encoding the enhancement layer. The difference between the enhancement layer and the up-sampled base layer is then transformed and the resulting transform coefficients are quantized. In case the quantized transform coefficients are all 0, the coding mode for the current coding unit is defined as inter-layer texture prediction skip mode.

It is noted that the inter-layer texture prediction skip mode is different from the existing skip mode in HEVC, since the prediction here is from the up-sampled base layer. Since there is no corresponding mode in the base layer HEVC, this mode has to be signaled in the SVC explicitly.

In HEVC, there is no skip mode in I slice. However, in an SVC enhancement layer I slice, the inter-layer texture prediction skip mode is possible, and needs a new way of signaling. In an HEVC P or B slice, there is already skip mode defined, so the texture prediction skip mode needs to differentiate itself from the already defined inter slice skip mode, and thus a new way of signaling.

The signaling for the inter-layer texture prediction skip can be summarized into the following syntax elements: skip_flag, skip_flag_interlayer_texture_prediction, and no_residual_syntax_flag. The skip_flag and no_residual_syntax_flag already exist in the HEVC draft specification, but the semantics need to be modified as shown below; and skip_flag_interlayer_texture_prediction below is newly added.

skip_flag[ x0 ][ y0 ] equal to 1 specifies that for the current coding unit

- when decoding an enhancement layer I slice, no more syntax elements are parsed after skip_flag[ x0 ][ y0 ].

- when decoding a base layer P or B slice, no more syntax elements except the merging candidate index merge_idx[ x0 ][ y0 ] are parsed after skip_flag[ x0 ][ y0 ].

- when decoding an enhancement layer P or B slice, depending on the skip_flag_interlayer_texture_prediction,  if the reconstruction comes from the up-sampled base layer (no more syntax element after skip_flag_interlayer_texture_prediction), or the same way as a base layer P/B slice (no more syntax elements except the merging candidate index merge_idx[ x0 ][ y0 ])

skip_flag[ x0 ][ y0 ] equal to 0 specifies that the coding unit is not skipped. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When skip_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

skip_flag_interlayer_texture_prediction[ x0 ][ y0 ] equal to 1 specifies that for the current coding unit, no more syntax elements are parsed after skip_flag_interlayer_texture_prediction[ x0 ][ y0 ]. skip_flag_interlayer_texture_prediction[ x0 ][ y0 ] equal to 0 specifies that for the current coding unit, no more syntax elements except the merging candidate index merge_idx[ x0 ][ y0 ] are parsed after skip_flag_interlayer_texture_prediction[ x0 ][ y0 ].

When skip_flag_interlayer_texture_prediction[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

no_residual_syntax_flag equal to 1 specifies that transform_tree() syntax is not present for the current coding unit. no_residual_syntax_flag equal to 0 specifies that transform_tree() syntax is present for the current coding unit.

The no_residual_syntax_flag is signaled under the condition that the current CU is not coded as the PCM mode or the Intra mode, and not as a merged 2Nx2N; or the current CU is in an enhancement layer P/B slice and as the inter layer texture prediction mode.

#### Enhancement layer texture prediction skip in I slice

In the case of enhancement layer I slice, the presence condition of skip_flag in the coding_unit() header is modified as the following, to signal the texture prediction skip mode in the enhancement layer.

coding_unit( x0, y0, log2CbSize ) {

if( transquant_bypass_enable_flag ) {

cu_transquant_bypass_flag

}

if( slice_type != I ¦ ¦ (layerID > 0 && (slice_type == I) ) )

skip_flag[ x0 ][ y0 ]

if( skip_flag[x0][y0] ) {

if( slice_type != I && (layerID > 0) ) skip_flag_interlayer_texture_prediction[x0][y0]

if(skip_flag_interlayer_texture_prediction[x0][y0] != 1)

prediction_unit( x0, y0, log2CbSize )

} else {

…

As seen from the above syntax table, the skip_flag can be used either in the case of a P/B slice skip or in the case of an I slice skip in the enhancement layer. Since the inter slice skip case is similar to that in HEVC, only the case of a skipped CU in an enhancement I slice needs to be further discussed and defined. In this case while skip_flag is 1, no more syntax element is needed to parse the current CU, and the up-sampled base layer is used as the reconstruction of the enhancement layer CU; otherwise when skip_flag is 0, the current CU is encoded either through intra prediction or the PCM mode, and more syntax elements are necessary for the reconstruction.

#### Enhancement layer texture prediction skip in P/B slice

In the case of enhancement layer P/B slice, the skip_flag is already defined in the current HEVC coding unit header. Thus, the case of enhancement layer texture prediction with skip has to be differentiated. For this purpose, a new flag, skip_flag_interlayer_texture_prediction is defined in CU header, as shown in the table above.

In case an enhancement layer CU in a P/B slice is decoded with skip_flag equal to 1: if the skip_flag_interlayer_texture_prediction is 1, the up-sampled base layer pixels should be used as the reconstruction of the current CU; otherwise, the inter skip reconstruction process as defined in HEVC should be used to reconstruct the current CU. The default value of skip_flag_interlayer_texture_prediction is 0.

#### Modification of the semantics of “no_residual_syntax_flag”

The “no_residue_syntax_flag” is originally signaled in HEVC, under the condition that the current CU is not coded as the PCM mode, and not as the Intra mode, and not as a merged 2Nx2N. The no_residual_syntax_flag equal to 1 specifies that transform_tree() syntax is not present for the current CU; no_residual_syntax_flag equal to 0 specifies that transform_tree() syntax is present for the current coding unit.

After the introduction of the base layer texture prediction mode, the no_residue_syntax_flag is also used to indicate, in the case of using base layer texture prediction, if there is transform_tree() syntax in the enhancement layer CU. The “no_residue_syntax_flag” is signaled under the condition that: 1) the condition as described in the above paragraph is true, or 2) the enhancement layer CU is coded as the inter layer texture prediction mode in an P/B slice.

In either case, the no residue_signal_syntax_flag equals to 1 specifies that transform_tree() syntax is not present for the current CU; no_residual_syntax_flag equals to 0 specifies that transform_tree() syntax is present for the current coding unit.

#### Contexts

When decoding the skip_flag of an I slice CU in the enhancement layer, a context is formed based on the directly neighboring left and above coding block. The context index, ctxIdxInc, is initialized to 0. If the directly left neighboring coding block is available and is skipped, ctxIdxInc increases by 1; and if the directly above neighboring coding block is available and is skipped, ctxIdxInc inscreased by 1.

When decoding the skip_flag_interlayer_texture_prediction of a P/B slice CU in the enhancement layer, a context is formed based on the directly neighboring left and above coding block. The context index, ctxIdxInc, is initialized to 0. If the directly left neighboring coding block is available and coded in texture prediction skip mode, ctxIdxInc increases by 1; and if the directly above neighboring coding block is available and coded in texture prediction skip mode, ctxIdxInc increases by 1.

When decoding the no_residual_syntax flag in a CU, the context is the same as in HEVC.

### Inter-layer motion vector prediction

In this proposal, inter-layer motion vector prediction is used in combination with AMVP and the MERGE mode. The basic idea is to identify the motion vector information at the collocated areas in the base-layer, and then to use the scaled motion vector as a candidate in AMVP mode or MREGE mode in encoding the current CU in the enhancement layer.

#### Retrieving motion information from base-layer

Each enhancement layer CU consists of multiple 4x4 blocks. For example, a 16x16 CU consists of 16 4x4 blocks. For each 4x4 block in an enhancement CU, the corresponding 4x4 block in the base layer can be determined and its motion information can be retrieved, including inter prediction direction, motion vector(s) and its reference index, and used as the motion information of the 4x4 block in the enhancement layer after scaling based on the spatial scalability ratio.

If the corresponding 4x4 block in base layer is encoded in intra mode and no motion information is available, the 4x4 block in the enhancement layer UC uses the motion information of its neighboring 4x4 block in the same enhancement layer CU as its own motion information. The neighboring block is determined in a recursive way.

However, if there is no motion information available for all the 4x4 blocks in the current enhancement layer CU, zero motion information is constructed and used for all the 4x4 blocks: For P slices, zero motion vector, zero reference index and motion prediction direction 0 is used. For B slices, zero motion vectors, zero reference index are used for both reference picture list and motion prediction direction is 2.

The motion obtained can be directly used in MODE_INTERLAYER_MODE_PRDICTION for motion prediction and compensation or used as one of merge or AMVP candidate.

#### Enhancement layer inter-layer AMVP candidate

For the AMVP in an enhancement layer CU, the motion information from the center of the corresponding base layer CU is scaled and used as the first MV candidate for the enhancement layer CU. The scaling operation is based on the distance between the reference picture of the corresponding base layer CU and the base layer picture and the distance between the reference picture of the enhancement layer CU and the enhancement layer picture.

The rest of the AMVP process is carried out in the same order as in HEVC, till the max number of AMVP candidate is filled.

#### Enhancement layer inter-layer MERGE candidate

For an enhancement layer CU using the MERGE mode for its motion information, the MERGE candidate list consists of one inter-layer MREGE candidate, three spatial MREGE candidates and one temporal MERGE candidate following the order of decreasing priority. The three spatial MERGE candidates can be obtained by following the same method as that in HEVC but only the first three spatial candidates in priority order are used. The temporal candidate can be obtained by following the HEVC method. If the number of MERGE candidates is less than 5, methods to search other MERGE candidates are the same as that in HEVC.

#### Inter-layer mode prediction for P/B Slices

The main idea of inter-layer mode prediction for P/B Slices is to retrieve the motion information from the corresponding base layer CU following the step in 2.2.3.1, to scale the motion information based on spatial scalability ratio and use them as the motion information for the enhancement layer CU.

For a 4x4 block in the enhancement layer CU, if its corresponding 4x4 block in base layer uses inter-frame prediction, motion compensation is used to obtain predicted samples of the 4x4 block in the enhancement layer CU. However, if the corresponding 4x4 block in base layer uses intra prediction, the inter-layer texture prediction method is used to obtain the predicted samples for the 4x4 block in the enhancement layer. Following the above approach, the predicted samples for the current CU in the enhancement layer can be obtained.

#### Inter-layer Intra mode prediction for I-Slice

##### Intra mode prediction for 2x spatial scalability

Inter-layer Intra mode prediction applies to I-Slice in the enhancement layer when the width and height of enhancement layer video pictures are exactly twice of the width and height of the base layer video pictures (in short, 2X spatial scalability). In 2X spatial scalability case, for a CU in the base layer with top-left coordinate (x1, y1) and bottom-right coordinate (x2, y2), its corresponding CU, collocated spatially and temporally,  in the enhancement layer for 2X case has top-left coordinate (2*x1, 2*y1) and bottom-right coordinate (2*x2, 2*y2).

For a base layer CU of size 32x32, 16x16 or 8x8 using intra prediction and 2Nx2N PU partition, the corresponding enhancement CU of size 64x64, 32x32 and 16x16 also uses intra prediction and reuse the Luma intra mode and Chroma intra mode of its base layer CU.

For a base layer CU of size 8x8 using intra prediction and NxN PU partition, the corresponding block in the enhancement layer becomes four CUs of size 8x8 with PU partition of 2Nx2N. Each of the four CUs reuses the Luma intra mode and the Chroma intra mode of the corresponding 4x4 PU in the base layer.

##### RQT mapping when Intra mode prediction is used

When a base layer CU uses 2Nx2N PU partition and its size is 16x16 or 8x8, its TU partition, i.e., its RQT, is reused in the enhancement layer CU.

When a base layer CU uses NxN PU partition and its size is 8x8, its RQT has four sub-trees. As mentioned in 2.3.6.5.1, the corresponding block in the enhancement layer becomes four CUs. Each of the four CUs in the enhancement layer reuses the corresponding sub-tree among the four sub-trees.

When the size of a base layer CU is 32x32 and the depth of its RQT is 0, the depth of RQT in the corresponding enhancement CU is set to 1 since 64x64 TU is not allowed in HEVC.

When the size of a base layer CU is 32x32 and the depth of its RQT is greater than or equal to 1, its RQT is reused in the corresponding enhancement layer CU.

## In-loop filtering

### De-blocking filter

#### Inter-layer texture prediction

The de-blocking filter operation is only applied to CU boundaries with filter strength as 2.

#### Inter-layer mode prediction

The de-blocking filter operation is not only applied to CU boundaries but also applied to internal 8x8 block within a CU.

# Syntax and semantics description

This section provides a summary of all syntax and semantics changes resulted from the proposed spatial scalable coding tools.

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_unit_type | u(6) |
| layer_id_partd | u(2) |
| layer_id_partq | u(2) |
| nuh_reserved_one_2bit | u(2) |
| nuh_temporal_id_plus1 | u(3) |
| } |  |

layer_id_partd part of the layer ID

layer_id_partq part of the layer ID

layerID is derived from layer_id_partd and layer_id_partq by (layer_id_partd << 2) + layer_id_partq.

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enable_flag ) { |  |
| cu_transquant_bypass_flag | ae(v) |
| } |  |
| if( slice_type != I ¦ ¦ (layerID > 0 && (slice_type == I) ) ) |  |
| skip_flag[ x0 ][ y0 ] | ae(v) |
| if( skip_flag[x0][y0] ) { |  |
| if( slice_type != I && (layerID > 0) ) skip_flag_interlayer_texture_prediction[x0][y0] |  |
| if(skip_flag_interlayer_text_prediction[x0][y0] != 1) |  |
| prediction_unit( x0, y0, log2CbSize ) |  |
| } else { |  |
| nCbS = ( 1 << log2CbSize ) |  |
| if( layerID > 0 ) |  |
| interlayer_texture_prediction_flag |  |
| if( !interlayer_texture_prediction_flag ) { |  |
| if( layerID > 0 && (slice_type != I ¦¦ is2xSpatialScalability) ) |  |
| interlayer_mode_prediction_flag |  |
| if( !interlayer_mode_prediction_flag ) { |  |
| pred_mode_flag |  |
| if( PredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ log2CbSize = = Log2MinCbSize ) |  |
| part_mode | ae(v) |
| if( PredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && log2CbSize >= Log2MinIPCMCUSize && log2CbSize <= Log2MaxIPCMCUSize ) |  |
| pcm_flag | ae(v) |
| if( pcm_flag ) { |  |
| num_subsequent_pcm | tu(3) |
| NumPCMBlock = num_subsequent_pcm + 1 |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( x0, y0, log2CbSize ) |  |
| } else { |  |
| pbOffset = ( PartMode = = PART_NxN ) ? ( nCbS / 2 ) : 0 |  |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) { |  |
| prev_intra_luma_pred_flag[ x0 + i ][ y0+ j ] | ae(v) |
| } |  |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) { |  |
| if( prev_intra_luma_pred_flag[ x0 + i ][ y0+ j ] ) |  |
| mpm_idx[ x0 + i ][ y0+ j ] | ae(v) |
| Else |  |
| rem_intra_luma_pred_mode[ x0 + i ][ y0+ j ] | ae(v) |
| } |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { |  |
| if( PartMode = = PART_2Nx2N ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else if( PartMode = = PART_2NxN ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS, nCbS / 2 ) |  |
| } else if( PartMode = = PART_Nx2N ) { |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS ) |  |
| } else if( PartMode = = PART_2NxnU ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 4 ), nCbS, nCbS *3 / 4 ) |  |
| } else if( PartMode = = PART_2NxnD ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS *3 / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS * 3 / 4 ), nCbS, nCbS / 4 ) |  |
| } else if( PartMode = = PART_nLx2N ) { |  |
| prediction_unit( x0, y0, nCbS /4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 4 ), y0, nCbS *3 / 4, nCbS) |  |
| } else if( PartMode = = PART_nRx2N ) { |  |
| prediction_unit( x0, y0, nCbS *3 / 4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS * 3 / 4 ), y0, nCbS / 4, nCbS ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS / 2) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |
| if( !pcm_flag ) { |  |
| if( PredMode[ x0 ][ y0 ] != MODE_INTRA && !(slice_type == I && PredMode[x0][y0] == MODE_INTERLAYER_MODE_PRED) && ( !(PartMode = = PART_2Nx2N && merge_flag[x0][y0]) ¦ ¦ (PartMode = = PART_2Nx2N && PredMode[ x0 ][ y0 ] = = MODE_INTERLAYER_TEXTURE_PRED) ) ) |  |
| no_residual_syntax_flag | ae(v) |
| if( !no_residual_syntax_flag ) { |  |
| MaxTrafoDepth = ( PredMode[ x0 ][ y0 ] = = MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : max_transform_hierarchy_depth_inter ) |  |
| transform_tree( x0, y0 x0, y0, log2CbSize, 0, 0 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |

Section 2.2.2 gives a detailed description of the new syntax element skip_flag_interlayer_texture_prediction, and the extended semantics on skip_flag and no_residual_syntax_flag.

interlayer_texture_prediction_flag[ x0 ][ y0 ] equal to 1 specifies that the enhancement layer CU is encoded in the mode that employs the up-sampled base layer reconstruction as the prediction.

The default value of interlayer_texture_prediction_flag is 0.

interlayer_mode_prediction_flag[ x0 ][ y0 ] equal to 1 specifies that the enhancement layer inter CU is encoded by employing the correspoding base layer CU information, including prediction direction, reference index, and MVs for P/B Slice and Luma intra mode, Chroma intra mode and RQT for I slice.

is2xSpatialScalability is a variable, which is set to 1 in an enhancement layer when the width and height of the enhancement layer video pictures are exactly twice of the width and height of its base layer video pictures (in short, 2X spatial scalability). Otherwise, it is set to 0.

# Compression performance discussion

## Category 1 (HEVC base layer)

### Spatial scalability

The coding performance of this proposal is shown in the tables below. Complete test results are available in the separate Excel file, JCTVC-K0046-test-results.xls. All of the test results presented in this contribution came from the simulations that were carried out strictly using the requirements and test conditions given in the CfP document. In average, the proposed HEVC scalable solution can achieve 27.5% BD rate saving when comparing the enhancement layer only bitstreams vs the simulcast HEVC SVC CfP Category 1 (HEVC base layer) anchor bitstreams, and 18.2% BD rate saving when comparing the combined scalable base and enhancement layer bitstreams vs. the same set of simulcast HEVC anchor bitstreams.

Table 4.1.1.1 BD rate improvement of enhancement layer only (actual rate) vs simulcast HEVC anchor bitstreams

|  | Random Access HEVC 2x |  |  | Random Access HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -30.2% | -19.8% | -20.2% |  |  |  |
| Class B | -24.7% | -17.9% | -16.0% | -45.1% | -38.1% | -36.4% |
| Overall | -27.4% | -18.9% | -18.1% | -45.1% | -38.1% | -36.4% |
|  | -27.5% | -19.0% | -18.2% | -45.3% | -38.2% | -36.7% |

Table 4.1.1.2 BD rate improvement of enhancement layer only (target rate) vs simulcast HEVC anchor bitstreams

|  | Random Access HEVC 2x |  |  | Random Access HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -30.0% | -19.6% | -20.0% |  |  |  |
| Class B | -24.6% | -17.8% | -15.9% | -45.0% | -38.0% | -36.3% |
| Overall | -27.3% | -18.7% | -18.0% | -45.0% | -38.0% | -36.3% |
|  | -27.3% | -18.8% | -18.1% | -45.2% | -38.1% | -36.6% |

Table 4.1.1.3 BD rate improvement of combined base and enhancement layers (actual rate) vs simulcast HEVC anchor bitstreams

|  | Random Access HEVC 2x |  |  | Random Access HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -20.1% | -11.3% | -11.6% |  |  |  |
| Class B | -16.2% | -10.4% | -8.8% | -26.4% | -20.1% | -18.8% |
| Overall | -18.2% | -10.8% | -10.2% | -26.4% | -20.1% | -18.8% |
|  | -18.2% | -10.9% | -10.2% | -26.4% | -20.0% | -18.7% |

Table 4.1.1.3 BD rate improvement of combined base and enhancement layers (target rate) vs simulcast HEVC anchor bitstreams

|  | Random Access HEVC 2x |  |  | Random Access HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -20.0% | -11.2% | -11.5% |  |  |  |
| Class B | -16.1% | -10.3% | -8.7% | -26.3% | -20.0% | -18.7% |
| Overall | -18.1% | -10.7% | -10.1% | -26.3% | -20.0% | -18.7% |
|  | -18.1% | -10.8% | -10.1% | -26.3% | -20.0% | -18.7% |

# Complexity analysis

## Encoding time and measurement methodology

Though the computers in the cluster described in Section 5.3 are of the same type, the encoding time statistics may not be accurate because of many factors such as the work load status of a computer when encoding a sequence. So the encoding time provided can only be used as a rough estimation.

The encoding of low resolution and high resolution sequences (simulcast encoding) were run on the same cluster. Encoding time reported in the encoder log files are used here.

In the encoder log files from the proposed scalable encoder, only the total time of encoding the base layer and enhancement layer sequences are reported. So it cannot be compared with the high resolution encoding time directly. Alternatively, we subtract the low resolution encoding time from the total encoding time, as a rough estimation of the enhancement layer encoding time. Please also keep in mind that the QP used for coding high resolution sequences in simulcast may be different from the QP used for coding enhancement layer in scalable video coding, so the comparison is far from accurate.

According to the above described measurement methodology, the encoding time of the enhancement layers is on average xxx% of the high resolution sequences. Please refer to the attached spreadsheet for the detailed data.

## Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM

Though the machines in the cluster described in Section 5.3 are of the same type, the timing statistics may not be accurate because of many factors such as the work load status of a machine when decoding a sequence. So the decoding time provided can only be used as a rough estimation.

The decoding of low resolution and high resolution sequences (simulcast decoding) were run on the same cluster. Decoding time reported in the decoder log files are used here.

In the decoder log files from the proposed scalable decoder, only the total time of decoding the base layer and enhancement layer sequences are reported. So it cannot be compared with the high resolution decoding time directly. Alternatively, we subtract the low resolution decoding time from the total decoding time, as a rough estimation of the enhancement layer decoding time. Please also keep in mind that the QP for coding the high resolution sequences in simulcast may be different from the QP for coding the enhancement layer sequences in scalable video coding, so the comparison is far from accurate.

According to the above described measurement methodology, the encoding time of the enhancement layers is on average xxx% of the high resolution sequences. Please refer to the attached spreadsheet for the detailed data.

## Description of computing platform used to determine encoding and decoding times reported in sections 5.1 and 5.2

The simulations, both encoding and decoding were run on a computer cluster that comprises multiple machines of the same type. The information of each machine is listed below.

- Processor: Interl(R) Xeon(R) CPU X5650 @ 2.67 GHz (2 processors, each has 6 cores)

- Installed memory (RAM): 22 GB

- Operation system: Windows Server 2008R2 Enterprise (64-bit)

All the tasks were distributed automatically to the cluster. Each machine would run at most 12 tasks simultaneously.

## Expected memory usage of encoder

The up-sampling filter is in 1/6 pixel precision, ignoring padding, the memory requirement for the storage of the up-sampled base layer picture is 36 times of that of the original base layer picture, and each up-sample pixel is stored in a “short”.

Because the base layer motion information is not compressed, the motion information memory requirement in the encoder also requires more memory than the HM-6.1.1 encoder. For a scaling factor of 4, the motion information memory requirement is 16 times that of the HM-6.1.1 base layer.

## Expected memory usage of decoder

The same as the encoder side.

## Complexity characteristics of encoder motion estimation and partitioning selection in enhancement layer(s)

The same as the base layer. No special motion estimation and partitioning selection is introduced in the enhancement layer.

## Complexity characteristics of decoder motion compensation in enhancement layer(s)

For the proposed texture prediction and texture prediction skip techniques, the base layer reconstruction is up-sampled and used as the prediction for the enhancement layer. Fetching the up-sampled base layer reconstructed samples can be seen as a kind of motion compensation. The partition size for this "motion compensation" is always 2Nx2N.

For the proposed mode prediction technique, the motion information is derived from the base layer, and the motion compensation is based on 4x4 blocks.

For the proposed MERGE extension in the enhancement layer, the first merge candidate is derived from the base layer following the same rules as the proposed mode prediction technique. So the motion compensation is also based on 4x4 block.

## Complexity characteristics of encoder intra-frame prediction type and partitioning selection in enhancement layer(s)

The same as the base layer. No special encoder side intra-frame prediction type and partitioning selection are introduced in the enhancement layer.

## Complexity characteristics of decoder intra-frame prediction operation in enhancement layer(s)

The same as the base layer. No special decoder side intra-frame prediction type and partitioning selection are introduced in the enhancement layer.

## Complexity characteristics of upsampling filters and transforms specific in enhancement layer(s)

Please refer to the algorithm description part on up-sampling filter in Section 2.

## Complexity characteristics of quantization and inverse quantization in enhancement layer(s)

The same as the base layer. No special quantization and inverse quantization are introduced in the enhancement layer.

## Complexity characteristics of encoder entropy coding operation in enhancement layer(s)

The same as the base layer. No special q encoder entropy coding operation is introduced in the enhancement layer.

## Complexity characteristics of decoder entropy decoding operation in enhancement layer(s)

The same as the base layer. No special decoder entropy decoding operation is introduced in the enhancement layer.

## Degree of capability for encoder parallel processing

The same as the base layer. No extra encoder parallel processing tool is introduced in the enhancement layer.

## Degree of capability for decoder parallel processing

The same as the base layer. No extra parallel processing tool is introduced in the enhancement layer.

# Software implementation description

HM6.1.1 comprises several bugfix compared with HM6.1, while the generated bitstreams are not changed. So HM6.1.1 was chosen as the base for the implementation of the proposed scalable coding codec. Guidelines in JCTVC-H1001 are followed in the software implementation.

At the encoder side, there is no built in down convert process, so both low resolution sequence and the high resolution sequence are input to the encoder. For each picture, the encoder firstly encodes the low resolution picture and then encodes the high resolution picture. For possibly different configurations for encoding the base layer and the enhancement layer, a bunch of parameters can be set separately for base layer and enhancement layer. These parameters include the coding tools enabling and setting parameters in the HE10 configuration in HM6.1.1. In the simulation, however, these HE10 coding tools are disabled, for easy evaluation of the proposed scalable coding tools described in Section 2.

A bitstream extractor is also implemented to extract the base layer bitstream from the layered bitstreams generated by the encoder. The extracted base layer bitstreams have been confirmed to be the same as those obtained from the ftp site.

At the decoder side, the layered bitstream is input to the decoder. For each picture, the decoder firstly decodes the low resolution picture and then decodes the high resolution picture.

# Closing remarks

This contribution presents a spatial scalable video coding solution for HEVC with detailed algorithm descriptions, syntax and semantics changes, performance results, and complexity analysis. As shown in Section 4, in average this solution can achieve 27.5% BD rate saving when comparing the enhancement layer only bitstreams vs the simulcast HEVC Category 1 (HEVC base layer) anchor bitstreams, and 18.2% BD rate saving when comparing the combined scalable base and enhancement layer bitstreams vs. the same set of simulcast HEVC anchor bitstreams.

We suggest the proposed spatial scalable coding tools be included in the new HM-SVC reference software for further study and testing.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

University of Science and Technology of China may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).