| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 11th Meeting: Shanghai, CN, 10-19 Oct., 2012 | Document: JCTVC-K0046_r1 |
| --- | --- |

| Title: | Description of scalable video coding technology proposal by Huawei Technologies |  |  |
| --- | --- | --- | --- |
| Status: | [Input Document to JCT-VC] |  |  |
| Purpose: | [Proposal] |  |  |
| Author(s) or Contact(s): | Haitao Yang Jinwen Zan Xianghui Wei Wen Gao Haoping Yu Huawei Technologies Co., Ltd Li Li Jinlei Zhang Bin Li Houqiang Li Univ. of Science and Technology of China Lei Feng Univ. of Southern California | Email: haitao.yang@huawei.com Email: jinwen.zan@huawei.com Email: weixianghui@huawei.com Email: wen.gao@huawei.com Email: haoping.yu@huawei.com Email: lilimao@mail.ustc.edu.cn Email: jlzhang@mail.ustc.edu.cn Email: yhlibin@mail.ustc.edu.cn Email: lihq@ustc.edu.cn Email: leif@usc.edu |  |
| Source: | [Huawei, USTC] |  |  |

_____________________________

# Abstract

This proposal is Huawei’s response to the Call for Proposals (CfP) on scalable video coding extensions of High Efficiency Video Coding (HEVC), jointly issued by ITU-T SG16 Q.6 (VCEG) and ISO/IEC JTC1/SC29/WG11 (MPEG). This proposal presents a scalable video coding solution with a set of coding tools exploiting inter-layer redundancies for efficient compression of the enhancement layer. The coding tools include inter-layer texture prediction, inter-layer texture skip, inter-layer motion copy, scalable extension of MERGE and AMVP, and inter-layer intra mode copy. These tools are developed jointly by Huawei and University of Science and Technology of China. In average, the proposed HEVC scalable solution can achieve 27.4% and 45.1% enhancement layer BD rate saving for Category 1 - 2x spatial scalability case and Category 1 - 1.5x spatial scalability case, respectively. When taking into account the base layer bitrate, the BD rate savings are 18.2% and 26.4% for the abovementioned two spatial scalability cases respectively.

# Contents

Abstract	1

Contents	2

1	Introduction	4

2	Algorithm description	4

2.1	CTU/TU/PU partitioning	4

2.2	Up-sampling filters	4

2.3	Inter-layer prediction	5

2.3.1	Inter-layer prediction of CU/TU/PU partitions	5

2.3.2	Inter-layer residual prediction	5

2.3.3	Inter-layer texture prediction	5

2.3.4	Inter-layer texture skip	6

2.3.5	Inter-layer motion copy/prediction	7

2.3.6	Inter-layer intra mode copy	11

2.4	Inter-frame prediction	12

2.5	Intra-frame prediction	12

2.6	In-loop filtering	12

2.6.1	De-blocking filter	12

2.6.2	SAO	12

2.7	Spatial transforms	12

2.8	Quantization	12

2.9	Motion/mode/parameter coding	13

2.9.1	Merge/skip	13

2.9.2	Motion vector coding	13

2.9.3	Intra prediction mode coding	13

2.9.4	SAO parameter coding	13

2.9.5	Other	13

2.10	Entropy coding of residual / transform coefficients	13

2.11	High-layer syntax	13

3	Syntax and semantics description	14

3.1.1	NAL unit syntax	14

3.1.2	Coding unit syntax	15

3.1.3	Transform tree syntax	16

3.1.4	NAL unit semantics	16

3.1.5	Coding unit semantics	16

4	Compression performance discussion	17

4.1	Category 1 (HEVC base layer)	17

4.1.1	Spatial scalability	17

5	Complexity analysis	18

5.1	Encoding time and measurement methodology	18

5.2	Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM	18

5.3	Description of computing platform used to determine encoding and decoding times reported in sections 5.1 and 5.2	19

5.4	Expected memory usage of encoder	19

5.5	Expected memory usage of decoder	19

5.6	Complexity characteristics of encoder motion estimation and partitioning selection in enhancement layer(s)	19

5.7	Complexity characteristics of decoder motion compensation in enhancement layer(s)	19

5.8	Complexity characteristics of encoder intra-frame prediction type and partitioning selection in enhancement layer(s)	20

5.9	Complexity characteristics of decoder intra-frame prediction operation in enhancement layer(s)	20

5.10	Complexity characteristics of upsampling filters and transforms specific in enhancement layer(s)	20

5.11	Complexity characteristics of quantization and inverse quantization in enhancement layer(s)	20

5.12	Complexity characteristics of encoder entropy coding operation in enhancement layer(s)	20

5.13	Complexity characteristics of decoder entropy decoding operation in enhancement layer(s)	20

5.14	Degree of capability for encoder parallel processing	20

5.15	Degree of capability for decoder parallel processing	20

6	Software implementation description	21

7	Closing remarks	21

8	Patent rights declaration(s)	21

# Introduction

The proposed HEVC scalable coding solution was developed in response to the joint Call for Proposals (CfP) on scalable video coding extensions of High Efficiency Video Coding (HEVC) issued by ITU-T SG16 Q.6 (VCEG) and ISO/IEC JTC1/SC29/WG11 (MPEG). A scalable coding framework was implemented on HM6.1.1, which can handle the encoding and decoding of multiple layers. A portion of coding parameters can be set separately for different layers. The temporal prediction structure of the enhancement layer is the same as the base layer.

On top of this framework, a set of coding tools were integrated to improve the compression efficiency of the enhancement layer by exploiting inter-layer redundancies. The integrated tools include inter-layer texture prediction, inter-layer texture skip, inter-layer motion copy, scalable extension of MERGE and AMVP, and inter-layer intra mode copy. The detailed descriptions of these coding tools as well as the up-sampling filters used are given in Section 2. Section 3 provides syntax and semantics changes resulted from these new tools. The coding performance discussion is given in Section 4, and all of the test results presented in this contribution came from the simulations that were carried out strictly using the requirements and test conditions given in the CfP document. Complexity analysis is given in Section 5, which is followed by Section 6 Software Implementation Description. As in Closing Remarks, we suggest the scalable coding framework as well as the set of coding tools be used as the starting point for further investigation of HEVC scalable extension.

Note that the same configuration as for the base layer, MAIN configuration for HM6.1 is used for coding the enhancement layer. In other words, all HE10 coding tools including AMP, NSQT, ALF, 10-bit coding, and LM Chroma are disabled in the enhancement layer.

The category covered by this proposal is marked by :

 	Category 1 (HEVC base layer) spatial scalability

 	Category 1 (HEVC base layer) intra-only spatial scalability

 	Category 1 (HEVC base layer) SNR scalability

 	Category 2 (AVC base layer) spatial scalability

 	The proposal obeys the constraints under section 5 of the CfP (if box is not ticked, explain cases where constraints are violated)

# Algorithm description

## CTU/TU/PU partitioning

The same as the HM6.1 MAIN profile.

## Up-sampling filters

DCT based interpolation method, DCT-IF, has been adopted in the current draft specification of HEVC. In this proposal, the same idea is used in the design of the inter-layer up-sampling filters, and the filter coefficients are calculated based on the DCT and inverse DCT transforms defined in (1) and (2) below.

- Ck=1Ml=−M+1Mp(l)cos((2l−1+2M)kπ4M)						(1)

p(α)=C02+k=12M−1Ckcos(π(2α−1+2M)k4M)						(2)

where M=4 and 2 for luma and chroma, respectively; and  α= i/6, i=0, 1, 2,…5.

In particular, 1/6 sub-pixel sample step is used in up-sampling both luma and chroma components in base-layer. Each luma filter has 8 taps, and each chroma filter has 4 taps. The precision of all the up-sampling filter coefficients is 6-bit. All filter coefficients are given in Table 1 and 2.

Table 1　Up-sampling filter coefficients for luma

| α | 8-tap |
| --- | --- |
| integer | { 0, 0, 0, 64, 0, 0, 0, 0 } |
| 1/6 | { −1, 3, −7, 61, 12, −5, 2, −1 } |
| 2/6 | { −1, 4, −11, 52, 26, −8, 3, −1 } |
| 3/6 | { −1, 4, −11, 40, 40, −11, 4, −1 } |
| 4/6 | { −1, 3, −8, 26, 52, −11, 4, −1 } |
| 5/6 | { −1, 2, −5, 12, 61, −7, 3, −1 } |

Table 2　Up-sampling filter coefficients for chroma

| α | 4-tap |
| --- | --- |
| integer | { 0, 64, 0, 0} |
| 1/6 | { −4, 59, 11, −2} |
| 2/6 | { −5, 50, 22, −3} |
| 3/6 | { −4, 36, 36, −4} |
| 4/6 | { −3, 22, 50, −5} |
| 5/6 | { −2, 11, 59, −4} |

In the implementation, horizontal up-sampling is performed first, which is followed by vertical up-sampling. So the base-layer picture is firstly up-sampled by a factor of 6 in both horizontal and vertical direction. Then the pixels at locations of Round(x×6r+0.5),Round(y×6r+0.5) are picked out as the output, where x and y are the coordinates of the pixel in the up-sampled picture which has the same resolution as the enhancement layer picture, r is the up-sampling ratio. In this proposal, r is either 2 or 3/2 for 2x or 1.5x spatial scalability cases, respectively.

To reduce computation complexity and memory usage, the up-sampling process alternatively can only be carried out at the pixel locations in the enhancement layer, i.e. Round(x×6r+0.5),Round(y×6r+0.5) Note that it is just an implementation issue, not affecting the coding performance.

## Inter-layer prediction

### Inter-layer prediction of CU/TU/PU partitions

Inter-layer prediction of CU/PU partitions are not used in this proposal. Inter-layer prediction of TU partitions is described in section 2.3.5.2.

### Inter-layer residual prediction

Not used in this proposal.

### Inter-layer texture prediction

By using the up-sampling filtering technique described above, the reconstructed base layer, after up-sampling, is used as the prediction of the enhancement layer CU. This coding mode, called the inter-layer texture prediction mode, applies to CUs in both inter and intra pictures in the enhancement layer, with partition size 2Nx2N. The residual coding of CUs using this mode is the same as ordinary inter mode in the current HEVC design. A flag, named inter_layer_texture_prediction_flag is inserted after the skip flag to indicate whether this mode is selected.

### Inter-layer texture skip

It is observed that the up-sampled base layer reconstructed picture exhibits rather good quality in some cases. For example, the base layer QP is lower than the enhancement layer QP and the high resolution picture itself doesn’t have too much high frequency signal. In such a case, the inter-layer texture prediction mode described in section 2.3.3 will be chosen, and the transform coefficient will be quantized to 0 because of the small residual for a large number of CUs. To address such a case, an additional coding mode, inter-layer texture skip mode is proposed and implemented.

The proposed inter-layer texture skip mode is applied to CUs in both intra and inter slices in the enhancement layer. A flag is used to indicate the usage of this mode. For CUs using this mode, neither prediction information nor residual syntax elements are written into the bitstream. So it is similar to the skip mode in the current HEVC design, except its prediction is from the base layer reconstructed texture.

As mentioned above, the proposed inter-layer texture skip mode can be applied to CUs in both intra and inter slices. But the signaling method for the two cases is different, as described in section 2.3.4.1 and section 2.3.4.2. Besides, when using the inter-layer texture skip mode and the inter-layer texture prediction mode simultaneously the no_residue_syntax_flag in the latter can be inferred, which is described in section 2.3.4.3.

Details of the syntax and semantics of this coding mode can be found in section 3.

#### Inter-layer texture skip for CUs in I slice

In the current HEVC design, the syntax element skip_flag is signaled at the beginning of a CU in P/B slices, indicating whether the skip mode is used. This SE is not present in I slices, because the ordinary intra prediction in general results in non-zero in most cases. In this proposal, we reuse the skip_flag in I slices in the enhancement layer to indicate whether the proposed Inter-layer texture skip is used or not.

When the skip_flag is equal to 1 for a CU in an enhancement layer I slice, no more syntax elements need to be parsed from the bitstream, and the up-sampled base layer texture is used as the reconstruction of the CU; otherwise when skip_flag is equal to 0, the current CU is encoded either through intra prediction or the PCM mode, and more syntax elements are necessary for the reconstruction.

#### Inter-layer texture skip for CUs in P/B slice

In the case of enhancement layer P/B slice, the skip_flag is used to indicate the usage of the skip mode, so it cannot be used to signal the proposed Inter-layer texture skip mode. Besides, the proposed inter-layer texture skip mode needs to be differentiated from the ordinary skip mode. For this purpose, an addition flag inter_layer_texture_skip_flag is added right after the skip_flag. Only when all the conditions listed below are true, this added flag is parsed.

- Enhancement layer

- P/B slice

- skip_flag is equal to 1

If the inter_layer_texture_skip_flag is equal to 1, the up-sampled base layer texture will be used as the reconstruction of the current CU; otherwise, the reconstruction process of the ordinary skip mode as defined in HEVC will be performed.

#### Remove the redundancy of the inter-layer texture prediction mode and the inter-layer texture skip mode

For the two proposed coding modes, the inter-layer texture prediction mode and the inter-layer texture skip mode, the prediction part is exactly the same. The difference is in that there is no residual for the inter-layer texture skip mode. So a CU choosing the inter-layer texture prediction mode instead of the inter-layer texture skip mode must have residual. In consequence, there is no need to signal the syntax element no_residue_data_flag for the inter-layer texture prediction mode, since it can be inferred to be 1.

### Inter-layer motion copy/prediction

Since the content of the base layer picture and the enhancement layer picture is the same though the resolution is different, the motion field of the two should be the same except for a constant scaling factor. However, the RD optimized motion information of the base layer and that of the enhancement layer may be different, because different coding parameters such as QP are used. But there is still a lot of similarity in the motion information between different layers. For some regions where the similarity is high, the base layer motion information can be directly used by the enhancement layer blocks. For some regions where the similarity is not that high, the base layer motion information may still be a good predictor for the enhancement layer blocks.

In order to fully exploit the similarity in the motion information between the base layer and the enhancement layer described above, three coding tools are proposed. The three tools are inter-layer motion copy mode, scalable extension of MERGE, and scalable extension of AMVP, and are described in detail in section 2.3.5.3, section 2.3.5.4 and section 2.3.5.5, respectively.

The inter-layer motion copy mode and the scalable extension of MERGE use the same method to retrieve motion information from the base layer, which is described in section 2.3.5.2, and the motion compensation is also the same. The difference is in the signaling of the two tools.

#### Inter-layer correspondence

In section 2.3.5.2, SPU is used as the basic unit to retrieve motion information from the base layer. So in this section, the method of identifying the collocated SPU in the base layer for an SPU in the enhancement layer is described.

For a position  in an enhancement layer, the corresponding position in the base layer  can be determined by (3) and (4),

(3)

(4)

where  and  is the scaling factor in horizontal and vertical directions, and are calculated through,

(5)

(6)

where  and  are the width and height of the enhancement layer picture;  and  are the width and height of the base layer picture.

The SPU containing the position  is determined as the collocated base layer SPU of the enhancement layer SPU containing the position.

#### Retrieving motion information from base-layer

Smallest Prediction Unit (SPU) is used as the basic unit to retrieve motion information from the base layer. It is set to 4x4 blocks in this proposal.

After proper scaling, the motion information of the collocated base layer SPU is used as the motion information of the enhancement layer SPU. The motion information include inter prediction direction, motion vector(s) and its reference index. The scaling of the motion vector is described in (7) and (8),

(7)

(8)

where  is the motion vector after scaling and  is the motion vector of the collocated base layer SPU..

Not all the SPUs in a CU can get valid motion information from the base layer. For example, if the collocated base layer SPU is in an intra coded CU, the enhancement layer SPU cannot get valid motion information. In such a case, a recursive padding method is proposed to derive the motion information for these SPUs. Shown in Fig. 1 is a CU containing 16 SPUs. The index of SPU is the depth first zigzag order. This CU is taken as an example to illustrate the recursive padding method.

Fig.1 An example of the processing unit (marked by blue square) 
and its four partitions (marked by green square)

Firstly, every four neighboring SPUs are grouped together as the current processing unit for the padding. For example, the SPU 0, 1, 2, and 3 are grouped together for processing, as shown in Fig. 1. In case one of the SPUs has no motion information, it will get the motion information from its neighboring SPUs within this group. It will check the three neighbors in the order of the horizontal neighbor, the vertical neighbor and the diagonal neighbor. If any of the three neighbors has motion information, the motion information is used as the motion information of the current SPU. For example, if SPU 3 has no motion information, it will firstly check SPU 2, and then SPU 1, and finally SPU 0, to see if any of them has motion information, and use the motion information as its motion information. For the current SPU with index, the index of the horizontal neighbor, the index of the vertical neighbor, and the index of the diagonal neighbor can be determined by (9), (10) and (11), respectively.

(9)

(10)

(1)

where  is the number of SPUs in the current processing unit divided by four. For example, N is equal to 1 if the current processing unit comprises 4 SPUs.

The same method can be used for padding the vacant motion information in other processing units. After the first round padding at the base level, for each processing unit, either all the four SPUs in it have motion information or all the four SPUs have no information.

Fig.2 Another example of the processing unit (marked by blue square) 
and its four partitions (marked by green square)

Then the second round padding will be started at a higher level. Specifically, every four processing units in the lower level form a new processing unit in the higher level. As shown in Fig. 2, the processing unit comprises four partitions, and each partition is a processing unit in the lower level. At this higher level, exactly the same method and the same equations can be used for motion information padding of each partition. For example, if the second partition (comprises SPU 4, 5, 6, and 7) has no motion information, it will check its neighbors in horizontal, vertical and diagonal direction to see if any of its neighbor has motion information, and use that motion information as its own motion information. Note that the motion information of the upper left SPU in a partition is considered as the representative motion information of the partition. The benefit is that equation (9), (10) and (11) can be used without modification to the calculation of the SPU index in a neighbor partition.

If the CU is composed of 16 SPUs as shown in Fig. 2, the padding process is terminated. In case the CU is bigger, the same padding process is performed recursively until the size of the processing unit is the same as the size of the CU.

After the above recursive padding process described above, either all the SPUs in a CU have valid motion information, or they have no motion information. In the latter case, zero motion information is constructed and used for the CU. For P slices where forward prediction is performed, motion vector (0,0) and reference index 0 are set for forward reference picture list. For B slices where bi-directional prediction is performed, motion vectors (0,0) and reference index 0 are set for both forward and backward reference picture list.

After the recursive padding process and the zero padding described above, all SPUs in a CU have valid information.

#### Inter-layer motion copy mode

The proposed inter-layer motion copy mode is applied to CUs in enhancement layer P/B slices. The motion information of a CU is derived by the method described in section 2.3.5.2. Then the motion compensation is performed separately for each SPU in the CU. The residual coding part is the same as the ordinary inter coded CUs in the base layer. A flag is put into the CU header, right after the flag for the inter-layer texture prediction mode, to indicate whether this mode is used. For details of the signaling of this mode, please refer to the syntax and semantics in section 3.

#### Scalable extension of MERGE

Using the method described in section 2.3.5.2, the motion information of all the SPUs in a CU can be derived, and it is treated as a whole – the base layer merge candidate. Other merge candidates are derived using the same method as the base layer. All merge candidates are illustrated in Fig. 3. The base layer merge candidate is the only new candidate, compared with the base layer coding.

Fig.3 Possible MERGE candidates

Considering the base layer merge candidate, the construction of the merge candidate list is illustrated in the right column of Table 3. The merge candidate list construction in HM6.1 is also shown in the left column of Table 3 for comparison.

Table 3 MERGE candidate list construction

| HM6.1 | Proposed |
| --- | --- |
| - A1 - B1 - B0 - A0 - if(N < 4), B2 - Col - first pruning - if(N < Max.), Combined bi-pred. candidates - if(N < Max.), Zero MV candidates | - Base layer merge candidate - A1 - B1 - B0 - if(N < 4), A0 - if(N < 4), B2 - Col - first pruning - if(N < Max.), Combined bi-pred. candidates - if(N < Max.), Zero MV candidates |

The max number of the merge candidate is set to 5. The binarization of the merge index is the same as HM6.1. The context modeling of the first bin indicating the usage of the base layer merge candidate is similar to that for the skip mode flag, while the context modeling for other bins is the same as HM6.1.

#### Scalable extension of AMVP

After the motion information of all the SPUs in a CU having been retrieved in section 2.3.5.2, the motion information of the center SPU is used to derive the base layer AMVP candidate. Specifically, the motion vector is scaled according to the POC difference, in the same way as the scaling operation on the temporal collocated motion vector for the derivation of the temporal AMVP candidate in HM6.1.

Given the base layer AMVP candidate, the AMVP candidate list is constructed as shown in the right column of Table 4. In the left column of Table 4, the construction method in HM6.1 is given for comparison.

Table 4 AMVP candidate list construction

| HM6.1 | Proposed |
| --- | --- |
| - A - B - Col - first pruning - if(N < Max.), Zero MV candidates | - Motion vector from base layer - A - if(N < Max.), B - Col - first pruning - if(N < Max.), Zero MV candidates |

### Inter-layer intra mode copy

The proposed intra mode copy is an additional coding mode for CUs in enhancement layer I slices. This new coding mode comprises two parts, inter-layer intra mode copy and RQT mapping, which are described in section 2.3.6.1 and section 2.3.6.2, respectively.

#### Intra mode copy

The proposed intra mode copy is currently implemented only for 2x spatial scalability (the width and height of enhancement layer video pictures are exactly twice of the width and height of the base layer video pictures). For a CU in the base layer with top-left coordinate (x1, y1) and bottom-right coordinate (x2, y2), its corresponding CU, collocated spatially and temporally, in the enhancement layer for 2X case has top-left coordinate (2*x1, 2*y1) and bottom-right coordinate (2*x2, 2*y2).

For a base layer CU of size 32x32, 16x16 or 8x8 using intra prediction and 2Nx2N PU partition, the encoder RDO process can decide whether the corresponding enhancement CU of size 64x64, 32x32 and 16x16 use the proposed inter-layer intra mode copy mode. With this mode, the enhancement layer CU reuses the luma intra mode and chroma intra mode of its corresponding base layer CU.

For a base layer CU of size 8x8 using intra prediction and NxN PU partition, the corresponding block in the enhancement layer becomes four CUs of size 8x8 with PU partition of 2Nx2N. The encoder RDO process can decide whether each of the four CUs uses the proposed inter-layer intra mode copy mode, separately. With this mode, the enhancement layer CU reuses the luma intra mode and the chroma intra mode of the corresponding 4x4 PU in the base layer.

Under the conditions described above, the proposed inter-layer intra mode copy mode is evaluated at the encoder. For clarity, the conditions are summarized in Table 5.

Table 5 Conditions for using the proposed inter-layer intra mode copy

| Enhancement layer CU size | Condition on the corresponding base layer CU |  |
| --- | --- | --- |
|  | CU size | Partition mode |
| 64x64 | 32x32 | 2Nx2N |
| 32x32 | 16x16 | 2Nx2N |
| 16x16 | 8x8 | 2Nx2N |
| 8x8 | 8x8 | NxN |

The flag signaling this mode could be sent for CUs meeting the conditions in Table 5. However, doing so would introduce parsing dependency between the base layer and the enhancement layer. To remove such a parsing dependency, the flag is sent for every CU in the enhancement layer.

#### RQT mapping when intra mode copy is used

When a base layer CU uses 2Nx2N PU partition and its size is 16x16 or 8x8, its TU partition, i.e., its RQT, is reused in the enhancement layer CU.

When a base layer CU uses NxN PU partition and its size is 8x8, its RQT has four sub-trees. As mentioned in Error: Reference source not found, the corresponding block in the enhancement layer becomes four CUs. Each of the four CUs in the enhancement layer reuses the sub-tree of the corresponding PU partition.

When the size of a base layer CU is 32x32 and the depth of its RQT is 0, the depth of RQT in the corresponding enhancement CU is set to 1 since 64x64 TU is not allowed in HEVC.

When the size of a base layer CU is 32x32 and the depth of its RQT is greater than or equal to 1, its RQT is reused in the corresponding enhancement layer CU.

## Inter-frame prediction

The same as HM6.1.

## Intra-frame prediction

The same as HM6.1.

## In-loop filtering

### De-blocking filter

For the proposed coding tools, the boundary decision and boundary strength are changed accordingly. The other parts of the deblocking filter are the same as HM6.1. The proposed coding tools that require deblocking filter modifications are inter-layer texture prediction, inter-layer texture skip, inter-layer motion copy, scalable extension of MERGE. The modifications are described in Table 6.

Table 6 Deblocking modifications for the proposal coding tools

| Coding tool | Boundary decision | BS decision |
| --- | --- | --- |
| Inter-layer texture prediction | No PU boundary | 2 |
| Inter-layer texture skip | No PU boundary | 2 |
| Inter-layer motion copy | SPU boundary | The same as HM6.1 |
| Scalable extension of MERGE | SPU boundary | The same as HM6.1 |

### SAO

The same as HM6.1.

## Spatial transforms

The same as HM6.1.

## Quantization

The same as HM6.1.

## Motion/mode/parameter coding

Described in section 2.3.4.3.

### Merge/skip

Described in section 2.3.4.5.

### Motion vector coding

Described in section 2.3.4.5.

### Intra prediction mode coding

Described in section 2.3.5.1.

### SAO parameter coding

The same as HM6.1.

### Other

The same as HM6.1.

## Entropy coding of residual / transform coefficients

The same as HM6.1.

## High-layer syntax

The reserved bits in the NALU header are used to distinguish different layers in the bitstream. Details are provided in section 3.

# Syntax and semantics description

### NAL unit syntax

| nal_unit( NumBytesInNALunit ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_ref_flag | u(1) |
| nal_unit_type | u(6) |
| NumBytesInRBSP = 0 |  |
| temporal_id | u(3) |
| layer_id_partd | u(2) |
| layer_id_partq | u(2) |
| reserved_one_1bits | u(1) |
| for( i = nalUnitHeaderBytes; i < NumBytesInNALunit; i++ ) { |  |
| if( i + 2 < NumBytesInNALunit && next_bits( 24 ) = = 0x000003 ) { |  |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| i += 2 |  |
| emulation_prevention_three_byte /* equal to 0x03 */ | f(8) |
| } else |  |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| } |  |
| } |  |

### Coding unit syntax

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| CurrCbAddrTS = MinCbAddrZS[ x0 >> Log2MinCbSize ][ y0 >> Log2MinCbSize ] |  |
| if( slice_type != I ¦ ¦ (layerID > 0 && (slice_type == I) ) ) |  |
| skip_flag[ x0 ][ y0 ] | ae(v) |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| if( slice_type != I && (layerID > 0) ) |  |
| inter_layer_texture_skip_flag[x0][y0] | ae(v) |
| if( !inter_layer_texture_skip_flag[x0][y0] ) | ` |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| } |  |
| else { |  |
| if( layerID > 0 ) { |  |
| inter_layer_texture_pred_flag | ae(v) |
| if( !inter_layer_texture_pred_flag ) |  |
| inter_layer_copy_flag | ae(v) |
| } |  |
| if( !inter_layer_texture_pred_flag && !inter_layer_copy_flag ) { |  |
| if( slice_type != I ) |  |
| part_mode_flag | ae(v) |
| if( PredMode != MODE_INTRA ¦ ¦ log2CbSize = = Log2MinCbSize ) |  |
| part_mode | ae(v) |
| x1 = x0 + ( ( 1 << log2CbSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2CbSize ) >> 1 ) |  |
| x2 = x1 − ( ( 1 << log2CbSize ) >> 2 ) |  |
| y2 = y1 − ( ( 1 << log2CbSize ) >> 2 ) |  |
| x3 = x1 + ( ( 1 << log2CbSize ) >> 2 ) |  |
| y3 = y1 + ( ( 1 << log2CbSize ) >> 2 ) |  |
| if( PartMode = = PART_2Nx2N ) |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| else if( PartMode = = PART_2NxN ) { |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x0, y1 , log2CbSize ) |  |
| } else if( PartMode = = PART_Nx2N ) { |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x1, y0 , log2CbSize ) |  |
| } else if( PartMode = = PART_2NxnU ) { |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x0, y2 , log2CbSize ) |  |
| } else if( PartMode = = PART_2NxnD ) { |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x0, y3 , log2CbSize ) |  |
| } else if( PartMode = = PART_nLx2N ) { |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x2, y0 , log2CbSize ) |  |
| } else if( PartMode = = PART_nRx2N ) { |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x3, y0 , log2CbSize ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x1, y0 , log2CbSize ) |  |
| prediction_unit( x0, y1 , log2CbSize ) |  |
| prediction_unit( x1, y1 , log2CbSize ) |  |
| } |  |
| } |  |
| if( !pcm_flag ) |  |
| transform_tree( x0, y0, x0, y0, log2CbSize, log2CbSize, log2CbSize, 0, 0 ) |  |
| } |  |
| } |  |

### Transform tree syntax

| transform_tree( x0, y0, xC, yC, log2CbSize, log2TrafoWidth, log2TrafoHeight, trafoDepth, blkIdx ) { | Descriptor |
| --- | --- |
| if( trafoDepth = = 0 && IntraSplitFlag = = 0 && PredMode != MODE_INTRA && !(slice_type == I && PredMode == INTER_LAYER_PRED_MODE) && (!(PartMode = = PART_2Nx2N && merge_flag[x0][y0]) ¦ ¦ (PartMode == PART_2Nx2N && PredMode == INTER_LAYER_TEXTURE_PRED) ) ) |  |
| no_residual_data_flag | ae(v) |
| … |  |
| } |  |

### NAL unit semantics

layer_id_partd specifies the ID of the dependency layer, and is in the range of 0..3, inclusive.

layer_id_partq specifies the ID of the quality layer, and is in the range of 0..3, inclusive.

layerID is derived from layer_id_partd and layer_id_partq by (layer_id_partd << 2) + layer_id_partq.

### Coding unit semantics

skip_flag[ x0 ][ y0 ] equal to 1 specifies that for the current coding unit

- when decoding an enhancement layer I slice, no more syntax elements are parsed after skip_flag[ x0 ][ y0 ].

- when decoding a base layer P or B slice, no more syntax elements except the merging candidate index merge_idx[ x0 ][ y0 ] are parsed after skip_flag[ x0 ][ y0 ].

- when decoding an enhancement layer P or B slice, inter_layer_texture_skip_flag[ x0 ][ y0 ] is parsed after skip_flag[ x0 ][ y0 ].

skip_flag[ x0 ][ y0 ] equal to 0 specifies that the coding unit is not skipped. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When skip_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

inter_layer_texture_skip_flag[ x0 ][ y0 ] equal to 1 specifies that for the current coding unit, no more syntax elements are parsed after inter_layer_texture_skip_flag[ x0 ][ y0 ]. inter_layer_texture_skip_flag[ x0 ][ y0 ] equal to 0 specifies that for the current coding unit, no more syntax elements except the merging candidate index merge_idx[ x0 ][ y0 ] are parsed after inter_layer_texture_skip_flag[ x0 ][ y0 ].

When inter_layer_texture_skip_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

inter_layer_texture_pred_flag[ x0 ][ y0 ] equal to 1 specifies that the up-sampled base layer reconstruction is used as the prediction for the enhancement layer coding unit.

The default value of interlayer_texture_prediction_flag is 0.

inter_layer_copy_flag[ x0 ][ y0 ] equal to 1 specifies that the enhancement information are copied from base layer. For coding units in I slices, the base layer intra mode information and residual quadtree information are copied. For P/B slices, the base layer motion information are copied.

# Compression performance discussion

## Category 1 (HEVC base layer)

### Spatial scalability

The coding performance of this proposal is shown in the tables below. Complete test results are available in the separate Excel file, JCTVC-K0046-test-results.xls. Only the spatial scalability case in HEVC SVC CfP Category 1 (HEVC base layer) is tested. All of the test results presented in this contribution came from the simulations that were carried out strictly using the requirements and test conditions given in the CfP document.

Considering only the enhancement layer bitrate, the proposed HEVC scalable solution can achieve 27.1% and 45.1% BD rate saving on average in case of 2x and 1.5x spatial scalability, when comparing with single layer coding of the high resolution sequences. Taking into account the base layer bitrate, the BD rate savings are 18.2% and 26.4% in case of 2x and 1.5x spatial scalability, respectively.

Table 7 BD rate improvement of enhancement layer only (actual rate) vs. simulcast HEVC anchor bitstreams

|  | Random Access HEVC 2x |  |  | Random Access HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -30.2% | -19.8% | -20.2% |  |  |  |
| Class B | -24.7% | -17.9% | -16.0% | -45.1% | -38.1% | -36.4% |
| Overall | -27.4% | -18.9% | -18.1% | -45.1% | -38.1% | -36.4% |
|  | -27.5% | -19.0% | -18.2% | -45.3% | -38.2% | -36.7% |

Table 8 BD rate improvement of enhancement layer only (target rate) vs. simulcast HEVC anchor bitstreams

|  | Random Access HEVC 2x |  |  | Random Access HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -30.0% | -19.6% | -20.0% |  |  |  |
| Class B | -24.6% | -17.8% | -15.9% | -45.0% | -38.0% | -36.3% |
| Overall | -27.3% | -18.7% | -18.0% | -45.0% | -38.0% | -36.3% |
|  | -27.3% | -18.8% | -18.1% | -45.2% | -38.1% | -36.6% |

Table 9 BD rate improvement of combined base and enhancement layers (actual rate) vs. simulcast HEVC anchor bitstreams

|  | Random Access HEVC 2x |  |  | Random Access HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -20.1% | -11.3% | -11.6% |  |  |  |
| Class B | -16.2% | -10.4% | -8.8% | -26.4% | -20.1% | -18.8% |
| Overall | -18.2% | -10.8% | -10.2% | -26.4% | -20.1% | -18.8% |
|  | -18.2% | -10.9% | -10.2% | -26.4% | -20.0% | -18.7% |

Table 10 BD rate improvement of combined base and enhancement layers (target rate) vs. simulcast HEVC anchor bitstreams

|  | Random Access HEVC 2x |  |  | Random Access HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -20.0% | -11.2% | -11.5% |  |  |  |
| Class B | -16.1% | -10.3% | -8.7% | -26.3% | -20.0% | -18.7% |
| Overall | -18.1% | -10.7% | -10.1% | -26.3% | -20.0% | -18.7% |
|  | -18.1% | -10.8% | -10.1% | -26.3% | -20.0% | -18.7% |

# Complexity analysis

## Encoding time and measurement methodology

Though the computers in the cluster described in Section 5.3 are of the same type, the encoding time statistics may not be accurate because of many factors such as the work load status of a computer when encoding a sequence. So the encoding time provided can only be used as a rough estimation.

The encoding of low resolution and high resolution sequences (simulcast encoding) were run on the same cluster. Encoding time reported in the encoder log files are used here for comparison.

In the encoder log files from the proposed scalable encoder, only the total time of encoding the base layer and enhancement layer sequences are reported. So it cannot be compared with the high resolution encoding time directly. Alternatively, we subtract the low resolution encoding time from the total encoding time, as a rough estimation of the enhancement layer encoding time. Please also keep in mind that the QP used for coding high resolution sequences in simulcast may be different from the QP used for coding enhancement layer in scalable video coding, so the comparison is far from accurate.

According to the above described measurement methodology, the encoding time of the enhancement layers is on average 112% and 111% of the encoding time of the high resolution sequences, for 2x scalability and 1.5x scalability. Please refer to the attached Excel file JCTVC-K0046-encoding&decoding-time.xls for the detailed data.

## Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM

Though the machines in the cluster described in Section 5.3 are of the same type, the decoding time statistics may not be accurate because of many factors such as the work load status of a machine when decoding a sequence. So the decoding time provided can only be used as a rough estimation.

The decoding of low resolution and high resolution sequences (simulcast decoding) were run on the same cluster. Decoding time reported in the decoder log files are used here for comparison.

In the decoder log files from the proposed scalable decoder, only the total time of decoding the base layer and enhancement layer sequences are reported. So it cannot be compared with the high resolution decoding time directly. Alternatively, we subtract the low resolution decoding time from the total decoding time, as a rough estimation of the enhancement layer decoding time. Please also keep in mind that the QP for coding the high resolution sequences in simulcast may be different from the QP for coding the enhancement layer sequences in scalable video coding, so the comparison is far from accurate.

According to the above described measurement methodology, the decoding time of the enhancement layers is on average 326% and 465% of the decoding time of the high resolution sequences, for 2x scalability and 1.5x scalability. Please refer to the attached Excel file JCTVC-K0046-encoding&decoding-time.xls for the detailed data.

By further investigation of the decoding time, we found out almost all the extra decoding time compared with single layer coding comes from the up-sampling the base layer picture. And we further identified the reason why the up-sampling operation costs so much time. First, the whole base layer picture is up-sampled no matter whether it will be used or not. Second, the up-sampling filter is performed to get all the sample in the 1/6 pel precision, although only a few of them are selected to construct the up-sampling output picture. The second reason is also described in section 2.2. Here we want to reiterate that this is just an implementation issue, not the inherent problem of the proposal scalable coding solution. For example, the decoding time would be greatly reduced if the two points mentioned above are improved, in an alternative implementation of the same coding performance.

## Description of computing platform used to determine encoding and decoding times reported in sections 5.1 and 5.2

The simulations, both encoding and decoding were run on a computer cluster that comprises multiple machines of the same type. The information of each machine is listed below.

- Processor: Interl(R) Xeon(R) CPU X5650 @ 2.67 GHz (2 processors, each has 6 cores)

- Installed memory (RAM): 22 GB

- Operation system: Windows Server 2008R2 Enterprise (64-bit)

All the tasks were distributed automatically to the cluster. Each machine would run at most 12 tasks simultaneously.

## Expected memory usage of encoder

The up-sampling filter is in 1/6 pixel precision, ignoring padding, the memory requirement for the storage of the up-sampled base layer picture is 36 times of that of the original base layer picture, and each up-sample pixel is stored in a “short”. However, it is an implementation issue and not the inherent problem of the proposal scalable coding solution. The required memory could be greatly reduced if only the final output picture of the up-sampling process is stored.

Because the base layer motion information is not compressed, the motion information memory requirement in the encoder also requires more memory than the HM6.1 encoder. For a scaling factor of 4, the motion information memory requirement is 16 times that of the HM6.1 by analysis. However, in the software implementation of HM6.1, the full resolution motion information is stored, though the 16 4x4 blocks in a 16x16 block share the same motion information. So in terms of the current software implementation, using the non-compressed motion information does not require addition memory.

## Expected memory usage of decoder

The same as the encoder side.

## Complexity characteristics of encoder motion estimation and partitioning selection in enhancement layer(s)

The same as HM6.1. No special motion estimation and partitioning selection is introduced in the enhancement layer.

## Complexity characteristics of decoder motion compensation in enhancement layer(s)

For the proposed inter-layer texture prediction and inter-layer texture techniques, the base layer reconstruction is up-sampled and used as the prediction for the enhancement layer. Fetching the up-sampled base layer reconstructed samples can be seen as a kind of motion compensation. The partition size for this "motion compensation" is always 2Nx2N.

For the proposed inter-layer motion copy mode, the motion compensation is performed on each SPU. The size of SPU is set to 4x4 in this proposal.

For the proposed MERGE extension in the enhancement layer, the first merge candidate is derived from the base layer following the same rules as the proposed inter-layer motion copy mode. So the motion compensation is also performed on each SPU.

## Complexity characteristics of encoder intra-frame prediction type and partitioning selection in enhancement layer(s)

The same as HM6.1. No special encoder side intra-frame prediction type and partitioning selection are introduced in the enhancement layer.

## Complexity characteristics of decoder intra-frame prediction operation in enhancement layer(s)

The same as HM6.1. No special decoder side intra-frame prediction type and partitioning selection are introduced in the enhancement layer.

## Complexity characteristics of upsampling filters and transforms specific in enhancement layer(s)

Please refer to the algorithm description part on up-sampling filter in section 2, section 5.2 and section 5.4.

## Complexity characteristics of quantization and inverse quantization in enhancement layer(s)

The same as HM6.1. No special quantization and inverse quantization are introduced in the enhancement layer.

## Complexity characteristics of encoder entropy coding operation in enhancement layer(s)

The same as HM6.1. No special encoder entropy coding operation is introduced in the enhancement layer.

## Complexity characteristics of decoder entropy decoding operation in enhancement layer(s)

The same as HM6.1. No special decoder entropy decoding operation is introduced in the enhancement layer.

## Degree of capability for encoder parallel processing

The same as HM6.1. No extra encoder parallel processing tool is introduced in the enhancement layer.

## Degree of capability for decoder parallel processing

The same as HM6.1. No extra parallel processing tool is introduced in the enhancement layer.

# Software implementation description

HM6.1.1 comprises several bugfix compared with HM6.1, while the generated bitstreams are not changed. So HM6.1.1 was chosen as the base for the implementation of the proposed scalable coding codec. Guidelines in JCTVC-H1001 are followed in the software implementation.

At the encoder side, there is no built in down convert process, so both low resolution sequence and the high resolution sequence are input to the encoder. For each picture, the encoder firstly encodes the low resolution picture and then encodes the high resolution picture. For possibly different configurations for encoding the base layer and the enhancement layer, a bunch of parameters can be set separately for base layer and enhancement layer. These parameters include the coding tools enabling and setting parameters in the HE10 configuration in HM6.1.1. In the simulation, however, these HE10 coding tools are disabled, for easy evaluation of the proposed scalable coding tools described in Section 2.

A bitstream extractor is also implemented to extract the base layer bitstream from the layered bitstreams generated by the encoder. The extracted base layer bitstreams have been confirmed to be the same as those obtained from the ftp site.

At the decoder side, the layered bitstream is input to the decoder. For each picture, the decoder firstly decodes the low resolution picture and then decodes the high resolution picture.

# Closing remarks

This contribution presents a spatial scalable video coding solution for HEVC with detailed algorithm descriptions, syntax and semantics changes, performance results, and complexity analysis. As shown in Section 4, for 2x scalability and 1.5x scalability, in average this solution can achieve 27.1% and 45.1% BD rate saving when comparing the enhancement layer only bitstreams vs the simulcast HEVC Category 1 (HEVC base layer) anchor bitstreams, and 18.2% and 26.4% BD rate saving when comparing the combined scalable base and enhancement layer bitstreams vs. the same set of simulcast HEVC anchor bitstreams. We suggest the scalable coding framework as well as the set of coding tools be used as the starting point for further investigation of HEVC scalable extension.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

University of Science and Technology of China may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

University of Southern California doesn’t have IPR relating to the technology described in this contribution.