| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 11th Meeting: Shanghai, CN, 10-19 Oct., 2012 | Document: JCTVC-K0040 |
| --- | --- |

| Title: | Description of scalable video coding technology proposal by Nokia (encoder configuration 2) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Oguz Bici, Done Bugdayci, Alireza Aminlou, Antti Hallapuro, Miska Hannuksela, Jani Lainema, Kemal Ugur Visiokatu 3, Tampere Finland | Email: | ext-oguz.bici@nokia.com; ext-done.bugdayci@nokia.com; ext-alireza.aminlou@nokia.com; miska.hannuksela@nokia.com; antti.hallapuro@nokia.com; jani.lainema@nokia.com; kemal.ugur@nokia.com; |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This document presents the details of the Nokia scalable video coding extension to HEVC technology (NSHEVC).

The main goal of the NSHEVC codec is to achieve scalability with high coding efficiency but with as small changes to HEVC design as possible. In order to achieve this, following HEVC tools are modified so that base layer information could be utilized to improve coding efficiency. Those tools could be summarized as:

- Enhancing the temporal and spatial prediction using base layer samples.

- Block based inter-layer prediction of samples.

- Using base layer intra prediction mode and motion vectors in coding the enhancement layer.

NSHEVC software will be made available to interested parties to enable studying of developed algorithms in more detail.

This contribution allows utilizing a different GOP structure for enhancement layer than the base layer GOP structure (the details of the enhancement layer GOP structure is explained in Section 2.4.2). An additional contribution is also submitted (JCTVC-K0040) using the same GOP length as the base layer for encoding the enhancement layer. Other than this change in encoding structure, both proposals have identical tools and encoder configurations.

The encoding times are updated in the second version of this document with accurate encoding time comparisons with the anchor.

# Contents

Abstract	1

Contents	1

1	Introduction	3

2	Algorithm description	3

2.1	CTU/TU/PU partitioning	3

2.2	Up-sampling filters	4

2.3	Inter-layer prediction	4

2.3.1	Inter-layer prediction of CU/TU/PU partitions	4

2.3.2	Inter-layer residual prediction	4

2.3.3	Inter-layer motion vector prediction	4

2.3.4	Inter-layer mode prediction	4

2.4	Inter-frame prediction	5

2.4.1	Base-layer enhanced motion compensated prediction (BEMCP)	5

2.4.2	Coding structure of enhancement layer	5

2.5	Intra-frame prediction	5

2.5.1	Mode-dependent enhancement of intra prediction	6

2.6	In-loop filtering	6

2.6.1	De-blocking filter	6

2.6.2	SAO	7

2.6.3	Other	7

2.7	Spatial transforms	7

2.8	Quantization	7

2.9	Motion/mode/parameter coding	7

2.9.1	Merge/skip and AMVP	7

2.9.2	Motion vector coding	7

2.9.3	Intra prediction mode coding	8

2.9.4	SAO parameter coding	8

2.10	Entropy coding of residual / transform coefficients	8

2.11	High-layer syntax	8

2.11.1	Layer identifier and layer dependencies	8

2.11.2	Picture marking and decoded picture buffering	8

2.11.3	Support for uneven GOP lengths between layers	9

3	Syntax and semantics description	9

4	Compression performance discussion	9

4.1	Category 1 (HEVC base layer)	9

4.1.1	Spatial scalability	9

4.1.2	Intra-only spatial scalability	9

5	Complexity analysis	10

5.1	Encoding time and measurement methodology	10

5.2	Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM	10

5.3	Description of computing platform used to determine encoding and decoding times reported in sections 5.1 and 5.2	10

5.4	Expected memory usage of encoder	10

5.5	Expected memory usage of decoder	10

5.6	Complexity characteristics of encoder motion estimation and partitioning selection in enhancement layer(s)	10

5.7	Complexity characteristics of decoder motion compensation in enhancement layer(s)	11

5.8	Complexity characteristics of encoder intra-frame prediction type and partitioning selection in enhancement layer(s)	11

5.9	Complexity characteristics of decoder intra-frame prediction operation in enhancement layer(s)	11

5.10	Complexity characteristics of upsampling filters and transforms specific in enhancement layer(s)	11

5.11	Complexity characteristics of quantization and inverse quantization in enhancement layer(s)	11

5.12	Complexity characteristics of encoder entropy coding operation in enhancement layer(s)	11

5.13	Complexity characteristics of decoder entropy decoding operation in enhancement layer(s)	11

5.14	Degree of capability for encoder parallel processing	11

5.15	Degree of capability for decoder parallel processing	11

6	Software implementation description	12

7	Highlighted aspects discussion	12

8	Closing remarks	12

9	Patent rights declaration(s)	12

# Introduction

Indicate the category/categories the proposal covers (tick box )

 	Category 1 (HEVC base layer) spatial scalability

	Category 1 (HEVC base layer) intra-only spatial scalability

 	Category 1 (HEVC base layer) SNR scalability

 	Category 2 (AVC base layer) spatial scalability

 	The proposal obeys the constraints under section 5 of the CfP (if box is not ticked, explain cases where constraints are violated)

The main goal of the NSHEVC codec is to achieve scalability with high coding efficiency but with as small changes to HEVC design as possible. This is achieved by introducing information from base layer to enhancement layer by making small modifications to some of the HEVC tools. Following are the list of tools included in the NSHEVC codec:

- Enhancing the temporal and spatial prediction using base layer samples.

- Block based inter-layer prediction of samples.

- Using base layer intra prediction mode and motion vectors in coding the enhancement layer.

# Algorithm description

Below is a brief description of the tools that were changed from HEVC, so that base layer information could be used. For more detailed description, please refer to the appendix where the syntax and semantics for those tools are provided.

## CTU/TU/PU partitioning

Same partitioning structure as HEVC is used. In addition, NSHEVC allows block based prediction of sample values from base layer in PU granularity. This is done by using a new syntax element, called pred_base_flag (which is entropy coded similar to skip_flag). If pred_base_flag is 1, the associated prediction unit performs the prediction by copying the co-located reconstructed samples from the upsampled base layer. pred_base_flag is used in the PU level by both I slices and inter slices (B and P slices). However, the signaling and utilization of the flag is different for intra and inter slices.  This is done to be able to exploit the rectangular prediction block granularity in inter PU’s maximally. Below figure shows the possible prediction modes allowed by this design:

(a)

Example CU in an intra slice

Intra PUs could be further enhanced by base layer

(b)

Example CUs in an inter slice

Inter PUs could be further enhanced from base layer using BEMCP

Figure 1 PU based sample prediction from base

Intra slices

The intra_skip_flag is present for the largest CU size in Intra slices. In the case the flag is set to 1, no other syntax elements are sent for the CU, the CU is fully predicted using the upsampled base layer and the residual decoding process is omitted. In the case the flag has a value of 0, parsing continues with the remaining syntax elements.

The pred_base_flag is placed before any intra prediction information such as luma/chroma intra direction mode. When the flag is 1, no other intra prediction syntaxes are then parsed and corresponding luma and chroma prediction blocks are predicted using the upsampled base layer. When the flag is 0, rest of the intra prediction information is parsed as usual. It should be noted that the intra prediction of the enhancement layer could be further enhanced by base layer samples using techniques presented in Section 2.6.

Inter slices

The flag is not parsed for intra PUs in an inter slice and it is always inferred to be zero. For an inter PU, pred_base_flag is parsed before any of the syntax elements in the PU. When the flag is 1, then no other syntax is parsed for the corresponding PU and corresponding samples of luma and chroma prediction blocks are predicted using the upsampled base layer picture. When the flag is 0, rest of the syntaxes are parsed as usual. For a skip CU, the pred_base_flag indicates whether the PU in the skip CU is a merge mode or the prediction from base layer mode. In addition, to remove redundancy parsing of no_residual_syntax_flag is modified.

It should be noted that the inter prediction could still be enhanced using the BEMCP technique described in Section 2.5.

cbf_luma and cbf_cb

The contexts of cbf_luma and cbf_cb flags depend on the value of pred_base_flag. More specifically, the new contexts are used if the top-left corner of the block whose cbf flag is being coded is within a PU whose pred_base_flag is equal to one, i.e. the PB predicted from the upsampled base layer.

## Up-sampling filters

Following upsampling filters are used in NSHEVC

1.5x upsampling:

To obtain the samples at sub-pixel position 1/3 in base layer, following filter taps are used (for 2/3 mirrored version of those taps are utilized)

Luma: -1,   4, -11, 52, 25, -7, 2, 0

Chroma: -5, 49, 24, -4

2x upsampling:

To obtain the samples at sub-pixel position 1/2 in base layer, following filter taps are used:

Luma: -1, 4, -11, 40, 40, -11, 4, -1

Chroma:  -4, 36, 36, -4

## Inter-layer prediction

### Inter-layer prediction of CU/TU/PU partitions

The CU/TU/PU partitions are not predicted from base layer.

### Inter-layer residual prediction

The residual in enhancement layer is not predicted from base layer.

## Inter-layer motion vector prediction

The coding of enhancement layer motion vectors are improved using base layer motion vectors by modifying the AMVP and merge modes. These are further explained in Section 2.10.1.

### Inter-layer mode prediction

The intra mode coding in enhancement layer is improved using the base layer intra modes. The techniques are further explained in Section 2.10.3.

## Inter-frame prediction

The inter-frame prediction process in NSHEVC is the same as in HEVC, but the inter prediction could be enhanced by using the reconstructed base layer samples. It should be noted that, unlike HM6.1, Asymmetric Partitions (AMP) mode is enabled while coding the enhancement layer.

### Base-layer enhanced motion compensated prediction (BEMCP)

The base-layer enhanced motion compensated prediction (BEMCP) is a tool improving the enhancement layer temporal prediction by adding a base layer component to the prediction. The base layer component approximates the new information appearing to (or disappearing from) the corresponding block in the base layer. The base layer component is derived by performing motion compensation on the base layer with the motion information identical to the enhancement layer and deducting the result from the reconstructed base layer signal. The codec supports adding no, full or half of the base layer component to the enhancement layer prediction – the first approach corresponding to the traditional motion compensated prediction. The reconstruction process for the three modes can be given as:

P0(x,y) = P’(x,y)

P1(x,y) = Clip ( P’(x,y) + B(x,y) - B’(x,y) )

P2(x,y) = Clip ( P’(x,y) + ( ( B(x,y) - B’(x,y) ) >> 1 ) )

where P’(x,y) corresponds a predicted pixel from enhancement layer motion compensation, B’(x,y) a predicted pixel utilizing enhancement layer motion information but upsampled base layer reference frames and B(x,y) the upsampled reconstructed base layer block co-locating with the enhancement layer block. The figure below illustrates the process for creating the BEMCP prediction with full base layer component added to the prediction.

### Coding structure of enhancement layer

We provide two sets of results with different GOP structures. This contribution (JCTVC-K0047) presents results using a longer GOP structure to code enhancement layer than the GOP structure of base layer, whereas contribution JCTVC-K0040 presents results of using the same GOP length for coding enhancement and base layers.

The GOP length of enhancement layer is selected as follows: For sequences having a dyadic intra period (32,64), GOP length of 32 is chosen. For sequences with non-dyadic intra period (24,48), same GOP length as the corresponding intra period is chosen. For those non-dyadic cases GOPs are created by concatenating two dyadic hierarchies (i.e. for 24, GOP structure is combination of 16 and 8). The encoder configurations for GOP lengths 24, 32 and 48 are attached for further study.

## Intra-frame prediction

The intra-frame prediction process in NSHEVC is the same as in HEVC, but the intra prediction could be enhanced using reconstructed base layer samples. This is done as follows:

### Mode-dependent enhancement of intra prediction

The intra prediction in enhancement layer could be enhanced using the sample values from base layer. This enhancement is different for different prediction modes and block sizes.

Figure 2 Enhancement of intra prediction from base layer

Let’s use the following notation:

P(x,y): denotes the sample to be predicted in the enhancement layer picture.

PR(xR,yR): denotes the enhancement layer reference sample in the enhancement layer picture based on the directionality of the intra prediction mode

B(x’,y’): in the base layer picture with location corresponding to the enhancement layer sample P(x,y)

Gradient based enhancement: For horizontal and vertical modes, intra prediction is enhanced by measuring how the base layer pixel values change in the corresponding direction and applying the generated differential values when reconstructing prediction samples for the enhancement layer video. This process is illustrated in Figure 3 for a 4x4 block using vertical prediction.

Figure 3 Gradient based enhancement of intra prediction for vertical prediction

DC correction:

DC correction is performed for those intra PUs which have a size of 8x8 or larger and are not utilizing the 0-angle horizontal and vertical modes. This mode adds a constant value to each sample of the enhancement layer block so that the DC of the enhancement layer prediction equals to the DC of the co-located base layer block. In other words, a dc_delta is calculated by taking the difference between enhancement layer block DC and corresponding base layer block DC, and added to all the samples of the enhancement layer block.

## In-loop filtering

### De-blocking filter

Same de-blocking filter as in HM6.0 is used.

### SAO

Same SAO as in HM6.0 is used.

### Other

No in-loop filters other than SAO and de-blocking filter are used.

## Spatial transforms

Same transforms as in HM6.0 are used.

## Quantization

Same quantization as in HM6.0 is used.

## Motion/mode/parameter coding

### Merge/skip and AMVP

Motion vectors in the base layer are quite correlated with the motion information in the enhancement layer as those are describing the movement of the same object. In order to exploit this correlation, AMVP and merge mode processes in HEVC are modified to utilize the base layer motion vector predictor (BMVP). BMVP is formed by scaling the motion vector from the co-located block in the base layer according the POC differences and the scalability factor. The position of base layer block, from which the BMVP is formed, is calculated by scaling the center coordinates and size of the enhancement layer block using scalability factor. The specific changes to AMVP and merge mode processes are given as:

AMVP mode

If available, the co-located motion vector in the base layer (BMVP) is placed as the first candidate in the AMVP list. In addition, base layer MVP does not participate in pruning process to reduce the complexity of list construction.

Merge mode

While coding enhancement layer motion vector, the motion vector prediction candidate list of an enhancement layer block is constructed in such a way that the base layer motion vector predictor and temporal motion vector predictor are always present in the candidate list if they are both available and the list size is larger than two. This means that the base layer motion vector predictor candidate and temporal motion vector predictor candidate are treated with higher priority than the spatial motion vector predictor candidates. This is done as follows:

- Place the base layer MVP in the beginning of the list

- Select maximally 3 spatial MVP candidates.

- Place the temporal MVP to the list.

- Base layer MVP does not participate in redundancy check process to reduce complexity

It should be noted that the GOP structure for base layer and enhancement layer could be different. For those cases, the same reference index could point to a picture with different POC value at base and enhancement layer. Therefore, using reference indices from base layer as it is reduces coding efficiency especially when different GOP structures are used for base and enhancement layer. To improve the coding efficiency, we use a table that maps the reference indices of the base layer (refIdxBase) to corresponding reference indices of the enhancement layer (refIdxEnh). The table is constructed in such a way that the POC value of reference picture at index refIdxBase (POC_refIdxBase) in base reference picture list corresponds to the same POC value of reference picture at index refIdxEnh in enhancement reference picture list. If any of the enhancement reference pictures list does not contain a reference picture with the same POC value (POC_refIdxBase) for the same list, then the corresponding base layer MVP is not  placed in the merge candidate list.

### Motion vector coding

No changes to motion vector coding were made.

### Intra prediction mode coding

Similar to motion vectors, intra prediction mode of an enhancement layer block is correlated with that of co-located intra block in base layer. In order to exploit this correlation, the intra prediction mode of enhancement layer is indicated by coding the difference with respect to the mode of base layer. Since the base layer intra prediction mode information is already present in the decoder side, the actual enhancement layer intra prediction mode information can be reconstructed by adding this difference to the base layer intra prediction mode. Probability distribution of the difference values can be exploited during binarization in order to improve the coding efficiency as follows:

- First bin indicates whether the difference is equal to zero, i.e. base layer intra prediction mode is equal to enhancement layer intra prediction mode or not.

- Second bin indicates the sign of the difference (either positive or negative)

- Third bin indicates whether the absolute value of the difference is equal to 1 or not.

- Fourth bin indicates whether the absolute value of the difference is equal to 2 or not.

- Since the difference values ranges from -34 to 34 and values 0, +-1 and +-2 are covered by the previous bins and the sign of the difference value is indicated by the second bin, there are a total of 32 absolute values that the difference can take. The remaining values can be coded by assigning absolute value of the difference minus 3 to 5 bins with equal probability (i.e. in bypass mode).

### SAO parameter coding

No changes to SAO parameter coding were made.

## Entropy coding of residual / transform coefficients

No changes to transform coefficient coding were made.

## High-layer syntax

### Layer identifier and layer dependencies

reserved_zero_6bits in the NAL unit header is renamed to layer_id. The semantics of layer_id values as well as the inter-layer prediction dependencies are specified in the active video parameter set.

Operation points specifying a list of decoded layers are specified in the Video Parameter Set and indexed in their order of appearance. The decoder operates at one of the listed operation points (hereafter, the target operation point). A leaf layer within an operation point is such a layer which no other layer within the operation point depends on.

inter_layer_reference_flag is included in the slice extension syntax. inter_layer_reference_flag equal to 1 specifies that the current picture may be used as reference for inter-layer prediction. inter_layer_reference_flag equal to 0 specifies that the current picture is not used as reference for inter-layer prediction.

### Picture marking and decoded picture buffering

Multi-loop decoding operation is used in NSHEVC. Consequently, decoded picture buffering and picture marking (e.g. as “used for reference”) is done independently for each layer. POC values are aligned across layers.

In addition to marking of pictures as “used for reference” and “unused for reference” for inter prediction within a layer, pictures can be marked as “used for inter-layer reference” and “unused for inter-layer reference”. A picture with LayerId equal to LID is marked as “used for inter-layer reference” immediately after its decoding if both of the following are true:

- The LayerId LID is among the layers of the target operation point and is not a leaf layer within the target operation point.

- The value of inter_layer_reference_flag is equal to 1.

Otherwise (the picture is not among the layers of the target operation point or the picture is on a leaf layer within the target operation point or the picture has inter_layer_reference_flag equal to 0), the picture is marked as “unused for inter-layer reference”.

A picture is subsequently marked as “unused for inter-layer reference”, when all pictures (of the same POC value and within the same target operation point) that use the picture as inter-layer reference (according to the dependencies provided in the active VPS) have been decoded.

The bumping process of the DPB is modified to remove a picture from the DPB when it is marked as “unused for reference”, “unused for inter-layer reference” and “not needed for output”.

### Support for uneven GOP lengths between layers

In NSHEVC, the GOP length (i.e. the number of pictures in a repetitive inter prediction hierarchy) need not be the same across layers. As a consequence, an access unit needs not contain all “component pictures” of the same output time, but rather component pictures are ordered in a dependency order that takes both inter and inter-layer prediction into account. The above-described inter-layer marking process is used to keep pictures required for inter-layer prediction long enough in the decoded picture buffer.

When the GOP size of an enhancement layer is twice the GOP size of the base layer, a typical decoding order is such where (GOP size + 1) base layer frames are consecutive and precede the respective enhancement layer frames, after which the base and enhancement layer pictures appear in conventional interleaved order. Such ordering of frames causes an increase in the frame buffers needed for storing base layer frames. For example, for base layer GOP size 8, the number of frames kept as inter prediction references is typically 4 to 5, whereas using a duplicated GOP size in the enhancement layer would increase the required frame buffers to 9. For 2x spatial scalability, memory use would therefore increase roughly by an amount equivalent to 1 decoded enhancement layer picture.

Decoders can reduce the amount of base layer frame buffers when an alternative decoding order of the base layer is used. The following syntax elements are used to support this feature:

- Pictures that follow the current picture in the alternative decoding order, specified by their LayerId and pic_order_cnt_lsb values. Let nextPic be the next picture in decoding order that follows the current picture. The listed pictures shall precede nextPic in the alternative decoding order and there shall be no other picture than the listed pictures that follow the current picture and precede nextPic in alternative decoding order. The pictures are listed in the slice extension.

- An additional reference picture set in the slice extension of base layer coded slices. The additional RPS specifies which pictures are additionally maintained in the RPS in the alternative decoding order.

- max_dec_pic_buffering, max_num_reorder_frames, and max_latency_increase for the alternative decoding order, indicated in VPS

For example, when dyadic temporal nesting with one reference frame per list is used, a duplicated GOP size in the enhancement layer would increase the required base layer frame buffers by 1.

# Syntax and semantics description

See attached appendix for the syntax description

# Compression performance discussion

## Category 1 (HEVC base layer)

The delta-bitrate under the "EL-only actual rate" section of the CfP excel template are reported here. Please refer to attached excel sheet for the full results.

### Spatial scalability

|  | Random Access HEVC 2x |  |  | Random Access HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -36.9% | -30.7% | -33.0% |  |  |  |
| Class B | -27.8% | -24.8% | -26.1% | -47.7% | -45.9% | -46.5% |
| Overall | -32.4% | -27.8% | -29.6% | -47.7% | -45.9% | -46.5% |
|  | -32.4% | -27.9% | -29.6% | -47.8% | -46.0% | -46.5% |

### Intra-only spatial scalability

|  | All Intra HEVC 2x |  |  | All Intra HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -38.3% | -34.3% | -34.2% |  |  |  |
| Class B | -31.2% | -26.7% | -25.8% | -53.6% | -50.7% | -50.6% |
| Overall | -34.7% | -30.5% | -30.0% | -53.6% | -50.7% | -50.6% |
|  | -34.8% | -30.5% | -30.0% | -53.8% | -50.8% | -50.9% |

# Complexity analysis

## Encoding time and measurement methodology

To obtain the encoding times, a heterogeneous cluster is used, therefore the encoding times are not very reliable. The reason for that is:

- The computational loads of the cluster were not the same during the entire simulations because of other ongoing computations.

- The nodes have different computational performance.

The encoding times are compared with respect to anchor at the second lowest rate point as follows.

| High Res | All Intra (average 2X – 1.5X) |  |  | Random Access (average 2X – 1.5X) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Sequence | Anchor Enc T [s] | Proposal Enc T [s] | Ratio | Anchor Enc T [s] | Proposal Enc T [s] | Ratio |
| Traffic QP 22 | 17014.86 | 17507.91 | 1.028978 | 35219.15 | 60047.95 | 1.704980103 |
| PeopleOnStreet QP 22 | 18595.79 | 19069.38 | 1.025468 | 93432.60 | 97307.48 | 1.041472463 |
| Kimono QP 22 | 6535.49 | 6703.05 | 1.025638 | 33622.76 | 52752.64 | 1.568956267 |
| ParkScene QP 22 | 7780.54 | 7647.88 | 0.98295 | 30780.18 | 48302.76 | 1.569281271 |
| Cactus QP 22 | 16775.37 | 16339.29 | 0.974005 | 69788.38 | 57558.54 | 0.824758219 |
| BasketballDrive QP 22 | 15251.98 | 15283.92 | 1.002094 | 46392.49 | 69410.58 | 1.496159831 |
| BQTerrace QP 22 | 21327.73 | 20900.39 | 0.979963 | 48288.74 | 86730.04 | 1.796071714 |

The encoding times will be updated in the coming days by performing the encodings on the same performance nodes, where the encodings are done in isolation.

Below table represents the accurate encoding times for spatial scalability results, where the simulations are performed in similar nodes both for the proposal and the anchor. Also the attached excel sheets are updated accordingly.

| High Res | All Intra (average 2X – 1.5X) |  |  | Random Access (average 2X – 1.5X) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Sequence | Anchor Enc T [s] | Proposal Enc T [s] | Ratio | Anchor Enc T [s] | Proposal Enc T [s] | Ratio |
| Traffic QP 22 | 17014.86 | 17507.91 | 1.028978 | 35219.15 | 60047.95 | 1.70498 |
| PeopleOnStreet QP 22 | 18595.79 | 19069.38 | 1.025468 | 49497.98 | 97307.48 | 1.965888 |
| Kimono QP 22 | 6535.49 | 6703.05 | 1.025638 | 17374.61 | 30296.24 | 1.743708 |
| ParkScene QP 22 | 7780.54 | 7647.88 | 0.98295 | 15994.98 | 28063.54 | 1.754522 |
| Cactus QP 22 | 16775.37 | 16339.29 | 0.974005 | 37117.88 | 61685.13 | 1.661871 |
| BasketballDrive QP 22 | 15251.98 | 15283.92 | 1.002094 | 41780.29 | 80682.28 | 1.931109 |
| BQTerrace QP 22 | 21327.73 | 20900.39 | 0.979963 | 48288.74 | 86730.04 | 1.796072 |

## Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM

The decoding times are measured by running the NSHEVC software and the HM 6.1 on the same machine. The decoding of bitstreams is performed in a sequential manner to make the results more reliable. It should be noted that the decoding time for NSHEVC includes decoding of both the enhancement layer and the base layer anchor. The average results are shown in Section 5.1.  For more details of the decoding times, please see attached excel sheet.

The decoding times are compared with respect to anchor at the second lowest rate point as follows.

| High Res | All Intra (average 2X – 1.5X) |  |  | Random Access (average 2X – 1.5X) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Sequence | Anchor Dec T [s] | Proposal Dec T [s] | Ratio | Anchor Dec T [s] | Proposal Dec T [s] | Ratio |
| Traffic QP 22 | 185.49 | 285.66 | 1.540029112 | 73.16 | 152.78 | 2.088299617 |
| PeopleOnStreet QP 22 | 190.53 | 300.31 | 1.576182229 | 113.80 | 209.07 | 1.837170475 |
| Kimono QP 22 | 55.83 | 105.32 | 1.886440982 | 32.12 | 72.29 | 2.250622665 |
| ParkScene QP 22 | 84.07 | 132.21 | 1.572618056 | 34.42 | 75.21 | 2.185066822 |
| Cactus QP 22 | 173.74 | 274.63 | 1.580695292 | 71.41 | 153.29 | 2.146618121 |
| BasketballDrive QP 22 | 151.17 | 256.23 | 1.694979163 | 76.36 | 156.11 | 2.044394971 |
| BQTerrace QP 22 | 235.06 | 357.76 | 1.521994384 | 112.16 | 204.11 | 1.819810984 |

## Description of computing platform used to determine encoding and decoding times reported in sections 5.1 and 5.2

For the NSHEVC encoder: A heterogeneous cluster is used therefore the encoding time comparisons are not very reliable. The encoding times will be updated and run on a machine with the following specifications:

- Intel Xeon CPU with 8 cores @ 2.53 GHz 96 Gb of RAM

- 64 bit executable is used

- No multicore parallel processing will be used

For the NSHEVC decoder: The anchor and the NSHEVC decoders are run on a node with the following specifications:

- Intel Xeon CPU with 8 cores @ 2.53 GHz 96 Gb of RAM

- 64 bit executable is used

- No multicore parallel processing is used: decoding of each base layer frame is sequentially proceeded by decoding of the corresponding enhancement layer frame.

## Expected memory usage of encoder

Each enhancement layer picture need to store its temporal reference pictures. In addition, an additional base layer picture at the current POC needs to be stored for enhancing the intra prediction and additional base layer reference pictures need to be stored for enhancing the motion compensated prediction.

## Expected memory usage of decoder

Each enhancement layer picture need to store its temporal reference pictures. In addition, an additional base layer picture at the current POC needs to be stored for enhancing the intra prediction and additional base layer reference pictures need to be stored for enhancing the motion compensated prediction.

## Complexity characteristics of encoder motion estimation and partitioning selection in enhancement layer(s)

Same as in HM6.0

## Complexity characteristics of decoder motion compensation in enhancement layer(s)

Only change to decoder motion compensation is due to BEMCP tool. In that case, the decoder need to perform motion compensation in base layer as well.

## Complexity characteristics of encoder intra-frame prediction type and partitioning selection in enhancement layer(s)

Same as in HM6.0

## Complexity characteristics of decoder intra-frame prediction operation in enhancement layer(s)

Same as in HEVC as no changes were made to intra-frame prediction. However, the intra frame prediction is enhanced either by gradient based enhancement or DC correction. Their complexity characteristics can be summarized as:

Gradient based enhancement: This tool is similar to side smoothing tool in HEVC intra prediction and the additional complexity is due to the adding the delta between the co-located base layer reference pixel and the corresponding base layer reference pixel.

DC correction: The additional complexity is due to averaging the enhancement and base layer block and adding the difference to each intra-layer prediction pixel.

## Complexity characteristics of upsampling filters and transforms specific in enhancement layer(s)

Upsampling filter complexity has very similar characteristics to the HEVC interpolation filter (chroma is upsampled using 4-tap filter and luma is upsampled using an 8-tap filter).

Transform complexity is the same as no changes to HEVC were introduced.

## Complexity characteristics of quantization and inverse quantization in enhancement layer(s)

Same as in HEVC as no changes were made to quantization.

## Complexity characteristics of encoder entropy coding operation in enhancement layer(s)

Very similar as in HEVC, only changes are due to the coding of additional scalability specific syntaxes. Care was taken to not increase the complexity by i) coding as many bins in bypass mode and ii) reducing the additional contexts.

## Complexity characteristics of decoder entropy decoding operation in enhancement layer(s)

Very similar as in HEVC, only changes are due to the coding of additional scalability specific syntaxes. Care was taken to not increase the complexity by i) coding as many bins in bypass mode and ii) reducing the additional contexts.

## Degree of capability for encoder parallel processing

Similar to HEVC.

## Degree of capability for decoder parallel processing

Similar to HEVC.

# Software implementation description

Nokia Scalable HEVC (NSHEVC) is built on HM 6.1.1. For the encoding of enhancement layer, the encoder additionally uses the reconstructed base layer samples, motion field information of the base layer and intra prediction mode information of the base layer. This information is used by the encoder software using three new input parameters: 1) YUV file location of the reconstructed base layer where the frames are in display order, 2) Supplementary text file for each base layer frame that includes the motion information of the base layer. 3) Supplementary text file for each base layer frame that includes intra mode information of the base layer. Motion and intra mode information of each picture are stored in raster scan order for each square size of smallest CU size (which is 4x4 in our case).

Unlike the NSHEVC encoder, decoder does not take this information from an external file but decodes the base layer during decoding of enhancement layer. The reconstructed base layer picture, motion and intra information data are then derived and used during decoding of the enhancement layer. Base layer and enhancement layer decoding blocks are implemented in the same software package.

# Highlighted aspects discussion

NSHEVC was designed to provide scalability with minimal changes to the HEVC design as follows:

- Enhancing the temporal and spatial prediction using base layer samples.

- Block based inter-layer prediction of samples.

- Using base layer intra prediction mode and motion vectors in coding the enhancement layer.

# Closing remarks

The coding efficiency results of NSHEVC show that significant gains could be achieved with rather small changes to the HEVC design. Therefore, we propose to take a simplistic approach when deciding on the starting point of the Scalable HEVC project and believe NSHEVC software represents a good candidate for the initial starting point. The NSHEVC software will be made available to interested parties to enable studying of developed algorithms and the software in more detail.

# Patent rights declaration(s)

Nokia may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).