| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 11th Meeting: Shanghai, CN, 10-19 Oct., 2012 | Document: JCTVC-K0050 |
| --- | --- |

| Title: | Description of scalable video coding technology proposal by LG Electronics and MediaTek (differential coding mode off) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chulkeun Kim, Joonyoung Park, Jungsun Kim, Hendry, Seungwook Park, Yongjoon Jeon, Jaehyun Lim, Naeri Park, Byeongmoon Jeon LG Electronics #221 Yangjae-dong, Seocho-gu, Seoul 137-130, Korea Shan Liu, Ximin Zhang, Mei Guo, Zhenzhong Chen MediaTek USA 2860 Junction Ave, San Jose, CA 95134, USA Tzu-Der Chuang, Shih-Ta Hsiang, Ching-Yeh Chen, Chia-Yang Tsai, Chih-Ming Fu, Chih-Wei Hsu, Yu-Wen Huang, Shawmin Lei MediaTek No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Byeongmoon Jeon +82-10-82031126 {chulkeun.kim, jy.park, jungsun.kim, hendry.hendry, seungwook.park, yongjoon.jeon, jaehyun.lim, naeri.park, bm.jeon}@lge.com Shan Liu +1-408-5261899 ext. 88129 {shan.liu, ximin.zhang, mei.guo, zhenzhong.chen, peter.chuang, shih-ta.hsiang, chingyeh.chen, chiayang.tsai, chihming.fu, cw.hsu, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | LG Electronics and MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution is the LG Electronics and MediaTek response to the Joint Call for Proposals (CfP) on Scalable Video Coding Extensions of High Efficiency Video Coding (HEVC) issued by ISO/IEC JTC1/SC29/WG11 (MPEG) and ITU-T SG16 WP3 (VCEG). The goal of this proposal is to provide scalable coding technologies on top of the new HEVC draft standard. It is a duplication of JCTVC-K0033 except that one of the proposed coding tools in JCTVC-K0033, inter-layer differential coding, is disabled in this proposal. With all other tools as proposed in JCTVC-K0033 enabled, the proposed scalable video codec reportedly achieves 32%, 49%, 37%, 56%, and 41% BD-rate savings for category 1 – 2x spatial scalability (namely RA-2x, where RA means “random access”), category 1 – 1.5x spatial scalability (namely RA-1.5x), category 1 – 2x intra-only spatial scalability (namely AI-2x, where AI means “all intra”), category 1 – 1.5x intra-only spatial scalability (namely AI-1.5x), and category 1 – SNR scalability (namely RA-SNR), respectively, when compared with the CfP “EL-only actual rate” anchors. When compared with the CfP “EL+BL actual rate” anchors, the corresponding BD-rate savings are 22%, 30%, 26%, 34%, and 28%. For each test point, the actual rate is within 0.2% variation of the target rate. The average encoding time increases for the proposed scalable encoder compared with HM6.1 single layer encoder using EL spatial resolution are 39%, 60%, 75%, 91%, and 111% for RA-2x, RA-1.5x, AI-2x, AI-1.5x, and RA-SNR, respectively. The corresponding average decoding time increases are 108%, 130%, 86%, 104%, and 164%.

# Contents

Abstract	1

Contents	2

1	Introduction	3

2	Algorithm description	3

2.1	CTU/TU/PU partitioning	3

2.2	Up-sampling filters	3

2.3	Inter-layer prediction	3

2.3.1	Inter-layer texture (intra) prediction	3

2.3.2	Inter-layer motion vector prediction	3

2.3.3	Inter-layer intra mode prediction	3

2.4	Inter-frame prediction	3

2.5	Intra-frame prediction	3

2.6	In-loop filtering	3

2.7	Spatial transforms	3

2.8	Quantization	3

2.9	Motion/mode/parameter coding	4

2.10	Entropy coding of residual / transform coefficients	4

2.11	High-layer syntax	4

2.12	Internal bit-depth	4

3	Syntax and semantics description	4

4	Compression performance discussion	4

4.1	Category 1 (HEVC base layer)	4

4.1.1	Spatial scalability	4

4.1.2	Intra-only spatial scalability	5

4.1.3	SNR scalability	6

4.1.4	Overall	6

4.2	Category 2 (AVC base layer)	7

4.2.1	Spatial scalability	7

4.2.2	Overall	7

5	Complexity analysis	7

5.1	Encoding time and measurement methodology	7

5.2	Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM	7

5.3	Description of computing platform used to determine encoding and decoding times reported in Sections 5.1 and 5.2	7

5.4	Expected memory usage of encoder	8

5.5	Expected memory usage of decoder	8

5.6	Complexity characteristics of encoder motion estimation and partitioning selection in enhancement layer(s)	8

5.7	Complexity characteristics of decoder motion compensation in enhancement layer(s)	8

5.8	Complexity characteristics of encoder intra-frame prediction type and partitioning selection in enhancement layer(s)	9

5.9	Complexity characteristics of decoder intra-frame prediction operation in enhancement layer(s)	9

5.10	Complexity characteristics of up-sampling filters and transforms specific in enhancement layer(s)	9

5.11	Complexity characteristics of quantization and inverse quantization in enhancement layer(s)	9

5.12	Complexity characteristics of encoder entropy coding operation in enhancement layer(s)	9

5.13	Complexity characteristics of decoder entropy coding operation in enhancement layer(s)	9

5.14	Degree of capability for encoder parallel processing	9

5.15	Degree of capability for decoder parallel processing	9

6	Software implementation description	9

7	Highlighted aspects discussion	9

8	Closing remarks	9

9	Patent rights declaration(s)	9

References	10

# Introduction

This contribution is the response to the “Joint Call for Proposals on Scalable Video Coding Extensions of High Efficiency Video Coding (HEVC)” [1] by LG and MediaTek. The proposed coding tools in this contribution are the same as those have been described in JCTVC-K0033 [2], except that one tool, inter-layer differential coding, is turned off in this proposal.

This proposal covers the categories which are checked below.

 	Category 1 (HEVC base layer) spatial scalability

 	Category 1 (HEVC base layer) intra-only spatial scalability

 	Category 1 (HEVC base layer) SNR scalability

 	Category 2 (AVC base layer) spatial scalability

 	The proposal obeys the constraints under Section 5 of the CfP (if box is not ticked, explain cases where constraints are violated)

# Algorithm description

## CTU/TU/PU partitioning

Same as Section 2.1 in JCTVC-K0033

## Up-sampling filters

Same as Section 2.2 in JCTVC-K0033

## Inter-layer prediction

### Inter-layer texture (intra) prediction

Same as Section 2.3.1 in JCTVC-K0033

### Inter-layer motion vector prediction

Same as Section 2.3.3 in JCTVC-K0033

### Inter-layer intra mode prediction

Same as Section 2.3.4 in JCTVC-K0033

## Inter-frame prediction

Same as Section 2.4 in JCTVC-K0033

## Intra-frame prediction

Same as Section 2.5 in JCTVC-K0033

## In-loop filtering

Same as Section 2.6 in JCTVC-K0033

## Spatial transforms

Same as Section 2.7 in JCTVC-K0033

## Quantization

Same as Section 2.8 in JCTVC-K0033

## Motion/mode/parameter coding

Same as Section 2.9 in JCTVC-K0033

## Entropy coding of residual / transform coefficients

Same as section 2.10 in JCTVC-K0033

## High-layer syntax

The high-layer syntax is described in section 3 in details.

## Internal bit-depth

Same as Section 2.12 in JCTVC-K0033

# Syntax and semantics description

This section is the same as section 3 in JCTVC-K0033, with inter_layer_differential_coding_enabled_flag always set to 0 in slice header. As a consequence, CU syntax element diff_coding_flag is not coded and is inferred to 0. Hence, inter-layer differential coding is disabled.

# Compression performance discussion

## Category 1 (HEVC base layer)

### Spatial scalability

“EL-only actual rate”

|  | Random Access HEVC 2x |  |  | Random Access HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -34.3% | -26.0% | -26.3% |  |  |  |
| Class B | -29.4% | -24.6% | -22.2% | -48.9% | -43.8% | -41.6% |
| Overall | -31.8% | -25.3% | -24.2% | -48.9% | -43.8% | -41.6% |
|  | -31.9% | -25.4% | -24.3% | -49.0% | -43.8% | -41.8% |

“EL-only target rate”

|  | Random Access HEVC 2x |  |  | Random Access HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -34.1% | -25.8% | -26.1% |  |  |  |
| Class B | -29.2% | -24.4% | -22.0% | -48.7% | -43.6% | -41.5% |
| Overall | -31.6% | -25.1% | -24.0% | -48.7% | -43.6% | -41.5% |
|  | -31.6% | -25.1% | -24.1% | -48.8% | -43.7% | -41.6% |

“EL+BL actual rate”

|  | Random Access HEVC 2x |  |  | Random Access HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -23.7% | -16.6% | -16.8% |  |  |  |
| Class B | -20.3% | -16.3% | -14.1% | -29.7% | -25.1% | -23.2% |
| Overall | -22.0% | -16.4% | -15.5% | -29.7% | -25.1% | -23.2% |
|  | -22.0% | -16.4% | -15.5% | -29.7% | -25.1% | -23.2% |

“EL+BL target rate”

|  | Random Access HEVC 2x |  |  | Random Access HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -23.5% | -16.4% | -16.6% |  |  |  |
| Class B | -20.2% | -16.1% | -14.0% | -29.6% | -25.0% | -23.1% |
| Overall | -21.9% | -16.3% | -15.3% | -29.6% | -25.0% | -23.1% |
|  | -21.9% | -16.3% | -15.3% | -29.6% | -24.9% | -23.1% |

### Intra-only spatial scalability

“EL-only actual rate”

|  | All Intra HEVC 2x |  |  | All Intra HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -40.4% | -40.9% | -41.4% |  |  |  |
| Class B | -33.0% | -32.8% | -32.9% | -55.6% | -55.4% | -55.7% |
| Overall | -36.7% | -36.9% | -37.1% | -55.6% | -55.4% | -55.7% |
|  | -36.7% | -36.9% | -37.2% | -55.8% | -55.5% | -55.9% |

“EL-only target rate”

|  | All Intra HEVC 2x |  |  | All Intra HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -40.4% | -40.9% | -41.3% |  |  |  |
| Class B | -33.0% | -32.8% | -32.9% | -55.6% | -55.4% | -55.7% |
| Overall | -36.7% | -36.8% | -37.1% | -55.6% | -55.4% | -55.7% |
|  | -36.7% | -36.9% | -37.2% | -55.8% | -55.5% | -55.8% |

“EL+BL actual rate”

|  | All Intra HEVC 2x |  |  | All Intra HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -28.5% | -28.6% | -29.1% |  |  |  |
| Class B | -23.0% | -22.8% | -22.9% | -33.7% | -33.5% | -33.9% |
| Overall | -25.7% | -25.7% | -26.0% | -33.7% | -33.5% | -33.9% |
|  | -25.7% | -25.8% | -26.0% | -33.7% | -33.4% | -33.7% |

“EL+BL target rate”

|  | All Intra HEVC 2x |  |  | All Intra HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -28.4% | -28.6% | -29.0% |  |  |  |
| Class B | -23.0% | -22.8% | -22.9% | -33.7% | -33.5% | -33.9% |
| Overall | -25.7% | -25.7% | -26.0% | -33.7% | -33.5% | -33.9% |
|  | -25.7% | -25.7% | -26.0% | -33.7% | -33.4% | -33.7% |

### SNR scalability

“EL-only actual rate”

|  | Random Access HEVC SNR |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A+ | -43.4% | -35.6% | -35.3% |
| Class B | -38.6% | -32.6% | -28.5% |
| Overall | -41.0% | -34.1% | -31.9% |
|  | -41.0% | -34.1% | -31.9% |

“EL-only target rate”

|  | Random Access HEVC SNR |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A+ | -43.1% | -35.3% | -35.1% |
| Class B | -38.4% | -32.4% | -28.3% |
| Overall | -40.8% | -33.9% | -31.7% |
|  | -40.7% | -33.9% | -31.7% |

“EL+BL actual rate”

|  | Random Access HEVC SNR |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A+ | -29.6% | -23.0% | -22.7% |
| Class B | -26.2% | -21.1% | -17.6% |
| Overall | -27.9% | -22.0% | -20.2% |
|  | -27.9% | -22.0% | -20.2% |

“EL+BL target rate”

|  | Random Access HEVC SNR |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A+ | -29.4% | -22.8% | -22.6% |
| Class B | -26.0% | -20.9% | -17.4% |
| Overall | -27.7% | -21.9% | -20.0% |
|  | -27.7% | -21.9% | -20.0% |

### Overall

The proposed scalable codec achieves approximately 32%, 49%, 37%, 56%, and 41% BD-rate savings on average for category 1 – 2x spatial scalability, category 1 – 1.5x spatial scalability, category 1 – 2x intra-only spatial scalability, category 1 – 1.5x intra-only spatial scalability, and category 1 – SNR scalability, respectively, compared with CfP “EL-only rate” anchors.

The proposed scalable codec achieves approximately 22%, 30%, 26%, 34%, and 28% BD-rate savings on average for category 1 – 2x spatial scalability, category 1 – 1.5x spatial scalability, category 1 – 2x intra-only spatial scalability, category 1 – 1.5x intra-only spatial scalability, and category 1 – SNR scalability, respectively, compared with CfP “EL+BL rate” anchors.

Please note that the actual rates of all the test points are within 0.2% variation of corresponding target rates.

## Category 2 (AVC base layer)

### Spatial scalability

This proposal does not respond to Category 2 (AVC base layer).

### Overall

This proposal does not respond to Category 2 (AVC base layer).

# Complexity analysis

## Encoding time and measurement methodology

The encoding time of the anchor is the run time of encoding single layer sequences of the enhancement layer (EL) spatial resolution by using HM6.1. The encoding time of the proposed scalable video encoder is the run time of encoding both base layer (BL) and EL sequences by using the proposed encoder software.

The encoding time starts after parsing the configuration file at the beginning and ends after recording coded bits for the last picture. For the proposed encoder software, both reconstructed YUVs of BL and EL are output. Also for the anchor, the reconstructed YUVs are output.

The run times of the proposed encoder software are shown in Table 1. Compared with the HM6.1 single layer encoding, the encoding times are 139%, 160%, 175%, 191%, and 211% for category 1 – 2x spatial scalability (namely RA-2x, where RA means “random access”), category 1 – 1.5x spatial scalability (namely RA-1.5x), category 1 – 2x intra-only spatial scalability (namely AI-2x, where AI means “all intra”), category 1 – 1.5x intra-only spatial scalability (namely AI-1.5x), and category 1 – SNR scalability (namely RA-SNR), respectively.

Table 1.  The encoding time of proposed encoder software compared with HM-6.1 anchor

|  | RA-2x | RA-1.5x | AI-2x | AI-1.5x | RA-SNR |
| --- | --- | --- | --- | --- | --- |
| Encoding Time | 139% | 160% | 175% | 191% | 211% |

## Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM

Comparison with HM 6.1 should be set to “YUV output enabled” and “reference input disabled.”

The decoding time of the anchor is the run time of decoding single layer sequences of the enhancement layer (EL) spatial resolution by using HM6.1. The decoding time of the proposed scalable video decoder is the run time of decoding both base layer (BL) and EL sequences by using the proposed decoder software.

The decoding time starts after parsing bitstream at the beginning and ends after recoding the last reconstructed YUV output before releasing all memories. For the proposed decoder software, the reconstructed YUVs of EL are output. For the anchor, the reconstructed YUVs of single layer decoding (at enhancement resolution) are output.

The run times of the proposed decoder software are shown in Table 2. Compared with the HM6.1 single layer decoding, the decoding times are 208%, 230%, 186%, 204%, and 264% for category 1 – 2x spatial scalability (namely RA-2x, where RA means “random access”), category 1 – 1.5x spatial scalability (namely RA-1.5x), category 1 – 2x intra-only spatial scalability (namely AI-2x, where AI means “all intra”), category 1 – 1.5x intra-only spatial scalability (namely AI-1.5x), and category 1 – SNR scalability (namely RA-SNR), respectively.

Table 2.  The decoding time of proposed decoder software compared with HM-6.1 anchor

|  | RA-2x | RA-1.5x | AI-2x | AI-1.5x | RA-SNR |
| --- | --- | --- | --- | --- | --- |
| Decoding Time | 208% | 230% | 186% | 204% | 264% |

## Description of computing platform used to determine encoding and decoding times reported in Sections 5.1 and 5.2

Computing platforms and assigned simulations are summarized in Table 3.

Table 3.  Summary of computing platform

|  | CPU | Memory | OS | Storage |
| --- | --- | --- | --- | --- |
| Platform | Intel Xeon Quad Core 5570(2.93GHz) | 12GB DDR3 | Red Hat 3.4.6-11 | NAS (IBM N6040) |

## Expected memory usage of encoder

In the proposed encoder software, we allocate two sets of buffers for the base layer (BL) and the enhancement layer (EL) encoding. Each set of buffers includes the picture buffers for original signals, prediction signals, residual signals, reconstructed signals, and reference pictures, mode information buffers for prediction modes and motion information, residual coefficient buffer, context buffer for context-based adaptive binary arithmetic coding, sequence parameter buffer, and filter coefficient buffer for sample adaptive offset and adaptive loop filter.

The sizes of the EL buffers are equal to the buffer sizes that are used in HM6.1 single layer encoding at the EL spatial resolution. The sizes of the BL buffers are proportional to the sizes of the EL buffers; the scale factor is the ratio between BL spatial resolution and EL spatial resolution. For example, in category 1 – 2x spatial scalability, the BL buffer sizes are roughly 1/4 of the EL buffer sizes. In addition to these two buffer sets, a temporary buffer is allocated for inter-layer texture prediction in EL encoding. The buffer size of each temporary buffer is equal to the picture size of the EL.

The total memory usage of the proposed encoder software is approximately (1 + BL picture size / EL picture size) * (buffer size of HM6.1 single layer encoding at EL spatial resolution) + EL picture size.

## Expected memory usage of decoder

In the proposed decoder software, we allocate two sets of buffers for the base layer (BL) and the enhancement layer (EL) decoding. Each set of buffers includes the picture buffers for reconstructed signals and reference pictures, mode information buffers for prediction modes and motion information, residual coefficient buffer, context buffer for context-based adaptive binary arithmetic coding, sequence parameter buffer, and filter coefficient buffer for sample adaptive offset and adaptive loop filter.

The sizes of the EL buffers are equal to the buffer sizes that are used in HM6.1 single layer decoding at the EL spatial resolution. The sizes of the BL buffers are proportional to the sizes of the EL buffers; the scale factor is the ratio between BL spatial resolution and EL spatial resolution. For example, in category 1 – 2x spatial scalability, the BL buffer sizes are roughly 1/4 of the EL buffer sizes. In addition to these two buffer sets, a temporary buffer is allocated for inter-layer texture prediction in EL decoding. The buffer size of each temporary buffer is equal to the picture size of the EL.

The total memory usage of the proposed decoder software is approximately (1 + BL picture size / EL picture size) * (buffer size of HM6.1 single layer decoding at EL spatial resolution) + EL picture size.

## Complexity characteristics of encoder motion estimation and partitioning selection in enhancement layer(s)

The encoder motion estimation process in the enhancement layer (EL) is the same as that in HM6.1 except for motion vector initialization. The starting point of motion estimation is determined as one of the AMVP candidates. As described in Section 2.3.3.3 of JCTVC-K0033, the scaled motion vectors from the base layer (BL) are added into the AMVP candidate list. The more accurate the candidate from the BL, the lower the complexity of encoder motion estimation.

The encoder partitioning selection in EL is the same as that used in HM6.1.

## Complexity characteristics of decoder motion compensation in enhancement layer(s)

Same as Section 5.7 in JCTVC-K0033

## Complexity characteristics of encoder intra-frame prediction type and partitioning selection in enhancement layer(s)

Same as Section 5.8 in JCTVC-K0033

## Complexity characteristics of decoder intra-frame prediction operation in enhancement layer(s)

Same as Section 5.9 in JCTVC-K0033

## Complexity characteristics of up-sampling filters and transforms specific in enhancement layer(s)

Same as Section 5.10 in JCTVC-K0033

## Complexity characteristics of quantization and inverse quantization in enhancement layer(s)

Same as Section 5.11 in JCTVC-K0033

## Complexity characteristics of encoder entropy coding operation in enhancement layer(s)

Same as Section 5.12 in JCTVC-K0033

## Complexity characteristics of decoder entropy coding operation in enhancement layer(s)

Same as Section 5.13 in JCTVC-K0033

## Degree of capability for encoder parallel processing

Same as Section 5.15 in JCTVC-K0033

## Degree of capability for decoder parallel processing

Same as Section 5.16 in JCTVC-K0033

# Software implementation description

The proposed coding tools were implemented using standard C++ programming language. HM6.1 was used as the reference software for implementing single layer coding modules. The software is modular and suitable for further tool experiments.

# Highlighted aspects discussion

All important techniques of this proposal have been described in above sections.

# Closing remarks

The previous section have described the joint proposal by LG and MediaTek in response to the CfP on Scalable Video Coding Extensions of HEVC in various aspects, including algorithm descriptions, syntax, compression performance, and complexity analysis, etc. In summary, the proposed scalable codec achieves average 32-56% BD-rate savings for category 1 scalabilities compared with the CfP “EL-only” anchors; average 22-34% BD-rate savings for category 1 scalabilities compared with the CfP “EL+BL” anchors. We believe the proposed codec can be a good starting point for further investigations and tests on scalable video coding extensions of HEVC.

# Patent rights declaration(s)

LG Electronics may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- ISO/IEC JTC1/SC29/WG11 (MPEG) and ITU-T SG16 WP3 (VCEG), “Joint Call for Proposals on Scalable Video Coding Extensions of High Efficiency Video Coding (HEVC),” ISO/IEC JTC1/SC29/WG11 N12957, July, 2012.

- C. Kim, J. Park, J. Kim, Hendry, S. Park, Y. Jeon, J. Lim, N. Park, B. Jeon, S. Liu, X. Zhang, M. Guo, Z. Chen, T.-D. Chuang, S.-T. Hsiang, C.-Y. Chen, C.-Y. Tsai, C.-M. Fu, C.-W. Hsu, Y.-W. Huang, S. Lei, “Description of scalable video coding technology proposal by LG Electronics and MediaTek (with differential coding mode),” Document of Joint Collaborative Team on Video Coding, JCTVC-K0033, October, 2012.