| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 11th Meeting: Shanghai, CN, 10-19 Oct., 2012 | Document: JCTVC-K0052 |
| --- | --- |

| Title: | Description of scalable video coding technology proposal by KDDI |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kei Kawamura Tomonobu Yoshino Sei Naito 2-1-15 Ohara, Fujimino-shi, Saitama, JAPAN Lei Sun, Takeshi Ikenaga 2-7 Hibikino, Wakamatsu, Kitakyushu, Fukuoka, JAPAN | Tel: Email: | +81 49 278 7411 ki-kawamura@kddi.com |
| Source: | KDDI Corp. (KDDI R&D Laboratories, Inc.) and Waseda University |  |  |

_____________________________

# Abstract

This contribution is the KDDI’s response to the Joint Call for Proposals on Scalable Video Coding Extensions of High Efficiency Video Coding (HEVC), jointly issued by ITU-T SG16 Q.6 (VCEG) and ISO/IEC JTC1/SC29/WG11 (MPEG). The goal of this contribution is to provide scalable video coding extensions of HEVC which has significantly higher compression capability than the state-of-the-art H.264/AVC standard, especially for high-definition (HD) video contents. A further goal is to obtain these gains with minimal increase in complexity over HEVC. To achieve these goals, three modified tools are proposed covering several aspects of scalable video coding technology. These include inter-layer intra prediction, inter-layer prediction mode and motion prediction, and inter-layer residual prediction. When all the proposed tools are used, the proposed scalable video codec achieve approximately 28% and 46% bit-saving on average compared to HEVC single layer in both RA x2.0 and RA x1.5 configurations.

# Contents

Abstract	1

Contents	2

1	Introduction	4

2	Algorithm description	4

2.1	CTU/TU/PU partitioning	4

2.2	Up-sampling filters	4

2.3	Inter-layer prediction	4

2.3.1	Inter-layer prediction of CU/TU/PU partitions	4

2.3.2	Inter-layer residual prediction	4

2.3.3	Inter-layer motion vector prediction	4

2.3.4	Inter-layer mode prediction	4

2.3.5	Other	4

2.4	Inter-frame prediction	4

2.5	Intra-frame prediction	5

2.6	In-loop filtering	5

2.6.1	De-blocking filter	5

2.6.2	SAO	5

2.6.3	Other	5

2.7	Spatial transforms	5

2.8	Quantization	5

2.9	Motion/mode/parameter coding	5

2.9.1	Merge/skip	5

2.9.2	Motion vector coding	5

2.9.3	Intra prediction mode coding	5

2.9.4	SAO parameter coding	5

2.9.5	Other	5

2.10	Entropy coding of residual / transform coefficients	5

2.11	High-layer syntax	5

2.12	Additional algorithmic description discussion topic #1	5

2.13	Additional algorithmic description discussion topic #2	5

2.14	…	5

3	Syntax and semantics description	5

4	Compression performance discussion	6

4.1	Category 1 (HEVC base layer)	6

4.1.1	Spatial scalability	6

4.1.2	Intra-only spatial scalability	6

4.1.3	SNR scalability	6

4.1.4	Overall	6

4.2	Category 2 (AVC base layer)	6

4.2.1	Spatial scalability	6

4.2.2	Overall	6

5	Complexity analysis	6

5.1	Encoding time and measurement methodology	6

5.2	Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM	6

5.3	Description of computing platform used to determine encoding and decoding times reported in sections 5.1 and 5.2	6

5.4	Expected memory usage of encoder	6

5.5	Expected memory usage of decoder	7

5.6	Complexity characteristics of encoder motion estimation and partitioning selection in enhancement layer(s)	7

5.7	Complexity characteristics of decoder motion compensation in enhancement layer(s)	7

5.8	Complexity characteristics of encoder intra-frame prediction type and partitioning selection in enhancement layer(s)	7

5.9	Complexity characteristics of decoder intra-frame prediction operation in enhancement layer(s)	7

5.10	Complexity characteristics of upsampling filters and transforms specific in enhancement layer(s)	7

5.11	Complexity characteristics of quantization and inverse quantization in enhancement layer(s)	7

5.12	Complexity characteristics of encoder entropy coding operation in enhancement layer(s)	7

5.13	Complexity characteristics of decoder entropy decoding operation in enhancement layer(s)	7

5.14	Degree of capability for encoder parallel processing	7

5.15	Degree of capability for decoder parallel processing	7

6	Software implementation description	8

7	Highlighted aspects discussion	8

8	Closing remarks	8

9	Patent rights declaration(s)	8

# Introduction

This contribution provides a description of our scalable video coding extensions of HEVC. We reply three categories such as spatial scalability with the HEVC base layer, intra-only spatial scalability with the HEVC base layer, and spatial scalability with the AVC base layer.

 	Category 1 (HEVC base layer) spatial scalability

 	Category 1 (HEVC base layer) intra-only spatial scalability

 	Category 1 (HEVC base layer) SNR scalability

 	Category 2 (AVC base layer) spatial scalability

 	The proposal obeys the constraints under section 5 of the CfP (if box is not ticked, explain cases where constraints are violated)

The proposed tools consist of three functions which are generally same as the scalable video coding extension of AVC (H.264 Annex G). They are inter-layer intra prediction, inter-layer prediction mode and motion prediction, and inter-layer residual prediction. Furthermore, we provide the encoder optimization technique.

# Algorithm description

In this proposal, modified tools are presented to exploit the property of spatial scalability video. Highlighted features of proposed codec are as follows.

- Inter-layer intra prediction: the tool is generally same as the SVC. Predicted values are generated from base layer by using up-sampling filter. Position of base layer is derived from the enhancement layer. Base layer shall consist of intra coding unit only.

- Inter-layer prediction mode and motion prediction: This alternative skip mode means that coding unit type is inherited from the base layer. When the all small units in BL are intra coding, current CU of EL inherits inter-layer intra prediction. In the other case, virtual small units in BL are filled by motion vector. Each small unit in EL is then compensated by using motion vector of small unit in BL. Motion compensation is performed by each small unit in EL. Transformation is performed as TU. RQT is also on.

- Inter-layer residual prediction: This function is residual prediction of motion compensation results. Final predicted value is sum of motion compensation and up-sampled residual values obtained from the BL. Residual value in BL means inverse transformed coefficients of BL. When small units in BL are intra coded, residual values are considered zero. This function is turned on/off by each coding unit.

## CTU/TU/PU partitioning

No change from HEVC.

## Up-sampling filters

Up-sampling filets are the same as that in JSVM. It is assumed that the zero-phase is utilized in the filter.

## Inter-layer prediction

### Inter-layer prediction of CU/TU/PU partitions

CU/TU/PU partitions are the same as that in HEVC. It means that partition is independent to the base layer.

Inter-layer sample prediction is introduced, which is same concept of inter-layer intra prediction is JSVM.

### Inter-layer residual prediction

It is the same as JSVM.

### Inter-layer motion vector prediction

It is the same to the JSVM. Base layer MV is enlarged with a given scaling ratio. Obtained MV is added to merge mode, described later.

### Inter-layer mode prediction

It is same as JSVM. When a base_mode_flag is 1, coding unit type is decided from base layer mode.

### Other

Nothing

## Inter-frame prediction

No change from HEVC.

## Intra-frame prediction

No change from HEVC.

## In-loop filtering

### De-blocking filter

No change from HEVC.

### SAO

No change from HEVC.

### Other

Nothing

## Spatial transforms

No change from HEVC.

## Quantization

No change from HEVC.

## Motion/mode/parameter coding

### Merge/skip

In the HEVC specification , predicted motion vectors (PMVs) are listed from left, up, upper left, and so on. In the proposed scheme, PMVs are listed from left, up, inter-layer, upper left, and so on. Therefore maximum number of merge mode candidate becomes six.

### Motion vector coding

No change from HEVC.

### Intra prediction mode coding

Inter-layer intra prediction is introduced. This mode is coded as base_mode_flag.

Furthermore, when base_mode_flag is equaled to 1 and PredMode is equaled to MODE_INTRA, chroma intra prediction mode is coded. It means that luma and chroma prediction modes are independent.

### SAO parameter coding

No change from HEVC.

### Other

Nothing

## Entropy coding of residual / transform coefficients

No change from HEVC.

## High-layer syntax

No change from HEVC.

# Syntax and semantics description

The proposed syntax table is described as follows. The Syntax change from the HEVC specification is located only “General coding unit syntax”. No additional syntax table is necessary.

7.3.9.1	General coding unit syntax

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enable_flag ) { |  |
| cu_transquant_bypass_flag | ae(v) |
| } |  |
| if( slice_type != I ) |  |
| skip_flag[ x0 ][ y0 ] | ae(v) |
| if( skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, log2CbSize ) |  |
| else { |  |
| nCbS = ( 1 << log2CbSize ) |  |
| base_mode_flag | ae(v) |
| if( !base_mode_flag ) { |  |
| if( slice_type != I ) |  |
| pred_mode_flag |  |
| if( PredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ log2CbSize = = Log2MinCbSize ) |  |
| part_mode | ae(v) |
| ... |  |
| } else if ( PredMode[ x0 ][ y0 ] == MODE_INTRA ) { |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( PredMode[ x0 ][ y0 ] != MODE_INTRA ) |  |
| residual_prediction_flag | ae(v) |
| if( !pcm_flag ) { |  |
| if( PredMode[ x0 ][ y0 ] != MODE_INTRA && !(PartMode = = PART_2Nx2N && merge_flag[x0][y0]) ) |  |
| no_residual_syntax_flag | ae(v) |
| if( !no_residual_syntax_flag ) { |  |
| MaxTrafoDepth = ( PredMode[ x0 ][ y0 ] = = MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : max_transform_hierarchy_depth_inter ) |  |
| transform_tree( x0, y0 x0, y0, log2CbSize, 0, 0 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |

base_mode_flag equal to 1 specifies that the coding unit, the prediction unit, and the corresponding motion data (when applicable) are inferred as specified in from base layer. base_mode_flag equal to 0 specifies that the syntax element pred_mode_flag and part_mode is present in the coding unit layer in scalable extension syntax structure.

residual_prediction_flag equal to 1 specifies that the residual signal of the current coding unit is predicted using the reference layer representation. residual_prediction_flag equal to 0 specifies that the residual signal of the current coding unit is not predicted. This flag is presented in response of intra-only spatial scalability.

As mentioned above, number of motion vector candidates is increased due to the inter-layer motion-vector prediction. The five_minus_max_num_merge_cand variable is changed to the followings.

six_minus_max_num_merge_cand specifies the maximum number of merging MVP candidates supported in the slice subtracted from 6. The maximum number of merging MVP candidates, MaxNumMergeCand is computed as

MaxNumMergeCand = 6 − six_minus_max_num_merge_cand	(7-1)

The value of six_minus_max_num_merge_cand shall be limited such that MaxNumMergeCand is in the range of 1 to 6, inclusive.

# Compression performance discussion

## Category 1 (HEVC base layer)

### Spatial scalability

|  | Random Access HEVC 2x |  |  | Random Access HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -29.3% | -24.7% | -25.6% |  |  |  |
| Class B | -26.7% | -20.8% | -20.4% | -45.5% | -40.8% | -40.8% |
| Overall | -28.0% | -22.8% | -23.0% | -45.5% | -40.8% | -40.8% |
|  | -28.1% | -22.8% | -23.1% | -45.5% | -41.4% | -41.5% |

### Intra-only spatial scalability

|  | All Intra HEVC 2x |  |  | All Intra HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -44.7% | -41.9% | -43.8% |  |  |  |
| Class B | -36.0% | -30.8% | -31.3% | -61.6% | -60.1% | -60.9% |
| Overall | -40.4% | -36.3% | -37.5% | -61.6% | -60.1% | -60.9% |
|  | -40.4% | -36.2% | -37.3% | -62.9% | -64.2% | -64.8% |

### SNR scalability

No response.

### Overall

The result of scale factor 1.5x configuration is higher coding performance because of the up-sampled image quality. Bitrate of high resolution sequence can be reduced by the proposed scalability. This trend highly depends on the test sequences.

## Category 2 (AVC base layer)

### Spatial scalability

|  | Hybrid vs HEVC 2x |  |  | Hybrid vs HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -25.7% | -23.2% | -23.6% |  |  |  |
| Class B | -23.1% | -19.1% | -18.1% | -42.5% | -40.5% | -40.1% |
| Overall | -24.4% | -21.2% | -20.9% | -42.5% | -40.5% | -40.1% |
|  | -24.5% | -21.2% | -20.9% | -42.7% | -40.9% | -40.6% |
|  | Hybrid vs AVC 2x |  |  | Hybrid vs AVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A+ | -48.1% | -44.5% | -45.6% |  |  |  |
| Class B | -54.4% | -51.9% | -51.0% | -66.4% | -66.3% | -65.6% |
| Overall | -51.2% | -48.2% | -48.3% | -66.3% | -66.0% | -65.3% |
|  | -51.2% | -48.2% | -48.3% | -66.4% | -66.3% | -65.6% |

### Overall

Because of the same coding tools and the same side information generated from base layer, coding performance of Category 2 is very similar to that of Category 1.

Although the base-layer image-qualities are very similar between the HEVC and AVC, difference ratio of AVC base is smaller than that of HEVC base. This trend is related to the motion vector quality. Furthermore, lambda factor determination is slightly different each other.

# Complexity analysis

## Encoding time and measurement methodology

Measurement methodology is the very similar to HM. An I/O time to read both base layer picture and side information is also included. Consequently both an encoding and a decoding time of base layer is not included.

## Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM

Measurement methodology is the very similar to HM. An I/O time to read both base layer picture and side information is also included. Consequently a decoding time of base layer is not included.

## Description of computing platform used to determine encoding and decoding times reported in sections 5.1 and 5.2

For the encoding time measurements, a clustered system with multiple CPUs and RAMs was used. The CPU is Intel Xeon E5-2680 (2.7 GHz). Each node PC has two CPUs and the 128 GB RAM. The 64-bit executable of HM6.1.1 and the proposed encoder were generated by Microsoft Visual Studio 2008. The execution time was measured on Windows 7 Professional SP1 using HM output time.

## Expected memory usage of encoder

Expected memory is sum of two codec such as base layer resolution and enhancement layer resolution. Some additional memory is also allocated due to the scalability. One is up-sampled motion vector with 4x4 pixel accuracy for one enhancement frame. The others are up-sampled pixels in intra block and residual pixels in inter blocks. A number of these up-sampled frames is the same as that of reference frames.

## Expected memory usage of decoder

In our implementation, expected memory is the same size of the encoder.

## Complexity characteristics of encoder motion estimation and partitioning selection in enhancement layer(s)

Complexity of motion estimation is twice due to the residual prediction. Decision process of partitioning is the same as HM.

## Complexity characteristics of decoder motion compensation in enhancement layer(s)

Motion compensation granularity is increased due to the inter-layer motion-vector prediction with 4x4 blocks. This scheme is the same as JSVM.

## Complexity characteristics of encoder intra-frame prediction type and partitioning selection in enhancement layer(s)

Inter-layer intra prediction method is added to the rate-distortion optimization loop. Because an insertion position is before the sum of absolute difference computations, an increase of total RDO process is negligible.

## Complexity characteristics of decoder intra-frame prediction operation in enhancement layer(s)

Due to the inter-layer intra prediction, up-sampling process is required. In our implementation, whole frame is always up-sampled.

## Complexity characteristics of upsampling filters and transforms specific in enhancement layer(s)

Since up-sampling filters are identical to JSVM, the complexity of them is same as that of JSVM. Transforms are identical to HM6.1.1. Therefore the complexity characteristics are identical to that of the HM6.1.1.

## Complexity characteristics of quantization and inverse quantization in enhancement layer(s)

No change from HM6.1.1.

## Complexity characteristics of encoder entropy coding operation in enhancement layer(s)

No change from HM6.1.1.

## Complexity characteristics of decoder entropy decoding operation in enhancement layer(s)

No change from HM6.1.1.

## Degree of capability for encoder parallel processing

No change from HM6.1.1.

## Degree of capability for decoder parallel processing

No change from HM6.1.1.

# Software implementation description

## External data generated from base layer bitstream

The proposed algorithms were implemented using standard C++ programming language.

We use HM6.1.1 as the base software. Before encoder/decoder running, base-layer bitstream shall be decoded by the modified decoder to produce the decoded yuv video and side information. For each frame, that information consists of picture type, QP value, POC, number of L0 and L1 frames, and POC of ref_index. For each 4x4 block, that information consists of PRED_MODE (intra/inter), ref_idx of L0 and L1, and motion vectors.

Although the encoder and decoder needs external data mentioned above, these information can be generated in the encoder and decoder from a base layer bitstream. Our architecture realizes the simplified implementation. Furthermore this scheme is common for AVC base and HEVC base.

Decoded yuv mentioned above is not motion compensated because of two reasons. One is that inter-layer intra prediction is allowed when all base layer block consist of intra blocks. The other is that inter-layer residual prediction only use the inverse transformed values.

## Some restriction by the implementation

A frame rate of enhancement layer requires to identical that of base layer. This restriction comes from the POC issues, which is not taken care into our implementation.

## Lambda value issues

The proposal obeys the constraints under section 5 of the CfP. In order to satisfy it, lambda values are modified for each condition. Condition set consists of base layer QP, delta QP, b-frame, special scale, and base layer codec.

For the ‘All intra’ category, lambda value is multiplied by dMult, as follows,

dMult=34−iQP12C3ratio+C2−C1

where iQP is QP of enhancement layer and ratio is interpolated ratio based on delta QP. The ratio is described in the CfP. Coefficient table is shown in Table 1.

|  | C1 | C2 | C3 |
| --- | --- | --- | --- |
| AI HEVC x2 | 1.24 | 0.81 | 2.53 |
| AI HEVC x1.5 | 1.04 | 0.58 | 2.14 |

For ‘Random access’ categoly, lambda value is multiplied by dMult, as follows,

dMult=42−iQP20C3ratio+C2−C1+C1

|  | C1 | C2 | C3 |
| --- | --- | --- | --- |
| RA HEVC x2 | 1.15 | 0.22 | 1.40 |
| RA HEVC x1.5 | 1.13 | 0.20 | 1.29 |
| RA AVC x2 | 1.06 | 0.22 | 1.27 |
| RA AVC x1.5 | 1.02 | 0.40 | 1.47 |

where iQP is QP of enhancement layer and ratio is interpolated ratio based on delta QP. The ratio is described in the CfP. Coefficient table is shown in Table 2.

# Highlighted aspects discussion

Our proposed algorithm is common for base layer codec such as AVC and HEVC. In the AVC case, coding order of picture is different between the AVC base layer and HEVC enhancement layer for the last GOP.

# Closing remarks

This contribution proposes to extend HEVC to realize scalability. In conclusion, we think that enough information has been provided to demonstrate the validity of this proposal and the reason for it to be considered as a strong candidate to be used for the Test Model that will be used as the basis of the Core Experiments in the standardization of scalable video coding extensions of HEVC.

# Patent rights declaration(s)

KDDI Corp. (KDDI R&D Laboratories, Inc.) may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).