| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0102 |
| --- | --- |

| Title: | HM-8.0 code speedup |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bin Li Houqiang Li | Email: | yhlibin@mail.ustc.edu.cn lihq@uste.edu.cn |
| Source: | University of Science and Technology of China |  |  |

_____________________________

# Abstract

This contribution proposes several aspects to speed up the HM encoder and decoder. The modifications in the document do not include any normative changes and do not change the R-D performance. The experimental results show that the modifications in this document can save about 14% encoding time (RA_Main, RA_HE10, LB_Main, and LB_HE10) and about 10% decoding time on average.

# Introduction

This contribution proposes several aspects to speed up the HM encoder and decoder. The modifications in the document do not include any normative changes and do not change the R-D performance. The experimental results show that the modifications in this document can save about 14% encoding time (RA_Main, RA_HE10, LB_Main, and LB_HE10) and about 10% decoding time on average.

# Proposed modifications

There are totally 4 modifications in this document.

## Interpolation filter

In the original HM-8.0, the coefficients of interpolation filter are passed as function parameters.

template<int N, bool isVertical, bool isFirst, bool isLast>

Void TComInterpolationFilter::filter(Short const *src, Int srcStride, Short *dst, Int dstStride, Int width, Int height, Short const *coeff)

In the proposed modification, the interpolation filter coefficients are passed as template parameters, adding a parameter “frac” to indicate which group of parameters will be used.

template<int N, int frac, bool isVertical, bool isFirst, bool isLast>

Void TComInterpolationFilter::filter(Short const *src, Int srcStride, Short *dst, Int dstStride, Int width, Int height)

One of the benefits is that with the proposed method, all the interpolation filter coefficients can be determined during the compiling time. Thus, compiler can do any optimization if it considers worth to.

This aspect brings about 10% encoding time saving and 5%~8% decoding time saving.

## Avoid repeated calculation

In the original HM-8.0 implementation, the “component bits” is calculated again and again in
UInt TComRdCost::xGetComponentBits( Int iVal )

In the proposed method, we use a look up table to get the “component bits”. The look up table only needs to be initialized once at the encoder side.

This aspect brings about 3% time saving at the encoder side.

## Different clip operation implementation

In the original HM-8.0, the clip operation is implemented with min() and max() functions. In the proposed method, the clip operation is implemented with if…else… directly.

About 1%~3% encoding time saving and 4%~8% decoding time saving are obtained from this aspect.

## SAD calculation improvement

The loops in TComRdCost::xCalcHADs4x4() and TComRdCost::xCalcHADs8x8() are unrolled in some degree. For example, in the proposed method, the following part in TComRdCost::xCalcHADs4x4() is rewritten without loop.

for (k=0; k<16; ++k)

{

satd += abs(d[k]);

}

It should be noticed that most of the functions in TComRdCost.cpp are with loop unrolling in some degree.

This aspect brings about 1% encoding time saving.

# Experimental results

The experiments are performed based on HM-8.0. The default HM-8.0 is used as anchor for all the tests. The common test conditions specified in [1] are followed. The MD5 checksum calculation is enabled at both encoder side and decoder side. The executable files are generated with Visual Studio 2010, x64, and the tests are running on Windows Server 2008. As the R-D performance is not changed, only the encoding/decoding time ratio shown in Summary (Full) (including Class F) is provided. The detailed results can be found in the associated spreadsheets of this document.

Table 1 Encoding/Decoding time ratio of the propose method

|  | Interpolation filter | Avoid repeated calculation | Clip implementation | SAD calculation | All (Enc. / Dec.) |
| --- | --- | --- | --- | --- | --- |
| AI_Main |  | 100% / 99% | 97% / 92% | 99% / 100% | 96% / 92% |
| AI_HE10 |  | 100% / 100% | 97% / 92% | 100% / 100% | 96% / 92% |
| RA_Main | 90% / 92% | 97% / 99% | 99% / 96% | 99% / 100% | 86% / 88% |
| RA_HE10 | 90% / 92% | 97% / 100% | 99% / 96% | 99% / 100% | 86% / 88% |
| LB_Main | 90% / 93% | 97% / 99% | 100% / 96% | 99% / 100% | 86% / 89% |
| LB_HE10 | 90% / 93% | 97% / 100% | 100% / 96% | 99% / 100% | 86% / 90% |
| LP_Main | 92% / 95% | 97% / 99% | 99% / 97% | 99% / 100% | 88% / 92% |
| LP_HE10 | 92% / 95% | 97% / 100% | 99% / 96% | 99% / 100% | 88% / 92% |

# Acknowledgement

We would like to thank Intel Corporation for providing a crosscheck on a different platform, i.e., Linux + GCC. Running time reduction is also obtained.

# References

- JCTVC-J1100, “HM 8 Common Test Conditions and Software Reference Configurations,” Stockholm, SE, 11–20 July 2012.

# Patent rights declaration(s)

University of Science and Technology of China may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).