| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0103 |
| --- | --- |

| Title: | Rate control by R-lambda model for HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bin Li Houqiang Li Li Li Jinlei Zhang | Email: | yhlibin@mail.ustc.edu.cn lihq@uste.edu.cn |
| Source: | University of Science and Technology of China |  |  |

_____________________________

# Abstract

This contribution proposes R-λ model based rate control for HEVC. The proposed rate control algorithm is implemented on HM-8.0. Compared with the existing rate control algorithm in HM-8.0, when targeting the bitrate of HM-8.0 default anchor (without rate control), the proposed method obtains 0.42 dB ~ 1.12 dB Y PSNR gain for different cases. The bitrate errors (the difference of the target bitrate and the actual bitrate) of the proposed method, 0.09%~0.22% for different cases, are also much smaller than those of the existing rate control algorithm in HM-8.0, which are 0.16%~1.09% for different cases.

# Introduction

The existing rate control algorithm in HM-8.0 applies a unified RQ model first proposed in JCTVC-H0213 [1]. But according to previous investigation in JCTVC-I0426 [2], λ is more important than QP value on determining the bitrate. Thus, this contribution proposes a new rate control algorithm for HEVC via R-λ model. The R-λ model will be described in Section 2, and the proposed rate control algorithm via R-λ model will be described in Section 3. Section 4 provides detailed experimental results and analysis.

# R-λ model

This contribution will build the relationship between R and λ. As we know, several types of R-D model have been proposed to characterize the relationship between R and D. One typical R-D model is Exponential function [4],

D(R)=Ce−KR		(1)

where C and K are model parameters related to the characteristic of the source. Some researchers also model the R-D relationship by Hyperbolic function [5][6],

D(R)=CR−K		(2)

where C and K are also model parameters related to the characteristic of the source, but are different from the C and K in (1).

This contribution performs some experiments to compare these two models based on HEVC. We use the simple IBBB coding structure with only one reference picture and flat QP in HM-7.0, and then fit the R-D curve according to the two different models, as shown in Fig. 1. R is expressed in terms of bpp, and D is expressed in terms of MSE (Mean Square Error) of Luma component. They are calculated by (3) and (4)

bpp=Rf⋅w⋅h		(3)

where f is the frame rate, w and h are the width and height of the picture respectively.

MSE=1Nireci−orgi2	(4)

where N is the total number of pixels in a picture, reci and orgi are the pixel values in reconstructed picture and original picture, respectively. Considering I picture and B picture may have different R-D characteristics, only the R and D in B pictures are taken into account. In Fig. 1, the solid lines are fitted with the Hyperbolic model in (2), while the dotted lines are fitted with the Exponential model in (1). We use correlation coefficient to measure how well these two statistical models fit the experimental observations. It is shown that the average correlation coefficient is greater than 0.99 when fitting with (2), whereas the value is only about 0.92 when fitting with (1). From both the fitting curves and the corresponding correlation coefficient values, we can conclude that for HEVC, Hyperbolic model is better than Exponential model in characterizing the R-D relationship. Thus, Hyperbolic model in (2) is used in this contribution.

Figure 1 Hyperbolic model and Exponential model

As is well known, λ is the slope of R-D curve, which can be expressed as

λ=−∂D∂R=CK⋅R−K−1=α⋅Rβ		(5)

where α and β are parameters related to the video source. (5) can also be expressed in another form that

R=λα−β=α1⋅λβ1			(6)

(6) shows that bitrate R is determined by λ value. To verify the derived R-λ model, we have done a number of experiments using the same test conditions and sequences as used in Fig. 1 to test (6). The fitting R-λ curves are shown in Fig. 2.

Figure 2 R-λ Curve Fitting for Different Sequences

Fig. 2 shows that the R-λ relationship matches the model in (6) very well, although the α1 and β1 are quite different for different video sequences. Based on the R-λ relationship, the rate control framework can be built based on the R-λ model.

# Rate control via R-λ model

The proposed rate control algorithm can be divided into two parts. The first part is bit allocation, and the second part is to adjusting encoding parameters to achieve the allocated bits. Only the second part relates to the proposed R-λ model.

## Bit Allocation

Suppose that the target bitrate is Rtar, the frame rate is f. Thus, the average target bits per picture is

RPicAvg=Rtarf

Denote the number of pictures already been coded Ncoded, and the bit cost on those pictures Rcoded.

- GOP level bit allocation

Suppose there are NGOP pictures in the current GOP. SW is the size of a smooth window, which is used to make the bitrate change smoother. The SW used in this contribution is 40. The target bits in a GOP are determined by

TAvgPic=RPicAvg⋅(Ncoded+SW)−RcodedSW	(7)

TGOP=TAvgPic⋅NGOP			(8)

The aim of the smooth window is to achieve the target bitrate R after SW pictures, if these SW pictures cost exactly TAvgPic bits per picture. The current buffer status and the target bitrate are jointly considered in (7). (7) can be written in another form

TAvgPic=RPicAvg+RPicAvg⋅Ncoded−RcodedSW

The first item (RPicAvg) is the target bitrate, and the second item (RPicAvg⋅Ncoded−Rcoded) is the buffer status.

- Picture level bit allocation

Denote TGOP is the target bits of the current GOP, determined by (8). CodedGOP is the coded bits of the current GOP. ω is the bit allocation weight of every picture in the current GOP. Thus, the target bit of the current picture is determined by

TCurrPic=TGOP−CodedGOPNotCodedPicturesωi⋅ωCurrPic	(9)

The purpose of (9) is to allocate the left bits according to the weight of different pictures. ω is determined by Table 1 and 2.

Table 1 ω of low delay encoding

| Coding order | Example POC | Equal allocation | Hierarchical allocation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | bpp>0.2 | 0.2≥bpp>0.1 | 0.1≥bpp>0.05 | Others |
| 1 | 1 | 1 | 2 | 2 | 2 | 2 |
| 2 | 2 | 1 | 3 | 3 | 3 | 3 |
| 3 | 3 | 1 | 2 | 2 | 2 | 2 |
| 4 | 4 | 1 | 6 | 10 | 12 | 14 |

Table 2 ω of random access encoding

| Coding order | Example POC | Equal allocation | Hierarchical allocation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | bpp>0.2 | 0.2≥bpp>0.1 | 0.1≥bpp>0.05 | Others |
| 1 | 8 | 1 | 15 | 20 | 25 | 30 |
| 2 | 4 | 1 | 5 | 6 | 7 | 8 |
| 3 | 2 | 1 | 4 | 4 | 4 | 4 |
| 4 | 1 | 1 | 1 | 1 | 1 | 1 |
| 5 | 3 | 1 | 1 | 1 | 1 | 1 |
| 6 | 6 | 1 | 4 | 4 | 4 | 4 |
| 7 | 5 | 1 | 1 | 1 | 1 | 1 |
| 8 | 7 | 1 | 1 | 1 | 1 | 1 |

Both the equal bit allocation and hierarchical bit allocation are supported in the proposed rate control algorithm.

In the experiments, hierarchical bit allocation is used for both low delay and random access coding structure. Additionally, the results of equal bit allocation are also provided for low delay cases, as we think it does not make much sense of applying equal bit allocation to hierarchical-B coding structure.

There is a special case, the bit allocation for intra pictures. When the QP and λ of intra pictures are not specified (in our simulation, the QP and λ values of intra pictures are fixed, using the values of HEVC anchor without rate control), the allocated bits (TCurrPic) are refined according to bpp, as shown in Table 3.

Table 3 Intra bit allocation refinement

| bpp | bpp>0.2 | 0.2≥bpp>0.1 | Others |
| --- | --- | --- | --- |
| Final TCurrPic | 5 x TCurrPic | 7 x TCurrPic | 10 x TCurrPic |

It should be noticed that, the refinement specified in Table 3 is not used when targeting the bitrate of HM-8.0 anchor (without rate control).

And as the intra pictures will cost much more bits than inter picture, which may result in the bit cost of intra picture is more than the bit allocated for the entire GOP. Thus, the bit cost of intra picture is not used to update CodedGOP (bit cost inside the current GOP), it is only used to update Rcoded (bit cost of the whole sequence). TCurrPic before refinement is used to update the current CodedGOP.

- LCU level bit allocation

When LCU level rate control is enabled, the target bits of each LCU is determined by

TCurrCU=TCurrPic−Bitheader−CodedPicNotCodedLCUsωi⋅ωCurrLCU

The Bitheader is the estimated bits of all headers, including slice header (and maybe APS if it comes back). Bitheader is estimated according to the actual header bits of previous coded pictures belonging to the same level.

Here ω is the weight of each LCU, estimated by the prediction error (in form of MAD) of collocated picture in the previous coded picture belonging to the same level.

MADLCU=1Npixelsipredi−orgi

ωLCU=MADLCU2

## Achieve the allocated bit

As shown in Section 2, R and λ have strong relationship. To achieve a target R, we only need to determine a corresponding λ. λ can be determined according to (5), as the target R is known here. We should determine α and β value here. As shown in Fig. 2, different sequences have quite different values. A simple solution is to pre-encode some pictures and fit the R-λ curve so as to get α and β values for a specified sequence. But it will introduce additional encoding complexity and greatly increase the delay. In addition, when the characteristic of the video sequence changes (e.g., scene changes), the predetermined α and β values will be no longer valid. To adapt to various video sequence with different characteristic, we design the following algorithms.

λ=α⋅bppβ		(10)

We use (10) to derive λ value from target R for a picture level or LCU level, but the model may have different α and β values for different cases. In addition, different picture level also has different model parameters if hierarchical bit allocation is applied. For instance, if GOP size is equal to 4, there exists three picture levels; and the three picture levels may have their different pairs of α and β values. At last, different LCUs also have different α and β values; however, it is assumed that collocated LCUs in different pictures but belong to the same picture level may share the same parameters α and β. The initial values of α and β used in (10) are 3.2003 and −1.367 respectively, obtained from experience and observation. It should be noticed that the initial values of α and β are not critical, as they will keep updating with the encoding.

The QP value can be determined by multiple-QP optimization, which may result in the best performance but increase the encoding complexity significantly. Instead, we use the following equation to determine the QP value according to the λ value, as proposed in [2] and [3].

QP=4.2005lnλ+13.7122

To keep the quality consistency, the λ value and the determined QP value will be clipped in a narrow range.

For picture level, the estimated λ is guaranteed that

λlastSameLevel⋅2−3.03.0≤λcurrPic≤λlastSameLevel⋅23.03.0

λlastPic⋅2−10.03.0≤λcurrPic≤λlastPic⋅210.03.0

For picture level, the QP is guaranteed that

QPlastSameLevel−3≤QPcurrPic≤QPlastSameLevel+3

QPlastPic−10≤QPcurrPic≤QPlastPic+10

For LCU level, the estimated λ is guaranteed that

λlastLCU⋅2−1.03.0≤λcurrLCU≤λlastLCU⋅21.03.0

λcurrPic⋅2−2.03.0≤λcurrLCU≤λcurrPic⋅22.03.0

For LCU level, the QP is guaranteed that

QPlastLCU−1≤QPcurrLCU≤QPlastLCU+1

QPcurrPic−2≤QPcurrLCU≤QPcurrPic+2

After encoding one LCU or one picture, we use the real encoded bpp (bppreal) and real used λ value (λreal) to update the values of α and β of the model. In a picture, each LCU has its own λ value, and we need to use the average value. It should be noticed that when referring to the average values in the rest of this document, it means arithmetic mean value for QP, while it means geometric mean value for λ. The α and β are updated by (11) to (13). In our implementation, δα and δβ are set to be 0.1 and 0.05, respectively.

λcomp=αold⋅bpprealβold				(11)

αnew=αold+δα⋅lnλreal−lnλcomp⋅αold		(12)

βnew=βold+δβ⋅lnλreal−lnλcomp⋅lnbppreal	(13)

It should be noticed that, for some special case, such as the actual encoding bpp is too small (e.g. skip mode for a LCU or too many skip mode for a picture), the updating process is performed as

αnew=0.96αold

βnew=0.98βold

No matter which method is used to update α and β, the final values are clipped in a predetermined range. α is clipped into [0.05, 20] and β is clipped into [−3.0, −0.1].

# Experimental results

The experiments are performed based on HM-8.0. The (rounded) bitrate of default HM-8.0 is used as the target bitrate of the tests. The common test conditions specified in [1] are followed. The MD5 checksum calculation is enabled at both encoder side and decoder side. The aspects to speed up the codec introduced in [8] are enabled in the simulation. The anchor used in the simulation is the rate control algorithm built in HM-8.0. Table 4 (JCTVC-K0103_AR.xlsm) shows the R-D performance of the proposed R-λ based rate control algorithm with hierarchical bit allocation enabled. Table 5 (JCTVC-K0103_AR_noH.xlsm) shows the R-D performance of the proposed R-λ based rate control algorithm without hierarchical bit allocation enabled. As we do not think it makes much sense of using equal bit allocation for hierarchical-B coding structure, only the results of the IBBB coding are provided when equal bit allocation is used.

Table 4 R-D performance of the proposed R-λ model based rate control (Hierarchical bit allocation)

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -24.1% | -26.8% | -27.0% | -22.0% | -4.4% | -3.8% |
| Class B | -26.8% | -33.1% | -35.0% | -20.2% | -6.9% | -9.5% |
| Class C | -26.5% | -32.1% | -31.5% | -7.6% | -3.4% | -4.7% |
| Class D | -20.3% | -33.7% | -33.0% | -0.4% | 2.8% | 0.9% |
| Class E |  |  |  |  |  |  |
| Class F | -37.7% | -37.3% | -38.1% | -19.7% | -13.5% | -15.5% |
| Overall | -27.1% | -32.6% | -33.0% | -14.3% | -5.1% | -6.7% |
|  | -25.2% | -15.9% | 11.3% | -14.1% | -4.9% | -6.3% |
| Enc Time[%] | 98% |  |  | 102% |  |  |
| Dec Time[%] | 98% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -23.1% | -14.2% | -14.7% | -51.5% | -27.2% | -26.2% |
| Class C | -10.7% | -10.5% | -10.1% | -23.3% | -7.6% | -6.6% |
| Class D | -6.9% | -12.3% | -11.1% | -6.8% | 9.5% | 9.7% |
| Class E | -22.7% | 5.6% | 12.1% | -31.5% | 2.6% | 10.1% |
| Class F | -3.7% | -1.2% | 2.6% | -12.6% | 0.7% | 2.7% |
| Overall | -13.4% | -7.5% | -5.6% | -26.2% | -5.9% | -3.9% |
|  | -16.3% | -11.2% | -8.2% | -26.0% | -6.6% | -4.7% |
| Enc Time[%] | 100% |  |  | 104% |  |  |
| Dec Time[%] | 99% |  |  | 102% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -21.7% | -13.0% | -13.9% | -50.4% | -26.2% | -26.5% |
| Class C | -10.6% | -11.9% | -11.7% | -23.6% | -7.3% | -6.1% |
| Class D | -8.3% | -15.4% | -14.2% | -7.8% | 7.8% | 8.4% |
| Class E | -22.8% | 8.3% | 13.9% | -31.1% | 6.7% | 14.7% |
| Class F | -5.2% | 0.1% | 4.1% | -13.2% | -0.5% | 1.2% |
| Overall | -13.7% | -7.5% | -5.7% | -26.2% | -5.5% | -3.7% |
|  | 81.9% | -9.9% | -8.5% | -26.1% | -6.4% | -4.8% |
| Enc Time[%] | 100% |  |  | 105% |  |  |
| Dec Time[%] | 99% |  |  | 101% |  |  |

Table 5 R-D performance of the proposed R-λ model based rate control (Equal bit allocation)

|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -20.0% | 5.0% | 6.2% | -49.3% | -12.0% | -9.6% |
| Class C | -0.3% | 12.4% | 11.8% | -13.8% | 16.7% | 17.0% |
| Class D | 3.9% | 13.5% | 12.2% | 4.7% | 36.7% | 36.4% |
| Class E | -18.5% | 18.7% | 27.5% | -28.2% | 13.3% | 22.9% |
| Class F | -1.1% | 8.3% | 11.3% | -11.2% | 10.9% | 11.9% |
| Overall | -7.3% | 10.9% | 12.7% | -20.6% | 11.9% | 14.1% |
|  | -10.2% | 6.5% | 9.4% | -20.5% | 10.2% | 12.5% |
| Enc Time[%] | 103% |  |  | 108% |  |  |
| Dec Time[%] | 101% |  |  | 104% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -19.1% | 8.6% | 10.1% | -48.3% | -8.4% | -7.2% |
| Class C | -0.6% | 11.7% | 11.4% | -14.5% | 18.7% | 19.9% |
| Class D | 2.5% | 9.8% | 9.1% | 3.5% | 37.2% | 39.2% |
| Class E | -18.3% | 22.2% | 32.1% | -27.4% | 18.6% | 29.0% |
| Class F | -2.2% | 11.4% | 14.8% | -11.7% | 11.8% | 12.2% |
| Overall | -7.6% | 12.1% | 14.4% | -20.7% | 14.2% | 16.8% |
|  | 93.6% | 8.8% | 10.8% | -20.7% | 12.4% | 14.7% |
| Enc Time[%] | 104% |  |  | 110% |  |  |
| Dec Time[%] | 101% |  |  | 103% |  |  |

We also performed experiments on picture level only rate control. The detailed results can be found in JCTVC-K0103_AR_picOnly.xlsm and JCTVC-K0103_AR_picOnly_noH.xlsm. Besides targeting the HEVC anchor bitrate, we also performed experiments on targeting the bitrate specified in HEVC Call for Proposal [9] ([9] specified 5 bitrate points per sequence, we only use 4 of them, excluding the middle one). The detailed results can be found in (JCTVC-K0103_CR.xlsm, JCTVC-K0103_CR_noH.xlsm, JCTVC-K0103_CR_picOnly.xlsm, and JCTVC-K0103_CR_picOnly_noH.xlsm).

The summary results on rate control accuracy when targeting the HM anchor bitrate are provided in Table 6. The Class F sequences are excluded in the average numbers shows in Table 6.

Table 6 The rate control accuracy of the proposed R-λ model based rate control algorithm

| Test case | Bitrate error (Avg./Max) |  | Y PSNR improvement |
| --- | --- | --- | --- |
| Hierarchical bit allocation | RC in HM-8.0 | Proposed RC |  |
| RA-Main | 0.78%/2.78% | 0.22%/3.33% | 1.08 dB |
| RA-HE10 | 1.09%/9.59% | 0.21%/2.84% | 0.42 dB |
| LB-Main | 0.16%/1.51% | 0.10%/1.30% | 0.55 dB |
| LB-HE10 | 0.73%/2.78% | 0.10%/1.09% | 1.12 dB |
| LP-Main | 0.22%/3.93% | 0.09%/1.06% | 0.52 dB |
| LP-HE10 | 0.75%/3.14% | 0.12%/1.14% | 1.08 dB |
| Equal bit allocation | RC in HM-8.0 | Proposed RC |  |
| LB-Main | 0.16%/1.51% | 0.06%/1.44% | 0.29 dB |
| LB-HE10 | 0.73%/2.78% | 0.08%/2.28% | 0.87 dB |
| LP-Main | 0.22%/3.93% | 0.13%/2.86% | 0.29 dB |
| LP-HE10 | 0.75%/3.14% | 0.07%/1.95% | 0.83 dB |

The following figures show some per-picture bit cost comparing the proposed R-λ model based rate control algorithm and the existing rate control algorithm in HM-8.0. In the first two figures, the equal bit allocation is used in the proposed rate control. And in the last two figures, the hierarchical bit allocation is used in the proposed rate control algorithm

# References

- JCTVC-H0213, “Rate control based on unified RQ model for HEVC,” San José, CA, USA, 1–10 February, 2012.

- JCTVC-I0426, “QP determination by lambda value,” Geneva, CH, 27 April – 7 May 2012.

- JCTVC-J0242, “Encoding with fixed Lagrange multipliers,” Stockholm, SE, 11–20 July 2012.

- G. Sullivan and T. Wiegand, “Rate-distortion optimization for video compression,” Signal Processing Magazine, IEEE, vol. 15, pp. 74 –90, Nov. 1998.

- S. Mallat and F. Falzon, “Analysis of low bit rate image transform coding,” Signal Processing, IEEE Transactions on, vol. 46, pp. 1027 –1042, Apr. 1998.

- M. Dai, D. Loguinov, and H. Radha, “Rate-distortion modeling of scalable video coders,” in Image Processing, 2004. ICIP’04, 2004 International Conference on, vol. 2, pp. 1093 – 1096 Vol.2, Oct. 2004.

- JCTVC-J1100, “HM 8 Common Test Conditions and Software Reference Configurations,” Stockholm, SE, 11–20 July 2012.

- JCTVC-K0102, “HM-8.0 code speedup,” Shanghai, CN, 10–19 Oct. 2012.

- VCEG-AM91, “Joint Call for Proposals on Video Compression Technology,” Kyoto, JP, Jan. 2010.

# Patent rights declaration(s)

University of Science and Technology of China may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).