| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0105 |
| --- | --- |

| Title: | FPA SEI message modification |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ohji Nakagami Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81 50 3750 2746 ohji.nakagami@jp.sony.com teruhikos@jp.sony.com |
|  | Giovanni Ballocca Vittorio Giovara Sisvel Technology, Via Castagnole 59, 10060 None Torinese (TO), ITALY | Tel: Email: | +39 011 9904770 giovanni.ballocca@sisveltech.com vittorio.giovara@sisveltech.com |
|  | Paola Sunna Marco Arena Centro Ricerche e Innovazione Tecnologica RAI, corso Giambone 68, 10135 Torino, ITALY | Tel: Email: | +39 011 8103189 p.sunna@rai.it marco.arena@rai.it |
|  | Jean Francois Travers TDF, 106 rue Max Dormoy Montrouge, 92 FRANCE | Email: | jean-francois.travers@tdf.fr |
|  | Klaus Illgner Institut für Rundfunktechnik GmbH, Floriansmühlstr. 60 80939 Munich, GERMANY | Tel: Email: | +49 (0)89 / 323 99 - 228 illgner@irt.de |
|  | Marco Pellegrinato Mediaset, Viale Europa 44/46/48, 20093 Cologno Monzese (MI), ITALY | Tel: Email: | +39 02 25149353 marco.pellegrinato@mediaset.it |
| Source: | Sony Corporation, Sisvel Technology S.r.l., RAI, TDF, IRT, Mediaset |  |  |

_____________________________

# Abstract

Documents JCTVC-J0070, J0073 and J0198 presented at the 11th (Stockholm) meeting discussed the 2D compatibility of 3D frame packing arrangements (FPAs). J0070 and J0073 presented similar solutions, based on the usage of the cropping rectangle and modified SEI messages syntax while J0073, among other things, proposed means to signal a plurality of cropping rectangles to enable both 2D and 3D decoders to crop the decoded picture correctly.

This contribution elaborates on the content of the above-mentioned papers and proposes an alternative approach based on the differentiation of SEI messages describing 2D compatible and non 2D compatible FPAs.

Introduction

Two different service scenarios based on the distribution of FPAs are envisioned and should be enabled:

- The service operator is willing to distribute stereoscopic video using one of the FPAs without 2D compatibility (e.g., a broadcaster simulcasting the same content in 2D and stereoscopic format). By external (applicative) means it is always possible to display just one of the two views.

- The service operator is willing to distribute stereoscopic video using FPAs with 2D compatibility (e.g., so as to avoid the bandwidth overhead needed for simulcast). A 2D decoder, will automatically present only one of the two views to the display, while a 3D decoder will be able to present the two images contained within the FPA to the display for further processing and stereoscopic visualization.

In case 1, the behavior of the 2D decoder when it receives the 3D FPA content is not taken into consideration since a 2D equivalent has been provided as a different service. The 2D decoder is not required to understand the signalling describing the FPS (FPA SEI messages).

In case 2, since no 2D service equivalent is provided, a technical solution is necessary to enable the 2D decoder (unaware of the FPA format) to output by default a 2D version from the FPA, while enabling a 3D decoder (aware of the different FPAs and able to interpret the SEI messages) to output video and proper signalling enabling stereoscopic visualization.

# Proposed solution

Case 1 doesn’t require any modification to the existing syntax and signalling.

Case 2 is easily covered by a slight modification of the already existing signalling: the cropping rectangle is set so as to enclose one of the two views, so that a 2D decoder only outputs the 2D content (see following figure). Proper scaling of the output picture can be suggested using the SAR parameter  in the VUI.

New SEI messages are available to tell a 3D decoder that part of the FPA is located outside the cropping rectangle.

The decoding process should work according to the following chart:

The new value of the cropping parameter might be suitably signaled in the new SEI messages.

# Text modification

The current draft applies the semantics specified in subclause D.2.25 of ITU-T Rec. H.264 | ISO/IEC 14496-10. The proposed modifications are highlighted in yellow.

  - SEI payload syntax

    - Frame packing arrangement SEI message syntax

| frame_packing_arrangement ( payloadSize ) { | Descriptor |
| --- | --- |
| frame_packing_arrangement_id | ue(v) |
| frame_packing_arrangement_cancel_flag | u(1) |
| if( !frame_packing_arrangement_cancel_flag ) { |  |
| frame_packing_arrangement_type | u(7) |
| quincunx_sampling_flag | u(1) |
| content_interpretation_type | u(6) |
| spatial_flipping_flag | u(1) |
| frame0_flipped_flag | u(1) |
| field_views_flag | u(1) |
| current_frame_is_frame0_flag | u(1) |
| frame0_self_contained_flag | u(1) |
| frame1_self_contained_flag | u(1) |
| if (!quincunx_sampling_flag && frame_packing_arrangement_type != 5) { |  |
| frame0_grid_position_x | u(4) |
| frame0_grid_position_y | u(4) |
| frame1_grid_position_x | u(4) |
| frame1_grid_position_y | u(4) |
| } |  |
| if (frame_packing_arrangement_type == N ¦¦ frame_packing_arrangement_type == N+1) { |  |
| large_crop_top_position_frame | ue(v) |
| large _crop_left_position_frame | ue(v) |
| large _crop_bottom_position_frame | ue(v) |
| large _crop_right_position_frame | ue(v) |
| } |  |
| frame_packing_arrangement_reserved_byte | u(8) |
| frame_packing_arrangement_repetition_period | ue(v) |
| } |  |
| frame_packing_arrangement_extension_flag | u(1) |
| } |  |

  - SEI payload semantics

    - Frame packing arrangement SEI message semantics

frame_packing_arrangement_type indicates the type of packing arrangement of the frames as specified in the table below.

Table ‎2-1: proposed modification to Table D-8 of the ISO/IEC 14496-10

| Value | Interpretation |
| --- | --- |
| 0 | Each component plane of the decoded frames contains a "checkerboard" based interleaving of corresponding planes of two constituent frames as illustrated in Figure D-1. |
| 1 | Each component plane of the decoded frames contains a column based interleaving of corresponding planes of two constituent frames as illustrated in Figure D-2. |
| 2 | Each component plane of the decoded frames contains a row based interleaving of corresponding planes of two constituent frames as illustrated in Figure D-3. |
| 3 | Each component plane of the decoded frames contains a side-by-side packing arrangement of corresponding planes of two constituent frames as illustrated in Figure D-4 and Figure D-6. |
| 4 | Each component plane of the decoded frames contains top-bottom packing arrangement of corresponding planes of two constituent frames as illustrated in Figure D-5. |
| 5 | The component planes of the decoded frames in output order form a temporal interleaving of alternating first and second constituent frames as illustrated in Figure D-7. |
| ... | New FPAs defined in pending amendment to AVC specification. |
| N | Each component plane of the decoded frames contains a side-by-side packing arrangement of corresponding planes of two constituent frames as illustrated in Figure D-4 and Figure D-6. This frame packing arrangement is intended to be made 2D compatible by suitable settings of the frame cropping parameter values in the SPS as exemplified in (2.16.X) and illustrated in figure D-X. |
| N+1 | Each component plane of the decoded frames contains top-bottom packing arrangement of corresponding planes of two constituent frames as illustrated in Figure D-5. This frame packing arrangement is intended to be made 2D compatible by suitable settings of the frame cropping parameter values in the SPS as exemplified in (2.16.Y) and illustrated in figure D-W. |

[...]

large_crop_top_position_frame_frame, large _crop_left_position_frame, large_crop_bottom_position_frame and large_crop_right_position_frame when frame_packing_arrangement_type is equal to N or N+1 it is necessary to set up the cropping rectangle on one of the two views to provide backward compatibility specify the samples of frame in the pictures in the coded video sequence that are output from the decoding process, in terms of a rectangular region specified in picture coordinates for output. This values can be used by 3D-aware decoders in order to display the stereoscopic video.

[…]

Figure D-X illustrates the 2D compatible rearrangement of the side by side arrangement scheme.

The cropping rectangle is depicted in dashed lines on the side by side frame packing arrangement.

Figure D-X – 2D compatible rearrangement and upconversion of side-by-side packing arrangement

Figure D-Y illustrates the 3D rearrangement of the side by side arrangement scheme.

The new values of the cropping rectangle parameter are depicted in dashed lines on the side by side frame packing arrangement.

Figure D-Y – 3D rearrangement and upconversion of side-by-side packing arrangement

Figure D-W illustrates the 2D compatible rearrangement of the top and bottom arrangement scheme.

The cropping rectangle is depicted in dashed lines on the top and bottom frame packing arrangement.

Figure D-W – 2D compatible rearrangement and upconversion of top and bottom packing arrangement

Figure D-Z illustrates the 3D rearrangement of the top-and-bottom arrangement scheme.

The new values of the cropping rectangle parameter are depicted in dashed lines on the side by side frame packing arrangement.

Figure D-Z – 3D rearrangement and upconversion of top-and-bottom packing arrangement

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sisvel Technology may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

RAI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

TDF may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

IRT may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Mediaset may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)