| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 October 2012 | Document: JCTVC-K0108 |
| --- | --- |

| Title: | On parallel decoding with SEI message containing Reference Dependency Tree |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Minwoo Kim Deokho Kim KyungAh Kim Won Woo Ro Yonsei University Chanyul Kim Youngo Park Kwangpyo Choi Samsung | Email: Email: | kenstars@yonsei.ac.kr nautes87@yonsei.ac.kr kyungah.kim@yonsei.ac.kr wro@yonsei.ac.kr dionism@samsung.com youngo.park@samsung.com kp5.choi@samsung.com |
| Source: | Yonsei University and Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In this contribution, a parallel decoding scheme for random access profile is proposed. Such parallel algorithm is implemented by using a simple data structure called Reference Dependency Tree (RDT), indicating the order of decoding with parallelism. RDT is built at the encoder side and is designed to be contained in the SEI message portion of the bitstream to be transmitted to the decoder side. The decoder replaces the former Reference Picture Set (RPS) with RDT for parallel decoding of the frames by resolving the reference dependency. The experimental results show an average of 35.1% increase in decoding time, thus efficiently improving the performance of the decoder.

# Introduction

Accelerated decoding in order to achieve real-time processing is essential for the current HEVC decoder. So as to accomplish such goal, the HEVC coding tools provide methods to support parallel processing, such as tiling, wavefront parallel processing, etc. The drawback of such tools however, is that the methods require tight synchronization when concurrent processing inside the frame is performed. In addition, parallel decoding using such methods is only possible if and only if the encoded bitstream is generated by using the same technique. For this reason, we propose a novel approach for the parallel processing of the decoder, which is available for any form of encoded bitstream generated by using the random access profile, and charges minimum synchronization overhead.

The first contribution of this proposal provides a method to find parallelism among the coding frames by using a simple data structure which we call Reference Dependency Tree (RDT). At the encoding side, RDT is built by analyzing the encoding order inside the GOP using RPS so that the parallel coding order is able to be defined. The second contribution is the utilization of the Supplement Enhancement Information (SEI) message for the transmission of RDT. RDT information is included at the SEI portion of the NAL so that at RDT can directly be utilized at the decoding side for parallelized decoding phase. Details of RDT construction and parallel decoding algorithm are given in Section 2 and the experimental results are provided in Section 3.

# Algorithm Description

## Reference Dependency Tree

First, we define the reference dependency relationships existing within the GOP, as shown in Fig. 1.

Figure 1. Frame reference order inside the GOP

The figure indicates the reference dependencies used in the random access profile with GOP size 8. The numbers indicated in each frame represents the display order within a GOP and the frames pointed by the arrow are required to encode or decode a frame. The frame which marked as 0 represents 8th frame of the previous GOP in the figure, thus the coding order of the example represented in the figure becomes 8-4-2-1-3-6-5-7.

Fig. 2 shows an example of a Reference Dependency Tree (RDT) of GOP size 8 using the random access profile with CRA picture. The tree is built by analyzing the reference order defined in RPS. The root node 0 is an intra-frame of which its appearance is decided by the intra period value, and the coding is solely performed within the frame. The digits in the node indicate the relative position of the frames in a GOP in play order and each node is able to be coded whenever all its parent nodes terminate the coding process. Unlike the original design in which the referencing frames wait for the referenced frames to finish coding process, in the proposed algorithm the terminating nodes (referenced frames) progressively call the coding tools of the children nodes (referencing frames). In addition, nodes that share the identical tree depth are able to be coded simultaneously because no reference dependency exists between the sibling nodes. The first encoded/decoded picture of the next GOP is pointed by the last referenced picture of the current GOP. In the example of Fig. 2, this is represented by the node 6 pointing the node 8. This enables the decoding process to move on to the next GOP.

Finding the parallelism inside the GOP is the core objective of the proposed RDT. As stated above, the frames according to the nodes with identical depths are able to be processed independently, thus possessing parallelism. By using the dependencies represented in Fig. 1, node 1, 3, and 6, and node 5, 7, and the next GOP’s 8 have possible parallelism in the example in Fig. 2. Concurrently processing these frames never violate the original coding rules, providing only the benefit of accelerating the coding speed.

Figure 2. RDT at random access profile using CRA picture

## Parallel Decoding Using RDT

First of all, RDT is constructed at the encoder side. The constructed RDT is used for either sequential/parallel encoding of the input video sequence. As the NAL bitstream is generated after the encoding process, RDT is additionally included inside the SEI message in order to be transmitted to the decoding side. At the decoding side, RDT is extracted from the SEI message and is utilized for the clarification of the frame reference order inside the GOPs. Because the main purpose of RDT is to define parallel coding order inside the GOP, the decoding stage utilizes this information for parallelization of the decoding process.

The overall usage of RDT in the encoding/decoding sequence is shown in Fig. 3. As shown in the figure and explained previously, RDT is directly acquired by the decoding stage to be used for parallel decoding. The proposed design utilizes RDT of which the size of the data structure is multiple of (GOP size +1), allowing only minimal addition in the bitstream sequence. Also, since RDT is directly acquired at the decoding stage, there exists no additional calculation overhead for the decoder. This proposal enables accelerated progressive parallel decoding with minimal supplementary overhead.

Figure 3. Block diagram of the parallel decoding using RDT on SEI message

# Proposed Syntax and Semantics

The syntax of the SEI message for RDT transmission is shown as follows. The SEI message basically holds four information representing RDT, which are 1) the number of tree nodes, 2) the depth value of each node, 3) the number of child nodes for each node, and 4) the node numbers of the child nodes of each node.

| reference_dependency_tree_info(payloadSize) { | Descriptor |
| --- | --- |
| number_of_reference_dependency_tree_node | u(v) |
| for (i=0; i< number_of_reference_dependency_tree_node; i++) { |  |
| reference_dependency_tree_node_depth[i] | u(v) |
| reference_dependency_tree_node_number_of_children[i] | u(v) |
| } |  |
| for (i=0;i<number_of_reference_dependency_tree_node; i++) { |  |
| for (j=0; j< reference_dependency_tree_node_number_of_children [i];j++) { |  |
| reference_dependency_tree_node_children[i][j] | u(v) |
| } |  |
| } |  |
| } |  |

The pseudo code above is the syntax for RDT type SEI message. RDT is constructed at the encoding side and is included in the SEI message according to the sequence represented in the syntax above. The four types of essential information to be included are described as follows.

number_of_reference_dependency_tree_node identifies the number of nodes within RDT. The value of number_of_reference_dependency_tree_node shall be identical to the size of the GOP.

reference_dependency_tree_node_depth[i] specifies the depth of each node of RDT, which shall be in the range of 0 to the size of the GOP. The nodes are decoded in the order represented by the depth value, in ascending order. The nodes those possess the identical value are able to be decoded concurrently, in parallel.

reference_dependency_tree_node_number_of_children[i] identifies the number of child nodes of a node in RDT, which shall be in the range of 0 to the size of the GOP. The parent node that holds multiple child nodes can issue simultaneous parallel decoding processes to its children.

reference_dependency_tree_node_children[i][j] identifies the jth child node of the ith reference dependency tree node. The value indicates the actual node number of the child nodes. The value of reference_dependency_tree_node_children[i][j] shall be in the range of 0 to the size of the GOP.

# Experimental Results

The proposed parallel decoding method using RDT on SEI message is implemented on top of HM8.0. The experiment is done for the random access profile using CRA pictures. The test sequences used are those defined in document JCTVC-I1100 with random access main configuration profile. The experiment was performed on a real machine containing two multicore processors, achieving total 8 operational cores. The encoding/decoding options for each test sequence were not modified from the basic configuration provided by HM8.0.

Tables 1 through 5 summarize the decoding time enhancement achieved by the proposed method, in which the performance of the proposed method is indicated by ‘Parallel (s)’. The results show that the decoding time enhancement of each sequence class spans between 24~41%. The average decoding time enhancement for the entire classes is 35.9% for the main profile and 34.3% for the HE profile, respectively. Especially for the sequences ‘Basketball Drill’ in Class C and ‘Basketball Drill Text’ in Class F with the Main profile, and ‘RaceHorses’ in Class C with the HE profile achieve real time decoding through the proposed parallelism. Also, increase in NAL bitstream size by adding RDT in the SEI message requires additional 230 bytes, which is minimal compared to the payload.

Table 1. Performance results of the proposed method for Class A

Table 2. Performance results of the proposed method for Class B

Table 3. Performance results of the proposed method for Class C

Table 4. Performance results of the proposed method for Class D

Table 5. Performance results of the proposed method for Class F

Tables 6 and 7 show the BD-rate values of the proposed parallel decoder. It can be seen that there is no difference between the original reconstructed video sequence and the parallel-decoded video sequence. To summarize, our contribution is the achievement of the accelerated decoding with the output video sequence that is exactly identical to that decoded with the original decoder provided by HM8.0.

Table 6. BD-rate results of the proposed method using the Main profile

Table 7. BD-rate results of the proposed method using the HE profile

# Conclusion

Parallel decoding achieved by RDT effectively enhances the decoding speed of HEVC without any distortion on the output reconstructed video sequence. In addition, using SEI message in order to transmit RDT data structure from the encoding side efficiently removes the calculation overhead at the decoding side with minimum increase in NAL bitstream size. The proposed method shows remarkable performance improvement to be recommended for adoption in HEVC parallel decoding.

# Patent rights declaration(s)

Yonsei University and Samsung Electronics Co., Ld. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).