| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC- K0110 |
| --- | --- |

| Title: | Comparison of Compression Performance of HEVC with AVC at different resolutions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Paola Sunna Marco Arena Centro Ricerche e Innovazione Tecnologica RAI, corso Giambone 68, 10135, Torino, ITALY | Tel: Email: | +39 011 8103189 p.sunna@rai.it marco.arena@rai.it |
| Source: | RAI |  |  |

_____________________________

# Abstract

This contribution provides the experimental results of the compression performance (PSNR-based ) of HEVC Main Profile versus AVC High Profile. The test material includes both progressive and interlaced sequences, from 1280x720 to 3840x2160 pixel resolution. All source video test material, 10 seconds long - 4:2:0 YCbCr color sampling with 8 bits per sample, was encoded using the common test condition.

Experimental results show the following HM-7.0 bit-rate saving respect to JM-18.3:

- Main Profile all-intra configuration: about 15%;

- Main Profile random-access configuration: about 26% ;

- Main Profile low-delay configuration: about 32%

In order to validate the results, preliminary subjective expert viewings session were performed  on 720p50 sequences encoded in random-access configuration: the perceived subjective quality improvement was about 40% in terms of bitrate saving.

# Introduction

This contribution presents the results of objective (PSNR-based) comparison tests between HEVC (HM-7.0) and  AVC High Profile (JM-18.3) as an anchor reference. JM-18.3 and preliminary subjective expert viewings session results.

# Test materials

Original content was retrieved from the public SVT multi format test set [3] that includes the following pixel resolutions and frame rate:

- 720p50

- 1080i25

- 1080p50

- 2160p50

The table below shows the coding difficulty for each sequence as reported in [4].

Table 1

| Name | Coding difficulty |
| --- | --- |
| CrowdRun | Difficult |
| ParkJoy | Difficult |
| DuksTakeOff | Difficult |
| IntoTree | Easy |
| OldTownCross | Easy |

# Objective measurement

The JM software was configured with the same coding structure and referencing structure as HM-7.0. JM and HM used the same QP settings. Both JM anchor and HM test encoding used 8-bit encoding and decoding. The detailed JM configuration is explained below.

Interlaced materials was encoded in two ways:

- frame mode (i.e. progressive_source_flag equal to 1)

- field mode (i.e. progressive_source_flag equal to 0)

In both cases the video sources were encoded as progressive materials also when AVC is used.

The average YUV PSNR (the right-most column of Table 2) was calculated as

PSNRavg=6×PSNRY+PSNRU+PSNRV8

The summary results are shown in the Tables 3, 4, and 5. Details can be found in the attached file JCTVC-K0110.xls.

The HEVC Main Profile is specified in [2], and we use the configuration files provided in [1] to generate the HEVC Main Profile bitstream.

Table 2

|  | All Intra HE |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y | U | V | Avg |
| 720p50 | -12,3% | -14,7% | -16,0% | -13,0% |
| 1080i25 frame mode | -15,1% | -18,5% | -19,0% | -15,8% |
| 1080i25 field mode | -13,0% | -17,0% | -18,9% | -14,0% |
| 1080p50 | -14,6% | -16,2% | -18,2% | -15,1% |
| 2160p50 | -16,6% | -16,7% | -19,4% | -16,8% |
| Progressive | -14,5% | -15,9% | -17,9% | -15,0% |
| Interlaced | -14,0% | -17,7% | -18,9% | -14,9% |
| All | -14,3% | -16,6% | -18,3% | -14,9% |

Table 3

|  | Random Access HE |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y | U | V | Avg |
| 720p50 | -20,7% | -29,5% | -28,7% | -22,1% |
| 1080i25 frame mode | -25,3% | -27,5% | -22,4% | -25,3% |
| 1080i25 field mode | -22,7% | -26,0% | -27,1% | -23,3% |
| 1080p50 | -25,6% | -32,8% | -30,9% | -26,6% |
| 2160p50 | -31,7% | -29,8% | -29,0% | -31,4% |
| Progressive | -26,0% | -30,7% | -29,6% | -26,7% |
| Interlaced | -24,0% | -26,8% | -24,7% | -24,3% |
| All | -25,2% | -29,1% | -27,6% | -25,7% |

Table 4

|  | Low delay P HE |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y | U | V | Avg |
| 720p50 | -27,3% | -35,7% | -35,2% | -28,5% |
| 1080i25 frame mode | -24,3% | -25,2% | -21,3% | -24,2% |
| 1080i25 field mode | -27,1% | -32,4% | -34,1% | -28,2% |
| 1080p50 | -28,1% | -34,1% | -33,9% | -29,0% |
| 2160p50 | -23,7% | -21,8% | -24,3% | -23,7% |
| Progressive | -26,4% | -30,5% | -31,1% | -27,1% |
| Interlaced | -25,7% | -28,8% | -27,7% | -26,2% |
| All | -26,1% | -29,8% | -29,7% | -26,7% |

Table 5

|  | Low delay B HE |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y | U | V | Avg |
| 720p50 | -30,8% | -38,7% | -38,2% | -32,0% |
| 1080i25 frame mode | -33,0% | -34,5% | -30,8% | -33,0% |
| 1080i25 field mode | -31,3% | -36,7% | -38,6% | -32,5% |
| 1080p50 | -32,5% | -38,1% | -38,0% | -33,3% |
| 2160p50 | -29,9% | -27,5% | -30,9% | -29,8% |
| Progressive | -31,1% | -34,8% | -35,7% | -31,7% |
| Interlaced | -32,2% | -35,6% | -34,7% | -32,7% |
| All | -31,5% | -35,1% | -35,3% | -32,1% |

# Subjective evaluations

In order to validate the objective measurements, preliminary subjective expert viewings sessions were performed on 720p50 video sequences so as to define at which bitrate the quality of the reference (AVC) and the tested videos (HEVC) were perceived comparable.

In particular, for the preliminary subjective expert viewings session, the sequences encoded with AVC at about 10 Mbps in random access configuration was used as reference respect to HEVC encoded materials at different bitrate ranging from 3 Mbps to 10 Mbps.

At the end of the session an average bitrate saving of 40% of HEVC against AVC was spotted.

# Conclusions

The experimental results indicated a reduction of the overall BD-Bitrate compared to that for common test materials [5] due to the fact that SVT sequences include video characteristics that were not present in common test materials.

Bit rate saving for the SVT sequences used are summarized in the following:

- Main Profile all-intra configuration, HM-7.0 can save about 15% in bit rate;

- Main Profile random-access configuration, HM-7.0 can save about 26% in bit rate;

- Main Profile low-delay configuration, HM-7.0 can save about 32% in bit rate.

Furthermore, our test confirmed that these results remain more or less constants when the resolution changes as already reported in [5].

Coding interlaced material as field mode introduces a huge loss in chroma compared to the progressive frame mode as reported in the following:

- Main Profile all-intra configuration: U about 27% of bit rate loss, V about 30% of bit rate loss

- Main Profile random-access configuration: U about 50% of bit rate loss, V about 45% of bit rate loss

- Main Profile low-delay configuration: U about 28% of bit rate loss, V about 31% of bit rate loss

Given these results, further investigation about how interlaced material will be processed by practical HEVC encoders implementation would be valuable.

In terms of preliminary subjective expert viewings results revealed an average bitrate saving of 40% in case of 720p50 material in random access configuration.

# Future works

Further comparisons (objective and subjective) will be carried out using the material available from the recent 4K EBU shooting.

# References

- JCTVC-I1100, “Common HM test conditions and software reference configurations,” Geneva, CH, 27 April – 7 May 2012.

- JCTVC-I1003, “High Efficiency Video Coding (HEVC) text specification draft 7,” Geneva, CH, 27 April – 7 May 2012.

- ftp://vqeg.its.bldrdoc.gov/HDTV/SVT_MultiFormat/

- “The SVT High Definition Multi Format Test Set”,
 ftp://vqeg.its.bldrdoc.gov/HDTV/SVT_MultiFormat/SVT_MultiFormat_v10.pdf, v1.0, Feb. 2006.

- JCTVC-J0236, “Comparison of Compression Performance of HEVC Draft 7 with AVC High Profile,” Stockholm, SE, 11-20 July 2012.