| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, China, October 2012 | Document: JCTVC-K0113 |
| --- | --- |

| Title: | The general structure and syntax of the slice extension |  |  |
| --- | --- | --- | --- |
| Status: | Input contribution |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | David Singer, Thanos Leontaris, Alexis Tourapis, Steve Zhou Apple Inc. Cupertino CA | Tel: Email: | +1 408 996 1010 singer@apple.com |
| Source: | Apple Inc. |  |  |

_____________________________

# Abstract

This contribution offers a general syntax for slice-extensions, and enables them to be found and used before entropy decoding of the rest of the slice. This enables pipelined transmission of data that can only be generated after encoding, yet is currently placed in the slice header or a preceding SEI message, which requires buffering before transmission.  Placing the data in the slice extension hence enables pipelined transmission and reduced delay.

This revision adds further options after discussion. See section 3 below.

# Introduction

In the recent Stockholm meeting, concern was expressed over the use of wavefront parallel processing when low delay is desired. One of the contributions to delay is the need to buffer the slice during encoding so that the entry points can be written into the slice header before transmission.

If, instead, the entry points can be transmitted after the encoded data, in the slice extension, and found when they arrive but before slice entropy decoding is complete, this delay may be avoided.

This contribution looks at the slice extension syntax and proposes a general structure for carrying slice extensions. A companion contribution then applies that for various cases, including the entry points.

Overall, this contribution proposes:

- permitting zero or more slice extensions, using a general structure for each, in a compact yet forward-compatible fashion;

- and enabling the location of those extensions before slice entropy decoding that starts at the slice beginning, is has reached them;

# Slice extension

## Formal syntax

We wish to enable multiple slice extension data elements, with forward compatibility, and be able to find them before we have forward-decoded the entire slice.

To find them, we need a back-pointer at the end of the slice. The back-pointer refers backward from itself to the beginning of the slice extensions.  To make this compact – particularly when no extensions are present, we encode this as a ue(v) (golomb coded). Unfortunately, these values can only be decoded and delimited when parsed from their first bit.  Since we desire being able to both forward-decode it, and also decode it when its last bit is found, we make it palindromic.

A palindromic 0-th order Golomb code has the string of initial zeroes and their terminating one bit, reflected as a string of a one bit followed by zeroes, and added to the end of the code. For example:

00…0001xx…xxx	becomes	00…0001xx…xxx1000…00

This is indicated by the pal-ue(v) syntax below, and a formal definition is given later.

We make the syntax future-proof by labelling each extension with a compact type indicator and a compact length indicator, and terminate the set with a tag-type value of 0.

In specific, replace:

| slice_layer_rbsp( ) { | Descriptor |
| --- | --- |
| slice_header( ) |  |
| slice_data( ) |  |
| if( slice_extension_present_flag ) { |  |
| slice_extention_flag /* (sic) */ | u(1) |
| if( slice_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| slice_extension_data_flag | u(1) |
| } |  |
| rbsp_slice_trailing_bits( ) |  |
| } |  |

with

| slice_layer_rbsp( ) { | Descriptor |
| --- | --- |
| slice_header( ) |  |
| slice_data( ) |  |
| if (slice_extension_present_flag) slice_extension() |  |
| rbsp_slice_trailing_bits( ) |  |
| } |  |

and define slice_extension as follows:

| slice_extension( ) { | Descriptor |
| --- | --- |
| encoded_length = 1 /* 1 is the number of bits occupied by the final 0 */ |  |
| se_idx = 0 /* slice extension index */ |  |
| slice_extension_block_tag[ se_idx ] | ue(v) |
| while (slice_extension_block_tag[ se _idx ] != 0) { |  |
| slice_extension_block_length[ se_idx ] | ue(v) |
| encoded_length += slice_extension_block_length[ se_idx ] + ue_length_of( slice_extension_block_tag[ se_idx ] ) + ue_length_of( slice_extension_block_length[ se_idx ] ); |  |
| slice_extension_block()[ se _idx ] | u(v) |
| se_idx++ |  |
| slice_extension_block_tag[ se_idx ] | ue(v) |
| } |  |
| if (encoded_length > 1) |  |
| extension_back_pointer = encoded_length-3 /* we assert that encoded_length must be >= 4 */ | pal-ue(v) |
| } |  |

The function ue_length_of(x) returns the number of bits that x occupies when encoded as a ue(v), as defined by the following pseudo-code (which works for k-th order codes as well when k==0, since the number of bits occupied by the two codes is then the same):

len = k = 0
absV = Abs( synVal )
stopLoop = 0
do {
	if( absV  >=  ( 1 << k ) ) {
		len++
		absV = absV − ( 1 << k )
		k++
	} else {
		len += 1 + k
		stopLoop = 1
	}
} while( !stopLoop )
return len

The palindromic code then has the following structure (this could be part of the slice-extension syntax above, since this is the only place it is currently used):

| pal-ue( x ) { | Descriptor |
| --- | --- |
| forward_code = x | ue(v) |
| code_stop_one_bit /* Always equal to 1 */ | f(1) |
| code_trailing_zero_bits /* Always equal to 0 */ /* v, the number of bits of 0, is (ue_length_of(x) >> 1) */ | u(v) |
| } |  |

The slice_extension_block_tag is an integer tag that indicates the syntax and meaning of the slice extension block.  The slice extension length indicates the length (in bits) of the block, and the extension back pointer is set to the value encoded_length, expressed as a palindromic ue(v).

Each slice_extension_block has the length indicated by the preceding slice_extension_block_length value.

Note particularly that in the common case of no extension data, encoded_length will be 1 and omitted. The last value in the stream will be the slice_extension_block_tag reading 0, which is the single bit ‘1’, which is also readable as saying that the back-pointer is absent (as any usable back-pointer will be greater than 1).

We can therefore subtract 3 from the palindromic back pointer value, when present, as any usable set of extension blocks will set encoded_length to at least 4:

- at least two bits for the non-zero tag of the block that is present;

- at least one bit for the length of that block;

- (maybe zero bits for that extension block, if its mere presence is the desired signal)

- one bit for the terminating tag value of 0.

After subtracting 3, we are assured that result will be 1 or greater, which, when expressed as a palindromic code will end with a 0 bit, thus distinguishing this case from the case when no extensions are present (signalled by the final 1 bit of the terminating tag with the value 0).

## Finding the extension ‘from the end’

The end of the slice looks like this:

| rbsp_slice_trailing_bits( ) { | Descriptor |
| --- | --- |
| rbsp_trailing_bits( ) |  |
| while( more_rbsp_trailing_data( ) ) |  |
| cabac_zero_word /* equal to 0x0000 */ | f(16) |
| } |  |

| rbsp_trailing_bits( ) { | Descriptor |
| --- | --- |
| rbsp_stop_one_bit /* equal to 1 */ | f(1) |
| while( !byte_aligned( ) ) |  |
| rbsp_alignment_zero_bit /* equal to 0 */ | f(1) |
| } |  |

In order to find the extensions from the end of a NAL unit, the following steps have to taken:

- (logically) remove start-code emulation prevention bytes (see below);

- find the end of the slice rbsp (in bytes), from the length field or the start-codes;

- while the last two bytes are zero, remove cabac_zero_words;

- while the last bit is zero, remove rbsp_alignment_zero_bit;

- remove rbsp_stop_one_bit;

- if the last bit is now ‘1’, there is no extension, so stop;

- otherwise the last bit you have in your hand is the first bit (reverse stored) of the unary-coded length of the palindromic code; accumulate 0s to a 1, as usual, and then step back that distance, and decode the back-pointer.

Then you have the back-pointer, and can step back that number of bits, and decode forwards.

## Start-code emulation prevention

This is somewhat complicated by the existence of start-code emulation prevention.  Logically, emulation prevention needs removing before the steps in the section above.

However, many decoders prefer to remove start-code emulation prevention bytes in the process of reading bits from the stream. There are many ways to do this; one would be to make a conservative estimate of the length of the extension (say, 32 bytes) and scan the last 32 bytes for start-code emulation (and build a ‘map’ or remove the bytes), and then run the algorithm above. If the back-pointer is within that range, the process terminates. Otherwise re-estimate the start point of the extensions, and check that range, and so on.

## Detailed Edits

In 7.4.2.3, adjust the text as follows:

slice_extension_present_flag equal to 0 specifies that no slice extension syntax elements are present in the slice layer RBSP for coded pictures referring to the picture parameter set. slice_extension_present_flag equal to 1 indicates that slice_extensions are present in the slice layer RBSP for coded pictures referring to the picture parameter set.slice_extension_present_flag shall be equal to 0 in bitstreams conforming to this Recommendation | International Standard. The value of 1 for slice_extension_present_flag is reserved for future use by ITU-T | ISO/IEC.

NOTE – when present, a slice extension may be only a single bit, indicating that no slice extension blocks are present in this slice extension.

Replace the text of 7.4.2.11 with the following

The slice layer RBSP consists of a slice header, slice data, and possibly slice extension data.

The slice extension is formatted as a series of slice extension blocks, each tagged with a tag-type. The value 0 (zero) as tag-type indicates the end of the series. If there is a block other than this terminating zero tag, then the series is followed by a value indicating the number of bits in the slice extension, encoded as a palindromic golomb code, as defined [above | here | below | in section XXX].

The following values are defined for slice extension tags:

[ TBD, see separate contribution(s) ]

# Further Options

## General

There was a request that extension blocks not have a declared length, but instead that they are parsed and their length found by recognizing the tag. This means that any unrecognized tag will force parsing to abort, but this should not happen as the permitted tags would be controlled by profiles.

There were also suggestions that any extension should be verifiable – that if there are bits after the slice data, there should be a way to determine that they are deliberate and not a mistake. We could have a signature, but the back-pointer provides that verification – the extension has just enough required internal self-consistency to be verifiable.

In both cases, when the slice_extension_present_flag is false, this reduces to today’s syntax.

## Byte-aligned extension start and end

In this variant, we byte-align after the slice data, so we have a byte-aligned extension, and the extension also needs to finish byte-aligned. That means an extension is always at least one byte, so we need not only the parameter set flag saying whether extensions are ever present, but also a flag to say whether this slice has an extension – we don’t want to waste a byte in all the slices with no extension. That flag has to be in the slice header.

So add to the slice header

if (slice_extension_present_flag) 
	slice_extension_flag 	f(1)

Define the slice as

| slice_layer_rbsp( ) { | Descriptor |
| --- | --- |
| slice_header( ) |  |
| slice_data( ) |  |
| if (slice_extension_flag) slice_extension( ) |  |
| rbsp_slice_trailing_bits( ) |  |
| } |  |

and define slice_extension as follows:

| slice_extension( ) { | Descriptor |
| --- | --- |
| byte_alignment( ) /* the initial 1 bit terminates the cabac */ |  |
| encoded_length = 1 /* 1 is the number of bits occupied by the final 0 */ |  |
| se_idx = 0 /* slice extension index */ |  |
| slice_extension_block_tag[ se_idx ] | ue(v) |
| while (slice_extension_block_tag[ se _idx ] != 0) { |  |
| slice_extension_block[ se _idx ] | u(v) |
| encoded_length += bit_length_of(slice_extension_block[ se _idx ]) ue_length_of( slice_extension_block_tag[ se_idx ] ) ; |  |
| se_idx++ |  |
| slice_extension_block_tag[ se_idx ] | ue(v) |
| } |  |
| if (encoded_length > 1) |  |
| extension_back_pointer /* must be encoded_length-1 */ | pal_ue(v) |
| } |  |

We should optimize the back pointer to be a byte-count with a bit of careful maths (right-shift and rounding of the encoded_length-1).

## Byte-align at end of slice only

This variant does not byte-align twice, but only at slice end.

Define a slice as (this is essentially the same, without the slice header flag)

| slice_layer_rbsp( ) { | Descriptor |
| --- | --- |
| slice_header( ) |  |
| slice_data( ) |  |
| if (slice_extension_present_flag) slice_extension( ) |  |
| rbsp_slice_trailing_bits( ) |  |
| } |  |

and define slice_extension as follows:

| slice_extension( ) { | Descriptor |
| --- | --- |
| cabac_stop_one_bit /* equal to 1 */ | f(1) |
| encoded_length = 1 /* 1 is the number of bits occupied by the final 0 */ |  |
| se_idx = 0 /* slice extension index */ |  |
| slice_extension_block_tag[ se_idx ] | ue(v) |
| while (slice_extension_block_tag[ se _idx ] != 0) { |  |
| slice_extension_block[ se _idx ] | u(v) |
| encoded_length += bit_length_of(slice_extension_block[ se _idx ]) ue_length_of( slice_extension_block_tag[ se_idx ] ) ; |  |
| se_idx++ |  |
| slice_extension_block_tag[ se_idx ] | ue(v) |
| } |  |
| if (encoded_length > 1) |  |
| extension_back_pointer /* must be encoded_length-1 */ | pal_ue(v) |
| } |  |

In the case where the slice_extension_present_flag is true, but this slice does not use an extension, it reduces to two bits equal to 1 (the cabac stop one bit and the tag with value 0).

The back-pointer can be found by discarding zero bits and bytes until the last one bit, discarding that, and looking for the preceding value. If it’s 0, then there is no extension, because this must be the tag of 0 you have found, as any extension is longer than 1 bit.

This design does not use whole bytes for extensions, and does not double byte-align, and does not need another flag in the slice header. It’s much tighter for today, and more compact in future. The parser does not need to check the header, in order to check from slice end for a slice extension, just the parameter set flag.

## Don’t give the extension ‘structure’ now

If we don’t have an immediate use for the extension, then we could omit documenting the tag/block structure now, and leave that to later.  So, for example, in option 2, we’d write

| slice_extension( ) { | Descriptor |
| --- | --- |
| cabac_stop_one_bit /* equal to 1 */ | f(1) |
| slice_extension_block | u(v) |
| encoded_length = 1+ bit_length_of(slice_extension_block); |  |
| if (encoded_length > 1) |  |
| extension_back_pointer /* must be encoded_length-1 */ | pal_ue(v) |
| } |  |

This retains the back-pointer.  One can still verify; you have to do the scan from the end to find the back-pointer, and verify it points to where the cabac_stop_one_bit was found when parsing forward.

(A similar variant to option 1 is also possible).

# Proposal

Specifically we propose:

- adopt one of these slice extension syntaxes, with the blocks, and the back-pointer; we suggest one of the non-byte-aligned options.

# Patent rights declaration(s)

Apple Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).