| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, China, October 2012 | Document: JCTVC-K0114 |
| --- | --- |

| Title: | Specific Uses of the Slice Extension |  |  |
| --- | --- | --- | --- |
| Status: | Input contribution |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | David Singer, Thanos Leontaris, Alexis Tourapis, Steve Zhou Apple Inc. Cupertino CA | Tel: Email: | +1 408 996 1010 singer@apple.com |
| Source: | Apple Inc. |  |  |

_____________________________

# Abstract

This contribution builds on the structure for slice extensions, to enable the transmission of entry point offsets (as used in wavefronts and tiles) to be sent in the slice extension. This avoids buffering the slice header to insert the entry points before transmission, enables potential pipelining of encoding and transmission, and hence can reduce delay when these techniques are used.

# Introduction

This contribution builds on the general syntax proposed in JCTVC-K0113, for specific cases.

In the recent Stockholm meeting, concern was expressed over the use of wavefront parallel processing when low delay is desired. One of the contributions to delay is the need to buffer the slice during encoding so that the entry points can be written into the slice header before transmission.

If, instead, the entry points can be transmitted in the slice extension, and found when they arrive but before slice entropy decoding is complete, this delay may be avoided.

When this is present, a decoder capable of parallel operation may start a single thread decoding, and then if the entry points arrive before that thread is complete, start threads for those entry points that have not yet been reached. If decoding is complete, then it is transmission, not decoding, that is dominating delay, and the loss of parallel decode is not serious.

Overall, therefore, this contribution proposes documenting the use of one form of slice extension for the transmission of wavefront or tile entry point positions.

What happens now at the receiver?

- for receivers that load from disk, or otherwise get the whole NAL unit in one go, and which are not using start-codes, the length of the NAL gives you the position of the back-pointer, and you are off;

- if you are using start-codes, you'll need to scan for the next start-code to find the end of this NAL (you probably do anyway, to put the slice into a sized buffer);

- if you are streaming (incremental reception), you can start decoding as data arrives – you just can't start parallel processes until the index arrives at the end;

- if you are decoding (single-thread) faster than the data arrival rate, you'll never start a thread, but this is probably the most efficient anyway (you don't power up a second core);

- otherwise, when the index arrives, you start threads for all the lines that haven't been started yet.  This is the only place where there is a downside; you did single-thread decoding for a while, waiting for the index to arrive, and so you were a bit slower. But overall as a system your latency is still lower (potentially) as the encoder was sending you data much sooner – as it was encoded.

We note that this general structure could also be used for other information that can only be computed by an encoder after encoding, and explore some of those possibilities also.

# Entry point slice extension

## Formal syntax

In the slice_header, the entry points are encoded as follows:

| if( tiles_enabled_flag ¦ ¦ entropy_coding_sync_enabled_flag ) { |  |
| --- | --- |
| num_entry_point_offsets | ue(v) |
| if( num_entry_point_offsets > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_entry_point_offsets; i++ ) |  |
| entry_point_offset[ i ] | u(v) |
| } |  |
| } |  |

We now define the structure of slice_extension_block( ) when slice_extension_tag is the value indicating entry_point_offsets.  We suggest the value 1, as it is the next available value and codes compactly.

We use the notation slice_extension_block( n ) to indicate the syntax of a slice_extension_block when the slice_extension_tag is the value n.

Since we have the length (in bits) of the block, we do not need an explicit count; we can rely on consuming all the bits:

| slice_extension_block( 1 ) { | Descriptor |
| --- | --- |
| offset_len_minus1 | ue(v) |
| num_entry_point_offsets = 0; |  |
| while (extension_data_remaining()) { |  |
| entry_point_offset[ i ] | u(v) |
| num_entry_point_offsets++; |  |
| } |  |
| } |  |

This encodes each entry_point in the same number of bits; it is suitable for indexed access, therefore. However, it’s not clear that we need indexed access to the array. If we make the reasonable assumption that each block is roughly the same length, then the deltas between the lengths might well provide a more compact coding, at the expense of indexed access:

| slice_extension_ block ( 1 ) { | Descriptor |
| --- | --- |
| i = 0; |  |
| while (extension_data_remaining()) { |  |
| if (i == 0) entry_point_offset[0] | ue(v) |
| else entry_point_offset_delta[ i ] | se(v) |
| entry_point_offset[i] = entry_point_offset[ i – 1] + entry_point_offset_delta[ i ]; |  |
| i++; |  |
| } |  |
| num_entry_point_offsets = i; |  |
| } |  |

In this variant, the first offset is golomb-coded unsigned (difference from zero), and then all the rest are coded as the difference from the previous length (signed).

## Usage

With this structure possible, we propose allowing that the num_entry_point_offsets in the slice header be permitted to be zero even when there are entry_points defined. Note that the entry point data is not required for normal forward decoding: the decoder knows when it has come to the end of a line or tile, and must reset the entropy decoder.

We propose permitting the presence of the slice extension documentation of entry points whether or not they were in the slice header, i.e. all possibilities are permitted: undocumented, header only, extension only, header and extension.

# Other slice extension blocks

## Introduction

This general design for the slice extension can be used to cover all cases where there is information currently in the header which can only be written after encoding is complete (not just entropy decoding entry points). This includes information to assist de-blocking, sample-adaptive-offset filtering, and so on. This section points out two other possibilities.

## De-blocking parameters

The current slice-header has the following de-blocking information. Under some circumstances it may be that these values are computed after encoding the slice, which again introduces delay.

if( deblocking_filter_control_present_flag ) {

if( deblocking_filter_override_enabled_flag )

deblocking_filter_override_flag

if( deblocking_filter_override_flag ) {

slice_header_disable_deblocking_filter_flag

if( !slice_header_disable_deblocking_filter_flag ) {

beta_offset_div2

tc_offset_div2

}

}

}

The slice extension syntax for this would be:

| slice_extension_block( 2 ) { | Descriptor |
| --- | --- |
| slice_extension_disable_deblocking_filter_flag | u(1) |
| if( !slice_extension_disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| } |  |
| } |  |

Following the current practice, the slice extension overrides the slice header, which in turn overrides the picture parameter set.

## Checksums and Signatures

The third case that is useful is checksums and signatures. These must be computed based on the actual compressed bits of a slice, yet at the moment are in an SEI – which, by rule, must precede the slice NAL unit. SEIs are also slightly more vulnerable to loss. We could define slice extension blocks for checksums and/or signatures of the slice up to but not including the slice_extension.

# Proposal

Specifically we propose:

- define a slice extension block that carries entry point offsets, using one of the forms above

- permit the entry point offset array in the slice header to be empty (count of 0), whether or not the extension entry point block is present

We think the JCT should consider:

- would the standard benefit from other slice extension blocks, particularly the de-blocking information?

# Patent rights declaration(s)

Apple Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).